3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.3

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-IAB-03]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-IAB-03] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-IAB-03] Email discussion/approval of proposal 4 in R1-2001866 till 4/24 – Ralf (ATT)

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-IAB-03] “Email discussion/approval of Proposal 4 in R1-2001866” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-IAB-03]

The following is the proposal in Error: Reference source not found for approval in this email discussion:

FL Proposal 4 (high priority): Support the following new feature group

| 20-8 | Guard symbols | 1) Support DesiredGuardSymbols reporting 2) Support ProvidedGuardSymbols reception |  | Yes | N/A | Guard symbols reporting and reception is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Companies are asked to provide their views and comments in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei | We support the proposal. |

…

# Conclusions

…

# References

- R1-2001866, Summary on UE features for IAB, Moderator (AT&T)