3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-01]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-eMIMO-01]  during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-eMIMO-01] Email discussion/approval till 4/24 – Ralf (ATT), (16-1 family) - Decide the number of FGs for “L1-SINR reporting” by down-selecting between - Alt. 1-1: one FG - Alt. 1-2: two FGs - Alt. 1-3: three FGs - Decide the number of FGs for “TCI state activation and spatial relation update” by down-selecting between - Alt. 2-1: one FG - Alt. 2-2: three FGs - Decide if “Resources for beam management, pathloss measurement, BFD, and BFR” is a feature group - Alt. 3-1: no dedicated FG - Alt. 3-2: one FG

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-eMIMO-01]  “Email discussion/approval” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-01]

The following is the proposal in Error: Reference source not found for discussion in this email approval:

FL Proposal 1: (16-1 family)

- Decide the number of FGs for “L1-SINR reporting” by down-selecting between

  - Alt. 1-1: one FG

  - Alt. 1-2: two FGs

  - Alt. 1-3: three FGs

- Decide the number of FGs for “TCI state activation and spatial relation update” by down-selecting between

  - Alt. 2-1: one FG

  - Alt. 2-2: three FGs

- Decide if “Resources for beam management, pathloss measurement, BFD, and BFR” is a feature group

  - Alt. 3-1: no dedicated FG

  - Alt. 3-2: one FG

## L1-SINR reporting

The following are the alternatives in Error: Reference source not found for discussion in this email approval for “L1-SINR reporting”:

Alt. 1:

| 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) - FFS: Support of group-based reporting for L1-SINR | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1h | Group-based reporting for L1-SINR | Support of group-based reporting for L1-SINR | 16-1a |  | N/A |  | Per band |  |  |  |  |  |

Alt. 2:

| 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) - FFS: Support of group-based reporting for L1-SINR - Supported RS for CMR/IMR configurations for L1-SINR - 2. Max. number of reference signal configurations for CMR and IMR for L1-SINR across all CCs - 3. Max. number of reference signal configurations for CMR and IMR for L-SINR within a slot - 4. Max. number of reference signal configurations for CMR for L1-SINR and L1-RSRP | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 3:

| 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or - Support of L1-SINR based beam measurement and reporting based on NZP IMR (FFS details on the sub-components, e.g., FG 2-24) - FFS: Support of group-based reporting for L1-SINR | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 4:

| 16-1a | L1-SINR reporting | - Support of The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) - FFS: Support of group-based reporting for L1-SINR | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 5:

| 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) - FFS: Support of group-based reporting for L1-SINR | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1h | Group-based reporting for L1-SINR | Support of group-based reporting for L1-SINR | 16-1a |  |  |  |  |  |  |  |  |  |
| 16-1i | Non-group-based reporting for L1-SINR | Support of non-group based reporting for L1-SINR | 16-1a |  |  |  |  |  |  |  |  |  |

Alt. 6:

| 16-1a | L1-SINR reporting | - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot - The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot - Supported density of CSI-RS for CMR to measure L1-SINR - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as ZP IMR to measure L1-SINR within a slot - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as ZP IMR to measure L1-SINR - The max number of CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as NZP IMR to measure L1-SINR within a slot - The max number of CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as NZP IMR to measure L1-SINR | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 7:

| 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) Supported type of interference measurement resource - Supported type of dedicated IMR - Max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR for L1-SINR - Max number of NZP CSI-RS and CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as IMR for L1-SINR - Maximum number of L1-SINR report setting per BWP - FFS: Support of group-based reporting for L1-SINR | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 8:

| 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) - L1-SINR based on CMR without dedicated IMR - L1-SINR based on CSI-RS as CMR and dedicated ZP IMR - L1-SINR based on CSI-RS as CMR and dedicated NZP IMR - L1-SINR based on SSB as CMR and dedicated ZP IMR - L1-SINR based on SSB as CMR and dedicated NZP IMR - FFS: Support of group-based reporting for L1-SINR for each supported component | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Looking at the comments in [1], a majority of companies supports alternatives with one row/FG. Note that the company supporting Alt. 5 with three rows is also supporting Alt. 3 with a single row.

| Alternative | Number rows/FGs | Supporting company |
| --- | --- | --- |
| 1 | 2 | 3 |
| 2 | 1 |  |
| 3 | 1 | 2 |
| 4 | 1 | 1 |
| 5 | 3 | 1 |
| 6 | 1 | 4 |
| 7 | 1 | 1 |
| 8 | 1 | 3 |

Can we agree on a single row/FG for “L1-SINR reporting”?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We would be OK with a single row as well – as soon as a UE supports L1-SINR reporting, it would also group-based reporting for L1-SINR. This is different from how the UE capabilities for L1-RSRP reporting are structured, but we are OK with that. |
| Apple | Following Rel-15, we prefer to have 3 FGs FG1: RS related capability for L1-SINR similar as FG2-24. FG2: Support of non-group based RSRP reporting with N_max RSRP values reported similar as FG2-29 FG3: group based L1-SINR similar as FG2-29a |
| Intel | Given the uncertainty on the reporting granularity consider 3 FGs as follows: - Type of L1-SINR measurements and supported number of RS across CC and within a slot - TBD components - Number of L1-SINR report for non group L1-SINR report - Group based L1-SINR reporting Note: Need to consider the total number of RS for L1-RSRP and L1-SINR |
| LGE | No strong view on the number of FGs, but we think that it will be good if we simply copy FGs and features from L1-RSRP as baseline. Otherwise, it will be very hard to converge the views. |
| MediaTek | We are not OK with single row. We are OK with either 2 FGs or 3 FGs. If there are 2 FGs, they are - RS related capability for L1-SINR including both “total number of configurations across cells” and “number of measurements to perform within a slot” - Support of group-based L1-SINR If there are 3 FGs, they are - RS related capability for L1-SINR including both “total number of configurations across cells” and “number of measurements to perform within a slot” - Support of non-group-based L1-SINR - Support of group-based L1-SINR |
| OPPO | We propose to follow the L1-RSRP FGs defined in Rel15 to have 3 FGs for L1-SINR. |
| ZTE/Sanechips | We can additionally consider the Rel-15 L1-RSRP FGs to be reused for L1-SINR, e.g., 3 FGs. If going with 2 FGs, as we proposed in Alt1, the number of RSs to be measured and support of group based reporting should be considered. The maximum number of L1-SINR values does not need to be further reported, i.e., up to 4 as defined in current spec. |
| vivo | We are ok with Alt 8. |
| Huawei, HiSilicon | We are supportive to follow Rel-15 design principle, i.e., having 3~4 FGs. Except for RS/group-reporting related FGs, we want to remark that in R15 UE capability on reporting settings is also separately reported. |
| Qualcomm | Support at most 2 FGs including RS type related capability and group-based report capability. They can also merge into one FG. The resource # and time density are better limited by one general limit in 16-1g for all types of measurement. The single general limit is fundamentally what needed by the UE, and also allows gNB to configure more/less resources for some measurements as long as the total general limit is satisfied. Support at least the combination of SSB as CMR + CSI-RS as IMR to be optional: At least for our architecture, this CMR/IMR combination is harder for implementation because SSB and CSI-RS can have different periods, which are not friendly for CMR/IMR pairing. At each reporting time, UE needs to search the latest SSB as CMR to pair with the IMR to compute the corresponding L1-SINR. To do the search, memory needs to be reserved to store previous SSBs. For CSI-RS as both CMR and IMR, the pairing is simpler, since they have same period, and UE knows the exact location based on the configuration without the need for search every time. It would be great to make at least SSB+CSI-RS to be optional, so more UEs can support L1-SINR. Perhaps other UE vendors may encounter similar issue in future. |
| Nokia, NSB | Single row is preferred |
| CATT | Support group-based reporting as a UE capability, so alt-8 is acceptable, but not alt-6. |

If so, can we agree on either Alt. 6 or Alt. 8 as new baseline/reference for further discussion on detailed component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, notes, consequences if a feature is not supported by a UE, and prerequisites?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Alt 8 and in particular alt 6 are written in a way that it looks like the components are signaled independently. In our understanding, this is not possible. One FG is signaled with one value. For L1-SINR, this should be either “support/not support” or alternatively with on value “maximum number of L1-SINR values signaled”, as for L1-RSRP |
| Apple | We prefer Alt. 6 with additional FG FG2: Support of non-group based RSRP reporting with N_max RSRP values reported similar as FG2-29 FG3: group based L1-SINR similar as FG2-29a |
| Intel | We propose to agree in high level the contents of FG. According to our preference it should be as follows: - Type of L1-SINR measurements and supported number of RS across CC and within a slot - TBD components - Number of L1-SINR report for non group L1-SINR report - Group based L1-SINR reporting |
| LGE | Regarding the way of making a progress, we think that the baseline should be the version what we have agreed so far, i.e. neither Alt6 nor Alt8. Then we need to discuss whether/how to split the existing components. In my observation, most companies are supportive on that the first component needs to be split with regards to the type of IMR (ZP IMR or NZP IMR), which is common for Alt2, Alt3, Alt6, Alt7 and Alt8. So, we suggest starting from Alt3 as a common ground for Alt2, Alt3, Alt6, Alt7 and Alt8 |
| MediaTek | We are OK with Alt.6 if additional FG about “support of group-based SINR” is added |
| OPPO | We propose to combine Alt 5 and Alt 6 by merging Alt6 into the first FG in Alt5. |
| ZTE/Sanechips | We can NOT support Alt.6 or Alt.8. FG2-24 should be assumed as baseline, and we should strive to unify the L1-SINR FGs/components with L1-RSRP FGs/components as much as possible. |
| Qualcomm | Prefer Alt. 8. The resource # and time density can be limited by a general limit as in 16-1g. This way is more efficient than individual limit per FG, especially when # of measurement types increases. |
| Nokia, NSB | We would prefer alt 3 as the baseline, and we can build on that if needed. But we would be fine to start discussion from alt 8 if needed. |
| CATT | We prefer group-based reporting to be a UE capability, either in the same or different FG. Hence OK with Alt.8, but not alt-6. Reusing Rel.15 L1-RSRP structure is also OK. |

If not, what would have to fundamentally change to agree the number of rows/FGs for the above proposal (new FG(s), new components, delete components, add components, move components to which other FG …)?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Based on the feedback in the previous tables, it appears thus far the majority view is to have more than one FG. On this line, maybe we can start from the suggestions from Apply/Intel. |

The original FL Proposal 1 included for “L1 SINR reporting”:

FL Proposal 1: (16-1 family)

- Decide the number of FGs for “L1-SINR reporting” by down-selecting between

  - Alt. 1-1: one FG

  - Alt. 1-2: two FGs

  - Alt. 1-3: three FGs

During the RAN1 #100bis-e conference call on April 22, 2020 the following agreement was reached:

Working assumption: There will be three FGs for L1-SINR reporting structured similarly to R15 L1-RSRP

- RS related capability

- Number of non-group based L1-SINR reports

- Group-based L1 SINR report

Note: candidate component values and other details to be discussed in second round

Thus, Alt. 1-3 of the FL Proposal 1 was agreed. We now have to work on the details. Since it’s hard to have tables within tables I suggest to solely focus on component descriptions and component candidate values, as necessary. Please also stay as close as possible to the Rel. 15 L1-RSRP FG to ensure commonality across companies which will make merging for an agreement easier.

Draft feature list from FL (Apple):

Continue discussion by email using R15 L1-RSRP as starting point for R16 L1-SINR description

- Apple to share wording for FL to update email discussion summary in Inbox

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Per band |  |  |  |  | Optional with capability signalling Component-1 to 4, candidate value set for MB_1 and MB_1-x is {0, 8, 16, 32, 64} Component-5 to 8, candidate value set for MC_1 and MC_1-x is {0, 4, 8, 16, 32, 64} Component-9, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-10: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} Component-11, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1a-2 | Non-group based L1-SINR reporting | Support of non-group based L1-SINR reporting with N_max L1-SINR values reported | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling Candidate value set is {1, 2, 4} |
| 16-1a-3 | Group based L1-SINR reporting | Support of group based L1-SINR reporting | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We support to confirm working assumption provided that the detailed description is up for further discussion On clarification is that for the second sub-bullet, t is about the maximum number of L1-SINR UE can report for one non-group based L1-SINR reporting |
| MediaTek | For the bullet of “RS related capability” in the working assumption, use Alt. 6 as shown below, which is quite similar to Rel-15 L1-RSRP as guided by FL. Alt. 6: - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot - The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot - Supported density of CSI-RS for CMR to measure L1-SINR - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as ZP IMR to measure L1-SINR within a slot - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as ZP IMR to measure L1-SINR - The max number of CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as NZP IMR to measure L1-SINR within a slot - The max number of CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as NZP IMR to measure L1-SINR. For the bullet of “Number of non-group based L1-SINR reports” in the working assumption, reword it as “The maximum number of SSBRI or CRI for one non-group based L1-SINR report” For the bullet of “Group-based L1 SINR report” in the working assumption, reword it as “Support of group-based L1 SINR report” |
| Huawei, HiSilicon | To facilitate the discussions, we copied the draft feature list from the FL (Apple) to above this table of comments. As discussed in the 1st online session, to mimic the Rel-15 UE capabilities in components 2/4/7 of FG 2-35, we suggest adding the following components to the 1st FG: - Maximum number of periodic CSI report setting per BWP for beam report with L1-SINR - Maximum number of aperiodic CSI report setting per BWP for beam report with L1-SINR - Maximum number of semi-persistent CSI report setting per BWP for beam report with L1-SINR |
| Intel | 16-1a-1 The current proposed list of components is a bit detailed and some split is not very clear to us. We propose to start from more high-level structure. In particular we think that the following components needs to be considered. 1. Type of L1-SINR measurements with candidate values TBD 2. The number of configured resources for CMR and IMR across CCs - The number of configured SS/PBCH resources - The number of configured NZP CSI-RS resources for CMR - The number of configured NZP CSI-RS resources for IMR - The number of configured CSI-IM resource for IMR 3. The number of configured resources for CMR within a slot - The number of configured SS/PBCH resources - The number of configured NZP CSI-RS resources for CMR Note: The RAN1 should also consider the UE capability restricting the total number of RS for L1-RSRP and L1-SINR |
| ZTE | We support to confirm working assumption with the following modification. Besides, the detailed component design for RS related capability can be further studied. There will be three FGs for L1-SINR reporting structured similarly to R15 L1-RSRP - RS related capability - Maximum number of L1-SINR values in a non-group based L1-SINR report - Support of group-based L1 SINR report |
| Ericsson | We support confirming the WA. ZTE’s modification is fine to us. The proposal from Intel seems like a good starting point. |
| vivo | We would like to point out that when the same NZP CSI-RS is used as CMR and IMR simultaneously, the number should also be bounded with a separate capability. The report of number of resources may depend on supported CMR/IMR combination type. Thus the candidate values may need to have zero. We would like to support the proposal from Huawei that number of CSI reporting settings for L1-SINR should also be reported. |
| CATT | Support confirmation of the working assumption. Fine with ZTE’s modification. |

## TCI state activation and spatial relation update

The following are the alternatives in Error: Reference source not found for discussion in this email approval for “TCI state activation and spatial relation update”:

Alt. 1:

| 16-1b | TCI state activation and spatial relation update | - [Support of / maximum number of lists for] Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH (FFS whether to be a separate UE feature, e.g. 16-1b) - [Support of / maximum number of lists for] Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - [Support of / The maximum number of] PUCCH resource groups per BWP for simultaneous spatial relation update - Maximum number of PUCCH resources within each PUCCH resource group - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-1, 2-4 Component 2: 2-59, 2-60 Component 3: 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y FR2 only for component 1, 2, 4, 5 |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 2:

| 16-1b | TCI state activation and spatial relation update | - [Support of / maximum number of lists for] Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH (FFS whether to be a separate UE feature, e.g. 16-1b) - [Support of / maximum number of lists for] Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - [Support of / The maximum number of] PUCCH resource groups per BWP for simultaneous spatial relation update - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-1, 2-4 Component 2: 2-59, 2-60 Component 3: 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-59, 2-60 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
| 16-1b-3 | Spatial relation update for PUCCH group | Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |

Alt. 3:

| 16-1b | TCI state activation and spatial relation update | - [Support of / maximum number of lists for] Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH (FFS whether to be a separate UE feature, e.g. 16-1b) - [Support of / maximum number of lists for] Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - [Support of / The maximum number of] PUCCH resource groups per BWP for simultaneous spatial relation update - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-1, 2-4 Component 2: 2-59, 2-60 Component 3: 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1b1 | Cross-CC TCI state activation | Support of simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH | 2-1, 2-4 |  | N/A |  | Per band | N/A | N/A |  |  | TBD |
| 16-1b2 | Cross-CC spatial relation update | Support of simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS | 2-59, 2-60 |  | N/A |  | Per band | N/A | N/A |  |  | TBD |
| 16-1b3 | PUCCH resource groups | Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | Per band | N/A | N/A |  |  | TBD |

Out of 11 companies only two supported Alt. 1 in [1]. All others either supported Alt. 2 or Alt. 3 both of which require three rows/FGs.

Can we agree on three rows/FGs for “TCI state activation and spatial relation update”?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Yes |
| Apple | Yes |
| Intel | Yes |
| LGE | Support |
| MediaTek | Yes |
| OPPO | Yes, support to use three FGs |
| ZTE/Sanechips | Yes |
| Huawei, HiSilicon | Prefer single row, but can live with three rows. |
| Qualcomm | Either single or 3 rows is fine. |
| Nokia, NSB | It is OK to use three FGs. |
| CATT | Yes |

If so, can we agree on either Alt. 2 or Alt. 3 as new baseline/reference for further discussion on detailed component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, notes, consequences if a feature is not supported by a UE, and prerequisites?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Yes. Either 2 or 3 can be used as baseline. The difference may be handled by subsequently agreeing on per band or per BC |
| Yes | We prefer Alt. 2 |
| Intel | Alt 2 and Alt 3 looks similar. We have slight preference on Alt 3. |
| LGE | Either Alt2 or Alt3 are fine as a starting point since the only difference is the two FFS points. We are supportive for the FFS parts since gNB can group the CCs based on the UE capability information. |
| MediaTek | We support Alt. 2 |
| OPPO | We prefer Alt3. Why is the first row in Alt 2 called “TCI state activation and spatial relation update”. Updating TCI and spatial relation shall be different FG. |
| ZTE/Sanechips | We support Alt. 2. Per band or per BC should be determined later. Meanwhile, the title of first bullet of Alt2 can be updated as “TCI state activation and spatial relation update across multiple CCs” |
| vivo | We are ok with three rows. However we would like to clarify the understanding regarding number of TCI states and spatial relation with different spatial parameters in multiple CCs. Our understanding is this is one of the major intentions for this feature. |
| Huawei, HiSilicon | Prefer Alt 2, where the title of 16-1b should be revised and maximum number of PUCCH resources within one group should be added to 16-1b-3. Per-band reporting in Alt 3 cannot support inter-band case, which has deviated from previous agreement. We agree with vivo that clarification on the number of different active TCI states and spatial relations over multiple CCs is needed. |
| Qualcomm | Support Alt.2. The FFS for UE feedback of band combinations sharing same QCL is a critical input for this feature to work in case of inter-band CA. Support UE feedback of bands sharing same QCL/analog beam. This input can be used for gNB to determine the CC list in case of inter-band CA, since the most reasonable gNB implementation is to select same TCI state ID corresponding to same QCL-TypeD, i.e. same UE Rx beam, across those CCs. This is not only saving the MAC-CE overhead, but also avoid scheduling different QCL-TypeD across those CCs. The latter is more critical for UE side. This feedback also does not mandate gNB must configure same QCL-D to all CCs in the list, but just a recommendation to help mitigate the QCL mismatch at UE. |
| CATT | Slight preference on Alt.3, but alt-2 is acceptable. |

If not, what would have to fundamentally change to agree the number of rows/FGs for the above proposal (new FG(s), new components, delete components, add components, move components to which other FG …)?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | We would like to clarify the understanding regarding number of TCI states and spatial relation with different spatial parameters in multiple CCs. Our understanding is this is one of the major intentions for this feature. |

During the RAN1 #100bis-e conference call on April 22, 2020 the following agreement was reached:

Working assumption: Assume previous FG 16-1b requires three FGs, use Alt. 2 for further email discussion

FFS: maximum component values and all other details FFS

With Alt. 2 as starting point and reference, please work on finalizing the following three FGs.

| 16-1b | TCI state activation and spatial relation update | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state | Component 1: 2-1, 2-4 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-59, 2-60 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
| 16-1b-3 | Spatial relation update for PUCCH group | - Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We support to confirm working assumption provided that the detailed description is up for further discussion |
| MediaTek | We support working assumption. |
| Huawei, HiSilicon | FG 16-1b: Update the index as ‘16-1b-1’, and the name as ‘TCI state update across multiple CCs’. Mark as ‘per BC’ and remove FFS point. FG 16-1b-2: Mark as ‘per BC’ and remove FFS point. FG 16-1b-3: Add additional component of ‘The max number of PUCCH resources within one PUCCH resource group’. Mark as ‘per band’. |
| Intel | 16-1b. Remove FFS. Granularity is BC. 16-1b-2 Remove FFS. Granularity is BC and only applicable to FR2. |
| ZTE | We support to confirm the working assumption except that the title of “16-1b TCI state activation and spatial relation update” should be revised as “16-1b-1 TCI state activation and spatial relation update across multiple CCs” We can NOT support “FFS” this meeting, considering that it may have RAN4 impact. |
| Ericsson | Support to confirm WA. The FFSs can be removed. This is only a signaling feature, and it has no impact on RF. Considering that this is a baseband feature (functionality in MAC) we propose to make it “per UE” |
| vivo | 16-1b/16-1b-2: we support the intention of FFS. We would also like to report number of different TCI states and spatial relation UE supports across a CC list. |
| CATT | 16-1b: Remove FFS. Granularity is BC. 16-1b-2: Remove FFS. Granularity is BC. |

## Resources for beam management, pathloss measurement, BFD, and BFR

The following are the alternatives in Error: Reference source not found for discussion in this email approval for “Resources for beam management, pathloss measurement, BFD, and BFR”:

Alt. 1: Delete FG 16-1g

Alt. 2:

| 16-1g | FFS: Resources for beam management, pathloss measurement, BFD, and BFR | - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, and new beam identification. - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | TBD | N |  |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 3:

| 16-1g | FFS: Resources for beam management, pathloss measurement, BFD, and BFR | - The total number (sum of periodic/semi-persistent/aperiodic) of - SSB/NZP-CSI-RS/CSI-IM resources configured for L1-RSRP/L1-SINR, and - BFR resources configured for new beam identification, and - BFD resources configured for beam failure detection, and - Pathloss RS resources configured for pathloss measurement across all CCs shall not exceed M_1 - The total number of aperiodic NZP-CSI-RS/CSI-IM resources configured for L1-RSRP/L1-SINR across all CCs shall not exceed M_2 - The total number (sum of periodic/semi-persistent/aperiodic) of - SSB/NZP-CSI-RS resources to perform measurement on CMR for L1-RSRP/L1-SINR, and - BFR resources to perform measurement for new beam identification, and - BFD resources to perform measurement for beam failure detection, and - Pathloss RS resources to perform measurement for pathloss across all CCs within a slot shall not exceed M_3 - The total number (sum of periodic/semi-persistent/aperiodic) of NZP-CSI-RS/CSI-IM resources to perform measurement on IMR for L1-SINR across all CCs within a slot shall not exceed M_4 - FFS: The maximum number of SSB/CSI-RS resources across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, and new beam identification. - FFS: The maximum number of SSB/CSI-RS resources across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification | 2.24 |  | N/A |  | TBD | N |  |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

According to the comments in [1] there is equal support for introducing a dedicated FG and deleting the FG. More discussion is clearly needed. For example, proponents of Alt. 1 to delete the FG argue it is readily included in other FGs. Please comment and make constructive suggestions to arrive at consensus via email.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Before new capabilities are introduced, the relation to the current capabilities should be taken into account. The following capabilities apply: maxNumberSSB-CSI-RS-ResourceOneTx, maxNumberCSI-RS-SSB-CBD, maxNumberCSI-RS-BFD, maxNumberSSB-BFD. There are currently no capability for pathloss measurements, and there is no increase in the number of pathloss reference RSs in R16, so it is unclear to us why this is needed. |
| Apple | We think the dedicate FG is needed. There is no capability reporting in Rel-15 for UE to indicate its capability in terms of RS processing to support the beam management in general. The beam management consists of multiple components (1) L1-RSRP (2) L1-SINR (3) CBD (4) BFD. A UE may not have separate engine or processing unit to handle each component independently. As results, it is beneficial to allow UE to report the total RS processing capability for beam management without forcing UE to report each component individually assuming there is independent processing unit for each component. maxNumberSSB-CSI-RS-ResourceOneTx is for L1-RSRP with single port maxNumberCSI-RS-SSB-CBD is for new beam identification maxNumberCSI-RS-BFD/maxNumberSSB-BFD is for beam failure detection |
| Intel | In our opinion dedicated FG is preferable, and Alt. 2 is our preference. For the case of CBD (new beam identification) for BFR, this capability will signal the maximum number of resources that the UE can support across all CCs for CBD. This, in addition to the maximum number of supported CBD RS per CC for SCell BFR in FG 16-1f completes the capability in terms of number and split of RS resources that the UE can support for BFR |
| LGE | Same view with Ericsson. Alt1 is preferred. |
| MediaTek | We think dedicated FG is needed, and Alt. 3 is our preference. Same view as Apple. The UE capability in the amounts of reference signal resources for various functions in NR-eMIMO can be considered altogether. It is not needed to have individual component for each, unless some of them have quite different characteristics from others and can be addressed separately in that case. |
| OPPO | Prefer to have an FG for that. Either Alt2 or Alt3 is preferred. |
| ZTE/Sanechips | Same view with Ericsson. Alt1 is preferred. |
| vivo | We would like to clarify UE reporting behavior on the following capability. It is clearly stated that this is limited to L1-RSRP report. How can UE report for other purpose? - maxNumberSSB-CSI-RS-ResourceOneTx indicates maximum total number of configured one port NZP CSI-RS resources and SS/PBCH blocks that are supported by the UE for 'CRI/RSRP' and 'SSBRI/RSRP' reporting within a slot and across all serving cells. On FR2, it is mandatory to report >=8; On FR1, it is mandatory with capability signalling to report >=8. |
| Huawei, HiSilicon | We prefer to have a dedicated FG, and are open to discuss between Alt 2 and 3. And, we agree with comments from Apple/Intel/MediaTek. Reply to Ericsson/LGE/ZTE/Sanechips: In R15, as the RS resources configured for different purposes may be different, UE will be conservative on capability reporting for each purpose. With dedicated FG on the total number of resources across different purposes in R16, UE can report larger values for each purpose, as long as the limit on total number of resources is not exceeded. In our view, this will not only help exploiting full potential at UE side, but also improve planning flexibility at NW side. Reply to vivo: It seems there is some misunderstanding here. The proposal here is to include additional components on the total number of resources across different purposes. |
| Qualcomm | Support dedicated FG. A general limit on various measurement types is essentially what UE needed, and also allows gNB to more flexibly allocate configured resources among different measurement types, as long as the general limit is satisfied. This becomes more critical when more and more measurement types are introduced. Individual limit per FG as defined in R15 is not efficient anymore. Support at least a general limit on max total resource # per slot across CCs for L1-RSRP, L1-SINR for both CMR and IMR, BFD RS, candidate beam RS. This is due to the measurement heavy features introduced in R16, e.g. L1-SINR needs two resources, SCell BFD is for multiple CCs. Defining separate limit on resource # per measurement type as in R15 is not efficient and may even get worse in future release. At least to our UE, the bottle neck is the total resource # and their time density. In order to meet the total limit, we have to report conservatively for each separate limit to consider the worst case. If a general limit can be introduced, gNB can fully utilize UE’s capability based on the general limit. For example, UE may report max 10 resources for L1-SINR and max 10 resources for L1-SINR. If a general limit can be introduced, UE can report max 20 in total, and gNB can configure 15 for L1-RSRP and 5 for L1-SINR, as long as the general limit is satisfied. So it benefits both UE and gNB. We are also fine to modify on top of existing FG, as long as the implementation issue can be addressed. |
| Nokia, NSB | Alt.1 is preferred. |
| CATT | Prefer a dedicated FG. Alt-2 is preferred over alt-3. |

# Conclusions

…

# References

- R1-2001868, Summary on UE features for eMIMO, Moderator (AT&T)