3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.9

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-Mobility-01]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-Mobility-01] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-Mobility-01] Email discussion/approval proposal 1 (high priority item) in R1-2001870 by 4/24 – Ralf (ATT) - Note: “FL Proposal 5“ in R1-2001870 can be discussed within the scope of this email discussion

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-Mobility-01] “Email discussion/approval proposal 1 (high priority item) in R1-2001870” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-Mobility-01]

The following is the proposal in Error: Reference source not found for approval in this email discussion:

FL Proposal 1 (21-1):

Alt.1:

| 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | Per Band | No | N/A | N/A |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-1b | Inter-frequency DAPS HO | Support of inter-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | Per BC | No | N/A | N/A |  | [Optional with capability signalling] |

Alt. 2:

| 21-1 | DAPS HO in non-overlapping frequency resources | For the Indicated support of intra-frequency DAPS-HO for a given [band] and indicated support of inter-frequency DAPS-HO for a given [band combination]. 1) Indicates support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO in overlapping OFDM symbols in non-overlapping frequency resources 2) Indicates support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | [Per BC] for inter-frequency case, Per Band for intra-frequency case | No | N/A | N/A | Not supported when both source/target cells are in FR2 | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-1a | DAPS HO in overlapped time and frequency resources | For the Indicated support of intra-frequency DAPS-HO for a given [band] and indicated support of inter-frequency DAPS-HO for a given [band combination]. 1) Indicates support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO in overlapped time and frequency resources 2) Indicates support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | [Per BC] for inter-frequency case, Per Band for intra-frequency case | No | N/A | N/A | Not supported when both source/target cells are in FR2 | [Optional with capability signalling] |

- High priority

  - Down-select between Alt. 1 and Alt. 2

  - If Alt. 2 is agreed, conclude type for FG 21-1 and 21-1a

“FL Proposal 5“ in [1] can also be discussed within the scope of this email discussion.

FL Proposal 5 (high priority, 21-1a): Discuss whether to introduce the following new feature group

| 21-1a | Simultaneous DL reception with both source and target PCells during resource collision | Indicate support of simultaneous DL reception with both source and target PCells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time | DAPS |  |  |  |  |  |  | 21-1a | Simultaneous DL reception with both source and target PCells during resource collision | Indicate support of simultaneous DL reception with both source and target PCells when the resources for PDCCH/PDSCH from the two cells overlap in frequency and time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Companies are asked to provide their views and comments in the following tables.

Regarding the down-selection between Alt. 1 and Alt. 2:

| Company | Comments/Questions/Suggestions |
| --- | --- |

If Alt. 2 is agreed, regarding the conclusion of the type for FG 21-1 and 21-1a:

| Company | Comments/Questions/Suggestions |
| --- | --- |

Regarding FL Proposal 5:

| Company | Comments/Questions/Suggestions |
| --- | --- |

…

# Conclusions

…

# References

- R1-2001870, Summary on UE features for NR mobility enhancements, Moderator (AT&T)