3GPP TSG RAN WG1 Meeting #100b-e			R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Title:	[Draft] LS on subcarrier spacing for CLI-RSSI measurement

Response to:

Release:	Release-16

Work Item:	NR_CLI_RIM-Core

Source:	LG Electronics [RAN1]

To:	RAN2

Cc:	RAN4

Contact Person:

#### Name:	Hyunsoo Ko

E-mail Address:	hyunsoo (dot) ko (at) lge (dot) com

1. Overall Description:

RAN1 concluded following RAN1 assumptions for CLI-RSSI measurement.

  - UE performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource.

    - CLI-RSSI measurement is applicable for RRC_CONNECTED intra-frequency only.

That is, for performing CLI-RSSI measurement, RAN1 assumes the SCS of the active bandwidth part is used. On the other hand, for the purpose of CLI-RSSI measurement resource configuration, RAN1 assumes the parameter rssi-SCS-r16 in RSSI-ResourceConfigCLI-r16 in the IE MeasObjectCLI is used.

In order to make clear the purpose of the parameter rssi-SCS-r16 and to avoid ambiguity which SCS is used for performing CLI-RSSI measurement, RAN1 decided to update the description of the parameter rssi-SCS-r16 as follow:

rssi-scs-r16 Reference subcarrier spacing for CLI-RSSI measurement. Only the values 15, 30 kHz or 60 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. UE performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource.

RAN1 kindly ask RAN2 to update the description of the parameter rssi-SCS-r16 in the IE MeasObjectCLI in TS38.331 as above in the future specification work.

2. Actions:

To 3GPP RAN2

ACTION: 	RAN1 kindly ask RAN2 to update the description of the parameter rssi-SCS-r16 in the IE MeasObjectCLI in TS38.331 as above in the future specification work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101-e	25 May - 5 June 2020				Online

TSG-RAN WG1 Meeting #102 	24 Aug - 28 Aug 2020				Toulouse, FR