3GPP TSG RAN WG1 Meeting #100bis	R1-2003113

# e-Meeting, 20th – 30th April 2020

Agenda Item:	6.2.1.3

Source:	Moderator (Ericsson)

Title:	TP for 36.213 on RV cycling and interleaving granularity for LTE-MTC multi-TB scheduling

Document for:	Discussion, Decision

# Introduction

RAN1#100bis-e discussed RV cycling as well as interleaving granularity for Multi-TB scheduling for Rel-16 LTE-MTC in the email discussion “[100b-e-LTE-eMTC5-Multi-TB-02] RV cycling, TB interleaving and frequency hopping”. For more background information, see “Issue #2” in the email discussion summary in R1-2002796. This contribution presents a 36.213 TP based on the outcome of the discussion.

| Reason for change: | RAN1#100bis-e made the following agreement for Rel-16 LTE-MTC which needs to be captured in the specification: - i0 is initialized at the beginning of each TB, and the corresponding includes all the subframes belonging to the TB and excludes those associated with other TBs scheduled by the DCI. - For the sub-PRB interleaving case, the granularity can be similar with NB-IOT which is based on the TB repetition. |
| --- | --- |
| Summary of change: | The agreement is implemented in the specification. |
| Consequences if not approved: | RV cycling and sub-PRB interleaving may be incorrectly implemented. |
| Clauses affected: | 7.1.7.1, 8, 8.6.1 |

# Text proposal for 36.213

#### 7.1.7.1	Modulation order and redundancy version determination

<unchanged parts omitted>

For BL/CE UEs, for a PDSCH not carrying SystemInformationBlockType1-BR or SI message, the same redundancy version is applied to PDSCH associated with a TB that is transmitted in a given block of  consecutive subframes associated with the TB, including subframes that are not BL/CE DL subframes. The subframe number of the first subframe in each block of such consecutive subframes, denoted as , satisfies , where  for FDD and  for TDD. Denote  as the subframe number of the first downlink subframe intended for PDSCH associated with a TB, as defined in Subclause 7.1.11. The PDSCH transmission associated with the TB spans  Nabs,TBPDSCH consecutive subframes associated with the TB, including subframes that are not BL/CE DL subframes where the PDSCH transmission is postponed and excluding subframes associated with other TBs scheduled by the DCI, if any. For the  block of consecutive subframes within the set of  Nabs,TBPDSCH subframes associated with the TB as described above, the redundancy version (rvidx) associated with the TB is determined according to Table 7.1.7.1-2 using , where , and JPDSCH=Nabs,TBPDSCH +i0-δ mod NaccNacc. The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured in CEModeA,  and  for a TB is determined by the 'Redundancy version' field in DCI format 6-1A, if present. For a BL/CE UE configured in CEModeA, if the UE is configured with higher layer parameter multi-TB-DL-config and multiple TB are scheduled in the corresponding DCI with CRC scrambled by C-RNTI, and the 'Redundancy version' field for a scheduled TB is not present in the corresponding DCI,  for all TBs scheduled by the DCI. For a BL/CE UE configured with CEModeB, or a BL/CE UE receiving PDSCH associated with P-RNTI,  for FDD and  for TDD, and .

<unchanged parts omitted>

# 8	Physical uplink shared channel related procedures

<unchanged parts omitted>

A BL/CE UE shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B scheduling PUSCH intended for the UE, perform a corresponding PUSCH transmission in subframe(s) ni = n+ki if a transport block(s) corresponding to the HARQ process(es) of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, NTBN-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted;

-	the value of is the number of scheduled TB determined by the corresponding DCI if present,  otherwise;

-	 and the value of  is determined by the repetition number field in the corresponding DCI, where

-	if the UE is configured with higher layer parameter ce-pdsch-puschEnhancement-config with value 'On' are given by {1,2,4,8,12,16,24,32}

-	otherwise, are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI is using uplink resource allocation type 5,  where N ≤ 32 for CE Mode A and N ≤ 2048 for CE Mode B,  is defined in [3] and  is determined according to procedure in subclause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE UL subframe(s) starting with subframe n+x, and in case N=1, k0=x;

-	for ,

-	if the UE is configured with higher layer parameter multi-TB-UL-Unicast-Interleaving-config, and PUSCH corresponding to a MPDCCH with DCI CRC scrambled by C-RNTI and

-	where C=MslotsUL2∙MRU if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15 and the PUSCH resource assignment is using uplink resource allocation type 5, otherwise  for UE configured with CEModeA and  for UE configured with CEModeB,

-	BL/CE UL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	BL/CE UL subframes  with  are associated with TBr+1 ,

<unchanged parts omitted>

### 8.6.1	Modulation order and redundancy version determination

<unchanged parts omitted>

For a BL/CE UE or for UEs configured with higher layer parameter PUSCH-EnhancementsConfig,

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment is using uplink resource allocation type 5, the redundancy version (rvidx) to use for the i-th BL/CE UL subframe associated with a TB in the physical uplink shared channel is determined according to Table 7.1.7.1-2 using  where i=0, 1, …, N-1, and N is the number of BL/CE UL subframes associated with the TB for the PUSCH transmission as determined in subclause 8.0. For a BL/CE UE configured in CEModeA,   is determined by the 'Redundancy version' field in DCI format 6-0A, if present. For a BL/CE UE configured in CEModeA, if the UE is configured with higher layer parameter multi-TB-UL-config and multiple TB are scheduled in the corresponding DCI, and the 'Redundancy version' field for a scheduled TB is not present in the corresponding DCI,  for all TBs scheduled by the DCI. For a BL/CE UE configured with CEModeB, . For a BL/CE UE,  for a PUSCH transmission using preconfigured uplink resource.

-	otherwise, the same redundancy version is applied to PUSCH associated with a TB that is transmitted in a given block of  consecutive subframes associated with a TB, including subframes that are not BL/CE UL subframes. The subframe number of the first subframe in each block of  such consecutive subframes, denoted as , satisfies . Denote  as the subframe number of the first uplink subframe intended for PUSCH associated with a TB. For BL/CE UEs, the PUSCH transmission associated with a TB spans  Nabs,TBPUSCH consecutive subframes associated with the TB, including subframes that are not BL/CE UL subframes where the PUSCH transmission is postponed and excluding subframes associated with other TBs scheduled by the DCI, if any. For the  block of consecutive subframes within the set of Nabs,TBPUSCH subframes associated with the TB as described above, the redundancy version (rvidx) associated with the TB is determined according to Table 7.1.7.1-2 using , where , and JPUSCH=Nabs,TBPUSCH +i0 mod NaccNacc. The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured in CEModeA,  and  for a TB is determined by the 'Redundancy version' field in DCI format 6-0A, if present. For a BL/CE UE configured in CEModeA, if the UE is configured with higher layer parameter multi-TB-UL-config and multiple TB are scheduled in the corresponding DCI, and the 'Redundancy version' field for a scheduled TB is not present in the corresponding DCI,  for all TBs scheduled by the DCI. For a BL/CE UE configured with CEModeB,  for FDD and  for TDD, and . For a UE configured with higher layer parameter PUSCH-EnhancementsConfig,  and  is determined by the 'Redundancy version' field in DCI format 0C. For UEs configured with higher layer parameter PUSCH-EnhancementsConfig,NabsPUSCH=NrepPUSCH Nabs,TBPUSCH=NrepPUSCH.

<unchanged parts omitted>