3GPP TSG RAN WG1 #101	R1-20xxxxx

e-Meeting, May 25th – June 5th, 2020

Agenda item:		5.1

Source:	Moderator (China Telecom)

Title:	[101-e-LS-TxSwitching-01] Email discussion/approval on remaining issues for inter-band UL CA

Document for:		Discussion and Decision

# Introduction

In [1], open issues are summarized for uplink Tx switching from RAN1 perspective. As per the guidance of Chairman, following issues are identified for email discussion/approval during RAN1 #101 e-meeting:

[101-e-LS-TxSwitching-01] Email discussion/approval on remaining issues for inter-band UL CA

- Granularity of determination on the presence of the switching period

- Support of codebook based PUSCH transmission.

- Whether UE can report support of both option 1 and option 2

by 5/28, with the corresponding TP by 6/3 – Jianchi (CT)

[101-e-LS-TxSwitching-02] Email discussion/approval on remaining issues for EN-DC

- General mechanism

- Mapping between UL transmission ports and Tx chain

- Switching mechanism

by 5/28, with the corresponding TP by 6/3 – Jianchi (CT)

[101-e-LS-TxSwitching-03] Email discussion/approval on other issues for uplink Tx switching

- Additional preparation time

- Switching period, including UE behaviour and location

- Twisted-order scheduling

- UE capability relation between operation w/ Tx switching and operation w/o Tx switching

- PUCCH multiplexing

- Clarification on the determination of the last transmission occasion

- UE behaviours related to simultaneousTxSUL-NonSUL for SUL

by 5/29, with the corresponding TP by 6/4 – Jianchi (CT)

[101-e-LS-TxSwitching-04] Email approval of TP capturing agreements from previous meetings by 6/2 (Mihai, Nokia)

This is email discussion thread #1 to discuss remaining issues for inter-band UL CA.

# Discussion

## Issue #1: Granularity of determination on the presence of the switching period

In RAN1 #100b-e, following three options for the granularity of determination on the presence of the switching period were discussed:

- Option 1: The presence of the switching period is determined one time every transmission occasion.

Supported by: OPPO, Nokia, Huawei, CATT

- Option 2: The presence of the switching period is determined one time every slot.

- Option 2a: The presence of the switching period is determined one time every slot with smaller numerology.

Supported by: MediaTek, Nokia

- Option 2b: The presence of the switching period is determined one time every slot with larger numerology.

- Option 3: The presence of the switching period is determined one time every UL phase. An UL phase is defined as consecutive UL symbols in the TDD carrier which is capable of 2 ports transmission. The state of Tx chains is not changed during the UL phase.

Supported by: China Telecom, ZTE, Qualcomm, Intel, Nokia, Samsung

In RAN1 #100b-e, it was agreed to make down selection on following two options:

- Option 1: The presence of the switching period is determined one time every transmission occasion.

- Option 2: The presence of the switching period is determined one time every slot or every UL phase.

Companies are invited to provide further views for the granularity of determination on the presence of the switching period, including pros/cons, and specification impacts.

| Companies | Option 1 (Support or object) | Option 2 (Support or object) | Comments |
| --- | --- | --- | --- |
| CATT | Support | Object | Option 2 is a special case of option 1. If overhead of Tx switching is a concern in some deployment, gNB can impose scheduling restriction to make sure that there is no switching within a UL phase/slot. It is up to gNB implementation. It is too restrictive from network perspective that switching can only happen at the boundary of a UL phase. |
| ZTE |  | Support | We think Option 2 is not a special case of Option 1. i.e. Option 2 cannot be achieved by gNB implementation if Option 1 is agreed. For Option 1, the location of switching period varies with the location of scheduled PUSCH (with the assumption that switching period is immediately before next UL transmission which triggers case switch). However, for Option2, switching period location can be only at the boundary of an UL phase. i.e. The location of switching period does not depend on the starting symbol of the PUSCH scheduled in the UL phase. This makes UE and network implementation easier since the switching period is semi-statically fixed. With Option 2, the network can do semi-static coordination on UL resources in the UL carriers e.g. by avoiding configuration of uplink resources (e.g. PUCCH/CG/PRACH) on the switching period location. Also, it would be beneficial if the switching period can be always located at the GP or downlink symbols (if there is no DL interruption) of the TDD carrier. This can be ensured by Option 2 regardless of the starting symbol of scheduled PUSCH in the UL phase. It cannot be achieved by Option1 if the scheduled PUSCH is not located at the first uplink symbol of the UL phase. In addition, it reduces the probability of misalignment between UE and network when one of the uplink scheduling DCI is dropped. |
| vivo |  | Support | For Option 1, we think it is not beneficial for UE to be always prepared for carrier switch every transmission occasion or every slot. Option 2 could be beneficial for UE implementation if the location for potential UL switch is pre-known by the UE such that some potential power saving can be possible. Therefore, we suggest to have some restrictions on the presence of switching period and prefer to not allow more than one switch per UL phase. |

## Issue #2: Support of codebook based PUSCH transmission for option 2

For UL CA option 2, for codebook based PUSCH transmission, the basic mechanism can be supported. PUCCH and PUSCH with 1-port transmission can be indicated by DCI format 0_0 and DCI format 0_1 can indicate PUSCH with 2-port transmission when nrofSRS-Ports is configured as 2 antenna ports.

However, nrofSRS-Ports is semi-statically configured while case 1 or case 2 can be dynamically changed. There is one problem that whether to indicate 1-port transmission using DCI format 0_1 when nrofSRS-Ports is configured as 2 antenna ports.

In RAN1 #100b-e, the majority thought for UL CA option 2, DCI format 0_1 can be used to indicate 1-port transmission when nrofSRS-Ports is configured as 2 antenna ports. Two options were discussed:

  - Option 1: DCI Format 0_1 can be used to indicate 1-port UL transmission on carrier 2 when 2-port SRS is configured for codebook-based transmission.

  - PUSCH transmission with TPMI= is considered as 1 port transmission.

  - PUSCH transmission with all other TPMI, e.g., TPMI= and TPMI=, are considered as 2 ports transmission.

Supported by: China Telecom, ZTE, Qualcomm, Nokia

  - Option 2: Reuse the functionality of ULFP (UL full power) transmission Mode 2, gNB configures 2 SRS resources, 1-port SRS resource for Case 1 and 2-port SRS resource for Case 2.

  - Note: no spec impacts for spatial relationship on carrier 2.

Supported by: China Telecom, OPPO, ZTE, LG, Intel, Huawei, Ericsson

In RAN1 #100b-e, there was extensive discussion on the above two options, but no consensus has been reached. Companies are invited to provide further views on the above two options, including pros/cons, and specification impacts.

| Companies | Option 1 (Support or object) | Option 2 (Support or object) | Comments |
| --- | --- | --- | --- |
| CATT |  | Object | If option 2 is adopted, a UE supporting ULFP mode 2 with antenna virtualization has to indicate ‘not support UL Tx switching’. This is restrictive and will delay deployment of this feature. |
| ZTE | Support | Support | Regardless which option to be adopted or even none of the Options is adopted, we need to further discuss the followings: - Clarify in the spec whether TPMIs [1, 0] and/or [0, 1] are regarded as 1 port or 2 port transmission. From our perspective, they are both 1-port transmission if it is not clarified. This would be problematic since [0, 1] cannot be achieved by Case 1. - Whether the spec allows Tx switching is supported together with Full Tx power mode 2. From our perspective, the spec does not need to prevent UEs from reporting the support of both. Virtualization is transparent in RAN1 spec. Supporting ULFPTx mode 2 does not necessarily mean the UE has to support virtualization. What we care is whether full power transmission can be supported. Since RAN4 has agreed that power class declaration will NOT be changed between case 1 and case 2.", it is pretty much limited to the architecture of [23dBm 23dBm] for PC 3 UEs. In this architecture, the UE can achieve full power in both case 1 with 1Tx state and case 2 with 2Tx state in carrier 2. Under the current UE feature on ULFPTx mode 2, the UE can report support of 2 SRS resources (i.e. one 1-port SRS resource and one 2-port SRS resources). Under the architecture, it has no problem for the UE to support full power with 1-port SRS resource even under 1Tx Case 1. Also, the UE can report full power transmission with all TPMIs (including [1,0] and [0,1]) under the TPMI group reporting. So the UE feature signalling design for ULFPTx already allows architecture for UL Tx Switching. So we should not prevent supporting ULFPTx and UL Tx Switching at the same time. Given the above analysis, our proposal is: Support 1-port UL transmission indicated by DCI Format 0_1 for UL CA with Tx switching. - If only one 2-port SRS resource is configured for codebook-based transmission on carrier 2, - PUSCH transmission with TPMI= is considered as 1 port transmission. - PUSCH transmission with all other TPMI, e.g., TPMI= and TPMI=, are considered as 2 ports transmission. - If 2 SRS resources (with one 1-port resource and one 2-port resource) are configured for codebook-based transmission on carrier 2, the 1-port SRS resource is used for Case 1 and the 2-port SRS resource is used for Case 2. - Support using the same UE capability signaling as that for UL full Tx power. |

## Issue #3: Whether UE can report support of both option 1 and option 2

In RAN1 #100b-e, for mapping between UL transmission ports and Tx chain, it has been agreed that for inter-band UL CA, if UE reports via capability signaling to support uplink Tx switching, UE further reports via capability signaling which option (between Option 1 and Option 2) is supported. However, it has not been discussed whether UE can report support of both option 1 and option 2 in UE capability signaling.

Companies are invited to provide views on whether UE can report support of both option 1 and option 2 in UE capability signaling.

| Companies | Comments |
| --- | --- |
| CATT | We don't see any reason to force UE not to support both option 1 and option 2. |
| ZTE | We think a UE should be allowed to support both option1 and option2 if they have the capability to do so. This may address the market need in the future. Therefore, we have the following proposal: A UE can report support of {Option1, Option2, Option1 and Option2} in UE capability signaling. An explicit RRC parameter is introduced to differentiate Option1 and Option2 for inter-band UL CA with Tx switching. |
| vivo | It is up to UE to indicate whether it support either option 1 or option 2, or both |

## Other issues

Companies are invited to provide views on other issues not covered above.

| Companies | Comments |
| --- | --- |
| ZTE | We may need to discuss whether and how SRS antenna switching works with UL Tx switching. |

# References

- R1-2004699, Summary#2 of uplink Tx switching, Moderator (China Telecom), RAN1 #101e, May 2020.

- R1-2003332, Remaining Issues on Support of Tx Switching between Two Uplink Carriers, ZTE, RAN1 #101e, May 2020.

- R1-2003356, Remaining issues on UL carrier switch, vivo, RAN1 #101e, May 2020.

- R1-2003590, Remaining issues on UL Tx switching, CATT, RAN1 #101e, May 2020.

- R1-2003831, Remaining issues on uplink Tx switching, China Telecom, RAN1 #101e, May 2020.

- R1-2004356, RAN1 aspects of UL Tx switching, Ericsson, RAN1 #101e, May 2020.

- R1-2004376, Discussion on Tx Switching between Two Uplink Carriers, OPPO, RAN1 #101e, May 2020.

- R1-2004434, Remaining issues for 1Tx-2Tx switching, Qualcomm Incorporated, RAN1 #101e, May 2020.

- R1-2004501, Remaining open issues on Tx switching between two uplink carriers, Nokia, Nokia Shanghai Bell, RAN1 #101e, May 2020.

- R1-2004603, Discussion on the remaining problems of supporting Tx switching between two uplink carriers, Huawei, HiSilicon, RAN1 #101e, May 2020.

# Appendix

| Companies | Views |
| --- | --- |
| ZTE (R1-2003332) | Proposal 6: UE can report support of {Option1, Option2, Option1 and Option2} in UE capability signaling. An explicit RRC parameter is introduced to differentiate Option1 and Option2 for inter-band UL CA with Tx switching. Proposal 7: Introduce UL phase for UL CA with Tx switching - An UL phase is defined as consecutive UL symbols in the TDD carrier which is capable for 2 ports transmission. - UE is not expected to switch the case within each UL phase. - The location of switching period is at the boundaries of the UL phase. - If the location is configured on carrier 1, the switching period is located immediately before the start of the UL phase corresponding to the non-uplink symbols on carrier 2 and immediately after the end of the UL phase to the downlink symbols on carrier 2. - If the location is configured on carrier 2, the switching period is located immediately after the start of the UL phase corresponding to the uplink symbols on carrier 2 and immediately before the end of the UL phase to the uplink symbols on carrier 2. Proposal 8: Support 1-port UL transmission indicated by DCI Format 0_1 for UL CA with Tx switching. - If only one 2-port SRS resource is configured for codebook-based transmission on carrier 2, - PUSCH transmission with TPMI= is considered as 1 port transmission. - PUSCH transmission with all other TPMI, e.g., TPMI= and TPMI=, are considered as 2 ports transmission. - If 2 SRS resources (with one 1-port resource and one 2-port resource) are configured for codebook-based transmission on carrier 2, the 1-port SRS resource is used for Case 1 and the 2-port SRS resource is used for Case 2. - Support using the same UE capability signaling as that for UL full Tx power. Proposal 9: Extend the agreement on the timing of decision on Tx switching to cover all switching cases for UL CA. i.e. - For UL CA - The current transmission occasion and the last transmission occasion assumed by the UE at T0 - Toffset are respectively case-1 transmission and case-2 transmission which needs switching period in between, where T0 and Toffset is the starting timing and UE preparation time of current transmission, respectively. - The current transmission occasion and the last transmission occasion assumed by the UE at T0 - Toffset are respectively case-2 transmission and case-1 transmission which needs switching period in between, where T0 and Toffset is the starting timing and UE preparation time of current transmission, respectively. Proposal 13: Adopt the TP1 for introducing Tx switching for EN-DC, CA and SUL in Section 3. |
| vivo (R1-2003356) | Proposal 1: The presence of the switching period is determined once per UL phase, where UL phase is defined as a set of contiguous UL slots in the carrier 2. |
| CATT (R1-2003590) | Proposal 1: - The presence of the switching period is determined one time every transmission occasion. Proposal 2: - When UL full power mode 2 and UL Tx switching are configured, 1-port SRS resource (if configured) is only used for Case 2. |
| China Telecom (R1-2003831) | Proposal 2: For inter-band UL CA, UL phase is defined to avoid unnecessary Tx switching. An UL phase is defined as consecutive UL symbols in the TDD carrier which is capable of 2 ports transmission. The state of Tx chains is not changed during the UL phase. Proposal 3: For UL CA option 2, for codebook based PUSCH transmission, DCI format 0_1 can indicate 1-port UL transmission when 2-port SRS is configured. gNB can configure 2 SRS resources, 1-port SRS resource for Case 1 and 2-port SRS resource for Case 2. |
| Ericsson (R1-2004356) | Comparing these two approaches, there are tradeoffs of UE, CSI accuracy, UE implementation, and SRS overhead. Using two SRS resources allows one port CSI for the uplink, since it is completely up to UE implementation if it transmits one port SRS by virtualizing the two Tx chains or by selecting either one. A UE with two half power PAs can virtualize the two Tx chains to one port, and transmit two port UL MIMO as normal without virtualization. By contrast, UEs that have one full power PA for carrier 2 can indicate full power for a TPMI and may be configured with only one two port SRS resource. The two SRS resource case for full power Mode 2 is therefore suitable to support UEs with half power PAs and/or to obtain better port CSI, which may be particularly beneficial DCI format 0_0 which can only be one port. Using one SRS resource for full power Mode 2 conserves SRS resource, and is suitable for UEs with one full power PA. The ‘Option 1’ proposal that one TPMI is considered as one port transmission, while other TPMIs are considered as two port transmission is not precisely clear. If indicating a TPMI changes the number of SRS ports used, then this is a new MIMO transmission scheme, since only SRI changes the number of SRS ports in Rel-15 and Rel-16. Changing the number of indicated SRS ports also implies that the transmit power scaling varies with the number of indicated ports, as can be seen in 38.213 section 7.1. If the intention of ‘is considered as 1 port’ is that only the power scaling changes with the TPMI, then this is already supported in full power Mode 2 when the UE is configured with one 2 port SRS resource, and so the UE only needs to indicate capability for Mode 2. Proposal 2 - CA Option 2 UL tx switching is supported by UL full power mode 2 - New MIMO transmission schemes dynamically switching between one and two port transmission or power scaling are not defined. |
| OPPO (R1-2004376) | Proposal 2: In order to support Option 2, RAN1 should support a SRS resource set with 1-port SRS resource(s) and 2-port SRS resource(s) is configured for codebook based PUSCH (i.e., Reuse the scheme introduced in full power transmission) - In case 1, 1-port SRS resource will be indicated by DCI for carrier 2 - In case 2, 2-port SRS resource will be indicated by DCI for carrier 2 - No spatial relation information is configured (since it is only for FR1) - Up to 2 SRS resources can be configured in the SRS resource (same restriction as Rel-15) - The power control scheme is the same as Rel-15 - The power scaling factor is the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource - Proposal 3: In Rel-16, a UE is not expected to be configured with Option 2 and non-codebook based PUSCH simultaneously. |
| Qualcomm (R1-2004434) | Companies propose to use 1-port SRS agreed in eMIMO to indicate Case 1 as it is intended for virtualization. However, we think this is contrary to the goals of ULFP Mode 2. In ULFP Mode 2, the 1-port SRS is virtualized across 2 or more physical ports to achieve full power. In the case of UL Tx switching, however, Case 1 implies that one of the two ports is switched away, therefore full power cannot be achieved by the port corresponding to the 1-port SRS. We understand the proponents are trying to use some agreement to “reduce” the standard efforts. However, as the intention is different or even contradict it is just an unnecessary repetition of an SRS the UE has already transmitted. It is a waste of power and resources while UE may not be able to support virtualization as it only with one Tx. Meanwhile, in the feature list session, there was no consensus yet to split eMIMO Mode 2 capabilities to ‘additional SRS resource’ and ‘full power TPMI report’. It would not be acceptable to us if a UE, which never had full power issues, now have to report list of TPMIs with no information content. With this, Option 2 will waste not only UE power, network resources but also signaling overhead. Proposal 1: Adopt option 2 - 2 Tx in CC2 (TDD) is used for these UL transmissions: PUSCH with TPMI=, PUSCH with TPMI=, 2-port SRS, 2-port configured grant PUSCH - 1 Tx in CC2 (TDD) is used for these UL transmissions: No grant, PUCCH, SR, PRACH, PUSCH with TPMI=, PUSCH scheduled by DCI 0_0, single port configured grant PUSCH Proposal 2: To simplify the specification discussion, we make the following proposal on timeline. - Only allowing one switch for an UL phase, i.e. for consecutive UL slots in CC2 - Allowed switch boundaries are the start and the end of the UL phase in CC2 Proposal 3: For placement of transient time - Relative placement of transient is RRC configured - Placing transient always in CC1 (FDD) should be default - Gap is created by gNB scheduling, the occurrence of a requested transmission in a gap is an error case |
| Nokia (R1-2004501) | Proposal: The UL Tx switching for UL CA is limited to the following two uplink configurations (additional DL-only SCells may be configured) - One uplink on an FDD carrier and another uplink on a TDD carrier - Two TDD carriers with uplink, where the uplink phases of the TDD carriers do not overlap Proposal: For UL, CA the determination of the switching gap is done once per slot. |
| Huawei (R1-2004603) | Proposal 2: For inter-band UL CA, if uplink Tx switching is configured, the presence of the switching period is determined only once for each transmission occasion. Proposal 3: For UL CA option 2, select alternative 1 to support codebook based PUSCH transmission on carrier 2, otherwise choose alternative 2. - Alt. 1: Reuse Rel-16 eMIMO ULFP Mode 2, i.e., gNB configures two SRS resources for carrier 2, 1-port SRS resource for Case 1 and 2-port SRS resource for Case 2. - Note: no spec impacts for spatial relationship on carrier 2. - Alt. 2: Add 1 bit in DCI format 0_1 to indicate the number of Tx chain used for the codebook based PUSCH transmission on carrier 2, i.e., if the value is set to 1, it means 2Tx transmission for Case 2, otherwise it means Case 1. |