Proposal 13:

- Adopt the following target data rates for eMBB performance evaluation for FR2.

-	Indoor: DL: 25Mbps, [100Mbps], UL:5Mbps, [10Mbps]

-	Urban: DL: 25Mbps, [100Mbps], UL: 5Mbps, [10Mbps]

-	Suburban: DL: 1Mbps, UL: 50kbps, [200kbps]

| Companies | Comments |
| --- | --- |

Proposal 14:

- For link level simulation, TBS of VoIP for FR2 is the same as FR1.

- For link level simulation, TBS of Msg3 for FR2 is the same as FR1.

| Companies | Comments |
| --- | --- |

Proposal 15:

- The evaluation methodology for FR2 is the same as FR1.

- The link budget template for FR2 is the same as FR1.

- The target performance metric for FR2 is the same as FR1.

| Companies | Comments |
| --- | --- |

Proposal 16:

- For link level simulation, adopt the following table for PUSCH and PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) |
| Subcarrier Space | 120kHz |
| BLER | 10% iBLER for eMBB, 2% rBLER for VoIP |
| UE velocity | Indoor scenario:3km/h Urban scenario: 3km/h for indoor, 30km/h for outdoor. Suburban scenario: 3km/h for indoor, 30km/h, [120km/h] for outdoor. |
| Occupied channel bandwidth for PDSCH | 100MHz, [400MHz] |
| Frequency hopping for PUSCH | Intra-slot, [inter-slot] frequency hopping is enabled |

| Companies | Comments |
| --- | --- |

Proposal 17:

- For link level simulation, adopt the following table for PUSCH and PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| BLER | 10% iBLER for eMBB, 2% rBLER for voice. |
| Number of UE antennas | 8 |
| Number of UE TRXUs | 1 or 2 for PUSCH, 2 for PDSCH |
| DMRS configuration | For 3km/h: Type I, one DMRS symbol, no multiplexing with data. For 30km/h, 120km/h: Type I, 2 or 3 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for PUSCH, CP-OFDM for PDSCH |
| Number of repetitions for PUSCH | For eMBB, no repetition is assumed. For VoIP, the maximum number of repetitions can be 2/4/8. |
| HARQ configuration for PUSCH | For eMBB, no retransmission is assumed. For VoIP, the maximum number of HARQ transmission can be 2/4/8. |

| Companies | Comments |
| --- | --- |

Proposal 18:

- For link level simulation, adopt the following table for PUSCH and PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Number of antenna elements for BS | Indoor scenario: 128 Urban scenario: 256 Suburban: 256 |
| Number of TxRUs for BS | 2 |
| Channel model for link-level simulation | CDL- A, [CDL-C], TDL-A |
| Delay spread | Indoor scenario: 30ns Urban scenario: 100ns Suburban scenario: 100ns |
| Latency requirements for voice | 50ms/100ms |
| PRBs/TBS/MCS | Reported by companies. |

| Companies | Comments |
| --- | --- |

Proposal 19:

- For link level simulation, adopt the following table for PUCCH for FR2.

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI Format 3, 11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%, ACK missed detection probability: 1%. For PUCCH format 3: Block error probability: 1% |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE antennas for PUCCH | 1 |
| Number of UE TRXUs for PUCCH | 1 |
| Number of receive antenna elements for BS | Indoor scenario: 128 Urban scenario: 256 Suburban: 256 |
| Number of receive TxRUs for BS | 2 |
| Number of repetitions for PUCCH | w/ and w/o repetition for PUCCH. The maximum number of repetitions can be 2/4/8. |

| Companies | Comments |
| --- | --- |

Proposal 20:

- For link level simulation, adopt the following table for PDCCH for FR2.

| Parameters | Values |
| --- | --- |
| aggregation level | 16 |
| payload | 40 bits |
| CORESET size | 2 symbols |
| CCE-to-REG mapping type | interleaved or non-interleaved mapping |

| Companies | Comments |
| --- | --- |

Proposal 21:

- For link level simulation, adopt the following table for PRACH for FR2.

| Parameters | Values |
| --- | --- |
| Format type | Format B4, Format C2 |
| Scheduled PRBs | 12 PRBs |
| Performance metric | 0.1% false alarm, 1% miss-detection |

| Companies | Comments |
| --- | --- |