3GPP TSG RAN WG1 #103		R1-2007679

e-meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Evaluation on NR coverage performance for FR2

Agenda Item:	8.8.1.2

Document for:	Discussion and Decision

- Introduction

In RAN1 102-e meeting, NR coverage evaluation assumptions are further discussed, and the specific evaluation metric is also confirmed with following agreements achieved [1].

The agreements of previous meeting aim to further align the evaluation assumptions of specific scenarios, including evaluation metric, antenna model, etc. The main objectives are as follows. Agreements: Further clarify the agreement on antenna gain and antenna gain components including antenna gain correction factors as follows: - For both TDL option 1 (table A below) and TDL option 2 & CDL (table B below) - The gain of antenna gain component 1 is included in LLS results - The gain of antenna gain component 2 is included in link budget template - The gain is expressed by 10 * log 10( N/k ) - 1 - For TDL option 2 & CDL, the gain is 0 dB - The gain of antenna gain component 3 is included in link budget template - The gain of antenna gain component 4 is included in link budget template - The gain of antenna gain components 3 and 4 is expressed by Antenna Element Gain + 10 * log 10( M/N ) -2 - For Tx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (4) - For Rx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (11) - Note: more appropriate name or explanation will be added to row No.(4) and (11). Details can be discussed when the link budget template is updated. Agreements: - Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters - [For LLS based methodology, ]coverage bottleneck(s) identification is performed using at least [MCL and] MIL. - [MCL values can also be considered to compare channels with similar antenna (and antenna array) gain] Agreements: - RAN1 strives for satisfying appropriate targets identified by companies particularly operators - The targets may be in the form of one or more of the following: - 1. Scenario dependent targets, e.g., ISD/MPL - 2. Service dependent targets, e.g., [MCL=147] dB for VoIP; - 3. Relative difference between channels, e.g, MIL(/[MCL]) - Further values and details of such targets will be clarified at RAN1#103-e - Note: there is no intention in RAN1 to update the study item objectives due to the identified targets.

According to above agreements, link budget template based on IMT-2020 self-evaluation is used for capturing MCL, MIL and MPL, which is then used for identifying the bottleneck channels. Therefore, we provide the evaluation results based on above metrics in urban and indoor hotspot scenario for NR FR2 in this contribution. Details on evaluation is listed in the attached spreadsheet.

- Consideration evaluation assumptions

  - Correction factor for broadcast channels

The DL coverage is highly dependent on the gNB transmission EIRP. While the EIRP is usually determined based on transmission power and beamforming gain. In Rel-15/Rel-16, for a UE specific downlink transmission, gNB may select beam with better quality to serve a UE through beam management procedure or taking advantage of channel reciprocity, hence large BF gain can be assumed for transmissions or receptions other than DL broadcast channels.

For DL broadcast channels, e.g. SSBs and PDCCH in type 0-2 CSSs, typically these channels are served by fixed and wider beams, the beamforming gain would be lower compared with UE specific channels. The difference is not considered in ITU link budget template. To reflect the coverage of the broadcast channels more accurately, the BF gain difference between broadcast channel and unicast channel should be reflected in link budget template.

In last meeting, correction factor is introduced to reflect the non-ideal factors in antenna array gain component gain 2, 3 and 4. The beamforming at gNB can be implemented in analog or digital domain, which depends on the antenna structure. In FR1 2.6GHz and 4GHz, the beamforming is realized by mapping from k RF chains to N TxRUs, which is defined as antenna component 2 in [1]. In this case, the correction factor for BF gain difference should be included in antenna gain component 2. While for FR2, k=N=2 is assumed based on previous agreements, analog beamforming is assumed, however there is no antenna gain component 2. Hence, for cases with k=N in FR2, both broadcast and unicast BF is reflected in antenna gain components 3&4. Specifically, the correction factor to reflect the difference between broadcast and unicast, is included in antenna gain component 2 for 2.6GHz and 4GHz in FR1, and is reflected in antenna gain components 3&4 for FR2.

Observation 1: The correction factor for the difference between broadcast and unicast is included in antenna gain components 3&4 for FR2, if k=N=2 is assumed.

We evaluated the BF gain difference between unicast BF and broadcast BF through system level simulation, and the difference is reflected by the RSRP difference measured at UEs in a serving cell. The number of broadcast beams and the beam width is selected to make sure every position in a cell is covered by a broadcast beam within 3dB beam width. For broadcast beams, 64 beams cover 16 horizontal directions and 4 vertical directions in FR2. For unicast beams, 128 DFT based beams composing 32 horizon directions and 4 vertical directions are modeled for FR2. The detailed evaluation assumptions for broadcast and unicast beams are given in [2] .

(a) FR2 urban                            (b) FR2 indoor

Figure 1. CDF of RSRP with broadcast BF and unicast BF

As shown in Figure 1, the RSRP of unicast BF is obviously higher than that of broadcast BF. In FR2, the difference is about 8dB and 5dB for urban and Indoor scenarios, respectively. Without loss of generality, we have the following proposal on correction factors for broadcast channels.

Proposal 1: The correction factor for gNB BF gain for broadcast channel should be considered in link budget template.

- In FR2, the correction factor is about 8dB for urban scenario and 5dB for indoor scenario;

For FR2, UE would transmit UL channel in corresponding to DL signals, hence the correction factor for gNB Rx beamforming should also be considered for PRACH and MSG3, the same correction factor values can be assumed as that for Tx beamforming.

Proposal 2: The correction factor for gNB Rx BF gain for UL channels, associated with DL broadcast channel, should be considered in FR2.

  - Correction factor for antenna gain loss due to tilt angle

As discussed in previous meetings, antenna gain component 4 should also be considered in the antenna array gains, which is determined by the antenna element gain and the antenna pattern. The actual antenna gain depends on the antenna pattern and the tilt angle from the boresight direction of the gNB pannel rather than simply using the maximum antenna element gain. In RAN1#102e, correction factor 2 is introduced in antenna components 3&4. We also obtain the correction factor due to tilt angle through system level evaluation using the following methodology.

- Step 1: Obtain the tilt angle of UE direction from the boresight of the gNB pannel through SLS.

- Step 2: Get the actual antenna gain based on the antenna radiation pattern.

We obtained average antenna gain loss of the evaluated UEs in each scenario, the results are provided in the following table.

Table 1  Average antenna gain loss due to tilt angle obtain from SLS

| Scenarios | 2(dB) |
| --- | --- |
| Urban 28GHz | 3.07 |
| Indoor 28GHz | 3.48 |

Observation 2: The loss in gNB antenna gain due to tilt angle from the boresight direction of antenna pannel is about 3.07dB for FR2 urban and 3.48dB for FR2 indoor.

- Evaluation results

In this section, evaluation results in terms of MCL, MIL and MPL for different scenarios are provided, and bottleneck channels are identified accordingly.

  - Discussion on Coverage Targets

As agreed in [1], targets may be in the form of scenario dependent targets, e.g. ISD/MPL or relative difference between MIL/MCL. In this contribution, we identify bottleneck channels based on the target ISD/MPL. Channel model B is considered as the pathloss model to derive the target MPL from the target ISD of each scenario recommended in [3], which are provided in Table 2.

Table 2 target ISD in each scenario

| scenarios | Urban | Indoor |
| --- | --- | --- |
| target ISD | 500m | 20m |

To intuitively reflect the gap between the MPL of the physical channels and target MPL, the performance gaps are calculated. Note that target ISD of 500m is too aggressive for all channels in urban scenario. For example, the gap between actual MPL and the target MPL can be as large as 35dB, which is unreachable considering any solutions. Hence, performance gap is calculated assuming the target ISD to be 200m for Urban scenario.

Proposal 3: Target ISD for each scenario provided in TR 38.913 can be considered as baseline to identify the coverage bottleneck.

- A reasonable target ISD for Urban scenario should be selected, e.g. 200m.

The performance gaps are provided in Table 3 and Table 4 for Urban and Indoor, respectively. The negative values in green means the coverage of the physical channels can meet the target requirement in corresponding scenario, and the positive values in red show performance gap compared with the target available path loss. Meanwhile, the values which are close to the target threshold with limited performance margin are marked with yellow.

- Urban

As agreed in previous meeting, both NLOS outdoor-to-outdoor and NLOS outdoor-to-indoor are considered for urban scenario. And following carrier frequency and frame structures are considered,

- 4GHz, DDDSU

- 4GHz, DDSU

In urban scenario, 5Mbps target data rate of PUSCH is assumed. The evaluation results of urban scenario are shown in appendix B, it can be observed that PUSCH eMBB is the most obvious bottleneck channel among all channels based on relative difference between MIL and MCL. Besides, almost all uplink channels may need to be enhanced if target ISD is set to 500m. Even if 200m target is assumed, the performance gap is still large, PUSCH and PRACH is below the coverage requirement with 4~5dB in O-to-O scenario, and up to 20dB need to be compensated in O-to-I scenario among uplink channels. It is also noted that downlink broadcast channels are below the target performance when ISD is set to 200m in O-to-I scenario, while the performance gap is no more than 5dB, which is much lower than uplink bottleneck channels. Meanwhile, some solutions can be adopted to effectively enhance downlink broadcast coverage performance, such of TB-scaling of MSG2, which may have around 3~4dB performance gain. Therefore, enhancement of uplink channels has higher priority than downlink broadcast channels in FR2 urban scenario.

Table 3 Performance gap between MPL and pathloss corresponding to the target ISD for FR2 Urban

| MCL gap(dB) | Freq/DL-UL-Config | PDSCH (eMBB) | PDSCH (voip) | PDCCH (UE) | PDSCH MSG2 | PDSCH MSG4 | PDCCH (BC) | SSB | PUSCH (eMBB) | PUSCH (voip) | PUSCH MSG3 | PRACH format B4 | PUCCH (PF1) | PUCCH (11bits) | PUCCH (22 bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban(500m) | 28GHz- DDDSU O2O | -3.57 | -11.61 | -3.57 | 2.56 | 1.68 | 4.43 | 1.78 | 20.62 | -0.24 | 13.77 | 20.03 | 4.48 | 6.37 | 9.75 |
| Urban(200m) |  | -19.13 | -27.17 | -19.13 | -13 | -13.88 | -11.13 | -13.78 | 5.06 | -15.8 | -1.79 | 4.47 | -11.08 | -9.19 | -5.81 |
| Urban(500m) | 28GHz- DDDSU O2I | 11.49 | 4.08 | 12.29 | 17.59 | 16.25 | 20.29 | 17.15 | 36.02 | 16.79 | 29.29 | 36.14 | 20.75 | 22.51 | 25.71 |
| Urban(200m) |  | -4.07 | -11.48 | -3.27 | 2.03 | 0.69 | 4.73 | 2.19 | 20.46 | 1.23 | 13.73 | 20.58 | 5.19 | 6.95 | 10.15 |
| Urban(500m) | 28GHz- DDSU O2O | -3.38 | -11.61 | -3.57 | 2.56 | 1.68 | 4.43 | 1.78 | 19.89 | -0.24 | 13.77 | 20.03 | 4.48 | 6.37 | 9.75 |
| Urban(200m) |  | -18.94 | -27.17 | -19.13 | -13 | -13.88 | -11.13 | -13.78 | 4.33 | -15.8 | -1.79 | 4.47 | -11.08 | -9.19 | -5.81 |
| Urban(500m) | 28GHz- DDSU O2I | 11.99 | 4.08 | 12.29 | 17.59 | 16.25 | 20.29 | 17.15 | 35.09 | 16.79 | 29.29 | 36.14 | 20.75 | 22.51 | 25.71 |
| Urban(200m) |  | -3.57 | -11.48 | -3.27 | 2.03 | 0.69 | 4.73 | 2.19 | 19.53 | 1.23 | 13.73 | 20.58 | 5.19 | 6.95 | 10.15 |

Observation 3: In urban scenario, at least PUSCH and PRACH should be the bottleneck channels based on MIL, MCL and MPL.

- Indoor hotspot

Only NLOS indoor-to-indoor is considered in this scenario. And following carrier frequency and frame structures are evaluated.

- 28GHz, DDDSU

- 28GHz, DDSU

In indoor scenario, downlink power transmission is 23dBm, which is not too much higher than uplink, thus performance difference between downlink and uplink will be close. The MCL/MIL/MPL for different channels in rural scenario in TDD are given in Table 4, it can be observed that the MPL for all channels can meet 20m target ISD. Although PUSCH coverage seems a bit lower than other channels based on MIL, it is not necessary to consider PUSCH as the bottleneck, as long as the target coverage can be fulfilled.

Table 4 Performance gap between MPL and pathloss corresponding to the target ISD for Indoor

| MCL gap(dB) | Freq/DL-UL-Config | PDSCH (eMBB) | PDSCH (voip) | PDCCH (UE) | PDSCH MSG2 | PDSCH MSG4 | PDCCH (BC) | SSB | PUSCH (eMBB) | PUSCH (voip) | PUSCH MSG3 | PRACH format B4 | PUCCH (PF1) | PUCCH (11 bits) | PUCCH (22 bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor 28GHz | DDDSU | -34.24 | -42.8 | -34.96 | -31.45 | -32.8 | -29.96 | -34.26 | -29.65 | -51.02 | -40.58 | -36.87 | -48.41 | -46.73 | -43.45 |
|  | DDSU | -34.05 | -42.8 | -34.96 | -31.45 | -32.8 | -29.96 | -34.26 | -30.36 | -51.02 | -40.58 | -36.87 | -48.41 | -46.73 | -43.45 |

Observation 4: In urban with long distance scenario, there are no bottleneck channels based on target MPL metric.

- Summary of the coverage performance of physical channels

Based on the evaluation results and discussion above, the UL channels have inferior coverage performance compared to DL channels in general. PUSCH eMBB and PRACH are the most significant bottleneck in urban scenarios, and almost all uplink channels need to be enhanced if target ISD of urban scenario is set to 500m. For PUSCH voip, since PUSCH repetition and retransmission is assumed, better coverage can be achieved. However, for a high load cell, it may be too ideal to use so many repetitions for voip transmission, as shown in Appendix A. Therefore, though it has reached the performance target in 28GHz NLOS O-to-O scenario, if limited number of PUSCH transmission occasions are allocated for PUSCH voip, coverage enhancement maybe still needed.

Proposal 4: all uplink channels need to be enhanced for coverage enhancements in FR2 urban scenario.

- Conclusion

In this contribution, we provide our evaluation results in urban scenario Indoor hotspot scenario with the following observations and proposals:

Observation 1: The correction factor for the difference between broadcast and unicast is included in antenna gain component 3&4 for FR2, if k=N=2 is assumed.

Observation 2: The loss in gNB antenna gain due to tilt angle from the boresight direction of antenna pannel, is about 3.07dB for FR2 urban and 3.48dB for FR2 indoor.

Observation 3: In urban scenario, at least PUSCH and PRACH should be the bottleneck channels based on MIL, MCL and MPL.

Observation 4: In urban with long distance scenario, there are no bottleneck channels based on target MPL metric.

Proposal 1: The correction factor for gNB BF gain for broadcast channel should be considered in link budget template.

- In FR2, the correction factor is about 8dB for urban scenario and 5dB for indoor scenario;

Proposal 2: The correction factor for gNB Rx BF gain for UL channels, associated with DL broadcast channel, should be considered in FR2.

Proposal 3: Target ISD for each scenario provided in TR 38.913 can be considered as baseline to identify the coverage bottleneck.

-	A reasonable target ISD for Urban scenario should be selected, e.g. 200m.

Proposal 4: all uplink channels need to be enhanced for coverage enhancements in FR2 urban scenario.

- Appendix A: Evaluation assumptions for physical channels

Table 5 evaluation assumption for FR2

| Parameter | Evaluation assumptions |
| --- | --- |
| Number of gNB antenna port in LLS | 2 |
| Number of UE antenna port in LLS | 1Tx/2Rx |
| PDSCH (eMBB) | Around 1/3 code rate. All DL slots allocated for PDSCH for eMBB |
| PDSCH (voip) | 4 repetitions, 4 HARQ transmission times within 20ms period |
| PUSCH (voip) | 4 repetitions, 4 HARQ retransmissions are confined within 20ms period, inter-slot frequency hopping is assumed |

- Appendix B: Evaluation results

- Urban

- 28GHz, DDDSU

- 28GHz, DDSU

- Indoor hotspot

- 28GHz, DDDSU

- 28GHz, DDSU

- References

- Chairman’s Notes RAN1#102-e final.

- R1-2007683, ‘Considerations on Parameters for coverage enhancements’, vivo.

- 3GPP TR 38.913, ‘Study on Scenarios and Requirements for Next Generation Access Technologies (Release 16)’.