3GPP TSG RAN WG1 #103-e                                                R1-2007743

e-Meeting, October 26th – November 13th, 2020

Source: 	ZTE Corporation

Title: 	Discussion on potential techniques for PUSCH

Agenda item:	8.8.2.1

Document for:	Discussion/Decision

Introduction

According to the revised SID on NR coverage enhancements [1], one objective is to identify the performance target for coverage enhancement for specific scenarios for both FR1 and FR2 and study the potential solutions for both DL and UL.

In the RAN1#102-e meeting, the potential enhancements for PUSCH were discussed [2], and the following agreements were reached [3].

Agreements: - Prioritize the study on the performance and specification impacts on time domain based solutions for PUSCH enhancements, including - Increase the number of repetitions for PUSCH repetition type A - PUSCH repetition with non-consecutive slots/on the basis of available slots for TDD - Note: whether increasing the number of PUSCH repetition for FDD depends on the outcome of AI 8.8.1.1. - Enhancement on PUSCH repetition Type B - E.g., actual repetition across the slot boundary, or the length of actual repetition larger than 14 symbols, etc. - TB processing at least over multi-slot PUSCH - e.g., single TB, sized for a single slot, but transmitted in parts over multiple slots; or single TB, sized for multiple slots, transmitted over multiple slots, and in conjunction with repetition, etc. - FFS - OCC spreading based repetition - Symbol-level repetition - TB interleaving - RV repetition - Early termination of PUSCH repetitions Agreements: - Following solutions are not considered for PUSCH enhancements in this study item in RAN1: - Enhancements to improve spherical coverage / beam correspondence - Reflective arrays - Polarization aspects of the UL and/or DL reference signals Agreements: - Prioritize the study on the performance and specification impacts on DM-RS enhancements for PUSCH, including - Cross-slot channel estimation - With a lower priority compared with cross-slot channel estimation (i.e., companies are encouraged to study it) - Lower density - E.g., DM-RS sharing among multiple PUSCH transmissions or lower DMRS density in the frequency domain. - Higher density - E.g., in time or frequency domain, e.g., 1-comb pattern - Adaptive configuration - DM-RS balancing among frequency hops Agreements: - Multiple layer PUSCH transmission with DFT-S-OFDM for PUSCH enhancements can be studied with low priority. - Study open-loop/closed loop Tx diversity for PUSCH enhancements with low priority. Agreements: - Study the performance and specification impacts on frequency domain based solutions for PUSCH, including - Inter-slot frequency hopping - with more frequency offsets - with more frequency hopping positions. - Inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation - Enhancements on frequency hopping for PUSCH repetition type B - Note that the above inter-slot frequency hopping enhancement can apply for PUSCH repetition type B - [Sub-PRB transmission for VoIP] - FFS: details, e.g., number of tones, multi-slot aggregation - FFS - Intra-slot frequency hopping - with more frequency offsets - with more frequency hopping positions. [Note: Appropriate simulation assumptions are expected.] Agreements: - Study following power domain based solution for PUSCH enhancements - Waveform design to optimize MPR/A-MPR - [FDD high power UE] - Power boosting for pi/2 BPSK Note: if a LS to RAN4 (for the last two bullets) is deemed necessary, target sending the LS in the 1st week of RAN1#103-e

In this contribution, we further analyze the potential techniques for PUSCH enhancements in coverage enhancement SI.

Time domain based enhancement

PUSCH repetition type A

For VoIP service with fixed packet size, PUSCH repetition is deemed an effective way to improve the performance. To look into the potential benefits compared to re-transmission scheme, the link level simulation results for VoIP with different number of repetitions and HARQ re-transmissions are provided in Figure 1. As VoIP has certain latency requirement, two cases are simulated, where one case for 4 repetitions with maximum 1 re-transmission and another case with 2 repetitions with maximum 3 re-transmissions. More detailed assumptions are listed in Table A-1 in the Annex.

Figure 1. Simulation results for VoIP with different number of repetitions and HARQ re-transmissions

As can be shown in Figure 1, the first case with larger number of repetitions while lower number of re-transmission can provide about 1~1.5 dB gain over the second case.

Observation 1: For VoIP service, 4 repetitions with maximum 1 re-transmission can provide about 1~1.5 dB gain over 2 repetitions with maximum 3 re-transmissions.

In NR Rel-15, PUSCH repetition type A is supported with semi-statically configured repetition factors. Dynamic indication of the repetition factors is introduced in Rel-16 URLLC. However, the nominal number of repetition is counted on contiguous slots. In case of collision with DL slots for TDD or BWP switching, the collided PUSCH transmission shall be canceled. So, the actual number of PUSCH repetition is less than the nominal number of repetition and the coverage performance of PUSCH will be significantly degraded. Thus, the number of repetition should be guaranteed for PUSCH repetition type A to improve the coverage performance. For instance, it should be counted on the basis of available U/X slots until the indicated number of repetitions is reached.

Proposal 1: For PUSCH repetition type A, the number of repetitions indicated by gNB should be guaranteed in case of collisions.

PUSCH repetition type B

For the existing PUSCH repetition type B transmission mechanism, when a nominal repetition crosses a slot boundary, it will be divided into multiple actual repetition transmissions. This may cause duration of one actual repetition is much shorter than the indicated duration for nominal transmission. Then, the imbalanced duration of each actual repetition may lead to performance deterioration.

Take the frame structure in Figure 2 for instance, there are 2 available UL symbols in the first slot and 14 UL symbols in the second slot. In such case, there could be the following four cases for scheduling.

- Case 1: Baseline scheme with a 14-symbol PUSCH in the second slot.

- Case 2: Baseline scheme with PUSCH of 2 repetitions, and the length of each nominal repetition is L=8. In such case, there are three actual repetitions with duration of 2, 6, 8 symbols respectively.

- Case 3: Enhanced scheme with actual repetition can cross slot boundary. The duration of each repetition is L=8. This could achieve a more balanced duration for each repetition.

- Case 4: Enhanced scheme with one transmission with length L>14. A 16-symbol PUSCH can be scheduled in such case. This could lower the coding rate for potential better performance.

Figure 2. Cases for PUSCH type B scheduling in frame structure ‘DDDSU’ (S: 10D:2G:2U)

Figure 3. Simulation results for above four scheduling cases in frame structure ‘DDDSU’ (S: 10D:2G:2U)

In Figure 3, the performance of above four cases are provided. In the simulation, different MCS indexes are used based on the available resources in one nominal repetition. More detailed simulation assumptions are listed in Table A-2 in the appendix. As can be observed, the performance of the enhanced Case 3 and Case 4 is similar and both can provide about 0.8 dB gain over the baseline schemes.

Observation 2: In scenario with TDD frame structure ‘DDDSU’ (S: 10D:2G:2U),

- PUSCH repetition type B with actual repetition cross slot boundary can provide 0.8dB gain, and

- the length of actual repetition with 16 symbols can provide 0.8dB gain.

Based on above analysis, we have the following proposal.

Proposal 2: For PUSCH repetition type B, support actual PUSCH transmission across the slot boundaries and the length of actual repetition larger than 14 symbols.

Early termination

As mentioned above, PUSCH repetitions is one straightforward way for coverage enhancement while causes inefficiency. Though repetition is for performance enhancement, it is still possible for gNB to successfully decode the PUSCH based on only part of repetitions. Take 4 repetitions for instance, if the overall reliability is 99%, i.e., 1-(1-P)^4=0.99. The reliability for one repetition is P= 0.6838. Then, the probability of successful decoding based on the first 2 and 3 repetitions is 1-(1-P)^2 = 0.9 and 1-(1-P)^3 = 0.9684 respectively. This makes the rest of repetitions needless. Normally, one repetition would have a long length for coverage enhancement. Thus, it is also possible for gNB to terminate some of the needless repetitions. This is also beneficial in terms of UE power saving and interference reduction.

In Rel-16, DFI in NR-U is introduced for explicit ACK, which can be used as the starting point for early termination for NR coverage.

Proposal 3: Early termination can be considered for NR coverage enhancement.

OCC spreading based PUSCH repetition

Though PUSCH repetitions can improve coverage, it would also lead to inefficiency by applying repetitions, especially the number of repetitions is expected to be very large for coverage enhancement. In case of repetitions, it is naturally spread by an orthogonal code with all ‘1’ for each repetition. To improve the efficiency, orthogonal code division multiplexing among different UEs can be considered in case of repetitions. This is similar to using time domain OCC among OFDM symbols to enable PUCCH multiplexing among different UEs. Take Figure 4 as an example. Each block in the figure is one repetition. There are three UEs, where UE#1 has 4 repetitions and encoded with orthogonal code [1, 1, 1, 1], UE#2 and UE#3 both have two repetitions and encoded with orthogonal code [1, -1]. In such example, the three UEs can be multiplexed together even there is overlapping resources.

Figure 4. An example for OCC spreading based PUSCH repetitions.

Proposal 4: OCC spreading based PUSCH can be considered for NR coverage enhancement.

Frequency domain based enhancement

Inter-slot Frequency hopping

In case of inter-slot frequency hopping in Rel-15, frequency hopping occurs between every two slot-based transmissions. For inter-repetition frequency hopping for PUSCH repetition type B in Rel-16, frequency hopping occurs between every two nominal repetitions. In both Rel-15 and Rel-16, the hopping locations in frequency is at most 2, which may be not sufficient for power limited UEs with a small number of RBs. To investigate the potential benefits for introducing more frequency hopping positions, the simulation results for different number of frequency hopping positions are provided in Figure 5. The simulation assumptions are listed in Table A-3 in the Annex.

Figure 5. Simulation results for inter-slot frequency hopping

Based on Figure 5, the case with 4 hopping positions can provide additional 0.5 dB gain over inter-slot hopping in Rel-15 at target BLER 0.1. The performance gain is expected to be larger at a lower BLER target.

Observation 3: Increasing the number of frequency hopping positions from 2 to 4 could provide additional performance gain for PUSCH repetition.

Proposal 5: Enhancement to inter-slot frequency hopping is supported for NR coverage enhancement.

- More frequency hopping positions can be considered.

Frequency hopping with cross-slot channel estimation

- For NR coverage, a more flexible frequency pattern may be desirable to mitigate the inter-cell interference for cell edge UEs. In addition, a flexible time domain hopping interval is beneficial to achieve better channel estimation performance or DMRS sharing. For example, the time domain hopping interval can be configured as more than one repetitions/slots so as to enable joint DMRS estimation per hopping interval. In Figure 6, the performance of frequency hopping with cross-slot channel estimation are provided.

Figure 6. Performance of PUSCH with cross-slot channel estimation and frequency hopping

As can be observed, cross-slot channel estimation could provide about 0.5 dB gain for PUSCH with 4 repetitions in urban 4GHz scenario

- Observation 4: Frequency hopping together with cross-slot channel estimation can provide performance improvement for PUSCH repetition.

Proposal 6: For PUSCH repetition, support enhanced frequency hopping schemes to enable cross-slot channel estimation among repetitions per hop.

DMRS enhancement

Cross-slot channel estimation

In LTE short TTI, DMRS sharing among different PUSCH transmissions across different sTTI within a subframe is adopted. Similarly, DMRS sharing across different repetitions/slots can be considered in NR. This could improve the channel estimation performance by enabling joint cross-slot channel estimation. Due to power control is controlled by each repetition and timing advance is adjusted by each slot, whether the power phase continuity can be kept across slot boundary need further study.

Figure 7 shows the performance comparison of PUSCH w/ or w/o cross-slot channel estimation for 8 repetitions in urban scenario. For the both cases, no frequency hopping is assumed and one DMRS is configured for every PUSCH repetition. More detailed simulation assumptions are listed in Table A-4 in the Annex.

Figure 7. Performance comparison of PUSCH w/ or w/o DMRS sharing in case frequency hopping is disabled

As shown in Figure 7, the case with cross-slot channel estimation can provide additional 1.8 dB gain over the baseline case with 8 inter-slot repetitions at target BLER 0.1 in urban scenario.

Observation 5: Cross-slot channel estimation among 8 PUSCH repetitions can provide 1.8 dB gain in urban scenario.

Proposal 7: Cross-slot channel estimation among PUSCH repetitions should be supported.

  - Further study whether phase continuity can be kept across slot boundary.

DMRS overhead reduction in the frequency domain

In Rel-15/16, two DMRS configuration types are supported, where two interlaced CDM groups are specified for type 1 and 3 CDM groups for type 2. Depending on the number of DMRS CDM groups without data, 0 dB, 3 dB or 4.77 dB power boosting could be applied for DMRS REs compared to the data RE for PUSCH. For instance, if there are two DMRS CDM groups without data for one port transmission, DMRS will be only transmitted on one of the CDM group (i.e., only even or odd REs for DMRS type 1). The power from the remaining REs could be borrowed to boost 3dB power one the DMRS REs. This could improve the DMRS channel estimation for better performance.

To investigate the performance gain by lower DMRS density in the frequency domain, the following two cases are considered.

- Case 1: DMRS type 1, one port transmission. 3 dB power boosting compared to data RE is applied. This is a baseline case achieved by Rel-15/16.

- Case 2, Enhanced scheme with only mapping DMRS type 1 on even PRBs. In means 50% DMRS REs are saved compared to Case 1. In such case, 6 dB power boosting compared to data RE is applied.

- In Figure 8, the performance results for above two cases are provided. More detailed simulation assumptions are provided in Table A-5 in the appendix.

Figure 8. Performance results for lower DMRS density

As can be observed, about 1 dB gain can be obtained by the enhanced scheme with only mapping DMRS on even PRBs. Thus, schemes for lower DMRS density in the frequency domain can be studied in this SI.

Observation 6: Lower DMRS density with only mapping DMRS on half of the PRBs could provide about 1 dB performance gain.

Proposal 8: Support lower DMRS density in the frequency domain for NR coverage enhancement.

Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: For VoIP service, 4 repetitions with maximum 1 re-transmission can provide about 1~1.5 dB gain over 2 repetitions with maximum 3 re-transmissions.

Observation 2: In scenario with TDD frame structure ‘DDDSU’ (S: 10D:2G:2U),

- PUSCH repetition type B with actual repetition cross slot boundary can provide 0.8dB gain, and

- the length of actual repetition with 16 symbols can provide 0.8dB gain.

Observation 3: Increasing the number of frequency hopping positions from 2 to 4 could provide additional performance gain for PUSCH repetition.

- Observation 4: Frequency hopping together with cross-slot channel estimation can provide performance improvement for PUSCH repetition.

Observation 5: Cross-slot channel estimation among 8 PUSCH repetitions can provide 1.8 dB gain in urban scenario.

Observation 6: Lower DMRS density with only mapping DMRS on half of the PRBs could provide about 1 dB performance gain.

Proposal 1: For PUSCH repetition type A, the number of repetitions indicated by gNB should be guaranteed in case of collisions.

Proposal 2: For PUSCH repetition type B, support actual PUSCH transmission across the slot boundaries and the length of actual repetition larger than 14 symbols.

Proposal 3: Early termination can be considered for NR coverage enhancement.

Proposal 4: OCC spreading based PUSCH can be considered for NR coverage enhancement.

Proposal 5: Enhancement to inter-slot frequency hopping is supported for NR coverage enhancement.

- More frequency hopping positions can be considered.

Proposal 6: For PUSCH repetition, support enhanced frequency hopping schemes to enable cross-slot channel estimation among repetitions per hop.

Proposal 7: Cross-slot channel estimation among PUSCH repetitions should be supported.

  - Further study whether phase continuity can be kept across slot boundary.

Proposal 8: Support lower DMRS density in the frequency domain for NR coverage enhancement.

Reference

3GPP RAN#88-e, RP-200861, “Revised SID on NR coverage enhancement”, China Telecom.

3GPP RAN1#102-e, R1-2007404, [102-e-NR-CovEnh-03] Email discussion/approval on PUSCH coverage enhancement, Moderator (China Telecom).

3GPP RAN1#102-e, Chairman’s notes.

- Appendix

- Table A-1 Simulation assumption for VoIP repetition

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 for Rural scenario |
| UE speed | 120 km/h |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1 Tx |
| System bandwidth | 100 MHz |
| Sub-carrier spacing | 15 kHz |
| TBS for VoIP | 320bits |
| Waveform | DFT-s-OFDM |
| Number of OS per repetition | 14 (PUSCH mapping type A) |
| DMRS overhead | Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |
| Number of repetitions | 2/4 |
| Frequency hopping | w/o |
| Channel estimation | Practical |
| Receiver type | MMSE |

- Table A-2 Simulation assumption for enhanced PUSCH repetition type B

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 for Urban scenario |
| UE speed | 3 km/h |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 2 Tx |
| System bandwidth | 100 MHz |
| Sub-carrier spacing | 30kHz |
| Occupied RB | 4 |
| TB Size | 384bit |
| Waveform | DFT-s-OFDM |
| Frequency hopping | Disabled |
| Channel estimation | Practical |
| Receiver type | MMSE |
| RV | (0, 2, 3) for case 2, (0, 2) for case 3, 0 for other cases |

- Table A-3 Simulation assumption for PUSCH frequency hopping and cross-slot channel estimation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 120 km/h for O2O 3km/h for O2I |
| BS antenna configuration | 2 Rx for Rma 4Rx for Uma |
| UE antenna configuration | 1 Tx |
| System bandwidth | 100 MHz |
| Sub-carrier spacing | 30 kHz |
| Occupied RB | 4 for Rma 30 for Uma |
| MCS | MCS2 for Rma MCS3 for Uma |
| Waveform | DFT-s-OFDM |
| Number of OS per repetition | 14 (PUSCH mapping type A) |
| DMRS overhead | 1 DMRS symbol per hop |
| Number of repetitions | 4 |
| Frequency hopping | Enabled |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Hopping RB offset | 1/2 BW offset for the case with two frequency hopping positions. 1/4BW、1/2 BW 、3/4BW offset for the case with two frequency hopping positions. |

- Table A-4 Simulation assumption for cross-slot channel estimation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 for Urban scenario |
| UE speed | 3 km/h |
| BS antenna configuration | 8 Rx |
| UE antenna configuration | 2 Tx |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30kHz |
| Occupied RB | 15 |
| MCS | 5 |
| Waveform | DFT-s-OFDM |
| Number of OS per repetition | 14 (PUSCH mapping type A) |
| DMRS overhead | 1 DMRS symbol in all repetitions |
| Number of repetitions | 8 |
| Frequency hopping | Enabled |
| Channel estimation | Practical |
| Receiver type | MMSE |

- Table A-5 Simulation assumption for low DMRS density

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2GHz |
| Channel model | TDL-A 100ns |
| UE speed | 3 km/h |
| BS antenna configuration | 1 Rx |
| UE antenna configuration | 1 Tx, one port transmission. |
| System bandwidth | 20 MHz |
| Sub-carrier spacing | 15kHz |
| Occupied RB | 25 |
| Waveform | DFT-s-OFDM |
| Number of OS per repetition | 14 (PUSCH mapping type A) |
| DMRS overhead | 1 DMRS symbol, DMRS type 1. |
| Number of repetitions | 1 |
| Frequency hopping | Disable |
| Channel estimation | Practical |
| Receiver type | MMSE |