3GPP TSG RAN WG1 #103-e		                        R1-2008500

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.11.3

Source: 	ASUSTeK

Title:  	Discussion on SL DRX impact on physical layer

Document for:	Discussion and Decision

- Introduction

In WID [1], the objective is to specify radio solutions that can enhance NR sidelink for the V2X, public safety and commercial use cases. One direction of sidelink DRX is quoted as below:

3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2]

- Define on- and off-durations in sidelink and specify the corresponding UE procedure

- Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other

- Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

In this contribution, we discuss some physical layer impact induced by SL DRX.

- Discussion

As described in WID Justification, SL DRX is RAN 2 objective which can be supported for power saving. Following the concept of Uu DRX, in case of enabling SL DRX, a RX UE may monitor PSCCH during SL　DRX wake-up time and not monitor PSCCH during SL sleep time. Because of the SL sleep time, SL DRX is able to reduce power consumption for RX UE. However, if any PSCCH/PSSCH for the RX UE occurs during the SL sleep time, the RX UE will loss such PSCCH/PSSCH. For unicast and groupcast, the paired TX UE will need to retransmit the PSCCH/PSSCH. It will induce impact on resource efficiency, reliability and latency degradation, and even power consumption of the TX UE. For broadcast, the RX UE may loss other vehicles’ message without chance to request retransmission.

Observation 1:  SL DRX may induce RX UE loss some PSCCH/PSSCH reception in all cast type.

Although SL DRX is handled by RAN2, there seems some physical layer impact. To avoid such PSCCH/PSSCH reception loss, the paired TX UE needs to know SL DRX wake-up time of the RX UE. During resource selection procedure for transmitting PSCCH/PSSCH to the RX UE, the TX UE should ensure selected sidelink resources being within SL DRX wake-up time of the RX UE. One alternative is that RAN1 excludes sidelink resources within SL sleep time, and another alternative is that RAN2 selects sidelink resources within SL DRX wake-up time. In order to guarantee enough candidate resource set for selection, e.g. 20%, the former alternative is preferred from our aspect.

Proposal 1:  SL DRX wake-up time of intended RX UE is considered for mode 2 resource selection.

In last RAN1 meeting, there is discussion on mode 2 enhancement of “a resource set”, such that when a set of resources determined at UE-A is sent to UE-B in mode 2 and UE-B takes this into account in the resource selection for its own transmission. Related proposal is quoted as below.

Proposal 1: - When a set of resources determined at UE-A is sent to UE-B in mode 2 and UE-B takes this into account in the resource selection for its own transmission, - for the definition of “a set of resources”, at least followings can be considered: - Resource set which is preferred for UE-B’s transmission - e.g., - Resource set which is preferred for UE-A’s reception - Resource set which is preferred for intended receiver(s) of UE-B’s transmission - Resource set which is preferred not to be used by UE-B’s transmission - e.g., - Resource set which is not preferred for UE-A’s reception - Resource set with a problem for intended receiver(s) of UE-B’s transmission - FFS: whether the “resource set” in above candidates can individually refer to the resources in the past, in the future, or in both past and future. - FFS details on how UE-A determines “a set of resources” in the above definitions of “a set of resources”.

From our aspect, SL DRX can be one consideration for deriving the resource set. For instance, the UE-A (such as the RX UE) can determine a white resource set being within SL DRX wake-up time, or derive a black resource set being within SL sleep time. When UE-B (such as the paired TX UE) receives the resource set, UE-B can take this into account in resource selection for transmitting PSCCH/PSSCH to the UE-A. Thus, such mode 2 enhancement can be utilized to solve SL DRX impact at least for unicast and groupcast. As for broadcast, TX UE may not easily acquire SL DRX wake-up time of all surrounding RX UEs. It is also complicated for TX UE to perform resource selection with considering SL DRX operations of all surrounding RX UEs. Thus, one possible solution may be to design a pool-specific SL DRX configuration at least for broadcast. When TX UE intends to transmit a V2X message for surrounding pedestrians or power saving UE, pool-specific SL DRX configuration can work to avoid loss of such V2X message.

Proposal 2:  For mode 2 enhancement, UE-A determines the resource set with consideration of its SL DRX wake-up time or sleep time.

- Conclusion

In this contribution, we have following observation and proposals about SL DRX impact on physical layer:

Observation 1:  SL DRX may induce RX UE loss some PSCCH/PSSCH reception in all cast type.

Proposal 1:  SL DRX wake-up time of intended RX UE is considered for mode 2 resource selection.

Proposal 2:  For mode 2 enhancement, UE-A determines the resource set with consideration of its SL DRX wake-up time or sleep time.

- Reference

- RP-201516, “WID revision: NR sidelink enhancement”