3GPP TSG RAN WG1 #103-e																	R1-2008618

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.5.2

Source: 	Qualcomm Incorporated

Title: 	Evaluation of achievable Positioning Accuracy & Latency

Document for:		Discussion and Decision

# Introduction

In this document we provide evaluation results for accuracy and analysis for PHY-layer latency.

# Accuracy Evaluation

The system level simulations for positioning were conducted based on the agreed simulations Assumptions in [1].

  - Processing Steps of the Applied Algorithm

## Basic Processing Flow

The blocks shown in the below diagram correspond to the main aspects of the applied algorithm with regards to TDOA-based positioning procedures: OTDOA, UTDOA, multi-RTT, multi-RTT+AoA.

Perform position estimation

CER/CFR reception, TOA estimation/pruning, outlier rejection

RS Transmission from each gNB to a specific UE and/or from a UE to the gNB

Fix “good” Tx/Rx Beam pair for each gNB/UE link

## TOA Estimation, Pruning & Outlier Rejection

After all TOAs are estimated (e.g., from 18 cell-sites for InF), the TOAs are sorted according to a quality metric, and all the TOAs with a quality metric smaller than a specific threshold are removed while ensuring that at least 4 different cell-sites are used. Following the TOA pruning, an optional Random Sample and Consensus (RANSAC) outlier rejection procedure is applied on the remaining TOAs.  The algorithm starts with a list of randomly selected TOA subsets, among which the one with most TOAs agreeing to its estimate on UE position is determined.  The TOAs showing inconsistent result when examined with the selected TOA subset are deemed unreliable and removed from the TOA list. As the process proceeds in the OTDOA and UTDOA procedures, the reference TOA is then identified as the TOA with the highest quality metric among those remaining in the list.

Run TOA outlier rejection to remove unreliable TOAs  after pruning (Optional)

Sort the TOAs based on a Quality metric, e.g.,:

- the estimated SINR

- median/TOA-peak ratio

- median/main peak ratio

Prune the TOAs with a quality metric smaller than a threshold, ensuring that

- At least X different cell-sites are used (e.g., 4) and at least Y different gNBs remain

Identify the reference TOA as the TOA with the highest quality metric, and derive the TDOA vector

  - Baseline Simulation Results for Indoor Factory Scenarios

The simulation results for achievable horizontal and/or vertical accuracy are provided in this section, where TOA pruning based on link quality (LQ) metric is considered as a baseline algorithm to be used, and in several cases additional results with a RANSAC-based outlier rejection algorithm is also shown, as a more advanced algorithm which is implementable within NR Rel-16 specification.

## Case 1: InF-DH, ISD=20m, FR1, 100MHz, 30kHz SCS

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, InF FR1 DH ISD20, 100MHz, Link Quality, DL TDOA | Convex UEs | 0.55m | 6.42m | 15.35m | 31.66m |
|  | (Optional) All UEs | 0.67m | 6.51m | 13.37m | 26.8m |
| Case 1, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.071m | 0.37m | 1.32m | 6.92m |
|  | (Optional) All UEs | 0.085m | 0.52m | 2.17m | 8.25m |

## Case 2: InF-SH, ISD=50m, FR1, 100MHz, 30kHz SCS,

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 2, InF FR1 SH ISD50, 100MHz, Link Quality, DL TDOA | Convex UEs | 0.035m | 0.06m | 0.25m | 2.92m |
|  | (Optional) All UEs | 0.04m | 0.09m | 0.49m | 4.16m |
| Case 2, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.03m | 0.065 | 0.16m | 0.28m |
|  | (Optional) All UEs | 0.05m | 0.11m | 0.17m | 0.54m |

## Case 3: InF-DH, ISD=20m, FR1, 100MHz, 30kHz SCS, unequal gNB Height

For this Case, 3D positioning is performed. The gNBs are placed with unequal heights, i.e., half of the gNBs are at height 4m and the other half at height 8m. Two scenarios are simulated for this case: Variable UEs heights scenario in which the UEs height is randomly selected with uniform distribution U[0.5m, 2m], and Fixed UEs heights scenario in which the UEs height is fixed at 1.5m.

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 0.96m | 2.18m | 4.48m | 12.66m |
|  | (Optional) All UEs | 1.67m | 5.39m | 12.11m | 30.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 1.33m | 3.56m | 5.32m | 13.1m |
|  | (Optional) All UEs | 1.89m | 3.99m | 7.67m | 17.62m |

| Vertical Positioning error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 2.75m | 6.79m | 10.12m | 20.6m |
|  | (Optional) All UEs | 4.6m | 8.91m | 17.21m | 38.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 2.35m | 6.78m | 12.5m | 18.44m |
|  | (Optional) All UEs | 3.07m | 7.34m | 13.92m | 22.98m |

## Case 4: InF-SH, ISD=50m, FR1, 100MHz, 30kHz SCS, unequal gNB Height

For this Case, 3D positioning is performed. The gNBs are placed with unequal heights, i.e., half of the gNBs are at height 4m and the other half at height 8m. Two scenarios are simulated for this case: Variable UEs heights scenario in which the UEs height is randomly selected with uniform distribution U[0.5m, 2m], and Fixed UEs heights scenario in which the UEs height is fixed at 1.5m.

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Variable UE heights | Convex UEs | 0.03m | 0.05m | 0.1m | 0.22m |
|  | (Optional) All UEs | 0.04m | 0.09m | 0.17m | 0.47m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.03m | 0.05m | 0.08m | 0.12m |
|  | (Optional) All UEs | 0.046m | 0.08m | 0.16m | 0.36m |

| Vertical Positioning error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, , Unequal gNBs heights, Variable UE heights | Convex UEs | 0.183m | 0.33m | 0.75m | 1.89m |
|  | (Optional) All UEs | 0.162m | 0.37m | 0.78m | 2.63m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, Unequal gNBs heights, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.178m | 0.298m | 0.41m | 0.9m |
|  | (Optional) All UEs | 0.19m | 0.34m | 0.64m | 1.34m |

| Parameter |  | Case 1, InF-DH, FR1, DL-TDOA | Case 2, InF-SH, FR1, DL-TDOA | Case 3, InF-DH, FR1, DL-TDOA | Case 4, InF-SH, FR1, DL-TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 18(1-sector) | 18(1-sector) | 18(1-sector) | 18(1-sector) |
| Number of symbols used per occasion |  | 18 | 18 | 18 | 18 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates.. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |
| Additional notes, if any |  | All gNB’s are at height 8m and All UEs are at height 1.5m | All gNB’s are at height 8m and All UEs are at height 1.5m | gNBs Heights: Half of gNBs at 4m height and half at 8m height. UEs Heights: either Fixed UE heights at 1.5m or uniformly distributed U[0.5m,2m] | gNBs Heights: Half of gNBs at 4m height and half at 8m height. UEs Heights: either Fixed UE heights at 1.5m or uniformly distributed U[0.5m,2m] |

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps [m] |
| --- | --- | --- | --- |
| Case 1, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | 5.92 | 6.72 | 6.42 |
| Case 2, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Yes | 0.08 | Yes |

Rel.16 NR positioning –FR1 horizontal accuracy performance summary

Rel.16 NR positioning –FR1 horizontal accuracy performance summary

| Simulation case (Horizontal Error) |  | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps [m] |
| --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Variable UE heights | 11.66 | 12.46 | 12.16 |
|  | Fixed UE heights | 12.1 | 12.9 | 12.6 |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Variable UE heights | Yes | 0.02 | Yes |
|  | Fixed UE heights | Yes | Yes | Yes |

Rel.16 NR positioning –FR1 vertical accuracy performance summary

| Simulation case (Vertical Error) |  | Commercial vertical accuracy requirements [3]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% [m] | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% [m] | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If no, provide performance gaps @[90]% [m] |
| --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Variable UE heights | 17.6 | 20.4 | 19.6 |
|  | Fixed UE heights | 15.4 | 18.24 | 17.4 |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Variable UE heights | Yes | 1.69 | 0.89 |
|  | Fixed UE heights | Yes | 0.7 | Yes |

## Case 5: InF-SH, ISD=20m, FR2, 400MHz, 120kHz SCS DL-TDOA

## Case 6: InF-SH, ISD=20m, FR2, 400MHz, 120kHz SCS 3D mRTT, equal gNB

## Case 7: InF-DH, ISD=20m, FR2, 400MHz, 120kHz SCS DL-TDOA

## Case 8: InF-DH, ISD=20m, FR2, 400MHz, 120kHz SCS 3D mRTT, equal gNB

Rel.16 NR positioning – FR2 horizontal location error results across all UEs (m)

| Case ID | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0058 | 0.0077 | 0.011 | 0.019 |
|  | Strongest | 0.0074 | 0.010 | 0.017 | 0.027 |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.0061 | 0.0085 | 0.011 | 0.015 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0054 | 0.0082 | 0.013 | 0.025 |
|  | Strongest | 0.0084 | 0.0132 | 0.0287 | 9.40 |
| Case 8 InF-DH FR2 3d mRTT | Earliest | 0.0061 | 0.0088 | 0.013 | 0.020 |

Rel.16 NR positioning – FR2 horizontal location error results across all UEs within gNB convex hull (m)

| horizontal | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0043 | 0.0059 | 0.0071 | 0.0087 |
|  | Strongest | 0.0057 | 0.0080 | 0.0104 | 0.0170 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0039 | 0.0054 | 0.0070 | 0.0096 |
|  | Strongest | 0.0058 | 0.0098 | 0.0138 | 1.6767 |

Rel.16 NR positioning – FR2 Vertical location error results across all UEs (m)

| Vertical | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.037 | 0.048 | 0.061 | 0.084 |
| Case 8 InF-SH FR2 3d mRTT | Earliest | 0.010 | 0.015 | 0.023 | 0.041 |

Rel.16 NR positioning –FR2 horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| Case 5, Earliest beam pair | Yes. | Yes. | Yes. |
| Case 5, Strongest beam pair | Yes. | Yes. | Yes. |
| Case 6, Earliest beam pair | Yes. | Yes. | Yes. |
| Case 7, Earliest beam pair | Yes. | Yes. | Yes. |
| Case 7, Strongest beam pair | Yes. | 86% | 87% |
| Case 8, Earliest beam pair | Yes. | Yes. | Yes. |

Rel.16 NR positioning – FR2 vertical accuracy performance summary

| Simulation case (Vertical Error) | Commercial vertical accuracy requirements [3]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| Case 6, Earliest beam pair | Yes. | Yes. | Yes. |
| Case 8, Earliest beam pair | Yes. | Yes. | Yes. |

| Parameter |  | Case 5, InF-SH, FR2, DL-TDOA | Case 6, InF-SH, FR2, multi-RTT | Case 7, InF-DH, FR2, DL-TDOA | Case 8, InF-DH, FR2, multi-RTT |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing |  | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth |  | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb2 | Comb2 | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS&SRS-Pos | DL-PRS | DL-PRS&SRS-Pos |
| Number of sites |  | 18(3-sector) | 18(3-sector) | 18(3-sector) | 18(3-sector) |
| Number of symbols used per occasion |  | 54 (2 FDMed PRS symbols with two TRPs ) | 54 (2 FDMed PRS symbols with two TRPs ) on DL/UL | 54 (2 FDMed PRS symbols with two TRPs ) | 54 (2 FDMed PRS symbols with two TRPs ) on DL/UL |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | Not applied (23 dBm Tx) | NA | Not applied (23 dBm Tx) |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting for DL, interference from 4 UEs in UL | Ideal Muting | Ideal muting for DL, interference from 4 UEs in UL |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | multi-RTT, the algebraic solution proposed in -Norrdine, Abdelmoumen. (55). An Algebraic Solution to the Multilateration Problem. 10.13140/RG.2.1.1681.3602. Equal weight is used in the TOA covariance matrix. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | multi-RTT, the algebraic solution proposed in -Norrdine, Abdelmoumen. (55). An Algebraic Solution to the Multilateration Problem. 10.13140/RG.2.1.1681.3602. Equal weight is used in the TOA covariance matrix. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. The earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |
| Additional notes, if any |  |  | The UE’s height is lower than gNBs’ height is assumed as prior knowledge in positioning |  | The UE’s height is lower than gNBs’ height is assumed as prior knowledge in positioning |

  - Baseline Simulation Results for UMI/UMA/InH Scenarios

## Case 9: UMI in FR1 without Δτ, DL-TDOA or RTT

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.4 | 0.6 |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 0.64 | 1.2 | 2 | 3.2 |
| Case 9, UMI, FR1, RTT, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.16 | 0.22 | 0.3 | 0.4 |

## Case 10: UMI in FR1 with Δτ, DL-TDOA or RTT

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 3.4 | 5 | 6.2 | 8 |
| Case 10, UMI, FR1,RTT, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 4 | 4.2 | 6.9 | 9.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 5.7 | 8.1 | 9.9 | 17.3 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, LQ Algorithm | (Optional) All UEs | 7.9 | 11.5 | 14.3 | 20.1 |

## Case 11: UMA in FR1 without Δτ

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 11, UMA, FR1, DL-TDOA, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Outdoor UEs | 0.31 | 0.5 | 0.8 | 1.5 |
| Case 11, UMA, FR1, RTT, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Indoor UEs | 1.7 | 6.9 | 0.8 | 96 |

| Parameter |  | Case 9, UMI, FR1, DL-TDOA or RTT | Case 10, UMI with Δτ, FR1, DL-TDOA and RTT | Case 11, UMA, FR1, DL-TDOA |  |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz |  |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz |  |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 |  |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS |  |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) |  |
| Number of symbols used per occasion |  | 48 | 48 | 48 |  |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 |  |
| Power-boosting level |  | 0 | 0 | 0 |  |
| Uplink power control (applied/not applied) |  | NA | NA | NA |  |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync |  |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | NA | NA | NA |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |

## Case 12: UMI in FR2 without Δτ

## Case 13: InH in FR2 without Δτ

Rel. 16 NR positioning – FR2 horizontal location error results across all UEs (m)

| Horizontal Error (All UEs) |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.011 | 0.018 | 0.025 | 0.040 |
|  | Strongest | 0.13 | 0.80 | 2.63 | 6.32 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0058 | 0.0093 | 0.015 | 0.024 |
|  | Strongest | 0.0092 | 0.016 | 0.028 | 0.053 |

Rel. 16 NR positioning – FR2 horizontal location error results across UEs within gNB Convex Hull (m)

| Horizontal Error (convex Hull UEs) |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.0106 | 0.0178 | 0.0251 | 0.0403 |
|  | Strongest | 0.1333 | 0.7972 | 2.6315 | 6.3233 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0035 | 0.0053 | 0.0069 | 0.0113 |
|  | Strongest | 0.0066 | 0.0101 | 0.0160 | 0.0279 |

Rel.16 NR positioning – FR2 horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| Case 12 UMi Earliest | Yes. | Yes | Yes |
| Case 12 UMi Strongest | 70% | 55% | 58% |
| Case 13 InH Earliest | Yes. | Yes | Yes |
| Case 13 InH Strongest | Yes | Yes | Yes |

| Parameter | Case 12, UMi, FR2, DL-TDOA | Case 13, InH, FR2, DL-TDOA |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |
| Carrier frequency | 30GHz | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS |
| Number of sites | 19(3-sector) | 12(3-sector) |
| Number of symbols used per occasion | 58(2 FDMed PRS symbols with two TRPs ) | 36(2 FDMed PRS symbols with two TRPs) |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | No calibration error | No calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60, 180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |
| Additional notes, if any |  |  |

  - Simulation Results with Timing Errors

## Case 14: UMI in FR1 with Δτ, RTT

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 1 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.4 | 4.8 | 6 | 7.3 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.5 | 4.8 | 6 | 7.4 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.6 | 4.8 | 6 | 7.6 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.7 | 4.8 | 6 | 7.9 |

## Case 15: UMI in FR1 with Δτ, DL-TDOA

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.4 | 5.8 | 7.2 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |

## Case 16: UMI in FR1 without Δτ, RTT

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 16, UMI, FR1, RTT, Without Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.38 | 0.52 | 0.66 | 0.87 |
| Case 16, UMI, FR1, RTT, Without Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.68 | 0.89 | 1.1 | 1.37 |
| Case 16, UMI, FR1, RTT, Without Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.11 | 1.45 | 1.75 | 2.22 |

## Case 17: UMI in FR1 without Δτ, DL-TDOA

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.41 | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.46 | 0.73 | 0.76 | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.2 | 1.55 | 1.89 | 2.3 |

| Parameter |  | Case 14, UMI with Δτ, FR1, RTT | Case 15, UMI with Δτ, FR1, DL-TDOA | Case 16, UMI without Δτ, FR1, RTT | Case 17, UMI without Δτ, FR1, DL-TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz | 4 GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) | 19(3-sector) |
| Number of symbols used per occasion |  | 48 | 48 | 48 | 48 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync |  |
| UE/gNB Tx/Rx Calibration Error |  | The realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings. The Tx errors at gNB are assumed to be independent |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | NA | NA | NA | NA |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |

## Case 18: InF-SH, ISD=50m, FR2, 400MHz, 120kHz SCS  DL-TDOA

In this scenario, we show results for 400 MHz InF-SH FR2, with the realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings.

- The Tx errors at gNB are assumed to be independent and follows with a truncated Gaussian distribution.

- No Rx timing error is added.

## Case 19: InF-DH, ISD=20m, FR2, 400MHz, 120kHz SCS DL-TDOA

In this scenario, we show results for 400 MHz InF-DH FR2, with the realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings.

- The Tx errors at gNB are assumed to be independent and follows with a truncated Gaussian distribution.

- No Rx timing error is added.

## Case 20: UMI in FR2 with DL-TDOA

In this scenario, we show results for 400 MHz UMi  FR2, with the realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings.

- The Tx errors at gNB are assumed to be independent and follows with a truncated Gaussian distribution.

- No Rx timing error is added.

## Case  21: InH in FR2 with DL-TDOA

In this scenario, we show results for 400 MHz InH FR2, with the realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings.

- The Tx errors at gNB are assumed to be independent and follows with a truncated Gaussian distribution.

- No Rx timing error is added.

Tx/Rx timing error – FR2 evaulation results

| Case ID | Beam Pair | Tx T1 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| Case 18 InF-SH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.01 |
|  |  | 0.1ns | 0.02 | 0.03 | 0.04 | 0.06 |
|  |  | 0.2ns | 0.05 | 0.07 | 0.09 | 0.13 |
|  |  | 0.5ns | 0.14 | 0.19 | 0.27 | 0.40 |
|  |  | 1.0ns | 0.28 | 0.40 | 0.55 | 0.96 |
|  |  | 2.0ns | 0.55 | 0.75 | 1.08 | 1.59 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.02 |
|  |  | 0.1ns | 0.03 | 0.05 | 0.07 | 0.11 |
|  |  | 0.2ns | 0.06 | 0.10 | 0.14 | 0.25 |
|  |  | 0.5ns | 0.19 | 0.29 | 0.47 | 0.94 |
|  |  | 1.0ns | 0.40 | 0.63 | 1.08 | 2.30 |
|  |  | 2.0ns | 0.82 | 1.22 | 1.87 | 5.19 |
| Case 19 InF-DH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.02 |
|  |  | 0.1ns | 0.03 | 0.04 | 0.06 | 0.10 |
|  |  | 0.2ns | 0.06 | 0.08 | 0.12 | 0.24 |
|  |  | 0.5ns | 0.16 | 0.24 | 0.40 | 0.76 |
|  |  | 1.0ns | 0.34 | 0.47 | 0.78 | 2.10 |
|  |  | 2.0ns | 0.74 | 1.17 | 2.15 | 6.31 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.05 | 43.94 |
|  |  | 0.1ns | 0.05 | 0.08 | 3.61 | 36.56 |
|  |  | 0.2ns | 0.10 | 0.21 | 7.91 | 35.28 |
|  |  | 0.5ns | 0.29 | 0.73 | 18.49 | 44.16 |
|  |  | 1.0ns | 0.74 | 1.96 | 18.90 | 40.33 |
|  |  | 2.0ns | 1.73 | 6.42 | 20.03 | 46.31 |
| Case 20 InH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.02 | 0.03 |
|  |  | 0.1ns | 0.05 | 0.08 | 0.13 | 0.31 |
|  |  | 0.2ns | 0.11 | 0.17 | 0.28 | 0.47 |
|  |  | 0.5ns | 0.23 | 0.40 | 0.60 | 1.16 |
|  |  | 1.0ns | 0.51 | 0.71 | 1.19 | 2.04 |
|  |  | 2.0ns | 0.97 | 1.49 | 2.23 | 4.11 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.03 | 0.21 |
|  |  | 0.1ns | 0.10 | 0.22 | 0.76 | 7.44 |
|  |  | 0.2ns | 0.22 | 0.48 | 1.94 | 8.22 |
|  |  | 0.5ns | 0.70 | 2.42 | 5.24 | 11.37 |
|  |  | 1.0ns | 1.26 | 3.45 | 6.88 | 14.94 |
|  |  | 2.0ns | 2.08 | 4.38 | 8.21 | 18.87 |
| Case 21 UMi, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.02 | 0.03 |
|  |  | 0.1ns | 0.04 | 0.05 | 0.08 | 0.13 |
|  |  | 0.2ns | 0.07 | 0.10 | 0.13 | 0.20 |
|  |  | 0.5ns | 0.15 | 0.20 | 0.26 | 0.38 |
|  |  | 1.0ns | 0.32 | 0.42 | 0.54 | 0.73 |
|  |  | 2.0ns | 0.63 | 0.82 | 1.02 | 1.38 |
|  | Strongest beam pair | 0.0ns | 0.47 | 3.08 | 7.73 | 16.21 |
|  |  | 0.1ns | 2.10 | 6.20 | 12.72 | 23.54 |
|  |  | 0.2ns | 1.20 | 4.23 | 9.16 | 16.59 |
|  |  | 0.5ns | 1.69 | 3.81 | 7.18 | 19.30 |
|  |  | 1.0ns | 2.08 | 4.00 | 7.50 | 14.27 |
|  |  | 2.0ns | 2.95 | 6.52 | 12.89 | 21.65 |

Tx/Rx timing error – FR2 horizontal accuracy performance summary

| Case ID | Beam Pair | Tx T1 | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- | --- |
| Case 18 InF-SH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 100% | 100% | 100% |
|  |  | 0.1ns | 100% | 99% | 100% |
|  |  | 0.2ns | 100% | 95% | 98% |
|  |  | 0.5ns | 99% | 70% | 84% |
|  |  | 1.0ns | 91% | 31% | 52% |
|  |  | 2.0ns | 78% | 11% | 20% |
|  | Strongest beam pair | 0.0ns | 100% | 100% | 100% |
|  |  | 0.1ns | 99% | 95% | 98% |
|  |  | 0.2ns | 98% | 87% | 92% |
|  |  | 0.5ns | 91% | 52% | 68% |
|  |  | 1.0ns | 79% | 22% | 36% |
|  |  | 2.0ns | 58% | 9% | 16% |
| Case 19 InF-DH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 98% | 98% | 98% |
|  |  | 0.1ns | 96% | 94% | 96% |
|  |  | 0.2ns | 94% | 89% | 92% |
|  |  | 0.5ns | 92% | 63% | 74% |
|  |  | 1.0ns | 84% | 27% | 42% |
|  |  | 2.0ns | 62% | 6% | 16% |
|  | Strongest beam pair | 0.0ns | 88% | 88% | 88% |
|  |  | 0.1ns | 84% | 80% | 82% |
|  |  | 0.2ns | 83% | 70% | 76% |
|  |  | 0.5ns | 73% | 38% | 53% |
|  |  | 1.0ns | 58% | 15% | 25% |
|  |  | 2.0ns | 39% | 3% | 7% |
| Case 20 InH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 100% | 100% | 100% |
|  |  | 0.1ns | 98% | 86% | 90% |
|  |  | 0.2ns | 97% | 72% | 81% |
|  |  | 0.5ns | 89% | 45% | 58% |
|  |  | 1.0ns | 78% | 13% | 28% |
|  |  | 2.0ns | 52% | 4% | 10% |
|  | Strongest beam pair | 0.0ns | 93% | 90% | 91% |
|  |  | 0.1ns | 82% | 64% | 71% |
|  |  | 0.2ns | 76% | 48% | 58% |
|  |  | 0.5ns | 55% | 21% | 35% |
|  |  | 1.0ns | 45% | 10% | 18% |
|  |  | 2.0ns | 30% | 3% | 4% |
| Case 21 UMi, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 100% | 100% | 100% |
|  |  | 0.1ns | 98% | 94% | 97% |
|  |  | 0.2ns | 99% | 90% | 94% |
|  |  | 0.5ns | 99% | 65% | 84% |
|  |  | 1.0ns | 95% | 25% | 45% |
|  |  | 2.0ns | 79% | 10% | 16% |
|  | Strongest beam pair | 0.0ns | 56% | 42% | 43% |
|  |  | 0.1ns | 42% | 25% | 28% |
|  |  | 0.2ns | 48% | 28% | 32% |
|  |  | 0.5ns | 43% | 15% | 23% |
|  |  | 1.0ns | 36% | 7% | 14% |
|  |  | 2.0ns | 25% | 2% | 5% |

| Parameter | Case 18, InF-SH, FR2, DL-TDOA | Case 19, InF-DH, FR2, DL-TDOA | Case 20, UMi, FR2, DL-TDOA | Case 21, InH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |
| Carrier frequency | 28GHz | 28GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites | 18(3-sector) | 18(3-sector) | 19(3-sector) | 12(3-sector) |
| Number of symbols used per occasion | 54(2 FDMed PRS symbols with two TRPs ) | 54(2 FDMed PRS symbols with two TRPs ) | 58(2 FDMed PRS symbols with two TRPs ) | 36(2 FDMed PRS symbols with two TRPs) |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. . |  |  |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | The realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings The Tx errors at gNB are assumed to be independent and T1 = [0, 0.1 0.2 0.5 1 2]. The UE is assumed to has one panel, and thus no Rx timing error with DL-TDOA. |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |
| Additional notes, if any |  |  |  |  |

  - Simulation Results for PRS Frequency Domain stitching

## Case 22: InH in FR1 with Phase Offset between 2 PFLs, LQ

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 22, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 2 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 1.1 | 1.4 | 2.5 |
| Case 22, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.5 | 1.2 | 1.4 | 2.5 |

## Case 23: InH in FR1 with Phase Offset between 4 PFLs, LQ

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 23, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.12 | 0.3 | 0.5 | 0.9 |
| Case 23, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.15 | 0.35 | 0.9 | 2 |
| Case 23, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.2 | 0.4 | 1.1 | 2.4 |
| Case 23, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 1 | 1.2 | 2.5 |

## Case 24: InH in FR1 with channel spacing between 2 PFLs, LQ

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 24, InH, FR1, Channel Spacing = 0 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.35 | 0.8 | 1.5 | 3 |
| Case 24, InH, FR1, Channel Spacing = 7.2 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 0.9 | 1.7 | 4 |
| Case 24, InH, FR1, Channel Spacing = 14.4 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.5 | 1.8 | 2.3 | 5 |
| Case 24, InH, FR1, Channel Spacing = 36 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.8 | 1.9 | 2.8 | 5.5 |

## Case 25: UMI in FR1 with Phase Offset, DL-TDOA, RANSAC

NR positioning enhancements - horizontal location error results

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 25, UMI, FR1,Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.6 m | 1 m |
| Case 25, UMI, FR1,Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.7 m | 1.2 m |
| Case 25, UMI, FR1,Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 m | 0.55 m | 0.7 m | 1.5 m |
| Case 25, UMI, FR1,Phase offset = 12 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.5 m | 0.7 m | 1.2 m | 2.2 m |
| Case 25, UMI, FR1,Phase offset = 18 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.6 m | 1 m | 1.8 m | 4 m |

## Case 26: UMI in FR1 with Time Offset, DL-TDOA, RANSAC

NR positioning enhancements - horizontal location error results

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 26, UMI, FR1,Time offset = 0 ns, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.6 m | 1 m |
| Case 26, UMI, FR1,Time offset = 0.5 ns, Frequency-domain Stitching | (Optional) All UEs | 0.38 m | 0.52 m | 0.7 m | 1 m |
| Case 26, UMI, FR1,Time offset = 1 ns, Frequency-domain Stitching | (Optional) All UEs | 0.38 m | 0.55 m | 0.7m | 1 m |
| Case 26, UMI, FR1,Time offset = 2 ns, Frequency-domain Stitching | (Optional) All UEs | 0.4 m | 0.58 m | 0.6 m | 1.5 m |
| Case 26, UMI, FR1,Time offset = 5 ns, Frequency-domain Stitching | (Optional) All UEs | 0.5 m | 0.8 m | 1 m | 1.5 m |

| Parameter |  | Case 22, InH, FR1, 100+100 MHz, No Frequency gap, Phase Offset, Link Quality, DL-TDOA | Case 23, InH, FR1, 100+100+100+100 MHz, No Frequency gap, Phase Offset, Link Quality, DL-TDOA | Case 24, InH, FR1, 100+100 MHz, Variable channel gap, RANSAC | Case 25, UMI, FR1, 100+100 MHz, No Frequency gap, Phase Offset, RANSAC | Case 26, UMI, FR1, 100+100 MHz, No Frequency gap, Time Offset, RANSAC |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz | 4 GHz | 4 GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) | 19(3-sector) | 19(3-sector) |
| Number of symbols used per occasion |  | 48 | 48 | 48 | 48 | 48 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates.. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |

PRS frequency domain stitching – FR1 InH/UMI simulation assumptions and setup

## Case 27: InF in FR1 with Phase Offset

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 0 | 0.117m | 4.9m | 13.2m | 24.4m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 45 | 0.108m | 5.3m | 13.3m | 25.6m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 90 | 0.126m | 6.09m | 13.8m | 25.5m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 0 | 0.067m | 4.47m | 14.26m | 25.7m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 45 | 0.09m | 3.79m | 13.1m | 25.46m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 90 | 1.04m | 7.02m | 15.1m | 28.9m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 0 | 0.04m | 0.07m | 0.17m | 3.4m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 45 | 0.04m | 0.07m | 0.17m | 2.9m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 90 | 0.04m | 0.08m | 0.22m | 3.4m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 0 | 0.02m | 0.06m | 0.14m | 4.25m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 45 | 0.03m | 0.06m | 0.21m | 4.26m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 90 | 0.03m | 0.06m | 0.83m | 4.43m |

| Parameter |  | Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100 MHz Gap | Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 18(1-sector) | 18(1-sector) | 18(1-sector) | 18(1-sector) |
| Number of symbols used per occasion |  | 18 | 18 | 18 | 18 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates.. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |
| Additional notes, if any |  | This case is simulated with phase shifts between the two aggregated CCs assumed to be 0, 45, 90 degrees | This case is simulated with phase shifts between the two aggregated CCs assumed to be 0, 45, 90 degrees | This case is simulated with phase shifts between the two aggregated CCs assumed to be 0, 45, 90 degrees | This case is simulated with phase shifts between the two aggregated CCs assumed to be 0, 45, 90 degrees |

PRS frequency domain stitching – FR1 InH simulation assumptions and setup

| Simulation case (Horizontal Error) | Phase Shift between the two aggregated component carriers | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps [m] |
| --- | --- | --- | --- | --- |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | 0 degrees | 23.4 | 24.2 | 23.9 |
|  | 45 degrees | 24.6 | 25.4 | 25.1 |
|  | 90 degrees | 24.5 | 25.3 | 25 |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap | 0 degrees | 24.7 | 25.5 | 25.2 |
|  | 45 degrees | 24.4 | 25.2 | 24.9 |
|  | 90 degrees | 27.9 | 28.7 | 28.4 |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | 0 degrees | 2.4 | 3.2 | 2.9 |
|  | 45 degrees | 1.9 | 2.7 | 2.4 |
|  | 90 degrees | 2.4 | 3.2 | 2.9 |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap | 0 degrees | 3.25 | 4.05 | 3.75 |
|  | 45 degrees | 3.26 | 4.06 | 3.76 |
|  | 90 degrees | 3.43 | 4.23 | 3.93 |

## Case  28: InF-SH in FR2 with Phase Offset

## Case  29: InF-DH in FR2 with Phase Offset

## Case  30: InH in FR2 with Phase Offset

## Case  31: UMI in FR2 with Phase Offset

## Case  32: UMI in FR2 with Time offset

## Case  33: InH in FR2 with Time offset

PRS frequency domain stitching – FR2 evaluation results

| Case ID | Phase Shift θ | Time offset t | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| Case 28 InF-SH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.14 |
|  | 10 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.15 |
|  | 20 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.12 |
|  | 40 | 0.0ns | 0.01 | 0.02 | 0.08 | 1.37 |
|  | 60 | 0.0ns | 0.01 | 0.02 | 0.18 | 1.14 |
|  | 80 | 0.0ns | 0.01 | 0.02 | 0.07 | 0.96 |
| Case 29 InF-DH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 2.72 | 12.41 | 28.39 |
|  | 10 | 0.0ns | 0.01 | 2.82 | 12.43 | 29.53 |
|  | 20 | 0.0ns | 0.01 | 2.82 | 12.41 | 28.39 |
|  | 40 | 0.0ns | 0.01 | 2.88 | 12.42 | 28.33 |
|  | 60 | 0.0ns | 0.02 | 3.95 | 13.62 | 30.92 |
|  | 80 | 0.0ns | 0.02 | 3.65 | 13.60 | 29.54 |
| Case 30 UMi, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.05 |
|  | 10 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.05 |
|  | 20 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.04 |
|  | 40 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.25 |
|  | 60 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.11 |
|  | 80 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.07 |
| Case 31 InH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.04 | 0.08 | 0.17 | 0.42 |
|  | 10 | 0.0ns | 0.04 | 0.09 | 0.20 | 0.53 |
|  | 20 | 0.0ns | 0.04 | 0.09 | 0.20 | 0.56 |
|  | 40 | 0.0ns | 0.05 | 0.12 | 0.35 | 0.72 |
|  | 60 | 0.0ns | 0.06 | 0.21 | 0.52 | 0.94 |
|  | 80 | 0.0ns | 0.07 | 0.28 | 0.53 | 0.97 |
| Case 32 UMi, FR2, DL-TDOA, RANSAC | 0 | 0.0ns | 0.011 | 0.018 | 0.025 | 0.040 |
|  | 0 | 0.1ns | 0.011 | 0.018 | 0.026 | 0.043 |
|  | 0 | 0.5ns | 0.011 | 0.018 | 0.029 | 0.046 |
|  | 0 | 1.0ns | 0.012 | 0.022 | 0.032 | 0.052 |
|  | 0 | 5.0ns | 0.016 | 0.030 | 0.050 | 0.120 |
|  | 0 | 10.0ns | 0.021 | 0.038 | 0.066 | 0.220 |
|  | 0 | 20.0ns | 0.035 | 0.059 | 0.111 | 0.269 |
| Case 33 InH, FR2, DL-TDOA, RANSAC | 0 | 0.0ns | 0.006 | 0.009 | 0.015 | 0.024 |
|  | 0 | 0.1ns | 0.006 | 0.009 | 0.015 | 0.027 |
|  | 0 | 0.5ns | 0.006 | 0.009 | 0.014 | 0.027 |
|  | 0 | 1.0ns | 0.006 | 0.011 | 0.017 | 0.027 |
|  | 0 | 5.0ns | 0.008 | 0.013 | 0.028 | 0.066 |
|  | 0 | 10.0ns | 0.008 | 0.014 | 0.027 | 0.057 |
|  | 0 | 20.0ns | 0.009 | 0.015 | 0.031 | 0.070 |

PRS frequency domain stitching – FR2 observations

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| 28 2*200MHz Perfect phase | Yes. | Yes. | Yes. |
| 28 200MHZ(baseline) | Yes. | Yes. | Yes. |
| 29 2*200MHz Perfect phase | 63% | 62% | 62% |
| 29 200MHZ(baseline) | 62% | 61% | 61% |
| 30 2*200MHz Perfect phase | Yes. | 78% | 83% |
| 30 200MHZ(baseline) | Yes. | 66% | 77% |
| 31 2*200MHz Perfect phase | Yes. | Yes. | Yes. |
| 31 200MHZ(baseline) | Yes. | Yes. | Yes. |
| 32 2*200MHz Perfect time | Yes. | 78% | 83% |
| 32 200MHZ(baseline) | Yes. | 66% | 77% |
| 33 2*200MHz Perfect time | Yes. | Yes. | Yes. |
| 33 200MHZ(baseline) | Yes. | Yes. | Yes. |

PRS frequency domain stitching – FR2 simulation setup

| Parameter | Case 28, InF-SH, FR2, DL-TDOA | Case 29, InF-DH, FR2, DL-TDOA | Case 30, UMi, FR2, DL-TDOA | Case 31, InH, FR2, DL-TDOA | Case 32, UMi, FR2, DL-TDOA |  | ase 33, InH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |  |  |
| Carrier frequency | 28GHz | 28GHz | 30GHz | 30GHz | 30GHz |  | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |  | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |  | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 | Comb2 | Comb2 | Comb2 |  | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS | DL-PRS | DL-PRS | DL-PRS |  | DL-PRS |
| Number of sites | 18(3-sector) | 18(3-sector) | 19(3-sector) | 12(3-sector) | 19(3-sector) |  | 12(3-sector) |
| Number of symbols used per occasion | 54(2 FDMed PRS symbols with two TRPs ) | 54(2 FDMed PRS symbols with two TRPs ) | 58(2 FDMed PRS symbols with two TRPs ) | 36(2 FDMed PRS symbols with two TRPs) | 58(2 FDMed PRS symbols with two TRPs ) |  | 36(2 FDMed PRS symbols with two TRPs) |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 |  | 1 |
| Power-boosting level | 0 | 0 | 0 | 0 | 0 |  | 0 |
| Uplink power control (applied/not applied) | NA | NA | NA | NA | NA |  | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting |  | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. |  |  |  |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |  | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | NA |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |  |  |
| PRS Frequency Domain stitching | 2X200MHz Continuous PFLs | 2X200MHz Continuous PFLs | 2X200MHz Continuous PFLs | 2X200MHz Continuous PFLs | 2X200MHz Continuous PFLs |  | 2X200MHz Continuous PFLs |
| Phase Shift | Randomly select a phase shift value within -θ,θ and apply the phase shift on the second PFL per PRS, θ= [0,10, 20, 40,60,80] |  |  |  | NA |  | NA |
| Phase Ramp(Time offset) | NA | NA | NA | NA | Randomly select a time offset value within -t,t and convert the time offset as a phase ramp offset on second PFL per PRS, t= [0,0.1, 0.5, 1, 5, 10,20]ns |  |  |

PRS frequency domain stitching – FR2 simulation assumptions and setup

  - Simulation Results for Kinematic constraint aware positioning

Two scenarios are considered for the case when one coordinate is known:

- In scenario 1, the node computing the position knows the value of one of the coordinates, and only computes the other one. This is denoted in the figure legends as ‘D unknown in estimation and RMS calculation’ (where D=X or Y, depending on which coordinate is unknown).

- In scenario 2, the node computing the position has no side information and computes both X and Y. However, the LCS client, i.e., the consumer of the computed (X,Y) position, knows the value of one of the coordinates, and thus ignores that one and uses only the other one.

## Case 34: InF-SH in FR2 with DL-TDOA

## Case 35: InF-DH in FR2 with DL-TDOA

## Case 36: InH in FR2 with DL-TDOA

Kinematic constraint aware positioning – FR2 evaluation and observation results

| Case ID | Kinematic constraint condition | 50% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 34 InF-SH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.024 | 0.033 | 0.049 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.017 | 0.024 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.017 | 0.024 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.017 | 0.024 | 0.031 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.017 | 0.026 | 0.036 |
| Case 35 InF-DH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.030 | 0.038 | 0.058 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.014 | 0.021 | 0.029 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.016 | 0.023 | 0.034 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.018 | 0.028 | 0.038 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.018 | 0.029 | 0.044 |
| Case 36 InH, FR2 DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.031 | 0.046 | 0.071 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.019 | 0.026 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.013 | 0.019 | 0.028 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.022 | 0.031 | 0.044 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.023 | 0.035 | 0.053 |

Kinematic constraint aware positioning – FR2 simulation assumptions and setup

| Kinematic constraint aware positioning – FR2 simulation assumptions and setupParameter | Case 34, InF-SH, FR2, DL-TDOA | Case 35, InF-DH, FR2, DL-TDOA | Case 36, InH, FR2, DL-TDOA |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |
| Carrier frequency | 28GHz | 28GHz | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS | DL-PRS |
| Number of sites | 18(3-sector) | 18(3-sector) | 12(3-sector) |
| Number of symbols used per occasion | 54 (2 FDMed PRS symbols with two TRPs ) | 54 (2 FDMed PRS symbols with two TRPs ) | 36 (2 FDMed PRS symbols with two TRPs ) |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 0 | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | NA |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |
| Kinematic Constraint awareness | In scenario 1, the node computing the position knows the value of one of the coordinates, and only computes the other one. This is denoted in the figure legends as ‘D unknown in estimation and RMS calculation’ (where D=X or Y, depending on which coordinate is unknown). In scenario 2, the node computing the position has no side information and computes both X and Y. However, the LCS client, i.e., the consumer of the computed (X,Y) position, knows the value of one of the coordinates, and thus ignores that one and uses only the other one. |  |  |

  - Simulation Results for RTT + AoA Fusion & Enhancement in the AoA and Additional Path Reporting

## Case 37: UMI in FR1 with Δτ, RTT+AoA

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 37, UMI, FR1,RTT+AoA, With Delta Tau, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 2.6 | 4.4 | 5.8 | 7.3 |
| Case 37, UMI, FR1,RTT, With Delta Tau, Perfect Sync, No Timing Errors, AoA & UL PDP Enhancement | (Optional) All UEs | 1.2 | 1.8 | 2.7 | 3.9 |

  - Simulation Results with Enhancement on the Additional Path Reporting

## Case 38: UMI in FR1 with Δτ, DL-TDOA

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 38, UMI, FR1,DL-TDOA, With Δτ, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 4.1 | 5.2 | 6.8 | 9 |
| Case 38, UMI, FR1,DL-TDOA, With Δτ, Perfect Sync, No Timing Errors, UL PDP Enhancement | (Optional) All UEs | 3.6 | 4.85 | 6 | 8 |

## Case 39: UMI in FR1 with Δτ, RTT

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 39, UMI, FR1, RTT, With Delta Tau, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 3.38 | 4.9 | 6.2 | 9 |
| Case 39, UMI, FR1, RTT, With Delta Tau, Perfect Sync, No Timing Errors, UL PDP Enhancement | (Optional) All UEs | 1.6 | 2.4 | 3.4 | 4.7 |

| Parameter |  | Case 37, UMI with Δτ, FR1, RTT+AoA | Case 38, UMI with Δτ, FR1, DL-TDOA | Case 39, UMI with Δτ, FR1, RTT |  |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz |  |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz |  |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 |  |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS |  |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) |  |
| Number of symbols used per occasion |  | 48 | 48 | 48 |  |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 |  |
| Power-boosting level |  | 0 | 0 | 0 |  |
| Uplink power control (applied/not applied) |  | NO | NA | NO |  |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync |  |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | NA | NA | NA |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |

  - Genie LOS and Outlier Rejection Comparison

## Case 40: InF-DH, FR1, 100 MHz

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 40, InF FR1 DH ISD20, 100MHz, LOS Genie + Link Quality | 0.36 | 4.08 | 10.28 | 35.65 |
| Case 40, InF FR1 DH ISD20, 100MHz, RANSAC | 0.085m | 0.52m | 2.17m | 8.25m |

| Parameter |  | Case 40, InF-DH, FR1, DL-TDOA |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |
| Carrier frequency |  | 3.5GHz |
| Subcarrier spacing |  | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS |
| Number of sites |  | 18(1-sector) |
| Number of symbols used per occasion |  | 18 |
| number of occasions used per positioning estimate |  | 1 |
| Power-boosting level |  | 0 |
| Uplink power control (applied/not applied) |  | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |
| Network synchronization assumptions |  | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |
| Additional notes, if any |  | All gNB’s are at height 8m and All UEs are at height 1.5m |

- Phy-layer Latency Analysis

With regards to the PHY-layer latency the following agreement was made previous meeting:

Agreement: Physical Layer Latency Start and End times are defined as follows: Method; Start; End. UE assisted DL-only & DL-ECID & Multi-RTT; Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message; Successful decoding of the PUSCH carrying the LPP Provide Location Information message. UL-only method & UL ECID & Multi-RTT; Reception by the gNB of the NRPPa measurement request message; The transmission by the gNB of the NRPPa measurement response message. UE-based; Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives.; Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE.

There were 3 alternatives written in the previous agreement with regards to the start time of the PHY-layer latency evaluation for UE-based methods. We make the following observations:

- For a  UE in RRC connected state and UE-based imitated by an internal client, the UE needs to request MGs in NR Rel-16. Such request should be considered as part of the PHY-layer latency discussions because it includes request towards the serving cell which can be optimized in several ways with solutions that can be, at least partially, within RAN1 scope (e.g., DCI or MAC-CE MG request vs. RRC-based MG request, PRS processing without MG).

  - Therefore, for such scenario Alt. 1 makes sense

- For a UE in RRC Idle/Inactive and UE-based imitated by an internal client, there is no need of MGs in NR Rel-16. Also the assistance data will be received through the already specified feature of broadcast of assistance data.

  - Therefore, for such scenario Alt. 3 makes more sense

## UE-Assisted DL-only Positioning, RRC Connected State

| Steps | “Best-case” Estimate of Duration (msec) in NR Rel-16 | “Conservative” Estimate of the Duration (msec) in NR Rel-16 | Short Explanation |
| --- | --- | --- | --- |
| Step 1: Serving gNB sends PDSCH that contains the Location Information Request and is successfully decoded in physical layer by the UE | [1] | [2] | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 2: UE interprets and applies the LPP message (Location Request) | 10 | 10 | RRC Processing time are captured in 38.331 Section 12 |
| Step 3: Request of Location Information has be received at the UE. UE transmits in PUSCH an MG-request through an RRC message | [1] | [4] | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 4: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 5: Serving gNB transmits the RRC configuration of the MG request | [1] | [2] | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 6: UE receives the message, interprets and applies the MG configuration | 10 | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 7: UE receives at least one occasion of the PRS within the MG. | 20 | 4*160 = 640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 8: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 | 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 9: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | [2] | [5] | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total PHY-layer Latency (msec) | [57] | [823] | Latency needed from the time applies the location request up to the decoding of the PUSCH carrying the Positionign report. |

## UE-based DL-only Positioning, RRC Inactive State, External Client

| Steps | “Best-case” Estimate of Duration (msec) in NR Rel-16 | “Conservative” Estimate of the Duration (msec) in NR Rel-16 | Short Explanation |
| --- | --- | --- | --- |
| Step 1: UE receives at least one occasion of the PRS | 20 | 4*160 = 640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 2: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 | 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 4: UE transisitions to RRC Connected state | 11.3 | 18.5 | Transition from RRC Inactive to Connected state. 11.3 ms – 18.5 ms dependent on configuration/deployment (TR 37.910 [8]). Assume an "average" value. |
| Step 5: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | [2] | [5] | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total PHY-layer Latency (msec) | [35.3] | [803.5] | Latency needed from the time applies the location request up to the decoding of the PUSCH carrying the Positionign report. |

## UE-based DL-only Positioning, RRC Connected State, UE Internal-client

| Steps | “Best-case” Estimate of Duration (msec) in NR Rel-16 | “Conservative” Estimate of the Duration (msec) in NR Rel-16 | Short Explanation |
| --- | --- | --- | --- |
| Step 1: Request of Location Information has be received at the UE. UE transmits in PUSCH an MG-request through an RRC message | [1] | [4] | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 2: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 3: Serving gNB transmits the RRC configuration of the MG request | [1] | [2] | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 4: UE receives the message, interprets and applies the MG configuration | 10 | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 5: UE receives at least one occasion of the PRS within the MG. | 20 | 4*160 = 640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 6: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 | 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 7: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | [2] | [5] | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total PHY-layer Latency (msec) | 46 | 811 |  |

## UE-based DL-only Positioning, RRC Inactive State, UE internal-client

| Steps | “Best-case” Estimate of Duration (msec) in NR Rel-16 | “Conservative” Estimate of the Duration (msec) in NR Rel-16 | Short Explanation |
| --- | --- | --- | --- |
| Step 1: UE receives at least one occasion of the PRS | 6 | 4*160 = 640 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (N,8), with N<8. We assume that a reasonable PRS instance in NR Rel-16 FR1 is 6 msec. Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 2: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 | 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Total PHY-layer Latency (msec) | [8] | [780] | Latency needed from the time applies the location request up to the decoding of the PUSCH carrying the Positionign report. |

## UE-Assisted MRTT Positioning, RRC Connected State

| Steps | “Best-case” Estimate of Duration (msec) in NR Rel-16 | “Conservative” Estimate of the Duration (msec) in NR Rel-16 | Short Explanation |
| --- | --- | --- | --- |
| Step 1: Serving gNB sends PDSCH that contains the Location Information Request and is successfully decoded in physical layer by the UE | [1] | [2] | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 2: UE interprets and applies the LPP message (Location Request) | 10 | 10 | RRC Processing time are captured in 38.331 Section 12 |
| Step 3: Request of Location Information has be received at the UE. UE transmits in PUSCH an MG-request through an RRC message | [1] | [4] | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 4: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 5: Serving gNB transmits the RRC configuration of the MG request | [1] | [2] | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 6: UE receives the message, interprets and applies the MG configuration | 10 | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 7: UE receives at least one occasion of the PRS within the MG. | 20 | 4*160 = 640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 8: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 | 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 9: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | [2] | [5] | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Step 10: All gNBs processes the transmitted SRS in parallel. | 2 | 2 | Assumption: SRS configuration is received before Step 1, and SRS can be triggered with DCI. |
| Total PHY-layer Latency (msec) | [59] | [823] | Latency needed from the time applies the location request up to the decoding of the PUSCH carrying the Positionign report. |

## Summary of NR Rel-16 L1-layer Latency

Overall we have the following Table:

| NR Rel-16 Scenario | L1-layer Latency Expected Range (ms) |
| --- | --- |
| ## UE-Assisted DL-only Positioning, RRC Connected State | [57 - 823] |
| ## UE-based DL-only Positioning, RRC Inactive State, External Client | [35.3 - 803.5] |
| ## UE-based DL-only Positioning, RRC Connected State, UE Internal-client | [46 - 811] |
| ## UE-based DL-only Positioning, RRC Inactive State, UE internal-client | [8 - 780] |
| ## UE-Assisted MRTT Positioning, RRC Connected State | [59 - 823] |

- Phy-layer Enhancements – Latency Analysis

In the [2] paper we make the following proposals to enhance the PHY-layer Latency. In this section, we summarize the amount of savings that can be achieved in msec by these proposals, along with an explanation

| Proposal | L1 Latency | Potential Gains from the “best-case” estimate of NR rel-16 | Explanation |
| --- | --- | --- | --- |
| Support Low-layer (e.g., unicast/group-common DCI, MAC-CE) triggering of DL/UL PRS transmission/muting/Location-Request for DL-only and DL/UL methods. | - < 1 slot (for DCI-based) - 3 msec (for DL MAC-CE) | 10 msec | Instead of the UE receiving through a PDSCH carrying an RRC message the Location Information Request, a DCI-based triggering may be interpreted by the UE, depending on the scenario within the same slot that the PDCCH is received. A MAC-CE Request may be interested within 3 msec. |
| Support DCI/MAC-CE triggering of Measurement gaps (MG) for the purpose of positioning measurements | - < 1 slot (for DCI-based) - 3 msec (for DL MAC-CE) | At least to 10 + 20 = 30 msec | Instead of the gNB transmitting an RRC configuration that includes the MG request, a DCI-based triggering may be interpreted by the UE, depending on the scenario within the same slot that the PDCCH is received. A MAC-CE Request may be interested within 3 msec. The smallest MG periodicity currently is 20 msec, so a UE may have to wait, after it has interpreted the RRC message, an additional 20 msec before receiving the MG in NR Rel-16. If the MG is triggered by DCI or MAC-CE, the UE would not have to wait for a long period of time for the first occasion of the MG. |
| Fast/real-time processing of short PRS instances: - Support Enhanced PRS processing capabilities - Support partially-staggered or no-staggered DL-PRS transmissions | [1] slot | - At least 1.5 msec | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before it is ready to report. Furthermore, the maximum capability with respect to the number of PRS resources within a slot that is currently supported is 64 resources. If we want the UE to process, e.g. 18 TRPs, each one with at least 4 beams, with single-symbol comb-6 PRS, we would need UEs that can process 72 resources per slot. |
| Support Low-layer (e.g. UL MAC-CE or UCI) Measurement Reporting towards the serving gNB | - < 1 slot (for UCI) - 3 msec (form UL MAC-CE) | 2 msec | Instead of the UE reporting the measurements as part of RRC in a NAS container (2-5 msec), a UCI-based would require < 1 slot, whereas a MAC-CE alternative would require 3 msec. |

- Summary of Results of Accuracy Evaluations

In this document, we presented our evaluation results across a variety of scenarios including InF, UMI, InH, UMA, in FR1 and FR2. A total of 40 cases were simulated with the following results:

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, InF FR1 DH ISD20, 100MHz, Link Quality, DL TDOA | Convex UEs | 0.55m | 6.42m | 15.35m | 31.66m |
|  | (Optional) All UEs | 0.67m | 6.51m | 13.37m | 26.8m |
| Case 1, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.071m | 0.37m | 1.32m | 6.92m |
|  | (Optional) All UEs | 0.085m | 0.52m | 2.17m | 8.25m |

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 2, InF FR1 SH ISD50, 100MHz, Link Quality, DL TDOA | Convex UEs | 0.035m | 0.06m | 0.25m | 2.92m |
|  | (Optional) All UEs | 0.04m | 0.09m | 0.49m | 4.16m |
| Case 2, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.03m | 0.065 | 0.16m | 0.28m |
|  | (Optional) All UEs | 0.05m | 0.11m | 0.17m | 0.54m |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 0.96m | 2.18m | 4.48m | 12.66m |
|  | (Optional) All UEs | 1.67m | 5.39m | 12.11m | 30.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 1.33m | 3.56m | 5.32m | 13.1m |
|  | (Optional) All UEs | 1.89m | 3.99m | 7.67m | 17.62m |

| Vertical Positioning error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 2.75m | 6.79m | 10.12m | 20.6m |
|  | (Optional) All UEs | 4.6m | 8.91m | 17.21m | 38.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 2.35m | 6.78m | 12.5m | 18.44m |
|  | (Optional) All UEs | 3.07m | 7.34m | 13.92m | 22.98m |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Variable UE heights | Convex UEs | 0.03m | 0.05m | 0.1m | 0.22m |
|  | (Optional) All UEs | 0.04m | 0.09m | 0.17m | 0.47m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.03m | 0.05m | 0.08m | 0.12m |
|  | (Optional) All UEs | 0.046m | 0.08m | 0.16m | 0.36m |

| Vertical Positioning error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, , Unequal gNBs heights, Variable UE heights | Convex UEs | 0.183m | 0.33m | 0.75m | 1.89m |
|  | (Optional) All UEs | 0.162m | 0.37m | 0.78m | 2.63m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, Unequal gNBs heights, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.178m | 0.298m | 0.41m | 0.9m |
|  | (Optional) All UEs | 0.19m | 0.34m | 0.64m | 1.34m |

| Horizontal Positioning Error (all UEs) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0058 | 0.0077 | 0.011 | 0.019 |
|  | Strongest | 0.0074 | 0.010 | 0.017 | 0.027 |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.0061 | 0.0085 | 0.011 | 0.015 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0054 | 0.0082 | 0.013 | 0.025 |
|  | Strongest | 0.0084 | 0.0132 | 0.0287 | 9.40 |
| Case 8 InF-DH FR2 3d mRTT | Earliest | 0.0061 | 0.0088 | 0.013 | 0.020 |

| Horizontal Positioning Error (convex Hull) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0043 | 0.0059 | 0.0071 | 0.0087 |
|  | Strongest | 0.0057 | 0.0080 | 0.0104 | 0.0170 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0039 | 0.0054 | 0.0070 | 0.0096 |
|  | Strongest | 0.0058 | 0.0098 | 0.0138 | 1.6767 |

| Vertical (Across All UEs) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.037 | 0.048 | 0.061 | 0.084 |
| Case 8 InF-SH FR2 3d mRTT | Earliest | 0.010 | 0.015 | 0.023 | 0.041 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.4 | 0.6 |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 0.64 | 1.2 | 2 | 3.2 |
| Case 9, UMI, FR1, RTT, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.16 | 0.22 | 0.3 | 0.4 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 3.4 | 5 | 6.2 | 8 |
| Case 10, UMI, FR1,RTT, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 4 | 4.2 | 6.9 | 9.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 5.7 | 8.1 | 9.9 | 17.3 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, LQ Algorithm | (Optional) All UEs | 7.9 | 11.5 | 14.3 | 20.1 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 11, UMA, FR1, DL-TDOA, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Outdoor UEs | 0.31 | 0.5 | 0.8 | 1.5 |
| Case 11, UMA, FR1, RTT, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Indoor UEs | 1.7 | 6.9 | 0.8 | 96 |

| Horizontal Error (All UEs) |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.011 | 0.018 | 0.025 | 0.040 |
|  | Strongest | 0.13 | 0.80 | 2.63 | 6.32 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0058 | 0.0093 | 0.015 | 0.024 |
|  | Strongest | 0.0092 | 0.016 | 0.028 | 0.053 |

| Horizontal Error (convex Hull UEs) |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.0106 | 0.0178 | 0.0251 | 0.0403 |
|  | Strongest | 0.1333 | 0.7972 | 2.6315 | 6.3233 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0035 | 0.0053 | 0.0069 | 0.0113 |
|  | Strongest | 0.0066 | 0.0101 | 0.0160 | 0.0279 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 1 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.4 | 4.8 | 6 | 7.3 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.5 | 4.8 | 6 | 7.4 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.6 | 4.8 | 6 | 7.6 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.7 | 4.8 | 6 | 7.9 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.4 | 5.8 | 7.2 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.4 | 5.8 | 7.2 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.38 | 0.52 | 0.66 | 0.87 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.68 | 0.89 | 1.1 | 1.37 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.11 | 1.45 | 1.75 | 2.22 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.38 | 0.52 | 0.66 | 0.87 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.68 | 0.89 | 1.1 | 1.37 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.11 | 1.45 | 1.75 | 2.22 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.41 | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.46 | 0.73 | 0.76 | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.2 | 1.55 | 1.89 | 2.3 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.41 | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.46 | 0.73 | 0.76 | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.2 | 1.55 | 1.89 | 2.3 |

| Case ID | Beam Pair | Tx T1 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| Case 18 InF-SH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.01 |
|  |  | 0.1ns | 0.02 | 0.03 | 0.04 | 0.06 |
|  |  | 0.2ns | 0.05 | 0.07 | 0.09 | 0.13 |
|  |  | 0.5ns | 0.14 | 0.19 | 0.27 | 0.40 |
|  |  | 1.0ns | 0.28 | 0.40 | 0.55 | 0.96 |
|  |  | 2.0ns | 0.55 | 0.75 | 1.08 | 1.59 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.02 |
|  |  | 0.1ns | 0.03 | 0.05 | 0.07 | 0.11 |
|  |  | 0.2ns | 0.06 | 0.10 | 0.14 | 0.25 |
|  |  | 0.5ns | 0.19 | 0.29 | 0.47 | 0.94 |
|  |  | 1.0ns | 0.40 | 0.63 | 1.08 | 2.30 |
|  |  | 2.0ns | 0.82 | 1.22 | 1.87 | 5.19 |
| Case 19 InF-DH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.02 |
|  |  | 0.1ns | 0.03 | 0.04 | 0.06 | 0.10 |
|  |  | 0.2ns | 0.06 | 0.08 | 0.12 | 0.24 |
|  |  | 0.5ns | 0.16 | 0.24 | 0.40 | 0.76 |
|  |  | 1.0ns | 0.34 | 0.47 | 0.78 | 2.10 |
|  |  | 2.0ns | 0.74 | 1.17 | 2.15 | 6.31 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.05 | 43.94 |
|  |  | 0.1ns | 0.05 | 0.08 | 3.61 | 36.56 |
|  |  | 0.2ns | 0.10 | 0.21 | 7.91 | 35.28 |
|  |  | 0.5ns | 0.29 | 0.73 | 18.49 | 44.16 |
|  |  | 1.0ns | 0.74 | 1.96 | 18.90 | 40.33 |
|  |  | 2.0ns | 1.73 | 6.42 | 20.03 | 46.31 |
| Case 20 InH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.02 | 0.03 |
|  |  | 0.1ns | 0.05 | 0.08 | 0.13 | 0.31 |
|  |  | 0.2ns | 0.11 | 0.17 | 0.28 | 0.47 |
|  |  | 0.5ns | 0.23 | 0.40 | 0.60 | 1.16 |
|  |  | 1.0ns | 0.51 | 0.71 | 1.19 | 2.04 |
|  |  | 2.0ns | 0.97 | 1.49 | 2.23 | 4.11 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.03 | 0.21 |
|  |  | 0.1ns | 0.10 | 0.22 | 0.76 | 7.44 |
|  |  | 0.2ns | 0.22 | 0.48 | 1.94 | 8.22 |
|  |  | 0.5ns | 0.70 | 2.42 | 5.24 | 11.37 |
|  |  | 1.0ns | 1.26 | 3.45 | 6.88 | 14.94 |
|  |  | 2.0ns | 2.08 | 4.38 | 8.21 | 18.87 |
| Case 21 UMi, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.02 | 0.03 |
|  |  | 0.1ns | 0.04 | 0.05 | 0.08 | 0.13 |
|  |  | 0.2ns | 0.07 | 0.10 | 0.13 | 0.20 |
|  |  | 0.5ns | 0.15 | 0.20 | 0.26 | 0.38 |
|  |  | 1.0ns | 0.32 | 0.42 | 0.54 | 0.73 |
|  |  | 2.0ns | 0.63 | 0.82 | 1.02 | 1.38 |
|  | Strongest beam pair | 0.0ns | 0.47 | 3.08 | 7.73 | 16.21 |
|  |  | 0.1ns | 2.10 | 6.20 | 12.72 | 23.54 |
|  |  | 0.2ns | 1.20 | 4.23 | 9.16 | 16.59 |
|  |  | 0.5ns | 1.69 | 3.81 | 7.18 | 19.30 |
|  |  | 1.0ns | 2.08 | 4.00 | 7.50 | 14.27 |
|  |  | 2.0ns | 2.95 | 6.52 | 12.89 | 21.65 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 22, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 2 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 1.1 | 1.4 | 2.5 |
| Case 22, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.5 | 1.2 | 1.4 | 2.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 23, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.12 | 0.3 | 0.5 | 0.9 |
| Case 23, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.15 | 0.35 | 0.9 | 2 |
| Case 23, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.2 | 0.4 | 1.1 | 2.4 |
| Case 23, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 1 | 1.2 | 2.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 24, InH, FR1, Channel Spacing = 0 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.35 | 0.8 | 1.5 | 3 |
| Case 24, InH, FR1, Channel Spacing = 7.2 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 0.9 | 1.7 | 4 |
| Case 24, InH, FR1, Channel Spacing = 14.4 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.5 | 1.8 | 2.3 | 5 |
| Case 24, InH, FR1, Channel Spacing = 36 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.8 | 1.9 | 2.8 | 5.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 25, UMI, FR1,Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.6 m | 1 m |
| Case 25, UMI, FR1,Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.7 m | 1.2 m |
| Case 25, UMI, FR1,Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 m | 0.55 m | 0.7 m | 1.5 m |
| Case 25, UMI, FR1,Phase offset = 12 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.5 m | 0.7 m | 1.2 m | 2.2 m |
| Case 25, UMI, FR1,Phase offset = 18 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.6 m | 1 m | 1.8 m | 4 m |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 26, UMI, FR1,Time offset = 0 ns, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.6 m | 1 m |
| Case 26, UMI, FR1,Time offset = 0.5 ns, Frequency-domain Stitching | (Optional) All UEs | 0.38 m | 0.52 m | 0.7 m | 1 m |
| Case 26, UMI, FR1,Time offset = 1 ns, Frequency-domain Stitching | (Optional) All UEs | 0.38 m | 0.55 m | 0.7m | 1 m |
| Case 26, UMI, FR1,Time offset = 2 ns, Frequency-domain Stitching | (Optional) All UEs | 0.4 m | 0.58 m | 0.6 m | 1.5 m |
| Case 26, UMI, FR1,Time offset = 5 ns, Frequency-domain Stitching | (Optional) All UEs | 0.5 m | 0.8 m | 1 m | 1.5 m |

| Horizontal Positioning Error | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 0 | 0.117m | 4.9m | 13.2m | 24.4m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 45 | 0.108m | 5.3m | 13.3m | 25.6m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 90 | 0.126m | 6.09m | 13.8m | 25.5m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 0 | 0.067m | 4.47m | 14.26m | 25.7m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 45 | 0.09m | 3.79m | 13.1m | 25.46m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 90 | 1.04m | 7.02m | 15.1m | 28.9m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 0 | 0.04m | 0.07m | 0.17m | 3.4m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 45 | 0.04m | 0.07m | 0.17m | 2.9m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 90 | 0.04m | 0.08m | 0.22m | 3.4m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 0 | 0.02m | 0.06m | 0.14m | 4.25m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 45 | 0.03m | 0.06m | 0.21m | 4.26m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 90 | 0.03m | 0.06m | 0.83m | 4.43m |

| Case ID | Phase Shift θ | Time offset t | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| Case 28 InF-SH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.14 |
|  | 10 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.15 |
|  | 20 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.12 |
|  | 40 | 0.0ns | 0.01 | 0.02 | 0.08 | 1.37 |
|  | 60 | 0.0ns | 0.01 | 0.02 | 0.18 | 1.14 |
|  | 80 | 0.0ns | 0.01 | 0.02 | 0.07 | 0.96 |
| Case 29 InF-DH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 2.72 | 12.41 | 28.39 |
|  | 10 | 0.0ns | 0.01 | 2.82 | 12.43 | 29.53 |
|  | 20 | 0.0ns | 0.01 | 2.82 | 12.41 | 28.39 |
|  | 40 | 0.0ns | 0.01 | 2.88 | 12.42 | 28.33 |
|  | 60 | 0.0ns | 0.02 | 3.95 | 13.62 | 30.92 |
|  | 80 | 0.0ns | 0.02 | 3.65 | 13.60 | 29.54 |
| Case 30 UMi, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.05 |
|  | 10 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.05 |
|  | 20 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.04 |
|  | 40 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.25 |
|  | 60 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.11 |
|  | 80 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.07 |
| Case 31 InH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.04 | 0.08 | 0.17 | 0.42 |
|  | 10 | 0.0ns | 0.04 | 0.09 | 0.20 | 0.53 |
|  | 20 | 0.0ns | 0.04 | 0.09 | 0.20 | 0.56 |
|  | 40 | 0.0ns | 0.05 | 0.12 | 0.35 | 0.72 |
|  | 60 | 0.0ns | 0.06 | 0.21 | 0.52 | 0.94 |
|  | 80 | 0.0ns | 0.07 | 0.28 | 0.53 | 0.97 |
| Case 32 UMi, FR2, DL-TDOA, RANSAC | 0 | 0.0ns | 0.011 | 0.018 | 0.025 | 0.040 |
|  | 0 | 0.1ns | 0.011 | 0.018 | 0.026 | 0.043 |
|  | 0 | 0.5ns | 0.011 | 0.018 | 0.029 | 0.046 |
|  | 0 | 1.0ns | 0.012 | 0.022 | 0.032 | 0.052 |
|  | 0 | 5.0ns | 0.016 | 0.030 | 0.050 | 0.120 |
|  | 0 | 10.0ns | 0.021 | 0.038 | 0.066 | 0.220 |
|  | 0 | 20.0ns | 0.035 | 0.059 | 0.111 | 0.269 |
| Case 33 InH, FR2, DL-TDOA, RANSAC | 0 | 0.0ns | 0.006 | 0.009 | 0.015 | 0.024 |
|  | 0 | 0.1ns | 0.006 | 0.009 | 0.015 | 0.027 |
|  | 0 | 0.5ns | 0.006 | 0.009 | 0.014 | 0.027 |
|  | 0 | 1.0ns | 0.006 | 0.011 | 0.017 | 0.027 |
|  | 0 | 5.0ns | 0.008 | 0.013 | 0.028 | 0.066 |
|  | 0 | 10.0ns | 0.008 | 0.014 | 0.027 | 0.057 |
|  | 0 | 20.0ns | 0.009 | 0.015 | 0.031 | 0.070 |

| Case ID | Kinematic constraint condition | 50% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 34 InF-SH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.024 | 0.033 | 0.049 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.017 | 0.024 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.017 | 0.024 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.017 | 0.024 | 0.031 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.017 | 0.026 | 0.036 |
| Case 35 InF-DH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.030 | 0.038 | 0.058 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.014 | 0.021 | 0.029 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.016 | 0.023 | 0.034 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.018 | 0.028 | 0.038 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.018 | 0.029 | 0.044 |
| Case 36 InH, FR2 DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.031 | 0.046 | 0.071 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.019 | 0.026 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.013 | 0.019 | 0.028 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.022 | 0.031 | 0.044 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.023 | 0.035 | 0.053 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 37, UMI, FR1,RTT+AoA, With Delta Tau, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 2.6 | 4.4 | 5.8 | 7.3 |
| Case 37, UMI, FR1,RTT, With Delta Tau, Perfect Sync, No Timing Errors, AoA & UL PDP Enhancement | (Optional) All UEs | 1.2 | 1.8 | 2.7 | 3.9 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 38, UMI, FR1,DL-TDOA, With Δτ, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 4.1 | 5.2 | 6.8 | 9 |
| Case 38, UMI, FR1,DL-TDOA, With Δτ, Perfect Sync, No Timing Errors, UL PDP Enhancement | (Optional) All UEs | 3.6 | 4.85 | 6 | 8 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 39, UMI, FR1, RTT, With Delta Tau, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 3.38 | 4.9 | 6.2 | 9 |
| Case 39, UMI, FR1, RTT, With Delta Tau, Perfect Sync, No Timing Errors, UL PDP Enhancement | (Optional) All UEs | 1.6 | 2.4 | 3.4 | 4.7 |

| Horizontal Positioning Error | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 40, InF FR1 DH ISD20, 100MHz, LOS Genie + Link Quality | 0.36 | 4.08 | 10.28 | 35.65 |
| Case 40, InF FR1 DH ISD20, 100MHz, RANSAC | 0.085m | 0.52m | 2.17m | 8.25m |

- Summary of Results of L1-latency Analysis

- NR Rel-16 Expected L1-layer Latency in ms:

| NR Rel-16 Scenario | L1-layer Latency Expected Range (ms) |
| --- | --- |
| ## UE-Assisted DL-only Positioning, RRC Connected State | [57 - 823] |
| ## UE-based DL-only Positioning, RRC Inactive State, External Client | [35.3 - 803.5] |
| ## UE-based DL-only Positioning, RRC Connected State, UE Internal-client | [46 - 811] |
| ## UE-based DL-only Positioning, RRC Inactive State, UE internal-client | [8 - 780] |
| ## UE-Assisted MRTT Positioning, RRC Connected State | [59 - 823] |

- NR Rel-17 L1-Enhancements for L1-layer Latency reduction

| Proposal | L1 Latency | Potential Gains from the “best-case” estimate of NR rel-16 |
| --- | --- | --- |
| Support Low-layer (e.g., unicast/group-common DCI, MAC-CE) triggering of DL/UL PRS transmission/muting/Location-Request for DL-only and DL/UL methods. | - < 1 slot (for DCI-based) - 3 msec (for DL MAC-CE) | 10 msec |
| Support DCI/MAC-CE triggering of Measurement gaps (MG) for the purpose of positioning measurements | - < 1 slot (for DCI-based) - 3 msec (for DL MAC-CE) | At least to 30 msec |
| Fast/real-time processing of short PRS instances: - Support Enhanced PRS processing capabilities - Support partially-staggered or no-staggered DL-PRS transmissions | [1] slot | At least 1.5 msec |
| Support Low-layer (e.g. UL MAC-CE or UCI) Measurement Reporting towards the serving gNB | - < 1 slot (for UCI) - 3 msec (form UL MAC-CE) | 2 msec |

# References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 59.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”,V16.0.0 (59-10)