3GPP TSG RAN WG1 Meeting #103-e	R1-2009108

e-Meeting, October 26th – Novermber 13th 2020

Agenda item:	8.10.1

Source: 	Lenovo, Motorola Mobility

Title: 	Enhancements to resource multiplexing for IAB

Document for:	Discussion and Decision

# Introduction

RAN1 started working on a new WI on enhancements for IAB ‎[1]‎[2] in RAN1#102-e. The main focus of IAB specification in Rel-16 was on TDM between communications of an IAB node with its child and parent IAB nodes. An enhancement considered for IAB specification in Rel-17 is to facilitate FDM/SDM between upstream and downstream operations.

The following cases of simultaneous operations were identified in RAN1#102-e ‎[3]:

Agreement - Based on the WID, the following multiplexing cases are in scope for potential support in Rel-17: - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx - Further study for for Case A and Case B at least the following scenarios: - Single or multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands) - Further study for Case C and Case D at least for the following scenarios: - Multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands) - FFS: Required level of specification impact to support the different cases. Any additional specification support in Rel-17 should be conditioned on feasibility from an interference and reliability perspective on a per-link and network basis

The different cases are illustrated below ‎[3]:

. Simultaneous operations cases for enhanced IAB

Further agreements and notes from the previous meeting are mentioned and sicussed by topic in the next section.In this paper, we provide our views on specification of simultaneous operation for an IAB-node’s child and parent links. Then, in a companion paper ‎[4], we provide our views on specification of other features for multiplexing and duplexing enhanacements.

# Discussion

The following was agreed in RAN1#102-e [3]:

Agreement The Rel-16 semi-static and dynamic resource allocation mechanisms are the starting point for supporting Rel-17 multiplexing cases. - FFS: Applicability for different IAB-DU resource types - FFS: Cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT

We discuss the proposed mechasnisms further next.

## Semi-static resource allocation

In IAB Rel-16, resources in the time domain (slots/symbols) can be configured by the RRC as hard (H), soft (S), or not available (NA) for each IAB node. These semi-static configurations are applied only in the time domain, meaning that either all the PRBs in the BWP (except the ones that are allocated by other signals such as SS/PBCH blocks) are available (H), not available (NA), or subject to availability indication (AI) by DCI signalling (S).

A similar approach can be taken in the frequency domain – new configurations can be specified to determine availability in the time and frequency domains for an IAB node. Each configuration should include paramaters such as the periodicity and available PRBs in each period for the IAB node as shown in the example of ‎Figure 3 for the scenario depicted below.

. Example scenario

. Example of semi-static resource configuration (partitioning) in time and frequency domain

This approach essentially partitions the available resources into hard (H) and not available (NA) for each IAB node in a semi-static manner that is fully controlled by the donor IAB-CU. The approach can be further extended to allow soft resource configurations in the frequency domain for each IAB node, hence allowing availability indication in the frequency domain by the parent node IAB-DU as well.

Support semi-static resource configuration (partitioning) in time and frequency domains.

## Dynamic resource allocation

A frequency-domain AI can be specified to allow an IAB node to coordinate with its child node(s), locally and dynamically, what resources to use for scheduling. The frequency-domain AI can be used with a semi-static configuration as explained in Section 2.1, or it can be simply used for indication of availability of frequency-domain resources (such as PRBs) on soft symbols, which would make it backward compatible with IAB Rel-16 resource configurations. This latter approach could allow a convenient operation of IAB systems with mixed Rel-16 and Rel-17 IAB nodes – resource configurations remain backward compatible, and frequency-domain availability indication is only used locally when an IAB node and its child node both support enhanced duplexing.

Frequency-domain duplexing may also be enabled by proper configuration and activation of bandwidth parts so as to avoid resource overlaps among adjacent hops, fulfill guard band requirements, avoid cross-link interference, and so on.

In all the above schemes, the resolution for frequency-domain resources could be PRBs, or otherwise a number of consecutive PRBs in order to reduce signalling overhead.

Extend hard/soft resource configuration and availability indication to the frequeny domain.

## Implementation characteristics

Companies were encouraged in the previous meeting to further consider the following aspects ‎[3]:

For companies to further consider: Whether the following characteristics of the IAB node implementation will impact the operation of different resource multiplexing cases, including resource partitioning (i.e. identify whether there is a need for potential specification impact/enhancements compared to Rel-16 if the characteristic is or is not supported by an IAB node): - Baseband (mis)timing alignment between IAB-MT and IAB-DU - Separate vs. shared antenna panels/RF front-end for IAB-MT/IAB-DU - Separate vs. shared baseband for IAB-MT/IAB-DU - Transmitter/receiver implementation - Self-interference cancellation - Power control mechanisms

As agreed in the past meeting, Case A (simultaneous transmission) and Case B (simultaneous reception) are considered for both single-panel and multi-panel IAB nodes. For these two cases, multi-panel IAB nodes are expected to impose less stringent requirements on baseband timing alignment. If the goal is to avoid any overlap between a transmission on one panel and a reception on another panel, even at symbol boundaries, the matter can be handled by allocating guard symbols between DL and UL resources.

However, an IAB node with a single panel/RF frontend, shared between IAB-MT and IAB-DU, may require a perfect alignment between FDM’ed operations. Enhanced time alignment cases being discussed under agenda item 8.10.2 can be employed to satisfy the timing alignment constraint for simultaneous transmsisions (Case-6) or simultaneous receptions (Case-7). Nevertheless, such a requirement can be relaxed if multiple baseband units are available or the baseband unit is equipped with multiple IFFT/FFT windows. Whether the IAB node supports multiple baseband processing can be specified as an IAB node capability.

Knowledge of IAB node capability of timing alignment, multi-panel and baseband characteristics may be beneficial from system operation perspective.

Moreover, there are considerations for simultaneous operation of an IAB node when operating at FR2. Communications of an IAB node with a single antenna panel (connected to a single RF chain) are spatially limited to one beam at a time. That means if two operations are scheduled simultaneously, the signals are constrained to be transmitted or received through a same beam.

Communications of an IAB node with multiple antenna panels (each connected to a separate RF chain) are not limited to one beam, but there are still constraints for simultaneous operations. Consider the example scenario in ‎‎Figure 2 where N has two antenna panels as shown in ‎Figure 4. (PN and CN in this example may also have multiple panels, but they are not shown for the sake of brevity.)

. Simultaneous operation for a multi-panel IAB node

Suppose that the parent IAB node (PN) schedules a PDSCH to the IAB node (N). A TCI indication determines which of the two panels of N are selected by PN. Since the PDSCH takes only one of the panels of N, the node can schedule a PUSCH from the child IAB node (CN) simultaneously. For example, if PN indicates a beam from panel P1, N will have panel P2 for communication with CN.

If N knows in advance whether and which panel will be used for communication with PN, it can schedule a communication with CN in a timely manner. However, if PN schedules the PDSCH in the same time slot for example, N will not have sufficient time to decode the DCI and transmit a DCI of its own to CN.

Consider mechanisms to identify which IAB node panels are available for its scheduling to facilitate SDM operation between parent IAB node and child IAB node.

From the list fo considerations, power control mechanisms are discussed under agenda item 8.10.2. The matter of self-interference will be discussed next under resource partitioning.

## Resource partitioning aspects

Companies were encouraged in the previous meeting to further consider the following aspects ‎[3]:

For companies to further consider: Different resource partitioning scenarios for access and backhaul links, including their respective implication on interference, for different resource multiplexing cases. Examples include: - Whether a given case is only applicable for certain resource types: e.g. DL only, UL only, DL + UL - Whether a given case is only applicable for backhaul links or both access and backhaul links - Note: This should have no impact on legacy UE behavior

Companies were encouraged in the previous meeting to further consider resource partitioning scenarios. One of the main aspects is how to handle interference and whether that affects DL/UL resources and backhaul/access links.

We propose to consider both downlink and uplink resources to cover all the four duplexing cases at least for the backhaul link. The different types of interference, including cross-link interference (CLI) and self-interference (SI), among adjacent IAB nodes and within an IAB node, can be handled by configuring reference signals that other IAB nodes in the vicinity can measure. These reference signals are naturally CSI-RS for DL resources and SRS for UL resources. This approach works well at least for handling interference among IAB nodes connected to the same IAB donor, because information of these designated CSI-RS/SRS configurations can be provided to all the IAB nodes by the IAB-CU. The case of interference among IAB nodes connected to different IAB donors can be further studied.

Furthermore, CLI where either the aggressor or the victim is a UE can also be handled by existing CLI specifications. Therefore, it can be argued that resource partitioning for duplexing cases need not be discriminated based on resource type (DL/UL) or link type (access/backhaul).

Support both downlink and uplink resources for backhaul links and access links.

Support interference management (including CLI and SI) for the four duplexing cases, at least between IAB nodes connected to the same IAB donor.

## Dual-connectivity

The following was agreed and proposed for further consideration in the previous meeting ‎[3]:

Conclusion At least the inter-carrier DC scenario can be considered in Rel-17. Further discussion in RAN3/RAN Plenary may be necessary for the intra-carrier DC scenario. Agreement Reuse by IAB-MT of existing Inter-frequency DC is considered as a starting point to support concurrent BH links to two parents. - FFS: Reuse of multi-TRP transmission resource allocation features (if intra-freq DC scenario is supported for IAB) - FFS: Additional specification effort to support IAB For companies to further consider: The following categories of enhancements have been proposed to support DC scenarios (not an exhaustive list): - Inter-parent DU resource coordination mechanisms and signaling - Resource allocation/scheduling conflict resolution rules at the parent or child node - Per-link IAB-DU resource configurations at the parent node

When an IAB node N establishes connections with two parent IAB nodes PN1 and PN2, both parent IAB nodes can be informed of resource configurations for N according to IAB Rel-16. That is, both PN1 and PN2 can be informed of which resources for N IAB-DU are always available (H), not available (NA), or subject to availability indication (S). However, for example, if PN1 indicates soft resources available for N through DCI 2_5 signaling, PN2 does not have access to this information.

That raises several issues. One issue is that if PN2 intends to avoid interference with transmissions scheduled by N on soft resources, it does not have a means to realize whether the soft resources are already available to N. Another issue is during transition times when the link between N and PN1 is not available and PN2 should take control of resource availability for N and other child nodes without knowing which soft resources are already indicated available to N.

In order to address these issues, signalling can be introduced between parent nodes, or from an IAB node to its parent node, that informs the parent node of the current status of AI for soft resources of a dually-connected IAB node.

Define signaling for IAB nodes in the DC mode to inform parent IAB nodes of the status of availability of soft resources.

# Conclusions

In this contribution, we discussed resource multiplexing enhancements for simultaneous operations on child and parent links of an IAB node and made the following proposals:

Proposal 1:	Support semi-static resource configuration (partitioning) in time and frequency domains.

Proposal 2:	Extend hard/soft resource configuration and availability indication to the frequeny domain.

Proposal 3:	Knowledge of IAB node capability of timing alignment, multi-panel and baseband characteristics may be beneficial from system operation perspective.

Proposal 4:	Consider mechanisms to identify which IAB node panels are available for its scheduling to facilitate SDM operation between parent IAB node and child IAB node.

Proposal 5:	Support both downlink and uplink resources for backhaul links and access links.

Proposal 6:	Support interference management (including CLI and SI) for the four duplexing cases, at least between IAB nodes connected to the same IAB donor.

Proposal 7:	Define signaling for IAB nodes in the DC mode to inform parent IAB nodes of the status of availability of soft resources.

# References

- RP-193251, “New WID on Enhancements to Integrated Access and Backhaul,” Qualcomm, RAN#86, Sitges, Spain, December 2019.

- RP-201293, “New WID on Enhancements to Integrated Access and Backhaul,” Qualcomm, RAN#88e, Electronic Meeting, June-July 2020.

- RAN1#102-e Chairman’s Notes.

- R1-2009109, “Enhancements for simultaneous operation in IAB systems,” Lenovo, Motorola Mobility, RAN1#103-e, Electronic Meeting, October/November 2020.