3GPP TSG-RAN WG1 #103-e	R1- 2009267

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.7.1.2

Source: 	Qualcomm Incorporated

Title: 	TRS/CSI-RS for idle/inactive UE power saving

Document for:	Discussion/Decision

# Introduction

In RAN meeting #88e, the Rel-17 power saving enhancements WID has identified potential TRS/CSI-RS occasions as one of the  major areas for RAN1 work [1].

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] … - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required …

In RAN1 #102-e, discussions were conducted for potential TRS/CSI-RS occasions. Several agreements and conclusions were made as follows. In principle, network can configure idle/inactive UE to reuse the RS that was first configured to connected mode UEs. One main open problem is whether UE should blindly detect the presence of the RS. Another one is whether and how network indicates availability/lifetime of the RS.

Agreements: Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as: - AGC, time/frequency tracking - FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication Agreements: - New types/patterns of TRS/CSI-RS are not introduced specifically for idle/inactive mode UE. Agreements: The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs. - Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share). - Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted. - Note: Always-on TRS/CSI-RS transmission by gNodeB is not required - At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported - FFS for other RS types - FFS: Whether UE blind detection is required or not. Observation: It is up to gNB implementation whether or not to transmit a TRS/CSI-RS to idle/inactive UEs even when the TRS/CSI-RS is not needed by connected UEs (e.g., when there is a connected mode UE in a cell but the UE is no longer using the TRS/CSI-RS, or when there is no longer connected mode UE in a cell, etc.) Agreements: The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling - FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.) - FFS for other signalling candidates (e.g., pre-configuration, etc.) - FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.) Agreements: Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly). - Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

In this contribution, we will discuss remaining issues for TRS/CSI-RS for idle/inactive UE power saving enhancements.

# TRS/CSI-RS design

With the TRS/CSI-RS for idle/inactive UEs, additional power saving gain may come from the reduced number of UE wakeups outside the paging occasion (PO) and extended deep sleep duration. In idle/inactive mode, the main reason for the UE to wake up is to decode SSBs for time, frequency tracking loops, AGC update and RRM measurement. In addition, if the SSB is very close to the PO, the UE can wake up once to receive the SSB and continue to process the PO.

Because the TRS has a smaller number of REs per RB than that of the SSB, the TRS may not always have as many REs as the SSB for example within a bandwidth of 52 RBs. On the other hand, the TRS has a wider time span than the SSB. Then whether the TRS can outperform the SSB depends on whether more processing gain can be achieved from channel frequency diversity and finer (but narrower range) frequency error estimation due to the wider time span. In comparison to PDSCH, there is more flexibility for PDCCH to be more reliably decoded in presence of timing and frequency errors. Therefore, PDSCH benefits more from additional TRS/CSI-RS than PDCCH if TRS/CSI-RS can provide better tracking loop quality than SSBs.

Based on these discussions, we have following observations.

Observation 1: Power saving gain of additional TRS/CSI-RS depends on the number of SSBs needed for loop update in the specific channel condition or for the specific UE implementation and location of the TRS/CSI-RS relative to the PO.

Observation 2: Whether a TRS/CSI-RS can provide more processing gain than a SSB depends on channel frequency selectivity, bandwidth of the TRS/CSI-RS and frequency drifting rate.

Observation 3: TRS/CSI-RS is especially beneficial for decoding broadcast PDSCH.

With the observation that the UE benefits from the additional TRS/CSI-RS mainly for PDSCH decoding, then, the TRS/CSI-RS is not needed and even the SSB needs not to be processed very frequently if PDSCH is not transmitted. It is reasonable that the UE only needs to assume the TRS/CSI-RS is present for a PO when paging message is transmitted to the UE.

Proposal 1: UE assumes TRS/CSI-RS is transmitted only when page message is transmitted for the UE. In this sense, the TRS/CSI-RS can act as a paging early indication.

As discussed above, it is beneficial to have a TRS/CSI-RS close to or even overlap with the page message. If TRS/CSI-RS is overlapping with the PDSCH, UE can minimize RF on time for processing the signal/channel together. This has distinct power saving advantage as evident from the power model [2]:

- For non-RRM evaluation, for “PDCCH+PDSCH” concurrent in the same slot as “SSB or CSI-RS processing”, assume “PDCCH+PDSCH” power

- For non-RRM evaluation, for “PDSCH only” concurrent in the same slot as “SSB or CSI-RS processing”, assume “PDSCH only” power

Observation 4: From power saving perspective, it is preferred to have the TRS overlap with or very close to the PDSCH.

|  | TRS not overlapping with PDSCH |
| --- | --- |
|  | Desirably, TRS is located close to the paging PDSCH, TDM or FDM with PDSCH |

Because there are generally multiple POs in each paging cycle, it is necessary to have multiple TRS/CSI-RS occasions in each paging cycle to facilitate tracking loop update for UEs associated with different POs. Besides, even for the same PO, configuring multiple TRS/CSI-RSs can provide additional flexibility to network and UE implementation for potential extra power gain.

Proposal 2: The TRS/CSI-RS can be UE group based for idle/inactive UEs.

The TRS/CSI-RS is intended for tracking loop update for idle/inactive UEs. It is also possible to use this TRS/CSI-RS for RRM measurement. Then the number of wakeups may be further reduced, and additional power saving gain may be achieved. However, the condition that (SNR, mobility) TRS/CSI-RS can provide RRM measurement as accurate as SSBs should be further studied.

Proposal 3: If TRS/CSI-RS is adopted, TRS/CSI-RS can be optionally used for RRM measurement.

For NR, the paging PDCCH and paging messages are transmitted on all beams for SSBs transmitted in the cell. This is shown by the figure below. When additional TRS/CSI-RS are configured to the UE, beams of the TRS/CSI-RS should match with beams of SSBs so that time/frequency tracking and AGC information obtained by the TRS/CSI-RS reflect channel condition for paging PDCCH and PDSCH on the SSB beams. Therefore, the additional TRS/CSI-RS should be configured with QCL source same as one of the transmitted SSBs.

Proposal 4: A TRS/CSI-RS configured to the idle/inactive UE should be QCL’ed with a transmitted SSB of the serving cell. At least one RS is QCL’ed with each transmitted SSB of the serving cell.

One FFS from RAN1 #102-e is whether UE blind detection is required or not. Blind detection of the presence of the TRS/CSI-RS should be avoided to the UE due to the additional complexity and potential robustness issue.

Proposal 5: The idle/inactive mode UE is not required to blindly detect the presence of TRS/CSI-RS unless the TRS/CSI-RS is used as paging early indication

- FFS: if TRS/CSI-RS is used both for tracking loop update and paging early indication, the UE should be able to be configured with enough information to explicitly differentiate the two different use cases.

# TRS/CSI-RS configuration

If TRS/CSI-RS is adopted, network needs to signal information of the TRS/CSI-RS to idle/inactive UEs. There are two potential relationships between the connected mode UE that is originally configured with the TRS/CSI-RS and the idle/inactive mode UE that reuses the TRS/CSI-RS

- They are the same UE

- They are different UEs

For the same UE case, the idle/inactive UE has already received the TRS/CSI-RS configured when it is in connected mode. Then network only needs to tell the UE whether the TRS/CSI-RS is still available with potential update of the RS configuration e.g., for overhead reduction of the RS transmission. Then the RS configuration can be included in the RRCRelease message.

For different UE case, network should configure both the full set of RS parameters and indicate the availability of the RS to the idle/inactive UE. In theory, network can use the same full RS configuration mechanism to configure and terminate the RS. However, when network disables a RS, it only needs to send a small number of bits (e.g., one bit) to the UE. This would help network effectively reduce the signaling overhead especially when dynamic enabling/disabling can be more frequent than a cell specific RS.

Observation 5: RS configuration signaling overhead can be reduced if network indicate availability for the RS that has been configured to the idle/inactive mode UE.

It would be more proper to use a L1 signaling to indicate the availability of additional TRS/CSI-RS. For paging enhancements, UE sub-grouping and paging early indication (PEI) can also be achieved by L1 signaling in PDCCH. Design of these three techniques should be jointly performed.

Proposal 6: Paging early indication, UE sub-grouping and availability indication of additional TRS/CSI-RS should be jointly designed if they are adopted.

If PEI is based on the legacy paging DCI, the RS availability information needs to be carried by unused bits of the DCI format 1_0. The reserved bits field can be used to indicate whether a UE’s associated UE group is paged. In addition, if value of the Short Message Indicator is 01, the Short Message field (8bits) is reserved. Otherwise, the last 5 bits of the Short Message field are still unused. If the DCI only includes the short message, scheduling field of paging PDSCH can be used. However, if UE sub-grouping indication is supported by the PEI, the unused bits should be partitioned into two sets for sub-grouping indication and RS availability indication, respectively.

Observation 6: Unused bits of the paging PDCCH can be used to indicate availability of the TRS/CSI-RS. These include

- Reserved bits to indicate the paged UE groups in the PO

- Unused bits 4 to 8 in the Short Message field

- If Short Message Indicator is 01, the entire Short Message field

- If Short Message Indicator is 10, the scheduling field.

Unused bits are partitioned between the sub-grouping indication and RS availability indication functions.

If DCI format 2_6 is used as the PEI, it is naturally a UE group based signaling. In addition, DCI format 2_6 can be used to indicate SCell dormancy for groups of SCells configured to connected mode UEs. For Rel-17 idle/inactive, it is convenient to replace the SCell dormancy indication function with the availability indication of additional CSI-RS/TRS. Similar to SCell dormancy, CSI-RS/TRS groups can also be defined to manage the signaling overhead.

Observation 7: If DCI format 2_6 is used as paging early indication for idle/inactive mode UEs, it can carry indication of availability of the additional TRS/CSI-RS in a similar way to SCell dormancy indication for connected mode UEs.

If RS/sequence-based PEI is adopted, information that can be carried by RS/sequences configured for the same UE sub-group is very limited. In this case, the PEI may be able to only indicate the availability of all configured TRS/CSI-RSs, i.e., either all TRS/CSI-RSs are available to the idle/inactive UE or not available.

Observation 8: If paging early indication is based on RS/sequence, only one bit information is provided to indicate whether all configured TRS/CSI-RSs are available to the idle/inactive UE or not available.

Based on these the discussions, we propose the following for TRS/CSI-RS configuration for idle/inactive mode UEs.

Proposal 7: TRS/CSI-RS can be configured to the idle/inactive mode UE by the following mechanisms

- Full set of configuration parameters is provided in SIB

- Lifetime of RS is indicated by RRCRelease message when the same UE switches from connected mode to idle/inactive

- Availability of the RS is indicated by L1 signaling in the paging early indication.

# Conclusions

In this paper, we discussed TRS/CSI-RS design and signalling for Rel-17 connected mode power saving. The following observations and proposals are made.

Observation 1: Power saving gain of additional TRS/CSI-RS depends on the number of SSBs needed for loop update in the specific channel condition or for the specific UE implementation and location of the TRS/CSI-RS relative to the PO.

Observation 2: Whether a TRS/CSI-RS can provide more processing gain than a SSB depends on channel frequency selectivity, bandwidth of the TRS/CSI-RS and frequency drifting rate.

Observation 3: TRS/CSI-RS is especially beneficial for decoding broadcast PDSCH.

Observation 4: From power saving perspective, it is preferred to have the TRS overlap with or very close to the PDSCH.

Observation 5: RS configuration signaling overhead can be reduced if network indicate availability for the RS that has been configured to the idle/inactive mode UE.

Observation 6: Unused bits of the paging PDCCH can be used to indicate availability of the TRS/CSI-RS. These include

- Reserved bits to indicate the paged UE groups in the PO

- Unused bits 4 to 8 in the Short Message field

- If Short Message Indicator is 01, the entire Short Message field

- If Short Message Indicator is 10, the scheduling field.

Unused bits are partitioned between the sub-grouping indication and RS availability indication functions.

Observation 7: If DCI format 2_6 is used as paging early indication for idle/inactive mode UEs, it can carry indication of availability of the additional TRS/CSI-RS in a similar way to SCell dormancy indication for connected mode UEs.

Observation 8: If paging early indication is based on RS/sequence, only one bit information is provided to indicate whether all configured TRS/CSI-RSs are available to the idle/inactive UE or not available.

Proposal 1: UE assumes TRS/CSI-RS is transmitted only when page message is transmitted for the UE. In this sense, the TRS/CSI-RS can act as a paging early indication.

Proposal 2: The TRS/CSI-RS can be UE group based for idle/inactive UEs.

Proposal 3: If TRS/CSI-RS is adopted, TRS/CSI-RS can be optionally used for RRM measurement.

Proposal 4: A TRS/CSI-RS configured to the idle/inactive UE should be QCL’ed with a transmitted SSB of the serving cell. At least one RS is QCL’ed with each transmitted SSB of the serving cell.

Proposal 5: The idle/inactive mode UE is not required to blindly detect the presence of TRS/CSI-RS unless the TRS/CSI-RS is used as paging early indication

- FFS: if TRS/CSI-RS is used both for tracking loop update and paging early indication, the UE should be able to be configured with enough information to explicitly differentiate the two different use cases.

Proposal 6: Paging early indication, UE sub-grouping and availability indication of additional TRS/CSI-RS should be jointly designed if they are adopted.

Proposal 7: TRS/CSI-RS can be configured to the idle/inactive mode UE by the following mechanisms

- Full set of configuration parameters is provided in SIB

- Lifetime of RS is indicated by RRCRelease message when the same UE switches from connected mode to idle/inactive

- Availability of the RS is indicated by L1 signaling in the paging early indication.

# References

- RP-200938, “Revised WID UE Power Saving Enhancements for NR”, RAN #88e, MediaTek Inc.

- TR 38.840, “Study on User Equipment (UE) power saving in NR”.