3GPP TSG-RAN WG1 #103-e		R1-20xxxxx

e-Meeting, Oct 26- Nov 13, 2020

Source: 	Moderator (Ericsson)

Title:      	Summary of Email discussion [103-e-NR-MRDC-CA-01]

Agenda item:	7.2.10

Document for:	Discussion and Decision

# 1 Introduction

This document provides summary of email discussion [103-e-NR-MRDC-CA-01] on following issues discussed during preparation phase of RAN1#103-eMeeting

[103-e-NR-MRDC-CA-01] Email discussion/approval on Dormancy and Unaligned CA on the following until 10/29 with potential CRs by 11/5 – Ravi (Ericsson)

- Dormancy Topic 1: Corrections to 38.213 related to SCell dormancy indication in DCI format 2_6

- Dormancy Topic 3: Corrections to 38.213 related to Case 2 dormancy indication

- Dormancy Topic 4: Corrections to 38.213 related to switching time during dormant/non-dormant BWP transition

- Dormancy Topic 5: Corrections to 38.212

- Unaligned CA Issue 1: whether the single offset duration limitation is applied to per CG or across CGs in case of NR-DC

- Unaligned CA Issue 2: Determination of Sync/Async NR-DC in the context of unaligned frame boundary CA

- Unaligned CA Issue 3: update of spec. TS 38.214 section 5.2.1.5.1 on aperiodic CSI-RS timing when the triggering PDCCH and the CSI-RS have the same numerology, to align with the agreement.

# 2. Discussion

### 2.1 Dormancy Topic 1

Please provide your input to below questions Q1-Q3 on this topic, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to TP1 from R1-2007578?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 1, Q1) |
| --- | --- | --- |
| OPPO | No | The behavior seems be sufficient by text in [10, TS 38.133], which state no receiving and transmission during the switching delay. |
| ZTE | No | It seems this is more like something nice to have. We prefer not to have this TP in Rel-16. Without this enhancement, if the BWP switching delay is longer than the gap between DCI 2-6 and the start of DRX on, UE may finish the BWP switching after the start of the DRX on. However, this is not an issue at all from our perspective. |

#### Question 2

Q2. Is it OK to agree to the TP in Proposal 4 of R1-2007737?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 1, Q2) |
| --- | --- | --- |
| OPPO | Yes | We are OK for that, we also propose the change in a set of changes, which the major part is not agreed. But the correction is still acceptable. |
| ZTE | Yes | This change was firstly approved in previous meetings. However, seems that it was missing somehow. |

#### Question 3

Q3. Is it OK to agree to the draft CR in R1-2008113?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 1, Q3) |
| --- | --- | --- |
| OPPO | Yes | Can be agreed with Q2 |
| ZTE | Yes | The second change is the same as that in Question2. |

### 2.2 Dormancy Topic 3

Please provide your input to below questions Q1-Q3 on this topic, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to draft CR2 of R1-2007737?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 3, Q1) |
| --- | --- | --- |
| OPPO | Yes | Ok to clarify. |
| ZTE | Yes | We are supportive of this CR. Without this CR, the UE behavior for HARQ-ACK for case 2 dormancy indication is not complete. |

#### Question 2

Q2. Is it OK to agree to draft CR3 of R1-2007737?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 3, Q2) |
| --- | --- | --- |
| OPPO | No | The feedback should not be taken into priority procedure. No need to include. |
| ZTE | Yes | Currently, DCI format 1-1 is allowed to be configured with priority indicator field and dormancy indication at the same time. It is not clear whether the HARQ-ACK for DCI format 1-1 indicating SCell dormancy without scheduling PDSCH should be put in the first HARQ-ACK codebook or the second HARQ-ACK codebook. Basically, there are following three alternatives to address this issue. No matter which way is adopted, a conclusion/CR is needed. Alt.1. Based on the priority indicator; Alt.2. Low/High priority by default. Alt.3. UE is not expected to be configured with priority indicator for the SCell dormancy indication. Our preference is Alt.1. Thus, we are supportive of this CR. |

#### Question 3

Q3. Is it OK to agree to draft CR in R1-2008566?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 3, Q3) |
| --- | --- | --- |
| OPPO | NO | The change is not needed. Base station should not indicate the SCell dormancy if it not configure the dormant BWP at all. |
| ZTE |  | We don’t have a strong view on this. We believe that network won’t transmit SCell dormancy indication if no SCell is configured with dormant BWP. From this perspective, it seems this CR is not needed. But if companies want to put it in the specification, we won’t object to do so. |

### 2.5 Dormancy Topic 4

Please provide your input to below questions Q1 and Q2 on this topic, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to the TP in section 2 of R1-2008145?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 4, Q1) |
| --- | --- | --- |
| OPPO | No | For power saving purpose, cancel the last transmission would be beneficial to UE. But, no need to clarify, could left for implementation. |
| ZTE | No | The same issue happens in Rel-15. For example, if DCI based BWP switching happens, Rel-15 doesn’t specify anything related to the UL cancellation. From this perspective, it seems not necessary to specify UL cancellation for Rel-16 dormant BWP change. |

#### Question 2

Q2. Is it OK to agree to the TP corresponding to Proposal 1 in R1-2008203?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 4, Q2) |
| --- | --- | --- |
| OPPO | NO | Usually, the delay defined in 133 is clear. |
| ZTE |  | We don’t have a strong view for this. Either way is fine. |

### 2.5 Dormancy Topic 5

Please provide your input to question Q1 below, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to draft CR1 of R1-2007737?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Dormancy Topic 5, Q1) |
| --- | --- | --- |
| OPPO | No | The first and last can means the configured order. |
| ZTE | Yes | The issue arises when discontinuous group index for SCell dormancy is configured by network. Basically, two approaches can be considered. Approach#1: Specify in RAN2 that only continuous group index for SCell dormancy is allowed. This may need LS exchange between RAN1 and RAN2. Approach#2: The SCell dormancy indication field corresponds to the ascending order of the group index. This is a pure RAN1 solution and is in line with what we did for BWP indication (as shown below). - Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where - if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id; - otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;. We prefer approach#2 and thus we are supportive of this CR. |

### 2.6 Unaligned CA Issue 1

Please provide your input to question Q1 below, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to below proposal (based on FL proposal 1 in [15])?

- Clarify the below RAN1#99 agreement that at most single non-zero offset duration can be configured within a CG in case of DC in the unaligned CA configuration.

Agreements:

At most single non-zero offset duration (independent on SCS) can be configured among CCs in the unaligned CA configuration.

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Unaligned CA Issue 1, Q1) |
| --- | --- | --- |
| ZTE | Yes | For Asynchronous NR-DC, the baseband and RF processing are almost independent in each CG. Thus, it makes sense to apply one single slot offset per CG. For Synchronous NR-DC, all Rel-16 UEs are required to support non-SFN-sync NR-DC. From our perspective, non-SFN-sync NR-DC is similar to have different slot offset between MCG and SCG. Thus, it seems UE is possible to support one slot offset per CG. |

### 2.6 Unaligned CA Issue 2

Please provide your input to question Q1 below, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to below proposal (based on FL proposal 2 in [15])?

- Synchronous or asynchronous NR-DC is determined according the PCell and PSCell.

- LS can be sent to RAN4 to confirm this.

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Unaligned CA Issue 2, Q1) |
| --- | --- | --- |
| ZTE | Yes | We are supportive of the above proposal. Since the synchronous and asynchronous NR-DC requirements are specified before the introduction of unaligned frame boundary, it is preferred to determine synchronous or asynchronous NR-DC before applying the slot offset in MCG and SCG. From our perspective, determining synchronous or asynchronous NR-DC before applying the slot offset in MCG and SCG is the same as determining according to the PCell and PSCell. Thus, we support the above proposal. To make it clear, maybe we can fine-tune the proposal as below. - If unaligned frame boundary CA is configured in either CG, Synchronous or asynchronous NR-DC is determined according the PCell and PSCell. - LS can be sent to RAN4 to confirm this. |

### 2.6 Unaligned CA Issue 3

Please provide your input to question Q1 below, preferably by 10/27 (11:59PM UTC).

#### Question 1

Q1. Is it OK to agree to the TP corresponding to Proposal 1 of R1-2008504?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Unaligned CA Issue 3, Q1) |
| --- | --- | --- |
| ZTE | No | It seems this TP is not necessary. Based on the following description from TS38.331, it has been specified that aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 is the slot offset between he slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. Based on this, it seems the current spec is clear. aperiodicTriggeringOffset, aperiodicTriggeringOffset-r16 Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. For aperiodicTriggeringOffset, the value 0 corresponds to 0 slots, value 1 corresponds to 1 slot, value 2 corresponds to 2 slots, value 3 corresponds to 3 slots, value 4 corresponds to 4 slots, value 5 corresponds to 16 slots, value 6 corresponds to 24 slots. For aperiodicTriggeringOffset-r16, the value indicates the number of slots. The network configures only one of the fields. When neither field is included, the UE applies the value 0. |

# 3 Conclusions

TBU

# 4 References

- R1-2007578	Remaining issues on CA	Huawei, HiSilicon

- R1-2007737	Remaining Issues of SCell Dormancy, Single Tx and Unaligned Frame Boundary ZTE

- R1-2008113	Correction to dormancy indication outside Active Time	NEC

- R1-2008145	Remaining issues for Scell dormancy	Samsung

- R1-2008203	Remaining issues on dormancy	Nokia, Nokia Shanghai Bell

- R1-2008275	BWP switching delay issues and Scell dormancy		OPPO

- R1-2008504	Remaining issues on Rel-16 carrier aggregation	MediaTek Inc.

- R1-2008566	Draft CR to 38.213 on corrections for SCell dormancy	Ericsson

- R1-2008613	Remaining issue on SCell dormancy	Qualcomm Incorporated

- R1-2007806	Remaining issues on inter-band CA with unaligned frame boundary	CATT

- R1-2008504	Remaining issues on Rel-16 carrier aggregation	MediaTek Inc.

- R1-2008567	Maintenance for other MR-DC topics	Ericsson

- R1-2008680	Remaining issues on MR-DC and CA enhancements	vivo

- R1-20xxxxx	Summary of maintenance issues for SCell Dormancy	Moderator (Ericsson)

- R1-20xxxxx	FL summary on support of unaligned frame boundary for R16 NR inter-band	Moderator (CMCC)