3GPP TSG RAN WG1 #103-e	R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Agenda item:	7.2.10

Source:	Moderator (Nokia)

Title:	Summary of Email discussion [103-e-NR-MRDC-CA-02]

Document for:	Discussion and Decision

Table of Contents

1	Introduction	1

2	UL Power Control for Dual Connectivity	2

2.1	PC-DC Issue 2	2

2.2	PC-DC Issue 3	2

2.3	PC-DC Issue 4	3

3	Cross carrier scheduling	4

3.1	XCC A-1 (R1-2007736)	4

3.2	XCC A-2 (R1-2007807)	4

3.3	XCC A-3 (R1-2007807)	5

3.4	XCC A-4 (R1-2008504)	5

4	1-Tx EN-DC enhancements	5

5	Conclusions	7

6	References	7

# 1	Introduction

This document facilitates and summarizes the AI 7.2.10	Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements email thread #2:

[103-e-NR-MRDC-CA-02] Email discussion/approval on PC for DC, X-CC scheduling and 1-Tx enhancements until 10/29 with potential CRs by 11/5 – Karri (Nokia)

- PC-DC Issue 2: (R1-2007736 and CR1 for power control) Clarification on the timing relation for Dynamic Power Sharing

- PC-DC Issue 3: (R1-2008503, P1 and TP) On FDD scenario for Semi-Static-mode2 power control

- PC-DC Issue 4: (R1-2008694, TP) Alignment between UE capability description on TS38.306 and UE’s behaviour on TS38.213.

- XCC A-1 (R1-2007736): Discuss if there is a need to add the PDSCH starting time to determine the last DCI in order to be able to indicate different PRIs in the same PUCCH slot for the two HARQ-ACKs in the scenario described in R1-2007736.

- XCC A-2 (R1-2007807): The clarification on the RRC parameter applicability between URLLC priority based codebook and secondary PUCCH group codebook would seem to benefit from the suggested clarification. Discuss the TP1 and TP2 to TS38.213 sections 7.2 and 9 respectively.

- XCC A-3 (R1-2007807): Interoperability of the simultaneous configuration of pdsch-HARQ-ACK-CodebookList-r16 and pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 would seem to be in a need of clarification. Discuss how to resolve the interoperability issue.

- XCC A-4 (R1-2008504): Discuss the need for introducing the additional delay ‘d’ for timeDurationForQCL in case of CCS when enableDefaultBeamForCCS is not configured as proposed in section 3 of R1-2008504 to 38.214 subclause 5.1.5

- 1TX C-1 (R1-2007737): There appears to be a discrepancy between the UE capabilities and the TS38.213 for the TDD and FDD PCell semi-static UL transmission in all subframes and a correction is needed. Discuss section 2.2 and CR4 in R1-2007737.

# 2	UL Power Control for Dual Connectivity

Three issues related to the UL power control for dual connectivity are in the scope of this email thread. The Email tread summary erroneously listed CR2 of ‘7736 as the text proposal in scope, but it should be obvious that this was a mistake as CR1 is the text proposal relevant to UL PC for DC.

- PC-DC Issue 2: (R1-2007736 and CR1 for power control) Clarification on the timing relation for Dynamic Power Sharing

- PC-DC Issue 3: (R1-2008503, P1 and TP) On FDD scenario for Semi-Static-mode2 power control

- PC-DC Issue 4: (R1-2008694, TP) Alignment between UE capability description on TS38.306 and UE’s behaviour on TS38.213.

### 2.1	PC-DC Issue 2

PC-DC Issue 2: (R1-2007736 and CR1 for power control) Clarification on the timing relation for Dynamic Power Sharing

Moderator proposal: Agree to CR1 in R1-2007736 to TS38.213 subclause 7.6.2

Company comments

| Company | Yes or No | Comments |
| --- | --- | --- |

### 2.2	PC-DC Issue 3

PC-DC Issue 3: (R1-2008503, P1 and TP) On FDD scenario for Semi-Static-mode2 power control

- Proposal 1 [R1-2008503]: To define the UE behaviour on FDD scenario for semi-static-mode2 power control in NR-DC, adopt the following TP to 38.213 Section 7.6.2:

If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2 - if the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG or SCG, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using or as the maximum transmission power, respectively. - if at least one symbol of slot of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot as described in Clauses 7.1 through 7.5 using or , respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot , as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering or , respectively

Moderator proposal: Agree to the TP of proposal 1 in R1-2008503 to TS38.213 subclause 7.6.2

Company comments

| Company | Yes or No | Comments |
| --- | --- | --- |

### 2.3	PC-DC Issue 4

PC-DC Issue 4: (R1-2008694, TP) Alignment between UE capability description on TS38.306 and UE’s behaviour on TS38.213.

[R1-2008694] To avoid misalignmed between UE capability description on TS38.306 and UE behaviour desctiption on TS38.213, we suggest text proposal for TS38.213 as below:

7.6.2 NR-DC <text omitted> If a UE - is provided dynamic for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, and - indicates a capability to support dynamic power sharing for intra-FR NR DC, - indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, or are configured by higher layers, and - overlap with the transmission occasion on the SCG the UE determines a maximum transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, or are configured by higher layers, and - overlap with the transmission occasion on the SCG The maximum transmission power on the SCG is determined as UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as - minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power - PTotalNR-DC, if the UE does not determine any transmissions on the MCG <text omitted>

Moderator proposal: Agree to the TP of R1-2008964 to TS38.213 subclause 7.6.2

Company comments

| Company | Yes or No | Comments |
| --- | --- | --- |

# 3	Cross carrier scheduling

Four issues related to cross carrier scheduling are in the scope of this email thread.

- XCC A-1 (R1-2007736): Discuss if there is a need to add the PDSCH starting time to determine the last DCI in order to be able to indicate different PRIs in the same PUCCH slot for the two HARQ-ACKs in the scenario described in R1-2007736.

- XCC A-2 (R1-2007807): The clarification on the RRC parameter applicability between URLLC priority based codebook and secondary PUCCH group codebook would seem to benefit from the suggested clarification. Discuss the TP1 and TP2 to TS38.213 sections 7.2 and 9 respectively.

- XCC A-3 (R1-2007807): Interoperability of the simultaneous configuration of pdsch-HARQ-ACK-CodebookList-r16 and pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 would seem to be in a need of clarification. Discuss how to resolve the interoperability issue.

- XCC A-4 (R1-2008504): Discuss the need for introducing the additional delay ‘d’ for timeDurationForQCL in case of CCS when enableDefaultBeamForCCS is not configured as proposed in section 3 of R1-2008504 to 38.214 subclause 5.1.5

### 3.1	XCC A-1 (R1-2007736)

XCC A-1 (R1-2007736): Discuss if there is a need to add the PDSCH starting time to determine the last DCI in order to be able to indicate different PRIs in the same PUCCH slot for the two HARQ-ACKs in the scenario described in R1-2007736.

“…if the DSCH starting time in addition to the existing MO index and Cell index is not applied to determine the “last DCI”, then network has to indicate the same PRI in these different DCIs in this MO. This kind of restriction on network implementation has no justification. Thus, it is preferred that the PDSCH starting time in addition to the existing MO index and Cell index can also be applied to determine the “last DCI”.” [R1-2007736]

Moderator thought:  The motivation for the gNB to need to be able to indicate different PRIs in this case is unclear.

Moderator proposal: Do NOT agree to the CR2 in R1-2007736

Company comments

| Company | Yes or No | Comments |
| --- | --- | --- |

### 3.2	XCC A-2 (R1-2007807)

XCC A-2 (R1-2007807): The clarification on the RRC parameter applicability between URLLC priority based codebook and secondary PUCCH group codebook would seem to benefit from the suggested clarification. Discuss the TP1 and TP2 to TS38.213 sections 7.2 and 9 respectively.

Moderator proposal: Agree to the TPs to TS 38.213 subclauses 7.2 and 9 in R1-2007807

Company comments

| Company | Yes or No | Comments |
| --- | --- | --- |

### 3.3	XCC A-3 (R1-2007807)

XCC A-3 (R1-2007807): Interoperability of the simultaneous configuration of pdsch-HARQ-ACK-CodebookList-r16 and pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 would seem to be in a need of clarification. Discuss how to resolve the interoperability issue.

Proposal 2 [R1-2007807]: “Discuss whether to introduce a new RRC parameter to separately configure the HARQ-ACK codebook type for the two HARQ-ACK codebooks for the secondary PUCCH group. If the new RRC parameter is not introduced, clarify that pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 should not be configured or should be ignored if pdsch-HARQ-ACK-CodebookList-r16 is configured.”

Moderator thought: The absolute necessity for the new RRC parameter is not clear, and the ASN.1 has been frozen since June/2020, hence the introduction of a new RRC parameter is not recommended.

Moderator proposal: Do NOT introduce a new RRC parameter. Agree that pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 should be ignored if pdsch-HARQ-ACK-CodebookList-r16 is configured.

Company comments

| Company | Yes or No | Comments |
| --- | --- | --- |

### 3.4	XCC A-4 (R1-2008504)

XCC A-4 (R1-2008504): Discuss the need for introducing the additional delay ‘d’ for timeDurationForQCL in case of CCS when enableDefaultBeamForCCS is not configured as proposed in section 3 of R1-2008504 to 38.214 subclause 5.1.5

Moderator thought: ASUSTeK made a compelling argument that the change is not needed as as base station would always configure enableDefaultBeamForCCS-r16 for cross carrier scheduling with different numerologies. That is, the proposed change is non-essential.

Moderator proposal: Do NOT agree to the TP in section 3 of the R1-2008504 to TS 38.214 subclause 5.1.5

Company comments

| Company | Yes or No | Comments |
| --- | --- | --- |

# 4	1-Tx EN-DC enhancements

One issue related to single Tx EN-DC enhancements is in the scope of this email thread.

- 1TX C-1 (R1-2007737): There appears to be a discrepancy between the UE capabilities and the TS38.213 for the TDD and FDD PCell semi-static UL transmission in all subframes and a correction is needed. Discuss section 2.2 and CR4 in R1-2007737.

Moderator proposal: Agree to the TP of CR4 in R1-2007737 to TS38.213 subclause 5.1 and 8

Company comments

| Company | Yes or No | Comments |
| --- | --- | --- |

# 5	Conclusions

To be written

# 6	References

- R1-2009219 Moderator summary of the 4 feature lead summaries of 7.2.10 in preparation of RAN1#103-e, Moderator (Nokia)

- R1-2008897 FL summary on support of unaligned frame boundary for R16 NR inter-band CA, Moderator (CMCC)

- R1-2009210 Summary of maintenance issues for SCell Dormancy, Moderator (Ericsson)

- R1-2009218 Feature Lead summary on singe Tx enhancements, and cross carrier scheduling and A-CSI RS triggering, Moderator (Nokia)

- R1-2009231 Feature lead summary #1 on UL Power Control for NN-DC, Moderator (Apple Inc.)