3GPP TSG RAN WG1 Meeting #103-e		R1-200xxxx

# E-meeting, October 26th – November 13th, 2020

Title:	Draft LS reply on cell-grouping UE capability for synchronous NR-DC

Response to:

Release:	Release 16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	Qualcomm [RAN1]

To:	RAN2

Cc:	RAN4

Contact person:	Fred Takeda

ktakeda (at) qti (dot) qualcomm (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

Regarding the question on the need of cell-grouping capability for synchronous NR-DC, RAN1 had no time to discuss it. However, RAN1 has discussed the need of PUCCH-grouping capability for NR-CA with two PUCCH groups and agreed to support the following new Rel.16 capabilities.

Alt.1: - Whether to support two PUCCH groups is reported per BC for NR-CA with 3 or more DL bands with at least two from {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2} - For the BC, the UE reports one or multiple of supported configuration(s) of {primary PUCCH group config, secondary PUCCH group config} where for each supported configuration, - the “primary PUCCH group config” includes following information: - One or multiple Band(s) mapped to the primary PUCCH group - One or multiple Band(s) that can be configured with the PUCCH transmission in the primary PUCCH group - the “secondary PUCCH group config” includes following information: - One or multiple Band(s) mapped to the secondary PUCCH group - One or multiple Band(s) that can be configured with the PUCCH transmission in the secondary PUCCH group - Note: for each {primary PUCCH group config, secondary PUCCH group config}, each band (i.e. FDD/TDD/SUL/SDL band as defined in RAN4) is mapped to either the primary PUCCH group config or the secondary PUCCH group config - For NR-CA with 2 bands with one or two from {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2}, or with 3 or more bands with one of {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2}: - The Rel.15 capabilities are used. Alt.2: - Whether to support two PUCCH groups is reported per BC for NR-CA with 3 or more DL bands with at least two from {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2} - For the BC, the UE reports one or multiple of supported configuration(s) of {primary PUCCH group config, secondary PUCCH group config} where for each supported configuration, - the “primary PUCCH group config” includes following information: - One or multiple from {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2} mapped to the primary PUCCH group - One or multiple from {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission in the primary PUCCH group - the “secondary PUCCH group config” includes following information: - One or multiple from {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2} mapped to the secondary PUCCH group - One or multiple from {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission in the secondary PUCCH group - Note: for each {primary PUCCH group config, secondary PUCCH group config}, each of {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2} is mapped to either or both of the primary PUCCH group config orand the secondary PUCCH group config. - Note: RAN1 will discuss on how to handle the SDL or SUL band - SDL overlapping with either TDD or FDD can follow the same principle with TDD or FDD accordingly - SDL having no overlapped TDD or FDD can follow the same principle with FDD - Note: When the NUL is indicated for PUCCH transmission location, corresponding SUL can also be used for PUCCH transmission - For NR-CA with 2 bands with one or two from {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2}, or with 3 or more bands with one of {FR1 licensed TDD, [FR1 unlicensed TDD], FR1 licensed FDD, FR2}: - The Rel.15 capabilities are used.

# 2	Actions

To 3GPP RAN2

ACTION: To take into account the RAN1’s feedback above for their specification work of cell-grouping capability for synchronous NR-DC.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #104-e	25 January – 5 February 2021, Electronic