3GPP TSG RAN WG1 #103-e			R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Title:	[draft] Reply LS to RAN2 on beamSwitchTiming

Response to:	R2-2008318

Release:	Rel-16

Work Item:	TEI16

Source:	vivo [RAN1]

To:	RAN2

Cc:

- Contact Person:

- Name:	                Rakesh Tamrakar

E-mail Address:	 rakesh@vivo.com

- 1. Overall Description:

RAN1 thanks RAN2 for the LS on beamSwicthTiming.

On Rel-15 UE capability of beamSwitchTiming,

- For Rel-15 UEs,

- if the UE indicates a value from the set {14, 28, 48} in Rel-15 capability signaling, the reported value is used for all cases, including CSI-RS with repetition is configured and set to “ON” or “OFF”.

- if the UE indicates a value from the set {224, 336} in Rel-15 capability signaling, there is no specified beahviour for value of 224 and 336.

- If the Rel-15 capability signaling (beamSwitchTiming) is not included, the beam switch timing is up to 48 OFDM symbols for each supported sub-carrier spacing.

Please find RAN1 understanding on above 3 bullet points as below:

  - In R15, if the UE indicates one value among {14, 28, 48} for beamSwitchTiming, the UE behavior for receiving AP-CSI-RS configured with repetition (set as ‘ON’ or ‘OFF’) with a scheduling offset smaller than reported beamSwitchTiming was unspecified, while the UE behavior for receiving AP-CSI-RS with a scheduling offset equal to or greater than reported beamSwitchTiming was specified.

  - [In R15, when UE reports one value among {224, 336} for beamSwitchTiming, it will be used to determine UE expectation/behavior for aperiodic CSI-RS for tracking and latency requirements for L1-RSRP reporting, while UE behavior for receiving AP-CSI-RS for CSI acquisition (without trs-Info and without repetition) with a scheduling offset smaller than, equal to or larger than reported beamSwitchTiming was unspecified.]

Option1: Agree on the text as above

Option2: Agree on following text: “UE behavior for receiving AP-CSI-RS for CSI acquisition (without trs-Info and without repetition) with a scheduling offset smaller than, equal to or larger than reported beamSwitchTiming was unspecified.”

Option3: There is no consensus in RAN1 on UE behavior when it indicates a values from {224, 336} in Rel-15 capability signalling beamSwitchTiming.

| Company | Comments |
| --- | --- |

  - In R15, RAN1 did not agree on using 48 as default value for beamSwitchTiming.

For Rel-16 UEs,

- For Rel-16 UEs, the UE signals beamSwitchTiming and beamSwitchTiming-r16. The first one, i.e., the Rel-15 capability signaling from the set {14, 28, 48}, is always signaled (if this capability needs to be reported). The second one, i.e., the newly introduced Rel-16 capability signaling from the set {224, 336}, is optional.

- If the UE doesn’t include the Rel-16 capability, it behaves in all respects as the Rel-15 UEs above.

- If the UE does include the Rel-16 capability, then, the UE may or may not receive configured parameter enableBeamSwitchTiming-r16 in the RRC configuration:

  - If the UE doesn’t receive parameter enableBeamSwitchTiming-r16, it behaves in all respect as the Rel-15 UE above.

  - If the UE receives parameter enableBeamSwitchTiming-r16, then, the UE does the following:

    - Question A: For CSI-RS configured with repetition and set to “ON”, whether the UE indicates switch time that is the same as the signalled value from the set {224, 336} in Rel-16 capability signaling or the default value of 48?

    - Question B: For CSI-RS configured with repetition and set to “OFF” (a.k.a. regular CSI-RS), whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling?

    - Question C: For CSI-RS configured without repetition, whether the UE indicates switch time of 48 (even if it signaled < 48 in the Rel-15 capability), or the signalled value in Rel-15 capability signaling?

  - Both beamSwitchTiming and beamSwitchTiming-r16 are optional to report

  - If the UE doesn’t include the beamSwitchTiming-r16, its behaviour is same as Rel-15

  - If the UE does include the Rel-16 capability, UE may or may not receive enableBeamSwitchTiming-r16

  - If the UE does include the Rel-16 capability but doesn’t receive enableBeamSwitchTiming-r16, its behaviour is same as Rel-15

Below please find RAN1’s Answers to Questions A, B and C.

Answer A: For CSI-RS configured with repetition “ON”, the UE applies switch time that is the same as the signalled value from the set {224, 336}

Answer B: For CSI-RS configured with repetition “OFF”, the UE applies switch time of 48 if beamSwitchTiming-r16 is reported.

Answer C: For CSI-RS configured without repetition and without trs-info, the UE applies switch time of 48 if beamSwitchTiming-r16 is reported.

- 2. Actions:

- To RAN WG2

- RAN1 respectfully requests RAN2 to take above answers into account.

- 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #104-e		25 Jan – 5 Feb 2021				e-Meeting

TSG RAN WG1 Meeting #104-bis-e		12 – 20 Apr 2021					e-Meeting