3GPP TSG RAN WG1 Meeting #103-e	R1- 200xxxx

e-Meeting, 26th October – 13th November 2020

Title:	Summary of NR UE Power Saving

Agenda item:	7.2.7

Source:	CATT

Document for:	Discussion

# Final Summary of Email Discussions and Agreements

# Email Discussion [102e-NR_UE_Pow_Sav_01]

# Email Discussion during Preparation[102e-Prep_NR_UE_Pow_Sav]

| Company | Supporting Issues | Comments |
| --- | --- | --- |
| Nokia | #1,#2,#3,#5 | Issue #1, #2 and issue #3 appear relatively straight forward, albeit I would expect some wording discussion. I think issue #4 could be covered under Editors CR, if not already. Issue#5, relates also to the RRC parameter name aligment, but adding a source parameter reference would in my view require agreement from RAN1. I also took the liberty of adding the draft TP from the Annex of R1-2008732 at the end to Section 5 as a reference. |
| Ericsson | #1,#2,#3,#4,#5 | Regarding issue#4, we understand it is simply to help the editor as per chairman’s guidance. - For all Rel-16 “editorial” CRs/updates, we will use the editors’ alignment CRs as a start to collect and incorporate these updates. There will be dedicated editors’ alignment CR email threads. - Please especially FLs help the editors collect any essential editorial-like CRs (per spec) |

# Summary from contributions reviews

## Summary of Open Issues

- Issue 1: Interference measurements are not part of L1-RSRP measurements when drx-OnDurationTimer does not start [5] in TS38.214

------------------------------------------------ Start of Text Proposal 1 ------------------------------------------------

#### 5.2.2.5 CSI reference resource definition

<Unchanged parts are omitted>

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

<Unchanged parts are omitted>

------------------------------------------------- End of Text Proposal 1 ------------------------------------------------

- Issue 2:  Clarification on minimumSchedulingOffsetK0-r16 is not configured for UL BWP.in Clause 5.2.1.5.1a of TS 38.214[5]

------------------------------------------------ Start of Text Proposal 2 ------------------------------------------------

##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

<Unchanged parts are omitted>

Aperiodic CSI-RS timing:

-	When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16, including the case that the UE is not configured with minimumSchedulingOffsetK0-r16 for any DL BWP or minimumSchedulingOffsetK2-r16 for any UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, …, 31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16,

-	μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and  μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5

<Unchanged parts are omitted>

------------------------------------------------- End of Text Proposal 2 ------------------------------------------------

- Issue 3: Align the Clause 10.3 of TS38.213 related to the minimum time gap with the corresponding minimum time gap capability parameter described in RAN2 specifications [6]

## 10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

<omitted unchanged text>

If a UE reports for an active DL BWP a MinTimeGap-r16 value requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, where X corresponds to the reported MinTimeGap-r16 value for the requirement of the SCS of the active DL BWP in Table 10.3-1.

Table 10.3-1 Minimum time gap value X

| SCS (kHz) | Minimum Time Gap X (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

<omitted unchanged text>

- Issue 4:  RRC parameter alignments [6]

38.213 subclause 10.3

- sizeDCI_2-6  → sizeDCI-2-6

- psPositionDCI-2-6  → ps-PositionDCI-2-6

- drx-onDuarationTimer → drx-onDurationTimer

- Issue 5:   Clarify the source of the parameter drx-onDurationTimer [8] in TS38.213 and TS38.214.

  - Note:

    - In 38.213 the parameter drx-onDurationTimer is referred in context of DCP/DCI format 2_6 triggered behaviour or monitoring, and DCP/DCI format 2_6 cannot be configured together with DRX-ConfigSecondaryGroup thus source for the parameter drx-onDurationTimer should be clear from the context.

    - In 38.214 the parameter drx-onDurationTimer is referred in context of DCP/DCI format 2_6 triggered start of timer but also used to define absolute time duration, thus it could be considered to clarify the source of the parameter.

# Contributions summary and proposals

| CATT [1] | - Observation 1: The number of aggregation levels for DCI format 2_6 should be restricted to reduce the number of PDCCH blind decoding and the additional power saving gain. - Observation2: DCI size alignment will degrade miss detection performance of DCI format 2_6 more than 2dB in AWGN channel for 12bits DCI size. |
| --- | --- |
| ZTE [2] | - Proposal 1: Adopt the following TP on TS 38.213. ----------------------------------------------- Start of TP of TS 38.213 -------------------------------------------------------- 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells *** Unchanged text is omitted *** If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time prior to a next long DRX cycle the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle. *** Unchanged text is omitted *** ----------------------------------------------- End of TP of TS 38.213 --------------------------------------------------------. <Moderator comment> The proposed change has same meaning as that in the specification. This is not an essential correction or editorial change. |
| Samsung [3] | Proposed TP1 for TS 38.213 in Section 10.3 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells […] The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321]. The UE does not monitor PDCCH for detecting DCI format 2_6 while short DRX cycle is used [11, TS 38.321]. […]. Proposed TP2 for TS 38.213 in Section 10.3 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells […] - an offset by ps-Offset indicating a time, where the UE starts monitoring PDCCH for detection of DCI format 2_6 according to the number of search space sets, prior to a slot where the drx-onDuarationTimer for long DRX cycle would start on the PCell or on the SpCell [11, TS 38.321] - for each search space set, the PDCCH monitoring occasions are the ones in the first Ts slots indicated by duration, or Ts=1 slot if duration is not provided, starting from the first slot of the first Ts slots and ending prior to the start of drx-onDurationTimer for long DRX cycle. On PDCCH monitoring occasions associated with a same long DRX Cycle, a UE does not expect to detect more than one DCI format 2_6 with different values of the Wake-up indication bit for the UE or with different values of the bitmap for the UE. The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321]. If a UE reports for an active DL BWP a requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer for long DRX cycle, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the requirement of the SCS of the active DL BWP in Table 10.3-1. […]. <Moderator comments> This issue was discussed in email discussion in issue 5-5 of RAN1#102-e [102-e_NR_NR_UE_Pow_Sav_02] email discussion. The conclusion was no consensus in additional specification change to capture “for long DRX cycle”. |
| Huawei, HiSilicon [4] | - Proposal 1: Make a conclusion in RAN1 that UE may use N Rx antennas for the reception of PDSCH on a DL active BWP when the per-BWP configured maxMIMO-Layers for the DL BWP is N. - Proposal 2: Suggest Editor to change ‘minimum scheduling offset restriction’ to ‘minimum scheduling offset restriction’ in TS 38.214. <Moderator’s comments> These are not essential correction. |
| MediaTeck[5] | - Proposal 1: Adopt TP1 in Section 5.2.2.5 of TS 38.214 to clarify interference measurement is unnecessary if the UE is configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP during the time duration indicated by drx-onDurationTimer outside DRX active time. ------------------------------------------------ Start of Text Proposal 1 ------------------------------------------------ #### 5.2.2.5 CSI reference resource definition <Unchanged parts are omitted> When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. <Unchanged parts are omitted> ------------------------------------------------- End of Text Proposal 1 ------------------------------------------------ - Proposal 2: Adopt TP2 in Section 5.2.1.5.1a of TS 38.214 to clarify that minimumSchedulingOffsetK0-r16 is not configured for UL BWP. ------------------------------------------------ Start of Text Proposal 2 ------------------------------------------------ ##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies <Unchanged parts are omitted> Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16, including the case that the UE is not configured with minimumSchedulingOffsetK0-r16 for any DL BWP or minimumSchedulingOffsetK2-r16 for any UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, …, 31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where - n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16, - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively, - Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5 <Unchanged parts are omitted> ------------------------------------------------- End of Text Proposal 2 ------------------------------------------------ |
| Ericsson [6] | Following TP is proposed to align the RAN1 specification related to the minimum time gap with the corresponding minimum time gap capability parameter described in RAN2 specifications. TP for 38.213-g30, subclause 10.3 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells <omitted unchanged text> If a UE reports for an active DL BWP a MinTimeGap-r16 value requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, where X corresponds to the reported MinTimeGap-r16 value for the requirement of the SCS of the active DL BWP in Table 10.3-1. Table 10.3-1 Minimum time gap value X SCS (kHz); Minimum Time Gap X (slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. <omitted unchanged text> # 2.2 RRC parameter alignment Following RRC parameter name alignment/corrections are needed in some places. These are mentioned here for editors’ convenience. 38.213 subclause 10.3 - sizeDCI_2-6 → sizeDCI-2-6 - psPositionDCI-2-6 → ps-PositionDCI-2-6 - drx-onDuarationTimer → drx-onDurationTimer 38.214 subclauses 5.1.6.1 and 5.2.2.5 - ps-TransmitOtherPeriodicCSI → ps-TransmitPeriodicCSI <Moderator’s comments> ps-TransmitOtherPeriodicCSI is in the DCI-config-R16 in PhysicalCellGroupConfig IE |
| vivo[7] | Proposal 1: Further clarification is needed for minimum time gap. Capture TP in Appendix 1 in R1-2008677 for TS38.213 ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ==============================Unchanged part omitted================================ If a UE reports for an active DL BWP a requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the requirement of the SCS of the active DL BWP smallest SCS among all SCS values of all configured CCs in Table 10.3-1. Table 10.3-1 Minimum time gap value X SCS (kHz); Minimum Time Gap X (slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24. ==============================Unchanged part omitted================================ <Moderator comments> minimum time gap value X is associated with DCP in PCell in CA or SpCell in DC. It is not count any associated SCell SCS in CA. |
| Nokia, NSB [8] | - Proposal: In light of parameter name duplication, discuss whether there is a need to clarify the source of the parameter drx-onDurationTimer. - Observation 1: In 38.213 the parameter drx-onDurationTimer is referred in context of DCP/DCI format 2_6 triggered behaviour or monitoring, and DCP/DCI format 2_6 cannot be configured together with DRX-ConfigSecondaryGroup thus source for the parameter drx-onDurationTimer should be clear from the context. - Observation 2: In 38.214 the parameter drx-onDurationTimer is referred in context of DCP/DCI format 2_6 triggered start of timer but also used to define absolute time duration, thus it could be considered to clarify the source of the parameter. Draft TP to 38.214: #### 5.1.6.1 CSI-RS reception procedure [unneccesary text omitted] If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer in DRX-Config also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported. [unneccesary text omitted] 5.1.6.1.3 CSI-RS for mobility [unneccesary text omitted] If the UE is configured with DRX, the UE is not required to perform measurement of CSI-RS resources other than during the active time for measurements based on CSI-RS-Resource-Mobility. When the UE is configured to monitor DCI format 2_6, the UE is not required to perform measurements other than during the active time and during the timer duration indicated by drx-onDurationTimer in DRX-Config also outside active time based on CSI-RS-Resource-Mobility. If the UE is configured with DRX and DRX cycle in use is larger than 80 ms, the UE may not expect CSI-RS resources are available other than during the active time for measurements based on CSI-RS-Resource-Mobility. If the UE is configured with DRX and configured to monitor DCI format 2_6 and DRX cycle in use is larger than 80ms, the UE may not expect that the CSI-RS resources are available other than during the active time and during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time for measurements based on CSI-RS-Resource-Mobility. Otherwise, the UE may assume CSI-RS are available for measurements based on CSI-RS-Resource-Mobility. [unneccesary text omitted] #### 5.2.2.5 CSI reference resource definition [unneccesary text omitted] When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.. |

# Reference

- R1-2007821		Remaining issues on UE Power Saving		CATT

- R1-2007970		Remaining issues of Rel-16 power saving		ZTE

- R1-2008143		Remaining issues for Rel-16 UE power saving		Samsung

- R1-2008331		Remaining issues for Rel-16 UE power saving		Huawei, HiSilicon

- R1-2008509		Remaining issues on UE power saving		MediaTek Inc.

- R1-2008565		Maintenance for UE power saving		Ericsson

- R1-2008677		Remaining issues on UE power saving		vivo

- R1-2008732		On open issues related to Rel-16 UE power saving	Nokia, Nokia Shanghai Bell