3GPP TR 38.857 V0.2.1 (2020-11)

Technical Report

3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on NR Positioning Enhancements; (Release 17)

| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |
| --- | --- |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	11

2	References	11

3	Definitions of terms, symbols and abbreviations	12

3.1	Terms	12

3.2	Symbols	12

3.3	Abbreviations	12

4	General description of NR positioning	12

5	Target requirements for NR positioning enhancements in Rel-17	13

5.1 	Target requirements	13

5.2 	Performance evaluation metrics	13

5.2.1	Horizontal accuracy	13

5.2.2	Vertical accuracy	13

5.2.3	 Other metrics	13

5.2.3.1	Latency	13

5.2.3.1.1	Physical layer Latency	13

5.2.3.1.2	Higher layer Latency	14

5.2.3.2	Network efficiency	14

5.2.3.3	Device efficiency	14

6 	Additional scenarios and channel models for NR positioning enhancements	14

6.1 	IIoT use cases	18

6.2 	General commercial use cases	21

7	Studied NR positioning enhancements	21

8	Performance evaluations for Rel-17 targets	22

8.1	Performance analysis of Rel-16 positioning solutions	22

8.1.1	Positioning accuracy analysis	22

8.1.1.1	Observations  from source [4]	22

8.1.1.2	Observations  from source [7]	26

8.1.1.3	Observations  from source [8]	27

8.1.1.4	Observations  from source [13]	30

8.1.1.5 Observations  from source [5]	31

8.1.1.6	Observations  from source [12]	39

8.1.1.7	Observations  from source [10]	40

8.1.1.8 Observations  from source [14]	41

8.1.1.9	Observations  from source [20]	42

8.1.1.10	Observations  from source [17]	43

8.1.1.11	Observations  from source [18]	50

8.1.2	Physical layer latency analysis for Rel-16	51

8.1.2.1	Observations  from source [4]	51

8.1.2.2	Observations  from source [7]	51

8.1.2.3	Observations  from source [8]	51

8.1.2.4	Observations  from source [13]	51

8.1.2.5 Observations  from source [11]	51

8.1.2.6	Observations  from source [5]	52

8.1.2.7	Observations  from source [12]	53

8.1.2.8	Observations  from source [10]	54

8.1.2.9 Observations  from source [16]	54

8.1.2.10 Observations  from source [17]	54

8.1.2.11 Observations  from source [15]	54

8.2	Performance analysis of studied NR positioning enhancements	55

8.2.1	Positioning accuracy analysis for NR positioning enhancements	55

8.2.1.1	Observations  from source [4]	55

8.2.1.2	Observations  from source [7]	58

8.2.1.3	Observations  from source [8]	58

8.2.1.4	Observations  from source [13]	59

8.2.1.5	Observations  from source [5]	59

8.2.1.6	Observations  from source [12]	73

8.2.1.7	Observations  from source [10]	73

8.2.1.8	Observations  from source [18]	75

8.2.1.9 Observations  from source [14]	78

8.2.1.10	Observations  from source [21]	79

8.2.1.11	Results from [17]	81

8.2.1.12	Results from [9]	84

8.2.2	Physical layer latency analysis for NR positioning enhancements	84

8.2.2.1	Observations  from source [4]	84

8.2.2.2	Observations  from source [8]	85

8.2.2.3	Observations  from source [5]	85

8.2.2.4	Observations  from source [10]	85

8.2.2.5 Observations  from source [16]	86

8.2.2.6	Observations from source [17]	86

8.2.2.7 Observations  from source [15]	87

8.3	Efficiency analysis for NR positioning enhancements	87

8.3.1	Network efficiency analysis for NR positioning enhancements	87

8.3.1.1	Observations  from source [4]	87

8.3.1.2	Observations  from source [8]	88

8.3.1.3	Observations  from source [5]	88

8.3.1.4	Observations  from source [12]	88

8.3.1.5	Observations  from source [21]	88

8.3.1.6 Observations  from source [18]	88

8.3.2	UE efficiency analysis for NR positioning enhancements	88

8.3.2.1	Observations  from source [4]	88

8.3.2.2	Observations  from source [5]	89

8.4	Summary of performance evaluations	92

9	Positioning integrity and reliability	98

10	Identified NR impacts in Rel-17	98

10.1 	NR positioning for UEs in RRC_INACTIVE state	98

10.2 On-demand transmission and reception of DL PRS	98

10.3 Aggregation of DL PRS resources	99

10.4 Aggregation of SRS for positioning resources	99

10.5 Enhancements for  UE Rx/Tx and gNB Rx/Tx timing delays	99

10.6 Enhancements for angle based methods	99

10.8 Aperiodic reception of DL PRS	99

10.9 Semi-persistent reception of DL PRS	100

11	Conclusions	100

Annex A: Change history	101

Annex B: Appendix to summary of performance evaluations	102

B.1 Evaluation of  horizontal positioning accuracy with gNB/UE TX/RX timing error	102

B.2 Evaluation of  physical layer latency	107

Annex C:	Appendix to Performance evaluations for R17 performance targets	121

C.1	Performance analysis of Rel-16 positioning solutions	121

C.1.1	Positioning accuracy analysis	121

C.1.1.1	Results from source [4]	121

C.1.1.1.1	Description of evaluation scenarios	121

C.1.1.1.2	Positioning accuracy evaluation results	121

C.1.1.2	Results from source [7]	121

C.1.1.2.1	Description of evaluation scenarios	121

C.1.1.2.2	Positioning accuracy evaluation results	123

C.1.1.3	 Results from source [8]	141

C.1.1.3.1	Description of evaluation scenarios	141

C.1.1.3.2	Positioning accuracy evaluation results	163

C.1.1.4	Results from source [13]	166

C.1.1.4.1	Description of evaluation scenarios	166

C.1.1.4.2	Positioning accuracy evaluation results	169

C.1.1.5 Results from source [5]	169

C.1.1.5.1	Description of evaluation scenarios	169

C.1.1.5.2	Positioning accuracy evaluation results	193

C.1.1.6	Results from source [12]	198

C.1.1.6.1	Description of evaluation scenarios	198

C.1.1.6.2	Positioning accuracy evaluation results	199

C.1.1.7	Results from source [10]	200

C.1.1.7.1	Description of evaluation scenarios	200

C.1.1.7.2	Positioning accuracy evaluation results	203

C.1.1.8 Results from source [14]	206

C.1.1.8.1 Description of evaluation scenarios	206

C.1.1.8.2 Positioning accuracy evaluation results	207

C.1.1.8.3 Observations on Rel-16 NR positioning accuracy	209

C.1.1.9	Results from source [20]	210

C.1.1.9.1	Description of evaluation scenarios	210

C.1.1.9.2	Positioning accuracy evaluation results	219

C.1.1.9.3	Observations on Rel-16 NR positioning accuracy	230

C.1.1.10	Results from source [17]	231

C.1.1.10.1	Description of evaluation scenarios	231

C.1.1.10.2	Positioning accuracy evaluation results	245

C.1.1.10.3	Observations on Rel-16 NR positioning accuracy	253

C.1.1.11	Results from source [18]	261

C.1.1.11.1	Description of evaluation scenarios	261

C.1.1.11.2	Positioning accuracy evaluation results	263

C.1.2	Physical layer latency analysis for Rel-16	264

C.1.2.1	Results from source [4]	264

C.1.2.1.1	Description of evaluation scenarios	264

C.1.2.1.2	Latency analysis of NR positioning enhancements	264

C.1.2.2	Results from source [7]	269

C.1.2.2.1	Description of evaluation scenarios	269

C.1.2.2.2	Latency analysis for Rel.16 solutions	270

C.1.2.3	Results from source [8]	276

C.1.2.3.1	Description of evaluation scenarios	276

C.1.2.3.2	Latency analysis for Rel.16 solutions	277

C.1.2.4	Results from source [13]	280

C.1.2.4.1	Description of evaluation scenarios	280

C.1.2.4.2	Latency analysis for Rel.16 solutions	281

C.1.2.5 Results from source [11]	283

C.1.2.5.1 Description of evaluation scenarios	283

C.1.2.5.2 Latency analysis for Rel.16 solutions	284

C.1.2.6	Results from source [5]	290

C.1.2.6.1	Description of evaluation scenarios	290

C.1.2.6.2	Latency analysis for Rel.16 solutions	290

C.1.2.7	Results from source [12]	301

C.1.2.7.1	Description of evaluation scenarios	301

C.1.2.7.2	Latency analysis for Rel.16 solutions	301

C.1.2.8	Results from source [10]	304

C.1.2.8.1	Description of evaluation scenarios	304

C.1.2.8.2	Latency analysis for Rel.16 solutions	305

C.1.2.9 Results from source [16]	313

C.1.2.9.1 Description of evaluation scenarios	313

C.1.2.9.2 Latency analysis for R.16 solution	314

C.1.2.10 Results from source [17]	317

C.1.2.10.1	Description of evaluation scenarios	317

C.1.2.10.2	Latency analysis for Rel.16 solutions	317

C.1.2.11 Results from source [15]	323

C.1.2.11.1	Description of evaluation scenarios	323

C.1.2.11.2	Latency analysis for Rel.16 solutions	324

C.2	Performance of studied NR positioning enhancements	334

C.2.1	Positioning accuracy analysis for NR positioning enhancements	334

C.2.1.1	Results from source [4]	334

C.2.1.1.1	Description of evaluation scenarios	334

C.2.1.1.2	Positioning accuracy evaluation results for NR positioning enhancements	339

C.2.1.2	Results from source [7]	347

C.2.1.2.1	Description of evaluation scenarios	347

C.2.1.2.2	Positioning accuracy evaluation results for NR positioning enhancements	348

C.2.1.3	Results from source [8]	350

C.2.1.3.1	Description of evaluation scenarios	350

C.2.1.3.2	Positioning accuracy evaluation results for NR positioning enhancements	351

C.2.1.4	Results from source [13]	352

C.2.1.4.1	Description of evaluation scenarios	352

C.2.1.4.2	Positioning accuracy evaluation results for NR positioning enhancements	352

C.2.1.5	Results from source [5]	352

C.2.1.5.1	Description of evaluation scenarios	352

C.2.1.5.2	Positioning accuracy evaluation results for NR positioning enhancements	405

C.2.1.6	Results from source [12]	413

C.2.1.6.1	Description of evaluation scenarios	413

C.2.1.6.2	Positioning accuracy evaluation results for NR positioning enhancements	415

C.2.1.7	Results from source [10]	415

C.2.1.7.1	Description of evaluation scenarios	415

C.2.1.7.2	Positioning accuracy evaluation results for NR positioning enhancements	421

C.2.1.8	Results from source [18]	425

C.2.1.8.1	Description of evaluation scenarios	425

C.1.1.8.2	Positioning accuracy evaluation results	432

C.2.1.9 Results from source [14]	435

C.2.1.9.1	Description of evaluation scenarios	435

C.2.1.9.2 Positioning accuracy evaluation results for NR positioning enhancements	439

C.2.1.10	Results from source [21]	443

C.2.1.10.1	Description of evaluation scenarios	443

C.2.1.10.2	Positioning accuracy evaluation results for NR positioning enhancements	466

C.2.1.11	Results from [17]	478

C.2.1.11.1	Description of evaluation scenarios	478

C.2.1.11.2	Positioning accuracy evaluation results for NR positioning enhancements	491

C.2.1.12	Results from [9]	496

C.2.1.12.1	Description of evaluation scenarios	496

C.2.1.12.2	Positioning accuracy evaluation results for NR positioning enhancements	497

C.2.2	Physical layer latency analysis for NR positioning enhancements	503

C.2.2.1	Results from source [4]	503

C.2.2.1.1	Description of evaluation scenarios	503

C.2.2.1.2	Latency analysis of NR positioning enhancements	503

C.2.2.2	Results from source [8]	504

C.2.2.2.1	Description of evaluation scenarios	504

C.2.2.2.2	Latency analysis of NR positioning enhancements	504

C.2.2.3	Results from source [5]	506

C.2.2.3.1	Description of evaluation scenarios	506

C.2.2.3.2	Latency analysis of NR positioning enhancements	506

C.2.2.4	Results from source [10]	512

C.2.2.4.1	Description of evaluation scenarios	512

C.2.2.4.2	Latency analysis of NR positioning enhancements	512

C.2.2.5 Results from source [16]	515

C.2.2.5.1 Description of evaluation scenarios	515

C.2.2.5.2 Latency analysis of NR positioning enhancements	515

C.2.2.6	Results from [17]	517

C.2.2.6.1	Description of evaluation scenarios	517

C.2.2.6.2	Latency analysis of NR positioning enhancements	517

C.2.2.7 Results from source [15]	517

C.2.2.7.1	Description of evaluation scenarios	517

C.2.2.7.2	Latency analysis of NR positioning enhancements	518

C.3	Efficiency analysis for NR positioning enhancements	525

C.3.1	Network efficiency analysis for NR positioning enhancements	525

C.3.1.1	Results from source [4]	525

C.3.1.1.1	Description of evaluation scenarios	525

C.3.1.1.2	Network efficiency analysis of NR positioning enhancements	526

C.3.1.2	Results from source [8]	526

C.3.1.2.1	Description of evaluation scenarios	526

C.3.1.2.2	Network efficiency analysis of NR positioning enhancements	527

C.3.1.3	Results from source [5]	529

C.3.1.3.1	Description of evaluation scenarios	529

C.3.1.3.2	Network efficiency analysis of NR positioning enhancements	529

C.3.1.4	Results from source [12]	531

C.3.1.4.1	Description of evaluation scenarios	531

C.3.1.5	Results from source [21]	532

C.3.1.5.1	Description of evaluation scenarios	532

C.3.1.5.2	Network efficiency analysis of NR positioning enhancements	532

C.3.1.6 Results from source [18]	532

C.3.1.6.1	Description of evaluation scenarios	532

C.3.1.6.2	Network efficiency analysis of NR positioning enhancements	532

C.3.2	UE efficiency analysis for NR positioning enhancements	534

C.3.2.1	Results from source [4]	534

C.3.2.1.1	Description of evaluation scenarios	534

C.3.2.1.2	UE efficiency analysis of NR positioning enhancements	537

C.3.2.2	Results from source [5]	538

C.3.2.2.1	Description of evaluation scenarios	538

C.3.2.2.2	UE efficiency analysis of NR positioning enhancements	541

1 - Baseline power consumption evaluation results	541

A - Power consumption for PRS measurement in idle state (baseline)	541

B - Power consumption for PRS measurement in connected state (baseline)	541

2 - Power consumption evaluation for different PRS configurations	542

A - PRS measurement impacted by DRX	542

B -Extending PRS period	543

C - Concentrated PRS distribution	544

D - Adding PRS measurement window (PRS-MTC)	545

E - Reducing number of TRPs to be measured	546

F - Reducing number of positioning frequency layers to be measured	547

3 - Power consumption evaluation for idle states positioning	548

A - Idle state measurement and report	548

B - Idle state measurement and connected state report	550

C- Connected state measurement and report	552

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document captures the findings of the study item "Study on NR positioning enhancements" [2]. The purpose of this technical report is to document the requirements, additional scenarios, evaluations and technical proposals treated during the study and provide a way forward toward enhancements to NR positioning in TSG RAN WGs.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-193237: "new SID on NR Positioning Enhancements".

[3]	3GPP TR 38.855: "Study on NR Positioning (Release 16)".

[4]		R1-2009433	Evaluation results for Rel-16 positioning and Rel-17 enhancement	Huawei, HiSilicon

[5]	R1-2007665	Evaluation of NR positioning performance	vivo

[6]	R1-2007720	Evaluation of achievable positioning accuracy	BUPT

[7]	R1-2007754	Evaluation of achievable accuracy and latency	ZTE

[8]	R1-2007859	Discussion of evaluation of NR positioning performance	CATT

[9]	R1-2007908	NLOS Identification and Mitigation	FUTUREWEI

[10]	R1-2009390	Update of Evaluation Results for NR Positioning Performance in I-IoT Scenarios	Intel Corporation

[11]	R1-2007997	NR Positioning Latency Evaluations	Lenovo, Motorola Mobility

[12]	R1-2008225	Evaluation of NR positioning in IIOT scenario	OPPO

[13]	R1-2009555	Results on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

[14]	R1-2009502 Discussion on Performance evaluation of Rel-17 positioning	Sony

[15]	R1-2008416	Discussions on evaluation of achievable positioning accuracy and latency for NR positioning	LG Electronics

[16]	R1-2008489	Evaluation of achievable positioning latency	InterDigital, Inc.

[17]	R1-2009708	Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

[18]	R1-2009428	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

[19]	R1-2008720	Positioning evaluation results on potential enhancements for additional use cases	CeWiT

[20]	R1-2008764	Evaluation of achievable positioning accuracy and latency	Ericsson

[21]	R1-2008765	Potential positioning enhancements	Ericsson

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	General description of NR positioning

(General description of NR positioning up to release 16 & NR positioning enhancements in rel17)

# 5	Target requirements for NR positioning enhancements in Rel-17

## 5.1 	Target requirements

## 5.2 	Performance evaluation metrics

(Includes horizontal accuracy vertical accuracy and other metrics)

For evaluating performance of NR positioning technologies, the following metrics apply. The following percentiles of positioning error are analyzed: 50%, 67%, 80%, 90%.

### 5.2.1	Horizontal accuracy

### 5.2.2	Vertical accuracy

### 5.2.3		Other metrics

#### 5.2.3.1	Latency

##### 5.2.3.1.1	Physical layer Latency

Latency includes higher layer and physical layer latency. Physical layer latency for DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted approaches are separately studied

The physical layer latency start- and end-time are defined for each positioning method in table 5.2.3.1-1

Table 5.2.3.1-1: Definition of physical layer latency start- and end-time

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| UL-only method & UL ECID & Multi-RTT | Reception by the gNB of the NRPPa measurement request message | The transmission by the gNB of the NRPPa measurement response message |
| UE-based | - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE |

##### 5.2.3.1.2	Higher layer Latency

#### 5.2.3.2	Network efficiency

PRS/SRS resource utilization is the metric used to evaluate network efficiency.

#### 5.2.3.3	Device efficiency

The UE power consumption models developed in TR38.840 can be considered as the starting point for defining the UE power consumption model for the evaluation for NR positioning. For evaluations, it is up to each company to detail their methodology (including the power model) for evaluation.

# 6 	Additional scenarios and channel models for NR positioning enhancements

From justification, for the evaluation of solutions, the Rel-16 scenarios and channel models in TR 38.855 are reused where applicable, and additional scenarios for IIoT use cases should be defined.

from objective 1a. Includes definition of  additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases e.g. (I)IoT)

The scenario parameters common to all the scenarios in the study are detailed in table 6-1. Additionally, blockage model is not considered. For evaluations including UE mobility, the spatial consistency procedure defined in TR 38.901 is taken into consideration.

The evaluation methodology does not define any baseline reference signals. Configurations of DL PRS and SRS supported by Rel-16 specifications are used for evaluation of the achievable performance based on Rel-16 positioning technologies.

Table 6-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: 4-panels UE: - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: [X] ns for gNB and [Y] ns for UE - X and Y are up to companies - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

## 6.1 	IIoT use cases

For evaluating baseline performance, the following scenarios (with various options/configurations) are defined for RAT-dependent positioning techniques for the NR positioning enhancements study

-	Scenario 1. InF-SH for FR1 and FR2

-	Scenario 2. InF-DH for FR1 and FR2

In the evaluation of all scenarios, the absolute-time-of arrival model defined in TR 38.901 is considered, without modification. Parameters specific to scenario 1and 2 are detailed in table 6.1-1

Table 6.1-1: Parameters common to InF scenarios

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be - (baseline) at least the convex hull of the horizontal BS deployment. - (optional) It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(Inf-SH) and X2= for scenario 2 (InF-DH) |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: - Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height - (Optional): {40%, 3m, 5m} - (Optional): {60%, 6m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

## 6.2 	General commercial use cases

For general commercial use cases, Rel-16 scenarios and channel models in TR 38.855 are reused. For the absolute time of arrival modelling in IOO, UMa, Umi, companies may provide the details of their model, if any.

# 7	Studied NR positioning enhancements

(from objective 1c. Includes positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency for both RAN1 and RAN2.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case.  )

The following enhancements have been considered during this study:

- Partial staggering and non-staggering RE mapping of SRS for positioning with different combinations of comb-factors and symbol lengths, including the methods/signalling for addressing potential time-domain aliasing due to the partial/non-staggering RE mapping.

- Semi-persistent and a-periodic transmission and reception of DL PRS

  - Semi-persistent means MAC-CE triggered

  - Aperiodic would correspond to DCI-triggered

- On-demand transmission and reception of DL PRS

  - On-demand corresponds to the UE-initiated or network-initiated request of PRS and/or SRS, i.e. UE or LMF request/suggesting/recommending specific PRS pattern, ON/OFF, periodicity, BW, etc.

- Multipath mitigation techniques including but not limited to the following:

  - The applicable scenarios and performance benefits of multipath mitigation techniques

  - The methods/measurement/signaling for the LOS/NLOS detection and identification

  - The measurements for supporting the multipath mitigation/utilization

  - The procedure and signaling for supporting the multipath mitigation/utilization

  - Implementation-based solutions (e.g., outlier rejection) without the need of any additional specified method/measurements/procedures/signaling.

  - Note: The above study applies to DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted positioning.

- NR positioning for UEs in RRC_IDLE state and UEs in RRC_INACTIVE state, including the benefits on latency, network/UE efficiency and UE power consumption

- For reducing NR positioning latency, more efficient signaling & procedures enabling a device to request and report positioning information, which may include, but not limited to, the following aspects:

  - DL PRS/SRS configuration, activation or triggering.

  - The request for positioning information (the assistance data, etc.).

  - The report of positioning information (the measurement report, etc.).

  - Note: It is not within RAN1 scope to analyze positioning architecture enhancements to enable such more efficient signaling & procedures.

  - Note: RAN1 does not make any assumptions on whether the LCS architecture specified in TS 23.273 is enhanced or not.

- Simultaneous transmission by the UE and reception by the gNB of the SRS for positioning across multiple CCs and multiple slots, including

  - The scenarios and performance benefits of the enhancement

  - The impact of channel spacing, TA and timing offset, phase offset, frequency error, and power imbalance across slots or CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

- Scenario, benefits, and methods for improving the accuracy of the UL AoA and DL-AoD methods for both UE-based and network-based (including UE-assisted) positioning

- Scenario, benefits, methods and signaling for improving positioning accuracy in the presence of the UE Rx/Tx transmission delays, and/or and gNB Rx/Tx transmission delays for UE-based and network-based (including UE-assisted) positioning.

- Aggregating multiple DL positioning frequency layers of the same or different bands for improving positioning performance for both intra-band and inter-band scenarios

- The scenarios and performance benefits of aggregating multiple DL positioning frequency layers

- The impact of channel spacing, timing offset, phase offset, frequency error, and power imbalance among CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

- UE complexity considerations

# 8	Performance evaluations for Rel-17 targets

## 8.1	Performance analysis of Rel-16 positioning solutions

Including accuracy and latency (objective 1b) performance, compared to rel17 performance targets

### 8.1.1	Positioning accuracy analysis

#### 8.1.1.1	Observations  from source [4]

Table 8.1.1.1-1 captures observations based on NR positioning evaluations results for horizontal location error for baseline scenarios.

Table 8.1.1.1-2 captures observations based on NR positioning evaluations results for horizontal location error for modified DH and 3D positioning.

Table 8.1.1.1-3 captures observations based on NR positioning evaluations results for horizontal location error for UE/gNB calibration error.

Table 8.1.1.1-4 captures observations based on NR positioning evaluations results for vertical location error for modified DH and 3D positioning.

Table 8.1.1.1-1: Rel.16 NR positioning (baseline) – horizontal accuracy performance summary [4]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| 1, InF-SH, FR1, DL-TDOA | 1.964 | 0.964 | 1.764 | 1.464 |
| 2, InF-SH, FR1, UL-TDOA | 1.0277 | 0.0277 | 0.8277 | 0.5277 |
| 3, InF-SH, FR1, UL-TDOA/AoA | 0.2682 | Yes | 0.0682 | Yes |
| 4, InF-SH, FR1, Multi-RTT | 1.6992 | 0.6992 | 1.4992 | 1.1992 |
| 5, InF-DH422, FR1, DL-TDOA | 15.635 | 14.635 | 15.435 | 15.135 |
| 6, InF- DH422, FR1, UL-TDOA | 9.6631 | 9.163 | 9.963 | 9.663 |
| 7, InF-DH422, FR1, UL-TDOA/AoA | 0.8016 | Yes | 0.6016 | 0.3016 |
| 8, InF- DH422, FR1, Multi-RTT | 7.311 | 6.311 | 7.111 | 6.811 |
| 9, InF-SH, FR2, DL-TDOA | 0.9633 | Yes | 0.7633 | 0.4633 |
| 10, InF-SH, FR2, DL-TDOA/AoD | 0.0654 | Yes | Yes | Yes |
| 11, InF-SH, FR2, UL-TDOA/AoA | 0.0694 | Yes | Yes | Yes |
| 12, InF-SH, FR2, Multi-RTT | 0.4496 | Yes | 0.2496 | Yes |
| 13, InF-DH422, FR2, DL-TDOA | 9.6798 | 8.6798 | 9.4798 | 9.1798 |
| 14, InF- DH422, FR2, DL-TDOA/AoD | 0.7197 | Yes | 0.5197 | 0.2197 |
| 15, InF-DH422, FR2, UL-TDOA/AoA | 0.7086 | Yes | 0.5086 | 0.2086 |
| 16, InF- DH422, FR2, Multi-RTT | 4.2895 | 3.2895 | 4.0895 | 3.7895 |

Table 8.1.1.1-2: Rel.16 NR positioning (modified DH and 3D positioning) – horizontal accuracy performance summary [4]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| 101, InF-DH435, FR1, UL-TDOA/AoA | 1.3012 | 0.3012 | 1.1012 | 0.8012 |
| 102, InF-DH435, FR1, Multi-RTT | 9.8411 | 8.8411 | 9.6411 | 9.3411 |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | 4.3405(H) | 3.3405 | 4.1405 | 3.8405 |
| 104, InF-DH435-3D, FR1, Multi-RTT | 16.0515(H) | 15.0515 | 15.8515 | 15.5515 |
| 105, InF-DH435, FR2, UL-TDOA/AoA | 1.1486 | 0.1486 | 0.9486 | 0.6486 |
| 106, InF-DH435, FR2, Multi-RTT | 5.46 | 4.46 | 5.26 | 4.96 |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | 2.4365(H) | 1.4365 | 2.2365 | 1.9365 |
| 108, InF-DH435-3D, FR2, Multi-RTT | 15.5828(H) | 14.5828 | 15.3828 | 15.0828 |

Table 8.1.1.1-3: Rel.16 NR positioning (UE/gNB calibration error) – horizontal accuracy performance summary [4]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| 201, InF-SH, FR1, DL-TDOA, Group Delay Error | 1.458 | 0.458 | 1.258 | 0.958 |
| 202, InF-SH, FR1, UL-TDOA, Group delay error | 1.2343 | 0.2343 | 1.0343 | 0.7343 |
| 203, InF-SH, FR1, UL-TDOA/AoA, Group delay error | 0.3251 | Yes | 0.1251 | Yes |
| 204, InF-SH, FR1, Multi-RTT, Group delay error | 4.2662 | 3.2662 | 4.0662 | 3.7662 |
| 205, InF-DH422, FR1, DL-TDOA, Group delay error | 15.039 | 14.039 | 14.839 | 14.539 |
| 206, InF-DH422, FR1, UL-TDOA, Group delay error | 9.4102 | 8.4102 | 9.2102 | 8.9102 |
| 207, InF-DH422, FR1, UL-TDOA/AoA, Group delay error | 0.8662 | Yes | 0.6662 | 0.3662 |
| 208, InF-DH422, FR1, Multi-RTT, Group delay error | 9.5701 | 8.5701 | 9.3701 | 9.0701 |
| 209, InF-SH, FR1, UL-AoA | 0.1119 | Yes | Yes | Yes |
| 210, InF-SH, FR1, UL-AoA, Angle error 1 degree | 1.1676 | 0.1676 | 0.9676 | 0.6676 |
| 211, InF-SH, FR1, UL-AoA, Angle error 2 degrees | 2.1732 | 1.1732 | 1.9732 | 1.6732 |
| 212, InF-SH, FR1, UL-AoA, Angle error 5 degrees | 5.3982 | 4.3982 | 5.1982 | 4.8982 |

Table 8.1.1.1-4: Rel.16 NR positioning (modified DH and 3D positioning) – vertical accuracy performance summary [4]

| Simulation case (Vertical Error) | Accuracy achieved @[90]% | Commercial vertical accuracy requirements [3]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | 1.1585(V) | Yes | 0.9585 | 0.1585 |
| 104, InF-DH435-3D, FR1, Multi-RTT | 1.6675(V) | Yes | 1.4675 | 0.6675 |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | 0.4593(V) | Yes | 0.2593 | Yes |
| 108, InF-DH435-3D, FR2, Multi-RTT | 1.8800(V) | Yes | 1.68 | 0.88 |

#### 8.1.1.2	Observations  from source [7]

Table 8.1.1.2-1: Rel.16 NR positioning – horizontal accuracy performance summary[7]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1 | 0.603 |
| Case 2 | 0.568 |
| Case 3 | 0.704 |
| Case 4 | 0.943 |
| Case 5 | 1.479 |
| Case 6 | 0.092 |
| Case 7 | 0.090 |
| Case 8 | 0.300 |
| Case 9 | 0.615 |
| Case 10 | 1.224 |
| Case 11 | 12.433 |
| Case 12 | 12.345 |
| Case13 | 12.386 |
| Case 14 | 12.368 |
| Case 15 | 12.458 |
| Case 16 | 14.759 |
| Case 17 | 12.174 |
| Case 18 | 10.815 |
| Case 19 | 12.285 |
| Case 20 | 14.845 |

Table 8.1.1.2-2: Rel.16 NR positioning – vertical accuracy performance summary [7]

| Simulation case (Vertical Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 21 | 0.979 |
| Case 22 | 0.459 |
| Case 23 | 1.419 |
| Case 24 | 1.271 |

#### 8.1.1.3	Observations  from source [8]

Table 8.1.1.3-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.3-1: Rel.16 NR positioning – horizontal accuracy performance summary [8]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If No, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If No, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | 0.1650 | Yes | Yes |
| [Case 2], [InF-SH-2D], [FR1], [UL-TDOA] | 0.1551 | Yes | Yes |
| [Case 3], [InF-SH-2D], [FR1], [Multi-RTT] | 0.1650 | Yes | Yes |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | 0.2045 | No （0.0045） | Yes |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | 0.2574 | No （0.0574） | Yes |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | 0.2677 | No （0.0677） | Yes |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | 0.2540 | No (0.054) | Yes |
| [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | 0.1693 | Yes | Yes |
| [Case 9], [InF-DH-2D], [FR1], [UL-TDOA] | 0.1184 | Yes | Yes |
| [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | 0.1237 | Yes | Yes |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | 0.7089 | No (0.5089) | No (0.2089) |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | 0.6937 | No （0.4937） | No (0.1937) |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | 0.151 | Yes | Yes |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | 0.692 | No (0.362) | No (0.062) |
| [Case 15], [IOO], [FR1], [DL-TDOA] | 0.2288 | —— | —— |
| [Case 16], [IOO], [FR1], [UL-TDOA] | 0.1836 | —— | —— |
| [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | 0.1219 | —— | —— |
| [Case 18], [IOO], [FR1], [Multi-RTT] | 0.283 | —— | —— |
| [Case 19], [IOO], [FR1], [DL-TDOA] | 32.4509 | —— | —— |
| [Case 20], [IOO], [FR1], [UL-TDOA] | 32.0927 | —— | —— |
| [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | 9.2356 | —— | —— |
| [Case 22], [IOO], [FR1], [Multi-RTT] | 1.3668 | —— | —— |
| [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | 0.0372 | Yes | Yes |
| [Case 24], [InF-SH-2D], [FR2], [UL-TDOA] | 0.0538 | Yes | Yes |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | 0.0789 | Yes | Yes |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | 0.0817 | Yes | Yes |
| [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | 0.0388 | Yes | Yes |
| [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | 0.0553 | Yes | Yes |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | 0.7033 | No （0.5033） | No （0.2033） |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | 0.6848 | No （0.4848） | No （0.1848） |
| [Case 31], [IOO], [FR2], [DL-TDOA] | 0.0406 | —— | —— |
| [Case 32], [IOO], [FR2], [UL-TDOA] | 0.0397 | —— | —— |
| [Case 33], [IOO], [FR2], [Multi-RTT] | 0.0402 | —— | —— |
| [Case 34], [IOO], [FR2], [DL-TDOA] | 32.3809 | —— | —— |
| [Case 35], [IOO], [FR2], [UL-TDOA] | 32.0887 | —— | —— |
| [Case 36], [IOO], [FR2], [Multi-RTT] | 1.2681 | —— | —— |

Table 8.1.1.3-2 captures observations based on NR positioning evaluations results for vertical location error.

Table 8.1.1.3-2: Rel.16 NR positioning – vertical accuracy performance summary [8]

| Simulation case (Vertical Error) | Accuracy achieved @[90]% | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If No, provide performance gaps@[90]% | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If No, provide performance gaps@[90]% |
| --- | --- | --- | --- |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | 1.8954 | No （1.6954） | No （0.8954） |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | 2.0599 | No （1.6599） | No （1.0599） |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | 2.256 | No (2.056) | No (1.256) |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | 2.18 | No (1.98) | No (1.18) |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | 2.9917 | No （2.7917） | No （1.9917） |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | 2.049 | No （1.849） | No （1.049） |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | 0.633 | No (0.433) | Yes |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | 2.82 | No (2.62) | No (1.82) |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | 0.6283 | No （0.4283） | Yes |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | 0.8304 | No （0.6304） | Yes |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | 3.0578 | No （2.8578） | No （2.0578） |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | 3.1267 | No （2.9267） | No （2.1267） |

#### 8.1.1.4	Observations  from source [13]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1, [InF-SH, DL-TDOA, FR1] | 4.35 |
| Case 2, [InF-DH, DL-TDOA, FR1] | 7.16 |
| Case 3, [IOO, DL-TDOA, FR1] | 4.31 |
| Case 4, [IOO, DL-TDOA, FR1] | 6.50 |
| Case 5, [UMi, DL-TDOA, FR1] | 23.81 |
| Case 6, [InF-SH, DL-TDOA, FR1] | 1.65 |
| Case 7, [InF-DH, DL-TDOA, FR1] | 4.99 |

#### 8.1.1.5 Observations  from source [5]

Table 8.1.1.5-1.1 to Table 8.1.1.5-1.3 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.5-1.1: Rel.16 NR positioning – horizontal accuracy performance summary for baseline with perfect synchronization [5]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| [Case 1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 4.15 | 3.95 |
| [Case 3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 2.97 | 2.77 |
| [Case 5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 5.92 | 5.72 |
| [Case 7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 5.77 | 5.57 |
| [Case 11], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | 0.094 | Yes |
| [Case 13], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | 0.031 | Yes |
| [Case 15], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | 0.60 | 0.4 |
| [Case 17], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | 0.049 | Yes |
| [Case 19], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | 4.22 | 4.02 |
| [Case 21], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | 4.07 | 3.87 |
| [Case 23], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | 5.85 | 5.65 |
| [Case 25], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | 5.76 | 5.56 |
| [Case 27], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | 0.087 | Yes |
| [Case 29], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | 0.032 | Yes |
| [Case 31], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | 0.60 | 0.40 |
| [Case 33], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | 0.051 | Yes |
| [Case 35], [SH, perfect sync], [FR1], [UL-AOA] | 5.93 | 5.73 |
| [Case 37], [DH, perfect sync], [FR1], [UL-AOA] | 5.48 | 5.28 |
| [Case 39], [SH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | 0.41 | 0.21 |
| [Case 41], [DH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | 0.68 | 0.48 |
| [Case 43], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | 4.25 | 4.05 |
| [Case 45], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | 3.96 | 3.76 |
| [Case 47], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | 5.88 | 5.68 |
| [Case 49], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | 5.74 | 5.54 |
| [Case 51], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | 0.10 | Yes |
| [Case 53], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | 0.031 | Yes |
| [Case 55], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | 0.60 | 0.40 |
| [Case 57], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | 0.051 | Yes |

Table 8.1.1.5-1.2: Rel.16 NR positioning – horizontal accuracy performance summary for baseline with 50ns synchronization error [5]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| [Case 2], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 24.06 | 23.86 |
| [Case 4], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 23.21 | 23.01 |
| [Case 6], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 23.79 | 23.59 |
| [Case 8], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 22.90 | 22.70 |
| [Case 12], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | 26.09 | 25.89 |
| [Case 14], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | 25.67 | 25.47 |
| [Case 16], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | 20.30 | 20.10 |
| [Case 18], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | 20.16 | 19.96 |
| [Case 20], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | 24.51 | 24.31 |
| [Case 22], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | 23.21 | 23.01 |
| [Case 24], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | 22.90 | 22.70 |
| [Case 26], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | 18.92 | 18.72 |
| [Case 28], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | 27.70 | 27.50 |
| [Case 30], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | 25.67 | 25.47 |
| [Case 32], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | 22.01 | 21.81 |
| [Case 34], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | 19.74 | 19.54 |
| [Case 36], [SH, sync error 50ns], [FR1], [UL-AOA] | 6.20 | 6.00 |
| [Case 38], [DH, sync error 50ns], [FR1], [UL-AOA] | 5.76 | 5.56 |
| [Case 40], [SH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | 0.43 | 0.23 |
| [Case 42], [DH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | 0.77 | 0.57 |
| [Case 44], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | 4.71 | 4.51 |
| [Case 46], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | 4.13 | 3.93 |
| [Case 48], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | 6.20 | 6.00 |
| [Case 50], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | 6.23 | 6.03 |
| [Case 52], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | 0.10 | Yes |
| [Case 54], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | 0.030 | Yes |
| [Case 56], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | 0.78 | 0.58 |
| [Case 58], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | 0.055 | Yes |

Table 8.1.1.5-1.3: Rel.16 NR positioning – horizontal accuracy performance summary for DH {60%,6,2} with perfect synchronization [5]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| [Case 9], [DH {0.6,6,2}, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 18.71 | 18.51 |
| [Case 10], [DH {0.6,6,2}, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 15.09 | 14.89 |

Table 8.1.1.5-2 captures observations based on NR positioning evaluations results for vertical location error.

Table 8.1.1.5-2: Rel.16 NR positioning – vertical accuracy performance summary [[5]]

| Simulation case (Vertical Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [1]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| [Case V1], [SH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 0.66 | Yes |
| [Case V2], [DH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 1.12 | 0.12 |
| [Case V3], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 0.82 | Yes |
| [Case V4], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 1.39 | 0.39 |
| [Case V5], [SH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 1.05 | 0.05 |
| [Case V6], [DH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 5.46 | 4.46 |
| [Case V7], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 1.21 | 0.21 |
| [Case V8], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 9.06 | 8.06 |

#### 8.1.1.6	Observations  from source [12]

Table 8.1.1.6-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.6-1: Rel.16 NR positioning – horizontal accuracy performance summary [12]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| Case1, (InF-SH, FR1) | 8.5 | No, 8.3 | No, 8.00 |
| Case2, (InF-DH, FR1) | 14.95 | No, 14.75 | No, 14.55 |

#### 8.1.1.7	Observations  from source [10]

Table 8.1.1.7-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.7-1: Rel.16 NR positioning – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1, InF-SH, FR1, DL-TDOA | 0.85 |
| Case 2, InF-DH, FR1, DL-TDOA | 6.2 |
| Case 3, InF-SH, FR2, DL-TDOA | 0.65 |
| Case 4, InF-DH, FR2, DL-TDOA | 17.3 |
| Case 5, InF-SH, FR1, UL-TDOA | 0.77 |
| Case 6, InF-DH, FR1, UL-TDOA | 6.13 |
| Case 7, InF-SH, FR2, UL-TDOA | 0.9 |
| Case 8, InF-DH, FR2, UL-TDOA | 16.9 |
| Case 9, InF-SH, FR1, Multi-RTT | 0.25 |
| Case 10, InF-DH, FR1, Multi-RTT | 16.3 |
| Case 11, InF-SH, FR2, Multi-RTT | 0.9 |
| Case 12, InF-DH, FR2, Multi-RTT | 7.72 |

Table 8.1.1.7-2 captures observations based on NR positioning evaluations results for vertical location error.

Table 8.1.1.7-2: Rel.16 NR positioning – vertical accuracy performance summary

| Simulation case (Vertical Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1, InF-SH, FR1, DL-TDOA | 8.5 |
| Case 2, InF-DH, FR1, DL-TDOA | 12.6 |
| Case 3, InF-SH, FR2, DL-TDOA | 12.88 |
| Case 4, InF-DH, FR2, DL-TDOA | 63.4 |
| Case 5, InF-SH, FR1, UL-TDOA | 12.9 |
| Case 6, InF-DH, FR1, UL-TDOA | 12.9 |
| Case 7, InF-SH, FR2, UL-TDOA | 13 |
| Case 8, InF-DH, FR2, UL-TDOA | 62.78 |
| Case 9, InF-SH, FR1, Multi-RTT | 13.1 |
| Case 10, InF-DH, FR1, Multi-RTT | 66 |
| Case 11, InF-SH, FR2, Multi-RTT | 6.45 |
| Case 12, InF-DH, FR2, Multi-RTT | 7.07 |

#### 8.1.1.8 Observations  from source [14]

Table 8.1.1.8-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.8-1: Rel.16 NR positioning – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| [Case 1], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.24 |
| [Case 2], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 0.70 |
| [Case 3], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 2.80 |
| [Case 4], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 3.87 |
| [Case 5], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.35 |
| [Case 6], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | 1.00 |

#### 8.1.1.9	Observations  from source [20]

Table 8.1.1.9-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.9-1: Rel.16 NR positioning – horizontal accuracy performance summary [20]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| [20], source1, UMa, FR1, DL-TDOA (50% UEs indoor) | 11.97m (1 occasion) 7.04m (9 occasions) |
| [20], source1, UMa, FR1, DL-TDOA (100% UEs outdoor) | 4.5m (1 occasion) 2.5m (9 occasions) |
| [20], source2, UMi, FR1, DL-TDOA | 1.7m (1 occasion) 1.06m (9 occasions) |
| [20], source3, UMi, FR1, UL-TDOA | 3.10m (1 occasion) 2.43m (9 occasions) |
| [20], source4, UMi, FR2, DL-TDOA | 0.13m (1 occasion) 0.06m (9 occasions) |
| [20], source5, IOO, FR1, DL-TDOA | 1.51m (1 occasion) 0.93m (9 occasions) |
| [20], source6, IOO, FR1, UL-TDOA | 1.52m (1 occasion) 0.98m (9 occasions) |
| [20], source7, IOO, FR2, DL-TDOA | 0.18m (1 occasion) 0.07m (9 occasions) |
| [20], source8, InF-SH, FR1, DL-TDOA | 0.19m (convex hull UEs) 0.36m (all UEs) |
| [20], source9, InF-SH , FR1, UL-TDOA | 0.18m (convex hull) 0.35m (all UEs) |
| [20], source10, InF-DH, FR1, DL-TDOA | 7m (convex hull UEs) 14.3m (all UEs) |
| [20], source11, InF-DH , FR1, UL-TDOA | 7.5m (convex hull UEs) 14.55m (all UEs) |
| [20], source12, InF-SH, FR2, DL-TDOA | 0.0172m(no RX/Tx error-convex hull UEs) 3.34m (8ns Rx/Tx error- convex hull UEs) 0.0349m (no RX/Tx error-all UEs) 3.68m (8ns Rx/Tx error-all UEs) |
| [20], source13, InF-SH , FR2, UL-TDOA | 0.0163m (no RX/Tx error-convex hull UEs) 3.36m (8ns Rx/Tx error-convex hull UEs) 0.0313m (no RX/Tx error-all UEs) 3.85m (8ns Rx/Tx error-all UEs) |
| [20], source14, InF-DH, FR2, DL-TDOA | 7.06m (convex hull UEs) 14.95m (all UEs) |
| Ericsson15, InF-DH , FR2, UL-TDOA | 6.98m (convex hull UEs) 13.48m (all UEs) |

#### 8.1.1.10	Observations  from source [17]

Table 8.1.1.10-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.10-1: Rel.16 NR positioning – horizontal accuracy performance summary

|  |  | 90% |
| --- | --- | --- |
| Case 1, InF FR1 DH ISD20, 100MHz, Link Quality, DL TDOA | Convex UEs | 31.66m |
|  | (Optional) All UEs | 26.8m |
| Case 1, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Convex UEs | 6.92m |
|  | (Optional) All UEs | 8.25m |
| Case 2, InF FR1 SH ISD50, 100MHz, Link Quality, DL TDOA | Convex UEs | 2.92m |
|  | (Optional) All UEs | 4.16m |
| Case 2, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.28m |
|  | (Optional) All UEs | 0.54m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 12.66m |
|  | (Optional) All UEs | 30.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 13.1m |
|  | (Optional) All UEs | 17.62m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Variable UE heights | Convex UEs | 0.22m |
|  | (Optional) All UEs | 0.47m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.12m |
|  | (Optional) All UEs | 0.36m |

| Horizontal Positioning Error (all UEs) | Beam Pair | 90% |
| --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.019 |
|  | Strongest | 0.027 |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.015 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.025 |
|  | Strongest | 9.40 |
| Case 8 InF-DH FR2 3d mRTT | Earliest | 0.020 |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0087 |
|  | Strongest | 0.0170 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0096 |
|  | Strongest | 1.6767 |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.040 |
|  | Strongest | 6.32 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.024 |
|  | Strongest | 0.053 |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.0403 |
|  | Strongest | 6.3233 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0113 |
|  | Strongest | 0.0279 |

| Horizontal Positioning Error |  | 90% |
| --- | --- | --- |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.6 |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 3.2 |
| Case 9, UMI, FR1, RTT, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 8 |
| Case 10, UMI, FR1,RTT, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 9.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 17.3 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, LQ Algorithm | (Optional) All UEs | 20.1 |
| Case 11, UMA, FR1, DL-TDOA, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Outdoor UEs | 1.5 |
| Case 11, UMA, FR1, RTT, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Indoor UEs | 96 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 1 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.3 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.4 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.6 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.9 |

| Horizontal Positioning Error |  | 90% |
| --- | --- | --- |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |

| Horizontal Positioning Error |  |  | 90% |
| --- | --- | --- | --- |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 0.87 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 1.37 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 2.22 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 0.87 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 1.37 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 2.22 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 2.3 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 2.3 |

| Case ID | Beam Pair | Tx T1 | 90% |
| --- | --- | --- | --- |
| Case 18 InF-SH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 |
|  |  | 0.1ns | 0.06 |
|  |  | 0.2ns | 0.13 |
|  |  | 0.5ns | 0.40 |
|  |  | 1.0ns | 0.96 |
|  |  | 2.0ns | 1.59 |
|  | Strongest beam pair | 0.0ns | 0.02 |
|  |  | 0.1ns | 0.11 |
|  |  | 0.2ns | 0.25 |
|  |  | 0.5ns | 0.94 |
|  |  | 1.0ns | 2.30 |
|  |  | 2.0ns | 5.19 |
| Case 19 InF-DH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.02 |
|  |  | 0.1ns | 0.10 |
|  |  | 0.2ns | 0.24 |
|  |  | 0.5ns | 0.76 |
|  |  | 1.0ns | 2.10 |
|  |  | 2.0ns | 6.31 |
|  | Strongest beam pair | 0.0ns | 43.94 |
|  |  | 0.1ns | 36.56 |
|  |  | 0.2ns | 35.28 |
|  |  | 0.5ns | 44.16 |
|  |  | 1.0ns | 40.33 |
|  |  | 2.0ns | 46.31 |
| Case 20 InH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.03 |
|  |  | 0.1ns | 0.31 |
|  |  | 0.2ns | 0.47 |
|  |  | 0.5ns | 1.16 |
|  |  | 1.0ns | 2.04 |
|  |  | 2.0ns | 4.11 |
|  | Strongest beam pair | 0.0ns | 0.21 |
|  |  | 0.1ns | 7.44 |
|  |  | 0.2ns | 8.22 |
|  |  | 0.5ns | 11.37 |
|  |  | 1.0ns | 14.94 |
|  |  | 2.0ns | 18.87 |
| Case 21 UMi, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.03 |
|  |  | 0.1ns | 0.13 |
|  |  | 0.2ns | 0.20 |
|  |  | 0.5ns | 0.38 |
|  |  | 1.0ns | 0.73 |
|  |  | 2.0ns | 1.38 |
|  | Strongest beam pair | 0.0ns | 16.21 |
|  |  | 0.1ns | 23.54 |
|  |  | 0.2ns | 16.59 |
|  |  | 0.5ns | 19.30 |
|  |  | 1.0ns | 14.27 |
|  |  | 2.0ns | 21.65 |

Table 8.1.1.10-2 captures observations based on NR positioning evaluations results for vertical location error.

Table 8.1.1.10-2: Rel.16 NR positioning – vertical accuracy performance summary

| Vertical Positioning error |  | 90% |
| --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 20.6m |
|  | (Optional) All UEs | 38.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 18.44m |
|  | (Optional) All UEs | 22.98m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, , Unequal gNBs heights, Variable UE heights | Convex UEs | 1.89m |
|  | (Optional) All UEs | 2.63m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, Unequal gNBs heights, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.9m |
|  | (Optional) All UEs | 1.34m |

| Vertical (Across All UEs) | Beam Pair | 90% |
| --- | --- | --- |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.084 |
| Case 8 InF-SH FR2 3d mRTT | Earliest | 0.041 |

#### 8.1.1.11	Observations  from source [18]

Table 8.1.1.11-1: Rel.16 NR positioning – horizontal accuracy performance summary from [18]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1- config ID 445 (FR1-InF DH) | ≥10 |
| Case 3- config ID 1112 (FR1-UMi) | 3.24 |
| Case 2- config ID 1011 (FR1-UMi with ATOA) | ≥10 |

### 8.1.2	Physical layer latency analysis for Rel-16

#### 8.1.2.1	Observations  from source [4]

No observation found

#### 8.1.2.2	Observations  from source [7]

Summary of latency performance analysis is provided in Table 8.1.2.2-1.

Table 8.1.2.2-1: NR positioning enhancements – latency performance summary

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Case PHY-L1, UE-A, DL-TDOA, FR1, FDD | 106.23 |
| Case PHY-L1, UE-A, DL-TDOA, FR2, FDD | 667.87 |
| Case PHY-L2, UE-B, DL-TDOA, FR1, FDD | 106.30 |
| Case PHY-L2, UE-B, DL-TDOA, FR2, FDD | 667.82 |
| Case PHY-L3, UE-A, DL-ECID, FR1,FDD | 10.43 |
| Case PHY-L3, UE-A, DL-ECID, FR2, FDD | 10.64 |

#### 8.1.2.3	Observations  from source [8]

Summary of latency performance analysis is provided in Table 8.1.2.3-1.

Table 8.1.2.3-1: NR Rel.16 positioning - latency performance summary [8]

| Description Evaluation Case | L1 Latency, ms | Commercial requirements [100]ms are met -Yes/No - If No, provide performance gaps | IIoT requirements of [10ms] are met - Yes/No. If No, provide performance gaps | IIoT requirements of [100]ms are met - Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case 1, 15kHz, FR1, DL-TDOA | 51.5 | Yes | No (41.5ms gaps) | Yes |
| Case 2, 15kHz, FR1, UL-TDOA | 5 | Yes | Yes | Yes |

#### 8.1.2.4	Observations  from source [13]

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Case 1, DL-TDOA/DL-AoD, FR1 | [44.35 – 10500] |
| Case 2, DL-TDOA/DL-AoD, FR2 | [35.08 – 2118.93] |
| Case 3, UL-TDOA/UL-AoA, FR1 | [2.78 – 81928.5] |

#### 8.1.2.5 Observations  from source [11]

A summary of the physical layer latency performance analysis for the DL-based positioning methods is provided in provided in Table 8.1.2.5-1.

Table 8.1.2.5-1: NR positioning enhancements – latency performance summary [11]

| Description Evaluation Case | L1 Latency1, ms |
| --- | --- |
| Case ID: 1, Scenario: UE-Assisted Positioning with MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA/R.16 DL-AoD | [38 - 235.6]: 30 kHz SCS |
|  | [35 - 229.6]: 120 kHz SCS |
| Case ID: 2, Scenario: UE-Assisted Positioning without MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDO/ R.16 DL-AoD | [17 - 5147.8]: 30 kHz SCS |
|  | [15.5 - 5144.8]: 120 kHz SCS |
| Case ID: 3, Scenario: UE-based Positioning with MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA/R.16 DL-AoD | [29 - 207.8]: 30 kHz SCS |
|  | [27.5 - 204.8]: 120 kHz SCS |
| Case ID: 4, Scenario: UE-based Positioning without MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA/R.16 DL-AoD | [8 – 5120]: 120 kHz SCS |
| Notes: 1: The presented L1 latency values correspond to the minimum and cautious estimates provided in Tables 8.1.2.5.2-1 - 8.1.2.5.2-4. Therefore, due to the assumptions of a single DL-PRS occasion, this may not correspond to an accurate positioning measurement and serves a guideline for the achievable physical layer latency. The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |

#### 8.1.2.6	Observations  from source [5]

Summary of latency performance analysis is provided in Table 8.1.2.6-1.

Table 8.1.2.6-1: NR positioning enhancements – latency performance summary [5]

| Description Evaluation Case | L1 Latency ms | Commercial requirements [100]ms are met -Yes/No - If no, provide performance gaps | IIoT requirements of [10ms] are met - Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- |
| [Case 1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-A] | 64ms~ |  | 54ms~ |  |
| [Case 1-1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-A], [idle,inactive] | 85.3ms~ or 104ms~ |  | 75.3ms~ or 94ms~ |  |
| [Case 2], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-B] Source [Network]/Destination [Network] | 66 ms ~ |  | 56ms~ |  |
| [Case 3], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-B] Source [UE]/Destination [UE] | 55.5ms~ |  | 45.5ms~ |  |
| [Case 4], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA], [periodic SRS] | 30.5ms~ |  | 20.5ms~ |  |
| [Case 5], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA], [A- SRS] | 11ms~ |  | 1ms~ |  |
| [Case 6], [IIoT/ Commercial], [Frequency Band], [Multi-RTT] | 94.5+ TAlign_DL_UL ~ |  | 84.5+TAlign_DL_UL~ |  |

#### 8.1.2.7	Observations  from source [12]

Summary of latency performance analysis is provided in Table 8.1.2.7-1.

Table 8.1.2.7-1: NR positioning enhancements – latency performance summary [12]

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Case1, DL-TDOA,FR1 | 54.125 |
| Case2, UL-TDOA,FR1 | 23.25 |
| Case3, UE- based method,FR1 | 54.125 |
| Case4, DL-TDOA,FR2 | 52.56 |
| Case5, UL-TDOA,FR2 | 23.125 |
| Case6, UE- based method,FR2 | 52.56 |

#### 8.1.2.8	Observations  from source [10]

Summary of latency performance analysis is provided in Table 8.1.2.8-1.

Table 8.1.2.8-1: NR positioning enhancements – latency performance summary

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Case 1, InF, FR1, R.16 DL-TDOA/DL-AoD | 4.5714 (L1 components) + [36] (L2/L3 components) + 88.5 (DL PRS processing) = 129.07 ms (total) |
| Case 2, InF, FR1, R.16 UL-TDOA/UL-AoA | 2.7678 (L1 components) + [16] (L2/L3 components) = 18.7678 (total) |
| Case 3, InF, FR1, R.16 Multi-RTT | 7.3393 (L1 components) + [45] (L2/L3 components) + 88.5 (DL PRS processing) = 140.8393 (total) |

#### 8.1.2.9 Observations  from source [16]

The latency analysis for each case is summarized in the following table:

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Latency analysis for UE-assisted DL methods (Case 1) | 33 |
| UE-based DL methods (Case 2) | 22-72 |
| UE-assisted UL methods (Case 3) | 12 |
| UE-assisted DL+UL methods (Case 4) | 45 |

#### 8.1.2.10 Observations  from source [17]

Summary of latency performance analysis is provided in Table 8.1.2.10-1.

Table 8.1.2.10-1: NR positioning enhancements – latency performance summary

| NR Rel-16 Scenario | L1-layer Latency Expected Range (ms) |
| --- | --- |
| UE-Assisted DL-only Positioning, RRC Connected State | 57 - 823 |
| UE-based DL-only Positioning, RRC Inactive State, External Client | 35.3 - 803.5 |
| UE-based DL-only Positioning, RRC Connected State, UE Internal-client | 46 - 811 |
| UE-based DL-only Positioning, RRC Inactive State, UE internal-client | 8 - 780 |
| UE-Assisted MRTT Positioning, RRC Connected State | 59 - 823 |

#### 8.1.2.11 Observations  from source [15]

Summary of latency performance analysis is provided in Table 8.1.2.11-1.

Table 8.1.2.11-1: NR positioning – latency analysis [15]

| Description Evaluation Case | Assumptions | L1 Latency, ms (including preparation/processing time at higher layer) | L1 Latency, ms (excluding preparation/processing time at higher layer) |
| --- | --- | --- | --- |
| Case 1, DL-TDOA, DL-AOD [NW initiated, UE-A] | FR1, 15kHz # of symbols for PUSCH: 1~14 OS # of symbols for PDSCH: 2~14 OS # of symbols for SRS: 2~12 OS Periodicity and offset for PUCCH: 2 OS ~ 80 slot The length of symbols for PUCCH: 1 OS~ 14 OS Slot for PDCCH Monitoring configured as periodicity and offset is 1slot. The first symbol(s) for PDCCH monitoring in the slots is zero # of symbols for CORESET: 1 OS ~3 OS -Uplink switching gap is not configured. -No BWP switching -No overlapping symbols of the PUCCH and the scheduled PUSCH -No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH *Note: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed. *Note: According to scheduling request configuration and UL grant configuration, the total values may change. For example, larger periodicity for SR and/or PDCCH monitoring periodicity are set. | For UE capability-1: 75.4+[X1]+[X2]ms ~ 300.33+[X1]+ [X2]ms For UE capability-2: 73.96+[X1]+[X2]ms ~ 298.47+[X1]+ [X2]ms | For UE capability-1: 23.4ms ~ 248.33ms For UE capability-2: 21.96ms ~ 246.47ms |
| Case 2, DL-TDOA, DL-AOD [UE initiated, UE-A] |  | For UE capability-1: 64.69+[X1]+[X2]ms ~ 288.55+[X1] +[X2]ms For UE capability-2: 63.61+[X1]+[X2]ms ~ 287.26+[X1] +[X2]ms | For UE capability-1: 22.69ms ~ 246.55ms For UE capability-2: 22.61ms ~ 245.26ms |
| Case 3, UL-TDOA, UL-AOA [NW initiated, UE-A] |  | For UE capability-1: 31.49+[X3] ms ~ 34.42+[X3] ms For UE capability-2: 30.77+[X3] ms ~ 33.28+[X3] ms | For UE capability-1: 1.49ms ~ 4.42ms For UE capability-2: 0.77ms ~ 3.28ms |
| Case 4, Multi-RTT [NW initiated, UE-A] |  | For UE capability-1: 147.62+[X1]+[X2]+[X3]ms ~ 376.53+[X1]+[X2]+[X3]ms For UE capability-2: 145.8+[X1]+[X2]+[X3]ms ~ 372.96+[X1]+[X2]+[X3] ms | For UE capability-1: 25.62ms ~ 254.53ms For UE capability-2: 23.08ms ~ 250.96ms |
| Case 5, E-CID [NW initiated, UE-A] |  | For UE capability-1: 23.84+[X2]ms ~ 106.77+[X2] ms For UE capability-2: 23.12+[X2]ms ~ 105.84+[X2] ms | For UE capability-1: 1.84ms ~ 84.77ms For UE capability-2: 1.12ms ~ 83.84ms |

## 8.2	Performance analysis of studied NR positioning enhancements

Including performance of positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency  ((objective 1c).

### 8.2.1	Positioning accuracy analysis for NR positioning enhancements

#### 8.2.1.1	Observations  from source [4]

Table 8.2.1.1-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.1-1: NR positioning enhancements – horizontal accuracy performance summary [4]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| 311, InF-DH422, FR1, UL-TDOA | Rel-16 baseline | 9.6631 | No | No | No |
| 312, InF-DH422, FR1, UL-TDOA w/ RAIM | 9.0062 | 0.6569 | Yes | 0.4569 | 0.1569 |
| 313, InF-DH422, FR1, UL-TDOA w/ LOS/NLOS identification | 9.452 | 0.2111 | Yes | 0.0111 | Yes |
| 321, InF-DH422, FR1, UL-TDOA, 100M contiguous | Rel-16 baseline | 0.2022 | Yes | No | Yes |
| 322, InF-DH422, FR1, UL-TDOA, 200M contiguous | 0.1638 | 0.0384 | Yes | Yes | Yes |
| 323, InF-DH422, FR1, UL-TDOA, 50MHz+100MHz (Gap)+50MHz | 0.111 | 0.0912 | Yes | Yes | Yes |
| 331, InF-SH, FR1, DL-TDOA, Comb-4 and 4-symbol | Rel-16 baseline | 0.0939 | Yes | Yes | Yes |
| 332, InF-SH, FR1, DL-TDOA, Comb-4 and 1-symbol | -0.0184 | 0.1123 | Yes | Yes | Yes |
| 333, InF-SH, FR1, DL-TDOA, Comb-12 and 12-symbol | Rel-16 baseline | 0.1091 | Yes | Yes | Yes |
| 334, InF-SH, FR1, DL-TDOA, Comb-12 and 1-symbol | -0.0108 | 0.1199 | Yes | Yes | Yes |
| 341, InF-SH, FR1, DL-TDOA | Rel-16 baseline | 0.0939 | Yes | Yes | Yes |
| 342, InF-SH, FR1, DL-TDOA, 20-RB of PRS punctured by SSB | -0.0151 | 0.109 | Yes | Yes | Yes |
| 362, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ legacy AoA) | Rel-16 baseline | 4.8161 | No | No | No |
| 363, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ modified AoA | 4.6467 | 0.1694 | Yes | Yes | Yes |
| 371, InF-DH422, FR1, Multi-RTT | Rel-16 baseline | 0.1694 | Yes | Yes | Yes |
| 372, InF-DH422, FR1, E-CID w/ single cell RTT/AoA | -0.0701 | 0.2395 | Yes | No | Yes |
| 381, InF-SH, FR1, UL-TDOA, 100M，0ns gNB Sync error | Rel-16 baseline | 0.1136 | Yes | Yes | Yes |
| 382, InF-SH, FR1, UL-TDOA, 100M，0.2ns gNB Sync error | -0.0316 | 0.1452 | Yes | Yes | Yes |
| 383, InF-SH, FR1, UL-TDOA, 100M，0.5ns gNB Sync error | -0.1692 | 0.2828 | Yes | No | Yes |
| 384, InF-SH, FR1, UL-TDOA, 100M，1ns gNB Sync error | -0.2544 | 0.5372 | Yes | No | Barely |
| 385, InF-SH, FR1, UL-TDOA, 200M, 0ns gNB Sync error | FR1 CA baseline | 0.025 | Yes | Yes | Yes |
| 386, InF-SH, FR1, UL-TDOA, 200M, 0.2ns gNB Sync error | -0.0852 | 0.1102 | Yes | Yes | Yes |
| 387 InF-SH, FR1, UL-TDOA, 200M, 0.5ns gNB Sync error | -0.2409 | 0.2659 | Yes | No | Yes |
| 388 InF-SH, FR1, UL-TDOA, 200M, 1ns gNB Sync error | -0.509 | 0.534 | Yes | No | Barely |

#### 8.2.1.2	Observations  from source [7]

Table 8.2.1.2-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.2-1: NR positioning enhancements – horizontal accuracy performance summary [7]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% |
| --- | --- | --- |
| Case 26 | 5.140 | 7.205 |
| Case 27 | 10.342 | 2.003 |
| Case 29 | 2.926 | 9.248 |
| Case 30 | 11.946 | 0.228 |

#### 8.2.1.3	Observations  from source [8]

Table 8.2.1.3-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.3-1: NR positioning enhancements – horizontal accuracy performance summary [8]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If No, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- |
| [Case 1], [InF-HH-2D], [FR1], [DL- TDOA+ DL-CPP] | 0.114m Vs CASE 1 in section 8.1.1.3 | 0.051 | YES | YES |
| [Case 2], [InF-HH-2D], [FR1], [UL- TDOA+ UL-CPP] | 0.1061m Vs CASE 2 in section 8.1.1.3 | 0.049 | YES | YES |

#### 8.2.1.4	Observations  from source [13]

#### 8.2.1.5	Observations  from source [5]

Table 8.2.1.5-1.1 to Table 8.2.1.5-1.8 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.5-1.1: NR positioning enhancements – horizontal accuracy performance summary for baseline with RAIM [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | 3.95 | 0.099 | Yes |
| [Case E3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | 2.95 | 0.024 | Yes |
| [Case E5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | 1.49 | 4.43 | 4.23 |
| [Case E7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | 1.42 | 4.35 | 4.15 |
| [Case E9], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0 | 0.094 | Yes |
| [Case E11], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.007 | 0.024 | Yes |
| [Case E13], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.43 | 0.17 | Yes |
| [Case E15], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.015 | 0.034 | Yes |
| [Case E17], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | 4.12 | 0.10 | Yes |
| [Case E19], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | 4.04 | 0.034 | Yes |
| [Case E21], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | 0.37 | 5.48 | 5.28 |
| [Case E23], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | 1.21 | 4.55 | 4.35 |
| [Case E25], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.004 | 0.083 | Yes |
| [Case E27], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0 | 0.032 | Yes |
| [Case E29], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.41 | 0.19 | Yes |
| [Case E31], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.008 | 0.043 | Yes |
| [Case E33], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | 4.14 | 0.11 | Yes |
| [Case E35], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | 3.91 | 0.049 | Yes |
| [Case E37], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | 0.99 | 4.89 | 4.69 |
| [Case E39], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | 1.62 | 4.12 | 3.92 |
| [Case E41], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.008 | 0.092 | Yes |
| [Case E43], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.008 | 0.030 | Yes |
| [Case E45], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.41 | 0.19 | Yes |
| [Case E47], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.003 | 0.048 | Yes |

Table 8.2.1.5-1.2: NR positioning enhancements – horizontal accuracy performance summary with RAIM and LOS detection [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E49], [SH, perfect sync], [FR1], [DL-TDOA, RAIM] |  | 0.094 | Yes |
| [Case E50], [SH, perfect sync], [FR1], [100% LOS detection probability without RAIM] |  | 0.096 | Yes |
| [Case E51], [SH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] |  | 0.083 | Yes |
| [Case E52], [SH, perfect sync], [FR1], [DL-TDOA, 95% LOS detection probability without RAIM] |  | 2.86 | 2.66 |
| [Case E53], [SH, perfect sync], [FR1], [DL-TDOA, 90% LOS detection probability without RAIM] |  | 4.54 | 4.34 |
| [Case E54], [SH, perfect sync], [FR1], [DL-TDOA, baseline no LOS detection without RAIM] |  | 4.62 | 4.42 |
| [Case E55], [DH, perfect sync], [FR1], [DL-TDOA, RAIM] |  | 0.17 | Yes |
| [Case E56], [DH, perfect sync], [FR1], [DL-TDOA, 100% LOS detection probability without RAIM] |  | 0.33 | 0.13 |
| [Case E57], [DH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] |  | 0.17 | Yes |
| [Case E58], [DH, perfect sync], [FR1], [DL-TDOA, 95% LOS detection probability without RAIM] |  | 3.40 | 3.20 |
| [Case E59], [DH, perfect sync], [FR1], [DL-TDOA, 90% LOS detection probability without RAIM] |  | 3.43 | 3.23 |
| [Case E60], [DH, perfect sync], [FR1], [DL-TDOA, baseline no LOS detection without RAIM] |  | 8.64 | 8.44 |

Table 8.2.1.5-1.3: NR positioning enhancements – horizontal accuracy performance summary with different timing measurement reporting granularity [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E61], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] |  | 0.16 | Yes |
| [Case E62], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] |  | 0.21 | 0.1 |
| [Case E63], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] |  | 0.47 | 0.27 |
| [Case E64], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] |  | 0.17 | Yes |
| [Case E65], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] |  | 0.35 | 0.15 |
| [Case E66], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] |  | 0.59 | 0.39 |

Table 8.2.1.5-1.4: NR positioning enhancements – horizontal accuracy performance summary with different Rx/Tx timing error [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E67], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] |  | 0.30 | 0.10 |
| [Case E68], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] |  | 0.34 | 0.14 |
| [Case E69], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] |  | 0.36 | 0.16 |
| [Case E70], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] |  | 0.35 | 0.15 |
| [Case E71], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] |  | 0.37 | 0.17 |
| [Case E72], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] |  | 0.42 | 0.22 |
| [Case E73], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] |  | 0.83 | 0.63 |
| [[Case E74], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] |  | 1.07 | 0.87 |
| [Case E75], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] |  | 1.87 | 1.67 |
| [Case E76], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] |  | 0.31 | 0.11 |
| [Case E77], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] |  | 0.32 | 0.12 |
| [Case E78], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] |  | 0.32 | 0.12 |
| [Case E79], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] |  | 0.28 | 0.08 |
| [Case E80], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] |  | 0.31 | 0.11 |
| [Case E81], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] |  | 0.40 | 0.20 |
| [Case E82 [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] |  | 0.76 | 0.56 |
| [[Case E83], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] |  | 0.88 | 0.68 |
| [Case E84], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] |  | 1.94 | 1.74 |
| [Case E85], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] |  | 0.24 | 0.04 |
| [Case E86], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] |  | 0.23 | 0.03 |
| [Case E87], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] |  | 0.27 | 0.07 |
| [Case E88], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] |  | 0.33 | 0.13 |
| [Case E89], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] |  | 0.44 | 0.24 |
| [Case E90], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] |  | 0.28 | 0.08 |
| [Case E91], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] |  | 0.34 | 0.14 |
| [[Case E92], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] |  | 0.76 | 0.56 |
| [Case E93], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] |  | 1.26 | 1.06 |
| [Case E94], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] |  | 0.24 | 0.04 |
| [Case E95], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] |  | 0.24 | 0.04 |
| [Case E96], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] |  | 0.28 | 0.08 |
| [Case E97], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] |  | 0.36 | 0.16 |
| [Case E98], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] |  | 0.46 | 0.26 |
| [Case E99], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] |  | 0.34 | 0.14 |
| [Case E100], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] |  | 0.48 | 0.28 |
| [[Case E101], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] |  | 0.87 | 0.67 |
| [Case E102], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] |  | 1.28 | 1.08 |

Table 8.2.1.5-1.5: NR positioning enhancements – horizontal accuracy performance summary with aggregation of DL positioning frequency layers [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E103], [SH, perfect sync], [FR1], [50M] |  | 0.31 | 0.11 |
| [Case E104], [SH, perfect sync], [FR1], [100M] |  | 0.094 | Yes |
| [Case E105], [SH, perfect sync], [FR1], [50M+50M] |  | 0.21 | 0.01 |
| [Case E106], [DH, perfect sync], [FR1], [50M] |  | 0.44 | 0.24 |
| [Case E107], [DH, perfect sync], [FR1], [100M] |  | 0.17 | Yes |
| Case E108], [DH, perfect sync], [FR1], [50M+50M] |  | 0.23 | 0.03 |

Table 8.2.1.5-1.6: NR positioning enhancements – horizontal accuracy performance summary with aggregation of DL positioning frequency layers with timing offset [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E121], [SH, perfect sync], [FR1], [50M+50M] [timing offset 1ns] |  | 0.46 | 0.26 |
| [Case E122], [SH, perfect sync], [FR1], [50M+50M] [timing offset 5ns] |  | 2.03 | 1.83 |
| [Case E123], [SH, perfect sync], [FR1], [50M+50M] [timing offset 10ns] |  | 5.46 | 5.26 |
| [Case E124], [SH, perfect sync], [FR1], [50M+50M] [timing offset 20ns] |  | 10.05 | 9.85 |
| [Case E125], [DH, perfect sync], [FR1], [50M+50M] [timing offset 1ns] |  | 0.96 | 0.76 |
| [Case E126], [DH, perfect sync], [FR1], [50M+50M] [timing offset 5ns] |  | 3.90 | 3.70 |
| [Case E127], [DH, perfect sync], [FR1], [50M+50M] [timing offset 10ns] |  | 8.34 | 8.14 |
| [Case E128], [DH, perfect sync], [FR1], [50M+50M] [timing offset 20ns] |  | 10.69 | 10.49 |

Table 8.2.1.5-1.7: NR positioning enhancements – horizontal accuracy performance with reduced Rx/Tx timing error and synchronization error [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E115], [SH, sync error 50ns], [FR1], [DL-TDOA] [sync error reduced by differential positioning] |  | 0.11 | Yes |
| [Case E116], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] [Rx/Tx timing error reduced by differential positioning] |  | 0.13 | Yes |
| [Case E117], [SH, sync error 50ns], [FR1], [UL-TDOA] [sync error reduced by UL-TDOA+AOA] |  | 3.16 | 2.96 |
| [Case E118], [SH, perfect sync], [FR1], [UL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] [Rx/Tx timing error reduced by UL-TDOA+AOA] |  | 1.50 | 1.30 |
| [Case E119], [DH, (60%,6,2), perfect sync], [FR1], [machine learning] |  | 4.60 | 4.40 |
| [Case E120], [DH, (60%,6,2), sync error 50ns], [FR1], [machine learning] |  | 5.12 | 4.92 |

Table 8.2.1.5-1.8: NR positioning enhancements – horizontal accuracy performance summary for IOO scenario [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E109], [IOO scenario, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] (Case 63 in [5]) |  | 0.80 | Yes |
| [Case E110], [IOO scenario, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] (Case 64 in [5]) |  | 0.54 | Yes |
| [Case E111], [IOO scenario, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] (Case 65 in [5]) |  | 0.84 | Yes |
| [Case E112], [IOO scenario, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] (Case 66 in [5]) |  | 0.56 | Yes |
| [Case E113], [IOO scenario, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] (Case 67 in [5]) |  | 0.68 | Yes |
| [Case E114], [IOO scenario, perfect sync], [FR2], [Multi-RTT MUSIC, select based on first/median peak] (Case 68 in [5]) |  | 0.50 | Yes |

Table 8.2.1.5-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

Table 8.2.1.5-2: NR positioning enhancements – vertical accuracy performance summary [[5]]

| Simulation case (Vertical Error) | Gain vs Rel16 solution @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [1]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| [Case E-V1], [SH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 0.58 | Yes |
| [Case E-V2], [DH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 0.64 | Yes |
| [Case E-V3], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 1.25 | 0.25 |
| [Case E-V4], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 4.62 | 3.62 |
| [Case E-V5], [SH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 0.66 | Yes |
| [Case E-V6], [DH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 3.16 | 2.16 |
| [Case E-V7], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 1.27 | 0.27 |
| [Case E-V8], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 4.93 | 3.93 |

#### 8.2.1.6	Observations  from source [12]

Table 8.2.1.6-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.6-1: NR positioning enhancements – horizontal accuracy performance summary [12]

| Simulation cases (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If No, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case1, (InF-SH, NLOS classification FR1) | 7.7 | 0.87 | No, 0.67 | No, 0.3 |
| Case2, (InF-SH, NLOS mitigation FR1) | 5.46 | 3.12 | No, 2.9 | No, 2.62 |
| Case3, (InF-SH, NLOS classification + NLOS mitigation FR1) | 8.27 | 0.31 | No, 0.11 | Yes |
| Case4, (InF-DH, NLOS classification FR1) | 2.4 | 12.5 | No, 12.3 | No, 12 |
| Case5, (InF-DH, NLOS mitigation FR1) | 9.4 | 5.49 | No, 5.29 | No, 4.66 |
| Case6, (InF-DH, NLOS classification FR1) | 12.43 | 2.47 | No, 2.27 | No, 1.97 |
| Case7, (InF-SH, Ideal NLOS classification FR1) | 8.27 | 0.23 | No, 0.03 | Yes, |
| Case8, (InF-DH, Ideal NLOS classification FR1) | 14.59 | 0.31 | No, 0.11 | Yes |

#### 8.2.1.7	Observations  from source [10]

Table 8.2.1.7-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.7-1: NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% |
| --- | --- | --- |
| Case 13, InF-SH, FR1, DL-TDOA | 0.48 | 0.37 |
| Case 14, InF-DH, FR1, DL-TDOA | 4.26 | 1.94 |
| Case 15, InF-SH, FR2, DL-TDOA | 0.44 | 0.21 |
| Case 16, InF-DH, FR2, DL-TDOA | 16.41 | 0.89 |
| Case 17, InF-SH, FR1, UL-TDOA | 0.37 | 0.4 |
| Case 18, InF-DH, FR1, UL-TDOA | 1.73 | 4.4 |
| Case 19, InF-SH, FR2, UL-TDOA | 0.66 | 0.24 |
| Case 20, InF-DH, FR2, UL-TDOA | 16.22 | 0.68 |
| Case 21, InF-SH, FR1, Multi-RTT | 0.08 | 0.17 |
| Case 22, InF-DH, FR1, Multi-RTT | 16.00 | 0.3 |
| Case 23, InF-SH, FR2, Multi-RTT | 0.80 | 0.1 |
| Case 24, InF-DH, FR2, Multi-RTT | 7.55 | 0.17 |
| Case 25, InF-SH, FR1, Multi-RTT + vertical AoA | 0.15 | 0.1 |
| Case 26, InF-DH, FR1, Multi-RTT + vertical AoA | 16.11 | 0.19 |
| Case 27, InF-SH, FR2, Multi-RTT + vertical AoA | 0.82 | 0.08 |
| Case 28, InF-DH, FR2, Multi-RTT + vertical AoA | 7.52 | 0.2 |
| Case 33, InF-SH, FR1, Multi-RTT | 0.19 | 0.06 |
| Case 34, InF-DH, FR1, Multi-RTT | 16.19 | 0.11 |
| Case 35, InF-SH, FR2, Multi-RTT | 0.85 | 0.05 |
| Case 36, InF-DH, FR2, Multi-RTT | 7.65 | 0.07 |

Table 8.2.1.7-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

Table 8.2.1.7-2: NR positioning enhancements – vertical accuracy performance summary

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% |
| --- | --- | --- |
| Case 13, InF-SH, FR1, DL-TDOA | 1.60 | 6.9 |
| Case 14, InF-DH, FR1, DL-TDOA | 0.2 | 12.4 |
| Case 15, InF-SH, FR2, DL-TDOA | 6.28 | 6.6 |
| Case 16, InF-DH, FR2, DL-TDOA | 57.60 | 5.8 |
| Case 17, InF-SH, FR1, UL-TDOA | 0.1 | 12.8 |
| Case 18, InF-DH, FR1, UL-TDOA | 0. | 12.9 |
| Case 19, InF-SH, FR2, UL-TDOA | 1.70 | 11.3 |
| Case 20, InF-DH, FR2, UL-TDOA | 62.14 | 0.64 |
| Case 21, InF-SH, FR1, Multi-RTT | 12.21 | 0.89 |
| Case 22, InF-DH, FR1, Multi-RTT | 65.51 | 0.49 |
| Case 23, InF-SH, FR2, Multi-RTT | 5.45 | 1 |
| Case 24, InF-DH, FR2, Multi-RTT | 6.62 | 0.45 |
| Case 25, InF-SH, FR1, Multi-RTT + vertical AoA | 12.83 | 0.27 |
| Case 26, InF-DH, FR1, Multi-RTT + vertical AoA | 65.80 | 0.2 |
| Case 27, InF-SH, FR2, Multi-RTT + vertical AoA | 6.37 | 0.08 |
| Case 28, InF-DH, FR2, Multi-RTT + vertical AoA | 7.01 | 0.06 |
| Case 33, InF-SH, FR1, Multi-RTT | 12.70 | 0.4 |
| Case 34, InF-DH, FR1, Multi-RTT | 65.81 | 0.19 |
| Case 35, InF-SH, FR2, Multi-RTT | 6.06 | 0.39 |
| Case 36, InF-DH, FR2, Multi-RTT | 6.82 | 0.25 |

#### 8.2.1.8	Observations  from source [18]

Table 8.2.1.8-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.8-1: NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case 4- config 220 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.70) | No (0.70) |
| Case 5- config 320 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.45) | Yes (0.45) |
| Case 6- config 420 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.43) | Yes |
| Case 7- config 421 (InF-DH, FR1,UL-TDOA) | N.A. | No(>3.00) | No(>3) | No(>3) |
| Case 8- config 422 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(0.44) | Yes |
| Case 9- config 423 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.59) | No (0.59) |
| Case 10- config 443 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.50) | Yes |
| Case 11- config 444 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.49) | Yes |
| Case 12- config 447 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(0.95) | Yes |
| Case 13- config 552 (InF-DH, FR1,UL-TDOA) | N.A. | No(1.70) | No(1.70) | Yes |
| Case 14- config 554 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | Yes | Yes |

Table 8.2.1.8-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

Table 8.2.1.8-2: NR positioning enhancements – vertical accuracy performance summary

| Simulation case (Vertical Error) | Gain vs Rel16 solution @[90]%, [m] | Commercial vertical accuracy requirements [3]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| Case 4- config 220 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.84) | No(1.84) |
| Case 5- config 320 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.19) | No(1.19) |
| Case 6- config 420 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.12) | No(1.12) |
| Case 7- config 421 (InF-DH, FR1,UL-TDOA) | N.A. | No(>3) | No(>3) | No(>3) |
| Case 8- config 422 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.15) | No(1.15) |
| Case 9- config 423 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.56) | No(1.56) |
| Case 10- config 443 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.12) | No(1.12) |
| Case 11- config 444 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.43) | No(1.43) |
| Case 12- config 447 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(2.23) | No(2.23) |
| Case 13- config 552 (InF-DH, FR1,UL-TDOA) | N.A. | No(>3) | No(>3) | No(>3) |
| Case 14- config 554 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.24) | Yes |

#### 8.2.1.9 Observations  from source [14]

Table 8.2.1.9-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.9-1: NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| [Case 7], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.074 |
| [Case 8], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 0.38 |
| [Case 9], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 1.96 |
| [Case 10], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 2.13 |
| [Case 11], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.31 |
| [Case 12], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | 0.69 |
| [Case 21], [InF-SH], [FR2], [400 MHz], [Comb-4], [6dB PB] | 0.086 |
| [Case 22], [InF-SH], [FR2], [100 MHz], [Comb-4], [6dB PB] | 0.36 |
| [Case 23], [InF-SH], [FR2], [50 MHz], [Comb-4], [6dB PB] | 0.77 |
| [Case 24], [InF-SH], [FR2], [20 MHz], [Comb-4], [6dB PB] | 1.45 |

Table 8.2.1.9-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

Table 8.2.1.9-2: NR positioning enhancements – vertical accuracy performance summary

| Simulation case (Vertical Error) | Accuracy achieved @[90]% |
| --- | --- |
| [Case 13], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB], [ UEH ∈ [0.5, 2] m] | 0.77 |
| [Case 14], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | 1.51 |
| [Case 15], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | 1.34 |
| [Case 16], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | 1.47 |
| [Case 17], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | 1.34 |
| [Case 18], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | 1.73 |
| [Case 19], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | 2.11 |
| [Case 20], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | 2.57 |

#### 8.2.1.10	Observations  from source [21]

Table 8.2.1.10-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.10-1: NR positioning enhancements – horizontal accuracy performance summary [21]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% |
| --- | --- | --- |
| [20] E2, Inf DH, FR1, DL TDOA LOS detection | 9.6m | 8.4m |
| [20] E20, InF SH FR2, UL TDOA multipanel | -0.002 | 0.034039 |
| [21] E21 InF SH FR2, UL TDOA multipanel | 3.81821 | 0.02989 |
| [21] E22 InF SH FR2, DL TDOA multipanel | -0.003 | 0.037037 |
| [21] E23 InF SH FR2, DL TDOA multipanel | 3.7881 | 0.03662 |
| [21] E30, Inf SH, FR1, DL TDOA 1-symbol PRS | -0.06 | 0.54665 |
| [21] E33, IOO, FR1, DL TDOA 1-symbol PRS | 0.02 | 1.404678 |
| [21] E36, Inf SH, FR1, DL TDOA 1-symbol PRS | 0.09 | 0.719702 |
| [21] E39, IOO, FR1, DL TDOA 1-symbol PRS | 0.08 | 1.512598 |
| [21] E42, Inf SH, FR1, DL TDOA 1-symbol PRS | 0.3 | 0.912933 |
| [21] E45, IOO, FR1, DL TDOA 1-symbol PRS | 0.31 | 1.732023 |
| [21] E48, Inf DH, FR1, DL TDOA thresholding | -0.05 | 15.788375 |
| [21] E49, Inf DH, FR1, DL TDOA thresholding | 0,29 | 16.022874 |
| [21] E50, Inf DH, FR1, DL TDOA thresholding | -1.25 | 16.984019 |
| [21] E51, Inf DH, FR1, DL TDOA thresholding | -1.68 | 17.412632 |
| [21] E52, Inf DH, FR1, DL TDOA thresholding | -4.05 | 19.787138 |
| [21] E53, Inf DH, FR1, DL TDOA thresholding | -6.88 | 22.610367 |
| [21] E54, Inf SH, FR1, DL TDOA SRS CS unfolding (PAPR preservation) | 13.4 | 2.52 |
| [21] E55, IOO, FR1, DL TDOA SRS CS unfolding (corr optimization) | 12.98 | 3.04 |
| [21] E58, Inf DH, FR1, DL TDOA PRS aggregation | 1.09 | 13.91966 |
| [21] E59, Inf DH, FR1, DL TDOA PRS aggregation | -0.005 | 15.01508 |
| [21] E61, Inf SH, FR1, DL TDOA PRS aggregation | 0.1798 | 0.079559 |
| [21] E62, Inf SH, FR1, DL TDOA PRS aggregation | -0.016 | 0.275798 |
| [21] E63, Inf SH, FR1, DL TDOA PRS aggregation | -1.18 | 1.447441 |

Table 8.2.1.10-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

#### 8.2.1.11	Results from [17]

Table 8.2.1.11-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.11-1: NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- |
| Case 22, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | 0.5 |
| Case 22, InH, FR1, Phase offset = 2 degrees, Frequency-domain Stitching | 0.5 |
| Case 22, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | 0.5 |
| Case 22, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | 1.5 |
| Case 22, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | 1.5 |
| Case 23, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | Yes |
| Case 23, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | 1 |
| Case 23, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | 1.4 |
| Case 23, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | 1.5 |
| Case 24, InH, FR1, Channel Spacing = 0 MHz, Frequency-domain Stitching | 2 |
| Case 24, InH, FR1, Channel Spacing = 7.2 MHz, Frequency-domain Stitching | 3 |
| Case 24, InH, FR1, Channel Spacing = 14.4 MHz, Frequency-domain Stitching | 4 |
| Case 24, InH, FR1, Channel Spacing = 36 MHz, Frequency-domain Stitching | 4.5 |
| Case 25, UMI, FR1,Phase offset = 0 degrees, Frequency-domain Stitching | Yes |
| Case 25, UMI, FR1,Phase offset = 4 degrees, Frequency-domain Stitching | 0.2 |
| Case 25, UMI, FR1,Phase offset = 8 degrees, Frequency-domain Stitching | 0.5 |
| Case 25, UMI, FR1,Phase offset = 12 degrees, Frequency-domain Stitching | 1.2 |
| Case 25, UMI, FR1,Phase offset = 18 degrees, Frequency-domain Stitching | 3 |
| Case 26, UMI, FR1,Time offset = 0 ns, Frequency-domain Stitching | Yes |
| Case 26, UMI, FR1,Time offset = 0.5 ns, Frequency-domain Stitching | Yes |
| Case 26, UMI, FR1,Time offset = 1 ns, Frequency-domain Stitching | Yes |
| Case 26, UMI, FR1,Time offset = 2 ns, Frequency-domain Stitching | 0.5 |
| Case 26, UMI, FR1,Time offset = 5 ns, Frequency-domain Stitching | 0.5 |

| Simulation case (Horizontal Error) | Phase Shift between the two aggregated component carriers | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps [m] |
| --- | --- | --- | --- | --- |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | 0 degrees | 23.4 | 24.2 | 23.9 |
|  | 45 degrees | 24.6 | 25.4 | 25.1 |
|  | 90 degrees | 24.5 | 25.3 | 25 |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap | 0 degrees | 24.7 | 25.5 | 25.2 |
|  | 45 degrees | 24.4 | 25.2 | 24.9 |
|  | 90 degrees | 27.9 | 28.7 | 28.4 |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | 0 degrees | 2.4 | 3.2 | 2.9 |
|  | 45 degrees | 1.9 | 2.7 | 2.4 |
|  | 90 degrees | 2.4 | 3.2 | 2.9 |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap | 0 degrees | 3.25 | 4.05 | 3.75 |
|  | 45 degrees | 3.26 | 4.06 | 3.76 |
|  | 90 degrees | 3.43 | 4.23 | 3.93 |

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| 28 2*200MHz Perfect phase | Yes. | Yes. | Yes. |
| 28 200MHZ(baseline) | Yes. | Yes. | Yes. |
| 29 2*200MHz Perfect phase | 63% | 62% | 62% |
| 29 200MHZ(baseline) | 62% | 61% | 61% |
| 30 2*200MHz Perfect phase | Yes. | 78% | 83% |
| 30 200MHZ(baseline) | Yes. | 66% | 77% |
| 31 2*200MHz Perfect phase | Yes. | Yes. | Yes. |
| 31 200MHZ(baseline) | Yes. | Yes. | Yes. |
| 32 2*200MHz Perfect time | Yes. | 78% | 83% |
| 32 200MHZ(baseline) | Yes. | 66% | 77% |
| 33 2*200MHz Perfect time | Yes. | Yes. | Yes. |
| 33 200MHZ(baseline) | Yes. | Yes. | Yes. |

| Case ID | Kinematic constraint condition | 90% |
| --- | --- | --- |
| Case 34 InF-SH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.049 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.024 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.024 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.031 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.036 |
| Case 35 InF-DH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.058 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.029 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.034 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.038 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.044 |
| Case 36 InH, FR2 DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.071 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.026 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.028 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.044 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.053 |

| Horizontal Positioning Error | Gain vs Rel.16 solution, @[90]%, [m] | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| Case 37, UMI, FR1,RTT, With Delta Tau, Perfect Sync, No Timing Errors, AoA & UL PDP Enhancement | 3.4 | 2.9 |
| Case 38, UMI, FR1,DL-TDOA, With Δτ, Perfect Sync, No Timing Errors, UL PDP Enhancement | 1 | 7 |
| Case 39, UMI, FR1, RTT, With Delta Tau, Perfect Sync, No Timing Errors, UL PDP Enhancement | 4.3 | 3.7 |

#### 8.2.1.12	Results from [9]

No observation found

### 8.2.2	Physical layer latency analysis for NR positioning enhancements

#### 8.2.2.1	Observations  from source [4]

Observations on NR positioning latency enhancements are provided in Table 8.2.2.1-1.

Table 8.2.2.1-1: NR positioning enhancements - physical layer latency performance summary [4]

| Description Evaluation Case | L1 Latency,ms | Gain over R16, ms | Commercial requirements [100]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [10]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| Case L101, UL E-CID w/ measurements available | 6-26ms | 0 | Yes | Yes/No | Yes |
| Case L102, UL E-CID w/o measurements available | 46-53.5ms | Negative | Yes | No | Yes |

#### 8.2.2.2	Observations  from source [8]

Table 8.2.2.2-1: NR positioning enhancements - physical layer latency performance summary [8]

| Description Evaluation Case | L1 Latency, ms | Gain over R16, ms | Commercial requirements [100]ms are met Yes/No. If No, provide performance gaps | IIoT requirements of [10]ms are met Yes/No. If No, provide performance gaps | IIoT requirements of [100]ms are met Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| Case 1, 15kHz, FR1, DL-TDOA | 13.5 | 38 | Yes | No (3.5ms gap) | Yes |

#### 8.2.2.3	Observations  from source [5]

Observations on NR positioning latency enhancements are provided in Table 8.2.2.3-1.

Table 8.2.2.3-1: NR positioning enhancements - physical layer latency performance summary [5]

| Description Evaluation Case | L1 Latency,ms | Gain over R16, ms | Commercial requirements [100]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [10]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| [Case 7], [on-demand/aperiodic PRS] | 44.5ms~ | 19.5ms~ |  | 34.5ms~ |  |
| [Case 8], [on-demand/aperiodic MG] | 27.5ms~ | 36.5ms~ |  | 17.5ms~ |  |
| [Case 9], [Positioning BWP] | 28.5ms~ | 35.5ms~ |  | 18.5ms~ |  |
| [Case 10], [physical layer triggered] | 44ms~ | 20ms~ |  | 32ms~ |  |
| [Case 11], [combination scheme] | 5ms~ | 59ms~ |  | Yes |  |
| [Case 12] [idle/inactive] | 27.3ms~ | 58ms or 76.7ms |  | 17.3ms |  |

#### 8.2.2.4	Observations  from source [10]

Observations on NR positioning latency enhancements are provided in Table 8.2.2.4-1.

Table 8.2.2.4-1: NR positioning enhancements - physical layer latency performance summary

| Description Evaluation Case | L1 Latency, ms | Gain over R16, ms |
| --- | --- | --- |
| Case 4, InF, FR1, R17, DL-TDOA/DL-AoD | 3.8839 (L1 components) + 4 (L2/L3 components) = 8 (total) | 0.6875 (L1 components) [32] (L2/L3 components) 121.07 (total) |
| Case 5, InF, FR1, R17, UL-TDOA/UL-AoA | 1.9018 (L1 components) + 3 (L2/L3 components) = 5 (total) | 0.8660 (L1 components) [13] (L2/L3 components) 13.7678 (total) |
| Case 6, InF, FR1, R17, Multi-RTT | 4.1875 (L1 components) + 4 (L2/L3 components) = 8.2 (total) | 3.1518 (L1 components) [41] (L2/L3 components) 132.6393 (total) |

#### 8.2.2.5 Observations  from source [16]

The latency analysis for each enhancement scenario is summarized in the following table:

| Description Evaluation Case | L1 Latency, ms | Gain over R16 (UE-assisted DL methods (Case 1)), ms |
| --- | --- | --- |
| No measurement gap DL methods | 16 | 17 |
| Measurement gap activation/deactivation DL methods | 24 | 09 |
| On-demand DL PRS methods | 22 | 11 |

#### 8.2.2.6	Observations from source [17]

Observations on NR positioning latency enhancements are provided in Table 8.2.2.1.6-1.

Table 8.2.2.6-1: NR positioning enhancements - physical layer latency performance summary

| Description Evaluation Case | Gain over R16, ms |
| --- | --- |
| Support Low-layer (e.g., unicast/group-common DCI, MAC-CE) triggering of DL/UL PRS transmission/muting/Location-Request for DL-only and DL/UL methods. | 10 msec |
| Support DCI/MAC-CE triggering of Measurement gaps (MG) for the purpose of positioning measurements | >30 msec |
| Fast/real-time processing of short PRS instances: Support Enhanced PRS processing capabilities Support partially-staggered or no-staggered DL-PRS transmissions | >1.5 msec |
| Support Low-layer (e.g. UL MAC-CE or UCI) Measurement Reporting towards the serving gNB | 2 msec |

#### 8.2.2.7 Observations  from source [15]

Summary of latency performance analysis is provided in Table 8.2.2.7-1.

Table 8.2.2.7-1: NR positioning enhancements – latency analysis [15]

| Description Evaluation Case | Assumptions | L1 Latency, ms (including preparation/processing time at higher layer) | L1 Latency, ms (excluding preparation/processing time at higher layer) |
| --- | --- | --- | --- |
| Case 1, DL-TDOA, DL-AOD [NW initiated, UE-A] | FR1, 15kHz # of symbols for PUSCH: 1~14 OS # of symbols for PDSCH: 2~14 OS # of symbols for SRS: 2~12 OS -Uplink switching gap is not configured. -No BWP switching -No overlapping symbols of the PUCCH and the scheduled PUSCH -No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH *Note: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed. | For UE capability-1: 73.12+[X1]+[X2]ms ~ 217.12+[X1]+ [X2]ms For UE capability-2: 71.68+[X1]+[X2]ms ~ 215.26+[X1]+ [X2]ms | For UE capability-1: 23.12ms ~ 167.12ms For UE capability-2: 21.68ms ~ 165.26ms |
| Case 2, DL-TDOA, DL-AOD [UE initiated, UE-A] |  | For UE capability-1: 62.41+[X1]+[X2]ms ~ 205.34+[X1] +[X2]ms For UE capability-2: 61.33+[X1]+[X2]ms ~ 204.05+[X1] +[X2]ms | For UE capability-1: 22.41ms ~ 165.34ms For UE capability-2: 22.33ms ~ 164.05ms |
| Case 3, UL-TDOA, UL-AOA [NW initiated, UE-A] |  | For UE capability-1: 31.49+[X3] ms ~ 34.42+[X3] ms For UE capability-2: 30.77+[X3] ms ~ 33.28+[X3] ms | For UE capability-1: 1.49ms ~ 4.42ms For UE capability-2: 0.77ms ~ 3.28ms |
| Case 4, Multi-RTT [NW initiated, UE-A] |  | For UE capability-1: 145.34+[X1]+[X2]+[X3]ms ~ 293.32+[X1]+[X2]+[X3]ms For UE capability-2: 142.8+[X1]+[X2]+[X3]ms ~ 289.75+[X1]+[X2]+[X3] ms | For UE capability-1: 25.34ms ~ 173.32ms For UE capability-2: 22.8ms ~169.75ms |
| Case 5, E-CID [NW initiated, UE-A] |  | For UE capability-1: 21.56+[X2]ms ~ 23.56+[X2] ms For UE capability-2: 20.84+[X2] ms~22.63+[X2] ms | For UE capability-1: 1.56ms ~ 3.56ms For UE capability-2: 0.84ms~2.63ms |

## 8.3	Efficiency analysis for NR positioning enhancements

In this report, Network efficiency and UE efficiency is evaluated either via analytically or via simulations.

### 8.3.1	Network efficiency analysis for NR positioning enhancements

#### 8.3.1.1	Observations  from source [4]

The single PRS symbol transmission achieves almost the same accuracy with that of multiple PRS symbols for InF scenarios, yet with reduced network overhead.

E-CID can achieve similar performance compared to Multi-RTT for InF-DH scenarios, yet without any additional network overhead compared to communication.

#### 8.3.1.2	Observations  from source [8]

Although the carrier phase positioning (CPP) scheme needs to measure the carrier phase to calculate the UE position, considering Rel.16 DL-PRS or UL-SRS can be reused to complete the carrier phase measurement, there is no increase in terms of reference signal resource overhead as the metric for network efficiency, compared with Rel.16 positioning solutions. It is observed that NR positioning enhancements with carrier phase measurements has no impact on UE and network RF resource usage efficiency.

#### 8.3.1.3	Observations  from source [5]

Companies are invited to present the observations/results based on their evaluation/analysis of network efficiency for NR positioning enhancements.

Observation

The network and device efficiency will be reduced by on-demand PRS within the same level latency compared to periodic PRS.

Observation

The network and device efficiency of aperiodic PRS is multiple of the number of activations.

#### 8.3.1.4	Observations  from source [12]

Observation: measurement gap is the major contribution for PRS resource utilization.

#### 8.3.1.5	Observations  from source [21]

No observation found

#### 8.3.1.6 Observations  from source [18]

Observation : 	Cyclic shift enhancements SRS may improve the network efficiency for UMi scenarios by a factor of 4-8

Observation : 	Multi-port SRS transmission may improve the network efficiency in terms of SRS resource compared to Rel-16 utilization by a factor:

- 4 in terms of SRS resource utilization for 4 ports SRS

- 2 in terms of SRS resource utilization for 2 ports SRS

### 8.3.2	UE efficiency analysis for NR positioning enhancements

#### 8.3.2.1	Observations  from source [4]

Observations on NR positioning UE efficiency enhancements are provided in Table 8.3.2.1-1.

Table 8.3.2.1-1: NR positioning enhancements – UE efficiency summary [4]

| Description Evaluation Case | Power consumption | Power saved |
| --- | --- | --- |
| Case P1: IDLE/INACTIVE state in every 1.28s | 26392 | 7.2% to Case P2 30.4% to Case P3 |
| Case P2: CONNECTED state inside on-duration in every 1.28s | 28432 | - |
| Case P3: CONNECTED state outside on-duration in every 1.28s | 37936 | - |
| Case P4: IDLE/INACTIVE state in every 1.28s | 11910 | 39.6% to Case P5 |
| Case P5: CONNECTED state outside on-duration in every 1.28s | 19712 | - |

#### 8.3.2.2	Observations  from source [5]

Observations of UE efficiency for power consumption

Table 8.3.2.2-1: NR positioning enhancements – power consumption performance in connected state [5]

| Power saving scheme description | Average power consumption (power unit) | Power reduction compared to baseline | Note |
| --- | --- | --- | --- |
| PRS measurement impacted by DRX | 53.5625(baseline) 2 PRS occasions every DRX cycle(160ms) | - | In this case, PRS period=80ms DRX cycle=160ms If PRS measurement is impacted by DRX, UE is only expected to measure PRS in DRX active time. |
|  | 35.2500 1 PRS occasion every DRX cycle (160ms) | 34.19% |  |
| Extending PRS period | 35.2500(baseline) PRS period=160ms | - | - |
|  | 27.4844 PRS period=320ms | 22.03% |  |
|  | 23.6016 PRS period=640ms | 33.05% |  |
| Concentrated PRS distribution | 43.3937(baseline) 4 distributed PRS occasion every 160ms | - | In this case, the duration of concentrated PRS distribution is 5ms with 4ms PRS length and 1ms MG switching time. While for distributed PRS, we divide the concentrated PRS occasion of 4ms (baseline) into 4 PRS occasions with 1ms, and the adjacent PRS occasions are separated by 40ms. |
|  | 35.2500 1 concentrated PRS occasion every 160ms | 18.77% |  |
| Adding PRS-MTC window | 35.2500(baseline) without PRS-MTC PRS occasion duration=4ms | - | - |
|  | 28.0313 PRS-MTC to limit PRS measurement PRS occasion duration=2ms | 20.48% | - |
| Reducing number of TRPs to be measured | 35.2500(baseline) Number of TRPs=8 | - | - |
|  | 32.2500 Number of TRPs=4 | 8.51% |  |
| Reducing number of positioning frequency layers to be measured | 82.4688 (baseline) Number of FLs=4 | - | - |
|  | 52.0313 Number of FLs=2 | 36.91% |  |
|  | 35.2500 Number of FLs=1 | 57.26% |  |

Table 8.3.2.2-2: NR positioning enhancements – power consumption comparison in idle state and connected state [5]

| Power saving scheme description | Average power consumption (power unit) | Power reduction compared to baseline | Additional assumptions | Note |
| --- | --- | --- | --- | --- |
| 1. Connected state measurement and report | 11.1367(baseline) | - | 1. UE starts positioning from idle state 2.LPP/RRC procedures for positioning are ignored. 3. Only one shot positioning measurement and report considered. 4.Once the positioning report is completed, the RRC connection is released 5. Measurement/report cycle is equal to idle state (1280ms). 6. The power unit for PRS measurement in connected state is equal to PRS bandwidth in idle state 7. Paging occasion power is equivalent to ‘PDCCH+PDSCH’, considering it may lead to RRC state transition | Considering that some assumptions are made to simplify power consumption evaluation, such as ignoring complicated steps for LPP procedures, aligning the bandwidth and period with idle state measurement, it will consume more power when positioning in the connected state in general. |
| 2. Idle state measurement and connected state report | 10.3246 | 7.29% | UE switches to connected mode to report. Once the positioning report is completed, the RRC connection is released |  |
| 3. Idle state measurement and idle state report | 5.7488 | 48.38% | - |  |

observation for UE overhead

The UE efficiency will be reduced by on-demand PRS with concentrated PRS compared to periodic PRS.

The UE efficiency will be reduced by Positioning BWP Without MG compared to periodic PRS.

## 8.4	Summary of performance evaluations

Performance analysis of baseline I-IoT InF scenarios shows that InF-SH scenario (Scenario 1) is characterized by high probability of LOS links. In InF-DH (Scenario 2) the probability of LOS links is reduced substantially while probability of NLOS links is increased accordingly.

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the InF-SH scenario (Scenario 1).

    - Based on the results provided by a majority of sources, sub-meter level @ 90% of horizontal positioning accuracy is achieved by Rel.16 solutions.

    - For horizontal accuracy, results were provided by [12] out of [17] sources for FR1 and by [9] sources out of [17] for FR2

- For NR positioning evaluations in FR1 band, the following is observed with respect to horizontal positioning accuracy:

  - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [3] sources  and is not achieved in contributions from [9] sources

  - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [7] sources and is not achieved in contributions from [5] sources

- For NR positioning evaluations in FR2 band, the following is observed with respect to horizontal positioning accuracy:

  - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [7] sources and is not achieved in contributions from [2] sources

  - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [9] sources and is not achieved in contributions from [0] sources

    - For vertical accuracy, results were provided by [4] sources ([7], [8], [5], [10]) out of [17] for FR1 and by [4] sources ([7], [8], [17], [10]) out of [17] for FR2 band

- For NR positioning evaluations in FR1 band, the following is observed with respect to vertical positioning accuracy:

  - Accuracy of ≤ 1m @ 90% is achieved in contribution from [2] sources ([7], [5]) and is not achieved from [2] sources ([8], [10])

- For NR positioning evaluations in FR2 band, the following is observed with respect to vertical positioning accuracy:

  - Accuracy of ≤ 1m @ 90% is achieved in contribution from [4] sources ([7], [8], [17], [10]) [and is not achieved by [0] sources]

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the baseline InF-DH scenario (Scenario 2), including evaluations with variable gNB/UE heights for vertical accuracy

- Based on the results provided by a majority of sources, sub-meter level @ 90% of horizontal positioning accuracy is not achieved by Rel.16 based solutions.

- For horizontal accuracy, results were provided by [14] sources ([4], [6], [7], [8], [9], [12], [13], [14], [19], [20], [17], [5], [10], [18]) out of [17] for FR1 and by [9] sources ([4], [6], [7], [8], [14], [20], [17], [5], [10]) out of [17] for FR2

- For NR positioning evaluations in FR1 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 0.2m @ 90% is achieved in contribution from [2] sources ([8],[17]) and is not achieved in contributions from [13] sources ([4], [6], [7], [9], [12], [13], [14], [19], [20], [17], [5], [10], [18])

- Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [4] sources ([6], [8], [17], [5]) and is not achieved in contributions from [10] sources ([4], [7], [9], [12], [13], [14], [19], [20], [10], [18])

- For NR positioning evaluations in FR2 band, the following is observed with respect to horizontal positioning accuracy:

  - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [3] sources ([6], [17], [5]) and is not achieved in contributions from [6] sources ([4], [7], [8], [14], [20], [10])

  - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [3] sources ([6], [17], [5]) and is not achieved in contributions from [6] sources ([4], [7], [8], [14], [20], [10])

    - For vertical accuracy, results were provided by [6] sources ([7], [8], [5], [10], [4], [18]) out of [17] for FR1 and by [4] sources ([7], [8], [10], [4]) out of [17] for FR2 band

- For NR positioning evaluations in FR1 band, the following is observed with respect to vertical positioning accuracy:

  - Accuracy of ≤ 1m @ 90% is achieved in contribution from [2] sources ([8], [5]) and is not achieved from [4] sources ([7], [10], [4], [18])

- For NR positioning evaluations in FR2 band, the following is observed with respect to vertical positioning accuracy:

  - Accuracy of ≤ 1m @ 90% is achieved in contribution from [1] source ([4]) and is not achieved from [3] sources ([7], [8], [10])

For the issues related to mitigating effects of multipath/NLOS for positioning

- Evaluation results for LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms in indoor factory scenarios were provided by [12] sources ([12], [9], [5], [10], [17], [7], [4], [19], [13], [14], [18], [20]) out of [17] sources

- NR positioning utilizing LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms improve performance of positioning accuracy with respect to solutions that do not apply these techniques

- From the evaluations,

  - [9] sources ([9], [10], [7], [4], [19], [13], [14], [18], [20]) evaluated LOS/NLOS identification with additional specification changes relative to Rel.16 solutions

  - [2] sources ([5], [17]) evaluated outlier rejection algorithm (implementation-based algorithm that can be applied for Rel.16 solutions without specification changes)

  - [1] source ([12]) evaluated NLOS mitigation using triangle-based inequality algorithm (implementation-based algorithm that can be applied for Rel.16 solutions without specification changes)

- Comparative analysis of LOS/NLOS identification with specification changes vs implementation based methods (outlier rejection algorithms) was done by 6 sources ([10], [4], [5], [17], [7], [12])

  - Three sources ([10], [4], [7]) observe that NR positioning based on LOS/NLOS identification outperforms NR positioning utilizing outlier rejection

  - Three sources ([5], [17], [12]) observe that NR positioning utilizing outlier rejection outperforms NR positioning utilizing LOS/NLOS identification

For issues related to gNB/UE TX/RX timing errors

- Evaluation results of gNB/UE TX/RX timing errors (as per the optional model) are provided by [7] sources ([4], [7], [17], [10], [8], [20], [5]) out of [17] sources)

- Summary of results is provided in tables B.1-1 to B.1-4

For the issues related to aggregation of DL positioning frequency layers:

- Evaluation results for aggregation of DL positioning frequency layers were provided by [5] sources ([10], [17], [4], [5], [20]) out of [17].

- Aggregation of NR positioning frequency layers improves positioning accuracy under certain scenarios, configurations, and assumptions on modelled impairments such as: bandwidth and spacing of aggregated layers, timing offset and frequency offset over frequency layers, phase discontinuity and possible amplitude imbalance.

  - One source ([4]) observes that aggregation with phase continuity can help to improve the positioning accuracy, and discontinuous aggregation can approach the performance of contiguous aggregation with the same frequency span

  - One source ([10]) has shown that  aggregation of frequency layers (without modeling impairements) improves the positioning accuracy for intra-band contiguous configuration and that further study is needed for other cases including impairments

  - One source ([20]) has observed that PRS aggregation shows potential gains without modeling phase error, but these gains are lost when the phase error between CCs becomes too large

  - One source ([17]) has analyzed aggregation of 2 and 4 frequency layers for different channel spacings, time and phase offset across frequency layers

  - One source ([5] R1-2007666) has analyzed aggregation of 2 frequency layers for different time offset values and observed that:

- For the case without impairements modeling, aggregation of multiple DL positioning frequency layers 50MHz+50MHz, performance target [0.2m @ 90%] cannot be achieved in both InF-SH and InF-DH.

- For the case without impairements modeling, aggregation of multiple DL positioning frequency layers 50MHz+50MHz, the performance is worse than 100MHz but better than 50MHz.

- The performance of aggregation of frequency layers degrades if timing offset is increased

For issues related to physical layer latency for Rel.16 DL-TDOA/DL-AOD

- Summary of results is provided in table B.2-1

- Summary of physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning in FR1 was provided by [11] sources

- Summary of physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning in FR2 was provided by [5] sources

- For evaluation in FR1,

  - results from [11] sources out of [11] sources ([17], [4], [7], [5], [11], [15], [8], [13], [12], [16], [10]) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 10ms

  - results from [2] ([7], [10]) sources out of [11] sources ([17], [4], [7], [5], [11], [15], [8], [13], [12], [16], [10]) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 100ms

- For evaluation in FR2,

  - results from [5] sources out of [5] sources ([7], [5], [11], [12],[13]) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 10ms

  - results from [2] ([11],[7], [5]) sources out of [4] sources ([7], [5], [11], [12]) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 100ms

- The following list provides the major physical layer latency components for Rel.16 DL TDOA/DL-AOD UE-assisted NR Positioning

  - DL PRS alignment, transmission, measurement (including processing time) and report delay

  - Measurement gap request, configuration and alignment time

  - UE/gNB higher layer (LPP/RRC) processing times

For issues related to physical layer latency for Rel.16 UL-TDOA/UL-AOA

- Summary of results is provided in table B.2-2

- Summary of physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning in FR1 was provided by [8] sources ([4], [5], [15], [8], [13], [12], [16], [10])

- Summary of physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning in FR2 was provided by [2] sources ([5], [12])

- For evaluation in FR1,

  - results from [3] sources ([4], [8], [13]) out of [8] sources ([4], [5], [15], [8], [13], [12], [16], [10]) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 10ms

  - results from [8] sources out of [8] sources ([4], [5], [15], [8], [13], [12], [16], [10]) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 100ms

- For evaluation in FR2,

  - results from [2] sources out of [2] sources ([5], [12]) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning exceeds 10ms

  - results from [1] ([12]) sources out of [2] sources ([5], [12]) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 UL-TDOA/UL-AOA NR Positioning

  - SRS for positioning processing time

  - SRS for positioning alignment time (depends on periodic or aperiodic SRS for positioning)

  - gNB higher layer processing delays (RRC/ NRPPa processing times)

For issues related to physical layer latency for Rel.16 Multi-RTT

- Summary of results is provided in table B.2-3

- Summary of physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning in FR1 was provided by [6] sources ([17], [4], [5], [15], [16], [10])

- Summary of physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning in FR2 was provided by [0] sources

- For evaluation in FR1,

  - results from [6] sources ([17], [4], [5], [15], [16], [10]) out of [6] sources ([17], [4], [5], [15], [16], [10]) show that minimum estimated physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning exceeds 10ms

  - results from [4] sources ([17], [4], [5], [16]) out of [6] sources ([17], [4], [5], [15], [16], [10]) show that minimum estimated physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 Multi-RTT UE-assisted NR positioning

  - DL PRS alignment, transmission, measurement time and report delay

  - Measurement gap request, configuration, alignment time

  - SRS for positioning processing time

  - SRS for positioning alignment time (depends on periodic or aperiodic SRS for positioning)

  - UE/gNB higher layer (LPP/RRC/NRPPa) processing times

For issues related to physical layer latency for Rel.16 E-CID NR positioning

- Summary of results is provided in table B.2-4

- Summary of physical layer latency for Rel.16 E-CID NR positioning in FR1 was provided by [3] sources ([4], [7], [15])

- Summary of physical layer latency for Rel.16 E-CID NR positioning in FR2 was provided by [0] sources

- For evaluation in FR1,

  - results from [2] sources ([7], [15]) out of [3] sources ([4], [7], [15]) show that minimum estimated physical layer latency for Rel.16 E-CID NR positioning exceeds 10ms

  - results from [3] sources ([4], [7], [15]) out of [3] sources ([4], [7], [15]) show that minimum estimated physical layer latency for Rel.16 E-CID NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 E-CID NR positioning

  - Higher layer signaling processing

For issues related to physical layer latency for Rel.16 DL-only UE-based NR positioning

- Summary of results is provided in table B.2-5

- Summary of physical layer latency for Rel.16 DL-only UE-based NR positioning in FR1 was provided by [6] sources ([17], [4], [5], [11], [12], [16])

- Summary of physical layer latency for Rel.16 DL-only UE-based NR positioning in FR2 was provided by [2] sources ([5], [11])

- For evaluation in FR1,

  - results from [4] sources ([4], [5], [12], [16]) out of [6] sources ([17], [4], [5], [11], [12], [16]) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 10ms

  - results from [6] sources out of [6] sources ([17], [4], [5], [11], [12], [16]) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning does not exceed 100ms

- For evaluation in FR2,

  - results from [2] sources out of [2] sources ([5], [11]) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 10ms

  - results from [1] ([5]) sources out of [2] sources ([5], [11]) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 100ms

- The following list provides the major physical layer latency components for Rel.16 DL-only UE-based NR positioning

  - DL PRS alignment, transmission, measurement time and, if requested, report delay

  - Measurement gap request, configuration, alignment time

  - Higher layer (LPP/RRC) processing times

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMa scenario

- Based on the results provided, 10 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMa scenario

- Results were provided by [2] sources ([20], [17]) out of [17] for FR1 band

- For NR positioning evaluations for UMa scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 80% is achieved for the outdoor UEs in contributions from [1] source ([17]) out of [2] sources ([20], [17]) in the scenario without absolute time of arrival modelling. Zero sources met an accuracy of ≤ 1m @ 90%.

- Accuracy of ≤ 10m @ 90% is achieved for the outdoor UEs in contributions from [2] sources ([20], [17]) out of [2] sources in the scenario without absolute time of arrival modelling

- Accuracy of ≤ 10m @ 90% is achieved for the indoor UEs in contributions from [1] source ([20]) out of [2] sources in the scenario without absolute time of arrival modelling

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMi scenario

- Results were provided by [4] sources ([13], [20], [17], [18]) out of [17] for FR1 band

- For NR positioning evaluations for UMi scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 90% is achieved in contributions from [2] sources ([20], [17]) and is not achieved from [2] sources ([13], [18]) in the scenario without absolute time of arrival modelling

- Accuracy of ≤ 1m @ 90% is not achieved from [2] sources ([17], [18]) in a scenario with absolute time of arrival modelling

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the IOO scenario

- Based on the results provided by a majority of the sources, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in IOO scenario

- Results were provided by [5] sources ([8], [13], [14], [20], [5]) out of [17] for FR1 and [5] sources ([8], [14], [20], [17], [5]) out of [17] for FR2 band

- For NR positioning evaluations for IOO scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 90% is achieved in contributions from [4] sources ([8], [14], [20], [5]) and is not achieved from [1] source ([13]) in the scenario without absolute time of arrival modelling

- Accuracy of ≤ 1m @ 90% is achieved from [1] source ([5]) in a scenario with absolute time of arrival modelling

- For NR positioning evaluations for IOO scenario in FR2 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 90% is achieved in contributions from [5] sources ([8], [14], [20], [17], [5]) in the scenario without absolute time of arrival modelling

- Accuracy of ≤ 1m @ 90% is achieved from [1] source ([5]) in a scenario with absolute time of arrival modelling

The results for the UE efficiency (power saving) in the RRC_IDLE/RRC_INACTIVE states were analyzed by 2 sources ([4], [5]) out of 17 sources (assumptions may be different between the different sources)

- In one source ([4]), the following observations were made:

  - RRC_IDLE/RRC_INACTIVE state positioning can save about 7%-40% power consumption compared to C-DRX configuration

- In one source ([5]), the following observations were made:

  - Positioning report in the RRC_IDLE state can provide 44.32 % of power saving gain compared to the report in the RRC_CONNECTED state

  - Positioning measurement and report in the RRC_IDLE state can provide at least 48.38 % of power saving gain compared to the measurement and report in the RRC_CONNECTED state

# 9	Positioning integrity and reliability

From objective 2: Includes solutions necessary to support integrity and reliability of assistance data and position information:

# 10	Identified NR impacts in Rel-17

## 10.1 	NR positioning for UEs in RRC_INACTIVE state

NR positioning for UEs in RRC_INACTIVE state is recommended for normative work, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

The details of how to enable the UE positioning in RRC_ INACTIVE state can be further discussed during normative work. These details may include, but are not limited to the following aspects:

  - UL reference signals (e.g., SRS for positioning, PRACH preambles) for UL measurements

  - Signalling and procedures for support the assistance data delivery, DL-PRS configuration, UL reference signals for positioning resource configuration, measurement reporting), which may be developed based on the enhancements of existing signalling and procedures (e.g., existing 2-step and/or 4-step PRACH procedures, paging procedure, small data transmission).

## 10.2 On-demand transmission and reception of DL PRS

From a physical layer perspective, on-demand transmission and reception of DL PRS, which includes at least the following is recommended

- UE-initiated request of on-demand DL PRS transmission

- LMF (network)-initiated request of on-demand DL PRS transmission

- Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

## 10.3 Aggregation of DL PRS resources

Simultaneous transmission by the gNB and reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs can be studied further and if needed, specified during normative work

- From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, bands and RF architectures, can be further studied

## 10.4 Aggregation of SRS for positioning resources

Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers can be studied further and if needed, specified during normative work.

- From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, particular bands and RF architectures, can be further studied.

## 10.5 Enhancements for  UE Rx/Tx and gNB Rx/Tx timing delays

The methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or and gNB Rx/Tx timing delays are recommended for normative work, including

- DL, UL and DL+UL positioning methods

- UE-based and UE-assisted positioning solutions

- Note: The details of the solutions are left for further discussion in normative work.

## 10.6 Enhancements for angle based methods

The enhancements of the procedure, measurements, reporting, and signalling for improving the accuracy of

- UL AoA is recommended for normative work for network-based positioning solutions.

- DL-AoD is recommended for normative work for UE-based and network-based (including UE-assisted) positioning solutions.

## 10.7 Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation

Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation can be studied further, and if needed, specified during normative work for improving positioning accuracy.

- Note: The details of the enhancements of reporting are left for further discussion in normative work, which may include, but are not limited to the following information associated with multi-path, e.g., LOS/NLOS identification, time of arrival of the multi-path components, signal power and/or relative power, power delay profile, angle, and/or polarization information, coherence bandwidth, etc.

## 10.8 Aperiodic reception of DL PRS

- Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work.

- Note: Aperiodic reception would correspond to DCI-triggered reception

## 10.9 Semi-persistent reception of DL PRS

- Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work.

- Note: Semi-persistent reception in the above corresponds to MAC-CE activated reception

# 11	Conclusions

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-05 | RAN1#101-e | R1-2004948 |  |  |  | Baseline TR skeleton. | 0.0.1 |
| 2020-10 | RAN1#103-e | R1-2009430 |  |  |  | Update of TR based on RAN1#101-e and RAN1#102-e agreements. | 0.1.0 |
| 2020-10 | RAN1#103-e | R1- 2009544 |  |  |  | Update of TR based on RAN1#103-e agreements. | 0.1.1 |
| 2020-11 | RAN1#103-e | R1- 2009670 |  |  |  | Update of TR based on RAN1#103-e agreements. | 0.2.0 |
| 2020-11 | RAN1#103-e | R1- 200NNNN |  |  |  | Update of TR based on RAN1#103-e agreements. | 0.2.1 |

Annex B:
Appendix to summary of performance evaluations

## B.1 Evaluation of  horizontal positioning accuracy with gNB/UE TX/RX timing error

Table B.1-1: Summary of evaluated gNB/UE TX/RX timing error parameters and achieved horizontal positioning accuracy in InF-SH baseline scenario for Rel.16 positioning method.

| Company name (Positioning method) | FR1 / FR2 | gNB/UE TX/RX timing error mitigation is on/off | Evaluated UE TX/RX timing error values (Y value) | Evaluated gNB TX/RX timing error values (X value) | Is horizontal positioning accuracy 0.2m @ 90% met? | Is horizontal positioning accuracy 0.5m @ 90% met? |
| --- | --- | --- | --- | --- | --- | --- |
| [10] (Multi-RTT) | FR1 | Off at gNB Off at UE | 10 ns | 5 ns | NO | NO |
|  | FR1 | Ideal at gNB On at UE | 0 ns | 5 ns | NO | YES |
| [7] (DL-TDOA) | FR1 | Off at gNB | 0 ns | 0.5 ns | NO | NO |
|  | FR2 | Off at gNB | 0 ns | 0.5 ns | NO | YES |
| [4] (DL/UL-TDOA) | FR1 | Off at gNB | N/A | 1.4ns (2ns inter-gNB difference) | NO | NO |
| [4] (UL-TDOA/AoA) | FR1 | Off at gNB | N/A | 1.4ns (2ns inter-gNB difference) | NO | YES |
| [4] (Multi-RTT) | FR1 | Off at gNB Off at UE | 5.6ns (8ns intra-UE Rx - Tx difference) | 1.4ns (2ns intra-gNB Rx – Tx difference) | NO | NO |
| [4] (UL-TDOA) | FR1 | On at gNB | N/A | 0ns inter-gNB difference | YES | YES |
|  |  |  |  | 0.2ns inter-gNB difference | YES | YES |
|  |  |  |  | 0.5ns inter-gNB difference | NO | YES |
|  |  |  |  | 1ns inter-gNB difference | NO | NO |
| [5] (DL-TDOA) | FR1 | Off at gNB Off at UE | 0 ns | 0 ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1ns | 0.5ns | NO | YES |
|  |  |  | 2ns | 0.5ns | NO | YES |
|  |  |  | 3ns | 0.5ns | NO | YES |
|  |  |  | 5ns | 0.5ns | NO | YES |
|  |  |  | 0.5ns | 1ns | NO | YES |
|  |  |  | 0.5ns | 2ns | NO | NO |
|  |  |  | 0.5ns | 3ns | NO | NO |
|  |  |  | 0.5ns | 5ns | NO | NO |
| [5] (Multi-RTT) | FR1 | Off at gNB Off at UE | 0 ns | 0 ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1ns | 0.5ns | NO | YES |
|  |  |  | 2ns | 0.5ns | NO | YES |
|  |  |  | 3ns | 0.5ns | NO | YES |
|  |  |  | 5ns | 0.5ns | NO | YES |
|  |  |  | 0.5ns | 1ns | NO | YES |
|  |  |  | 0.5ns | 2ns | NO | YES |
|  |  |  | 0.5ns | 3ns | NO | NO |
|  |  |  | 0.5ns | 5ns | NO | NO |
| [17] (DL-TDOA) | FR2 | Off at gNB Off at UE | 0.0ns | 0.0ns | YES | YES |
|  |  |  | 0.1ns | 0.1ns | YES | YES |
|  |  |  | 0.2ns | 0.2ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1.0ns | 1.0ns | NO | NO |
|  |  |  | 2.0ns | 2.0ns | NO | NO |
| [20] (DL-TDOA) | FR2 | Off at gNB Off at UE | N/A | 0ns | YES | YES |
|  |  | Off at gNB Off at UE | N/A | 1ns | NO | YES |
|  |  | Off at gNB Off at UE | N/A | 2ns | NO | NO |
|  |  | Off at gNB Off at UE | N/A | 4ns | NO | NO |
|  |  | Off at gNB Off at UE | N/A | 8ns | NO | NO |
|  |  | On at gNB | N/A | 0ns | YES | YES |
|  |  | On at gNB | N/A | 8ns | YES | YES |

Table B.1-2: Summary of evaluated gNB/UE TX/RX timing error parameters and achieved horizontal accuracy in InF-DH baseline scenario for Rel.16 positioning methods.

| Company name (Positioning method) | FR1 / FR2 | gNB/UE TX/RX timing error mitigation is on/off | Evaluated UE TX/RX timing error values (Y value) | Evaluated gNB TX/RX timing error values (X value) | Is positioning accuracy 0.2m @ 90% met? | Is positioning accuracy 0.5m @ 90% met? |
| --- | --- | --- | --- | --- | --- | --- |
| [10] (Multi-RTT) | FR1 | Off at gNB Off at UE | 10 ns | 5 ns | NO | NO |
|  | FR1 | Ideal at gNB On at UE | 0 ns | 5 ns | NO | NO |
| [7] (DL-TDOA) | FR1 | Off at gNB | 0 ns | 0.5 ns | NO | NO |
|  | FR2 | Off at gNB | 0 ns | 0.5 ns | NO | NO |
| [4] (DL/UL-TDOA) | FR1 | Off at gNB | N/A | 1.4ns (2ns inter-gNB difference) | NO | NO |
| [4] (UL-TDOA/AoA) | FR1 | Off at gNB | N/A | 1.4ns (2ns inter-gNB difference) | NO | NO |
| [4] (Multi-RTT) | FR1 | Off at gNB Off at UE | 5.6ns (8ns intra-UE Rx - Tx difference) | 1.4ns (2ns intra-gNB Rx – Tx difference) | NO | NO |
| [5] (DL-TDOA) | FR1 | Off at gNB Off at UE | 0 ns | 0ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1ns | 0.5ns | NO | YES |
|  |  |  | 2ns | 0.5ns | NO | YES |
|  |  |  | 3ns | 0.5ns | NO | YES |
|  |  |  | 5ns | 0.5ns | NO | YES |
|  |  |  | 0.5ns | 1ns | NO | YES |
|  |  |  | 0.5ns | 2ns | NO | NO |
|  |  |  | 0.5ns | 3ns | NO | NO |
|  |  |  | 0.5ns | 5ns | NO | NO |
| [5] (Multi-RTT) | FR1 | Off at gNB Off at UE | 0 ns | 0ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1ns | 0.5ns | NO | YES |
|  |  |  | 2ns | 0.5ns | NO | YES |
|  |  |  | 3ns | 0.5ns | NO | YES |
|  |  |  | 5ns | 0.5ns | NO | YES |
|  |  |  | 0.5ns | 1ns | NO | YES |
|  |  |  | 0.5ns | 2ns | NO | YES |
|  |  |  | 0.5ns | 3ns | NO | NO |
| [17] (DL-TDOA) | FR2 | Off at gNB Off at UE | 0.0ns | 0.0ns | YES | YES |
|  |  |  | 0.1ns | 0.1ns | YES | YES |
|  |  |  | 0.2ns | 0.2ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | No | No |
|  |  |  | 1.0ns | 1.0ns | No | No |
|  |  |  | 2.0ns | 2.0ns | No | No |

Table B.1-3: Summary of evaluated gNB/UE TX/RX timing error parameters and achieved horizontal positioning accuracy in IOO scenario for Rel.16 positioning method.

| Company name (Positioning method) | FR1 / FR2 | gNB/UE TX/RX timing error mitigation is on/off | Evaluated UE TX/RX timing error values (Y value) | Evaluated gNB TX/RX timing error values (X value) | Is horizontal positioning accuracy 0.2m @ 90% met? | Is horizontal positioning accuracy 0.5m @ 90% met? |
| --- | --- | --- | --- | --- | --- | --- |
| [8] (DL-TDOA) | FR1 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (UL-TDOA) | FR1 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (Multi-RTT) | FR1 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (DL-TDOA) | FR2 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (UL-TDOA) | FR2 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (Multi-RTT) | FR2 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |

Table B.1-4: Summary of evaluated gNB/UE TX/RX timing error parameters and achieved horizontal positioning accuracy in UMi scenario for Rel.16 positioning method.

| Company name (Positioning method) | FR1 / FR2 | gNB/UE TX/RX timing error mitigation is on/off | Evaluated UE TX/RX timing error values (Y value) | Evaluated gNB TX/RX timing error values (X value) | Is horizontal positioning accuracy 1m @ 90% met? |
| --- | --- | --- | --- | --- | --- |
| [17], UMI with Δτ (RTT) | FR1 | Off at gNB Off at UE | 1 ns | 1 ns | No |
| [17], UMI with Δτ (RTT) | FR1 | Off at gNB Off at UE | 2 ns | 2 ns | No |
| [17], UMI with Δτ (RTT) | FR1 | Off at gNB Off at UE | 5 ns | 5 ns | No |
| [17], UMI with Δτ (RTT) | FR1 | Off at gNB Off at UE | 10 ns | 10 ns | No |
| [17], UMI without Δτ (RTT) | FR1 | Off at gNB Off at UE | 2 ns | 2 ns | Yes |
| [17], UMI without Δτ (RTT) | FR1 | Off at gNB Off at UE | 5 ns | 5 ns | No |
| [17], UMI without Δτ (RTT) | FR1 | Off at gNB Off at UE | 10 ns | 10 ns | No |

## B.2 Evaluation of  physical layer latency

Table B.2-1: physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-Assisted NR positioning

| Source Reference to Tdoc # | Physical layer latency for DL-TDOA/DL-AOD, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| [17] | [57-823] | Major Assumption: Connected state, FR1, (N,T) = (6,8) PRS capability Major components: Location Request reception, MG request & configuration, PRS/MG Alignment, PRS processing capabilities |
| [4] | FR1: 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) | Major assumptions: PRS periodicity is 20ms MG is requested Major components PRS measurement |
| [4] | FR1: 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) | Major assumptions: PRS periodicity is 160ms MG is not requested (sharing with existing RRM gap 6ms/40ms) Major components PRS measurement |
| [7] | FR1:106.23 ms FR2: 667.87 ms | Major assumptions: RRC Connected;4 samples;CSSF=1;Measurement Gap Repetition Period is 20ms. Major components: Measurement gap request procedures UE positioning measurement |
| [5] R1-2007665 | FR1: [64-11556] FR2： [728-328996] | Major assumptions and components: For FR1: DL measurement &process delay=LCMTPRS, T measGap∪ TProcess time, PRS and MG is periodicity The minimum value is 22ms for LCMTPRS, T measGap=20ms，(N,T) = (6,8) The maximum value is 11514 ms for LCMTPRS, T measGap=10240ms，(N,T) = (6,1280) For FR2: DL measurement &process delay=LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time , NRxBeam=8 , NPosOccasion=4 The minimum value is 20*4*8+2ms =642ms The maximum value is (10240×8×4+1280-6)=328954ms MG request and configuration Location Request and report |
| [11] | FR1: [38-235.6] FR2: [35-229.6] | Major Assumptions: Start and End States: RRC_CONNECTED, MG configuration enabled, MGRP = 20ms-160ms, 1 DL PRS occasion, T=8-160 ms DL PRS processing time. Major Components: Request Location reception and processing, MG request & configuration, DL PRS Measurement and Processing, Provide Location transmission and processing. |
| [11] | FR1: [17-5147.8] FR2: [15.5-5144.8] | Major Assumptions: Start and End States: RRC_CONNECTED, Without MG configuration, DL PRS periodicity =4-5120ms, 1 DL PRS occasion, T=8ms DL PRS processing time. Major Components: Request Location reception and processing, DL PRS Measurement and Processing, Provide Location transmission and processing. |
| [15] | FR1: For UE capability-1: 73.12+[X]ms ~ 217.12+[X]ms For UE capability-2: 71.68+ [X]ms ~ 215.26+[X]ms | Major assumptions: -For PUSCH transmission: Uplink switching gap is not configured. No BWP switching No overlapping symbols of the PUCCH and the scheduled PUSCH # of PUSCH symbols = from 4 to 14 for Type A # of PUSCH symbols = from 1 to 14 for Type B -For PDSCH transmission: No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH # of PDSCH symbols = from 3 to 14 for Type A # of PDSCH symbols = from 2 to 14 for Type B -[X]: Processing delay at gNB in terms of physical layer (Up to gNB capability) Major components RRC processing time for LPP message at both gNB and UE (LPP request location information message, measurement gap request message, LPP provide location information message) PRS measurement (LCM of PRS resource periodicity and repetition periodicity of the measurement gap) If the latency components related with higher layer are excluded, the physical layer latency is described as follows: For UE capability-1: 23.12ms ~ 167.12ms (FR1) For UE capability-2: 21.68ms ~ 165.26ms (FR1) |
| [8] | FR1: 51.5ms | Major Assumptions: Case 1, 15kHz, FR1, DL-TDOA Source UE/Destination NW Positioning technique DL-TDOA, type DL, mode UE-assisted, Initial and Final RRC States CONNECTED. Major Components: require measurement gap, measurement gap configuration, the delay between the time when DL PRS is received and the time when measurement gap configuration is received, the time from UE begins to measure PRS until the measurement result is ready to report, measurement reporting. |
| [13] | FR1: [44.35 – 10500] ms FR2: [35.08 – 2118.93 ms] | Major Assumptions: 15 kHz SCS for FR1 120 kHz SCS for FR2 Source NW/ Destination NW. UE-assisted. Including MG configuration. Major components: DL PRS periodicity DL PRS processing time SR related steps |
| [12] | FR1: 54.125ms for 60KHz FR2: 52.56ms for 120KHz | Major Assumptions: 60KHz for FR1 and 120KHz for FR2 Major components: Process Location Request reception, MG request & configuration, PRS measurement and processing PUSCH carrying measurement report |
| [16] | FR1: 33ms | Major assumptions: 30kHz SCS Initial and final state: RRC_CONNECTED. The UE is configured with MG of 1.5ms, receives the PRS within the MG to conduct positioning measurement. The UE uses a configured grant having periodicity of 1ms to report the measurement. Best case scenario Major components: Decoding the LPP request location by the UE Decoding the MG request by the gNB Receiving the MG configuration and apply the configuration. |
| [10] | FR1: 129.07 ms | Major assumptions: 30kHz SCS / FDD Initial and final state: RRC_CONNECTED. DL PRS: 18 resources / 4 symbols per resource / 12 Comb-6 symbols per period. Periodicity – 20 ms. UE DL PRS processing capability – N = 0.5 ms (~12 symbols @30kHz), T = 8 ms Dynamic DL/UL scheduling based on SR – based on URLLC assumptions [3GPP 38.824, v16.0.0] Measurement gap: MGL = 5.5 ms, MGRP = 20ms DL PRS processing Nsample = 4 (RAN4 core measurements requirements) UE is expected to perform measurements on DL PRS resource 4 times (i.e. across 4 periods) Higher layer latency components (RRC/LPP processing) are included into the physical layer analysis Major components: MG configuration and alignment time DL PRS processing time and report delay Multiple DL/UL transactions for location request, assistance information, measurement gap request and configuration and associated UE/gNB higher layer processing delays (RRC/LPP) Summary: 4.5714 (L1 components) + 36 (L2/L3 components) + 88.5 (DL PRS processing) = 129.07 ms (total) |

Table B.2-2: physical layer latency for Rel.16 UL-TDOA/DL-AOA UE-Assisted NR positioning

| Source Reference to Tdoc # | Physical layer latency for UL-TDOA/UL-AOA, ms | Comments on major assumptions and physical layer latency components |  |
| --- | --- | --- | --- |
| [4] | FR1: 6.5-26ms (1 samp.) 66.5-86.5ms (4 samp) | Major assumptions: SRS periodicity is 20ms Major components SRS measurement |  |
| [5] | FR1: 30.5-2570.5 FR2: 650.5-10250.5 | Major assumptions: FR1:SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560}slots - 15kHz 1ms-2560ms - 30kHz 0.5ms-1280ms - 60kHz 0.25ms-640ms - 120kHz 0.125ms-320ms FR2: Multiple positioning occasion (4) and beam sweeping (8) UL measurement equals to the periodicity of SRS×NRxBeam×NPosOccasion gNB processing delay is assumed as zero； The minimum periodicity of SRS is 20ms and the same as the DL minimum periodicity. Major components: SRS measurement; NRPPa process time |  |
| [5] | FR1: 11-43 | Major assumptions: SRS is aperiodic and the slot offset of aperiodic is 0-32 slots - 15kHz 0ms-32ms - 30kHz 0ms-16ms - 60kHz 0ms-8ms 120kHz 0ms-4ms Major components: SRS measurement; NRPPa process time; Activation; |  |
| [15] | FR1: For UE capability-1: 31.49+[Y] ms ~ 34.42+[Y] ms For UE capability-2: 30.77+[Y] ms ~ 33.28+[Y] ms | Major assumptions: -For SRS transmission: One shot transmission (2 OS ~ 12 OS) -For PDSCH transmission: No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH # of PDSCH symbols = from 3 to 14 for Type A # of PDSCH symbols = from 2 to 14 for Type B -[Y]: Processing delay at gNB in terms of physical layer (Up to gNB capability) Major components RRC processing time for LPP message at both gNB and UE (SRS configuration, SRS activation message) -When the latency related with higher layer is excluded, physical layer latency is described as follows: For UE capability-1: 1.49ms ~ 4.42ms (FR1) For UE capability-2: 0.77ms ~ 3.28ms (FR1) |  |
| [8] | FR1: 5ms | Major Assumptions: Case 2, 15kHz, FR1, UL-TDOA Source UE/Destination NW Positioning technique UL-TDOA, type UL, mode UE-assisted, Initial and Final RRC States CONNECTED Major Components: the time to activate the SRS transmission, the delay from effective time of SRS activation until UE begins to transmit SRS, the time from gNB begins to measure SRS until the measurement result is ready. |  |
| [13] | FR1: [2.78 – 81928.5] ms | Major Assumptions: 15 kHz SCS Source NW/ Destination NW. Excluding SRS-Pos RRC configuration Major Components: SRS-Pos periodicity Processing of SRS-Pos at gNB/RP-only |  |
| [13] | FR1: [2.78 – 81928.5] ms | Major Assumptions: 15 kHz SCS Source NW/ Destination NW. Excluding SRS-Pos RRC configuration Major Components: SRS-Pos periodicity Processing of SRS-Pos at gNB/RP-only |
| [13] | FR1: [2.35 – 81925] ms | Major Assumptions: 15 kHz SCS Source NW/ Destination NW. Excluding SRS-Pos RRC configuration Major Components: SRS-Pos periodicity Processing of SRS-Pos at gNB/RP-only |  |
| [12] | FR1: 23.25 ms for 60kHz FR2: 23.125ms for 120kHz | Major Assumptions: 60KHz for FR1 and 120KHz for FR2. Major Components: gNB process NPPa measurement request Configure SRS SRS-Pos periodicity gNB processing SRS |  |
| [16] | FR1: 12ms | Major assumptions: Initial and final state: RRC_CONNECTED. SRS transmission resources occur immediately after decoding the SRS configuration. 30kHz SCS Best case scenario Major components: Decoding the SRS configuration message. |  |
| [10] | FR1: 18.77 ms | Major assumptions: 30kHz SCS / FDD Initial and final state: RRC_CONNECTED. Dynamic DL/UL scheduling based on SR – see URLLC assumptions [3GPP 38.824, v16.0.0] PUSCH: Any symbol, subject to slot boundary constraint (i.e. transmission does not cross slot boundary); Duration – 2, 4, 7 symbols (Type B mapping w/ front loaded DMRS) PUCCH: 7 occasions per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0] for SR and HARQ feedback, Duration – 1 symbol. No HARQ – initial transmission is successful SRS for positioning: Single resource, 1 symbol duration, Periodicity – each slot Higher layer latency components (RRC/LPP processing) are included into the physical layer latency analysis Major components: SRS for positioning configuration UE/gNB higher layer processing delays (RRC/LPP processing) Summary = 2.7678 (L1 components) + 16 (L2/L3 components) = 18.7678 ms (total) |  |

Table B.2-3: physical layer latency for Rel.16 UE-Assisted Multi-RTT Positioning

| Source Reference to Tdoc # | Physical layer latency for Multi-RTT, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| [17] | [59-823] | Major assumptions: Connected state, FR1, (N,T) = (6,8) PRS capability Major components: Location Request Reception, MG Request & Configuration, PRS/MG Alignment, PRS processing capabilities |
| [4] | FR1: 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) | Major assumptions: PRS periodicity is 20ms MG is requested Major components PRS measurement |
| [4] | FR1: 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) | Major assumptions: PRS periodicity is 160ms MG is not requested (sharing with existing RRM gap 6ms/40ms) Major components PRS measurement |
| [5] | FR1: [94.5-14126.5] +TAlign_DL_UL | Major assumptions and components: For FR1: DL measurement &process delay =LCMTPRS, T measGap∪ TProcess time, PRS and MG is periodicity the minimum value is 22ms for LCMTPRS, T measGap=20ms，(N,T) = (6,8) the maximum value is 11514 ms for LCMTPRS, T measGap=10240ms，(N,T) = (6,1280) FR1:SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560}slots - 15kHz 1ms-2560ms - 30kHz 0.5ms-1280ms - 60kHz 0.25ms-640ms - 120kHz 0.125ms-320ms TAlign_DL_UL: The alignment delay is the gap between End trigger of DL positioning and Start trigger of UL positioning. MG request and configuration Location Request and report |
| [15] | FR1: For UE capability-1: 145.34ms+[Z] ~ 293.32+[Z]ms For UE capability-2: 142.8+[Z]ms ~ 289.75+[Z] ms | Major assumptions: -For PUSCH transmission: Uplink switching gap is not configured. No BWP switching No overlapping symbols of the PUCCH and the scheduled PUSCH # of PUSCH symbols = from 4 to 14 for Type A # of PUSCH symbols = from 1 to 14 for Type B -For PDSCH transmission: No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH # of PDSCH symbols = from 3 to 14 for Type A # of PDSCH symbols = from 2 to 14 for Type B -For SRS transmission: One shot transmission (2 OS ~ 12 OS) -[Z]: Processing delay at gNB in terms of physical layer (Up to gNB capability) Major components RRC processing time for LPP message at both gNB and UE (SRS configuration, SRS activation message, LPP request location information message, measurement gap request message, LPP provide location information message) PRS measurement (LCM of PRS resource periodicity and repetition periodicity of the measurement gap) -When the latency related with higher layer is excluded, physical layer latency is described as follows: For UE capability-1: 25.34ms ~ 173.32ms (FR1) For UE capability-2: 22.8ms ~169.75ms (FR1) |
| [16] | FR1: 45ms | Major assumptions: Initial and final state: RRC_CONNECTED. The UE is configured with MG of 1.5ms, receives the PRS within the MG to conduct positioning measurement. The UE uses a configured grant having periodicity of 1ms to report the measurement. SRS transmission resources occur immediately after decoding the SRS configuration. 30kHz SCS Best case scenario Major components: Decoding the LPP request location by the UE Decoding the MG request by the gNB Receiving the MG configuration and apply the configuration. Receiving PRS in the MG Decoding the SRS configuration message. |
| [10] | 140.84 ms | Major assumptions: 30kHz SCS / FDD Initial and final state: RRC_CONNECTED. DL PRS: 18 resources / 4 symbols per resource / 12 Comb-6 symbols per period. Periodicity – 20 ms. UE DL PRS processing capability – N = 0.5 ms (~12 symbols @30kHz), T = 8 ms Dynamic DL/UL scheduling based on SR – based on URLLC assumptions [3GPP 38.824, v16.0.0] Measurement gap: MGL = 5.5 ms, MGRP = 20ms DL PRS processing - Nsample = 4 (RAN4 core measurements requirements) - UE is expected to perform measurements on DL PRS resource 4 times (i.e. across 4 periods) PUSCH: Any symbol, subject to slot boundary constraint (i.e. transmission does not cross slot boundary); Duration – 2, 4, 7 symbols (Type B mapping w/ front loaded DMRS) PUCCH: 7 occasions per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0] for SR and HARQ feedback, Duration – 1 symbol. No HARQ – initial transmission is successful SRS for positioning: Single resource, 1 symbol duration, Periodicity – each slot Higher layer latency components (RRC/LPP processing) are included into the physical layer latency analysis Major components: MG configuration and alignment time DL PRS processing time and report delay Multiple DL/UL transactions and associated UE/gNB RRC/LPP processing delays Summary: 7.3393 (L1 components) + 45 (L2/L3 components) + 88.5 (DL PRS processing) = 140.8393 (total) |

Table B.2-4: physical layer latency for Rel.16 UE-Assisted E-CID Positioning

| Source Reference to Tdoc # | Physical layer latency for ECID, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| [4] | FR1 8.5-15ms | Major assumptions: DL E-CID RRM measurement available Major components Higher layer signaling processing |
| [4] | FR1 6-26ms | Major assumptions: UL E-CID RRM measurement available Major components Higher layer signaling processing, or Additional AoA measurement at gNB |
| [7] | FR1 10.30 ms | Major assumptions: DL E-CID RRM measurement is available at UE side. Major components: UE interprets and applies the measurement configuration |
| [15] | FR1: For UE capability-1: 21.56+[M]ms ~ 23.56+[M] ms For UE capability-2: 20.84+[M] ms~22.63+[M] ms | Major assumptions: -For PUSCH transmission: - Uplink switching gap is not configured. - No BWP switching - No overlapping symbols of the PUCCH and the scheduled PUSCH - # of PUSCH symbols = from 4 to 14 for Type A - # of PUSCH symbols = from 1 to 14 for Type B -[M]: Processing delay at gNB in terms of physical layer (Up to gNB capability) Major components - RRC processing time for LPP message at both gNB and UE (LPP provide location information message) -When the latency related with higher layer is excluded, physical layer latency is described as follows: - For UE capability-1: 1.56ms ~ 3.56ms (FR1) - For UE capability-2: 0.84ms~2.63ms (FR1) |

Table B.2-5: physical layer latency for Rel.16 UE-Based DL Only Positioning

| Source Reference to Tdoc # | Physical layer latency for UE-based DL only positioning, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| Source #1: | FR1: FR2: | Major assumptions: Major components: |
| [17]1 | [46-811] | Major assumptions: Start from RRC Connected, FR1, (N,T)=(6,8), External client Major components: Location Request reception, MG request & configuration, MG/PRS alignment, PRS processing capabilities |
| [17]2 | [8-780] | Major assumptions: Start from RRC Inactive, FR1, (N,T)=(6,8) , Internal client Major components: PRS alignment time, PRS processing capabilities |
| [4] | FR1 51-58.5ms (1 samp.) | Major assumptions: PRS periodicity is 20ms MG is requested MO-LR Major components PRS measurement |
| [5] 1 | FR1: [66-11558] FR2： [730-328998] | Major assumptions and components: For FR1: DL measurement &process delay=LCMTPRS, T measGap∪ TProcess time, PRS and MG is periodicity MG request and configuration Calculation of Location Estimate at the UE Location Request and report MT-LR |
| [5] 2 R1-2007665 | FR1: [55.5-11547.5] FR2： [719.5-328987.5] | Major assumptions and components: For FR1: DL measurement &process delay=LCMTPRS, T measGap∪ TProcess time, PRS and MG is periodicity MG request and configuration Location Request Calculation of Location Estimate at the UE MO-LR |
| [11] | FR1: [29-207.8] FR2: [27.5 -204.8] | Major Assumptions: Start and End States: RRC_CONNECTED, MGRP = 20ms-160ms, 1 DL PRS occasion, T=8ms-160ms PRS processing time, Request and provide location information messages omitted. Major Components: MG request & configuration, DL PRS Measurement and Processing. |
| [11] | [8-5120] | Major Assumptions: Start and End States: RRC_CONNECTED, Without MG configuration, DL PRS periodicity=4-5120ms, 1 DL PRS occasion, T=8ms DL PRS processing time, Request and provide location information messages omitted. Major Components: DL PRS Measurement and Processing. |
| [12] | 44ms | Major Assumption: - Start time: UE sends MG request - End time: UE finish location calculation Major component: - MG request and configuration - Measurement gap periodicity - UE calculating location |
| [16] | FR1 : [39-61] ms for Alt. 1, [50-72] ms for Alt. 2, [22-44] ms for Alt. 3, where different alternatives correspond to different starting points for latency evaluation of UE-B positioning | Major assumptions: - 30kHz SCS - Initial and final state: RRC_CONNECTED. - The UE is configured with MG of 1.5ms, receives the PRS within the MG to conduct positioning measurement. (for Alt 1 & 2) - The UE uses a configured grant having periodicity of 1ms to report the measurement. (for Alt 1 & 2 & 3) - Best case scenario Major components: - Decoding the LPP request location by the UE (for Alt 2) - Decoding the MG request by the gNB (for Alt 1 & 2) - Receiving the MG configuration and apply the configuration. (for Alt 1 & 2) - UE calculating location (for Alt 1 & 2 & 3) |

# Annex C:	Appendix to Performance evaluations for R17 performance targets

## C.1	Performance analysis of Rel-16 positioning solutions

Including accuracy and latency (objective 1b) performance, compared to rel17 performance targets

### C.1.1	Positioning accuracy analysis

#### C.1.1.1	Results from source [4]

##### C.1.1.1.1	Description of evaluation scenarios

The scenarios to be presented in this contribution include

- Baseline scenarios with InF-SH/InF-DH with fixed UE/gNB height and without UE/gNB calibration error (Case 0xx series)

- InF-DH with variable UE/gNB height (Case 1xx series)

- InF-SH/In-DH with UE/gNB calibration error (Case 2xx series)

Evaluation assumptions for system level analysis are provided in Table C.1.1.1.1-1 to C.1.1.1.1-5.

##### C.1.1.1.2	Positioning accuracy evaluation results

Table C.1.1.1.2-1 provides summary of NR positioning evaluations results for horizontal location error under the baseline scenarios.

Table C.1.1.1.2-2 provides summary of NR positioning evaluations results for horizontal location error under the modified DH and 3D positioning.

Table C.1.1.1.2-3 provides summary of NR positioning evaluations results for horizontal location error under UE/gNB calibration error.

Table C.1.1.1.2-4 provides summary of NR positioning evaluations results for vertical location error under the 3D positioning.

#### C.1.1.2	Results from source [7]

##### C.1.1.2.1	Description of evaluation scenarios

The evaluation scenarios for verifying achievable accuracy based on Rel-16 positioning methods in this contribution  include,

- Baseline scenarios with InF-SH/InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, for  comparing results when UEs are uniformly distributed and inside the covex hull over the factory.

- Baseline scenarios with InF-SH/InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, for  comparing results when gNBs have different Tx calibration errors (0ns ,0.5 ns, 1 ns and 2 ns).

- InF-SH/InF-DH (both FR1 and FR2) with discrete  gNB (staggered distribution) height and uniformly distributed UE height, for investigating vertical positioning accuracy.

Some scenario common parameters can be found in Table C.1.2.2.1-1. In addition, Table C.1.2.2.1-2 reveals some controlled variables of all simulation cases (Case 1-24).

Table C.1.1.2.1-1 Scenario common parameters

Table C.1.1.2.1-2 All simulation cases for positioning accuracy evaluation based on Rel-16 positioning methods

| Simulation cases | Scenario | FR1/FR2 | UE horizontal drop procedure | gNB Tx calibration error | Clutter parameters |
| --- | --- | --- | --- | --- | --- |
| Case 1 | InF-SH | FR1 | Uniformly distributed | 0ns | {20%, 2m, 10m} |
| Case 2 | InF-SH | FR1 | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 3 | InF-SH | FR1 | Inside convex hull | 0.5ns | {20%, 2m, 10m} |
| Case 4 | InF-SH | FR1 | Inside convex hull | 1ns | {20%, 2m, 10m} |
| Case 5 | InF-SH | FR1 | Inside convex hull | 2ns | {20%, 2m, 10m} |
| Case 6 | InF-SH | FR2 | Uniformly distributed | 0ns | {20%, 2m, 10m} |
| Case 7 | InF-SH | FR2 | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 8 | InF-SH | FR2 | Inside convex hull | 0.5ns | {20%, 2m, 10m} |
| Case 9 | InF-SH | FR2 | Inside convex hull | 1ns | {20%, 2m, 10m} |
| Case 10 | InF-SH | FR2 | Inside convex hull | 2ns | {20%, 2m, 10m} |
| Case 11 | InF-DH | FR1 | Uniformly distributed | 0ns | {40%, 2m, 2m} |
| Case 12 | InF-DH | FR1 | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 13 | InF-DH | FR1 | Inside convex hull | 0.5ns | {40%, 2m, 2m} |
| Case 14 | InF-DH | FR1 | Inside convex hull | 1ns | {40%, 2m, 2m} |
| Case 15 | InF-DH | FR1 | Inside convex hull | 2ns | {40%, 2m, 2m} |
| Case 16 | InF-DH | FR2 | Uniformly distributed | 0ns | {40%, 2m, 2m} |
| Case 17 | InF-DH | FR2 | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 18 | InF-DH | FR2 | Inside convex hull | 0.5ns | {40%, 2m, 2m} |
| Case 19 | InF-DH | FR2 | Inside convex hull | 1ns | {40%, 2m, 2m} |
| Case 20 | InF-DH | FR2 | Inside convex hull | 2ns | {40%, 2m, 2m} |
| Case 21 | InF-SH | FR1 | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 22 | InF-SH | FR2 | Inside convex hull | 0ns | {20%, 2m, 10m} |
| Case 23 | InF-DH | FR1 | Inside convex hull | 0ns | {40%, 3m,5m} |
| Case 24 | InF-DH | FR2 | Inside convex hull | 0ns | {40%, 3m, 5m} |

##### C.1.1.2.2	Positioning accuracy evaluation results

Table C.1.1.2.2-1 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-SH, FR1 scenario, and corresponding CDF curves can be found in Figure C.1.1.2.2-1-1.

Table C.1.1.2.2-2 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-SH, FR2 scenario, and corresponding CDF curves can be found in Figure C.1.1.2.2-1-2.

Table C.1.1.2.2-3 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-DH, FR1 scenario,  and corresponding CDF curves can be found in Figure C.1.1.2.2-1-3.

Table C.1.1.2.2-4 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-DH, FR2 scenario, and corresponding CDF curves can be found in Figure C.1.1.2.2-1-4.

Table C.1.1.2.2-5 provides CDF of vertical positioning accuracy at some specific percentiles  for InF-SH and InF-DH, and corresponding CDF curves can be found in Figure C.1.1.2.2-2.

Table C.1.1.2.2-1 CDF of horizontal positioning accuracy at some specific percentiles for InF-SH, FR1 scenario

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 1 | 0.282 | 0.370 | 0.467 | 0.603 |
| Case 2 | 0.264 | 0.350 | 0.469 | 0.568 |
| Case 3 | 0.301 | 0.411 | 0.549 | 0.704 |
| Case 4 | 0.475 | 0.616 | 0.767 | 0.943 |
| Case 5 | 0.841 | 1.042 | 1.239 | 1.479 |

Table C.1.1.2.2-2 CDF of horizontal positioning accuracy at some specific percentiles for InF-SH, FR2 scenario

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 6 | 0.044 | 0.059 | 0.072 | 0.092 |
| Case 7 | 0.041 | 0.055 | 0.070 | 0.090 |
| Case 8 | 0.171 | 0.223 | 0.257 | 0.300 |
| Case 9 | 0.419 | 0.480 | 0.545 | 0.615 |
| Case 10 | 0.833 | 0.971 | 1.086 | 1.224 |

Table C.1.1.2.2-3 CDF of horizontal positioning accuracy at some specific percentiles for InF-DH, FR1 scenario

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 11 | 0.552 | 5.708 | 10.120 | 12.433 |
| Case 12 | 0.414 | 1.060 | 7.337 | 12.345 |
| Case13 | 0.885 | 1.719 | 7.544 | 12.386 |
| Case 14 | 1.242 | 3.560 | 8.249 | 12.368 |
| Case 15 | 1.632 | 4.127 | 7.701 | 12.458 |

Table C.1.1.2.2-4 CDF of horizontal positioning accuracy at some specific percentiles for InF-DH, FR2 scenario

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 16 | 0.076 | 0.150 | 10.255 | 14.759 |
| Case 17 | 0.062 | 0.097 | 0.174 | 12.174 |
| Case 18 | 0.323 | 0.458 | 0.665 | 10.815 |
| Case 19 | 0.542 | 0.738 | 1.045 | 12.285 |
| Case 20 | 0.958 | 1.220 | 11.075 | 14.845 |

Table C.1.1.2.2-5 CDF of vertical positioning accuracy at some specific percentiles  for InF-SH and InF-DH

| Simulation cases | Vertical positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 21 | 0.411 | 0.623 | 0.815 | 0.979 |
| Case 22 | 0.185 | 0.293 | 0.362 | 0.459 |
| Case 23 | 0.522 | 0.814 | 1.062 | 1.419 |
| Case 24 | 0.222 | 0.414 | 0.705 | 1.271 |

Figure C.1.1.2.2-1-1 InF-SH, FR1                        Figure C.1.1.2.2-1-2 InF-SH, FR2

Figure C.1.1.2.2-1-3 InF-DH, FR1                         Figure C.1.1.2.2-1-4 InF-DH, FR2

Figure C.1.1.2.2-1 Horizontal positioning accuracy based on Rel-16 DL-TDOA method

Figure C.1.1.2.2-2-1 InF-SH                      Figure C.1.1.2.2-2-2 InF-DH

Figure C.1.1.2.2-2 Vertical positioning accuracy based on Rel-16 DL-TDOA method

Table C.1.1.1.2-1: Rel.16 NR positioning (baseline) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 1, InF-SH, FR1, DL-TDOA | (Optional) All UEs | 0.077 | 0.153 | 0.419 | 2.789 |
|  | Convex UEs | 0.053 | 0.090 | 0.224 | 1.964 |
| 2, InF-SH, FR1, UL-TDOA | (Optional) All UEs | 0.0871 | 0.1708 | 0.4013 | 1.8678 |
|  | Convex UEs | 0.0621 | 0.1096 | 0.2410 | 1.0277 |
| 3, InF-SH, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0465 | 0.0731 | 0.1271 | 0.299 |
|  | Convex UEs | 0.0434 | 0.0648 | 0.1116 | 0.2682 |
| 4, InF-SH, FR1, Multi-RTT | (Optional) All UEs | 0.0721 | 0.2035 | 0.6628 | 1.9481 |
|  | Convex UEs | 0.0665 | 0.1798 | 0.5628 | 1.6992 |
| 5, InF-DH422, FR1, DL-TDOA | (Optional) All UEs | 0.861 | 3.448 | 8.103 | 15.212 |
|  | Convex UEs | 0.703 | 2.904 | 8.399 | 15.635 |
| 6, InF-DH422, FR1, UL-TDOA | (Optional) All UEs | 0.4309 | 1.6317 | 4.9693 | 9.9453 |
|  | Convex UEs | 0.3354 | 1.2393 | 4.6237 | 9.6631 |
| 7, InF-DH422, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0702 | 0.1673 | 0.3825 | 1.0453 |
|  | Convex UEs | 0.0643 | 0.1529 | 0.3206 | 0.8016 |
| 8, InF-DH422, FR1, Multi-RTT | (Optional) All UEs | 0.0699 | 0.1260 | 0.3916 | 7.8992 |
|  | Convex UEs | 0.0667 | 0.1164 | 0.3338 | 7.3110 |
| 9, InF-SH, FR2, DL-TDOA | (Optional) All UEs | 0.0089 | 0.0329 | 0.1935 | 1.1281 |
|  | Convex UEs | 0.0077 | 0.0265 | 0.1586 | 0.9633 |
| 10, InF-SH, FR2, DL-TDOA/AoD | (Optional) All UEs | 0.0118 | 0.0203 | 0.0345 | 0.0918 |
|  | Convex UEs | 0.0143 | 0.0231 | 0.0387 | 0.0654 |
| 11, InF-SH, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0102 | 0.0191 | 0.0365 | 0.0991 |
|  | Convex UEs | 0.0084 | 0.0156 | 0.0296 | 0.0694 |
| 12, InF-SH, FR2, Multi-RTT | (Optional) All UEs | 0.0049 | 0.0143 | 0.0881 | 0.4697 |
|  | Convex UEs | 0.0048 | 0.0137 | 0.0841 | 0.4496 |
| 13, InF-DH422, FR2, DL-TDOA | (Optional) All UEs | 0.0466 | 0.5795 | 3.6866 | 9.2401 |
|  | Convex UEs | 0.0321 | 0.4003 | 2.7974 | 9.6798 |
| 14, InF- DH422, FR2, DL-TDOA/AoD | (Optional) All UEs | 0.0755 | 0.1861 | 0.4228 | 0.9619 |
|  | Convex UEs | 0.0587 | 0.1452 | 0.3221 | 0.7197 |
| 15, InF-DH422, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0732 | 0.1728 | 0.3750 | 0.9277 |
|  | Convex UEs | 0.0603 | 0.1309 | 0.2717 | 0.7086 |
| 16, InF- DH422, FR2, Multi-RTT | (Optional) All UEs | 0.0317 | 0.1905 | 1.0572 | 4.1555 |
|  | Convex UEs | 0.0259 | 0.1425 | 0.8398 | 4.2895 |

Table C.1.1.1.2-2: Rel.16 NR positioning (modified DH and 3D positioning) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 101, InF-DH435, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0742 | 0.1792 | 0.4392 | 1.3838 |
|  | Convex UEs | 0.0628 | 0.1571 | 0.3844 | 1.3012 |
| 102, InF-DH435, FR1, Multi-RTT | (Optional) All UEs | 0.0713 | 0.1249 | 0.4734 | 8.6557 |
|  | Convex UEs | 0.0663 | 0.1082 | 0.4952 | 9.8411 |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.3405(H) | 1.0103(H) | 2.3157(H) | 5.1203(H) |
|  | Convex UEs | 0.2923(H) | 0.8122(H) | 2.0088(H) | 4.3405(H) |
| 104, InF-DH435-3D, FR1, Multi-RTT | (Optional) All UEs | 0.1721(H) | 1.3411(H) | 7.4143(H) | 15.4966(H) |
|  | Convex UEs | 0.1258(H) | 1.4345(H) | 8.9295(H) | 16.0515(H) |
| 105, InF-DH435, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0943 | 0.2314 | 0.4973 | 1.3007 |
|  | Convex UEs | 0.0865 | 0.2101 | 0.4707 | 1.1486 |
| 106, InF-DH435, FR2, Multi-RTT | (Optional) All UEs | 0.3364 | 0.9128 | 2.4777 | 5.7209 |
|  | Convex UEs | 0.3079 | 0.8594 | 2.2392 | 5.4600 |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.1297(H) | 0.4173(H) | 1.1042(H) | 2.6878(H) |
|  | Convex UEs | 0.1109(H) | 0.3273(H) | 0.8667(H) | 2.4365(H) |
| 108, InF-DH435-3D, FR2, Multi-RTT | (Optional) All UEs | 0.5012(H) | 2.2343(H) | 6.6096(H) | 15.6304(H) |
|  | Convex UEs | 0.4304(H) | 2.4466(H) | 7.9926(H) | 15.5828(H) |

Table C.1.1.1.2-3: Rel.16 NR positioning (UE/gNB calibration error) - horizontal location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 201, InF-SH, FR1, DL-TDOA, Group Delay Error | (Optional) All UEs | 0.587 | 0.874 | 1.462 | 3.540 |
|  | Convex UEs | 0.462 | 0.614 | 0.829 | 1.458 |
| 202, InF-SH, FR1, UL-TDOA, Group delay error | (Optional) All UEs | 0.5937 | 0.8672 | 1.3135 | 2.6395 |
|  | Convex UEs | 0.4653 | 0.6145 | 0.8120 | 1.2343 |
| 203, InF-SH, FR1, UL-TDOA/AoA, Group delay error | (Optional) All UEs | 0.0699 | 0.1180 | 0.2034 | 0.3878 |
|  | Convex UEs | 0.0661 | 0.1094 | 0.1750 | 0.3251 |
| 204, InF-SH, FR1, Multi-RTT, Group delay error | (Optional) All UEs | 1.7167 | 2.3902 | 3.3592 | 4.4076 |
|  | Convex UEs | 1.7105 | 2.3963 | 3.3574 | 4.2662 |
| 205, InF-DH422, FR1, DL-TDOA, Group delay error | (Optional) All UEs | 1.719 | 4.034 | 8.588 | 15.330 |
|  | Convex UEs | 1.391 | 3.407 | 8.570 | 15.039 |
| 206, InF-DH422, FR1, UL-TDOA, Group delay error | (Optional) All UEs | 1.0065 | 2.0631 | 4.7037 | 10.459 |
|  | Convex UEs | 0.8741 | 1.5739 | 4.2751 | 9.4102 |
| 207, InF-DH422, FR1, UL-TDOA/AoA, Group delay error | (Optional) All UEs | 0.0810 | 0.184 | 0.4251 | 1.1961 |
|  | Convex UEs | 0.0717 | 0.1577 | 0.3371 | 0.8662 |
| 208, InF-DH422, FR1, Multi-RTT, Group delay error | (Optional) All UEs | 2.2640 | 3.2695 | 4.7279 | 8.9874 |
|  | Convex UEs | 2.2784 | 3.3381 | 5.0728 | 9.5701 |
| 209, InF-SH, FR1, UL-AoA | (Optional) All UEs | 0.0446 | 0.0615 | 0.0879 | 0.1383 |
|  | Convex UEs | 0.0410 | 0.0552 | 0.0758 | 0.1119 |
| 210, InF-SH, FR1, UL-AoA, Angle error 1 degree | (Optional) All UEs | 0.4960 | 0.6675 | 0.8724 | 1.3003 |
|  | Convex UEs | 0.4629 | 0.6280 | 0.8016 | 1.1676 |
| 211, InF-SH, FR1, UL-AoA, Angle error 2 degrees | (Optional) All UEs | 0.9357 | 1.2729 | 1.6915 | 2.6204 |
|  | Convex UEs | 0.8848 | 1.1961 | 1.5487 | 2.1732 |
| 212, InF-SH, FR1, UL-AoA, Angle error 5 degrees | (Optional) All UEs | 2.3274 | 3.1571 | 4.2839 | 5.9433 |
|  | Convex UEs | 2.2187 | 2.9534 | 4.0582 | 5.3982 |

Table C.1.1.1.2-4: Rel.16 NR positioning (modified DH and 3D positioning) - altitude location error results from [4]

| Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0905(V) | 0.2923(V) | 0.7287(V) | 1.3239(V) |
|  | Convex UEs | 00682(V) | 0.2182(V) | 0.5301(V) | 1.1585(V) |
| 104, InF-DH435-3D, FR1, Multi-RTT | (Optional) All UEs | 0.2176(V) | 0.6510(V) | 1.1742(V) | 1.7622(V) |
|  | Convex UEs | 0.1666(V) | 0.5005(V) | 1.0346(V) | 1.6675(V) |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | (Optional) All UEs | 0.0361(V) | 0.1075(V) | 0.2471(V) | 0.5573(V) |
|  | Convex UEs | 0.0288(V) | 0.0796(V) | 0.1879(V) | 0.4593(V) |
| 108, InF-DH435-3D, FR2, Multi-RTT | (Optional) All UEs | 0.3436(V) | 0.9022(V) | 1.5000(V) | 1.9555(V) |
|  | Convex UEs | 0.2871(V) | 0.7927(V) | 1.3316(V) | 1.8800(V) |

Figures C.1.1.1.2-1 to C.1.1.1.2-4 provide positioning evaluations results for the baseline scenario.

Figures C.1.1.1.2-5 to C.1.1.1.2-8 provide positioning evaluation results for the modified DH and 3D positioning (including horizontal and vertical error).

Figures C.1.1.1.2-9 to C.1.1.1.2-11 provide positioning evaluation results for the UE/gNB calibration error.

Figure C.1.1.1.2-1: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-2: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-3: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-4: Rel.16 NR positioning error results (baseline) from [4]

Figure C.1.1.1.2-5: Rel.16 NR positioning error results (modified DH and 2D) from [4]

Figure C.1.1.1.2-6: Rel.16 NR positioning error results (modified DH and 3D) from [4]

Figure C.1.1.1.2-7: Rel.16 NR positioning error results (modified DH and 2D) from [4]

Figure C.1.1.1.2-8: Rel.16 NR positioning error results (modified DH and 3D) from [4]

Figure C.1.1.1.2-9: Rel.16 NR positioning error results (UE/gNB calibration error) from [4]

Figure C.1.1.1.2-10: Rel.16 NR positioning error results (UE/gNB calibration error) from [4]

Figure C.1.1.1.2-11: Rel.16 NR positioning error results (gNB angle calibration error) from [4]

Table C.1.1.1.1-1: Rel.16 NR positioning (baseline) - evaluation scenarios and parameters

| Parameter | Case 1 (InF-SH, FR1) | Case 2 (InF-SH, FR1) | Case 3 (InF-SH, FR1) | Case 4 (InF-SH, FR1) | Case 5 (InF-DH, FR1) | Case 6 (InF-DH, FR1) | Case 7 (InF-DH, FR1) | Case 8 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-2: Rel.16 NR positioning (baseline) - evaluation scenarios and parameters

| Parameter | Case 9 (InF-SH, FR2) | Case 10 (InF-SH, FR2) | Case 11 (InF-SH, FR2) | Case 12 (InF-SH, FR2) | Case 13 (InF-DH, FR2) | Case 14 (InF-DH, FR2) | Case 15 (InF-DH, FR2) | Case 16 (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port | Gold, single port | Gold, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | DL-TDOA+DL-AoD PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-3: Rel.16 NR positioning (modified DH and 3D positioning) - evaluation scenarios and parameters

| Parameter | Case 101 (InF-DH, FR1) | Case 102 (InF-DH, FR1) | Case 103 (InF-DH, FR1) | Case 104 (InF-DH, FR1) | Case 105 (InF-DH, FR2) | Case 106 (InF-DH, FR2) | Case 107 (InF-DH, FR2) | Case 108 (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) | InF-DH (40%, 3, 5) |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping | Tx beam sweeping Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based | Tx codebook-based Rx codebook based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} | Fixed UE/gNB height. | Fixed UE/gNB height. | UE height within [0.5, 3] gNB height {4, 8} | UE height within [0.5, 3] gNB height {4, 8} |

Table C.1.1.1.1-4: Rel.16 NR positioning (UE/gNB calibration error) - evaluation scenarios and parameters

| Parameter | Case 201 (InF-SH, FR1) | Case 202 (InF-SH, FR1) | Case 203 (InF-SH, FR1) | Case 204 (InF-SH, FR1) | Case 205 (InF-DH, FR1) | Case 206 (InF-DH, FR1) | Case 207 (InF-DH, FR1) | Case 208 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz | 100Mhz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold, single port | ZC, single port | ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | DL-TDOA PSO | UL-TDOA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | gNB Rx/Tx Time error T1=1.4ns UE Rx/Tx time error T1=5.6ns |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

Table C.1.1.1.1-5: Rel.16 NR positioning (gNB angle calibration error) - evaluation scenarios and parameters

| Parameter | Case 209 (InF-SH, FR1) | Case 210 (InF-SH, FR1) | Case 211 (InF-SH, FR1) | Case 212 (InF-SH, FR1) |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | In-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHzHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100Mhz | 100Mhz | 100Mhz | 100Mhz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-AOA PSO | UL-AOA PSO | UL-AOA PSO | UL-AOA PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | gNB Rx Angle error θ=1° | gNB Rx Angle error θ=2° | gNB Rx Angle error θ=5° |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. | Fixed UE/gNB height. |

#### C.1.1.3	 Results from source [8]

##### C.1.1.3.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis are provided from Table C.1.1.3.3-1a to Table C.1.1.3.3-1f.

- Case IDs: 1 to 36.

- Scenarios: InF-SH-2D, InF-DH-2D, InF-SH-3D, InF-DH-3D, IOO.

- Frequency Bands: FR1, FR2.

- Positioning Techniques: R.16 DL-TDOA, R.16 UL-TDOA, R.16 Multi-RTT, R.16 DL-AOD, R.16 UL-TDOA +R.16 UL-AOA.

Note 1：

- InF-SH-2D: 300x150 m, Horizontal: gNB antenna height (8m), UE antenna height (1.5m), low clutter density {20%, 2m, 10m}.

- InF-SH-3D: 300x150 m, Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 2m], low clutter density {20%, 2m, 10m}

- InF-DH-2D: 120x60 m, Horizontal: gNB antenna height (8m), UE antenna height (1.5m), high clutter density #1{40%, 2m, 2m}

- InF-DH-3D: 120x60 m, Horizontal and vertical: gNB antenna height (4m+8m), UE antenna height uniformly distributed [0.5m, 3m], high clutter density #2{40%, 3m, 5m}.

Table C.1.1.3.3-1a: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR1]

| Parameter | [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | [Case 2], [InF- SH-2D], [FR1], [UL-TDOA] | [Case 3], [InF- SH-2D], [FR1], [Multi-RTT] | [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | [Case 5], [InF- SH-3D], [FR1], [UL-TDOA] | [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-SH-2D | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |

Table C.1.1.3.3-1b: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR1]

| Parameter | [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | [Case 9], [InF- DH-2D], [FR1], [UL-TDOA] | [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-DH-2D | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |

Table C.1.1.3.3-1c: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR1]

| Parameter | [Case 15], [IOO], [FR1], [DL-TDOA] | [Case 16], [IOO], [FR1], [UL-TDOA] | [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 18], [IOO], [FR1], [Multi-RTT] | [Case 19], [IOO], [FR1], [DL-TDOA] | [Case 20], [IOO], [FR1], [UL-TDOA] | [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 22], [IOO], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | IOO | IOO | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | Yes | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.3.3-1d: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR2]

| Parameter | [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | [Case 24], [InF- SH-2D], [FR2], [UL-TDOA] | [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | [Case 26], [InF- SH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any |  |  |  |  |

Table C.1.1.3.3-1e: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR2]

| Parameter | [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any |  |  |  |  |

Table C.1.1.3.3-1f: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR2]

| Parameter | [Case 31], [IOO], [FR2], [DL-TDOA] | [Case 32], [IOO], [FR2], [UL-TDOA] | [Case 33], [IOO], [FR2], [Multi-RTT] | [Case 34], [IOO], [FR2], [DL-TDOA] | [Case 35], [IOO], [FR2], [UL-TDOA] | [Case 36], [IOO], [FR2], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |

Table C.1.1.3.3-1a: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR1]

| Parameter | [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | [Case 2], [InF- SH-2D], [FR1], [UL-TDOA] | [Case 3], [InF- SH-2D], [FR1], [Multi-RTT] | [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | [Case 5], [InF- SH-3D], [FR1], [UL-TDOA] | [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-SH-2D | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |

Table C.1.1.3.3-1b: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR1]

| Parameter | [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | [Case 9], [InF- DH-2D], [FR1], [UL-TDOA] | [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | InF-DH-2D | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No |
| Precoding assumptions | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |

Table C.1.1.3.3-1c: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR1]

| Parameter | [Case 15], [IOO], [FR1], [DL-TDOA] | [Case 16], [IOO], [FR1], [UL-TDOA] | [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 18], [IOO], [FR1], [Multi-RTT] | [Case 19], [IOO], [FR1], [DL-TDOA] | [Case 20], [IOO], [FR1], [UL-TDOA] | [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | [Case 22], [IOO], [FR1], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline) | IOO | IOO | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 UL-TDOA+ R.16 UL-AOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | Yes | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.3.3-1d: Rel.16 NR positioning - evaluation scenarios and parameters [InF-SH-FR2]

| Parameter | [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | [Case 24], [InF- SH-2D], [FR2], [UL-TDOA] | [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | [Case 26], [InF- SH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH-2D | InF-SH-2D | InF-SH-3D | InF-SH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any |  |  |  |  |

Table C.1.1.3.3-1e: Rel.16 NR positioning - evaluation scenarios and parameters [InF-DH-FR2]

| Parameter | [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH-2D | InF-DH-2D | InF-DH-3D | InF-DH-3D |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 DL-TDOA | R.16 UL-TDOA |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No |
| Additional Notes, if any |  |  |  |  |

Table C.1.1.3.3-1f: Rel.16 NR positioning - evaluation scenarios and parameters [IOO-FR2]

| Parameter | [Case 31], [IOO], [FR2], [DL-TDOA] | [Case 32], [IOO], [FR2], [UL-TDOA] | [Case 33], [IOO], [FR2], [Multi-RTT] | [Case 34], [IOO], [FR2], [DL-TDOA] | [Case 35], [IOO], [FR2], [UL-TDOA] | [Case 36], [IOO], [FR2], [Multi-RTT] |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 | DL PRS, Comb- 6 | UL SRS, Comb- 8 | DL PRS, Comb- 6; UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence, Port-1 | Gold sequence, Port-1 | ZC sequence, Port-1 | Gold & ZC sequence Port-1 |
| Number of sites | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of symbols used per occasion | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 | 1 for FR1; 12 for FR2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB | 7.78 / 9 dB | 7.78dB | 9dB | 7.78 / 9 dB |
| Uplink power control (applied/Not applied) | —— | —— | —— | —— | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC | MUSIC |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT | R.16 DL-TDOA | R.16 UL-TDOA | R.16 Multi-RTT |
| Network synchronization assumptions | Perfect | Perfect | Perfect | Not Perfect | Not Perfect | Not Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | Yes | Yes | Yes |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No | No | No | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No | No | No | No | No |
| Additional Notes, if any |  |  |  |  |  |  |

##### C.1.1.3.2	Positioning accuracy evaluation results

Evaluation results are presented from Table C.1.1.3.2-1 to Table C.1.1.3.2-2.

Table C.1.1.3.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.3.2-1: Rel.16 NR positioning - horizontal location error results from [8]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | Convex UEs | 0.0418 | 0.0603 | 0.0836 | 0.1650 |
|  | (Optional) All UEs | 0.0996 | 0.104 | 0.1196 | 0.3159 |
| [Case 2], [InF-SH-2D], [FR1], [UL-TDOA] | Convex UEs | 0.0430 | 0.0621 | 0.0823 | 0.1551 |
| [Case 3], [InF-SH-2D], [FR1], [Multi-RTT] | Convex UEs | 0.03969 | 0.05828 | 0.08561 | 0.1650 |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.079 | 0.1083 | 0.1379 | 0.2045 |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.0754 | 0.1143 | 0.1728 | 0.2574 |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.033 | 0.054 | 0.1428 | 0.2677 |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.08 | 0.108 | 0.153 | 0.2540 |
| [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | Convex UEs | 0.0599 | 0.088 | 0.1185 | 0.1693 |
| [Case 9], [InF-DH-2D], [FR1], [UL-TDOA] | Convex UEs | 0.0488 | 0.0695 | 0.0856 | 0.1184 |
| [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | Convex UEs | 0.04744 | 0.06356 | 0.08704 | 0.1237 |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.1201 | 0.2499 | 0.4116 | 0.7089 |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.108 | 0.155 | 0.3692 | 0.6937 |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.044 | 0.062 | 0.118 | 0.151 |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.115 | 0.179 | 0.391 | 0.692 |
| [Case 15], [IOO], [FR1], [DL-TDOA] | Convex UEs | 0.079 | 0.1028 | 0.1338 | 0.2288 |
| [Case 16], [IOO], [FR1], [UL-TDOA] | Convex UEs | 0.0646 | 0.0984 | 0.127 | 0.1836 |
| [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.013 | 0.031 | 0.049 | 0.1219 |
| [Case 18], [IOO], [FR1], [Multi-RTT] | Convex UEs | 0.073 | 0.1024 | 0.1331 | 0.283 |
| [Case 19], [IOO], [FR1], [DL-TDOA] | Convex UEs | 7.4192 | 13.0891 | 18.592 | 32.4509 |
| [Case 20], [IOO], [FR1], [UL-TDOA] | Convex UEs | 6.6813 | 12.6521 | 18.281 | 32.0927 |
| [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 2.4591 | 3.7312 | 5.603 | 9.2356 |
| [Case 22], [IOO], [FR1], [Multi-RTT] | Convex UEs | 0.3828 | 0.4918 | 0.6728 | 1.3668 |
| [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | Convex UEs | 0.0136 | 0.0223 | 0.028 | 0.0372 |
| [Case 24], [InF-SH-2D], [FR2], [UL-TDOA] | Convex UEs | 0.022 | 0.0296 | 0.041 | 0.0538 |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.025 | 0.0324 | 0.046 | 0.0789 |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.0223 | 0.0276 | 0.0383 | 0.0817 |
| [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | Convex UEs | 0.0178 | 0.023 | 0.0277 | 0.0388 |
| [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | Convex UEs | 0.0204 | 0.0296 | 0.0416 | 0.0553 |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.0314 | 0.089 | 0.2996 | 0.7033 |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.0282 | 0.0561 | 0.2827 | 0.6848 |
| [Case 31], [IOO], [FR2], [DL-TDOA] | Convex UEs | 0.0166 | 0.0225 | 0.0301 | 0.0406 |
| [Case 32], [IOO], [FR2], [UL-TDOA] | Convex UEs | 0.011 | 0.0179 | 0.0243 | 0.0397 |
| [Case 33], [IOO], [FR2], [Multi-RTT] | Convex UEs | 0.01636 | 0.022 | 0.03 | 0.0402 |
| [Case 34], [IOO], [FR2], [DL-TDOA] | Convex UEs | 7.3189 | 13.0765 | 18.572 | 32.3809 |
| [Case 35], [IOO], [FR2], [UL-TDOA] | Convex UEs | 6.6798 | 12.5968 | 18.2581 | 32.0887 |
| [Case 36], [IOO], [FR2], [Multi-RTT] | Convex UEs | 0.3475 | 0.4718 | 0.6565 | 1.2681 |

Table C.1.1.3.2-2 provides summary of NR positioning evaluations results for vertical location error.

Table C.1.1.3.2-2: Rel.16 NR positioning - altitude location error results from [8]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.608 | 0.9636 | 1.3383 | 1.8954 |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.608 | 1.036 | 1.3383 | 2.0599 |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.8572 | 1.154 | 1.516 | 2.256 |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.639 | 0.917 | 1.60 | 2.18 |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | Convex UEs | 0.461 | 0.8138 | 1.3474 | 2.9917 |
| [Case 12], [InF- DH-3D], [FR1], [UL-TDOA] | Convex UEs | 0.4167 | 0.6408 | 1.0091 | 2.049 |
| [Case 13], [InF- DH-3D], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.334 | 0.486 | 0.593 | 0.633 |
| [Case 14], [InF- DH-3D], [FR1], [Multi-RTT] | Convex UEs | 0.507 | 0.723 | 1.59 | 2.82 |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.1581 | 0.277 | 0.3961 | 0.6283 |
| [Case 26], [InF- SH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.2018 | 0.3084 | 0.4611 | 0.8304 |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | Convex UEs | 0.1337 | 0.2703 | 0.6779 | 3.0578 |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | Convex UEs | 0.116 | 0.2358 | 0.5051 | 3.1267 |

#### C.1.1.4	Results from source [13]

##### C.1.1.4.1	Description of evaluation scenarios

In this section results are provided for the InF-SH, InF-DH, IOO, and UMi scenarios.

Table C.1.1.4.1-1 Description of evaluation scenarios

| Parameter | Case 1 [InF-SH, DL-TDOA, FR1, 100 MHz] | Case 2 [InF-DH, DL-TDOA, FR1, 100 MHz] | Case 3 [IOO, DL-TDOA, FR1, 100 MHz] | Case 4 [IOO, DL-TDOA, FR1, 100 MHz] | Case 5 [UMi, DL-TDOA, FR1, 100 MHz] | Case 6 [InF-SH, DL-TDOA, FR1, 100 MHz] | Case 7 [InF-DH, DL-TDOA, FR1, 100 MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline | Baseline | Baseline | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS | Comb-6, 6 symbol NR PRS |
| Reference signal (type of sequence, number of ports, …) | NR PRS, single port | NR PRS, single port | NR PRS, single port | NR PRS, single port | NR PRS, single port | NR PRS, single port | NR PRS, single port |
| Number of sites | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| Number of symbols used per slot per positioning estimate | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot | 6 symbols per slot |
| Number of slots per positioning estimate | 1 slot | 1 slot | 1 slot | 1 slot | 1 slot | 1 slot | 1 slot |
| Power-boosting level | 6 dB | 6 dB | 6 dB | 6 dB | 6 dB | 6 dB | 6 dB |
| Uplink power control (applied/not applied) | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Thresholding, 0.5, (No oversampling) | Thresholding, 0.5, (No oversampling) | Thresholding, 0.5, (Oversampling x4) | Thresholding, 0.5, (No oversampling) | Thresholding, 0.5, (No oversampling) | Thresholding, 0.5, (Oversampling x4) | Thresholding, 0.5, (Oversampling x4) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Taylor Series, Least Squares | Taylor Series, Least Squares | Series, Least Square | Taylor Series, Least Squares | Taylor Series, Least Squares | Taylor Series, Least Squares | Taylor Series, Least Squares |
| Network synchronization assumptions | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization | Perfect Synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nr of antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

##### C.1.1.4.2	Positioning accuracy evaluation results

Table C.1.1.4.2-1 Positioning accuracy evaluation results

| Case | UEs | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, [InF-SH, DL-TDOA, FR1] | Convex UEs | 0.98 | 1.47 | 2.13 | 4.35 |
| Case 2, [InF-DH, DL-TDOA, FR1] | Convex UEs | 1.71 | 3.15 | 4.39 | 7.16 |
| Case 3, [IOO, DL-TDOA, FR1] | All UEs | 0.46 | 0.82 | 1.68 | 4.31 |
| Case 4, [IOO, DL-TDOA, FR1] | All UEs | 1.17 | 1.92 | 3.24 | 6.50 |
| Case 5, [UMi, DL-TDOA, FR1] | All UEs | 5.29 | 9.59 | 14.92 | 23.81 |
| Case 6, [InF-SH, DL-TDOA, FR1] | Convex UEs | 0.26 | 0.38 | 0.66 | 1.65 |
| Case 7, [InF-DH, DL-TDOA, FR1] | Convex UEs | 1.29 | 2.04 | 3.34 | 4.99 |

#### C.1.1.5 Results from source [5]

##### C.1.1.5.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis are provided in Table C.1.1.5.1-1.1- Table C.1.1.5.1-1.8.

Table C.1.1.5.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 1] (InF-SH, FR1) | [Case 2] (InF-SH, FR1) | [Case 3] (InF-SH, FR2) | [Case 4] (InF-SH, FR2) | [Case 5] (InF-DH, FR1) | [Case 6] (InF-DH, FR1) | [Case 7] (InF-DH, FR2) | [Case 8] (InF-DH, FR2) | [Case 9] (InF-DH, FR1) | [Case 10] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (60%, 6, 2) | InF-DH (60%, 6, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) |  |  |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-2: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 11] (InF-SH, FR1) | [Case 12] (InF-SH, FR1) | [Case 13] (InF-SH, FR2) | [Case 14] (InF-SH, FR2) | [Case 15] (InF-DH, FR1) | [Case 16] (InF-DH, FR1) | [Case 17] (InF-DH, FR2) | [Case 18] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-3: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 19] (InF-SH, FR1) | [Case 20] (InF-SH, FR1) | [Case 21] (InF-SH, FR2) | [Case 22] (InF-SH, FR2) | [Case 23] (InF-DH, FR1) | [Case 24] (InF-DH, FR1) | [Case 25] (InF-DH, FR2) | [Case 26] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-4: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 27] (InF-SH, FR1) | [Case 28] (InF-SH, FR1) | [Case 29] (InF-SH, FR2) | [Case 30] (InF-SH, FR2) | [Case 31] (InF-DH, FR1) | [Case 32] (InF-DH, FR1) | [Case 33] (InF-DH, FR2) | [Case 34] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-5: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 35] (InF-SH, FR1) | [Case 36] (InF-SH, FR1) | [Case 37] (InF-DH, FR1) | [Case 38] (InF-DH, FR1) | [Case 39] (InF-SH, FR1) | [Case 40] (InF-SH, FR1) | [Case 41] (InF-DH, FR1) | [Case 42] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-AOA Least square first/median peak | UL-AOA Least square first/median peak | UL-AOA Least square first/median peak | UL-AOA Least square first/median peak | UL-TDOA+AOA Least square first/median peak | UL-TDOA+AOA Least square first/median peak | UL-TDOA+AOA Least square first/median peak | UL-TDOA+AOA Least square first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-6: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 43] (InF-SH, FR1) | [Case 44] (InF-SH, FR1) | [Case 45] (InF-SH, FR2) | [Case 46] (InF-SH, FR2) | [Case 47] (InF-DH, FR1) | [Case 48] (InF-DH, FR1) | [Case 49] (InF-DH, FR2) | [Case 50] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-7: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 51] (InF-SH, FR1) | [Case 52] (InF-SH, FR1) | [Case 53] (InF-SH, FR2) | [Case 54] (InF-SH, FR2) | [Case 55] (InF-DH, FR1) | [Case 56] (InF-DH, FR1) | [Case 57] (InF-DH, FR2) | [Case 58] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.1.1.5.1-8: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case V1] (InF-SH, FR1) | [Case V2] (InF-DH, FR1) | [Case V3] (InF-SH, FR1) | [Case V4] (InF-DH, FR1) | [Case V5] (InF-SH, FR1) | [Case V6] (InF-DH, FR1) | [Case V7] (InF-SH, FR1) | [Case V8] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH (BS height = 8m UE height =1.5m) | InF-DH (40%, 2, 2) (BS height = 8m UE height =1.5m) | InF-SH (BS height = {4,8}m UE height =1.5m) | InF-DH (40%, 2, 2) (BS height = {4,8}m UE height =1.5m) | InF-SH (BS height =8m UE height =[0.5,2]m) | InF-DH (40%, 2, 2), (BS height =8m UE height =[0.5,2]m) | InF-SH (BS height = {4,8}m UE height =[0.5,2]m) | InF-DH (40%, 2, 2) (BS height = {4,8}m UE height =[0.5,2]m) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak | UL-AOA+ZOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |  |  |

##### C.1.1.5.2	Positioning accuracy evaluation results

Table C.1.1.5.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.5.2-1: Rel.16 NR positioning - horizontal location error results from [5]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.041 | 0.055 | 0.099 | 4.15 |
|  | (Optional) All UEs | 0.059 | 0.17 | 1.40 | 6.73 |
| [Case 2], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.49 | 14.48 | 19.44 | 24.06 |
|  | (Optional) All UEs | 13.41 | 18.91 | 25.07 | 35.48 |
| [Case 3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.021 | 2.97 |
|  | (Optional) All UEs | 0.014 | 0.029 | 0.35 | 4.82 |
| [Case 4], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.67 | 13.44 | 17.49 | 23.21 |
|  | (Optional) All UEs | 13.45 | 17.71 | 23.46 | 33.34 |
| [Case 5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.056 | 0.33 | 2.97 | 5.92 |
|  | (Optional) All UEs | 0.10 | 1.23 | 4.60 | 9.80 |
| [Case 6], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.95 | 13.09 | 16.43 | 23.79 |
|  | (Optional) All UEs | 12.65 | 16.29 | 20.59 | 30.46 |
| [Case 7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.012 | 0.23 | 2.72 | 5.77 |
|  | (Optional) All UEs | 0.023 | 1.06 | 4.08 | 9.67 |
| [Case 8], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.39 | 12.89 | 15.05 | 22.90 |
|  | (Optional) All UEs | 11.79 | 14.79 | 17.49 | 22.93 |
| [Case 9], [DH {0.6,6,2}, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 6.13 | 7.76 | 13.38 | 18.71 |
|  | (Optional) All UEs | 6.49 | 10.35 | 15.33 | 24.59 |
| [Case 10], [DH {0.6,6,2}, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 6.63 | 9.01 | 13.6 | 15.09 |
|  | (Optional) All UEs | 8.04 | 12.84 | 15.23 | 23.12 |
| [Case 11], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.038 | 0.049 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.052 | 0.080 | 0.15 | 0.30 |
| [Case 12], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.20 | 14.93 | 18.32 | 26.09 |
|  | (Optional) All UEs | 13.82 | 18.12 | 23.77 | 36.35 |
| [Case 13], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.013 | 0.017 | 0.031 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.046 | 0.11 |
| [Case 14], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.12 | 13.01 | 17.49 | 25.67 |
|  | (Optional) All UEs | 12.27 | 17.54 | 23.28 | 32.26 |
| [Case 15], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.062 | 0.079 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.093 | 0.15 | 0.25 | 3.23 |
| [Case 16], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.14 | 13.00 | 14.93 | 20.30 |
|  | (Optional) All UEs | 12.24 | 15.42 | 20.30 | 31.59 |
| [Case 17], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.015 | 0.018 | 0.049 |
|  | (Optional) All UEs | 0.014 | 0.023 | 0.041 | 0.60 |
| [Case 18], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 9.71 | 12.14 | 15.19 | 20.16 |
|  | (Optional) All UEs | 11.36 | 14.65 | 17.97 | 25.55 |
| [Case 19], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.041 | 0.051 | 0.087 | 4.22 |
|  | (Optional) All UEs | 0.057 | 0.13 | 1.40 | 6.68 |
| [Case 20], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.50 | 14.77 | 19.49 | 24.51 |
|  | (Optional) All UEs | 13.81 | 19.44 | 26.32 | 35.51 |
| [Case 21], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.033 | 4.07 |
|  | (Optional) All UEs | 0.015 | 0.036 | 0.54 | 5.61 |
| [Case 22], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.70 | 13.82 | 18.11 | 23.21 |
|  | (Optional) All UEs | 13.65 | 18.95 | 23.67 | 33.34 |
| [Case 23], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.060 | 0.19 | 2.77 | 5.85 |
|  | (Optional) All UEs | 0.10 | 1.04 | 4.48 | 9.14 |
| [Case 24], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.92 | 13.47 | 16.02 | 22.90 |
|  | (Optional) All UEs | 12.31 | 15.40 | 19.04 | 23.85 |
| [Case 25], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.013 | 0.35 | 2.85 | 5.76 |
|  | (Optional) All UEs | 0.025 | 1.09 | 4.60 | 9.95 |
| [Case 26], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 10.52 | 13.46 | 16.09 | 18.92 |
|  | (Optional) All UEs | 12.22 | 15.26 | 18.72 | 23.64 |
| [Case 27], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.038 | 0.055 | 0.070 | 0.087 |
|  | (Optional) All UEs | 0.055 | 0.078 | 0.13 | 0.29 |
| [Case 28], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.23 | 15.14 | 20.06 | 27.70 |
|  | (Optional) All UEs | 13.95 | 18.53 | 24.10 | 36.35 |
| [Case 29], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.013 | 0.017 | 0.032 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.038 | 0.11 |
| [Case 30], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.11 | 14.30 | 18.27 | 25.67 |
|  | (Optional) All UEs | 13.38 | 18.28 | 23.63 | 32.51 |
| [Case 31], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.058 | 0.075 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.088 | 0.16 | 0.26 | 2.83 |
| [Case 32], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.14 | 13.35 | 16.32 | 22.01 |
|  | (Optional) All UEs | 12.14 | 15.29 | 18.76 | 27.05 |
| [Case 33], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.014 | 0.019 | 0.051 |
|  | (Optional) All UEs | 0.015 | 0.023 | 0.052 | 0.89 |
| [Case 34], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 9.93 | 13.20 | 16.64 | 19.74 |
|  | (Optional) All UEs | 12.15 | 15.34 | 18.35 | 24.03 |
| [Case 35], [SH, perfect sync], [FR1], [UL-AOA] | Convex UEs | 1.25 | 2.15 | 3.28 | 5.93 |
|  | (Optional) All UEs | 1.35 | 2.21 | 3.38 | 6.08 |
| [Case 36], [SH, sync error 50ns], [FR1], [UL-AOA] | Convex UEs | 1.31 | 2.24 | 3.75 | 6.20 |
|  | (Optional) All UEs | 1.43 | 2.41 | 4.02 | 6.47 |
| [Case 37], [DH, perfect sync], [FR1], [UL-AOA] | Convex UEs | 2.28 | 3.39 | 4.29 | 5.48 |
|  | (Optional) All UEs | 2.45 | 3.63 | 4.70 | 6.86 |
| [Case 38], [DH, sync error 50ns], [FR1], [UL-AOA] | Convex UEs | 2.45 | 3.56 | 4.45 | 5.76 |
|  | (Optional) All UEs | 2.63 | 3.68 | 4.75 | 7.06 |
| [Case 39], [SH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.067 | 0.11 | 0.21 | 0.41 |
|  | (Optional) All UEs | 0.078 | 0.14 | 0.25 | 0.41 |
| [Case 40], [SH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.098 | 0.17 | 0.26 | 0.43 |
|  | (Optional) All UEs | 0.11 | 0.21 | 0.33 | 0.63 |
| [Case 41], [DH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.047 | 0.094 | 0.18 | 0.68 |
|  | (Optional) All UEs | 0.055 | 0.11 | 0.41 | 4.12 |
| [Case 42], [DH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.059 | 0.10 | 0.26 | 0.77 |
|  | (Optional) All UEs | 0.083 | 0.17 | 0.44 | 4.23 |
| [Case 43], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.039 | 0.050 | 0.095 | 4.25 |
|  | (Optional) All UEs | 0.054 | 0.12 | 1.34 | 6.70 |
| [Case 44], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.042 | 0.055 | 0.11 | 4.71 |
|  | (Optional) All UEs | 0.060 | 0.14 | 1.49 | 6.72 |
| [Case 45], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.011 | 0.033 | 3.96 |
|  | (Optional) All UEs | 0.014 | 0.027 | 0.43 | 5.19 |
| [Case 46], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.034 | 4.13 |
|  | (Optional) All UEs | 0.014 | 0.037 | 0.50 | 5.87 |
| [Case 47], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.068 | 0.13 | 2.89 | 5.88 |
|  | (Optional) All UEs | 0.092 | 1.09 | 4.55 | 9.82 |
| [Case 48], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.068 | 0.23 | 3.18 | 6.20 |
|  | (Optional) All UEs | 0.091 | 1.34 | 5.49 | 10.71 |
| [Case 49], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.013 | 0.28 | 2.61 | 5.74 |
|  | (Optional) All UEs | 0.024 | 1.04 | 3.31 | 9.24 |
| [Case 50], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.30 | 2.80 | 6.23 |
|  | (Optional) All UEs | 0.025 | 1.51 | 4.53 | 10.10 |
| [Case 51], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.040 | 0.050 | 0.069 | 0.10 |
|  | (Optional) All UEs | 0.052 | 0.074 | 0.12 | 0.25 |
| [Case 52], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.042 | 0.058 | 0.069 | 0.10 |
|  | (Optional) All UEs | 0.048 | 0.080 | 0.15 | 0.32 |
| [Case 53], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.014 | 0.017 | 0.031 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.037 | 0.11 |
| [Case 54], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.012 | 0.017 | 0.030 |
|  | (Optional) All UEs | 0.013 | 0.021 | 0.039 | 0.10 |
| [Case 55], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.061 | 0.082 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.076 | 0.12 | 0.20 | 3.13 |
| [Case 56], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.063 | 0.092 | 0.12 | 0.78 |
|  | (Optional) All UEs | 0.087 | 0.13 | 0.28 | 4.72 |
| [Case 57], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.013 | 0.018 | 0.051 |
|  | (Optional) All UEs | 0.014 | 0.022 | 0.039 | 0.63 |
| [Case 58], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.015 | 0.021 | 0.055 |
|  | (Optional) All UEs | 0.016 | 0.023 | 0.041 | 0.69 |

Table C.1.1.5.2-2 provides summary of NR positioning evaluations results for vertical location error.

Table C.1.1.5.2-2: Rel.16 NR positioning - altitude location error results from [5]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case V1], [SH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.28 | 0.43 | 0.52 | 0.66 |
|  | (Optional) All UEs | 0.48 | 0.54 | 0.58 | 0.90 |
| [Case V2], [DH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.48 | 0.63 | 0.83 | 1.12 |
|  | (Optional) All UEs | 0.44 | 0.63 | 0.83 | 1.27 |
| [Case V3], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.30 | 0.38 | 0.64 | 0.82 |
|  | (Optional) All UEs | 0.33 | 0.48 | 0.68 | 0.98 |
| [Case V4], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.43 | 0.64 | 0.90 | 1.39 |
|  | (Optional) All UEs | 0.46 | 0.74 | 1.13 | 3.61 |
| [Case V5], [SH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.39 | 0.57 | 0.74 | 1.05 |
|  | (Optional) All UEs | 0.43 | 0.63 | 0.95 | 1.35 |
| [Case V6], [DH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 2.07 | 3.19 | 4.83 | 5.46 |
|  | (Optional) All UEs | 2.58 | 4.15 | 4.89 | 5.46 |
| [Case V7], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 0.45 | 0.62 | 0.96 | 1.21 |
|  | (Optional) All UEs | 0.45 | 0.70 | 1.06 | 1.33 |
| [Case V8], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | Convex UEs | 3.80 | 4.39 | 4.93 | 9.06 |
|  | (Optional) All UEs | 3.83 | 4.44 | 5.09 | 9.06 |

#### C.1.1.6	Results from source [12]

##### C.1.1.6.1	Description of evaluation scenarios

The following scenarios are included for evaluate the performance of rel16 baseline:

- Baseline scenarios with InF-SH/InF-DH with fixed UE/gNB height and without UE/gNB calibration error

Evaluation assumptions for system level analysis are provided in Table C.1.1.6.1-1.

Table C.1.1.6.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | Case1 ( InF-SH, FR1) | Case2 ( InF-DH, FR1) |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-DH (40%,2,2) |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS | DL-PRS |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port |
| Number of sites | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | ML | ML |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA Chan | DL-TDOA Chan |
| Network synchronization assumptions | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based |
| Additional notes, if any |  |  |

##### C.1.1.6.2	Positioning accuracy evaluation results

Table C.1.1.6.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.1.2-1: Rel.16 NR positioning - horizontal location error results from [12]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case1, (InF-SH, FR1, DL-TDOA) | Convex UEs | 0.26 | 0.4 | 2.14 | 8.58 |
| Case2, (InF-DH, FR1, DL-TDOA) | Convex UEs | 1.86 | 4.95 | 8.75 | 14.9 |

#### C.1.1.7	Results from source [10]

##### C.1.1.7.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis are provided in Table C.1.1.7.1-1.

Table C.1.1.7.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | Case 1, InF-SH, FR1, DL-TDOA | Case 2, InF-DH, FR1, DL-TDOA | Case 3, InF-SH, FR2, DL-TDOA | Case 4, InF-DH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 | DL PRS: Comb-2 | DL PRS: Comb-2 | DL PRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port | DL PRS: single port | DL PRS: single port | DL PRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols | DL PRS: 2 symbols | DL PRS: 2 symbols | DL PRS: 2 symbols |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB | DL PRS: 3 dB | DL PRS: 3 dB | DL PRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any |  |  |  |  |

| Parameter | Case 5, InF-SH, FR1, UL-TDOA | Case 6, InF-DH, FR1, UL-TDOA | Case 7, InF-SH, FR2, UL-TDOA | Case 8, InF-DH, FR2, UL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | UL SRS: Comb-2 | UL SRS: Comb-2 | UL SRS: Comb-2 | UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | UL SRS: single port | UL SRS: single port | UL SRS: single port | UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | UL SRS: 1 symbol | UL SRS: 1 symbol | UL SRS: 1 symbol | UL SRS: 1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | UL SRS: 3 dB | UL SRS: 3 dB | UL SRS: 3 dB | UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | Rx beam sweeping | Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any |  |  |  |  |

| Parameter | Case 9, InF-SH, FR1, Multi-RTT | Case 10, InF-DH, FR1, Multi-RTT | Case 11, InF-SH, FR2, Multi-RTT | Case 12, InF-DH, FR2, Multi-RTT |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any |  |  |  |  |

##### C.1.1.7.2	Positioning accuracy evaluation results

Table C.1.1.7.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.7.2-1: Rel.16 NR positioning - horizontal location error results from

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, InF-SH, FR1, DL-TDOA | Convex UEs | 0.16 | 0.25 | 0.45 | 0.85 |
| Case 2, InF-DH, FR1, DL-TDOA | Convex UEs | 1.04 | 2 | 3.6 | 6.2 |
| Case 3, InF-SH, FR2, DL-TDOA | Convex UEs | 0.1 | 0.19 | 0.36 | 0.65 |
| Case 4, InF-DH, FR2, DL-TDOA | Convex UEs | 0.87 | 1.9 | 5.17 | 17.3 |
| Case 5, InF-SH, FR1, UL-TDOA | Convex UEs | 0.15 | 0.25 | 0.42 | 0.77 |
| Case 6, InF-DH, FR1, UL-TDOA | Convex UEs | 0.97 | 1.7 | 4.7 | 6.13 |
| Case 7, InF-SH, FR2, UL-TDOA | Convex UEs | 0.6 | 0.09 | 0.27 | 0.9 |
| Case 8, InF-DH, FR2, UL-TDOA | Convex UEs | 0.87 | 2.08 | 3.6 | 16.9 |
| Case 9, InF-SH, FR1, Multi-RTT | Convex UEs | 0.07 | 0.11 | 0.14 | 0.25 |
| Case 10, InF-DH, FR1, Multi-RTT | Convex UEs | 1.35 | 4.2 | 9.6 | 16.3 |
| Case 11, InF-SH, FR2, Multi-RTT | Convex UEs | 0.06 | 0.09 | 0.27 | 0.9 |
| Case 12, InF-DH, FR2, Multi-RTT | Convex UEs | 0.44 | 1.45 | 3.2 | 7.72 |

Table C.1.1.7.2-2 provides summary of NR positioning evaluations results for vertical location error.

Table C.1.1.7.2-2: Rel.16 NR positioning - altitude location error results from [10]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, InF-SH, FR1, DL-TDOA | Convex UEs | 1.17 | 6.4 | 8.5 | 8.5 |
| Case 2, InF-DH, FR1, DL-TDOA | Convex UEs | 6.4 | 6.5 | 8.4 | 12.6 |
| Case 3, InF-SH, FR2, DL-TDOA | Convex UEs | 6.3 | 6.4 | 6.5 | 12.88 |
| Case 4, InF-DH, FR2, DL-TDOA | Convex UEs | 10.82 | 13.38 | 29.7 | 63.4 |
| Case 5, InF-SH, FR1, UL-TDOA | Convex UEs | 1.1 | 6.4 | 6.7 | 12.9 |
| Case 6, InF-DH, FR1, UL-TDOA | Convex UEs | 6.4 | 6.5 | 8.45 | 12.9 |
| Case 7, InF-SH, FR2, UL-TDOA | Convex UEs | 6.3 | 6.4 | 6.5 | 13 |
| Case 8, InF-DH, FR2, UL-TDOA | Convex UEs | 10.75 | 13.38 | 29.68 | 62.78 |
| Case 9, InF-SH, FR1, Multi-RTT | Convex UEs | 0.3 | 0.67 | 4.3 | 13.1 |
| Case 10, InF-DH, FR1, Multi-RTT | Convex UEs | 3.2 | 12.88 | 27.75 | 66 |
| Case 11, InF-SH, FR2, Multi-RTT | Convex UEs | 0.37 | 0.51 | 0.89 | 6.45 |
| Case 12, InF-DH, FR2, Multi-RTT | Convex UEs | 0.33 | 1.52 | 5.03 | 7.07 |

| Analysis for FR1 |  |
| --- | --- |
| Analysis for FR2 |  |

Figure C.1.1.7.2-1. Accuracy for Rel 16 positioning solutions

#### C.1.1.8 Results from source [14]

##### C.1.1.8.1 Description of evaluation scenarios

In this section results are provided for the following scenarios.

- InF-SH FR1/ FR2: 300x150 m, gNB height = 8m, UE antenna height = 1.5m, Low clutter density, {20%, 2m, 10m},

- InF-DH FR1/ FR2: 120x60 m, gNB height = 8m, UE antenna height = 1.5m, High clutter density, {40%, 2m, 2m},

- InH-OO FR1/ FR2: 120x50 m, gNB height = 3m.

Evaluation assumptions for system level analysis are provided in Table C.1.1.8.1-1.

Table C.1.1.8.1-1: Rel.16 NR positioning – vertical accuracy performance summary [14]

| Parameter | [Case1, InF- SH, FR2], [Case3, InF-DH, FR2], [Case5, InH-OO, FR2] | [Case2, InF- SH, FR1], [Case4, InF-DH, FR1], [Case6, InH-OO, FR1] |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline (See Table 3) | Baseline (See Table 3) |
| Carrier frequency | 28 GHz | 1.5 GHz |
| Subcarrier spacing | 120 kHz | 30 kHz |
| Reference Signal Transmission Bandwidth | 400 MHz | 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6, 6 symbols | Comb-6, 6 symbols |
| Reference signal (type of sequence, number of ports, …) | Pseudo-random sequence, 1 antenna port |  |
| Number of sites | 18 sites in InF, 12 sites in InH |  |
| Number of symbols used per occasion | 6 | 6 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 6 dB | 6 dB |
| Uplink power control (applied/not applied) | not applied |  |
| interference modelling (ideal muting, or other) | ideal muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Adaptive threshold-based first peak detection for TOA measurement |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA + AoD positioning Select the TRP by Fast peak average ratio (FPAR), Maximum likelihood positioning algorithm, AoD measurement |  |
| Network synchronization assumptions | Perfect synchronization |  |
| UE/gNB Tx/Rx Calibration Error | 0 |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Beam sweeping at Tx |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT Codebook |  |
| Additional notes, if any |  |  |

##### C.1.1.8.2 Positioning accuracy evaluation results

Table C.1.1.8.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.8.2-1: Rel.16 NR positioning - horizontal location error results from [14]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.028 | 0.038 | 0.052 | 0.24 |
|  | (Optional) All UEs | 0.036 | 0.059 | 0.12 | 0.39 |
| [Case 2], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.11 | 0.16 | 0.24 | 0.70 |
|  | (Optional) All UEs | 0.14 | 0.21 | 0.37 | 1.06 |
| [Case 3], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.24 | 0.72 | 1.42 | 2.80 |
|  | (Optional) All UEs | 0.30 | 0.68 | 1.31 | 2.83 |
| [Case 4], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.35 | 0.82 | 1.79 | 3.87 |
|  | (Optional) All UEs | 0.31 | 1.14 | 1.93 | 4.29 |
| [Case 5], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.045 | 0.070 | 0.14 | 0.35 |
|  | (Optional) All UEs | 0.068 | 0.13 | 0.28 | 0.50 |
| [Case 6], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.20 | 0.31 | 0.50 | 1.00 |
|  | (Optional) All UEs | 0.21 | 0.36 | 0.64 | 1.17 |

Figure C.1.1.8.2-1: Rel.16 positioning evaluation in InF-SH scenario.

Figure C.1.1.8.2-2: Rel.16 positioning evaluation in InF-DH scenario.

Figure C.1.1.8.2-3: Rel.16 positioning evaluation in InH-OO scenario.

##### C.1.1.8.3 Observations on Rel-16 NR positioning accuracy

Table C.1.1.8.3-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table C.1.1.8.3-1: Rel.16 NR positioning – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| [Case 1], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.24 |
| [Case 2], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 0.70 |
| [Case 3], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 2.80 |
| [Case 4], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 3.87 |
| [Case 5], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.35 |
| [Case 6], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | 1.00 |

#### C.1.1.9	Results from source [20]

##### C.1.1.9.1	Description of evaluation scenarios

In this section, we present simulation results for positioning accuracies in Rel.16 under baseline assumptions. The scope of evaluation is limited to DL-TDOA and UL-TDOA. Both frequency ranges (FR1 and FR2) are considered.  The scenarios evaluated are:

- Urban Macro (UMa)

- Urban Micro (UMi)

- Indoor Open Office ( IOO)

- Indoor Factory SH

- Indoor Factory DH

Evaluation assumptions for system level analysis are provided in Table C.1.1.9.1-1a-b.

Table C.1.1.9.1-1a: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [20], source1, UMa, FR1, DL-TDOA | [20], source2, UMi, FR1, DL-TDOA | [20], source3, UMi, FR1, UL-TDOA | [20], source4, UMi, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline |  | Baseline |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 272 PRB | 272 PRB | 272 PRB | 272 PRB |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Staggered | Staggered | Staggered | Staggered |
| Reference signal (type of sequence, number of ports, …) | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 |
| Number of sites | 19 (3 sectors each) | 19 (3 sectors each) | 19 (3 sectors each) | 19 (3 sectors each) |
| Number of symbols used per occasion | DL: comb-12, 12 symbol | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol | DL: comb-12, 12 symbol |
| number of occasions used per positioning estimate | 1 & 9 | 1 & 9 | 1 & 9 | 1 & 9 |
| Power-boosting level | No power boosting | DL: No power boosting | UL: Power boosting by comb factor. | DL: No power boosting |
| Uplink power control (applied/not applied) | No power control | No power control | No power control | No power control |
| interference modelling (ideal muting, or other) | Idea muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares |
| Network synchronization assumptions | Perfect synch | Perfect synch | Perfect synch | Perfect synch |
| UE/gNB Tx/Rx Calibration Error | N/A |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No beamforming | No beamforming | No beamforming | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e |
| Additional notes, if any | Two scenarios 50% indoor UEs and 100% outdoors UEs considered | N/A |  | N/A |

Table C.1.1.9.1-1b: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [20], source5, IOO, FR1, DL-TDOA | [20], source6, IOO, FR1, UL-TDOA | [20], source7, IOO, FR2, DL-TDOA | [20], source8, InF-SH, FR1, DL-TDOA | [20], source9, InF-SH , FR1, UL-TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |  | Baseline | Baseline (InF SH) |  |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 272 PRB | 272 PRB | 272 PRB | 272 PRB | 272 PRB |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Staggered | Staggered | Staggered | Staggered | Staggered |
| Reference signal (type of sequence, number of ports, …) | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 |
| Number of sites | 12 (single sector) | 12 (single sector) | 12 (3 sectors each) | 18 (single sector) | 18 (single sector) |
| Number of symbols used per occasion | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol | DL: comb-12, 12 symbol | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol |
| number of occasions used per positioning estimate | 1 & 9 | 1 & 9 | 1 & 9 | 1 | 1 |
| Power-boosting level | DL: No power boosting | UL: Power boosting by comb factor. | DL: No power boosting | Power boosting by comb factor. | Power boosting by comb factor. |
| Uplink power control (applied/not applied) | No power control | No power control | No power control | No power control | No power control |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Idea muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares |
| Network synchronization assumptions | Perfect synch | Perfect synch | Perfect synch | Perfect synch | Perfect synch |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No beamforming | No beamforming | Ideal | No beamforming | No beamforming |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e |
| Additional notes, if any | N/A |  | N/A | N/A |  |

Table C.1.1.9.1-1c: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [20], source10, InF-DH, FR1, DL-TDOA | [20], source11, InF-DH , FR1, UL-TDOA | [20], source12, InF-SH, FR2, DL-TDOA | [20], source13, InF-SH , FR2, UL-TDOA | [20], source14, InF-DH, FR2, DL-TDOA | [20], source15, InF-DH , FR2, UL-TDOA |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline (InF DH) | Baseline (InF DH) | Baseline (InF SH) | Baseline (InF SH) | Baseline (InF DH) | Baseline (InF DH) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 272 PRB | 272 PRB | 272 PRB | 272 PRB | 272 PRB | 272 PRB |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Staggered | Staggered | Staggered | Staggered | Staggered | Staggered |
| Reference signal (type of sequence, number of ports, …) | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 | Rel. 16 |
| Number of sites | 18 (single sector) | 18 (single sector) | 18 (3 sectors each) | 18 (3 sectors each) | 18 (3 sectors each) | 18 (3 sectors each) |
| Number of symbols used per occasion | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol | DL: comb-12, 12 symbol | UL: comb-2, 2 symbol |
| number of occasions used per positioning estimate | 1 | 11 | 1 |  | 1 | 1 |
| Power-boosting level | Power boosting by comb factor. | Power boosting by comb factor. | Power boosting by comb factor. | Power boosting by comb factor. | Power boosting by comb factor. | Power boosting by comb factor. |
| Uplink power control (applied/not applied) | No power control | No power control | No power control | No power control | No power control | No power control |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 | Interpolation by factor 10 |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares | Iterative Re-weighted Least Squares |
| Network synchronization assumptions | Perfect synch | Perfect synch | Perfect synch | Perfect synch | Perfect synch | Perfect synch |
| UE/gNB Tx/Rx Calibration Error | 0ns | 0ns | 0ns and 8ns | 0ns and 8ns | 0ns | 0ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No beamforming | No beamforming | Ideal | Ideal | Ideal | Ideal |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e | Baseline agreed in #101e |
| Additional notes, if any | N/A |  | N/A |  | N/A |  |

##### C.1.1.9.2	Positioning accuracy evaluation results

Table C.1.1.9.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.9.2-1: Rel.16 NR positioning - horizontal location error results from [20]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [20], source1, UMa, FR1, DL-TDOA (50% UEs indoor) | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 2.2m (1 occasion) 1.6m (9 occasions) | 3.2m (1 occasion) 2.35m (9 occasions) | 5.3m (1 occasion) 3.4m (9 occasions) | 11.97m (1 occasion) 7.04m (9 occasions) |
| [20], source1, UMa, FR1, DL-TDOA (100% UEs outdoor) | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 1.87m (1 occasion) 1.26m (9 occasions) | 2.5m (1 occasion) 1.61m (9 occasions) | 3.25m (1 occasion) 1.98m (9 occasions) | 4.5m (1 occasion) 2.5m (9 occasions) |
|  | (Optional) All UEs |  |  |  |  |
| [20], source2, UMi, FR1, DL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.58m (1 occasion) 0.41m (9 occasions) | 0.8m (1 occasion) 0.59m (9 occasions) | 1.14m (1 occasion) 0.77m (9 occasions) | 1.7m (1 occasion) 1.06m (9 occasions) |
| [20], source3, UMi, FR1, UL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 1.4m (1 occasion) 1.01m (9 occasions) | 1.86m (1 occasion) 1.4m (9 occasions) | 2.39m (1 occasion) 1.84m (9 occasions) | 3.10m (1 occasion) 2.43m (9 occasions) |
| [20], source4, UMi, FR2, DL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.05m (1 occasion) 0.02m (9 occasions) | 0.07m (1 occasion) 0.03m (9 occasions) | 0.09m (1 occasion) 0.04m (9 occasions) | 0.13m (1 occasion) 0.06m (9 occasions) |
|  | (Optional) All UEs |  |  |  |  |
| [20], source5, IOO, FR1, DL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.41m (1 occasion) 0.26m (9 occasions) | 0.62m (1 occasion) 0.4m (9 occasions) | 0.9m (1 occasion) 0.58m (9 occasions) | 1.51m (1 occasion) 0.93m (9 occasions) |
| [20], source6, IOO, FR1, UL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.39m (1 occasion) 0.24m (9 occasions) | 0.61m (1 occasion) 0.36m (9 occasions) | 0.93m (1 occasion) 0.55m (9 occasions) | 1.52m (1 occasion) 0.98m (9 occasions) |
| [20], source7, IOO, FR2, DL-TDOA | Convex UEs | N/A | N/A | N/A | N/A |
|  | (Optional) All UEs | 0.04m (1 occasion) 0.017m (9 occasions) | 0.06m (1 occasion) 0.02m (9 occasions) | 0.1m (1 occasion) 0.04m (9 occasions) | 0.18m (1 occasion) 0.07m (9 occasions) |
|  | (Optional) All UEs |  |  |  |  |
| [20], source8, InF-SH, FR1, DL-TDOA | Convex UEs | 0.06m | 0.089m | 0.13m | 0.19m |
|  | (Optional) All UEs | 0.08m | 0.13m | 0.21m | 0.36m |
| [20], source9, InF-SH, FR1, UL-TDOA | Convex UEs | 0.06m | 0.09m | 0.13m | 0.18m |
|  | (Optional) All UEs | 0.08m | 0.13m | 0.2m | 0.35m |
| [20], source10, InF-DH, FR1, DL-TDOA | Convex UEs | 1.88m | 3.63m | 5.4m | 7m |
|  | (Optional) All UEs | 2.94m | 5.09m | 7.48m | 14.3m |
| [20], source11, InF-DH , FR1, UL-TDOA | Convex UEs | 1.74m | 3.53m | 4.95m | 7.50m |
|  | (Optional) All UEs | 3.03m | 5.06m | 7.91m | 14.55m |
| [20], source12, InF-SH, FR2, DL-TDOA | Convex UEs | 0.0062m (no RX/Tx error) 1.24m (8ns Rx/Tx error) | 0.0088m(no RX/Tx error) 1.88m (8ns Rx/Tx error) | 0.0120m(no RX/Tx error) 2.57m (8ns Rx/Tx error) | 0.0172m(no RX/Tx error) 3.34m (8ns Rx/Tx error) |
|  | (Optional) All UEs | 0.0081m (no RX/Tx error) 1.01m (8ns Rx/Tx error) | 0.0122m (no RX/Tx error) 1.81m (8ns Rx/Tx error) | 0.0196m (no RX/Tx error) 2.66m (8ns Rx/Tx error) | 0.0349m (no RX/Tx error) 3.68m (8ns Rx/Tx error) |
| [20], source13, InF-SH , FR2, UL-TDOA | Convex UEs | 0.0059m (no RX/Tx error) 1.25m (8ns Rx/Tx error) | 0.0081m (no RX/Tx error) 1.90m (8ns Rx/Tx error) | 0.0112m (no RX/Tx error) 2.5m (8ns Rx/Tx error) | 0.0163m (no RX/Tx error) 3.36m (8ns Rx/Tx error) |
|  | (Optional) All UEs | 0.0075m (no RX/Tx error) 1.16m (8ns Rx/Tx error) | 0.0114m (no RX/Tx error) 1.95m (8ns Rx/Tx error) | 0.0174m (no RX/Tx error) 2.76m (8ns Rx/Tx error) | 0.0313m (no RX/Tx error) 3.85m (8ns Rx/Tx error) |
| [20], source14, InF-DH, FR2, DL-TDOA | Convex UEs | 1.78m | 3.28m | 5.08m | 7.06m |
|  | (Optional) All UEs | 2.92m | 5.14m | 7.76m | 14.95m |
| [20], source15, InF-DH , FR2, UL-TDOA | Convex UEs | 1.58m | 3.05m | 4.72m | 6.98m |
|  | (Optional) All UEs | 2.78m | 4.65m | 7.11m | 13.48m |

Simulations results showing accuracies reported in Table C.1.1.9.2-1 are showing in figures C.1.1.9.2-1-16.

Figure C.1.1.9.2-1 UMa FR1 50% UEs indoor DL-TDOA.

Figure C.1.1.9.2-2. UMa FR1 100% UEs outdoor DL-TDOA.

Figure C.1.1.9.2-3. UMi FR1 DL-TDOA.

Figure C.1.1.9.2-4. UMi FR2 DL-TDOA.

Figure C.1.1.9.2-5. UMi FR1 UL-TDOA.

Figure C.1.1.9.2-6. IOO FR1 DL-TDOA.

Figure C.1.1.9.2-7. IOO FR2 DL-TDOA.

Figure C.1.1.9.2-8. IOO FR1 UL-TDOA.

Figure C.1.1.9.2-9 InF-SH FR1 DL-TDOA.

Figure C.1.1.9.2-10. InF-SH FR1 UL-TDOA.

Figure C.1.1.9.2-11. InF-DH FR1 DL-TDOA.

Figure C.1.1.9.2-12. InF-DH FR1 UL-TDOA.

Figure C.1.1.9.2-13 InF-DH FR2 DL-TDOA.

Figure C.1.1.9.2-14 InF-DH FR2 UL-TDOA.

Figure C.1.1.9.2-15. InF-SH FR2 DL-TDOA.

Figure C.1.1.9.2-16. InF-SH FR2 UL-TDOA

##### C.1.1.9.3	Observations on Rel-16 NR positioning accuracy

Table C.1.1.9.3-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table C.1.1.9.3-1: Rel.16 NR positioning – horizontal accuracy performance summary [20]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| [20], source1, UMa, FR1, DL-TDOA (50% UEs indoor) | 11.97m (1 occasion) 7.04m (9 occasions) |
| [20], source1, UMa, FR1, DL-TDOA (100% UEs outdoor) | 4.5m (1 occasion) 2.5m (9 occasions) |
| [20], source2, UMi, FR1, DL-TDOA | 1.7m (1 occasion) 1.06m (9 occasions) |
| [20], source3, UMi, FR1, UL-TDOA | 3.10m (1 occasion) 2.43m (9 occasions) |
| [20], source4, UMi, FR2, DL-TDOA | 0.13m (1 occasion) 0.06m (9 occasions) |
| [20], source5, IOO, FR1, DL-TDOA | 1.51m (1 occasion) 0.93m (9 occasions) |
| [20], source6, IOO, FR1, UL-TDOA | 1.52m (1 occasion) 0.98m (9 occasions) |
| [20], source7, IOO, FR2, DL-TDOA | 0.18m (1 occasion) 0.07m (9 occasions) |
| [20], source8, InF-SH, FR1, DL-TDOA | 0.19m (convex hull UEs) 0.36m (all UEs) |
| [20], source9, InF-SH , FR1, UL-TDOA | 0.18m (convex hull) 0.35m (all UEs) |
| [20], source10, InF-DH, FR1, DL-TDOA | 7m (convex hull UEs) 14.3m (all UEs) |
| [20], source11, InF-DH , FR1, UL-TDOA | 7.5m (convex hull UEs) 14.55m (all UEs) |
| [20], source12, InF-SH, FR2, DL-TDOA | 0.0172m(no RX/Tx error-convex hull UEs) 3.34m (8ns Rx/Tx error- convex hull UEs) 0.0349m (no RX/Tx error-all UEs) 3.68m (8ns Rx/Tx error-all UEs) |
| [20], source13, InF-SH , FR2, UL-TDOA | 0.0163m (no RX/Tx error-convex hull UEs) 3.36m (8ns Rx/Tx error-convex hull UEs) 0.0313m (no RX/Tx error-all UEs) 3.85m (8ns Rx/Tx error-all UEs) |
| [20], source14, InF-DH, FR2, DL-TDOA | 7.06m (convex hull UEs) 14.95m (all UEs) |
| [20], source15, InF-DH , FR2, UL-TDOA | 6.98m (convex hull UEs) 13.48m (all UEs) |

#### C.1.1.10	Results from source [17]

##### C.1.1.10.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis are provided in Table C.1.1.10.1-1.

Table C.1.1.10.1-1.

| Parameter |  | Case 1, InF-DH, FR1, DL-TDOA | Case 2, InF-SH, FR1, DL-TDOA | Case 3, InF-DH, FR1, DL-TDOA | Case 4, InF-SH, FR1, DL-TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 18(1-sector) | 18(1-sector) | 18(1-sector) | 18(1-sector) |
| Number of symbols used per occasion |  | 18 | 18 | 18 | 18 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates.. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |
| Additional notes, if any |  | All gNB’s are at height 8m and All UEs are at height 1.5m | All gNB’s are at height 8m and All UEs are at height 1.5m | gNBs Heights: Half of gNBs at 4m height and half at 8m height. UEs Heights: either Fixed UE heights at 1.5m or uniformly distributed U[0.5m,2m] | gNBs Heights: Half of gNBs at 4m height and half at 8m height. UEs Heights: either Fixed UE heights at 1.5m or uniformly distributed U[0.5m,2m] |

| Parameter |  | Case 5, InF-SH, FR2, DL-TDOA | Case 6, InF-SH, FR2, multi-RTT | Case 7, InF-DH, FR2, DL-TDOA | Case 8, InF-DH, FR2, multi-RTT |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 28GHz | 28GHz | 28GHz | 28GHz |
| Subcarrier spacing |  | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth |  | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb2 | Comb2 | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS&SRS-Pos | DL-PRS | DL-PRS&SRS-Pos |
| Number of sites |  | 18(3-sector) | 18(3-sector) | 18(3-sector) | 18(3-sector) |
| Number of symbols used per occasion |  | 54 (2 FDMed PRS symbols with two TRPs ) | 54 (2 FDMed PRS symbols with two TRPs ) on DL/UL | 54 (2 FDMed PRS symbols with two TRPs ) | 54 (2 FDMed PRS symbols with two TRPs ) on DL/UL |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | Not applied (23 dBm Tx) | NA | Not applied (23 dBm Tx) |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting for DL, interference from 4 UEs in UL | Ideal Muting | Ideal muting for DL, interference from 4 UEs in UL |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | multi-RTT, the algebraic solution proposed in -Norrdine, Abdelmoumen. (55). An Algebraic Solution to the Multilateration Problem. 10.13140/RG.2.1.1681.3602. Equal weight is used in the TOA covariance matrix. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | multi-RTT, the algebraic solution proposed in -Norrdine, Abdelmoumen. (55). An Algebraic Solution to the Multilateration Problem. 10.13140/RG.2.1.1681.3602. Equal weight is used in the TOA covariance matrix. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. The earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |
| Additional notes, if any |  |  | The UE’s height is lower than gNBs’ height is assumed as prior knowledge in positioning |  | The UE’s height is lower than gNBs’ height is assumed as prior knowledge in positioning |

| Parameter |  | Case 9, UMI, FR1, DL-TDOA or RTT | Case 10, UMI with Δτ, FR1, DL-TDOA and RTT | Case 11, UMA, FR1, DL-TDOA |  |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz |  |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz |  |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 |  |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS |  |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) |  |
| Number of symbols used per occasion |  | 48 | 48 | 48 |  |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 |  |
| Power-boosting level |  | 0 | 0 | 0 |  |
| Uplink power control (applied/not applied) |  | NA | NA | NA |  |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync |  |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | NA | NA | NA |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |

| Parameter | Case 12, UMi, FR2, DL-TDOA | Case 13, InH, FR2, DL-TDOA |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |
| Carrier frequency | 30GHz | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS |
| Number of sites | 19(3-sector) | 12(3-sector) |
| Number of symbols used per occasion | 58(2 FDMed PRS symbols with two TRPs ) | 36(2 FDMed PRS symbols with two TRPs) |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | No calibration error | No calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60, 180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |
| Additional notes, if any |  |  |

| Parameter |  | Case 14, UMI with Δτ, FR1, RTT | Case 15, UMI with Δτ, FR1, DL-TDOA | Case 16, UMI without Δτ, FR1, RTT | Case 17, UMI without Δτ, FR1, DL-TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz | 4 GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) | 19(3-sector) |
| Number of symbols used per occasion |  | 48 | 48 | 48 | 48 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync |  |
| UE/gNB Tx/Rx Calibration Error |  | The realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings. The Tx errors at gNB are assumed to be independent |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | NA | NA | NA | NA |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |

| Parameter | Case 18, InF-SH, FR2, DL-TDOA | Case 19, InF-DH, FR2, DL-TDOA | Case 20, UMi, FR2, DL-TDOA | Case 21, InH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |
| Carrier frequency | 28GHz | 28GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites | 18(3-sector) | 18(3-sector) | 19(3-sector) | 12(3-sector) |
| Number of symbols used per occasion | 54(2 FDMed PRS symbols with two TRPs ) | 54(2 FDMed PRS symbols with two TRPs ) | 58(2 FDMed PRS symbols with two TRPs ) | 36(2 FDMed PRS symbols with two TRPs) |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. . |  |  |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | The realistic Tx/Rx timing error and/or network sync error according to truncated Gaussian Distribution [-2*T1,2*T1] nsec, as agreed in previous 3GPP RAN1 meetings The Tx errors at gNB are assumed to be independent and T1 = [0, 0.1 0.2 0.5 1 2]. The UE is assumed to has one panel, and thus no Rx timing error with DL-TDOA. |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. In the strongest beam pair scheme, the strongest RX beam is identified based on the criteria of receiving the strongest path with the serving cell. And Tx beam is further identified in respect to the strongest RX beam per sector. |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |
| Additional notes, if any |  |  |  |  |

##### C.1.1.10.2	Positioning accuracy evaluation results

Tables C.1.1.10.2-1 provides summary of NR positioning evaluations results for horizontal location error.

Table C.1.1.1.2-1: Rel.16 NR positioning - horizontal location error results from [17]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, InF FR1 DH ISD20, 100MHz, Link Quality, DL TDOA | Convex UEs | 0.55m | 6.42m | 15.35m | 31.66m |
|  | (Optional) All UEs | 0.67m | 6.51m | 13.37m | 26.8m |
| Case 1, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.04m | 0.07m | 0.16m | 0.44m |
|  | (Optional) All UEs | 0.05m | 0.09m | 0.18m | 0.59m |

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 2, InF FR1 SH ISD50, 100MHz, Link Quality, DL TDOA | Convex UEs | 0.035m | 0.06m | 0.25m | 2.92m |
|  | (Optional) All UEs | 0.04m | 0.09m | 0.49m | 4.16m |
| Case 2, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.03m | 0.065 | 0.16m | 0.28m |
|  | (Optional) All UEs | 0.05m | 0.11m | 0.17m | 0.54m |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 0.96m | 2.18m | 4.48m | 12.66m |
|  | (Optional) All UEs | 1.67m | 5.39m | 12.11m | 30.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 1.33m | 3.56m | 5.32m | 13.1m |
|  | (Optional) All UEs | 1.89m | 3.99m | 7.67m | 17.62m |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Variable UE heights | Convex UEs | 0.03m | 0.05m | 0.1m | 0.22m |
|  | (Optional) All UEs | 0.04m | 0.09m | 0.17m | 0.47m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.03m | 0.05m | 0.08m | 0.12m |
|  | (Optional) All UEs | 0.046m | 0.08m | 0.16m | 0.36m |

| Horizontal Positioning Error (all UEs) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0058 | 0.0077 | 0.011 | 0.019 |
|  | Strongest | 0.0074 | 0.010 | 0.017 | 0.027 |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.0061 | 0.0085 | 0.011 | 0.015 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0054 | 0.0082 | 0.013 | 0.025 |
|  | Strongest | 0.0084 | 0.0132 | 0.0287 | 9.40 |
| Case 8 InF-DH FR2 3d mRTT | Earliest | 0.0061 | 0.0088 | 0.013 | 0.020 |

| Horizontal Positioning Error (convex Hull) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0043 | 0.0059 | 0.0071 | 0.0087 |
|  | Strongest | 0.0057 | 0.0080 | 0.0104 | 0.0170 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0039 | 0.0054 | 0.0070 | 0.0096 |
|  | Strongest | 0.0058 | 0.0098 | 0.0138 | 1.6767 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.4 | 0.6 |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 0.64 | 1.2 | 2 | 3.2 |
| Case 9, UMI, FR1, RTT, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.16 | 0.22 | 0.3 | 0.4 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 3.4 | 5 | 6.2 | 8 |
| Case 10, UMI, FR1,RTT, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 4 | 4.2 | 6.9 | 9.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 5.7 | 8.1 | 9.9 | 17.3 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, LQ Algorithm | (Optional) All UEs | 7.9 | 11.5 | 14.3 | 20.1 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 11, UMA, FR1, DL-TDOA, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Outdoor UEs | 0.31 | 0.5 | 0.8 | 1.5 |
| Case 11, UMA, FR1, RTT, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Indoor UEs | 1.7 | 6.9 | 0.8 | 96 |

| Horizontal Error (All UEs) |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.011 | 0.018 | 0.025 | 0.040 |
|  | Strongest | 0.13 | 0.80 | 2.63 | 6.32 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0058 | 0.0093 | 0.015 | 0.024 |
|  | Strongest | 0.0092 | 0.016 | 0.028 | 0.053 |

| Horizontal Error (convex Hull UEs) |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.0106 | 0.0178 | 0.0251 | 0.0403 |
|  | Strongest | 0.1333 | 0.7972 | 2.6315 | 6.3233 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0035 | 0.0053 | 0.0069 | 0.0113 |
|  | Strongest | 0.0066 | 0.0101 | 0.0160 | 0.0279 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 1 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.4 | 4.8 | 6 | 7.3 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.5 | 4.8 | 6 | 7.4 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.6 | 4.8 | 6 | 7.6 |
| Case 14, UMI, FR1, RTT, With Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 3.7 | 4.8 | 6 | 7.9 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.4 | 5.8 | 7.2 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.1 | 5.4 | 6.9 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 4.4 | 5.8 | 7.2 | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 6.5 | 8.4 | 10.4 | 12.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.38 | 0.52 | 0.66 | 0.87 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.68 | 0.89 | 1.1 | 1.37 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.11 | 1.45 | 1.75 | 2.22 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.38 | 0.52 | 0.66 | 0.87 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.68 | 0.89 | 1.1 | 1.37 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.11 | 1.45 | 1.75 | 2.22 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.41 | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.46 | 0.73 | 0.76 | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.2 | 1.55 | 1.89 | 2.3 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.23 | 0.31 | 0.41 | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 0.46 | 0.73 | 0.76 | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 1.2 | 1.55 | 1.89 | 2.3 |

| Case ID | Beam Pair | Tx T1 | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| Case 18 InF-SH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.01 |
|  |  | 0.1ns | 0.02 | 0.03 | 0.04 | 0.06 |
|  |  | 0.2ns | 0.05 | 0.07 | 0.09 | 0.13 |
|  |  | 0.5ns | 0.14 | 0.19 | 0.27 | 0.40 |
|  |  | 1.0ns | 0.28 | 0.40 | 0.55 | 0.96 |
|  |  | 2.0ns | 0.55 | 0.75 | 1.08 | 1.59 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.02 |
|  |  | 0.1ns | 0.03 | 0.05 | 0.07 | 0.11 |
|  |  | 0.2ns | 0.06 | 0.10 | 0.14 | 0.25 |
|  |  | 0.5ns | 0.19 | 0.29 | 0.47 | 0.94 |
|  |  | 1.0ns | 0.40 | 0.63 | 1.08 | 2.30 |
|  |  | 2.0ns | 0.82 | 1.22 | 1.87 | 5.19 |
| Case 19 InF-DH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.01 | 0.02 |
|  |  | 0.1ns | 0.03 | 0.04 | 0.06 | 0.10 |
|  |  | 0.2ns | 0.06 | 0.08 | 0.12 | 0.24 |
|  |  | 0.5ns | 0.16 | 0.24 | 0.40 | 0.76 |
|  |  | 1.0ns | 0.34 | 0.47 | 0.78 | 2.10 |
|  |  | 2.0ns | 0.74 | 1.17 | 2.15 | 6.31 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.05 | 43.94 |
|  |  | 0.1ns | 0.05 | 0.08 | 3.61 | 36.56 |
|  |  | 0.2ns | 0.10 | 0.21 | 7.91 | 35.28 |
|  |  | 0.5ns | 0.29 | 0.73 | 18.49 | 44.16 |
|  |  | 1.0ns | 0.74 | 1.96 | 18.90 | 40.33 |
|  |  | 2.0ns | 1.73 | 6.42 | 20.03 | 46.31 |
| Case 20 InH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.02 | 0.03 |
|  |  | 0.1ns | 0.05 | 0.08 | 0.13 | 0.31 |
|  |  | 0.2ns | 0.11 | 0.17 | 0.28 | 0.47 |
|  |  | 0.5ns | 0.23 | 0.40 | 0.60 | 1.16 |
|  |  | 1.0ns | 0.51 | 0.71 | 1.19 | 2.04 |
|  |  | 2.0ns | 0.97 | 1.49 | 2.23 | 4.11 |
|  | Strongest beam pair | 0.0ns | 0.01 | 0.01 | 0.03 | 0.21 |
|  |  | 0.1ns | 0.10 | 0.22 | 0.76 | 7.44 |
|  |  | 0.2ns | 0.22 | 0.48 | 1.94 | 8.22 |
|  |  | 0.5ns | 0.70 | 2.42 | 5.24 | 11.37 |
|  |  | 1.0ns | 1.26 | 3.45 | 6.88 | 14.94 |
|  |  | 2.0ns | 2.08 | 4.38 | 8.21 | 18.87 |
| Case 21 UMi, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 | 0.01 | 0.02 | 0.03 |
|  |  | 0.1ns | 0.04 | 0.05 | 0.08 | 0.13 |
|  |  | 0.2ns | 0.07 | 0.10 | 0.13 | 0.20 |
|  |  | 0.5ns | 0.15 | 0.20 | 0.26 | 0.38 |
|  |  | 1.0ns | 0.32 | 0.42 | 0.54 | 0.73 |
|  |  | 2.0ns | 0.63 | 0.82 | 1.02 | 1.38 |
|  | Strongest beam pair | 0.0ns | 0.47 | 3.08 | 7.73 | 16.21 |
|  |  | 0.1ns | 2.10 | 6.20 | 12.72 | 23.54 |
|  |  | 0.2ns | 1.20 | 4.23 | 9.16 | 16.59 |
|  |  | 0.5ns | 1.69 | 3.81 | 7.18 | 19.30 |
|  |  | 1.0ns | 2.08 | 4.00 | 7.50 | 14.27 |
|  |  | 2.0ns | 2.95 | 6.52 | 12.89 | 21.65 |

Table C.1.1.10.2-2 provides summary of NR positioning evaluations results for vertical location error.

Table C.1.1.10.2-2: Rel.16 NR positioning - altitude location error results from [17]

| Vertical Positioning error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 2.75m | 6.79m | 10.12m | 20.6m |
|  | (Optional) All UEs | 4.6m | 8.91m | 17.21m | 38.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 2.35m | 6.78m | 12.5m | 18.44m |
|  | (Optional) All UEs | 3.07m | 7.34m | 13.92m | 22.98m |

| Vertical Positioning error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, , Unequal gNBs heights, Variable UE heights | Convex UEs | 0.183m | 0.33m | 0.75m | 1.89m |
|  | (Optional) All UEs | 0.162m | 0.37m | 0.78m | 2.63m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, Unequal gNBs heights, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.178m | 0.298m | 0.41m | 0.9m |
|  | (Optional) All UEs | 0.19m | 0.34m | 0.64m | 1.34m |

| Vertical (Across All UEs) | Beam Pair | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.037 | 0.048 | 0.061 | 0.084 |
| Case 8 InF-SH FR2 3d mRTT | Earliest | 0.010 | 0.015 | 0.023 | 0.041 |

##### C.1.1.10.3	Observations on Rel-16 NR positioning accuracy

Table C.1.1.10.3-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table C.1.1.10.3-1: Rel.16 NR positioning – horizontal accuracy performance summary

|  |  | 90% |
| --- | --- | --- |
| Case 1, InF FR1 DH ISD20, 100MHz, Link Quality, DL TDOA | Convex UEs | 31.66m |
|  | (Optional) All UEs | 26.8m |
| Case 1, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Convex UEs | 6.92m |
|  | (Optional) All UEs | 8.25m |
| Case 2, InF FR1 SH ISD50, 100MHz, Link Quality, DL TDOA | Convex UEs | 2.92m |
|  | (Optional) All UEs | 4.16m |
| Case 2, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.28m |
|  | (Optional) All UEs | 0.54m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 12.66m |
|  | (Optional) All UEs | 30.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 13.1m |
|  | (Optional) All UEs | 17.62m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Variable UE heights | Convex UEs | 0.22m |
|  | (Optional) All UEs | 0.47m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.12m |
|  | (Optional) All UEs | 0.36m |

| Horizontal Positioning Error (all UEs) | Beam Pair | 90% |
| --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.019 |
|  | Strongest | 0.027 |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.015 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.025 |
|  | Strongest | 9.40 |
| Case 8 InF-DH FR2 3d mRTT | Earliest | 0.020 |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0087 |
|  | Strongest | 0.0170 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0096 |
|  | Strongest | 1.6767 |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.040 |
|  | Strongest | 6.32 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.024 |
|  | Strongest | 0.053 |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.0403 |
|  | Strongest | 6.3233 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0113 |
|  | Strongest | 0.0279 |

| Horizontal Positioning Error |  | 90% |
| --- | --- | --- |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.6 |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 3.2 |
| Case 9, UMI, FR1, RTT, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 8 |
| Case 10, UMI, FR1,RTT, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 9.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 17.3 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, LQ Algorithm | (Optional) All UEs | 20.1 |
| Case 11, UMA, FR1, DL-TDOA, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Outdoor UEs | 1.5 |
| Case 11, UMA, FR1, RTT, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Indoor UEs | 96 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 1 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.3 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.4 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.6 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.9 |

| Horizontal Positioning Error |  | 90% |
| --- | --- | --- |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |

| Horizontal Positioning Error |  |  | 90% |
| --- | --- | --- | --- |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 0.87 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 1.37 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 2.22 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 0.87 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 1.37 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 2.22 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 2.3 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 2.3 |

| Case ID | Beam Pair | Tx T1 | 90% |
| --- | --- | --- | --- |
| Case 18 InF-SH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 |
|  |  | 0.1ns | 0.06 |
|  |  | 0.2ns | 0.13 |
|  |  | 0.5ns | 0.40 |
|  |  | 1.0ns | 0.96 |
|  |  | 2.0ns | 1.59 |
|  | Strongest beam pair | 0.0ns | 0.02 |
|  |  | 0.1ns | 0.11 |
|  |  | 0.2ns | 0.25 |
|  |  | 0.5ns | 0.94 |
|  |  | 1.0ns | 2.30 |
|  |  | 2.0ns | 5.19 |
| Case 19 InF-DH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.02 |
|  |  | 0.1ns | 0.10 |
|  |  | 0.2ns | 0.24 |
|  |  | 0.5ns | 0.76 |
|  |  | 1.0ns | 2.10 |
|  |  | 2.0ns | 6.31 |
|  | Strongest beam pair | 0.0ns | 43.94 |
|  |  | 0.1ns | 36.56 |
|  |  | 0.2ns | 35.28 |
|  |  | 0.5ns | 44.16 |
|  |  | 1.0ns | 40.33 |
|  |  | 2.0ns | 46.31 |
| Case 20 InH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.03 |
|  |  | 0.1ns | 0.31 |
|  |  | 0.2ns | 0.47 |
|  |  | 0.5ns | 1.16 |
|  |  | 1.0ns | 2.04 |
|  |  | 2.0ns | 4.11 |
|  | Strongest beam pair | 0.0ns | 0.21 |
|  |  | 0.1ns | 7.44 |
|  |  | 0.2ns | 8.22 |
|  |  | 0.5ns | 11.37 |
|  |  | 1.0ns | 14.94 |
|  |  | 2.0ns | 18.87 |
| Case 21 UMi, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.03 |
|  |  | 0.1ns | 0.13 |
|  |  | 0.2ns | 0.20 |
|  |  | 0.5ns | 0.38 |
|  |  | 1.0ns | 0.73 |
|  |  | 2.0ns | 1.38 |
|  | Strongest beam pair | 0.0ns | 16.21 |
|  |  | 0.1ns | 23.54 |
|  |  | 0.2ns | 16.59 |
|  |  | 0.5ns | 19.30 |
|  |  | 1.0ns | 14.27 |
|  |  | 2.0ns | 21.65 |

Table C.1.1.10.3-2 captures observations based on NR positioning evaluations results for vertical location error.

Table C.1.1.10.3-2: Rel.16 NR positioning – vertical accuracy performance summary

| Vertical Positioning error |  | 90% |
| --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 20.6m |
|  | (Optional) All UEs | 38.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 18.44m |
|  | (Optional) All UEs | 22.98m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, , Unequal gNBs heights, Variable UE heights | Convex UEs | 1.89m |
|  | (Optional) All UEs | 2.63m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, Unequal gNBs heights, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.9m |
|  | (Optional) All UEs | 1.34m |

| Vertical (Across All UEs) | Beam Pair | 90% |
| --- | --- | --- |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.084 |
| Case 8 InF-SH FR2 3d mRTT | Earliest | 0.041 |

#### C.1.1.11	Results from source [18]

##### C.1.1.11.1	Description of evaluation scenarios

Table C.1.1.11.1-1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | Case 1- config 445 (InF-DH, FR1,UL-TDOA) | Case 2- config1011 (UMi, FR1,UL-TDOA) | Case 3- config config1112 (UMi, FR1,UL-TDOA) |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | UMi (ATOA modelling with mu=-8) | UMi (No ATOA modelling) |
| Carrier frequency | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 4 | KTC = 8 nbSymb = 8 | KTC = 8 nbSymb = 8 (o/w KTC = 4 nbSymb = 4; or KTC = 2 nbSymb = 1) |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port |
| Number of sites | 18 | 19 | 19 |
| Number of symbols used per occasion | 4 | 8 | 8/4/1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | Power boosting by comb factor | Power boosting by comb factor | Power boosting by comb factor |
| Uplink power control (applied/not applied) | +23dBm | +23dBm | +23dBm |
| interference modelling (ideal muting, or other) | No Interference (no resource sharing) | No Interference (no resource sharing) | No Interference (no resource sharing) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |
| Network synchronization assumptions | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Per site (Rx): 3 sectors (3 panels, 5 dual polarized beams 1 TXRU per panel per polarization dimension UE (Tx): Omni pattern per element, 2 dual polarized antennas (1,2,2), random orientation of UE 4 sequential SRS transmissions | Per site (Rx): 3 sectors (= 3 panels, 5 dual polarized beams per panel) 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna One polarization used for UE-TX Random UE orientation (random polarization orientation | Per site (Rx): Omni antenna 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna One polarization used for UE-TX Random UE orientation (random polarization orientation |
| Additional notes, if any | No LOS detection | LOS detection not applicable (number of LOS links not sufficient, all Links are used) | LOS detection not applicable (number of LOS links not sufficient, all Links are used) |

##### C.1.1.11.2	Positioning accuracy evaluation results

| FR1 Results |  |
| --- | --- |
| Figure C.1.1.11.2-1- CDF 2D (horizontal) error for UTDOA in InF for Rel-16 evaluation. | Figure C.1.1.11.2-2 -CDF 2D (horizontal) error for UTDOA in UMi for Rel-16 evaluation |

Table C.1.1.11.2-3: NR positioning enhancements - horizontal location error results from [18]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1- config ID 445 (FR1-InF DH) | Convex UEs | 2.45 | 4.18 | 6.87 | ≥10 |
| Case 3- config ID 1112 (FR1-UMi) | All UEs | 0.87 | 1.39 | 2.07 | 3.24 |
| Case 2- config ID 1011 (FR1-UMi with ATOA) | All UEs | 3.32 | 5.12 | 7.34 | ≥10 |

Table C.1.1.11.2-3: NR positioning enhancements - altitude location error results from [18]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1- config ID 445 (FR1-InF DH) | Convex UEs | 4.54 | 6.33 | 6.33 | ≥10 |
| Case 3- config ID 1112 (FR1-UMi) | All UEs | 8.50 | 8.50 | 8.50 | ≥10 |
| Case 2- config ID 1011 (FR1-UMi with ATOA) | All UEs | 8.46 | 8.46 | ≥10 | ≥10 |

### C.1.2	Physical layer latency analysis for Rel-16

#### C.1.2.1	Results from source [4]

##### C.1.2.1.1	Description of evaluation scenarios

The physical layer latency for the following positioning methods are provided

- UE-A DL-only positioning

- UL-only positioning

- DL E-CID

- UL E-CID

- UE-B DL-only positioning

##### C.1.2.1.2	Latency analysis of NR positioning enhancements

Latency analysis for the Rel.16 NR positioning is provided in Tables C.1.2.1.2-1 to C.1.2.1.2-6.

The latency for multi-RTT positioning should be the maximum between DL-TDOA/DL-AoD and UL-TDOA/UL-AoA, which is the same as DL-TDOA/DL-AoD considering UL delay is much smaller than DL.

Table C.1.2.1.2-1: Rel.16 NR positioning latency [4]

| Case L1, DL-TDOA/DL-AoD/Multi-RTT w/ Gap request and PRS periodicity 20ms Source NW/Destination NW Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-A Initial and Final RRC States CONNECTED Gap is requested, and PRS periodicity is 20ms This can also be used for Multi-RTT positioning. |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for gap request |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Decoding measurement gap request |
| gNB Tx higher layer processing | 3ms | Reconfiguration with a measurement gap of MGRP = 20ms, MGL = 3ms (MG pattern 10) |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| PRS measurement delay | 23 (1 samp.) 83ms (4 samp. CSSF = 1) 143 (4 samp. CSSF = 2) | According to RAN4, assuming PRS periodicity being 20ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 Ti = 20ms as the UE capability T Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5ms-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) |  |

Table C.1.2.1.2-2: Rel.16 NR positioning latency [4]

| Case L2, DL-TDOA/DL-AoD/Multi-RTT w/o Gap request and PRS periodicity 160ms Source NW/Destination NW Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-A Initial and Final RRC States CONNECTED Gap is not requested, and PRS periodicity is 160ms This can also be used for Multi-RTT positioning. |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| PRS measurement delay | 163ms (1 samp.) 643ms (4 samp. CSSF = 1) | According to RAN4, assuming PRS periodicity being 160ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 Ti = 20ms as the UE capability T Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) |  |

Table C.1.2.1.2-3: Rel.16 NR positioning latency [4]

| Case L3, UL-TDOA/UL-AoA Source NW/Destination NW Positioning technique UL-TDOA and/or UL-AoA, type UL Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| SRS measurement delay | 0.5-20ms (1 samp.) 60.5-80ms (4 samp.) | Assuming SRS periodicity is 20ms. Using 1 or 4 samples. |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6.5-26ms (1 samp.) 66.5-86.5ms (4 samp) |  |

Table C.1.2.1.2-4: Rel.16 NR positioning latency [4]

| Case L4, DL E-CID Source NW/Destination NW Positioning technique DL E-CID, type DL, mode UE-A Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| DL E-CID measurement | 0ms | Using only existing RRM measurement |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 8.5-15ms |  |

Table C.1.2.1.2-5: Rel.16 NR positioning latency [4]

| Case L5, UL E-CID Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| AoA measurement delay | 0-20ms | Assuming SRS periodicity is 20ms. Either AoA is not measured or AoA is measured by 1 sample. |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6-26ms |  |

Table C.1.2.1.2-6: Rel.16 NR positioning latency [4]

| Case L6, UE-based DL-TDOA/DL-AoD w/ gap request t and PRS periodicity 20ms Source UE/Destination UE Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-B Initial and Final RRC States CONNECTED MO-LR, i.e. UE as the location consumer |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for gap request |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Decoding measurement gap request |
| gNB Tx higher layer processing | 3ms | Reconfiguration with a measurement gap of MGRP = 20ms, MGL = 3ms (MG pattern 10) |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| PRS measurement delay | 23 (1 samp.) | According to RAN4, assuming PRS periodicity being 20ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 Ti = 20ms as the UE capability T Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| Location calculation | 3ms | UE |
| End trigger |  | Calculation of Location Estimate at the UE |
| Total values | 51-58.5ms (1 samp.) |  |

##### C.1.2.1.3	Latency analysis for Rel.16 solutions

Summary of latency performance analysis is provided in Table 8.1.2.1.3-1.

Table C.1.2.1.3-1: NR positioning enhancements – latency performance summary [4]

| Description Evaluation Case | L1 Latency, ms | Commercial requirements [100]ms are met -Yes/No - If no, provide performance gaps | IIoT requirements of [10ms] are met - Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case L1, DL-TDOA/DL-AoD/Multi-RTT w/ Gap request and PRS periodicity 20ms | 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) | Yes (1 samp.) >=11.5ms (4 samp. CSSF = 1) >=71.5ms (4 samp. CSSF = 2) | >=41.5ms (1 samp.) >=101.5ms (4 samp. CSSF = 1) >=161.5ms (4 samp. CSSF = 2) | Yes (1 samp.) >=11.5ms (4 samp. CSSF = 1) >=71.5ms (4 samp. CSSF = 2) |
| Case L2, DL-TDOA/DL-AoD/Multi-RTT w/o Gap request and PRS periodicity 160ms | 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) | No (1 samp.) No (4 samp. CSSF = 1) | No (1 samp.) No (4 samp. CSSF = 1) | No (1 samp.) No (4 samp. CSSF = 1) |
| Case L3, UL-TDOA/UL-AoA | 6.5-26ms (1 samp.) 66.5-86.5ms (4 samp) | Yes (1 samp.) Yes (4 samp.) | Yes/No (1 samp.) >=56.5ms (4 samp.) | Yes (1 samp.) Yes (4 samp.) |
| Case L4, DL E-CID | 8.5-15ms | Yes | Yes/No | Yes |
| Case L5, UL E-CID | 6-26ms | Yes | Yes/No | Yes |
| Case L6, UE-based DL-TDOA/DL-AoD w/ gap request t and PRS periodicity 20ms | 51-58.5ms (1 samp.) | Yes | >=41ms | Yes |

#### C.1.2.2	Results from source [7]

##### C.1.2.2.1	Description of evaluation scenarios

In this contribution, the following positioning methods are included in physical layer latency analysis,

- DL-TDOA, UE-A

- DL-TDOA, UE-B

- DL-ECID

##### C.1.2.2.2	Latency analysis for Rel.16 solutions

Table C.1.2.2.2-1 physical layer latency analysis for UE-assisted positioning based on DL-TDOA method

| [Case PHY-L1] Source [UE]/Destination [NW] Positioning technique [DL-TDOA],type [DL], mode [UE-A] Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Components | Value Range (ms) | Description of Latency Component |
| Start trigger (step 1) | N/A | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE decodes and applies the Location Request (step 2) | 10 | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| Measurement gap request (step 3) | Corresponding values can be found in right column. | UL user plane latency defined in 37.910 clause 5.7.1.2.2 is applicable to measurement gap request, where the latency includes UE processing delay UL frame alignment (transmission alignment) TTI for UL data packet transmission BS processing delay The grant free based UL transmission is assumed in this contribution, some typical cases can be found in 37.910 clause 5.7.1.2.2. We choose following cases as examples, UL user plane latency for NR FDD with grant free transmission: UL user plane latency (Grant free)(ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 0.43; 0.30. UL user plane latency for NR TDD with grant free transmission: UL user plane latency (Grant free,DDDXU, with ‘X’=’U’ for UL traffic)(ms) – NR TDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 1.09; 0.64. |
| Serving gNB decodes and interprets the measurement gap request (step 4) | 3 | Regarding the RRC processing time at the gNB, 37.910 clause 5.7.1.1.1 defines processing delay in gNB for L2 and RRC. |
| Measurement gap configuration(step 5) | Corresponding values can be found in right column. | DL user plane latency defined in 37.910 clause 5.7.1.2.1 is applicable to measurement gap configuration, where the latency includes BS processing delay DL Frame alignment (transmission alignment) TTI for DL data packet transmission UE processing delay The latency may be different according to various configurations,some typical cases can be found in in 37.910 clause 5.7.1.2.2. We choose following cases as examples, DL user plane latency for NR FDD: DL user plane latency (ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=4 (7OS non-slot); p=0; 0.37; 0.27. DL user plane latency for NR TDD: DL user plane latency (ms – NR TDD (DDDXU,’X’=’D’ for DL traffic); UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=4 (7OS non-slot); p=0; 0.45; 0.32. |
| UE interprets and applies the measurement gap configuration(step 6) | 10 | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| UE positioning measurement(step 7) | 82 ( FR1) or 644 ( FR2) | Measurement period requirements defined in 38.133 clause 9.9.2.5 should be satisfied, more detailed configurations can refer to Table C.1.2.2.2-2. |
| UE positioning measurement transmission(step 8) | Corresponding values can be found in right column. | The same latency is assumed as “Measurement gap request”. |
| End trigger (step 9) | N/A | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | Corresponding values can be found in right column. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.23. FDD and 60KHz SCS; 667.87. TDD and 30KHz SCS; 107.63. TDD and 60KHz SCS; 668.60. |

Table C.1.2.2.2-2 Measurement period requirements for DL-TDOA method

| Parameters | FR1 | FR2 | Additional Explanations |
| --- | --- | --- | --- |
| MGRP ( Measurement Gap Repetition Period ) | 20 ms | 20 ms |  |
| MGL ( Measurement Gap Length ) | 4 ms | 4 ms | PRS is configured in a compact way, we assume the MGL can cover all PRS occasions from different TRPs. |
| TPRS ( DL PRS periodicity ) | 4 ms | 4 ms | All PRS resource sets have the same periodicity. |
| (N,T) ( Duration of DL PRS symbols N in units of ms a UE can process every T ms ) | (4, 20) ms | (4, 20) ms | The DL PRS buffering time fully aligns the MGL. |
| L ( Number of positioning frequency layer ) | 1 | 1 |  |
| Number of TRP | 4 | 4 |  |
| SCS ( Sub-carrier Spacing ) | 30 KHz | 60 KHz | MGL will span 8 slots for FR1, while the value is 16 slots for FR2. |
| LPRS ( the duration of DL PRS symbols within any a window ) | 2 ms | 4 ms | Type II DL PRS buffering capability is assumed. |
| ( carrier-specific scaling factor ) | 1 | 1 |  |
| ( UE Rx beam sweeping factor ) | 1 | 8 |  |
| ( Number of PRS RSTD samples ) | 4 | 4 |  |
| Total measurement period | 82 ms | 644 ms |  |
| Other assumptions | The number of PRS resources in a slot as configured by the assistance data is smaller the UE capability of max number of DL PRS resources that UE can process in a slot. UE is not required to measure PRS resources from multiple TRPs on the same OFDM symbol in FR2. |  |  |

Table C.1.2.2.2-3 physical layer latency for UE-based positioning based on DL-TDOA method

| [Case PHY-L2] Source [UE]/Destination [NW] Positioning technique [DL-TDOA],type [DL], mode [UE-B] Initial and Final RRC States [CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Start trigger | End trigger | Components that are different from UE-assisted positioning in Table C.1.2.2.2-1. | Total values (ms) |
| Transmission of the PDSCH from the gNB carrying the LPP Request Location Information | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message | Update the start and end triggers accordingly. Step 8 should be removed since UE is the location consumer, which is replaced by “ UE location calculation” ( assume the latency is [1 slot ]) All other steps are the same as UE-assisted positioning in Table C.1.2.2.2-1. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.73. FDD and 60KHz SCS; 668.02. TDD and 30KHz SCS; 108.13. TDD and 60KHz SCS; 668.85. |
| Transmission of the PDSCH from the gNB carrying the LPP Request Location Information | Calculation of Location Estimate at the UE | Update the start and end triggers accordingly. Step 8 should be removed since UE is the location consumer, which is replaced by “ UE location calculation” ( assume the latency is [1 slot ]) All other steps are the same as UE-assisted positioning in Table C.1.2.2.2-1. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.30. FDD and 60KHz SCS; 667.82. TDD and 30KHz SCS; 107.08. TDD and 60KHz SCS; 668.51. |
| Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message | Update the start and end triggers accordingly. Step 2 is replaced by “UE decodes and applies the LPP message containing the assistance data”. Another step for “ UE location calculation” ( assume the latency is [1 slot ] ) is added between step 7 and step 8. All other steps are the same as UE-assisted positioning in Table C.1.2.2.2-1. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.73. FDD and 60KHz SCS; 668.02. TDD and 30KHz SCS; 108.13. TDD and 60KHz SCS; 668.85. |
| Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data | Calculation of Location Estimate at the UE | Update the start and end triggers accordingly. Step 2 is replaced by “UE decodes and applies the LPP message containing the assistance data”. Step 8 should be removed since UE is the location consumer, which is replaced by “ UE location calculation” ( assume the latency is [1 slot ] ) All other steps are the same as UE-assisted positioning in Table C.1.2.2.2-1. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.30. FDD and 60KHz SCS; 667.82. TDD and 30KHz SCS; 107.08. TDD and 60KHz SCS; 668.51. |

Table C.1.2.2.2-3 physical layer latency for UE-based positioning based on DL-ECID method

| [Case PHY-L3] Source [UE]/Destination [NW] Positioning technique [DL-ECID],type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | N/A | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE interprets and applies the measurement configuration | 10 | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| UE ECID measurement time | 0 | If RRM measurement is available (i.e. buffered) at UE side, UE can forward the ECID measurement directly without consuming additional time. |
| UE positioning measurement transmission | Corresponding values can be found in right column. | UL user plane latency defined in 37.910 clause 5.7.1.2.2 is applicable to measurement gap request, where the latency includes UE processing delay UL frame alignment (transmission alignment) TTI for UL data packet transmission BS processing delay The grant free based UL transmission is assumed in this contribution, some typical cases can be found in 37.910 clause 5.7.1.2.2. We choose following cases as examples, UL user plane latency for NR FDD with grant free transmission: UL user plane latency (Grant free)(ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 0.43; 0.30. UL user plane latency for NR TDD with grant free transmission: UL user plane latency (Grant free,DDDXU, with ‘X’=’U’ for UL traffic)(ms) – NR TDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 1.09; 0.64. |
| End trigger | N/A | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | Corresponding values can be found in right column. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 10.43 ms. FDD and 60KHz SCS; 10.30 ms. TDD and 30KHz SCS; 11.09 ms. TDD and 60KHz SCS; 10.64 ms. |

#### C.1.2.3	Results from source [8]

##### C.1.2.3.1	Description of evaluation scenarios

##### C.1.2.3.2	Latency analysis for Rel.16 solutions

In this section, we analyse the physical layer latency for both DL-TDOA and UL-TDOA. For DL-TDOA, the physical layer procedure is shown in Figure C.1.2.3.2-1.

Figure C.1.2.3.2-1: PHY procedure for DL-TDOA

According to Figure C.1.2.3.2-1, the total PHY delay could be calculated as

Ttotal=Tgap,req+Tgap,cfg+Tariv+Tprocess+Treport

where,

- Tgap,req denotes the time to require measurement gap, which is 1ms according to the result of URLLC latency in TR 38.824.

- Tgap,cfg denotes the time for measurement gap configuration, which could be assumed as 10ms according to RRC reconfiguration procedure delay in TS 38.331.

- Tariv denotes the delay between the time when DL PRS is received and the time when measurement gap configuration is received, which is related to the maximum value of the configured periodicity of PRS and measurement gap.

- Tprocess denotes the time from UE begins to measure PRS until the measurement result is ready to report. Tprocessis related to UE capability and the number of PRS resources needed to be measured.

- Treport denotes the time for measurement reporting, which includes SR request, PDCCH-based UL grant and PUSCH-based measurement result reporting.

The value of Treport could be assumed as 0.5ms for URLLC according to section 5.2.1 of TR 36.881. The value of Tprocess may be inferred from the minimum periodicity of PRS and measurement gap, since the measurement result has to be reported before the next PRS occasion.

In Rel-16, the periodicity of PRS includes the following values: 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots. For the periodicity of measurement gap, the following values are supported: 20, 40, 80, 160, [320, 640]ms.

For example, assume SCS is 15kHz, the periodicity of PRS is 4 slots and the periodicity of measurement gap is 20ms, the PHY latency could be summarized in the following table.

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning (DL-TDOA) is provided in Table C.1.2.3.2-1.

Table C.1.2.3.2-1: Rel.16 NR positioning latency for DL-TDOA [8]

| Case 1, 15kHz, FR1, DL-TDOA Source UE/Destination NW Positioning technique DL-TDOA, type DL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger | 1 | Measurement gap request |
| Measurement gap configuration | 10 | Measurement gap configuration delay by RRC |
| PRS arrival delay | ≤X1, X1=max⁡{TcPRS,Tcmgap} (e.g. X1=4 when TcPRS=4 and Tcmgap=20) | DL PRS arrived for measurement, which is related to the maximum value of the configured periodicity of PRS TcPRS and the configured periodicity of measurement gap Tcmgap |
| Process time | ≤X2, X2=max⁡{min⁡(TPRS),min⁡(Tmgap)} (X2=20, as min⁡(TPRS)=4 and min⁡(Tmgap)=20) | UE begins to measure PRS until the measurement result is ready. This value depends on UE capability the number of PRS resources to be measured. The value is assumed to be determined by the supported minimum periodicity of PRS TPRS and measurement gap Tmgap. |
| End trigger | 0.5 | SR request, PDCCH UL grant and PUSCH-based measurement result reporting are included, wherein URLLC is assumed |
| Total values | ≤51.5 |  |

For UL-TDOA, the physical layer procedure is shown in Figure C.1.2.3.2-2.

Figure C.1.2.3.2-2: PHY procedure for UL-TDOA

The total delay could be calculated as

Ttotal=TSRS,act+Tprep+Tprocess

where,

- TSRS,act denotes the time to activate the SRS transmission.

- Tprep denotes the delay from effective time of SRS activation until UE begins to transmit SRS, which is related to the value of the configured periodicity of SRS.

- Tprocess denotes the time from gNB begins to measure SRS until the measurement result is ready, which is related to UE capability and the number of SRS resources needed to be measured.

If SRS is activated by MAC-CE, 3ms delay could be assumed. Similar to Case 1, the value of Tprocess may be inferred from the minimum periodicity of SRS. In Rel-16, the periodicity of SRS for 15 kHz SCS includes the following values:

- {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots. Assume the periodicity of SRS is 1 slot, the PHY latency could be get.

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning (UL-TDOA) is provided in Table C.1.2.3.2-2.

Table C.1.2.3.2-2: Rel.16 NR positioning latency for UL-TDOA [8]

| Case 2, 15kHz, FR1, UL-TDOA Source UE/Destination NW Positioning technique UL-TDOA, type UL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger | 3 | UE SRS transmission is activated |
| UL SRS arrival delay | ≤X1, X1=TcSRS (e.g. X1=1 when TcSRS=1) | UL SRS arrived for measurement, which is related to the value of the configured periodicity of SRS TcSRS |
| End trigger | ≤X2, X2=min⁡(TSRS) (X2=1, as min⁡(TSRS)=1) | gNB begins to measure SRS until the measurement result is ready. This value depends on gNB capability, and the number of SRS resources to be measured. The value is assumed to be determined by the minimum periodicity of SRS, TSRS. |
| Total values | ≤5 |  |

#### C.1.2.4	Results from source [13]

##### C.1.2.4.1	Description of evaluation scenarios

We analyse the case of Network Initiated location request where the positioning method is DL-TDOA/DL-AoD in Case 1/2 and also for UL-TDOA/UL-AoA in Case 3. We only consider the RRC state as connected mode. Note that this analysis assumes that the higher layer components are taken into account in separate analysis and the definition of start and stop times for PHY layer latency are used. For Case 3 we assume that SRS-Pos already being configured (e.g., higher layer latency takes this into account, see RAN2 analysis).

##### C.1.2.4.2	Latency analysis for Rel.16 solutions

| Case 1 (FR1)/ Case 2 (FR2) Source: NW/Destination: NW Positioning technique: DL-TDOA, type: DL, mode: UE-A Initial and Final RRC States: Initial/Final - CONNECTED |  |  |  |
| --- | --- | --- | --- |
|  | Latency Component | Value Range | Description of Latency Component |
| Start trigger |  |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| Step 1: Processing of “LPP Request Location Information message” | 1a. Processing of PDSCH | FR1: [3-8 symbols] FR2: [20-24 symbols] | UE processing – based on N1 values from 38.214 |
|  | 1b. Processing of LPP message | [10 ms] | From 38.331 |
| Step 2 (optional): measurement gap request and configuration | 2a. SR (for MG request) preparation and transmission | FR1: [2 symbols – 80 slots] FR2: [2 symbols – 640 slots] | generate SR, as the message of measurement gap request may be new data coming Wait for SR transmission occasion and transmit SR to gNB |
|  | 2b. SR processing at gNB | [1 symbol] | SR decoding, should be much smaller than N2. |
|  | 2c. UL grant (for MG request) preparation and transmission | [3 symbols – 1 slot ] | UL grant generation and waiting for scheduling opportunity (based on CORESET configuration). Typical value. |
|  | 2d. UE processing of PDCCH and UL preparation | FR1: [5-10 symbols] FR2: 36 symbols | UL data includes requested measurement gap request. Based on N2 values from 38.214. |
|  | 2e. Scheduling delay - K2 | [0-32 slots] | Scheduling delay from UL grant to PUSCH |
|  | 2f. PUSCH reception and decoding | [1-2 symbols] | Including 1. PUSCH (containing MG request information) processing at gNB |
|  | 2g. RRC processing of MG request | [10 ms] | gNB RRC processing time |
| Step 3 (optional): measurement gap configuration | 3a. gNB prepares to transmits RRC (for MG configuraiton) | [3 symbols – 1 slots] | PDCCH and PDSCH preparation |
|  | 3b. UE PHY processing for RRC (for MG configuraiton) | FR1: [3-8 symbols] FR2: [20-24 symbols] | PDSCH decoding; not specified, but should be smaller than N1 |
|  | 3c. UE time to applie the MG configuration | [10 ms] | Based on 38.331 |
| Step 4: PRS processing | 4a. Transmission of DL PRS | [4-10240 slots] | Worst case time between periodic DL PRS based on values from 38.211 |
|  | 4b. Processing of DL PRS at UE | [8-1280 ms] | Assume average value of T=8 (lowest value) is reported by UE as capability and UE is able to process all DL PRS resources (i.e., N is sufficient). |
| Step 5: measurement report processing | 5a. Transmit SR | FR1: [2 symbols – 80 slots] FR2: [2 symbols – 640 slots] | Wait for SR transmission occasion and transmit SR to gNB |
|  | 5b. Processing of SR at gNB | [1 symbol] | SR decoding |
|  | 5c. UL grant preparation and transmission of UL grant by gNB | [3 symbols – 1 slot ] | UL grant generation and waiting for scheduling opportunity (based on CORESET configuration). Typical value. |
|  | 5d. UE processing of PDCCH and UL preparation | FR1: [5-10 symbols] FR2: 36 symbols | Based on N2 values from 38.214. |
|  | 5e. Scheduling delay - K2 | [0-32 slots] | Scheduling delay from UL grant to PUSCH |
|  | 5f. gNB processing of RSTD Report | [1-2 symbol] |  |
| End trigger |  |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total PHY latency |  | FR1: [44.35 – 10500] ms FR2: [35.08 – 2118.93 ms] | Assuming 15 kHz SCS for FR1 and 120 kHz SCS for FR2 |

| Case 3 Source: NW/Destination: NW Positioning technique: UL-TDOA, type: UL, mode: UE-A Initial and Final RRC States: Initial/Final - CONNECTED |  |  |  |
| --- | --- | --- | --- |
|  | Latency Component | Value Range | Description of Latency Component |
| Start trigger |  |  | Reception by the gNB of the NRPPa measurement request message (assumption is that UE has already been configured with periodic SRS-Pos) |
| Step 1: SRS-pos configuration | 1a. gNB prepares and transmits RRC (for SRS-pos configuraiton) | [3 symbols – 1 slot ] | PDCCH and PDSCH preparation |
|  | 1b. UE PHY processing for RRC | [3-24 symbols] | PDSCH decoding |
| Step 2: SRS-pos processing | 2a. Wait for transmission of SRS-Pos | [1-81920 slots] | Periodicity of SRS-Pos |
|  | 2b. Transmission of SRS-Pos | [2 symbols- 8 symbols] | Assuming effective comb-1. In FR2 case this will be much longer (e.g., multiple resources per set) |
|  | 2c. Processing of SRS-Pos at gNB/RP-only | [1-5 slots] |  |
| Step 3: positioning report | 3a. Preparation of UL measurement report | [3 symbols ] | Typical value |
| End trigger |  |  | The transmission by the gNB of the NRPPa measurement response message |
| Total PHY latency |  | [2.78 – 81928.5] ms | Assuming 15 kHz SCS |

#### C.1.2.5 Results from source [11]

##### C.1.2.5.1 Description of evaluation scenarios

The physical layer latency evaluations are captured for the following Rel-16 DL-based positioning scenarios:

- Case ID: 1- DL-TDOA / DL-AoD, UE-Assisted (UE-A) Positioning with MG configuration.

- Case ID: 2- DL-TDOA/ DL-AoD, UE-Assisted (UE-A) Positioning without MG configuration.

- Case ID: 3- DL-TDOA/ DL-AoD, UE-Based (UE-B) Positioning with MG configuration.

- Case ID: 4- DL-TDOA/ DL-AoD, UE-Based (UE-B) Positioning without MG configuration.

##### C.1.2.5.2 Latency analysis for Rel.16 solutions

Table C.1.2.5.2-1 presents the UE-Assisted DL-TDOA/DL-AoD physical layer latency evaluations by taking into account the minimum and cautious latency estimates (within the value range) of the various delay components (starting with the earliest component) with a Measurement Gap (MG) configuration, while Table C.1.2.5.2-2 presents the same scenario without a Measurement Gap (MG) configuration.

Table C.1.2.5.2-3 presents the UE-Based DL-TDOA/DL-AoD physical layer latency evaluations by taking into account the minimum and cautious latency estimates (within the value range) of the various delay components (starting with the earliest component) with a Measurement Gap (MG) configuration, while Table C.1.2.5.2-4 presents the same scenario without a Measurement Gap (MG) configuration.

Table C.1.2.5.2-1: Rel.16 NR positioning latency [11]

| Case ID: 1, Scenario: UE-Assisted Positioning with MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA or R.16 DL-AoD Source: NW, Destination: NW Positioning technique(s): DL-TDOA or DL-AoD, Type: DL, Mode: UE-Assisted (UE-A) Initial RRC and Final RRC States: CONNECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Latency Component | Value Range (ms) |  | Description of Latency Component |
|  |  | Min Value | Cautious Estimate7 |  |
| PHY Start |  |  |  |  |
| 1.1. | Start trigger (Air Interface1 Latency) | 1 – FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message (Air Interface Latency) Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 – FR2: 120 kHz SCS |  |
| 1.2. | UE Processing4 of LPP Request Location Information message | 3 | 10 | UE Processing of the PDSCH carrying the LPP Request Location Information message |
| PUSCH Preparation Time2 |  |  |  |  |
| 2. | UL Transmission of Measurement Gap Request (SR-based Scheduling) | 1 | 3.8 | Cautious Estimate = max (FDD, TDD) = max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| Measurement Gap Configuration |  |  |  |  |
| 3.1 | Measurement Gap Request (Air Interface Latency1) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the MG Request/Need message (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2:120 kHz SCS | 0.5 – FR2: 120 kHz SCS |  |
| 3.2 | gNB Processing3 of Measurement Gap Request | 3 | 10 | gNB Processing of the PUSCH carrying the MG Request message |
| 3.3 | Measurement Gap Configuration (Air Interface1 Latency) | 1 - FR1:30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the Measurement Gap Configuration (Air Interface Latency) |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 – FR2: 120 kHz SCS |  |
| 3.4 | UE Processing4 of Measurement Gap Configuration message | 3 | 10 | UE Processing of the PDSCH carrying the Measurement Gap Configuration message |
| DL-PRS Configuration within TMGRP |  |  |  |  |
| 4.1 | MG Gap Repetition Period (TMGRP), NOcc5= 1 | 1×20 | 1×160 | UE processing of PRS within a MG |
| 4.2 | UE Processing of DL-PRS Units (NProc,T) in addition to the TMGRP | (NProc,T) = (6,8) 0 | (NProc,T)6 = (20, 160) 20 | Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. UE Processing time =max (0, T-(TMGRP-N)) |
| PUSCH Preparation Time2 |  |  |  |  |
| 5.1 | UL Transmission of Measurement Report without UL grant | 1 | 3.8 | Cautious Estimate = max (FDD, TDD) = max (2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| PHY End |  |  |  |  |
| 6.1 | End Trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the LPP Provide Location Information (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 – FR2: 120 kHz SCS |  |
| 6.2 | End Trigger (Processing) | 3 | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message containing the measurement report. |
|  | Total Values | 38 - FR1: 30 kHz SCS | 235.6- FR1: 30 kHz SCS |  |
|  |  | 35 – FR2: 120 kHz SCS | 229.6 – FR2: 120 kHz SCS |  |
| Notes: 1: The Air Interface is a function dependent on the selected numerology, TDD/FDD configuration, and FR1/FR2 band. 2: The PUSCH processing time is based on the SR-based scheduling of PUSCH in [4]. 3: gNB Processing: The minimum processing estimates in [4] served as a guideline. The cautious estimates of the gNB processing times of the RRC messages are based on the delay requirements in [5]. The value is used for evaluation only; gNB processing delay may vary depending on implementation/deployment. 4: UE Processing: The minimum estimate in [4] served as a guideline. The cautious estimates of the UE processing times of the RRC messages are based on the delay requirements in [5]. 5: The number of DL-PRS occasions, NOcc = 1, although this may not be a typical value for obtaining accurate positioning measurements. The positioning measurements would need to be averaged over a higher number of DL-PRS occasions for accurate measurements. 6: The symbol processing time of N=20 ms was selected to align with the associated MGL=20, which has a corresponding TMGRP = 160 ms. 7: The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |  |  |  |

Table C.1.2.5.2-2: Rel.16 NR positioning latency [11]

| Case ID: 2, Scenario: UE-Assisted Positioning without MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA or R.16 DL-AoD Source: NW, Destination: NW Positioning technique(s): DL-TDOA, DL-AoD, Type: DL, Mode: UE-Assisted (UE-A) Initial RRC and Final RRC States: CONNECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Latency Component | Value Range (ms) |  | Description of Latency Component |
|  |  | Min Value | Cautious Estimate6 |  |
| PHY Start |  |  |  |  |
| 1.1. | Start trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message (Air Interface Latency) Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 - FR1: 120 kHz SCS |  |
| 1.2. | UE Processing4 of LPP Request Location Information message | 3 | 10 | UE Processing of the PDSCH carrying the LPP Request Location Information message |
| DL-PRS Configuration = TPRS × NOcc5 |  |  |  |  |
| 2.1 | DL-PRS Resources (DL-PRS Periodicity (TPRS) & (NOcc5= 1)) | 1×4 | 1×5120 | TPRS of 5120 selected as cautious estimate. It is a function of the PRS periodicity and number of occasions (NOcc5=1) over which the measurements will be averaged. |
| 2.2 | UE Processing4 of DL-PRS Units (NProc,T) | 4 | 0 | Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz. Minimum estimate is based on minimum T of 8 ms while cautious estimate is based on maximum T of 1280 ms. Processing time =max (0, T-TPRS ) |
| PUSCH Preparation Time2 |  |  |  |  |
| 3.1 | UL Transmission of Measurement Report without UL grant | 1 | 3.8 | Cautious Estimate = max (FDD, TDD)= max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| PHY End |  |  |  |  |
| 4.1 | End Trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the LPP Provide Location Information (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 4.2 | End Trigger (Processing) | 3 | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message containing the measurement report. |
|  | Total Values | 17 - FR1: 30 kHz SCS | 5147.8 - FR1: 30 kHz SCS |  |
|  |  | 15.5 - FR2: 120 kHz SCS | 5144.8 - FR2: 120 kHz SCS |  |
| Notes: 1: The Air Interface is a function dependent on the selected numerology, TDD/FDD configuration, and FR1/FR2 band. 2: The PUSCH processing time is based on SR-based scheduling of PUSCH in [4]. 3: gNB Processing: The minimum estimate in [4] served as a guideline. The value is used for evaluation only; gNB processing delay may vary depending on implementation/deployment. 4: UE Processing: The minimum estimate in [4] served as a guideline. The cautious estimates UE processing times of the RRC messages are based on the delay requirements in [5]. 5: The number of DL-PRS occasions, NOcc = 1, although this may not be a typical value for obtaining accurate positioning measurements. The positioning measurements would need to be averaged over a higher number of DL-PRS occasions for accurate measurements. 6: The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |  |  |  |

Table C.1.2.5.2-3: Rel.16 NR positioning latency [11]

| Case ID: 3, Scenario: UE-based Positioning with MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA or R.16 DL-AoD Source: UE, Destination: UE Positioning technique(s): DL-TDOA, DL-AoD Type: DL, Mode: UE-Based (UE-B) Initial RRC and Final RRC States: CONNECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Latency Component | Value Range (ms) |  | Description of Latency Component |
|  |  | Min Value | Cautious Estimate9 |  |
| PHY Start (If applicable)7 |  |  |  |  |
| 1.1. | Start trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message (Air Interface Latency) Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 1.2. | UE Processing4 of LPP Request Location Information message | 3 | 10 | UE Processing of the PDSCH carrying the LPP Request Location Information message |
| PHY Start (Alt. 1): PUSCH Preparation Time2 |  |  |  |  |
| 2.1 | UL Transmission Preparation of Measurement Gap Request (SR-based Scheduling) | 1 | 3.8 | Cautious Estimate = max (FDD, TDD)= max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| Measurement Gap Configuration |  |  |  |  |
| 3.1 | Measurement Gap Request (Air Interface Latency1) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the MG Request/Need message (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 3.2 | gNB Processing3 of Measurement Gap Request | 3 | 10 | gNB Processing of the PUSCH carrying the MG Request message |
| 3.3 | Measurement Gap Configuration (Air Interface Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the Measurement Gap Configuration (Air Interface Latency) |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 3.4 | UE Processing4 of Measurement Gap Configuration message | 3 | 10 | UE Processing of the PDSCH carrying the Measurement Gap Configuration message |
| With MG Gap of DL-PRS Configuration |  |  |  |  |
| 4.1 | MG Gap Repetition Period (TMGRP), , NOcc5= 1 | 1×20 | 1×160 | UE processing of PRS within a MG |
| 4.2 | UE Processing of DL-PRS Units (NProc,T) in addition to the TMGRP | (NProc,T) = (6,8) 0 | (NProc,T)8 = (20, 160) 20 | Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. UE Processing time =max (0, T-(TMGRP-N)) |
| PUSCH Preparation Time (If applicable) 2 |  |  |  |  |
| 5.1 | UL Transmission of Measurement Report without UL grant | 1 | 3.8 | Cautious Estimate = max (FDD, TDD)= max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| PHY End (If applicable) 7 |  |  |  |  |
| 6.1 | End Trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the LPP Provide Location Information (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 6.2 | End Trigger (Processing) | 3 | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message containing the UE location estimate. |
|  | Total Values without Request and Provide Location Information Message Components = {2-5} | 29 - FR1: 30 kHz SCS | 207.8- FR1: 30 kHz SCS |  |
|  |  | 27.5 - FR2: 120 kHz SCS | 204.8 - FR2: 120 kHz SCS |  |
|  | With MG Gap and with Request and Provide Location Information message (If applicable) Components = {1-6} | 38 - FR1: 30 kHz SCS | 265.6- FR1: 30 kHz SCS |  |
|  |  | 35 - FR2: 120 kHz SCS | 259.6 - FR2: 120 kHz SCS |  |
| Notes: 1: The Air Interface is a function dependent on the selected numerology, TDD/FDD configuration, and FR1/FR2 band. 2: The PUSCH processing time is based on SR-based scheduling of PUSCH in [4]. 3: gNB Processing: The minimum estimate in [4] served as a guideline. The value is used for evaluation only; gNB processing delay may vary depending on implementation/deployment. 4: UE Processing: The minimum estimate in [4] served as a guideline. The cautious estimates UE processing times of the RRC messages are based on the delay requirements in [5]. 5: The number of DL-PRS occasions, NOcc = 1, although this may not be a typical value for obtaining accurate positioning measurements. The positioning measurements would need to be averaged over a higher number of DL-PRS occasions for accurate measurements. 6: For the evaluation scenario, the starting trigger for UE-based positioning considers the transmission of the PUSCH carrying the MG Request from the UE. 7: The LPP Request Location Information and Provide Location Information messages are considered in the evaluation as the UE may transmit the location estimate to the LMF, when applicable. 8: The symbol processing time of N=20 ms was selected to align with the associated MGL=20, which has a corresponding TMGRP = 160 ms. 9: The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |  |  |  |

Table C.1.2.5.2-4: Rel.16 NR positioning latency [11]

| Case ID: 4, Scenario: UE-based Positioning without MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA or R.16 DL-AoD Source: UE, Destination: UE Positioning technique(s): DL-TDOA, DL-AoD Type: DL, Mode: UE-Based (UE-B) Initial RRC and Final RRC States: CONNECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Latency Component | Value Range (ms) |  | Description of Latency Component |
|  |  | Min Value | Cautious Estimate8 |  |
| PHY Start (If applicable)7 |  |  |  |  |
| 1.1. | Start trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message (Air Interface Latency) Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 1.2. | UE Processing4 of LPP Request Location Information message | 3 | 10 | UE Processing of the PDSCH carrying the LPP Request Location Information message |
| DL-PRS Configuration = TPRS × NOcc5 |  |  |  |  |
| 2.1 | DL-PRS Resources (DL-PRS Periodicity (TPRS) & (NOcc5= 1)) | 1×4 | 1×5120 | TPRS of 5120 selected as cautious estimate. It is a function of the PRS periodicity and number of occasions (NOcc5=1) over which the measurements will be averaged. |
| 2.2 | UE Processing4 of DL-PRS Units (N,T) | 4 | 0 | Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz. Minimum estimate is based on minimum T of 8 ms while cautious estimate is based on maximum T of 1280 ms. Processing time =max(0, T-TPRS ) |
| PUSCH Preparation Time (If applicable) 2 |  |  |  |  |
| 3.1 | UL Transmission of Measurement Report without UL grant | 1 | 3.8 | Cautious Estimate = max (FDD, TDD)= max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| PHY End (If applicable) 7 |  |  |  |  |
| 4.1 | End Trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the LPP Provide Location Information (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 4.2 | End Trigger (Processing) | 3 | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message containing the UE location estimate. |
|  | Total Values Components = {2} | 8 | 5120 |  |
|  | Without MG Gap and with Request and Provide Location Information message (If applicable) Components = {1-4} | 17 - FR1: 30 kHz SCS | 5147.8 - FR1: 30 kHz SCS |  |
|  |  | 15.5 - FR2: 120 kHz SCS | 5144.8 - FR2: 120 kHz SCS |  |
| Notes: 1: The Air Interface is dependent on the selected numerology, TDD/FDD configuration, and FR1/FR2 band. 2: The PUSCH processing time is based on SR-based scheduling of PUSCH in [4]. 3: gNB Processing: The minimum estimate in [4] served as a guideline. The value is used for evaluation only; gNB processing delay may vary depending on implementation/deployment. 4: UE Processing: The minimum estimate in [4] served as a guideline. The cautious estimates UE processing times of the RRC messages are based on the delay requirements in [5]. 5: The number of DL-PRS occasions, NOcc = 1, although this may not be a typical value for obtaining accurate positioning measurements. The positioning measurements would need to be averaged over a higher number of DL-PRS occasions for accurate measurements. 6: For the evaluation scenario, the starting trigger for UE-based positioning considers the transmission of the PUSCH carrying the MG Request from the UE. 7: The LPP Request Location Information and Provide Location Information messages are considered in the evaluation as the UE may transmit the location estimate to the LMF, when applicable. 8: The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |  |  |  |

#### C.1.2.6	Results from source [5]

##### C.1.2.6.1	Description of evaluation scenarios

##### C.1.2.6.2	Latency analysis for Rel.16 solutions

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest one. Latency analysis for the Rel.16 NR positioning is provided in Table C.1.2.6.2-1 to Table C.1.2.6.2-7.

Table C.1.2.6.2-1: Rel.16 NR positioning latency [5]

| [Case 1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE, Network]/Destination [UE,Network] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 64~ | The minimum total value is 64ms if only consider the minimum value for every step. |

Table C.1.2.6.2-2: Rel.16 NR positioning latency [5]

| [Case 1-1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE, Network]/Destination [UE,Network] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-A], Initial and Final RRC States [Idle/inactive] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Additional latency for idle/inactive state to connected state | Inactive: [21.3-178.5] Idle:[40-200] | The latency for inactive to connected is 11.3-18.5 based on Table 5.7.2.1-1 of 37.910 which don’t consider the periodicity of PRACH, So the total latency is 21.3-178.5 as the periodicity of PRACH is 10 ms -160 ms; In addition, compared to the latency for inactive to connected, the latency for idle to connected may have additional high layer steps, such as steps of gNB to AMF interactive, AMF processing etc. Therefore, we assume that additional 20ms for higher layer latency is needed at least, so the latency for idle to connected is assumed as 40~200ms. |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 85.3/104~ | The minimum total value is 85.3 for inactive and 104ms for idle state if only consider the minimum value for every step. |

Table C.1.2.6.2-3: Rel.16 NR positioning latency [5]

| [Case 2], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [Network]/Destination [Network] Positioning technique [DL-TDOA], type [DL], mode [UE-B], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | For Physical Layer Latency Start time of UE based method, Alt.2 is selected. Transmission of the PDSCH from the gNB carrying LPP message containing the assistance data. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Calculation of Location Estimate at the UE | 2 | Calculation of Location Estimate at the UE |
| 8: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 66~ | The minimum total value is 66ms if only consider the minimum value for every step. |

Table C.1.2.6.2-4: Rel.16 NR positioning latency [5]

| [Case 3], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE]/Destination [UE] Positioning technique [DL-TDOA], type [DL], mode [UE-B], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying LPP message containing the assistance data. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Calculation of Location Estimate at the UE | 2 | Calculation of Location Estimate at the UE |
| Total values | 55.5~ | The minimum total value is 55.5ms if only consider the minimum value for every step. |

Table C.1.2.6.2-5: Rel.16 NR positioning latency [5]

| [Case 4], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA] Source [UE,Network]/Destination [UE,Network] Positioning technique [UL-TDOA/UL-AoA], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 10 | Reception and Successful decode the NRPPa measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 1: UL measurement &process delay. | FR1: 0.5-2560 FR2: 0.125~640 | For case 1: one occasion and without considering beam sweeping UL measurement equals to the periodicity of SRS SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560}slots 15kHz 1ms-2560ms 30kHz 0.5ms-1280ms 60kHz 0.25ms-640ms 120kHz 0.125ms-320ms For case 2: Multiple positioning occasion and beam sweeping UL measurement equals to the periodicity of SRS×NRxBeam×NPosOccasion gNB processing delay is assumed as zero |
| End trigger | 0.5 | The transmission by the gNB of the NRPPa measurement response message. Which value less than the DL/UL data transmission time. It can be seen as the gNB processing time (Tprocess time_pdsch) and smaller than 0.5ms. |
| Total values | 30.5~ | The minimum total value is 30.5 ms if the periodicity of SRS is 20ms and the same as the DL minimum periodicity. |

Table C.1.2.6.2-6: Rel.16 NR positioning latency [5]

| [Case 5], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA] Source [UE,Network]/Destination [UE,Network] Positioning technique [UL-TDOA/UL-AoA], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 10 | Reception and Successful decode the NRPPa SRS activation and measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 1. Transmission of the PDCCH from the gNB carrying activate SRS message | 0.5 | Transmission of the PDCCH from the gNB carrying activate SRS message. Which value less than the DL/UL data transmission time. It needs to consider Tprocess time_pdcch, Talign time andTPDCCH as below and smaller than 0.5ms. - the gNB processing time (Tprocess time_pdcch )and the alignment delay(Talign time ) - The alignment delay includes the gap between the two consecutive PDCCH monitoring occasions for FDD, the UL/DL configuration for TDD, and the scheduling constraint due to the slot boundaries. - PDCCH duration (TPDCCH ) |
| 2: UL measurement &process delay. | 0-32 | The slot offset of aperiodic is 0-32 slots 15kHz 0ms-32ms 30kHz 0ms-16ms 60kHz 0ms-8ms 120kHz 0ms-4ms |
| End trigger | 0.5-1 | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 11~ | The minimum total value is 11ms if only considers the minimum value for every step. |

Table C.1.2.6.2-7: Rel.16 NR positioning latency [5]

| [Case 6], [IIoT/ Commercial], [Frequency Band], [Multi-RTT] Source [UE,Network]/Destination [UE,Network] Positioning technique [Multi-RTT], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 8. Successful decoding of the PUSCH carrying the LPP Provide Location Information message | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| 9. The alignment delay is the gap between End trigger of DL positioning and Start trigger of UL positioning | [TBD] | TAlign_DL_UL: The alignment delay is the gap between End trigger of DL positioning and Start trigger of UL positioning. If TAlign_DL_UL>0 , Start trigger timing of UL positioning is later than End trigger timing of DL positioning and the multi-RTT latency will bigger than the sum(TUE assisted DL only, TUE assisted UL only). If - TUE assisted DL only < TAlign_DL_UL<0, Start trigger timing of UL positioning is earlier than End trigger timing of DL positioning and the multi-RTT latency will less than the sum(TUE assisted DL only, TUE assisted UL only). |
| 10. Reception and Successful decode the NRPPa measurement request message from the LMF at gNB | 10 | Reception and Successful decode the NRPPa measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 11: UL measurement &process delay. | 0.125-2560 | For case 1: one occasion and without considering beam sweeping UL measurement equals to the periodicity of SRS SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560} slots 15kHz 1ms-2560ms 30kHz 0.5ms-1280ms 60kHz 0.25ms-640ms 120kHz 0.125ms-320ms For case 2: Multiple positioning occasion and beam sweeping UL measurement equals to the periodicity of SRS×NRxBeam×NPosOccasion gNB processing delay is assumed as zero |
| End trigger | 0.5 | The transmission by the gNB of the NRPPa measurement response message. Which value less than the DL/UL data transmission time. It can be seen as the gNB processing time (Tprocess time_pdsch) and smaller than 0.5ms. |
| Total values | 94.5+TAlign_DL_UL |  |

#### C.1.2.7	Results from source [12]

##### C.1.2.7.1	Description of evaluation scenarios

The following positioning method are included for physical layer latency analysis

- UE-assisted DL-based method: DL-TDOA

- UE-assisted UL-based method: UL-TDOA

- UE- based method: UE-based positioning calculation

##### C.1.2.7.2	Latency analysis for Rel.16 solutions

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning are provided in Tables C.1.2.7.2-1~3.

Table C.1.2.7.2-1: Rel.16 NR positioning latency DL-TDOA

| Case 1 and 4: DL-TDOA. 60KHz and 120 KHz Source UE/Destination NW Positioning technique:DL-TDOA, Initial and Final RRC States:CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | gNB sending PDSCH carrying LPP location request message |
| Process LPP msg | 10ms | UE process LPP location request information message |
| measurement gap configuration | 10ms+10ms: RRC reconfiguration delay | UE requests MG and gNB configures MG |
| DL PRS reception and prcoess | Minimum value is 20ms + 1ms DL PRS periodicity can be 4, 5, 8, 10, 16, 20, 32, 40, 64, 80,160, 320,640, 1280, 2560, 5120 or10240 ms MG periodicity can be 20, 40, 80 or 160ms Processing time depends on the UE capability: Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms | DL PRS reception and measurement |
| End trigger | 12.5ms for 15KHz 6.25ms for 30KHz 3.125ms for 60KHz 1.56 ms for 120KHz UE sends PUCCH SR to request UL grant UE receives DCI scheduling PUSCH UE sends PUSCH carrying measurement report PUSCH decodes PUSCH | UE sends PUSCH carrying location measurement report and gNB decode PUSCH |
| Total values | For FR1 (60kHz): 54.125ms For FR2 (120KHz): 52.56ms |  |

Table C.1.2.7.2-2: Rel.16 NR positioning latency of UL-TDOA

| Case 2 and 5: UL-TDOA. 60KHz and 120 KHz Source UE/Destination NW Positioning technique:DL-TDOA, Initial and Final RRC States:CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | gNB receives NPPa measurement request |
| gNB process NPPa msg | 10ms | gNB process the NPPa measurement request |
| SRS configuration | 10ms+10ms: RRC reconfiguration delay | gNB configures SRS |
| SRS transmission | Assume periodic SRS resource. The periodicity of SRS can be 1, 2, 4, 5, 8, 10, 16, 20 .. slots. So the minimum latency is: 1ms for 15KHz 0.25ms for 60KHz. 0.125ms for 120KHz | UE transmits SRS for positioning |
| End trigger | Assume that 3ms is needed by gNB to measure SRS and process the NPPPa measurement response | gNB measures SRS for positioning and reports the NPPPa measurement response |
| Total values | For FR1 (60kHz): 23.25ms For FR2 (120KHz): 23.125ms |  |

Table C.1.2.7.2-3: Rel.16 NR positioning latency of UE-based method

| Case 3 and 6 :UE-based method 60KHz and 120 KHz Source UE/Destination NW Positioning technique:DL-TDOA, Initial and Final RRC States:CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger | 10ms RRC processing delay | UE sends RRC Location measurement indication |
| measurement gap configuration | 10ms: RRC reconfiguration delay | gNB sends RRC measurement gap configuration |
| DL PRS reception and measurement | Minimum value is 20ms + 1ms DL PRS periodicity can be 4, 5, 8, 10, 16, 20, 32, 40, 64, 80,160, 320,640, 1280, 2560, 5120 or 10240ms. However, the measurement gap periodicity can be 20, 40, 80 or 160ms Processing time depends on the UE capability. Assume extra 1ms is needed for the UE to finish measurement | DL PRS reception and measurement |
| End trigger | 3 ms | UE completes location calculation |
| Total values | For FR1 (60kHz): 54.125ms For FR2 (120KHz): 52.56ms |  |

#### C.1.2.8	Results from source [10]

##### C.1.2.8.1	Description of evaluation scenarios

In order to provide NR positioning physical layer latency analysis, we use the following reference system configuration which is in favor of low latency positioning and were used in NR URLLC studies [3GPP 38.824, v16.0.0]

- Spectrum – FDD / FR1

- SCS – 30 kHz

- PDCCH,

  - Monitoring occasions – 7 per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0]

  - Duration – 1 symbol

- PDSCH

  - One symbol overlap with PDCCH

  - Duration – 2, 4, 7 symbols (Type B mapping)

  - Slot boundary is respected (i.e. transmission does not cross slot boundary)

- PUSCH

  - Any symbol, subject to slot boundary constraint (i.e. transmission does not cross slot boundary)

  - Duration – 2, 4, 7 symbols (Type B mapping w/ front loaded DMRS)

- PUCCH

  - 7 occasions per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0] for SR and HARQ feedback

  - Duration – 1 symbol

  - No HARQ – initial transmission is successful

- SRS for positioning

  - Single resource, 1 symbol duration

  - Periodicity – each slot

- DL PRS

  - 18 resources / 4 symbols per resource / 12 Comb-6 symbols per period

  - Periodicity – 20 ms

  - UE DL PRS processing capability – N = 0.5 ms (~12 symbols @30kHz), T = 8 ms

- UE capability for Tproc,1 Tproc,2

  - Capability # 2 (for low latency)

- Measurement gap

  - Type 1: BWP switching = 2 slots @ 30kHz

  - MGL = 5.5 ms, MGRP = 20ms

    - 2 BWP switching times per occasion (from and to active DL BWP) + DL PRS duration => 5 slots

  - Nsample = 4 (RAN4 core measurements requirements)

    - UE is expected to perform measurements on DL PRS resource 4 times (i.e. across 4 periods)

##### C.1.2.8.2	Latency analysis for Rel.16 solutions

Latency analysis for the Rel.16 NR positioning is provided in Table C.1.2.8.2-1, C.1.2.8.2-2 and C.1.2.8.2-3.

Table C.1.2.8.2-1: Rel.16 NR positioning latency for DL-TDOA/DL-AoD

| Case 1, InF, FR1, R.16 DL-TDOA/DL-AoD Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: DL-TDOA/DL-AoD, type DL only, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  |  |
| Step 1. Location Request | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [38.824] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.25 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.25 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L3 UE LPP processing] | [~280symbols ~10 ms] | Not specified. [The following number is assumed – 5ms DL RRC processing time and 5ms LPP processing time. TBD by RAN WG2] |
| Step 2. Measurement Gap Request | 39.75 symbols 1.4196 ms | Value is provided for L1 components only |
| 2A: [L3 UE RRC processing delay] | 56-140 symbols 2-5ms | [Not defined: 2-5ms UE RRC processing delay is assumed in this analysis TBD by RAN WG2] |
| 2B: SR preparation time | 0 symbols 0 ms | 0ms is assumed since SR can be prepared in advance based on UE implementation |
| 2C: SR alignment time | 2 symbols 0.0714 ms | Depends on configuration of SR monitoring occasions, TDD configuration Worst case: 2 symbols@30kHz for selected reference system configuration |
| 2D: SR transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2E: SR processing time | 4.5 symbols 0.1607 ms | Not specified. For analysis, the assumption from [38.824] is used. For SR-based PUSCH, gNB's processing time for SR is UE's N1. |
| 2F: PDCCH preparation time | 2.25 symbols 0.0804 ms | Not specified. For analysis, the assumptions from the [38.824] are used to derive PDCCH preparation time equal to N1/2: gNB's PUSCH-to-PDCCH processing time (PDCCH alignment has to be included separately) is UE's N1 + X; gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| 2G: PDCCH alignment time | 2 symbols 0.0714 ms | Depends on configuration of PDCCH monitoring occasions and TDD configuration |
| 2H: PDCCH transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2I: PUSCH preparation time | 6.5 symbols 0.2321 ms | Specified as UE PUSCH preparation time Tproc,2 = N2+d21+d2 (if no BWP switch) N2 = 5.5@30kHz, d21 = 1, d2 = 0. |
| 2J: PUSCH alignment time | 7 symbols 0.2500 ms | Worst case:7 symbols for given reference configuration |
| 2K: PUSCH transmission time | 7 symbols 0.2500 ms | 7 symbols for selected reference system configuration |
| 2L: PUSCH processing time | 6.5 symbols 0.2321 ms | Not specified. For analysis, the assumption from the [38.824] is used. gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |

| 2M: [L3 gNB RRC processing delay] | [~84 symbols ~3 ms] | [Not specified. 3ms are assumed in this analysis for gNB RRC processing delay TBD by RAN WG2] |
| --- | --- | --- |
| Step 3. Measurement Gap Configuration | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 3A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Same as for 1A |
| 3B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Same as for 1B |
| 3C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Same as for 1C |
| 3D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Same as for 1D |
| 3E: [L3 UE RRC processing delay] | [~280symbols 10 ms] | [Specified - 10ms TBD by RAN WG2] |
| Step 4. DL PRS Processing | 88,5 ms | Value is provided for L1 components only |
| 4A: DL PRS alignment time | 560symbols 20 ms | Depends on DL PRS periodicity settings Worst case: 20ms for selected reference system configuration |
| 4B: DL PRS processing and report delay | 1918 symbols 68.5 ms | Depends on DL PRS configuration, MG configuration (length and period) and UE DL PRS processing capabilities(N,T). MG should accommodate DL BWP switching. The report delay is defined by 38.133 in Section 9.9.2.5. For considered, reference system configuration it is equal to 3*20+8+0.5 = 68.5ms Note: may be revised by RAN WG4 |
| Step 5. DL PRS Measurement Report | 39.75 symbols 1.4196 ms | Value is provided for L1 components only |
| 5A: [L3: UE RRC + LPP processing delay] | [56-140 symbols 2-5ms] | [Not defined 2-5 ms UE RRC processing time and 5ms UE LPP processing time are assumed TBD by RAN WG2] |
| 5B: SR preparation time | 0 symbols 0 ms | Same as for 2B |
| 5C: SR alignment time | 2 symbols 0.0714 ms | Same as for 2C |
| 5D: SR transmission time | 1 symbol 0.0357 ms | Same as for 2D |
| 5E: SR processing time | 4.5 symbols 0.1607 ms | Same as for 2E |
| 5F: PDCCH preparation time | 2.25 symbols 0.0804 ms | Same as for 2F |
| 5G: PDCCH alignment time | 2 symbols 0.0714 ms | Same as for 2G |
| 5H: PDCCH transmission time | 1 symbol 0.0357 ms | Same as for 2H |
| 5I: PUSCH preparation time | 6.5 symbols 0.2321 ms | Same as for 2I |
| 5J: PUSCH alignment time | 7 symbol 0.2500 ms | Same as for 2J |
| 5K: PUSCH transmission time | 7 symbols 0.2500 ms | Same as for 2K |
| 5L: PUSCH processing time | 6.5 symbols 0.2321 ms | Same as for 2L |
| 5M: [L3 gNB LPP processing time] | [84 symbols 3 ms] | [Not defined. 3 ms are assumed for gNB LPP processing time TBD by RAN WG2] |

Table C.1.2.8.2-2: Rel.16 NR positioning latency for UL-TDOA/UL-AoA

| Case 2, InF, FR1, R.16 UL-TDOA/UL-AoA Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: UL-TDOA/UL-AoA, type UL only, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| Step 1. Configuration of SRS for positioning | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [7] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,g1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L3 UE RRC configuration delay] | [280 symbols 10ms] | [Specified - 10ms TBD by RAN WG2] |
| Step 2 (Alt.1): Activation of SRS for positioning (DCI) | 8.5 symbols 0.3036 ms | Value is provided for L1 components only |
| 2A: PDCCH preparation time | 2.25 symbols 0.0804 ms | Not specified. For analysis, the assumptions from the [7] are used to derive PDCCH preparation time equal to N1/2: gNB's PUSCH-to-PDCCH processing time (PDCCH alignment has to be included separately) is UE's N1 + X; gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| 2B: PDCCH alignment time | 2 symbols 0.0714 ms | Depends on configuration of PDCCH monitoring occasions and TDD configuration |
| 2C: PDCCH transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2D: PDCCH processing time | 3.25 symbols 0.1161 ms | Not specified: For analysis, it is assumed to be equal to ½ of UE PUSCH preparation time Tproc,2 = N2+d21+d2 (if no BWP switch) i.e. Tproc,2/2 N2 = 5.5@30kHz, d21 = 1, d2 = 0. |
| Step 2’(Alt.2): Activation of SRS for positioning (MAC CE) | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 2’A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Same as for 1A |
| 2’B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Same as for 1B |
| 2’C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Same as for 1C |
| 2’D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Same as for 1D |
| 2’E: [L2/L3 component: UE MAC processing delay] | [28-84 symbols 1-3ms] | [Not defined: 1-3ms are assumed TBD by RAN WG2] |
| Step 3. SRS for positioning Tx & Rx | 29 symbols 1.0357 ms | Value is provided for L1 components only |
| 3A: SRS for positioning preparation time | 0 ms | Not defined. 0ms is assumed for analysis |
| 3B: SRS for positioning alignment time | 14 symbols 0.5ms | Depends on SRS resource allocation. Worst case: 14 symbols for selected reference configuration |
| 3C: SRS for positioning transmission time | 1 symbol 0.0357 | 1 symbol for selected reference configuration |
| 3D: SRS for positioning processing time by gNB | [14 symbols 0.5 ms] | [Not defined: 0.5 ms is assumed. Definition is in scope of RAN WG4] |
| Step 4. gNB Measurement Report | 0 | Value is provided for L1 components only |
| 4A: [L3 gNB NRPPa preparation time] | [84symbols 3 ms] | [Not defined: 3 ms is assumed. TBD by RAN WG2] |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |

Table C.1.2.8.2-3: Rel.16 NR positioning latency for Multi-RTT

| Case 3, InF, FR1, R.16 Multi-RTT Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: Multi-RTT, type DL+UL, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| Step 1. Configuration of SRS for positioning | L1: 0.8661 [L3: 10.0] | L1: gNB signaling to UE to configure SRS for positioning – 0.5088 ms [L3: UE RRC configuration delay – 10ms - TBD by RAN WG2] |
| Step 2: Activation of SRS for positioning (MAC CE) | L1: 0.8661 [L2: 1.0] | L1: gNB signaling to UE to activate SRS for positioning transmission – 0.5088 ms [L2: UE MAC CE processing delay – 1 ms - TBD by RAN WG2] |
| Step 3. SRS for positioning TX & RX | L1: 1.0357 | L1: transmission of SRS for positioning by UE and its reception by gNB – 1.0357 ms |
| Step 4. gNB NRPPa preparation time for report to LMF | [L3: 3.0] | [L3: gNB NRPPa preparation time for report to LMF – 3 ms - TBD by RAN WG2] |
| Step 5. Location information request | L1: 0.8661 [L3: 10.0] | L1: gNB transmits to UE location request - 0.5088 ms [L3: 5 ms DL RRC processing time and 5ms LPP processing time - TBD by RAN WG2] |
| Step 6. MG Request | L1: 1.4196 [L3: 5-8] | L1: UE requests gNB to provide MG – 1.0266 ms [L3: UE RRC processing delay – 2-5 ms and gNB RRC processing delay – 3ms TBD by RAN WG2] |
| Step 7. MG Configuration | L1: 0.8661 [L3: 10.0] | L1: gNB provides MG configuration to UE RRC signaling – 0.5088 ms [L3: UE RRC configuration delay – 10 ms - TBD by RAN WG2] |
| Step 8. DL PRS Processing | L1: 88.5 | DL PRS alignment time and DL PRS processing and report delay |
| Step 9. DL PRS Measurement Report | L1: 1.4196 [L3: 3.0] | L1: UE reports measurement results to gNB [L3: gNB LPP processing time - 3ms; 2-5ms UL RRC processing time; 5ms UE LPP processing time are assumed. TBD by RAN WG2] |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |

#### C.1.2.9 Results from source [16]

##### C.1.2.9.1 Description of evaluation scenarios

The latency analysis of DL, UL, and DL+UL methods are presented here. In each method, we also perform the analysis of UE-Assisted and UE-Based (if applicable). All analysis are based on 30kHz SCS and minimum value for latency is presented.

##### C.1.2.9.2 Latency analysis for R.16 solution

Table C.1.2.9.2-1: Latency analysis for UE-assisted DL methods (Case 1) [16]

| Source [NW]/Destination [NW] Positioning technique: DL methods Initial and Final RRC States [CONNECTED], mode: UE-A |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | 1 | Estimate |
| Step 2: The UE decodes the LPP Request location | 10 | NAS message, based on RC message decoding time |
| Step 3: The UE transmits measurement gap (MG) request in an RRC message. | 1 | Estimate |
| Step 4: The gNB successfully decode the MG request message | 5 | RRC message |
| Step 5: The gNB configures MG in an RRC message | 1 | Estimate |
| Step 6: The UE receive the MG configuration message and apply the MG configuration. | 10 | RRC message decoding including MG configuration |
| Step 7: The UE receives PRS in the MG | 2 | According to RRC, the UE can be configured with MGL of 1.5ms. Thefore, the best scenario is that the UE can be configured MG of 1.5ms, performing PRS monitoring and finish the MG within 2ms. |
| Step 8: The UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 | The UE prepares PUSCH (1ms), uses type-1 configured grant to report the positioning measurement (1ms). Then gNB successfully decodes PUSCH (1ms). Note that the minimum periodicity of type-1 configured grant is 2 symbols for all SCSs. |
| Total values | 33ms |  |

Table C.1.2.9.2-2: Latency analysis for UE-based DL methods (Case 2) [16]

| Source [NW]/Destination [UE, NW] Positioning technique: DL methods Initial and Final RRC States [CONNECTED] ], mode: UE-B |  |  |  |
| --- | --- | --- | --- |
| Latency Component |  | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |  | 1 | Estimate |
| Step 2: The UE decodes the LPP Request location |  | 10 | NAS message |
| Step 3: The UE transmits MG request in an RRC message. |  | 1 | Estimate |
| Step 4: The gNB decodes the MG request message. |  | 5 | NAS message |
| Step 5: The gNB configures MG in an RRC message |  | 1 | RRC message |
| Step 6: The UE receive the MG configuration message and apply the MG configuration |  | 10 | RRC message decoding including MG configuration |
| Step 7: The UE receives PRS in the MG |  | 2 | Estimate |
| Step 8: UE calculates its location |  | 20-40 | Estimate |
| Step 9: UE reports its location |  | 2 | The UE prepares PUSCH (1ms), uses type-1 configured grant to report the positioning measurement (1ms). |
| Total values | From Step 1 to Step 8 | 50-70ms | Applicable for the scenario starting at the step “the UE receives PDSCH from the gNB carrying the LPP Request Location Information message” and also applicable for Alt 2 |
|  | From Step 1 to Step 9 | 52-72ms |  |
|  | From Step 3 to Step 8 | 39-59ms | Applicable for Alt 1 |
|  | From Step 3 to Step 9 | 41-61ms |  |
|  | From Step 7 to Step 8 | 22-42ms | Applicable for Alt 3. |
|  | From Step 7 to Step 9 | 24-44ms |  |

Table C.1.2.5.2-3: Latency analysis for UE-assisted UL methods (Case 3) [16]

| Source [NW]/Destination [NW] Positioning technique: UL methods Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Reception by the gNB of the NRPPa measurement request message from LMF | NA | Delay at core. |
| Step 1: gNB send SRS configuration via RRC | 1 | Estimate |
| Step 2: UE decodes the RRC message including SRS configuration message | 10 | RRC message |
| Step 3: UE transmit SRS | 1 | Estimate |
| gNB perform measurement and transmit the NRPPa measurement response message | NA | Delay at core |
| Total values | 12ms | Note: In this analysis we only consider the periodic SRS configuration. For aperiodic and semi-persistent SRS configuration, the PHY latency will be increased. |

Table C.1.2.9.2-4: Latency analysis for UE-assisted DL+UL methods (Case 4) [16]

| Source [NW]/Destination [NW] ], mode: UE-A Positioning technique: DL+UL methods |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | 1 | Estimate |
| Step 2: The UE decodes the LPP Request location | 10 | NAS message |
| Step 3: The UE transmits measurement gap (MG) request in an RRC message. | 1 | Estimate |
| Step 4: The gNB successfully decode the MG request message | 5 | RRC message |
| Step 5: The gNB configures MG in an RRC message | 1 | Estimate |
| Step 6: The UE receive the MG configuration message and apply the MG configuration | 10 | RRC message |
| Step 7: The UE receives PRS in the MG | 2 |  |
| Reception by the gNB of the NRPPa measurement request message | NA | Core message. |
| Step 8: The gNB send SRS configuration via RRC | 1 | Estimate |
| Step 9: The UE decodes the RRC message including SRS configuration | 10 | RRC message |
| Step 10: The UE transmit SRS | 1 | Estimate |
| Step 11: The UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 |  |
| Total values | 45ms |  |

#### C.1.2.10 Results from source [17]

##### C.1.2.10.1	Description of evaluation scenarios

##### C.1.2.10.2	Latency analysis for Rel.16 solutions

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning is provided in Table C.1.2.1.20-1.

Table C.1.2.1.20-1: Rel.16 NR positioning latency

| UE-Assisted DL-only Positioning, RRC Connected State |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Step 1: Serving gNB sends PDSCH that contains the Location Information Request and is successfully decoded in physical layer by the UE | 1 - 2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 2: UE interprets and applies the LPP message (Location Request) | 10 | RRC Processing time are captured in 38.331 Section 12 |
| Step 3: Request of Location Information has be received at the UE. UE transmits in PUSCH an MG-request through an RRC message | 1 - 4 | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 4: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 5: Serving gNB transmits the RRC configuration of the MG request | 1 - 2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 6: UE receives the message, interprets and applies the MG configuration | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 7: UE receives at least one occasion of the PRS within the MG. | 20 - 640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 8: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 - 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 9: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | 2 - 5 | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total values | 57 - 823 |  |

| UE-based DL-only Positioning, RRC Inactive State, External Client |  |  |
| --- | --- | --- |
| Steps | “Best-case” Estimate of Duration (msec) in NR Rel-16 | “Conservative” Estimate of the Duration (msec) in NR Rel-16 |
| Step 1: Request of Location Information has been received at the UE. UE transmits in PUSCH an MG-request through an RRC message | 1-4 | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 2: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 3: Serving gNB transmits the RRC configuration of the MG request | 1-2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 4: UE receives the message, interprets and applies the MG configuration | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 5: UE receives at least one occasion of the PRS within the MG. | 20-640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 6: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2-140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 7: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | 2-5 | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total PHY-layer Latency (msec) | 46-811 |  |

| UE-based DL-only Positioning, RRC Connected State, UE Internal-client |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Step 1: UE receives at least one occasion of the PRS | 20 - 640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 2: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 - 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 3: UE transition to RRC Connected state | 11.3 - 18.5 | Transition from RRC Inactive to Connected state. 11.3 ms – 18.5 ms dependent on configuration/deployment (TR 37.910 [8]). Assume an "average" value. |
| Step 4: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | 2 - 5 | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total values | 35.3 - 803.5 |  |

| UE-based DL-only Positioning, RRC Inactive State, UE internal-client |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Step 1: UE receives at least one occasion of the PRS | 6 - 640 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (N,8), with N<8. We assume that a reasonable PRS instance in NR Rel-16 FR1 is 6 msec. Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 2: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 - 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Total values | 8 - 780 |  |

| UE-Assisted MRTT Positioning, RRC Connected State |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Step 1: Serving gNB sends PDSCH that contains the Location Information Request and is successfully decoded in physical layer by the UE | 1 – 2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 2: UE interprets and applies the LPP message (Location Request) | 10 | RRC Processing time are captured in 38.331 Section 12 |
| Step 3: Request of Location Information has be received at the UE. UE transmits in PUSCH an MG-request through an RRC message | 1 - 4 | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 4: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 5: Serving gNB transmits the RRC configuration of the MG request | 1 – 2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 6: UE receives the message, interprets and applies the MG configuration | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 7: UE receives at least one occasion of the PRS within the MG. | 20-640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 8: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 - 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 9: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | 2 - 5 | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Step 10: All gNBs processes the transmitted SRS in parallel. | 2 | Assumption: SRS configuration is received before Step 1, and SRS can be triggered with DCI. |
| Total values | 59 - 823 | Latency needed from the time applies the location request up to the decoding of the PUSCH carrying the Positionign report. |

#### C.1.2.11 Results from source [15]

##### C.1.2.11.1	Description of evaluation scenarios

We assumed the UEs in the RRC connected mode for the physical layer latency analysis, and we provide the latency analysis on the physical layer only as well as physical layer latency with the consideration of higher layer. The following cases are considered.

- Case 1: DL-TDOA/DL-AoD positioning method for the NW initiated

- Case 2: DL-TDOA/DL-AoD positioning method for the UE initiated

- Case 3: UL-TDOA/UL-AoA positioning method for the NW initiated

- Case 4: Multi-RTT positioning method for the NW initiated

- Case 5: E-CID positioning method for the NW initiated

To evaluate physical layer latency for the enhanced NR positioning solution(s), we use the following assumptions as below:

- Numerology: 15 kHz (for PDCCH, PDSCH, PUCCH, PUSCH, SRS)

- For PUSCH transmission:

- Uplink switching gap is not configured.

- No BWP switching

- No overlapping symbols of the PUCCH and the scheduled PUSCH

- The number of PUSCH symbols = from 4 to 14 for Type A

- The number of PUSCH symbols = from 1 to 14 for Type B

- According to the size of data (e.g. LPP message) to be transmitted, the number of symbols can be changed.

- For PDSCH transmission:

- No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH

- The number of PDSCH symbols = from 3 to 14 for Type A

- The number of PDSCH symbols = from 2 to 14 for Type B

- According to the size of data (e.g. LPP message) to be transmitted, the number of symbols can be changed.

- For SRS transmission:

- One shot transmission (2 OS ~ 12 OS)

- According to resource type (periodic, Semi-persistent and aperiodic), the number of symbols can be changed.

- For PUCCH transmission (Scheduling request):

- One shot transmission

- Periodicity and offset for PUCCH: 2 OS ~ 80 slots

- The length of symbols for PUCCH: 1 OS~ 14 OS (format 0 and 2)

- According to PUCCH configuration (e.g. periodicity and offset, symbol length of PUCCH), the latency can be changed.

- For PDCCH transmission (UL grant):

- Slot for PDCCH Monitoring configured as periodicity and offset is 1slot (format 0_0, 0_1) even though 2560 slots can be configured.

- The first symbol(s) for PDCCH monitoring in the slots is zero

- The length of symbols for CORESET: 1 OS ~3 OS

- According to PUCCH configuration (e.g. periodicity and offset, symbol length of PDCCH), the latency can be changed.

##### C.1.2.11.2	Latency analysis for Rel.16 solutions

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning is provided in Table C.1.2.11.2-1- Table C.1.2.11.2-5.

Table C.1.2.11.2-1: NR positioning – latency analysis [15]

| Source/Destination: NW/UE Positioning technique DL-TDOA and DL-AOD, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Request Location Information message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Request Location Information message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Constructing measurement gap request message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | [X1] | [X1] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| *Transportation of Measurement gap request message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5. The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| *Constructing Provide Location information message(RRC processing time, UE) | 10 ms | 38.331 section 12. |
| Transmission of scheduling request for measurement report (UEgNB) | 0.21ms~81ms | 38.213 section 9.2.4 38.331 section 6.3.2 |
| *Reception of scheduling request And preparation time for UL grant (gNB) | 2ms | L2/L1 processing time |
| Transmission of UL grant(gNB UE ) | 0.07ms~0.21ms | 38.213 section 10.1 |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | [X2] | [X2] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 75.4+[X1]+[X2]ms ~ 300.33+[X1]+ [X2]ms For UE capability-2: 73.96+[X1]+[X2]ms ~ 298.47+[X1]+ [X2]ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 23.4ms ~ 248.33ms For UE capability-2: 21.96ms ~ 246.47ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.1.2.11.2-2: NR positioning – latency analysis [15]

| Source/Destination: UE/NW Positioning technique DL-TDOA, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | [X1] | [X1] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| *Transportation of measurement gap request message (RRC processing time, gNB) | 10ms | 38.331 section 12. |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Measurement gap configuration message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5 The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| *Provide Location information message (UE) | 10 ms | 38.331 section 12. |
| Transmission of scheduling request for measurement report (UEgNB) | 0.21ms~81ms | 38.213 section 9.2.4 38.331 section 6.3.2 |
| *Reception of scheduling request And preparation time for UL grant (gNB) | 2ms | L2/L1 processing time |
| Transmission of UL grant(gNB UE ) | 0.07ms~0.21ms | 38.213 section 10.1 |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | [X2] | [X2] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 64.69+[X1]+[X2]ms ~ 288.55+[X1] +[X2]ms For UE capability-2: 63.61+[X1]+[X2]ms ~ 287.26+[X1] +[X2]ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 22.69ms ~ 246.55ms For UE capability-2: 22.61ms ~ 245.26ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.1.2.11.2-3: NR positioning – latency analysis [15]

| Source/Destination: NW/UE Positioning technique UL-TDOA and UL-AOA, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| SRS configuration (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS configuration (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS configuration message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of SRS activation message (RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Transmission of SRS activation (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS activation (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS activation message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| SRS transmission (UE) | 0.07ms ~ 0.86ms | 38.214 section 6.2.1. |
| *Reception of SRS and preparation time for gNB to make NRPPa message (gNB) | [X3] | [X3] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 31.49+[X3] ms ~ 34.42+[X3] ms For UE capability-2: 30.77+[X3] ms ~ 33.28+[X3] ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 1.49ms ~ 4.42ms For UE capability-2: 0.77ms ~ 3.28ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.1.2.11.2-4: NR positioning – latency analysis [15]

| Source/Destination: NW/UE Positioning technique: Multi-RTT, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| SRS configuration (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS configuration (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS configuration message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of SRS activation message (RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Transmission of SRS activation (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS activation (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS activation message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of provide assistance message (RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Transmission of LPP provide assistance data (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of LPP provide assistance data (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of provide assistance message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of request location information (RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Request Location Information message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Location Information message Reception (PHY processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of LPP message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of measurement gap request (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | [X1] | [X1] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| *Transportation of measurement gap request message (RRC processing time, gNB) | 10ms | 38.331 section 12. |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Measurement gap configuration message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5 The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| SRS transmission (UE) | 0.07ms ~ 0.86ms | 38.214 section 6.2.1. |
| *Reception of SRS and preparation time for gNB to make NRPPa message (gNB) | [X3] | [X3] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| *Constructing Provide Location information message(RRC processing time, UE) | 10 ms | 38.331 section 12. |
| Transmission of scheduling request for measurement report (UEgNB) | 0.21ms~81ms | 38.213 section 9.2.4 38.331 section 6.3.2 |
| *Reception of scheduling request And preparation time for UL grant (gNB) | 2ms | L2/L1 processing time |
| Transmission of UL grant(gNB UE ) | 0.07ms~0.21ms | 38.213 section 10.1 |
| measurement report transmission(including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | [X2] | [X2] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 147.62+[X1]+[X2]+[X3]ms ~ 376.53+[X1]+[X2]+[X3]ms For UE capability-2: 145.8+[X1]+[X2]+[X3]ms ~ 372.96+[X1]+[X2]+[X3] ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 25.62ms ~ 254.53ms For UE capability-2: 23.08ms ~ 250.96ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.1.2.11.2-5: NR positioning – latency analysis [15]

| Source/Destination: NW/UE Positioning technique: E-CID, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| LPP Request Location Information message transmission(gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of LPP Request Location Information message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Constructing Provide Location information message(RRC processing time, UE) | 10 ms | 38.331 section 12. |
| Transmission of scheduling request for measurement report (UEgNB) | 0.21ms~81ms | 38.213 section 9.2.4 38.331 section 6.3.2 |
| *Reception of scheduling request And preparation time for UL grant (gNB) | 2ms | L2/L1 processing time |
| Transmission of UL grant(gNB UE ) | 0.07ms~0.21ms | 38.213 section 10.1 |
| measurement report transmission(including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement report, SRS and preparation time for gNB to make NRPPa message (gNB) | [X2] | [X2]is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 23.84+[X2]ms ~ 106.77+[X2] ms For UE capability-2: 23.12+[X2]ms ~ 105.84+[X2] ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 1.84ms ~ 84.77ms For UE capability-2: 1.12ms ~ 83.84ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

## C.2	Performance of studied NR positioning enhancements

Including performance of positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency ((objective 1c).

### C.2.1	Positioning accuracy analysis for NR positioning enhancements

#### C.2.1.1	Results from source [4]

##### C.2.1.1.1	Description of evaluation scenarios

The following enhancements are evaluated for accuracy:

- LOS/NLOS identification

- PRS/SRS frequency aggregation

- 1-symbol PRS

- PRS punctured by SSB

- AoA with ULA

- E-CID enhancement

- Synchronization error compensation

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Tables C.2.1.1.1-1 to Table C.2.1.1.1-4.

Table C.2.1.1.1-1: NR positioning enhancements (LOS/NLOS identification 31x and PRS/SRS frequency aggregation 32x) - evaluation scenarios and parameters

| Parameter | Case 311 (InF-DH, FR1) | Case 312 (InF-DH, FR1) | Case 313 (InF-DH, FR1) | Case 314 (InF-DH, FR1) | Case 321 (InF-DH, FR1) | Case 322 (InF-DH, FR1) | Case 323 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 200MHz | 50MHz+100MHz (gap)+50MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution RAIM | Super resolution LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution No LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any |  |  |  |  |  |  |  |

Table C.2.1.1.1-2: NR positioning enhancements (1-symbol PRS 33x and PRS punctured by SSB 34x) - evaluation scenarios and parameters

| Parameter | Case 331 (InF-SH, FR1) | Case 332 (InF-SH, FR1) | Case 333 (InF-SH, FR1) | Case 334 (InF-SH, FR1) | Case 341 (InF-SH, FR1) | Case 342 (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 1 symbol) | DL-PRS (Comb-12, 12 symbol) | DL-PRS (Comb-12, 1 symbol) | DL-PRS (Comb-4, 4 symbol) | DL-PRS (Comb-4, 1 symbol) |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port | Gold, single port | Gold, single port | Gold, single port | Gold, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 10.8dB | 10.8dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO | DL-TDOA PSO |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any |  |  |  |  |  | PRS center 20RBs are punctured by SSB |

Table C.2.1.1.1-3: Rel.16 NR positioning (UL-AoA enhancement with ULA 36x and E-CID enhancement 37x) - evaluation scenarios and parameters

| Parameter | Case 361 (InF-DH, FR1) | Case 362 (InF-DH, FR1) | Case 363 (InF-DH, FR1) | Case 371 (InF-DH, FR1) | Case 372 (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | DL-PRS+PosSRS (Comb-4, 4 symbol) | CSI-RS+SRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | Gold/ZC, single port | Gold/ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 1 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA+UL-AoA PSO | UL-TDOA+UL-AoA PSO | UL-TDOA+UL-AoA PSO | Multi-RTT PSO | E-CID (RTT+AoA only from the serving cell) |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | UPA 4x4x2 | ULA 1x4x1 assuming legacy AoA | ULA 1x4x1 with modified AoA |  |  |

Table C.2.1.1.1-4: NR positioning enhancements (Synchronization error compensation) - evaluation scenarios and parameters

| Parameter | Case 381 (InF-SH, FR1) | Case 382 (InF-SH, FR1) | Case 383 (InF-SH, FR1) | Case 384 (InF-SH, FR1) | Case 385 (InF-SH, FR1) | Case 386 (InF-SH, FR1) | Case 387 (InF-SH, FR1) | Case 388 (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port | ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO |
| Network synchronization assumptions | 0ns | 0.2ns | 0.5ns | 1ns | 0ns | 0.2ns | 0.5ns | 1ns |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based | Tx codebook-based |
| Additional notes, if any | 100MHz contiguous | 100MHz contiguous | 100MHz contiguous | 100MHz contiguous | 200MHz contiguous | 200MHz contiguous | 200MHz contiguous | 200MHz contiguous |

##### C.2.1.1.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.1.2-1:

Table C.2.1.1.2-1: NR positioning enhancements - horizontal location error results from [4]

| Features | Cases |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| LOS/NLOS identification | 311, InF-DH422, FR1, UL-TDOA | (Optional) All UEs | 0.4309 | 1.6317 | 4.9693 | 9.9453 |
|  |  | Convex UEs | 0.3354 | 1.2393 | 4.6237 | 9.6631 |
|  | 312, InF-DH422, FR1, UL-TDOA w/ RAIM | (Optional) All UEs | 0.0371 | 0.0748 | 0.1912 | 1.4517 |
|  |  | Convex UEs | 0.0301 | 0.0554 | 0.1217 | 0.6569 |
|  | 313, InF-DH422, FR1, UL-TDOA w/ LOS/NLOS identification | (Optional) All UEs | 0.0397 | 0.0701 | 0.1354 | 0.4836 |
|  |  | Convex UEs | 0.0311 | 0.0505 | 0.0862 | 0.2111 |
|  | 314, InF-DH422, FR1, UL-TDOA w/ ideal LOS selection | (Optional) All UEs | 0.0394 | 0.0608 | 0.1399 | 0.4452 |
|  |  | Convex UEs | 0.0310 | 0.0506 | 0.0855 | 0.2022 |
| PRS/SRS frequency aggregation | 321, InF-DH422, FR1, UL-TDOA, 100M contiguous | (Optional) All UEs | 0.0394 | 0.0608 | 0.1399 | 0.4452 |
|  |  | Convex UEs | 0.0310 | 0.0506 | 0.0855 | 0.2022 |
|  | 322, InF-DH422, FR1, UL-TDOA, 200M contiguous | (Optional) All UEs | 0.0106 | 0.0179 | 0.0332 | 0.0827 |
|  |  | Convex UEs | 0.0082 | 0.0122 | 0.0197 | 0.0384 |
|  | 323, InF-DH422, FR1, UL-TDOA, 50MHz+100MHz (Gap)+50MHz | (Optional) All UEs | 0.0148 | 0.0273 | 0.0570 | 0.2244 |
|  |  | Convex UEs | 0.0109 | 0.0189 | 0.0337 | 0.0912 |
| 1-symbol PRS | 331, InF-SH, FR1, DL-TDOA, Comb-4 and 4-symbol | (Optional) All UEs | 0.0252 | 0.0425 | 0.0718 | 0.1531 |
|  |  | Convex UEs | 0.0196 | 0.0314 | 0.0504 | 0.0939 |
|  | 332, InF-SH, FR1, DL-TDOA, Comb-4 and 1-symbol | (Optional) All UEs | 0.0268 | 0.0473 | 0.0836 | 0.1693 |
|  |  | Convex UEs | 0.0210 | 0.0347 | 0.0567 | 0.1123 |
|  | 333, InF-SH, FR1, DL-TDOA, Comb-12 and 12-symbol | (Optional) All UEs | 0.0279 | 0.0492 | 0.0835 | 0.1647 |
|  |  | Convex UEs | 0.0219 | 0.0347 | 0.0573 | 0.1091 |
|  | 334, InF-SH, FR1, DL-TDOA, Comb-12 and 1-symbol | (Optional) All UEs | 0.0284 | 0.0484 | 0.0866 | 0.1784 |
|  |  | Convex UEs | 0.0209 | 0.0359 | 0.0595 | 0.1199 |
| PRS punctured by SSB | 341, InF-SH, FR1, DL-TDOA | (Optional) All UEs | 0.0252 | 0.0425 | 0.0718 | 0.1531 |
|  |  | Convex UEs | 0.0196 | 0.0314 | 0.0504 | 0.0939 |
|  | 342, InF-SH, FR1, DL-TDOA, 20-RB of PRS punctured by SSB | (Optional) All UEs | 0.0293 | 0.0475 | 0.0846 | 0.1699 |
|  |  | Convex UEs | 0.0213 | 0.0346 | 0.0539 | 0.1090 |
| UL-AoA enhancement with ULA | 361, InF-DH422, FR1, UL-TDOA/AoA | (Optional) All UEs | 0.0216 | 0.0318 | 0.0460 | 0.0735 |
|  |  | Convex UEs | 0.0203 | 0.0276 | 0.0396 | 0.0665 |
|  | 362, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ legacy AoA) | (Optional) All UEs | 2.4977 | 3.3503 | 4.1807 | 4.9986 |
|  |  | Convex UEs | 2.2164 | 3.1038 | 3.9067 | 4.8161 |
|  | 363, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ modified AoA | (Optional) All UEs | 0.0406 | 0.0626 | 0.1027 | 0.2058 |
|  |  | Convex UEs | 0.0348 | 0.0525 | 0.0821 | 0.1694 |
| E-CID enhancement | 371, InF-DH422, FR1, Multi-RTT | (Optional) All UEs | 0.0423 | 0.0591 | 0.0901 | 0.1756 |
|  |  | Convex UEs | 0.0388 | 0.0555 | 0.0871 | 0.1694 |
|  | 372, InF-DH422, FR1, E-CID w/ single cell RTT/AoA | (Optional) All UEs | 0.0471 | 0.0651 | 0.1068 | 0.2497 |
|  |  | Convex UEs | 0.0473 | 0.0647 | 0.1040 | 0.2395 |
| Synchronization error compensation | 381, InF-SH, FR1, UL-TDOA, 100M，0ns gNB Sync error | (Optional) All UEs | 0.0369 | 0.0622 | 0.1026 | 0.1867 |
|  |  | Convex UEs | 0.0263 | 0.0433 | 0.0654 | 0.1136 |
|  | 382, InF-SH, FR1, UL-TDOA, 100M，0.2ns gNB Sync error | (Optional) All UEs | 0.0828 | 0.1153 | 0.1643 | 0.2672 |
|  |  | Convex UEs | 0.0662 | 0.0855 | 0.1096 | 0.1452 |
|  | 383, InF-SH, FR1, UL-TDOA, 100M，0.5ns gNB Sync error | (Optional) All UEs | 0.1811 | 0.2482 | 0.3330 | 0.5173 |
|  |  | Convex UEs | 0.1454 | 0.1877 | 0.2306 | 0.2828 |
|  | 384, InF-SH, FR1, UL-TDOA, 100M，1ns gNB Sync error | (Optional) All UEs | 0.3500 | 0.4709 | 0.6348 | 1.0315 |
|  |  | Convex UEs | 0.2731 | 0.3530 | 0.4353 | 0.5372 |
|  | 385, InF-SH, FR1, UL-TDOA, 200M, 0ns gNB Sync error | (Optional) All UEs | 0.0086 | 0.0144 | 0.0240 | 0.0456 |
|  |  | Convex UEs | 0.0065 | 0.0101 | 0.0149 | 0.0250 |
|  | 386, InF-SH, FR1, UL-TDOA, 200M, 0.2ns gNB Sync error | (Optional) All UEs | 0.0710 | 0.0971 | 0.1330 | 0.2158 |
|  |  | Convex UEs | 0.0558 | 0.0726 | 0.0901 | 0.1102 |
|  | 387 InF-SH, FR1, UL-TDOA, 200M, 0.5ns gNB Sync error | (Optional) All UEs | 0.1725 | 0.2328 | 0.3055 | 0.4895 |
|  |  | Convex UEs | 0.1406 | 0.1790 | 0.2178 | 0.2659 |
|  | 388 InF-SH, FR1, UL-TDOA, 200M, 1ns gNB Sync error | (Optional) All UEs | 0.3490 | 0.4658 | 0.6330 | 0.9466 |
|  |  | Convex UEs | 0.2735 | 0.3582 | 0.4407 | 0.5340 |

Figure C.2.1.1.2-1 provides the positioning evaluation results for LOS/NLOS identification.

Figure C.2.1.1.2-2 provides the positioning evaluation results for PRS/SRS frequency aggregation.

Figure C.2.1.1.2-3 provides the positioning evaluation results for 1-symbol PRS.

Figure C.2.1.1.2-4 provides the positioning evaluation results for PRS punctured by SSB.

Figure C.2.1.1.2-5 provides the positioning evaluation results for UL-AoA enhancement with ULA.

Figure C.2.1.1.2-6 provides the positioning evaluation results for E-CID enhancement.

Figure C.2.1.1.2-7 and Figure C.2.1.1.2-8 provide the positioning evaluation results for synchronization error compensation.

Figure C.2.1.1.2-1: Positioning error results for LOS/NLOS identification from [4]

Figure C.2.1.1.2-2: Positioning error results for PRS/SRS frequency aggregation from [4]

Figure C.2.1.1.2-3: Positioning error results for 1-symbol PRS from [4]

Figure C.2.1.1.2-4: Positioning error results for PRS punctured by SSB from [4]

Figure C.2.1.1.2-5: Positioning error results for UL-AoA enhancement with ULA from [4]

Figure C.2.1.1.2-6: Positioning error results for E-CID enhancement from [4]

Figure C.2.1.1.2-7: Positioning error results for synchronization error compensation from [4]

Figure C.2.1.1.2-8: Positioning error results for synchronization error compensation from [4]

#### C.2.1.2	Results from source [7]

##### C.2.1.2.1	Description of evaluation scenarios

The evaluation scenarios in this contribution for verifying achievable accuracy based on enhanced positioning methods  include,

- InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, ideal LOS identification is assumed.

- InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, Rician K-factor as assistance information to determine LOS and NLOS links.

- InF-DH (both FR1 and FR2) with fixed UE/gNB height and without network synchronization, coherence bandwidth as assistance information to determine LOS and NLOS links.

Some scenario common parameters can be found in Table C.1.2.2.1-1. In addition, Table C.2.1.2.1-1 reveals some controlled variables of all simulation cases.

Table C.2.1.2.1-1 All simulation cases for positioning accuracy evaluation based on enhanced positioning methods

| Simulation cases | Scenario | FR1/FR2 | Whether Rel-17 enhancement is implemented | UE horizontal drop procedure | gNB Tx calibration error | Clutter parameters |
| --- | --- | --- | --- | --- | --- | --- |
| Case 12 | InF-DH | FR1 | NO | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 25 | InF-DH | FR1 | Ideal LOS identification | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 26 | InF-DH | FR1 | Rician K-factor | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 27 | InF-DH | FR1 | Coherence bandwidth | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 17 | InF-DH | FR2 | NO | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 28 | InF-DH | FR2 | Ideal LOS identification | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 29 | InF-DH | FR2 | Rician K-factor | Inside convex hull | 0ns | {40%, 2m, 2m} |
| Case 30 | InF-DH | FR2 | Coherence bandwidth | Inside convex hull | 0ns | {40%, 2m, 2m} |

##### C.2.1.2.2	Positioning accuracy evaluation results for NR positioning enhancements

Table C.1.1.2.2-1 provides CDF of horizontal positioning accuracy at some specific percentiles for InF-DH scenario based on enhanced positioning method, and corresponding CDF curves can be found in Figure C.2.1.2.2-1.

Table C.2.1.2.2-1: NR positioning enhancements - horizontal location error results from [7]

| Cases | Horizontal positioning accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% | 67% | 80% | 90% |
| Case 12 | 0.414 | 1.060 | 7.337 | 12.345 |
| Case 25 | 0.261 | 0.371 | 0.493 | 0.648 |
| Case 26 | 0.294 | 0.4207 | 0.786 | 7.205 |
| Case 27 | 0.300 | 0.4232 | 0.659 | 2.003 |
| Case 17 | 0.062 | 0.097 | 0.174 | 12.174 |
| Case 28 | 0.047 | 0.062 | 0.080 | 0.107 |
| Case 29 | 0.053 | 0.074 | 0.101 | 9.248 |
| Case 30 | 0.051 | 0.069 | 0.091 | 0.228 |

Figure C.2.1.2.2-1-1 InF-DH, FR1                      Figure C.2.1.2.2-2 InF-DH, FR2

Figure C.2.1.2.2-1 Rel-17 enhancements with LOS identification

#### C.2.1.3	Results from source [8]

##### C.2.1.3.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.3.1-1.

- Case ID: 1 to 2.

- Scenario: InF-HH-2D.

- Frequency Band: FR1.

- Positioning Technique: DL-TDOA +DL-CPP, UL-TDOA + UL-CPP [R1-2007860].

Table C.2.1.3.1-1: NR positioning enhancements - evaluation scenarios and parameters [8]

| Parameter | [Case 1], [InF-HH-2D], [FR1], [DL-TDOA+ DL-CPP] | [Case 2], [InF-HH-2D], [FR1], [UL-TDOA+ UL-CPP] |
| --- | --- | --- |
| Channel model (baseline) | InF-HH-2D | InF-HH-2D |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 |
| Number of symbols used per occasion | 12 | 12 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 7.78dB | 9dB |
| Uplink power control (applied/Not applied) | —— | —— |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm | MUSIC + PLL | MUSIC + PLL |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | EKF + Chan | EKF + Chan |
| Network synchronization assumptions | Perfect | Perfect |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No | No |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | No | No |
| Evaluated Enhancement for Rel.17 | DL-TDOA+ DL-CPP | UL-TDOA+ UL-CPP |
| Additional Notes, if any | Double differential with reference UE to overcome network synchronization error [R1-2007860] | Double differential with reference UE to overcome network synchronization error [R1-2007860] |

##### C.2.1.3.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.3.2-1:

Table C.2.1.3.2-1: NR positioning enhancements - horizontal location error results from [8]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-HH-2D], [FR1], [DL- TDOA+ DL-CPP] | Convex UEs | 0.0218 | 0.0321 | 0.0387 | 0.051 |
| [Case 2], [InF-HH-2D], [FR1], [UL- TDOA+ UL-CPP] | Convex UEs | 0.0196 | 0.0306 | 0.0368 | 0.049 |

#### C.2.1.4	Results from source [13]

##### C.2.1.4.1	Description of evaluation scenarios

| Parameter | Case 1 [InF-SH, UL-TDOA, FR1, 100 MHz] | Case 1 [InF-DH, UL-TDOA, FR1, 100 MHz] |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-2, 2 symbol SRS-Pos | Comb-2, 2 symbol SRS-Pos |
| Reference signal (type of sequence, number of ports, …) | NR SRS-Pos | NR SRS-Pos |
| Number of sites | 18 | 18 |
| Number of symbols used per slot per positioning estimate | 2 symbols per slot | 2 symbols per slot |
| Number of slots per positioning estimate | 1 slot | 1 slot |
| Power-boosting level | fixed power (max) | fixed power (max) |
| Uplink power control (applied/not applied) | Not applicable | Not applicable |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Thresholding, 0.5, (Oversampling x4) | Thresholding, 0.5, (Oversampling x4) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Taylor Series, Least Squares | Taylor Series, Least Squares |
| Network synchronization assumptions | Perfect Synchronization | Perfect Synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A |
| Precoding assumptions (codebook, nr of antenna elements used, etc) | N/A | N/A |
| Additional notes, if any | LoS detector and Outlier rejection algorithm based on relative path powers | LoS detector and Outlier rejection algorithm based on relative path powers |

##### C.2.1.4.2	Positioning accuracy evaluation results for NR positioning enhancements

| Case | UEs | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1, [InF-SH, UL-TDOA, FR1,100 MHz] | Convex UEs | 0.38 | 0.55 | 1.06 | 1.94 |
| Case 2, [InF-DH, UL-TDOA, FR1, 100 MHz] | Convex UEs | 1.60 | 2.35 | 3.08 | 4.2 |

#### C.2.1.5	Results from source [5]

##### C.2.1.5.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.5.1-1- Table C.2.1.5.1-18.

Table C.2.1.5.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E1] (InF-SH, FR1) | [Case E2] (InF-SH, FR1) | [Case E3] (InF-SH, FR2) | [Case E4] (InF-SH, FR2) | [Case E5] (InF-DH, FR1) | [Case E6] (InF-DH, FR1) | [Case E7] (InF-DH, FR2) | [Case E8] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP | DL-TDOA taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-2: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E9] (InF-SH, FR1) | [Case E10] (InF-SH, FR1) | [Case E11] (InF-SH, FR2) | [Case E12] (InF-SH, FR2) | [Case E13] (InF-DH, FR1) | [Case E14] (InF-DH, FR1) | [Case E15] (InF-DH, FR2) | [Case E16] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-3: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E17] (InF-SH, FR1) | [Case E18] (InF-SH, FR1) | [Case E19] (InF-SH, FR2) | [Case E20] (InF-SH, FR2) | [Case E21] (InF-DH, FR1) | [Case E22] (InF-DH, FR1) | [Case E23] (InF-DH, FR2) | [Case E24] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP | UL-TDOA taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-4: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E25] (InF-SH, FR1) | [Case E26] (InF-SH, FR1) | [Case E27] (InF-SH, FR2) | [Case E28] (InF-SH, FR2) | [Case E29] (InF-DH, FR1) | [Case E30] (InF-DH, FR1) | [Case E31] (InF-DH, FR2) | [Case E32] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-5: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E33] (InF-SH, FR1) | [Case E34] (InF-SH, FR1) | [Case E35] (InF-SH, FR2) | [Case E36] (InF-SH, FR2) | [Case E37] (InF-DH, FR1) | [Case E38] (InF-DH, FR1) | [Case E39] (InF-DH, FR2) | [Case E40] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP | Multi-RTT taylor series RSRP |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-6: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E41] (InF-SH, FR1) | [Case E42] (InF-SH, FR1) | [Case E43] (InF-SH, FR2) | [Case E44] (InF-SH, FR2) | [Case E45] (InF-DH, FR1) | [Case E46] (InF-DH, FR1) | [Case E47] (InF-DH, FR2) | [Case E48] (InF-DH, FR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 28GHz | 28GHz | 3.5GHz | 3.5GHz | 28GHz | 28GHz |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz | 120kHz | 30kHz | 30kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400kHz | 400kHz | 100MHz | 100MHz | 400kHz | 400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-7: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E49] (InF-SH, FR1) | [Case E50] (InF-SH, FR1) | [Case E51] (InF-SH, FR1) | [Case E52] (InF-SH, FR1) | [Case E53] (InF-SH, FR1) | [Case E54] (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | 100% LOS detection probability without RAIM | 100% LOS detection probability with RAIM | 95% LOS detection probability without RAIM | 90% LOS detection probability without RAIM | no LOS detection without RAIM |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-8: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E55] (InF-DH, FR1) | [Case E56] (InF-DH, FR1) | [Case E57] (InF-DH, FR1) | [Case E58] (InF-DH, FR1) | [Case E59] (InF-DH, FR1) | [Case E60] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | 100% LOS detection probability without RAIM | 100% LOS detection probability with RAIM | 95% LOS detection probability without RAIM | 90% LOS detection probability without RAIM | no LOS detection without RAIM |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-9: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E61] (InF-DH, FR1) | [Case E62] (InF-DH, FR1) | [Case E63] (InF-DH, FR1) | [Case E64] (InF-DH, FR1) | [Case E65] (InF-DH, FR1) | [Case E66] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM timing report granularity 0.5ns | RAIM timing report granularity 1ns | RAIM timing report granularity 2ns | RAIM timing report granularity 0.5ns | RAIM timing report granularity 1ns | RAIM timing report granularity 2ns |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-10: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E67] (InF-SH, FR1) | [Case E68] (InF-SH, FR1) | [Case E69] (InF-SH, FR1) | [Case E70] (InF-SH, FR1) | [Case E71] (InF-SH, FR1) | [Case E72] (InF-SH, FR1) | [Case E73] (InF-SH, FR1) | [Case E74] (InF-SH, FR1) | [Case E75] (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | BS timing error 0.5ns, UE timing error 0.5ns | BS timing error 0.5ns, UE timing error 1ns | BS timing error 0.5ns, UE timing error 2ns | BS timing error 0.5ns, UE timing error 3ns | BS timing error 0.5ns, UE timing error 5ns | BS timing error 1ns, UE timing error 0.5ns | BS timing error 2ns, UE timing error 0.5ns | BS timing error 3ns, UE timing error 0.5ns | BS timing error 5ns, UE timing error 0.5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-11: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E76] (InF-DH, FR1) | [Case E77] (InF-DH, FR1) | [Case E78] (InF-DH, FR1) | [Case E79] (InF-DH, FR1) | [Case E80] (InF-DH, FR1) | [Case E81] (InF-DH, FR1) | [Case E82] (InF-DH, FR1) | [Case E83] (InF-DH, FR1) | [Case E84] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | BS timing error 0.5ns, UE timing error 0.5ns | BS timing error 0.5ns, UE timing error 1ns | BS timing error 0.5ns, UE timing error 2ns | BS timing error 0.5ns, UE timing error 3ns | BS timing error 0.5ns, UE timing error 5ns | BS timing error 1ns, UE timing error 0.5ns | BS timing error 2ns, UE timing error 0.5ns | BS timing error 3ns, UE timing error 0.5ns | BS timing error 5ns, UE timing error 0.5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-12: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E85] (InF-SH, FR1) | [Case E86] (InF-SH, FR1) | [Case E87] (InF-SH, FR1) | [Case E88] (InF-SH, FR1) | [Case E89] (InF-SH, FR1) | [Case E90] (InF-SH, FR1) | [Case E91] (InF-SH, FR1) | [Case E92] (InF-SH, FR1) | [Case E93] (InF-SH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH | InF-SH |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | BS timing error 0.5ns, UE timing error 0.5ns | BS timing error 0.5ns, UE timing error 1ns | BS timing error 0.5ns, UE timing error 2ns | BS timing error 0.5ns, UE timing error 3ns | BS timing error 0.5ns, UE timing error 5ns | BS timing error 1ns, UE timing error 0.5ns | BS timing error 2ns, UE timing error 0.5ns | BS timing error 3ns, UE timing error 0.5ns | BS timing error 5ns, UE timing error 0.5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-13: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E94] (InF-DH, FR1) | [Case E95] (InF-DH, FR1) | [Case E96] (InF-DH, FR1) | [Case E97] (InF-DH, FR1) | [Case E98] (InF-DH, FR1) | [Case E99] (InF-DH, FR1) | [Case E100] (InF-DH, FR1) | [Case E101] (InF-DH, FR1) | [Case E102] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | BS timing error 0.5ns, UE timing error 0.5ns | BS timing error 0.5ns, UE timing error 1ns | BS timing error 0.5ns, UE timing error 2ns | BS timing error 0.5ns, UE timing error 3ns | BS timing error 0.5ns, UE timing error 5ns | BS timing error 1ns, UE timing error 0.5ns | BS timing error 2ns, UE timing error 0.5ns | BS timing error 3ns, UE timing error 0.5ns | BS timing error 5ns, UE timing error 0.5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-14: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E103] (InF-SH, FR1) | [Case E104] (InF-SH, FR1) | [Case E105] (InF-SH, FR1) | [Case E106] (InF-DH, FR1) | [Case E107] (InF-DH, FR1) | [Case E108] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 50MHz | 100MHz | 50MHz+50MHz | 50MHz | 100MHz | 50MHz+50MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN + 1 port, ZC sequence |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | DL:7.78dB UL:6dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM Aggregation of DL positioning frequency layers | RAIM | RAIM | RAIM Aggregation of DL positioning frequency layers |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-15: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E109] (IOO, FR1) | [Case E110] (IOO, FR2) | [Case E111] (IOO, FR1) | [Case E112] (IOO, FR2) | [Case E113] (IOO, FR1) | [Case E114] (IOO, FR2) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | IOO | IOO | IOO | IOO | IOO | IOO |
| Carrier frequency | 4GHz | 30GHz | 4GHz | 30GHz | 4GHz | 30GHz |
| Subcarrier spacing | 30kHz | 120kHz | 30kHz | 120kHz | 30kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) + SRS (comb-4 4 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | UL-TDOA taylor series first/median peak | Multi-RTT taylor series first/median peak | Multi-RTT taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-16: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E115] (InF-SH, FR1) | [Case E116] (InF-SH, FR1) | [Case E117] (InF-SH, FR1) | [Case E118] (InF-SH, FR1) | [Case E119] (InF-DH, FR1) | [Case E120] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (60%, 6, 2) | InF-DH (60%, 6, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | SRS (comb-4 4 symbols) | SRS (comb-4 4 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN | 1 port, QPSK-PN | 1 port, ZC sequence | 1 port, ZC sequence | 1 port, QPSK-PN | 1 port, QPSK-PN |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |
| Power-boosting level | DL:7.78dB | DL:7.78dB | UL:6dB | UL:6dB | DL:7.78dB | DL:7.78dB |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution | super resolution | super resolution | Machine learning | Machine learning |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | UL-AOA+AOA taylor series first/median peak | UL-AOA+AOA taylor series first/median peak | Machine learning | Machine learning |
| Network synchronization assumptions | sync error 50ns | Perfect sync | sync error 50ns | Perfect sync | Perfect sync | sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 | BS timing error 5ns, UE timing error 0.5ns | 0 | BS timing error 5ns, UE timing error 0.5ns | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM Sync error reduced by differential positioning | RAIM Rx/Tx timing error reduced by differential positioning | RAIM Sync error reduced by UL-TDOA+AOA | RAIM Rx/Tx timing error reduced by UL-TDOA+AOA | Machine learning | Machine learning |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.5.1-17: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E121] (InF-SH, FR1) | [Case E122] (InF-SH, FR1) | [Case E123] (InF-SH, FR1) | [Case E124] (InF-SH, FR1) | [Case E125] (InF-DH, FR1) | [Case E126] (InF-DH, FR1) | [Case E127] (InF-DH, FR1) | [Case E128] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | InF-SH | InF-SH | InF-DH (40%, 2, 2) | InF-DH (40%, 2, 2) | nF-DH (40%, 2, 2) | nF-DH (40%, 2, 2) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 50M+50M |  |  |  |  |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) |  |  |  |  |  |  |  |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak |  |  |  |  |  |  |  |
| Network synchronization assumptions | Perfect sync |  |  |  |  |  |  |  |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM Aggregation of DL positioning frequency layers Timing offset 1ns | RAIM Aggregation of DL positioning frequency layers Timing offset 5ns | RAIM Aggregation of DL positioning frequency layers Timing offset 10ns | RAIM Aggregation of DL positioning frequency layers Timing offset 20ns | RAIM Aggregation of DL positioning frequency layers Timing offset 1ns | RAIM Aggregation of DL positioning frequency layers Timing offset 5ns | RAIM Aggregation of DL positioning frequency layers Timing offset 10ns | RAIM Aggregation of DL positioning frequency layers Timing offset 20ns |
| Additional notes, if any |  |  |  |  |  |  |  |  |

Table C.2.1.5.1-18: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E-V1] (InF-SH, FR1) | [Case E-V2] (InF-DH, FR1) | [Case E- V3] (InF-SH, FR1) | [Case E-V4] (InF-DH, FR1) | [Case E- V5] (InF-SH, FR1) | [Case E-V6] (InF-DH, FR1) | [Case E-V7] (InF-SH, FR1) | [Case E-V8] (InF-DH, FR1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH (BS height = 8m UE height =1.5m) | InF-DH (40%, 2, 2) (BS height = 8m UE height =1.5m) | InF-SH (BS height = {4,8}m UE height =1.5m) | InF-DH (40%, 2, 2) (BS height = {4,8}m UE height =1.5m) | InF-SH (BS height =8m UE height =[0.5,2]m) | InF-DH (40%, 2, 2), (BS height =8m UE height =[0.5,2]m) | InF-SH (BS height = {4,8}m UE height =[0.5,2]m) | InF-DH (40%, 2, 2) (BS height = {4,8}m UE height =[0.5,2]m) |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) | R16 PRS (comb-6 6 symbols) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN |  |  |  |  |  |  |  |
| Number of sites | 18 (4 sites are chosen) |  |  |  |  |  |  |  |
| Number of symbols used per occasion | 1 |  |  |  |  |  |  |  |
| number of occasions used per positioning estimate | 1 |  |  |  |  |  |  |  |
| Power-boosting level | 7.78dB |  |  |  |  |  |  |  |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |  |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak | DL-TDOA taylor series first/median peak |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides |  |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | codebook |  |  |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM | RAIM |
| Additional notes, if any |  |  |  |  |  |  |  |  |

##### C.2.1.5.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.5.2-1:

Table C.2.1.5.2-1: NR positioning enhancements - horizontal location error results from [5]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.039 | 0.049 | 0.072 | 0.099 |
|  | (Optional) All UEs | 0.049 | 0.080 | 0.20 | 0.89 |
| [Case E2], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.25 | 13.23 | 17.16 | 26.57 |
|  | (Optional) All UEs | 13.09 | 17.89 | 25.44 | 34.70 |
| [Case E3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.009 | 0.012 | 0.016 | 0.024 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.042 | 0.34 |
| [Case E4], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.78 | 12.77 | 15.84 | 26.39 |
|  | (Optional) All UEs | 13.52 | 17.85 | 23.58 | 33.52 |
| [Case E5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.058 | 0.10 | 0.18 | 4.43 |
|  | (Optional) All UEs | 0.11 | 0.26 | 0.85 | 5.72 |
| [Case E6], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.32 | 11.97 | 14.63 | 21.94 |
|  | (Optional) All UEs | 10.80 | 14.46 | 17.66 | 23.84 |
| [Case E7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.013 | 0.026 | 4.35 |
|  | (Optional) All UEs | 0.015 | 0.027 | 0.21 | 4.98 |
| [Case E8], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.52 | 13.61 | 16.58 | 18.89 |
|  | (Optional) All UEs | 11.74 | 15.26 | 18.18 | 22.95 |
| [Case E9], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.30 |
| [Case E10], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.42 | 13.97 | 17.67 | 25.67 |
|  | (Optional) All UEs | 13.67 | 17.73 | 23.65 | 37.47 |
| [Case E11], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.009 | 0.012 | 0.017 | 0.024 |
|  | (Optional) All UEs | 0.012 | 0.019 | 0.033 | 0.10 |
| [Case E12], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 10.84 | 13.91 | 17.53 | 23.80 |
|  | (Optional) All UEs | 12.57 | 16.84 | 24.54 | 34.93 |
| [Case E13], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
|  | (Optional) All UEs | 0.087 | 0.14 | 0.25 | 0.62 |
| [Case E14], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 8.91 | 12.07 | 14.26 | 19.69 |
|  | (Optional) All UEs | 11.42 | 15.01 | 17.54 | 24.96 |
| [Case E15], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.011 | 0.015 | 0.018 | 0.034 |
|  | (Optional) All UEs | 0.015 | 0.022 | 0.040 | 0.32 |
| [Case E16], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 9.94 | 13.35 | 15.10 | 18.17 |
|  | (Optional) All UEs | 11.78 | 14.66 | 18.03 | 21.48 |
| [Case E17], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.037 | 0.049 | 0.061 | 0.10 |
|  | (Optional) All UEs | 0.051 | 0.086 | 0.17 | 0.98 |
| [Case E18], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.80 | 13.23 | 17.04 | 26.38 |
|  | (Optional) All UEs | 13.09 | 17.30 | 25.36 | 35.25 |
| [Case E19], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.009 | 0.012 | 0.016 | 0.034 |
|  | (Optional) All UEs | 0.012 | 0.019 | 0.036 | 0.31 |
| [Case E20], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.40 | 13.17 | 19.07 | 26.42 |
|  | (Optional) All UEs | 13.17 | 17.91 | 24.74 | 35.56 |
| [Case E21], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.058 | 0.098 | 0.18 | 5.48 |
|  | (Optional) All UEs | 0.10 | 0.21 | 1.14 | 5.98 |
| [Case E22], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.91 | 13.28 | 16.02 | 22.90 |
|  | (Optional) All UEs | 12.26 | 15.28 | 18.57 | 23.85 |
| [Case E23], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.020 | 0.089 | 4.55 |
|  | (Optional) All UEs | 0.016 | 0.029 | 0.50 | 5.96 |
| [Case E24], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.52 | 13.61 | 16.21 | 18.75 |
|  | (Optional) All UEs | 12.23 | 15.25 | 18.43 | 23.60 |
| [Case E25], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.037 | 0.048 | 0.060 | 0.083 |
|  | (Optional) All UEs | 0.050 | 0.072 | 0.13 | 0.25 |
| [Case E26], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.50 | 14.13 | 17.81 | 26.09 |
|  | (Optional) All UEs | 13.61 | 17.81 | 23.77 | 37.64 |
| [Case E27], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.009 | 0.010 | 0.016 | 0.032 |
|  | (Optional) All UEs | 0.011 | 0.018 | 0.033 | 0.11 |
| [Case E28], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.50 | 14.38 | 18.06 | 26.01 |
|  | (Optional) All UEs | 14.13 | 16.70 | 24.49 | 36.40 |
| [Case E29], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.053 | 0.073 | 0.11 | 0.19 |
|  | (Optional) All UEs | 0.086 | 0.16 | 0.27 | 0.65 |
| [Case E30], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 10.0 | 13.35 | 16.53 | 22.54 |
|  | (Optional) All UEs | 12.77 | 15.12 | 18.62 | 26.28 |
| [Case E31], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.011 | 0.014 | 0.019 | 0.043 |
|  | (Optional) All UEs | 0.015 | 0.022 | 0.051 | 0.34 |
| [Case E32], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 9.93 | 13.21 | 16.55 | 20.23 |
|  | (Optional) All UEs | 12.11 | 15.60 | 19.18 | 24.03 |
| [Case E33], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.040 | 0.052 | 0.071 | 0.11 |
|  | (Optional) All UEs | 0.054 | 0.076 | 0.12 | 0.51 |
| [Case E34], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.043 | 0.055 | 0.069 | 0.14 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.62 |
| [Case E35], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.014 | 0.019 | 0.049 |
|  | (Optional) All UEs | 0.013 | 0.021 | 0.036 | 0.14 |
| [Case E36], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.011 | 0.015 | 0.019 | 0.12 |
|  | (Optional) All UEs | 0.014 | 0.021 | 0.034 | 0.19 |
| [Case E37], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.068 | 0.12 | 0.23 | 4.89 |
|  | (Optional) All UEs | 0.091 | 0.14 | 0.78 | 6.39 |
| [Case E38], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.063 | 0.092 | 0.26 | 5.12 |
|  | (Optional) All UEs | 0.092 | 0.16 | 0.88 | 6.84 |
| [Case E39], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.013 | 0.019 | 0.11 | 4.12 |
|  | (Optional) All UEs | 0.018 | 0.065 | 0.59 | 4.82 |
| [Case E40], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.013 | 0.019 | 0.039 | 4.26 |
|  | (Optional) All UEs | 0.033 | 0.081 | 0.78 | 5.49 |
| [Case E41], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.040 | 0.050 | 0.068 | 0.092 |
|  | (Optional) All UEs | 0.044 | 0.066 | 0.097 | 0.24 |
| [Case E42], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.048 | 0.058 | 0.067 | 0.092 |
|  | (Optional) All UEs | 0.049 | 0.069 | 0.12 | 0.23 |
| [Case E43], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.010 | 0.014 | 0.018 | 0.030 |
|  | (Optional) All UEs | 0.013 | 0.019 | 0.030 | 0.055 |
| [Case E44], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.012 | 0.016 | 0.021 | 0.037 |
|  | (Optional) All UEs | 0.016 | 0.021 | 0.033 | 0.072 |
| [Case E45], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.059 | 0.083 | 0.11 | 0.19 |
|  | (Optional) All UEs | 0.078 | 0.12 | 0.18 | 0.31 |
| [Case E46], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.060 | 0.091 | 0.13 | 0.20 |
|  | (Optional) All UEs | 0.086 | 0.13 | 0.19 | 0.35 |
| [Case E47], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.012 | 0.016 | 0.020 | 0.048 |
|  | (Optional) All UEs | 0.013 | 0.018 | 0.029 | 0.11 |
| [Case E48], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.013 | 0.016 | 0.021 | 0.051 |
|  | (Optional) All UEs | 0.014 | 0.020 | 0.026 | 0.11 |
| [Case E49], [SH, perfect sync], [FR1], [DL-TDOA, RAIM] (Case E9) | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
| [Case E50], [SH, perfect sync], [FR1], [DL-TDOA, known LOS without RAIM] | Convex UEs | 0.037 | 0.049 | 0.073 | 0.096 |
| [Case E51], [SH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] | Convex UEs | 0.037 | 0.047 | 0.063 | 0.083 |
| [Case E52], [SH, perfect sync], [FR1], [DL-TDOA, 95% LOS without RAIM] | Convex UEs | 0.039 | 0.052 | 0.080 | 2.86 |
| [Case E53], [SH, perfect sync], [FR1], [DL-TDOA, 90% LOS without RAIM] | Convex UEs | 0.041 | 0.052 | 0.083 | 4.54 |
| [Case E54], [SH, perfect sync], [FR1], [DL-TDOA, unknown LOS without RAIM] | Convex UEs | 0.042 | 0.055 | 0.11 | 4.62 |
| [Case E55], [DH, perfect sync], [FR1], [DL-TDOA, RAIM] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
| [Case E56], [DH, perfect sync], [FR1], [DL-TDOA, known LOS without RAIM] | Convex UEs | 0.060 | 0.097 | 0.15 | 0.33 |
| [Case E57], [DH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] | Convex UEs | 0.055 | 0.071 | 0.11 | 0.17 |
| [Case E58], [DH, perfect sync], [FR1], [DL-TDOA, 95% LOS without RAIM] | Convex UEs | 0.042 | 0.064 | 0.30 | 3.40 |
| [Case E59], [DH, perfect sync], [FR1], [DL-TDOA, 90% LOS without RAIM] | Convex UEs | 0.043 | 0.065 | 1.82 | 3.43 |
| [Case E60], [DH, perfect sync], [FR1], [DL-TDOA, unknown LOS without RAIM] | Convex UEs | 0.048 | 1.05 | 3.49 | 8.64 |
| [Case E61], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] | Convex UEs | 0.070 | 0.092 | 0.12 | 0.16 |
| [Case E62], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] | Convex UEs | 0.10 | 0.13 | 0.16 | 0.21 |
| [Case E63], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] | Convex UEs | 0.21 | 0.28 | 0.32 | 0.47 |
| [Case E64], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] | Convex UEs | 0.063 | 0.086 | 0.11 | 0.17 |
| [Case E65], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] | Convex UEs | 0.12 | 0.15 | 0.21 | 0.35 |
| [Case E66], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] | Convex UEs | 0.22 | 0.27 | 0.33 | 0.59 |
| [Case E67], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.19 | 0.23 | 0.30 |
| [Case E68], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.34 |
| [Case E69], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.36 |
| [Case E70], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.14 | 0.18 | 0.22 | 0.35 |
| [Case E71], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.14 | 0.17 | 0.23 | 0.37 |
| [Case E72], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.18 | 0.23 | 0.29 | 0.42 |
| [Case E73], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.37 | 0.45 | 0.60 | 0.83 |
| [[Case E74], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.47 | 0.61 | 0.76 | 1.07 |
| [Case E75], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.80 | 1.00 | 1.24 | 1.87 |
| [Case E76], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.31 |
| [Case E77], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.13 | 0.17 | 0.21 | 0.32 |
| [Case E78], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.13 | 0.18 | 0.21 | 0.32 |
| [Case E79], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.20 | 0.28 |
| [Case E80], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.12 | 0.18 | 0.22 | 0.31 |
| [Case E81], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.18 | 0.24 | 0.33 | 0.40 |
| [Case E82 [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.40 | 0.48 | 0.62 | 0.76 |
| [[Case E83], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.54 | 0.62 | 0.75 | 0.88 |
| [Case E84], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.78 | 1.08 | 1.19 | 1.94 |
| [Case E85], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.089 | 0.13 | 0.18 | 0.24 |
| [Case E86], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.094 | 0.13 | 0.17 | 0.23 |
| [Case E87], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.12 | 0.15 | 0.19 | 0.27 |
| [Case E88], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.23 | 0.33 |
| [Case E89], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.16 | 0.25 | 0.34 | 0.44 |
| [Case E90], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.12 | 0.17 | 0.22 | 0.28 |
| [Case E91], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.17 | 0.21 | 0.28 | 0.34 |
| [[Case E92], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.39 | 0.49 | 0.61 | 0.76 |
| [Case E93], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.56 | 0.69 | 0.88 | 1.26 |
| [Case E94], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.10 | 0.12 | 0.17 | 0.24 |
| [Case E95], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.10 | 0.13 | 0.17 | 0.24 |
| [Case E96], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.11 | 0.15 | 0.20 | 0.28 |
| [Case E97], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.23 | 0.36 |
| [Case E98], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.18 | 0.25 | 0.33 | 0.46 |
| [Case E99], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.20 | 0.26 | 0.34 |
| [Case E100], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.21 | 0.26 | 0.35 | 0.48 |
| [[Case E101], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.37 | 0.51 | 0.62 | 0.87 |
| [Case E102], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.57 | 0.71 | 0.98 | 1.28 |
| [Case E103], [SH, perfect sync], [FR1], [50M] | Convex UEs | 0.12 | 0.16 | 0.20 | 0.31 |
|  | (Optional) All UEs | 0.16 | 0.25 | 0.38 | 0.95 |
| [Case E104], [SH, perfect sync], [FR1], [100M] (Case E9) | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.25 |
| [Case E105], [SH, perfect sync], [FR1], [50M+50M] | Convex UEs | 0.072 | 0.12 | 0.16 | 0.21 |
|  | (Optional) All UEs | 0.11 | 0.17 | 0.28 | 0.58 |
| [Case E106], [DH, perfect sync], [FR1], [50M] | Convex UEs | 0.11 | 0.16 | 0.21 | 0.44 |
|  | (Optional) All UEs | 0.17 | 0.26 | 0.50 | 0.98 |
| [Case E107], [DH, perfect sync], [FR1], [100M] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
|  | (Optional) All UEs | 0.087 | 0.14 | 0.25 | 0.62 |
| [Case E108], [DH, perfect sync], [FR1], [50M+50M] | Convex UEs | 0.069 | 0.095 | 0.15 | 0.23 |
|  | (Optional) All UEs | 0.11 | 0.20 | 0.39 | 0.91 |
| [Case E109], [IOO scenario, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] (Case 63 in [5]) | all UEs | 0.11 | 0.19 | 0.35 | 0.80 |
| [Case E110], [IOO scenario, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] (Case 64 in [5]) | all UEs | 0.03 | 0.06 | 0.13 | 0.54 |
| [Case E111], [IOO scenario, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] (Case 65 in [5]) | all UEs | 0.11 | 0.19 | 0.38 | 0.84 |
| [Case E112], [IOO scenario, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] (Case 66 in [5]) | all UEs | 0.03 | 0.06 | 0.13 | 0.56 |
| [Case E113], [IOO scenario, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] (Case 67 in [5]) | all UEs | 0.11 | 0.17 | 0.37 | 0.68 |
| [Case E114], [IOO scenario, perfect sync], [FR2], [Multi-RTT MUSIC, select based on first/median peak] (Case 68 in [5]) | all UEs | 0.03 | 0.05 | 0.11 | 0.50 |
| [Case E115], [SH, sync error 50ns], [FR1], [DL-TDOA] [sync error reduced by differential positioning] | Convex UEs | 0.046 | 0.061 | 0.077 | 0.11 |
| [Case E116], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] [Rx/Tx timing error reduced by differential positioning] | Convex UEs | 0.060 | 0.074 | 0.087 | 0.13 |
| [Case E117], [SH, sync error 50ns], [FR1], [UL-TDOA] [sync error reduced by UL-TDOA+AOA] | Convex UEs | 1.24 | 1.39 | 1.69 | 3.16 |
| [Case E118], [SH, perfect sync], [FR1], [UL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] [Rx/Tx timing error reduced by UL-TDOA+AOA] | Convex UEs | 0.64 | 0.85 | 1.08 | 1.50 |
| [Case E119], [DH, (60%,6,2), perfect sync], [FR1], [machine learning] | all UEs | 2.49 | 3.05 | 3.64 | 4.60 |
| [Case E120], [DH, (60%,6,2), sync error 50ns], [FR1], [machine learning] | all UEs | 2.24 | 2.99 | 3.68 | 5.12 |
| [Case E121], [SH, perfect sync], [FR1], [50M+50M] [timing offset 1ns] | Convex UEs | 0.19 | 0.25 | 0.33 | 0.46 |
| [Case E122], [SH, perfect sync], [FR1], [50M+50M] [timing offset 5ns] | Convex UEs | 0.93 | 1.17 | 1.58 | 2.03 |
| [Case E123], [SH, perfect sync], [FR1], [50M+50M] [timing offset 10ns] | Convex UEs | 2.15 | 2.57 | 3.55 | 5.46 |
| [Case E124], [SH, perfect sync], [FR1], [50M+50M] [timing offset 20ns] | Convex UEs | 4.60 | 5.21 | 6.81 | 10.05 |
| [Case E125], [DH, perfect sync], [FR1], [50M+50M] [timing offset 1ns] | Convex UEs | 0.24 | 0.36 | 0.51 | 0.96 |
| [Case E126], [DH, perfect sync], [FR1], [50M+50M] [timing offset 5ns] | Convex UEs | 1.03 | 1.40 | 2.15 | 3.90 |
| [Case E127], [DH, perfect sync], [FR1], [50M+50M] [timing offset 10ns] | Convex UEs | 2.08 | 2.79 | 3.97 | 8.34 |
| [Case E128], [DH, perfect sync], [FR1], [50M+50M] [timing offset 20ns] | Convex UEs | 3.70 | 5.94 | 7.48 | 10.69 |

Evaluation results of vertical location error for NR positioning enhancements are provided in Table C.2.1.5.2-2:

Table C.2.1.5.2-2: NR positioning enhancements - vertical location error results from [5]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E-V1], [SH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.19 | 0.30 | 0.46 | 0.58 |
|  | (Optional) All UEs | 0.19 | 0.33 | 0.54 | 0.84 |
| [Case E-V2], [DH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.12 | 0.20 | 0.41 | 0.64 |
|  | (Optional) All UEs | 0.13 | 0.22 | 0.41 | 1.39 |
| [Case E-V3], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.24 | 0.42 | 0.62 | 1.25 |
|  | (Optional) All UEs | 0.24 | 0.39 | 0.70 | 1.38 |
| [Case E-V4], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 1.03 | 2.05 | 2.68 | 4.62 |
|  | (Optional) All UEs | 1.21 | 2.50 | 2.94 | 5.24 |
| [Case E-V5], [SH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.16 | 0.25 | 0.45 | 0.66 |
|  | (Optional) All UEs | 0.19 | 0.30 | 0.46 | 0.76 |
| [Case E-V6], [DH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.82 | 1.17 | 2.03 | 3.16 |
|  | (Optional) All UEs | 0.91 | 1.68 | 2.75 | 3.47 |
| [Case E-V7], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.28 | 0.45 | 0.72 | 1.27 |
|  | (Optional) All UEs | 0.28 | 0.45 | 0.76 | 1.48 |
| [Case E-V8], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.67 | 1.77 | 3.02 | 4.93 |
|  | (Optional) All UEs | 0.86 | 2.39 | 3.12 | 5.98 |

#### C.2.1.6	Results from source [12]

##### C.2.1.6.1	Description of evaluation scenarios

The following enhancements for NR positioning are evaluated:

- NLOS classification to mitigate NLOS

- Multipath mitigation method

- Combination of NLOS classification and multipath mitigation

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.6.1-1.

Table C.2.1.6.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | Case1 ( InF-SH, FR1) | Case2 ( InF-DH, FR1) |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-DH (40%,2,2) |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL-PRS | DL-PRS |
| Reference signal (type of sequence, number of ports, …) | Gold, single port | Gold, single port |
| Number of sites | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal | Ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | ML | ML |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA Chan | DL-TDOA Chan |
| Network synchronization assumptions | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Ideal | Ideal |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based | Tx codebook-based |
| Evaluated Enhancement for Rel.17 |  |  |
| Additional notes, if any |  |  |

The following cases are evalueated:

| Case number | Scenarios | Band | Positioning method | Enhancement |
| --- | --- | --- | --- | --- |
| Case 1 | InF-SH | FR1 | DL-TDOA | NLOS classification |
| Case 2 | InF-SH | FR1 | DL-TDOA | NLOS mitigation |
| Case 3 | InF-SH | FR1 | DL-TDOA | NLOS mitigation + NLOS classification |
| Case 4 | InF-DH | FR1 | DL-TDOA | NLOS classification |
| Case 5 | InF-DH | FR1 | DL-TDOA | NLOS mitigation |
| Case 6 | InF-DH | FR1 | DL-TDOA | NLOS mitigation + NLOS classification |
| Case 7 | InF-SH | FR1 | DL-TDOA | Ideal NLOS classification |
| Case 8 | InF-DH | FR1 | DL-TDOA | Ideal NLOS classification |

##### C.2.1.6.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.6.2-1:

Table C.2.1.6.2-1: NR positioning enhancements - horizontal location error results from [12]

| Cases1 | UEs | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case1, (InF-SH, NLOS classification FR1) | Convex UEs | 0.17 | 0.22 | 0.33 | 0.87 |
| Case2, (InF-SH, NLOS mitigation FR1) | Convex UEs | 0.20 | 0.32 | 1.07 | 3.12 |
| Case3, (InF-SH, NLOS classification + NLOS mitigation FR1) | Convex UEs | 0.16 | 0.20 | 0.24 | 0.31 |
| Case4, (InF-DH, NLOS classification FR1) | Convex UEs | 2.08 | 4.13 | 7.29 | 12.5 |
| Case5, (InF-DH, NLOS mitigation FR1) | Convex UEs | 0.73 | 1.73 | 3.0 | 5.49 |
| Case6, (InF-DH, NLOS classification+ NLOS mitigation FR1) | Convex UEs | 0.21 | 0.29 | 0.75 | 2.47 |
| Case7, (InF-SH, Ideal NLOS classification FR1) | Convex UEs | 0.14 | 0.17 | 0.21 | 0.23 |
| Case8, (InF-DH, Ideal NLOS classification FR1) | Convex UEs | 0.16 | 0.20 | 0.25 | 0.31 |

#### C.2.1.7	Results from source [10]

##### C.2.1.7.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.7.1-1.

Table C.2.1.7.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | Case 13, InF-SH, FR1, DL-TDOA | Case 14, InF-DH, FR1, DL-TDOA | Case 15, InF-SH, FR2, DL-TDOA | Case 16, InF-DH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 | DL PRS: Comb-2 | DL PRS: Comb-2 | DL PRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port | DL PRS: single port | DL PRS: single port | DL PRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols | DL PRS: 2 symbols | DL PRS: 2 symbols | DL PRS: 2 symbols |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB | DL PRS: 3 dB | DL PRS: 3 dB | DL PRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | Tx beam sweeping | Tx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 17, InF-SH, FR1, UL-TDOA | Case 18, InF-DH, FR1, UL-TDOA | Case 19, InF-SH, FR2, UL-TDOA | Case 20, InF-DH, FR2, UL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | UL SRS: Comb-2 | UL SRS: Comb-2 | UL SRS: Comb-2 | UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | UL SRS: single port | UL SRS: single port | UL SRS: single port | UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | UL SRS: 1 symbol | UL SRS: 1 symbol | UL SRS: 1 symbol | UL SRS: 1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | UL SRS: 3 dB | UL SRS: 3 dB | UL SRS: 3 dB | UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | Rx beam sweeping | Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 21, InF-SH, FR1, Multi-RTT | Case 22, InF-DH, FR1, Multi-RTT | Case 23, InF-SH, FR2, Multi-RTT | Case 24, InF-DH, FR2, Multi-RTT |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 25, InF-SH, FR1, Multi-RTT+ vertical AoA | Case 26, InF-DH, FR1, Multi-RTT+ vertical AoA | Case 27, InF-SH, FR2, Multi-RTT+ vertical AoA | Case 28, InF-DH, FR2, Multi-RTT+ vertical AoA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold Vertical AoA estimation based of detected FAP | FAP timing estimation based on adaptive threshold Vertical AoA estimation based of detected FAP | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 29, InF-SH, FR1, Multi-RTT with Tx/Rx timing error compensation | Case 30, InF-DH, FR1, Multi-RTT RTT with Tx/Rx timing error compensation | Case 31, InF-SH, FR2, Multi-RTT RTT with Tx/Rx timing error compensation | Case 32, InF-DH, FR2, Multi-RTT RTT with Tx/Rx timing error compensation |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 400 MHz | 400 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | UE Tx/Rx timing error is modeled with X = 10 ns gNB Tx/Rx timing error is modeled with Y = 5 ns | UE Tx/Rx timing error is modeled with X = 10 ns gNB Tx/Rx timing error is modeled with Y = 5 ns | UE Tx/Rx timing error is modeled with X = 10 ns gNB Tx/Rx timing error is modeled with Y = 5 ns | UE Tx/Rx timing error is modeled with X = 10 ns gNB Tx/Rx timing error is modeled with Y = 5 ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification | LOS/NLOS classification |

| Parameter | Case 33, InF-SH, FR1, Multi-RTT | Case 34, InF-DH, FR1, Multi-RTT | Case 35, InF-SH, FR2, Multi-RTT | Case 36, InF-DH, FR2, Multi-RTT |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 3.5 GHz | 3.5 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 400 MHz | 400 MHz | 800 MHz | 800 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 | DL PRS: Comb-2 UL SRS: Comb-2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port | DL PRS: single port UL SRS: single port |
| Number of sites | 1 | 1 | 3 | 3 |
| Number of symbols used per occasion | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol | DL PRS: 2 symbols UL SRS:1 symbol |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB | DL PRS: 3 dB UL SRS: 3 dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold | FAP timing estimation based on adaptive threshold |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Taylor series | Taylor series | Taylor series | Taylor series |
| Network synchronization assumptions | Ideal synchronization | Ideal synchronization | Ideal synchronization | Ideal synchronization |
| UE/gNB Tx/Rx Calibration Error | No timing calibration error | No timing calibration error | No timing calibration error | No timing calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Tx/Rx beam sweeping | No Tx/Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping | DL PRS: Tx beam sweeping UL SRS: Rx beam sweeping |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Tx port | Single Tx port | Single Tx port | Single Tx port |
| Additional notes, if any | Ideal LOS classification Adjacent bandwidth | Ideal LOS classification Adjacent bandwidth | Ideal LOS classification Adjacent bandwidth | Ideal LOS classification Adjacent bandwidth |

##### C.2.1.7.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.7.2-1:

Table C.2.1.7.2-1: NR positioning enhancements - horizontal location error results from

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 13, InF-SH, FR1, DL-TDOA | Convex UEs | 0.11 | 0.15 | 0.23 | 0.37 |
| Case 14, InF-DH, FR1, DL-TDOA | Convex UEs | 0.22 | 0.56 | 1.2 | 1.94 |
| Case 15, InF-SH, FR2, DL-TDOA | Convex UEs | 0.06 | 0.08 | 0.11 | 0.21 |
| Case 16, InF-DH, FR2, DL-TDOA | Convex UEs | 0.08 | 0.14 | 0.22 | 0.89 |
| Case 17, InF-SH, FR1, UL-TDOA | Convex UEs | 0.12 | 0.17 | 0.25 | 0.4 |
| Case 18, InF-DH, FR1, UL-TDOA | Convex UEs | 0.39 | 0.81 | 1.45 | 4.4 |
| Case 19, InF-SH, FR2, UL-TDOA | Convex UEs | 0.08 | 0.12 | 0.15 | 0.24 |
| Case 20, InF-DH, FR2, UL-TDOA | Convex UEs | 0.12 | 0.17 | 0.28 | 0.68 |
| Case 21, InF-SH, FR1, Multi-RTT | Convex UEs | 0.07 | 0.1 | 0.13 | 0.17 |
| Case 22, InF-DH, FR1, Multi-RTT | Convex UEs | 0.1 | 0.15 | 0.2 | 0.3 |
| Case 23, InF-SH, FR2, Multi-RTT | Convex UEs | 0.05 | 0.07 | 0.08 | 0.1 |
| Case 24, InF-DH, FR2, Multi-RTT | Convex UEs | 0.07 | 0.09 | 0.12 | 0.17 |
| Case 25, InF-SH, FR1, Multi-RTT + vertical AoA | Convex UEs | 0.05 | 0.07 | 0.09 | 0.1 |
| Case 26, InF-DH, FR1, Multi-RTT + vertical AoA | Convex UEs | 0.07 | 0.1 | 0.13 | 0.19 |
| Case 27, InF-SH, FR2, Multi-RTT + vertical AoA | Convex UEs | 0.04 | 0.05 | 0.06 | 0.08 |
| Case 28, InF-DH, FR2, Multi-RTT + vertical AoA | Convex UEs | 0.06 | 0.07 | 0.09 | 0.2 |
| Case 29, InF-SH, FR1, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.09 | 0.13 | 0.19 | 0.28 |
| Case 30, InF-DH, FR1, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.15 | 0.24 | 0.36 | 0.55 |
| Case 31, InF-SH, FR2, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.06 | 0.08 | 0.11 | 0.14 |
| Case 32, InF-DH, FR2, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.09 | 0.14 | 0.5 | 8.7 |
| Case 33, InF-SH, FR1, Multi-RTT | Convex UEs | 0.03 | 0.04 | 0.05 | 0.06 |
| Case 34, InF-DH, FR1, Multi-RTT | Convex UEs | 0.05 | 0.07 | 0.09 | 0.11 |
| Case 35, InF-SH, FR2, Multi-RTT | Convex UEs | 0.02 | 0.03 | 0.04 | 0.05 |
| Case 36, InF-DH, FR2, Multi-RTT | Convex UEs | 0.04 | 0.05 | 0.06 | 0.07 |

Evaluation results of vertical location error for NR positioning enhancements are provided in Table C.2.1.7.2-2:

Table C.2.1.7.2-2: NR positioning enhancements - altitude location error results from

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 13, InF-SH, FR1, DL-TDOA | Convex UEs | 0.64 | 1.2 | 6.2 | 6.9 |
| Case 14, InF-DH, FR1, DL-TDOA | Convex UEs | 0.65 | 6.2 | 6.3 | 12.4 |
| Case 15, InF-SH, FR2, DL-TDOA | Convex UEs | 0.35 | 0.78 | 2.5 | 6.6 |
| Case 16, InF-DH, FR2, DL-TDOA | Convex UEs | 0.28 | 0.6 | 1.8 | 5.8 |
| Case 17, InF-SH, FR1, UL-TDOA | Convex UEs | 1.1 | 6.4 | 6.5 | 12.8 |
| Case 18, InF-DH, FR1, UL-TDOA | Convex UEs | 6.2 | 6.3 | 6.4 | 12.9 |
| Case 19, InF-SH, FR2, UL-TDOA | Convex UEs | 0.44 | 0.81 | 6.1 | 11.3 |
| Case 20, InF-DH, FR2, UL-TDOA | Convex UEs | 0.29 | 0.47 | 0.71 | 0.64 |
| Case 21, InF-SH, FR1, Multi-RTT | Convex UEs | 0.19 | 0.34 | 0.52 | 0.89 |
| Case 22, InF-DH, FR1, Multi-RTT | Convex UEs | 0.12 | 0.2 | 0.3 | 0.49 |
| Case 23, InF-SH, FR2, Multi-RTT | Convex UEs | 0.38 | 0.53 | 0.7 | 1 |
| Case 24, InF-DH, FR2, Multi-RTT | Convex UEs | 0.18 | 0.25 | 0.32 | 0.45 |
| Case 25, InF-SH, FR1, Multi-RTT + vertical AoA | Convex UEs | 0.08 | 0.12 | 0.18 | 0.27 |
| Case 26, InF-DH, FR1, Multi-RTT + vertical AoA | Convex UEs | 0.04 | 0.08 | 0.12 | 0.2 |
| Case 27, InF-SH, FR2, Multi-RTT + vertical AoA | Convex UEs | 0.01 | 0.03 | 0.05 | 0.08 |
| Case 28, InF-DH, FR2, Multi-RTT + vertical AoA | Convex UEs | 0.01 | 0.02 | 0.03 | 0.06 |
| Case 29, InF-SH, FR1, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.38 | 0.64 | 1.1 | 1.86 |
| Case 30, InF-DH, FR1, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.34 | 0.64 | 1.15 | 2.64 |
| Case 31, InF-SH, FR2, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.33 | 0.58 | 1.37 | 12.7 |
| Case 32, InF-DH, FR2, Multi-RTT + Tx/Rx timing error compensation | Convex UEs | 0.31 | 0.92 | 6.4 | 12.9 |
| Case 33, InF-SH, FR1, Multi-RTT | Convex UEs | 0.15 | 0.22 | 0.29 | 0.4 |
| Case 34, InF-DH, FR1, Multi-RTT | Convex UEs | 0.07 | 0.1 | 0.14 | 0.19 |
| Case 35, InF-SH, FR2, Multi-RTT | Convex UEs | 0.17 | 0.25 | 0.31 | 0.39 |
| Case 36, InF-DH, FR2, Multi-RTT | Convex UEs | 0.11 | 0.15 | 0.2 | 0.25 |

| Analysis for FR1 |  |
| --- | --- |
| Analysis for FR2 |  |

Figure C.2.1.7.2-1. Accuracy for Rel 17 positioning solutions with LOS/NLOS classification

| Analysis for FR1 and FR2 |  |
| --- | --- |

Figure C.2.1.7.2-2. Accuracy for Rel 17 positioning solutions based on Multi-RTT and vertical AoA measurements with LOS/NLOS classification

| Analysis for FR1 and FR2 |  |
| --- | --- |

Figure C.2.1.7.2-3. Accuracy for Rel 17 positioning solutions for Multi-RTT with Tx/Rx timing errors

| Analysis for FR1 |  |
| --- | --- |
| Analysis for FR2 |  |

Figure C.2.1.7.2-4. Accuracy for Rel 17 positioning solutions with aggregated bandwidth

#### C.2.1.8	Results from source [18]

##### C.2.1.8.1	Description of evaluation scenarios

The evaluation on the Rel-17 UL-TDOA performance in InF-DH scenario focuses on the following test cases:

- Impact of gNB antenna configuration (case 4, case 5 and case 6)

- Impact UE Tx Power and UE antenna configuration (case 8, case 9 , case 10 and case 11)

- Uplink beam management (case 7, case 13 and case 14)

- Ideal LOS detection is used for the position calculation for all test cases.

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.8.1-1.

Table C.2.1.8.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | Case 4- config 220 (InF-DH, FR1,UL-TDOA) | Case 5- config 320 (InF-DH, FR1,UL-TDOA) | Case 6- config 420 (InF-DH, FR1,UL-TDOA) | Case 7- config 421 (InF-DH, FR1,UL-TDOA) |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) |
| Carrier frequency | 4GHz | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port |
| Number of sites | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | +23dBm | +23dBm | +23dBm | +23dBm |
| interference modelling (ideal muting, or other) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied | Not applied |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps | RSRP used for Rx beam selection |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Per site (Rx): Omni antenna 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna | Per site (Rx): 3 sectors (3 panels, one element per panel) 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna | Per site (Rx): 3 panels, 5 dual polarized beams per panel 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna | Per site (Rx): 3 panels, 5 dual polarized beams per panel 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna |
| Evaluated Enhancement for Rel.17 | Ideal LOS detection | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment | Ideal LOS detection |
| Additional notes, if any | If the number of LOS links is sufficient, only LOS links are used for the position calculation (ideal LOS detection). If the number of LOS links is not sufficient all ToA-Values together with a signal power based signal quality are used by the positioning algorithm | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) |

| Parameter | Case 8- config 422 (InF-DH, FR1,UL-TDOA) | Case 9- config 423 (InF-DH, FR1,UL-TDOA) | Case 10- config 443 (InF-DH, FR1,UL-TDOA) | Case 11- config 444 (InF-DH, FR1,UL-TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) |
| Carrier frequency | 4GHz | 4GHz | 4GHz | 4GHz |
| Subcarrier spacing | 30kHz | 30kHz | 30kHz | 30kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port (4 resources) | SRS, 1 Port (4 resources) |
| Number of sites | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | 0dBm | -20dBm | -20dBm | -20dBm |
| interference modelling (ideal muting, or other) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied | Not applied |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Per site (Rx): 3 panels, 5 dual polarized beams per panel 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna | Per site (Rx): 3 panels, 5 dual polarized beams per panel 1 TXRU per panel per polarization dimension UE (Tx): Omni antenna | Per site (Rx): 3 panels, 5 dual polarized beams per panel 1 TXRU per panel per polarization dimension UE (Tx): 4 non-ideal antennas at different positions | Per site (Rx): 3 panels, 5 dual polarized beams per panel 1 TXRU per panel per polarization dimension UE (Tx): 4 ideal antennas at different positions |
| Evaluated Enhancement for Rel.17 | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment Gain of TX diversity with non-ideal UE antenna | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment Gain of TX diversity using 4 ideal UE antennas |
| Additional notes, if any | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) |

| Parameter | Case 12- config 447 (InF-DH, FR1,UL-TDOA) | Case 13- config 552 (InF-DH, FR2,UL-TDOA) | Case 14- config 554 (InF-DH, FR2,UL-TDOA |  |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) | InF-DH (r =0.4,hc=2m, dClutter=2m) |  |
| Carrier frequency | 4GHz | 28GHz | 28GHz |  |
| Subcarrier spacing | 30kHz | 30kHz | 120kHz |  |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 400Mhz |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 1 | KTC = 4 nbSymb = 4 |  |
| Reference signal (type of sequence, number of ports, …) | SRS, 1 Port | SRS, 1 Port | SRS, 1 Port (sequential transmissions) |  |
| Number of sites | 18 | 18 | 18 |  |
| Number of symbols used per occasion | 4 | 4 | 12*4 (12 different UE TX beams, 4 symbols per SRS resource) |  |
| number of occasions used per positioning estimate | 1 | 1 | 1 |  |
| Power-boosting level | 6dB | 6dB | 6dB |  |
| Uplink power control (applied/not applied) | 23dBm | 23dBm | 0dBm |  |
| interference modelling (ideal muting, or other) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) | Interference between UEs sharing the same REs (“cyclic shift multiplex”) |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function | IFP: First arrival path detection and evaluation of the first rising edge of the correlation function |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Levenberg Marquardt | Levenberg Marquardt | Levenberg Marquardt |  |
| Network synchronization assumptions | Ideal | Ideal | Ideal |  |
| UE/gNB Tx/Rx Calibration Error | Not applied | Not applied | Not applied |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Power of the first arriving taps | Power of the first arriving taps | Power of the first arriving taps |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Per site (Rx): 3 panels, 5 dual polarized beams per panel Virtualization model: 1 TXRU per panel per polarization dimension UE (Tx): 4 antenna panels (same characteristic as FR2 antenna), one element per panel | Per site (Rx): 3 panels, 5 dual polarized beams per panel Virtualization model: 1 TXRU per panel per polarization dimension UE (Tx): 4 antenna panels | Per site (Rx): 3 panels, 5 dual polarized beams per panel Virtualization model: 1 TXRU per panel per polarization dimension UE (Tx): 4 antenna panels |  |
| Evaluated Enhancement for Rel.17 | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment | Ideal LOS detection CIR analysis is used for the beam selection/spatial relationship establishment |  |
| Additional notes, if any | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) | Same note on the used LOS links as for case1 (config220) |  |

##### C.1.1.8.2	Positioning accuracy evaluation results

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.8.2-1:

Table C.2.1.8.2-1: NR positioning enhancements - horizontal location error results from [18]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4- config 220 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.15 | 0.23 | 0.36 | 0.70 |
| Case 5- config 320 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.11 | 0.18 | 0.27 | 0.45 |
| Case 6- config 420 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.09 | 0.15 | 0.23 | 0.43 |
| Case 7- config 421 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.16 | 0.54 | 2.68 | >3.00 |
| Case 8- config 422 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.09 | 0.15 | 0.23 | 0.44 |
| Case 9- config 423 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.10 | 0.16 | 0.27 | 0.59 |
| Case 10- config 443 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.07 | 0.13 | 0.22 | 0.50 |
| Case 11- config 444 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.07 | 0.12 | 0.21 | 0.49 |
| Case 12- config 447 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.08 | 0.14 | 0.28 | 0.95 |
| Case 13- config 552 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.12 | 0.21 | 0.43 | 1.70 |
| Case 14- config 554 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.02 | 0.03 | 0.05 | 0.09 |

Evaluation results of vertical location error for NR positioning enhancements are provided in Table C.2.1.8.2-2:

Table C.2.1.8.2-2: NR positioning enhancements - altitude location error results from [18]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4- config 220 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.33 | 0.55 | 0.95 | 1.84 |
| Case 5- config 320 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.24 | 0.41 | 0.68 | 1.19 |
| Case 6- config 420 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.18 | 0.33 | 0.57 | 1.12 |
| Case 7- config 421 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.39 | 1.29 | 6.33 | 10.0 |
| Case 8- config 422 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.21 | 0.35 | 0.61 | 1.15 |
| Case 9- config 423 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.21 | 0.36 | 0.65 | 1.56 |
| Case 10- config 443 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.15 | 0.27 | 0.48 | 1.12 |
| Case 11- config 444 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.15 | 0.26 | 0.49 | 1.43 |
| Case 12- config 447 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.17 | 0.35 | 0.75 | 2.23 |
| Case 13- config 552 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.26 | 0.54 | 1.14 | 4.28 |
| Case 14- config 554 (InF-DH, FR1,UL-TDOA) | Convex UEs | 0.04 | 0.07 | 0.12 | 0.24 |

| FR1 Results |  |
| --- | --- |
| Figure 3-CDF 2D (horizontal) error for UTDOA, comparison of different gNB antenna configurations | Figure 4 - CDF horizontal error , Tx Antenna diverstity |

| FR1 Results |  |
| --- | --- |
| Figure 3 - CDF horizontal error for UTDOA, comparison of different UE TX Power levels | Figure 6 - CDF horizontal error for UTDOA, Beam pair selection |

FR2 Results

Figure 12-CDF horizontal error UTDOA

Figure 14 - CDF 2D error for UTDOA, FR2 (400MHz BW)

#### C.2.1.9 Results from source [14]

##### C.2.1.9.1	Description of evaluation scenarios

In this section results are provided for the following scenarios.

- Baseline Scenarios for horizontal positioning evaluation: InF-SH, InF-DH, InH-OO, FR2/FR1, Convex UEs/ All UEs, Fixed UE height = 1.5 m,

- Baseline Scenarios for vertical positioning evaluation: InF-SH, InF-DH, FR2/FR1, Various UE height: UEH ∈ [0.5, 2] m/ UEH ∈ [0, 8] m, Fixed gNB height (8 m)/ Various gNB height ({4,8}),

- InF-SH FR2 using proposed various gNB height for vertical positioning,

- InF-SH FR2 using different size of bandwidth: BW = 400MHz/ 100MHz/ 50MHz/ 20MHz.

Evaluation assumptions for system level analysis are provided in Table C.2.1.9.1-1 and Table C.2.1.9.1-2.

Table C.2.1.9.1-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case7, InF- SH, FR2], [Case9, InF-DH, FR2], [Case11, InH-OO, FR2] | [Case8, InF- SH, FR1], [Case10, InF-DH, FR1], [Case12, InH-OO, FR1] | Case13, InF- SH, FR2], [Case15, InF-DH, FR2], | [Case14, InF- SH, FR1], [Case16, InF-DH, FR1], | Case17, InF- SH, FR2], [Case19, InF-DH, FR2], | [Case18, InF- SH, FR1], [Case20, InF-DH, FR1], |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 28 GHz | 1.5 GHz | 28 GHz | 1.5 GHz | 28 GHz | 1.5 GHz |
| Subcarrier spacing | 120 kHz | 30 kHz | 120 kHz | 30 kHz | 120 kHz | 30 kHz |
| Reference Signal Transmission Bandwidth | 400 MHz | 100 MHz | 400 MHz | 100 MHz | 400 MHz | 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6, 6 symbols | Comb-6, 6 symbols | Comb-6, 6 symbols | Comb-6, 6 symbols | Comb-6, 6 symbols | Comb-6, 6 symbols |
| Reference signal (type of sequence, number of ports, …) | Pseudo-random sequence, 1 antenna port |  |  |  |  |  |
| Number of sites | 18 sites in InF, 12 sites in InH |  |  |  |  |  |
| Number of symbols used per occasion | 6 | 6 | 6 | 6 | 6 | 6 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 6 dB | 6 dB | 6 dB | 6 dB | 6 dB | 6 dB |
| Uplink power control (applied/not applied) | not applied |  |  |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Adaptive threshold-based first peak detection for TOA measurement |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Filtering the TOA measurements by FPAR using a predefined threshold, AoD measurement from the estimated angular spectrum, NLOS Detection algorithms, TDOA-AOD based maximum likelihood positioning algorithm. |  |  |  |  |  |
| Network synchronization assumptions | Perfect synchronization |  |  |  |  |  |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Beam sweeping at Tx, MUSIC based signal combining at Rx |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker Product Codebook (Kronecker product of two oversampled DFT codewords) |  |  |  |  |  |
| Evaluated Enhancement for Rel.17 | NLOS Detection algorithms (NLOS state estimator) |  |  |  |  |  |
| Additional notes, if any | Horizontal positioning only, Fixed gNB height, Fixed UE height |  | Horizontal and vertical positioning, Fixed gNB height or various gNB height, UE height ∈ [0.5, 2] m |  | Horizontal and vertical positioning, Fixed gNB height or various gNB height, UE height ∈ [0, 8] m |  |

Table C.2.1.9.1-2: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case21, InF- SH, FR2] | [Case21, InF- SH, FR2] | [Case22, InF- SH, FR2] | [Case23, InF- SH, FR2] |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline | Baseline | Baseline | Baseline |
| Carrier frequency | 28 GHz | 28 GHz | 28 GHz | 28 GHz |
| Subcarrier spacing | 120 kHz | 120 kHz | 120 kHz | 120 kHz |
| Reference Signal Transmission Bandwidth | 100 MHz | 100 MHz | 50 MHz | 20 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4, 4 symbols | Comb-4, 4 symbols | Comb-4, 4 symbols | Comb-4, 4 symbols |
| Reference signal (type of sequence, number of ports, …) | Pseudo-random sequence, 1 antenna port |  |  |  |
| Number of sites | 18 sites in InF, 12 sites in InH |  |  |  |
| Number of symbols used per occasion | 6 | 6 | 6 | 6 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 6 dB | 6 dB | 6 dB | 6 dB |
| Uplink power control (applied/not applied) | not applied |  |  |  |
| interference modelling (ideal muting, or other) | ideal muting |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Adaptive threshold-based first peak detection for TOA measurement |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Filtering the TOA measurements by FPAR using a predefined threshold, AoD measurement from the estimated angular spectrum, NLOS Detection algorithms, TDOA-AOD based maximum likelihood positioning algorithm. |  |  |  |
| Network synchronization assumptions | Perfect synchronization |  |  |  |
| UE/gNB Tx/Rx Calibration Error | 0 |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Beam sweeping at Tx, MUSIC based signal combining at Rx |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker Product Codebook (Kronecker product of two oversampled DFT codewords) |  |  |  |
| Evaluated Enhancement for Rel.17 | NLOS Detection algorithms (NLOS state estimator) |  |  |  |
| Additional notes, if any | Horizontal positioning only, Fixed gNB height, Fixed UE height |  |  |  |

##### C.2.1.9.2 Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.9.2-1:

Table C.2.1.9.2-1: NR positioning enhancements - horizontal location error results

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 7], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.025 | 0.0344 | 0.046 | 0.074 |
|  | (Optional) All UEs | 0.033 | 0.048 | 0.075 | 0.23 |
| [Case 8], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.10 | 0.14 | 0.20 | 0.38 |
|  | (Optional) All UEs | 0.13 | 0.19 | 0.30 | 0.67 |
| [Case 9], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.044 | 0.075 | 0.53 | 1.96 |
|  | (Optional) All UEs | 0.063 | 0.31 | 1.31 | 2.12 |
| [Case 10], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.20 | 0.46 | 1.49 | 2.13 |
|  | (Optional) All UEs | 0.29 | 0.75 | 1.60 | 3.62 |
| [Case 11], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.041 | 0.062 | 0.11 | 0.31 |
|  | (Optional) All UEs | 0.054 | 0.095 | 0.18 | 0.39 |
| [Case 12], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.20 | 0.29 | 0.42 | 0.69 |
|  | (Optional) All UEs | 0.20 | 0.33 | 0.53 | 0.98 |

Evaluation results of vertical location error for NR positioning enhancements are provided in Table C.2.1.9.2-2:

Table C.2.1.9.2-2: NR positioning enhancements - Vertical location error results

| Case | gNB Deployment | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 13], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB], [ UEH ∈ [0.5, 2] m] | gNBH ∈ {4,8} m | 0.17 | 0.29 | 0.48 | 0.77 |
|  | gNBH = 8 m | 0.17 | 0.31 | 0.48 | 0.83 |
| [Case 14], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | gNBH ∈ {4,8} m, | 0.78 | 1.06 | 1.29 | 1.51 |
|  | gNBH = 8 m, | 0.86 | 1.12 | 1.38 | 1.59 |
| [Case 15], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | gNBH ∈ {4,8} m, | 0.69 | 0.91 | 1.10 | 1.34 |
|  | gNBH = 8 m, | 0.44 | 0.69 | 1.01 | 1.42 |
| [Case 16], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | gNBH ∈ {4,8} m, | 0.77 | 1.01 | 1.23 | 1.47 |
|  | gNBH = 8 m, | 0.84 | 1.08 | 1.34 | 1.58 |
| [Case 17], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | gNBH ∈ {4,8} m | 0.84 | 1.01 | 1.18 | 1.34 |
|  | gNBH = 8 m | 0.75 | 1.12 | 1.51 | 1.88 |
|  | gNBH ∈ {2, 4, 6, 8} m | 0.45 | 0.58 | 0.72 | 0.93 |
|  | gNBH = {1, 2, 3, 4, 5, 6, 7, 8} m, | 0.35 | 0.48 | 0.65 | 0.86 |
|  | gNBH = {1: 0.5: 8} m | 0.26 | 0.39 | 0.57 | 0.79 |
| [Case 18], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | gNBH ∈ {4,8}m | 0.96 | 1.25 | 1.50 | 1.73 |
|  | gNBH = 8 m | 0.98 | 1.38 | 1.78 | 2.16 |
| [Case 19], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | gNBH ∈ {4,8} m | 0.85 | 1.22 | 1.62 | 2.11 |
|  | gNBH = 8 m | 1.56 | 2.02 | 2.53 | 3.04 |
| [Case 20], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | gNBH ∈ {4,8}m | 0.85 | 1.25 | 1.77 | 2.57 |
|  | gNBH = 8 m | 1.31 | 1.74 | 2.18 | 2.69 |

Evaluation results of horizontal location error for NR positioning enhancements with various bandwidths are provided in Table C.2.1.9.2-3:

Table C.2.1.9.2-3: NR positioning enhancements - horizontal location error results in InF-SH FR2 scenario using different bandwidth

| Case | Bandwidth | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 21], [InF-SH], [FR2], [400 MHz], [Comb-4], [6dB PB] | 400 MHz | 0.025 | 0.034 | 0.046 | 0.086 |
| [Case 22], [InF-SH], [FR2], [100 MHz], [Comb-4], [6dB PB] | 100 MHz | 0.14 | 0.19 | 0.25 | 0.36 |
| [Case 23], [InF-SH], [FR2], [50 MHz], [Comb-4], [6dB PB] | 50 MHz | 0.20 | 0.27 | 0.38 | 0.77 |
| [Case 24], [InF-SH], [FR2], [20 MHz], [Comb-4], [6dB PB] | 20 MHz | 0.39 | 0.60 | 0.90 | 1.45 |

Simulation results given by the CDF of the positioning error can be found from figure C.2.1.9.2-1 to C.2.1.9.2-6.

Figure C.2.1.9.2-1: Rel-17 horizontal positioning evaluation in InF-SH and InF-DH scenario, referring to Case 7 to Case 10.

Figure C.2.1.9.2-2: Rel-17 horizontal positioning evaluation in InH-OO scenario, referring to Case 11 to Case 12.

Figure C.2.1.9.2-3: Rel-17 vertical positioning evaluation in InF-SH and InF-DH scenarios, UE height within a range [0.5, 2] m, referring to Case 13 to Case 16.

Figure C.2.1.9.2-4: Rel-17 vertical positioning evaluation in InF-SH and InF-DH scenarios, UE height within a range [0, 8] m, referring to Case 17 to Case 20.

Figure C.2.1.9.2-5: Rel-17 vertical positioning evaluation in InF-SH FR2 scenario, UE height within a range [0, 8] m, referring to Case 17.

Figure C.2.1.9.2-6: Rel-17 horizontal positioning evaluation in InF-SH FR2 scenario, referring to Case 21 to Case 24.

#### C.2.1.10	Results from source [21]

##### C.2.1.10.1	Description of evaluation scenarios

The following enhancements were evaluated for rel17:

- Line of Sight (LOS) detection

- Rx-Tx timing error mitigation for multi-panel UEs

- 1-symbol PRS

- Threshold based Channel peaks detection

- Cyclic shift enhancements for SRS

- PRS aggregation

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.10.1-1a-k.

Table C.2.1.10.1-1a: NR positioning enhancements - evaluation scenarios and parameters for line of sight detection

| Parameter | [21], source E1, Inf DH, FR1, DL TDOA | [[21], source E2, Inf DH, FR1, DL TDOA | [[21], source E3, Inf DH, FR1, DL TDOA |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Inf DH, 120x60m | Inf DH, 120x60m | Inf DH, 120x60m |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS staggered comb 12 | PRS staggered comb 12 | PRS staggered comb 12 |
| Reference signal (type of sequence, number of ports, …) | DL PRS | DL PRS | DL PRS |
| Number of sites | 18 | 18 | 18 |
| Number of symbols used per occasion | 12 | 12 | 12 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 10.8dB | 10.8dB | 10.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | ideal | ideal | ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Genie peak detection | Strongest peak LOS detection | No LOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | ideal | ideal | Idea |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | LOS detection | LOS detection | LOS detection |
| Additional notes, if any |  |  |  |

Table C.2.1.10.1-1c: NR positioning enhancements - evaluation scenarios and parameters for timing error evaluations baseline Inf SH

| Parameter | [21], source 10, InF SH FR2, UL TDOA | [21], source 11 InF SH FR2, UL TDOA | [21], source 12 InF SH FR2, UL TDOA | [21], source 13 InF SH FR2, UL TDOA | [21], source 14 InF SH FR2, UL TDOA | [21], source 15 InF SH FR2, DL TDOA | [21], source 16 InF SH FR2, DL TDOA | [21], source 17 InF SH FR2, DL TDOA | [21], source 18 InF SH FR2, DL TDOA | [21], source 19 InF SH FR2, DL TDOA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline | baseline | baseline | baseline | baseline | baseline | baseline | baseline |
| Carrier frequency | 28Ghz | 28Ghz | 28Ghz | 28Ghz | 28Ghz | 28Ghz | 28Ghz | 28Ghz | 28Ghz | 28Ghz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 |
| Reference signal (type of sequence, number of ports, …) | SRS pos | SRS pos | SRS pos | SRS pos | SRS pos | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS |
| Number of sites | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 3dB | 3dB | 3dB | 3dB | 3dB | 3dB | 3dB | 3dB | 3dB | 3dB |
| Uplink power control (applied/not applied) | none | none | none | none | none | X | X | X | X | X |
| interference modelling (ideal muting, or other) | ideal | ideal | ideal | ideal | ideal | ideal | ideal | ideal | ideal | ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | UL TDOA | UL TDOA | UL TDOA | UL TDOA | UL TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL TDOA | UL TDOA | UL TDOA | UL TDOA | UL TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | ideal | ideal | Idea | ideal | ideal | ideal | ideal | Idea | ideal | ideal |
| UE/gNB Tx/Rx Calibration Error | 0ns | 1ns | 2ns | 4ns | 8ns | 0ns | 1ns | 2ns | 4ns | 8ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Additional notes, if any |  |  |  |  |  |  |  |  |  |  |

Table C.2.1.10.1-1d: NR positioning enhancements - evaluation scenarios and parameters for timing error evaluations Inf SH with enhancements

| Parameter | [21], source 20, InF SH FR2, UL TDOA | [21], source 21 InF SH FR2, UL TDOA | [21], source 22 InF SH FR2, DL TDOA | [21], source 23 InF SH FR2, DL TDOA |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline | baseline |
| Carrier frequency | 28Ghz | 28Ghz | 28Ghz | 28Ghz |
| Subcarrier spacing |  |  |  |  |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS | PRS staggered comb 2 | PRS staggered comb 2 |
| Reference signal (type of sequence, number of ports, …) | SRS pos | SRS pos | DL PRS | DL PRS |
| Number of sites | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 2 | 2 | 2 | 2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 3dB | 3dB | 3dB | 3dB |
| Uplink power control (applied/not applied) | none | none | none | none |
| interference modelling (ideal muting, or other) | ideal | ideal | ideal | ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | UL TDOA | UL TDOA | DL-TDOA | DL-TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL TDOA | UL TDOA | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | ideal | ideal | ideal | ideal |
| UE/gNB Tx/Rx Calibration Error | 0ns | 8ns | 0ns | 8ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | Multi panel Tx | Multi panel Tx | Multi panel Tx | Multi panel Tx |
| Additional notes, if any |  |  |  |  |

Table C.2.1.10.1-1e: NR positioning enhancements - evaluation scenarios and parameters for timing error evaluations baseline Inf DH

| Parameter | [21], source 24, InF DH FR2, UL TDOA | [21], source 25 InF DH FR2, UL TDOA | [21], source 26 InF DH | [21], source 27 InF DH FR2, UL TDOA | [21], source 28 InF DH FR2, UL TDOA |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline | baseline | baseline |
| Carrier frequency | 28Ghz | 28Ghz | 28Ghz | 28Ghz | 28Ghz |
| Subcarrier spacing |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS | 2 symbols comb 2 SRS |
| Reference signal (type of sequence, number of ports, …) | SRS pos | SRS pos | SRS pos | SRS pos | SRS pos |
| Number of sites | 18 | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 2 | 2 | 2 | 2 | 2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 3dB | 3dB | 3dB | 3dB | 3dB |
| Uplink power control (applied/not applied) | none | none | none | none | none |
| interference modelling (ideal muting, or other) | ideal | ideal | ideal | ideal | ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | UL TDOA | UL TDOA | UL TDOA | UL TDOA | UL TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL TDOA | UL TDOA | UL TDOA | UL TDOA | UL TDOA |
| Network synchronization assumptions | ideal | ideal | Idea | ideal | ideal |
| UE/gNB Tx/Rx Calibration Error | 0ns | 1ns | 2ns | 4ns | 8ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | N/A | N/A | N/A | N/A | N/A |
| Additional notes, if any |  |  |  |  |  |

Table C.2.1.10.1-1f: NR positioning enhancements - evaluation scenarios and parameters for 1-symbol PRS

| Parameter | [21], source E29, Inf SH, FR1, DL TDOA | [21], source E30, Inf SH, FR1, DL TDOA | [21], source E31, Inf SH, FR1, DL TDOA | [21], source E32, IOO, FR1, DL TDOA | [21], source E33, IOO, FR1, DL TDOA | [21], source E34, IOO, FR1, DL TDOA |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Inf SH, 300x150m | Inf SH, 300x150m | Inf SH, 300x150m | baseline | baseline | baseline |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS staggered comb 12 reuse 1 | PRS staggered comb 12 | PRS staggered comb 12 | PRS staggered comb 12 reuse 1 | PRS staggered comb 12 | PRS staggered comb 12 |
| Reference signal (type of sequence, number of ports, …) | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS |
| Number of sites | 18 | 18 | 18 | 12 | 12 | 12 |
| Number of symbols used per occasion | 1 | 12 | 12 | 1 | 12 | 12 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 10.8dB | 10.8dB | 10.8dB | 10.8dB | 10.8dB | 10.8dB |
| Uplink power control (applied/not applied) | none | none | none | none | none | none |
| interference modelling (ideal muting, or other) | interference | interference | No interference | interference | interference | No interference |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.10.1-1g: NR positioning enhancements - evaluation scenarios and parameters for 1-symbol PRS

| Parameter | [21], source E35, Inf SH, FR1, DL TDOA< | [21], source E36, Inf SH, FR1, DL TDOA | [21], source E37, Inf SH, FR1, DL TDOA | [21], source E38, Inf SH, FR1, DL TDOA< | [21], source E39, IOO, FR1, DL TDOA | [21], source E40, IOO, FR1, DL TDOA |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Inf SH, 300x150m | Inf SH, 300x150m | Inf SH, 300x150m | baseline | baseline | baseline |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS staggered comb 6 reuse 3 | PRS staggered comb 6 | PRS staggered comb 6 | PRS staggered comb 6 reuse 2 | PRS staggered comb 6 | PRS staggered comb 6 |
| Reference signal (type of sequence, number of ports, …) | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS |
| Number of sites | 18 | 18 | 18 | 12 | 12 | 12 |
| Number of symbols used per occasion | 1 | 6 | 6 | 1 | 6 | 6 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 7.7dB | 7.7dB | 7.7dB | 7.7dB | 7.7dB | 7.7dB |
| Uplink power control (applied/not applied) | none | none | none | none | none | none |
| interference modelling (ideal muting, or other) | interference | interference | No interference | interference | interference | No interference |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.10.1-1h: NR positioning enhancements - evaluation scenarios and parameters for 1-symbol PRS

| Parameter | [21], source E41, Inf SH, FR1, DL TDOA< | [21], source E42, Inf SH, FR1, DL TDOA | [21], source E43, Inf SH, FR1, DL TDOA | [21], source E44, Inf SH, FR1, DL TDOA< | [21], source E45, IOO, FR1, DL TDOA | [21], source E46, IOO, FR1, DL TDOA |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Inf SH, 300x150m | Inf SH, 300x150m | Inf SH, 300x150m | baseline | baseline | baseline |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing |  |  |  |  |  |  |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS staggered comb 2 reuse 9 | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 reuse 6 | PRS staggered comb 2 | PRS staggered comb 2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS |
| Number of sites | 18 | 18 | 18 | 12 | 12 | 12 |
| Number of symbols used per occasion | 1 | 2 | 2 | 1 | 2 | 2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 3dB | 3dB | 3dB | 3dB | 3dB | 3dB |
| Uplink power control (applied/not applied) | none | none | none | none | none | none |
| interference modelling (ideal muting, or other) | interference | interference | No interference | interference | interference | No interference |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS | 1 symbol PRS |
| Additional notes, if any |  |  |  |  |  |  |

Table C.2.1.10.1-1i: NR positioning enhancements - evaluation scenarios and parameters for 1-symbol PRS with thresholding

| Parameter | [21], source E47, Inf DH, FR1, DL TDOA | [21], source E48, Inf DH, FR1, DL TDOA | [21], source E49, Inf DH, FR1, DL TDOA | [21], source E50, Inf DH, FR1, DL TDOA | [21], source E51, Inf DH, FR1, DL TDOA | [21], source E52, Inf DH, FR1, DL TDOA | [21], source E53, Inf DH, FR1, DL TDOA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Inf DH, | Inf DH, | Inf DH, | Inf DH, | Inf DH, | Inf DH, | Inf DH, |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS staggered comb 12 | PRS staggered comb 12 | PRS staggered comb 12 | PRS staggered comb 12 | PRS staggered comb 12 | PRS staggered comb 12 | PRS staggered comb 12 |
| Reference signal (type of sequence, number of ports, …) | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS |
| Number of sites | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 10.8dB | 10.8dB | 10.8dB | 10.8dB | 10.8dB | 10.8dB | 10.8dB |
| Uplink power control (applied/not applied) | none | none | none | none | none | none | none |
| interference modelling (ideal muting, or other) | ideal | ideal | ideal | ideal | ideal | ideal | ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | 1 symbol PRS with peak threasholding | 1 symbol PRS with peak threasholding | 1 symbol PRS with peak threasholding | 1 symbol PRS with peak threasholding | 1 symbol PRS with peak threasholding | 1 symbol PRS with peak threasholding | 1 symbol PRS with peak threasholding |
| Additional notes, if any | No thresholding | Relative threshold 30dB | Relative threshold 20dB | Relative threshold 10dB | Relative threshold 7dB | Relative threshold 5dB | Relative threshold 4dB |

Table C.2.1.10.1-1j: NR positioning enhancements - evaluation scenarios and parameters for SRS cyclic shifts

| Parameter | [21], source E54, Inf SH, FR1, DL TDOA< | [21], source E55, IOO, FR1, DL TDOA | [21], source E56, IOO, FR1, DL TDOA |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline |
| Carrier frequency | 2GHz | 2GHz | 2GHz |
| Subcarrier spacing |  |  |  |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | UL SRS comb 2 | UL SRS comb 2 | UL SRS comb 2 |
| Reference signal (type of sequence, number of ports, …) | UL SRS | UL SRS | UL SRS |
| Number of sites | 12 | 12 | 12 |
| Number of symbols used per occasion | 2 | 2 | 2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 3dB | 3dB | 3dB |
| Uplink power control (applied/not applied) | none | none | none |
| interference modelling (ideal muting, or other) | interference | interference | No interference |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | UL TDOA | UL TDOA | UL TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | UL TDOA | UL TDOA | UL TDOA |
| Network synchronization assumptions | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | none | Cyclic shifts enh., PAPR conservation | Cyclic shifts enh. Cross corr improvement |
| Additional notes, if any |  |  |  |

Table C.2.1.10.1-1k: NR positioning enhancements - evaluation scenarios and parameters for PRS aggregation

| Parameter | [21], source E57, Inf DH, FR1, DL TDOA | [21], source E58, Inf DH, FR1, DL TDOA | [21], source E59, Inf DH, FR1, DL TDOA | [21], source E60, Inf SH, FR1, DL TDOA | [21], source E61, Inf SH, FR1, DL TDOA | [21], source E62, Inf SH, FR1, DL TDOA | [21], source E63, Inf SH, FR1, DL TDOA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Inf DH, | Inf DH, | Inf DH, | Inf SH, | Inf SH, | Inf SH, | Inf SH, |
| Carrier frequency | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 | PRS staggered comb 2 |
| Reference signal (type of sequence, number of ports, …) | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS | DL PRS |
| Number of sites | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| Number of symbols used per occasion | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level | 3dB | 3dB | 3dB | 3dB | 3dB | 3dB | 3dB |
| Uplink power control (applied/not applied) | none | none | none | none | none | none | none |
| interference modelling (ideal muting, or other) | ideal | ideal | ideal | ideal | ideal | ideal | ideal |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA | DL TDOA |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA | DL-TDOA |
| Network synchronization assumptions | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal | Ideal |
| UE/gNB Tx/Rx Calibration Error | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Evaluated Enhancement for Rel.17 | PRS aggregation, 1CC | PRS Agg, 2CC, no interf | PRS agg, 2CC, interf | PRS aggregation, 1CC | PRS Agg, 2CC, no interf | PRS agg, 2CC, interf | PRS agg, 2CC, interf |
| Additional notes, if any |  |  | Max phase error =pi |  |  | Max phase error =pi | Max phase error =0.2pi |

##### C.2.1.10.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.10.2-1:

Table C.2.1.10.2-1a: NR positioning enhancements - horizontal location error results from [21] for line of sight detection

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [21], source E1, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.07m | 0.11m | 0.16m | 0.26m |
| [21], source E2, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.09m | 0.18m | 1.9m | 8.4m |
| [21], source E3, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 3.3m | 5.4m | 8.2m | 18m |

Figure C.2.1.10.2-1a:  Comparison of DL TDOA performance for the InF-DH scenario with genie LOS detection [[21], source E1, Inf DH, FR1, DL TDOA], with LOS detection based on whether the first peak is also the strongest peak [[21], source E2, Inf DH, FR1, DL TDOA], and without LOS detection[[21], source E2, Inf DH, FR1, DL TDOA]. The figure to the right is zoomed in at small positioning errors.

Table C.2.1.10.2-1c: NR positioning enhancements - horizontal location error results from [21] for timing error compensation

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [21], source E10, InF SH FR2, UL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.007454 | 0.011409 | 0.017341 | 0.031218 |
| [21], source E11 InF SH FR2, UL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.153485 | 0.245942 | 0.348854 | 0.500084 |
| [21], source E12 InF SH FR2, UL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.300045 | 0.48218 | 0.7123 | 0.989034 |
| [21], source E13 InF SH FR2, UL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.557293 | 0.950828 | 1.400275 | 1.924776 |
| [21], source E14 InF SH FR2, UL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 1.162661 | 1.947078 | 2.761667 | 3.848105 |
| [21], source E15 InF SH FR2, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.007943 | 0.012145 | 0.019341 | 0.033648 |
| [21], source E16 InF SH FR2, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.140761 | 0.239327 | 0.346505 | 0.501348 |
| [21], source E17 InF SH FR2, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.262914 | 0.461862 | 0.67501 | 0.989885 |
| [21], source E18 InF SH FR2, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.547018 | 0.951728 | 1.378088 | 1.98219 |
| [21], source E19 InF SH FR2, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 1.054968 | 1.852011 | 2.700799 | 3.824875 |
| [21], source E20, InF SH FR2, UL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.008428 | 0.012536 | 0.019082 | 0.034039 |
| [21], source E21 InF SH FR2, UL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.00793 | 0.01199 | 0.016988 | 0.02989 |
| [21], source E22 InF SH FR2, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.008819 | 0.013365 | 0.019739 | 0.037037 |
| [21], source E23 InF SH FR2, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.008404 | 0.012772 | 0.018612 | 0.03662 |

Figure C.2.1.10.2-1b:  UL TDOA positioning accuracy for dual UE antenna panels with a random TX timing error with a normal distribution with std  = 0 ([21], source E10, InF SH FR2, UL TDOA), 1 ([21], source E11, InF SH FR2, UL TDOA), 2 ([21], source E12, InF SH FR2, UL TDOA), 4 ([21], source E13, InF SH FR2, UL TDOA) and  8ns  ([21], source E14, InF SH FR2, UL TDOA) truncated at 2 in the InF-SH scenario. A 2-symbol comb-2 UL SRS was used at 28GHz carrier, 120kHz subcarrier spacing and 400MHz bandwidth. Positioning accuracy is severely reduced for large TX timing errors.

Figure C.2.1.10.2-1c:  DL TDOA positioning accuracy for dual UE antenna panels with a random RX timing error with a normal distribution with std s = 0 ([21], source E15 InF SH FR2, DL TDOA), 1([21], source E16 InF SH FR2, DL TDOA), 2 ([21], source E17 InF SH FR2, DL TDOA), 4 ([21], source E18 InF SH FR2, DL TDOA) and  8ns ([21], source E19 InF SH FR2, DL TDOA) truncated at 2s in the InF-SH scenario. A 2-symbol comb-2 DL PRS was used at 28GHz carrier, 120kHz subcarrier spacing and 400MHz bandwidth. Positioning accuracy is severely reduced for large RX timing errors.

Figure C.2.1.10.2-1d:  UL TDOA positioning accuracy for dual UE antenna panels with a random TX timing error with a normal distribution with std  = 0ns and  8ns truncated at 2 in the InF-SH scenario. A 2-symbol comb-2 UL SRS was used at 28GHz carrier, 120kHz subcarrier spacing and 400MHz bandwidth. Results are given both with  ([21], source E22, InF SH FR2, UL TDOA for 0ns, [21], source E23, InF SH FR2, UL TDOA) and without with  ([21], source E20, InF SH FR2, UL TDOA for 0ns, [21], source E22, InF SH FR2, UL TDOA) multi panel transmission (with multi panel transmission is denoted as ‘multi-tx’ in the figure). Multipanel transmission allows joint estimation of the UE position and of the TX timing errors per UE antenna panel and is seen to remove the problem of TX timing errors in this scenario.

Figure 9 DL TDOA positioning accuracy for dual UE antenna with a random TX timing error with a normal distribution with std  = 0, 1, 2, 4 and  8ns truncated at 2 in the InF-dH scenario. A 2-symbol comb-2 DL PRS was used at 28GHz carrier, 120kHz subcarrierspacing and 400MHz bandwidth. In this scenario positioning accuracy is limited by NLOS excess delays and the impact of TX timing errors is therefore not visible. However, if solutions such as LOS-detection are found to handle NLOS excess delays, TX timing errors would become important also in the INF-DH scenario.

Table C.2.1.10.2-1d: NR positioning enhancements - horizontal location error results from [21] for 1 symbol PRS

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [21], source E29, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.134169 | 0.209873 | 0.332673 | 0.615954 |
| [21], source E30, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.143561 | 0.215242 | 0.326055 | 0.54665 |
| [21], source E31, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.157217 | 0.232257 | 0.358399 | 0.681864 |
| [21], source E32, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.365266 | 0.585002 | 0.883262 | 1.384617 |
| [21], source E33, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.369137 | 0.591664 | 0.901767 | 1.404678 |
| [21], source E34, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.380713 | 0.617761 | 0.924331 | 1.447296 |
| [21], source E35, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.133153 | 0.207466 | 0.338269 | 0.629094 |
| [21], source E36, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.152067 | 0.227031 | 0.365366 | 0.719702 |
| [21], source E37, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.146765 | 0.227981 | 0.345524 | 0.642431 |
| [21], source E38, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.369406 | 0.595909 | 0.890289 | 1.429679 |
| [21], source E39, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.370563 | 0.579921 | 0.89593 | 1.512598 |
| [21], source E40, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.36838 | 0.60413 | 0.89382 | 1.420364 |
| [21], source E41, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.132011 | 0.209833 | 0.33191 | 0.614715 |
| [21], source E42, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.209148 | 0.319092 | 0.527592 | 0.912933 |
| [21], source E43, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.134027 | 0.206734 | 0.336959 | 0.616703 |
| [21], source E44, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.367821 | 0.598939 | 0.898759 | 1.445729 |
| [21], source E45, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.38739 | 0.619138 | 1.032782 | 1.732023 |
| [21], source E46, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.374937 | 0.594308 | 0.914879 | 1.425011 |
| [21], source E47, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 3.175204 | 5.128706 | 7.670416 | 15.736090 |
| [21], source E48, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 3.163860 | 5.148267 | 7.667818 | 15.788375 |
| [21], source E49, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 3.170297 | 5.214206 | 7.922149 | 16.022874 |
| [21], source E50, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 3.558847 | 5.525303 | 8.616365 | 16.984019 |
| [21], source E51, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 3.813672 | 6.093375 | 9.361212 | 17.412632 |
| [21], source E52, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 4.178469 | 6.653492 | 10.148000 | 19.787138 |
| [21], source E53, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 4.531061 | 7.078220 | 10.973041 | 22.610367 |

Figure C.2.1.10.2-1e:  Evaluation results for 1-symbol DL PRS compared to Rel. 16 DL PRS ([21], source E29-46)

Figure C.2.1.10.2-1f:  ([21], source E47-53) The use of a threshold relative to the strongest peak in the UE search of the first peak give no reduction in positioning performance with a relative threshold 20dB below the strongest peak.

Table C.2.1.10.2-1e: NR positioning enhancements - horizontal location error results from [21] for SRS CS enhancements

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [21], source E54, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.3 | .076 | 1.02 | 2.52 |
| [21], source E55, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.3 | 0.76 | 1.24 | 3.04 |
| [21], source E56, IOO, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.8 | 1.96 | 8.57 | 16.02 |

Figure C.2.1.10.2-1g: ([21], source E54 -56) Performance of release 16 SRS for positioning baseline (yellow curve) and proposed enhancements to the cyclic shift implementation (blue curve) and to the ZC sequence length (red line)

Table C.2.1.10.2-1f: NR positioning enhancements - horizontal location error results from [21] for PRS aggregation

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [21], source E57, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 3.091467 | 5.167042 | 7.670441 | 15.00989 |
| [21], source E58, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 2.829843 | 4.715137 | 7.400995 | 13.91966 |
| [21], source E59, Inf DH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 3.349368 | 5.183635 | 7.605808 | 15.01508 |
| [21], source E60, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.057696 | 0.089176 | 0.136153 | 0.259396 |
| [21], source E61, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.019322 | 0.027955 | 0.042136 | 0.079559 |
| [21], source E62, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.097234 | 0.12627 | 0.174131 | 0.275798 |
| [21], source E63, Inf SH, FR1, DL TDOA | Convex UEs |  |  |  |  |
|  | (Optional) All UEs | 0.516435 | 0.701014 | 0.973997 | 1.447441 |

Figure C.2.1.10.2-1h: ([21], source E57-59) (Comparison of DL TDOA positioning accuracy for two perfectly coherent 100MGHz carriers with a single 100MHz carrier. In addition the accuracy is given for the case when the UE tries to combine two 100MGHz carriers coherently but the two carriers are in fact incoherent

Figure C.2.1.10.2-1i: ([21], source E60-63)  Comparison of DL TDOA positioning accuracy for a single 100MHz carrier with two 100MGHz carriers with a phase difference which is a homogenously random number in the interval (-max_phase_difference, + max_phase_difference) for different values of max_phase_difference. Thus max_phase_difference=pi corresponds to fully incoherent carriers while max_phase_difference=0 corresponds to fully coherent carriers.

#### C.2.1.11	Results from [17]

##### C.2.1.11.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.1.11-1.

Table C.2.1.1.11-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter |  | Case 22, InH, FR1, 100+100 MHz, No Frequency gap, Phase Offset, Link Quality, DL-TDOA | Case 23, InH, FR1, 100+100+100+100 MHz, No Frequency gap, Phase Offset, Link Quality, DL-TDOA | Case 24, InH, FR1, 100+100 MHz, Variable channel gap, RANSAC | Case 25, UMI, FR1, 100+100 MHz, No Frequency gap, Phase Offset, RANSAC | Case 26, UMI, FR1, 100+100 MHz, No Frequency gap, Time Offset, RANSAC |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz | 4 GHz | 4 GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) | 19(3-sector) | 19(3-sector) |
| Number of symbols used per occasion |  | 48 | 48 | 48 | 48 | 48 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates.. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |  |

| Parameter |  | Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100 MHz Gap | Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 3.5GHz | 3.5GHz | 3.5GHz | 3.5GHz |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS | DL-PRS |
| Number of sites |  | 18(1-sector) | 18(1-sector) | 18(1-sector) | 18(1-sector) |
| Number of symbols used per occasion |  | 18 | 18 | 18 | 18 |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 | 1 |
| Power-boosting level |  | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) |  | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates.. |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. | In the earliest beam pair is identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles. | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |
| Additional notes, if any |  | This case is simulated with phase shifts between the two aggregated CCs assumed to be 0, 45, 90 degrees | This case is simulated with phase shifts between the two aggregated CCs assumed to be 0, 45, 90 degrees | This case is simulated with phase shifts between the two aggregated CCs assumed to be 0, 45, 90 degrees | This case is simulated with phase shifts between the two aggregated CCs assumed to be 0, 45, 90 degrees |

| Parameter | Case 28, InF-SH, FR2, DL-TDOA | Case 29, InF-DH, FR2, DL-TDOA | Case 30, UMi, FR2, DL-TDOA | Case 31, InH, FR2, DL-TDOA | Case 32, UMi, FR2, DL-TDOA |  | ase 33, InH, FR2, DL-TDOA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |  |  |
| Carrier frequency | 28GHz | 28GHz | 30GHz | 30GHz | 30GHz |  | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz | 120KHz | 120KHz |  | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz | 400MHz | 400MHz |  | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 | Comb2 | Comb2 | Comb2 |  | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS | DL-PRS | DL-PRS | DL-PRS |  | DL-PRS |
| Number of sites | 18(3-sector) | 18(3-sector) | 19(3-sector) | 12(3-sector) | 19(3-sector) |  | 12(3-sector) |
| Number of symbols used per occasion | 54(2 FDMed PRS symbols with two TRPs ) | 54(2 FDMed PRS symbols with two TRPs ) | 58(2 FDMed PRS symbols with two TRPs ) | 36(2 FDMed PRS symbols with two TRPs) | 58(2 FDMed PRS symbols with two TRPs ) |  | 36(2 FDMed PRS symbols with two TRPs) |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 | 1 |  | 1 |
| Power-boosting level | 0 | 0 | 0 | 0 | 0 |  | 0 |
| Uplink power control (applied/not applied) | NA | NA | NA | NA | NA |  | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting |  | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. |  |  |  |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |  | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | NA |  |  |  |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |  |  |  |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |  |  |  |  |
| PRS Frequency Domain stitching | 2X200MHz Continuous PFLs | 2X200MHz Continuous PFLs | 2X200MHz Continuous PFLs | 2X200MHz Continuous PFLs | 2X200MHz Continuous PFLs |  | 2X200MHz Continuous PFLs |
| Phase Shift | Randomly select a phase shift value within -θ,θ and apply the phase shift on the second PFL per PRS, θ= [0,10, 20, 40,60,80] |  |  |  | NA |  | NA |
| Phase Ramp(Time offset) | NA | NA | NA | NA | Randomly select a time offset value within -t,t and convert the time offset as a phase ramp offset on second PFL per PRS, t= [0,0.1, 0.5, 1, 5, 10,20]ns |  |  |

| Kinematic constraint aware positioning – FR2 simulation assumptions and setup | Case 34, InF-SH, FR2, DL-TDOA | Case 35, InF-DH, FR2, DL-TDOA | Case 36, InH, FR2, DL-TDOA |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |
| Carrier frequency | 28GHz | 28GHz | 30GHz |
| Subcarrier spacing | 120KHz | 120KHz | 120KHz |
| Reference Signal Transmission Bandwidth | 400MHz | 400MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb2 | Comb2 | Comb2 |
| Reference signal (type of sequence, number of ports, …) | DL-PRS | DL-PRS | DL-PRS |
| Number of sites | 18(3-sector) | 18(3-sector) | 12(3-sector) |
| Number of symbols used per occasion | 54 (2 FDMed PRS symbols with two TRPs ) | 54 (2 FDMed PRS symbols with two TRPs ) | 36 (2 FDMed PRS symbols with two TRPs ) |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 0 | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | NA |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | For a 3-sector scenario, the directions of beams are within [-60, 60] degrees in azimuth and [60,180] degrees in zenith. In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |  |
| Kinematic Constraint awareness | In scenario 1, the node computing the position knows the value of one of the coordinates, and only computes the other one. This is denoted in the figure legends as ‘D unknown in estimation and RMS calculation’ (where D=X or Y, depending on which coordinate is unknown). In scenario 2, the node computing the position has no side information and computes both X and Y. However, the LCS client, i.e., the consumer of the computed (X,Y) position, knows the value of one of the coordinates, and thus ignores that one and uses only the other one. |  |  |

| Parameter |  | Case 37, UMI with Δτ, FR1, RTT+AoA | Case 38, UMI with Δτ, FR1, DL-TDOA | Case 39, UMI with Δτ, FR1, RTT |  |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |  |
| Carrier frequency |  | 4 GHz | 4 GHz | 4 GHz |  |
| Subcarrier spacing |  | 30KHz | 30KHz | 30KHz |  |
| Reference Signal Transmission Bandwidth |  | 100MHz | 100MHz | 100MHz |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 | Comb6 | Comb6 |  |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS | DL-PRS | DL-PRS |  |
| Number of sites |  | 19(3-sector) | 19(3-sector) | 19(3-sector) |  |
| Number of symbols used per occasion |  | 48 | 48 | 48 |  |
| number of occasions used per positioning estimate |  | 1 | 1 | 1 |  |
| Power-boosting level |  | 0 | 0 | 0 |  |
| Uplink power control (applied/not applied) |  | NO | NA | NO |  |
| interference modelling (ideal muting, or other) |  | Ideal Muting | Ideal muting | Ideal Muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). |  |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |  |
| Network synchronization assumptions |  | Perfect Sync | Perfect Sync | Perfect Sync |  |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error | No calibration error | No calibration error |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | NA | NA | NA |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single port wideband Hadamar precoder |  |  |  |  |
| Additional notes, if any |  |  |  |  |  |

| Parameter |  | Case 40, InF-DH, FR1, DL-TDOA |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |
| Carrier frequency |  | 3.5GHz |
| Subcarrier spacing |  | 30KHz |
| Reference Signal Transmission Bandwidth |  | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  | Comb6 |
| Reference signal (type of sequence, number of ports, …) |  | DL-PRS |
| Number of sites |  | 18(1-sector) |
| Number of symbols used per occasion |  | 18 |
| number of occasions used per positioning estimate |  | 1 |
| Power-boosting level |  | 0 |
| Uplink power control (applied/not applied) |  | NA |
| interference modelling (ideal muting, or other) |  | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Case 40-A: TOA estimation with TOA pruning, before the positioning engine using the ratio of the estimated TOA peak over the median of the Channel Energy Response (CER). Case 40-B: Matrix pencil algorithm for TOA Estimation |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) |  | TDOA: Chan's Algorithm according to: Y. T. Chan, K. C. Ho, " A Simple and Efficient Estimator for Hyperbolic Location", IEEE Transactions on Signal Processing, vol. 42, pp. 1905-1915, Aug. 1994. Outlier Rejection: RANSAC according to: https://en.wikipedia.org/wiki/Random_sample_consensus#:~:text=Random%20sample%20consensus%20(RANSAC)%20is,the%20values%20of%20the%20estimates. |
| Network synchronization assumptions |  | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error |  | No calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  | In the earliest beam pair, scheme, the Tx and Rx beam pair are identified based on the criteria of receiving the earliest path given that the received power is larger than a threshold using the genie CDL channel profiles per TRP. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker product between vertical and horizontal weight vectors taken from DFT, with oversampling factor r. The DFT beam candidate in beam selection method is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN where r denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. For FR2, Tx oversampling factor r equals to 1 and Rx oversampling factor equals to 1. |  |
| Additional notes, if any |  | All gNB’s are at height 8m and All UEs are at height 1.5m |

##### C.2.1.11.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.11.2-1:

Table C.2.1.11.2-1: NR positioning enhancements - horizontal location error results

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 22, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 2 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.3 | 0.8 | 1 | 1.5 |
| Case 22, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 1.1 | 1.4 | 2.5 |
| Case 22, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.5 | 1.2 | 1.4 | 2.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 23, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.12 | 0.3 | 0.5 | 0.9 |
| Case 23, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.15 | 0.35 | 0.9 | 2 |
| Case 23, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.2 | 0.4 | 1.1 | 2.4 |
| Case 23, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 1 | 1.2 | 2.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 24, InH, FR1, Channel Spacing = 0 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.35 | 0.8 | 1.5 | 3 |
| Case 24, InH, FR1, Channel Spacing = 7.2 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.4 | 0.9 | 1.7 | 4 |
| Case 24, InH, FR1, Channel Spacing = 14.4 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.5 | 1.8 | 2.3 | 5 |
| Case 24, InH, FR1, Channel Spacing = 36 MHz, Frequency-domain Stitching | (Optional) All UEs | 0.8 | 1.9 | 2.8 | 5.5 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 25, UMI, FR1,Phase offset = 0 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.6 m | 1 m |
| Case 25, UMI, FR1,Phase offset = 4 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.7 m | 1.2 m |
| Case 25, UMI, FR1,Phase offset = 8 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.4 m | 0.55 m | 0.7 m | 1.5 m |
| Case 25, UMI, FR1,Phase offset = 12 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.5 m | 0.7 m | 1.2 m | 2.2 m |
| Case 25, UMI, FR1,Phase offset = 18 degrees, Frequency-domain Stitching | (Optional) All UEs | 0.6 m | 1 m | 1.8 m | 4 m |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 26, UMI, FR1,Time offset = 0 ns, Frequency-domain Stitching | (Optional) All UEs | 0.37 m | 0.5 m | 0.6 m | 1 m |
| Case 26, UMI, FR1,Time offset = 0.5 ns, Frequency-domain Stitching | (Optional) All UEs | 0.38 m | 0.52 m | 0.7 m | 1 m |
| Case 26, UMI, FR1,Time offset = 1 ns, Frequency-domain Stitching | (Optional) All UEs | 0.38 m | 0.55 m | 0.7m | 1 m |
| Case 26, UMI, FR1,Time offset = 2 ns, Frequency-domain Stitching | (Optional) All UEs | 0.4 m | 0.58 m | 0.6 m | 1.5 m |
| Case 26, UMI, FR1,Time offset = 5 ns, Frequency-domain Stitching | (Optional) All UEs | 0.5 m | 0.8 m | 1 m | 1.5 m |

| Horizontal Positioning Error | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 0 | 0.117m | 4.9m | 13.2m | 24.4m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 45 | 0.108m | 5.3m | 13.3m | 25.6m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 90 | 0.126m | 6.09m | 13.8m | 25.5m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 0 | 0.067m | 4.47m | 14.26m | 25.7m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 45 | 0.09m | 3.79m | 13.1m | 25.46m |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 90 | 1.04m | 7.02m | 15.1m | 28.9m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 0 | 0.04m | 0.07m | 0.17m | 3.4m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 45 | 0.04m | 0.07m | 0.17m | 2.9m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap, Phase Offset = 90 | 0.04m | 0.08m | 0.22m | 3.4m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 0 | 0.02m | 0.06m | 0.14m | 4.25m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 45 | 0.03m | 0.06m | 0.21m | 4.26m |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap, Phase Offset = 90 | 0.03m | 0.06m | 0.83m | 4.43m |

| Case ID | Phase Shift θ | Time offset t | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| Case 28 InF-SH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.14 |
|  | 10 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.15 |
|  | 20 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.12 |
|  | 40 | 0.0ns | 0.01 | 0.02 | 0.08 | 1.37 |
|  | 60 | 0.0ns | 0.01 | 0.02 | 0.18 | 1.14 |
|  | 80 | 0.0ns | 0.01 | 0.02 | 0.07 | 0.96 |
| Case 29 InF-DH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 2.72 | 12.41 | 28.39 |
|  | 10 | 0.0ns | 0.01 | 2.82 | 12.43 | 29.53 |
|  | 20 | 0.0ns | 0.01 | 2.82 | 12.41 | 28.39 |
|  | 40 | 0.0ns | 0.01 | 2.88 | 12.42 | 28.33 |
|  | 60 | 0.0ns | 0.02 | 3.95 | 13.62 | 30.92 |
|  | 80 | 0.0ns | 0.02 | 3.65 | 13.60 | 29.54 |
| Case 30 UMi, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.05 |
|  | 10 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.05 |
|  | 20 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.04 |
|  | 40 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.25 |
|  | 60 | 0.0ns | 0.01 | 0.01 | 0.03 | 0.11 |
|  | 80 | 0.0ns | 0.01 | 0.01 | 0.02 | 0.07 |
| Case 31 InH, FR2, DL-TDOA, LQ-based | 0 | 0.0ns | 0.04 | 0.08 | 0.17 | 0.42 |
|  | 10 | 0.0ns | 0.04 | 0.09 | 0.20 | 0.53 |
|  | 20 | 0.0ns | 0.04 | 0.09 | 0.20 | 0.56 |
|  | 40 | 0.0ns | 0.05 | 0.12 | 0.35 | 0.72 |
|  | 60 | 0.0ns | 0.06 | 0.21 | 0.52 | 0.94 |
|  | 80 | 0.0ns | 0.07 | 0.28 | 0.53 | 0.97 |
| Case 32 UMi, FR2, DL-TDOA, RANSAC | 0 | 0.0ns | 0.011 | 0.018 | 0.025 | 0.040 |
|  | 0 | 0.1ns | 0.011 | 0.018 | 0.026 | 0.043 |
|  | 0 | 0.5ns | 0.011 | 0.018 | 0.029 | 0.046 |
|  | 0 | 1.0ns | 0.012 | 0.022 | 0.032 | 0.052 |
|  | 0 | 5.0ns | 0.016 | 0.030 | 0.050 | 0.120 |
|  | 0 | 10.0ns | 0.021 | 0.038 | 0.066 | 0.220 |
|  | 0 | 20.0ns | 0.035 | 0.059 | 0.111 | 0.269 |
| Case 33 InH, FR2, DL-TDOA, RANSAC | 0 | 0.0ns | 0.006 | 0.009 | 0.015 | 0.024 |
|  | 0 | 0.1ns | 0.006 | 0.009 | 0.015 | 0.027 |
|  | 0 | 0.5ns | 0.006 | 0.009 | 0.014 | 0.027 |
|  | 0 | 1.0ns | 0.006 | 0.011 | 0.017 | 0.027 |
|  | 0 | 5.0ns | 0.008 | 0.013 | 0.028 | 0.066 |
|  | 0 | 10.0ns | 0.008 | 0.014 | 0.027 | 0.057 |
|  | 0 | 20.0ns | 0.009 | 0.015 | 0.031 | 0.070 |

| Case ID | Kinematic constraint condition | 50% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 34 InF-SH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.024 | 0.033 | 0.049 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.017 | 0.024 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.017 | 0.024 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.017 | 0.024 | 0.031 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.017 | 0.026 | 0.036 |
| Case 35 InF-DH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.030 | 0.038 | 0.058 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.014 | 0.021 | 0.029 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.016 | 0.023 | 0.034 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.018 | 0.028 | 0.038 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.018 | 0.029 | 0.044 |
| Case 36 InH, FR2 DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.031 | 0.046 | 0.071 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.012 | 0.019 | 0.026 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.013 | 0.019 | 0.028 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.022 | 0.031 | 0.044 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.023 | 0.035 | 0.053 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 37, UMI, FR1,RTT+AoA, With Delta Tau, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 2.6 | 4.4 | 5.8 | 7.3 |
| Case 37, UMI, FR1,RTT, With Delta Tau, Perfect Sync, No Timing Errors, AoA & UL PDP Enhancement | (Optional) All UEs | 1.2 | 1.8 | 2.7 | 3.9 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 38, UMI, FR1,DL-TDOA, With Δτ, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 4.1 | 5.2 | 6.8 | 9 |
| Case 38, UMI, FR1,DL-TDOA, With Δτ, Perfect Sync, No Timing Errors, UL PDP Enhancement | (Optional) All UEs | 3.6 | 4.85 | 6 | 8 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 39, UMI, FR1, RTT, With Delta Tau, Perfect Sync, No Timing Errors, NR Rel-16 Baseline | (Optional) All UEs | 3.38 | 4.9 | 6.2 | 9 |
| Case 39, UMI, FR1, RTT, With Delta Tau, Perfect Sync, No Timing Errors, UL PDP Enhancement | (Optional) All UEs | 1.6 | 2.4 | 3.4 | 4.7 |

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Case 40, InF FR1 DH ISD20, 100MHz, LOS Genie + Link Quality | 0.36m | 4.08m | 10.28m | 35.65m |
| Case 40, InF FR1 DH ISD20, 100MHz, RANSAC | 0.05m | 0.09m | 0.18m | 0.59m |
| Case 40, InF FR1 DH ISD20, 100MHz, Matrix Pencil, LOS Genie + Link Quality | 0.22m | 0.56m | 1.15m | 5.11m |
| Case 40, InF FR1 DH ISD20, 100MHz, Matrix Pencil, RANSAC | 0.03m | 0.06m | 0.12m | 0.34m |

#### C.2.1.12	Results from [9]

##### C.2.1.12.1	Description of evaluation scenarios

Evaluation assumptions for system level analysis of NR positioning accuracy enhancements are provided in Table C.2.1.1.12-1.

Table C.2.1.1.12-1: NR positioning enhancements - evaluation scenarios and parameter

| Parameter |  | Case 1, InF-DH, FR1, 120x60m Clutter=40%, hc= 2m, dc = 2m | Case 2, InF-DH, FR1, 120x60m Clutter=40%, hc= 3m, dc = 5m | Case 3, InF-DH, FR1, 120x60m Clutter=60%, hc= 6m, dc = 2m | Case 4, InF-SH, FR1, 300x150m Clutter=20%, hc= 2m, dc = 10m |
| --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline Channel Model based on common assumptions defined related to the channel models of 3GPP TRs 38.901 / 38.802 / 37.857. |  |  |  |
| Carrier frequency | 3.5 GHz |  |  |  |
| Subcarrier spacing | 30KHz |  |  |  |
| Reference Signal Transmission Bandwidth | 100MHz |  |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 |  |  |  |
| Reference signal (type of sequence, number of ports, …) | DL-PRS |  |  |  |
| Number of sites | 18(1-sector, ceiling mount) |  |  |  |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 1 | 1 | 1 | 1 |
| Power-boosting level | 0 | 0 | 0 | 0 |
| Uplink power control (applied/not applied) | NA | NA | NA | NA |
| interference modelling (ideal muting, or other) | Ideal Muting | Ideal Muting | Ideal Muting | Ideal Muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | TOA estimation is based on the average power over the RMS delay-spread determined over the CIR window (CP length of one OFDM symbol). The first sample which exceeds the average-power threshold will determine the FP and TOA. |  |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Take TOA from the 10 strongest sites/links sorted by RSRP. We eliminate potential NLOS links by polarization methods presented in [22] and use the first 3 strongest TOA measurements for horizontal trilateration position estimation: A.) Baseline (No Polarization-LOS detection, no NLOS elimination) B.) Polarization-LOS detection (Angle-based with ρangle=0.1), Dual Polarized UE Receiver C.) Polarization-LOS detection (Power-based with ρpower=0.16), Dual Polarized UE Receiver D.) Polarization-LOS detection (combined with ρangle=0.16, ρpower=0.26), Dual Polarized UE Receiver E.) Polarization-LOS detection (d4 with ρ4=0.13), Single Polarized UE Receiver |  |  |  |
| Network synchronization assumptions | Perfect Sync | Perfect Sync | Perfect Sync | Perfect Sync |
| UE/gNB Tx/Rx Calibration Error | No calibration error | No calibration error | No calibration error | NO calibration error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | No Beam sweeping |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Single Antenna Element, 2 ports with orthogonal linear polarizations (H and V) |  |  |  |
| Additional notes, if any | UE height fixed to 1.5m, gNB height fixed to 8m |  |  |  |

##### C.2.1.12.2	Positioning accuracy evaluation results for NR positioning enhancements

Evaluation results of horizontal location error for NR positioning enhancements are provided in Table C.2.1.12.2-1:

Table C.2.1.12.2-1: NR positioning enhancements - horizontal location error results [m]

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 1A, InF-DH, FR1 {40%, 2m,2m} No Pol-LOS detection (Red) | Convex UEs | 1.35 | 1.75 | 2.35 | 3.82 |
| Case 1B, InF-DH, FR1 {40%, 2m,2m} Angle-based Pol-LOS (Black) | Convex UEs | 1.32 | 1.64 | 2.11 | 2.76 |
| Case 1C, InF-DH, FR1 {40%, 2m,2m} Power-based Pol-LOS (Magenta) | Convex UEs | 1.26 | 1.61 | 2.06 | 2.76 |
| Case 1D, InF-DH, FR1 {40%, 2m,2m} Combined Angle - Power Pol-LOS (Green) | Convex UEs | 1.26 | 1.58 | 1.97 | 2.66 |
| Case 1E, InF-DH, FR1 {40%, 2m,2m} 1-Rx Polarization Pol-LOS (Dashed-Blue) | Convex UEs | 1.32 | 1.68 | 2.15 | 2.93 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 2A, InF-DH, FR1 {40%, 3m,5m} No Pol-LOS detection (Red) | Convex UEs | 1.43 | 1.92 | 2.61 | 7.02 |
| Case 2B, InF-DH, FR1 {40%, 3m,5m} Angle-based Pol-LOS (Black) | Convex UEs | 1.36 | 1.73 | 2.32 | 3.16 |
| Case 2C, InF-DH, FR1 {40%, 3m,5m} Power-based Pol-LOS (Magenta) | Convex UEs | 1.33 | 1.75 | 2.41 | 3.75 |
| Case 2D, InF-DH, FR1 {40%, 3m,5m} Combined Angle - Power Pol-LOS (Green) | Convex UEs | 1.32 | 1.71 | 2.24 | 2.91 |
| Case 2E, InF-DH, FR1 {40%, 3m,5m} 1-Rx Polarization Pol-LOS (Dashed-Blue) | Convex UEs | 1.36 | 1.8 | 2.4 | 3.69 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 3A, InF-DH, FR1 {60%,6m,2m} No Pol-LOS detection (Red) | Convex UEs | 18.82 | 24.51 | 31.39 | 38.02 |
| Case 3B, InF-DH, FR1 {60%, 6m,2m} Angle-based Pol-LOS (Black) | Convex UEs | 18.39 | 24.54 | 31.38 | 39.85 |
| Case 3C, InF-DH, FR1 {60%, 6m,2m} Power-based Pol-LOS (Magenta) | Convex UEs | 19.24 | 25.61 | 32.64 | 40.03 |
| Case 3D, InF-DH, FR1 {60%, 6m,2m} Combined Angle - Power Pol-LOS (Green) | Convex UEs | 19.01 | 25.12 | 31.64 | 40.03 |
| Case 3E, InF-DH, FR1 {60%, 6m,2m} 1-Rx Polarization Pol-LOS (Dashed-Blue) | Convex UEs | 18.79 | 23.85 | 31.16 | 40.63 |

| Horizontal Positioning Error |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| Case 4A, InF-SH, FR1 {20%, 2m,10m} No Pol-LOS detection (Red) | Convex UEs | 2 | 3.02 | 4.73 | 6.88 |
| Case 4B, InF-SH, FR1 {20%, 2m,10m} Angle-based Pol-LOS (Black) | Convex UEs | 1.94 | 2.86 | 4.42 | 6.45 |
| Case 4C, InF-SH, FR1 {20%, 2m,10m} Power-based Pol-LOS (Magenta) | Convex UEs | 1.83 | 2.75 | 4.28 | 6.51 |
| Case 4D, InF-SH, FR1 {20%, 2m,10m} Combined Angle - Power Pol-LOS (Green) | Convex UEs | 1.89 | 2.83 | 4.39 | 6.53 |
| Case 4E, InF-SH, FR1 {20%, 2m,10m} 1-Rx Polarization Pol-LOS (Dashed-Blue) | Convex UEs | 1.76 | 2.73 | 4.23 | 6.61 |

To see the improvement in NLOS link elimination out of the 3 used links for positioning estimation, we provide the percentile improvement in NLOS elimination in Table C.2.1.12.2-2:

Table C.2.1.12.2-2: NR positioning enhancements – NLOS mitigation results

|  | (Avg Number of NLOS paths out of the 3 selected, % NLOS elimination over Baseline) |  |  |  |
| --- | --- | --- | --- | --- |
| Case\LOS detection | Case 1: InF-DH baseline {40%, 2m,2m} | Case 2: InF-DH optional {40%, 3m,5m} | Case 3: InF-DH optional {60%, 6m,2m} | Case 4: InF-SH baseline {20%,2m,10m} |
| A) No Pol-LOS detection (Red) | (0.1, -) | (0.15, -) | (2.85, -) | (0.0045, -) |
| B) Angle-based Pol-LOS (Black) | (0.05, 53.94%) | (0.1, 31.67%) | (2.87, -0.53%) | (0, 100%)* |
| C) Power-based Pol-LOS (Magenta) | (0.07, 35.86%) | (0.11, 24.7%) | (2.86, -0.24%) | (0, 100%) |
| D) Combined Angle and Power Pol-LOS (Green) | (0.03, 72.01%) | (0.06, 56.97%) | (2.86, -0.21%) | (0, 100%)* |
| E) 1-Rx Polarization Pol-LOS (Dashed-Blue) | (0.07, 35.86%) | (0.11, 29.28%) | (2.86, -0.21%) | (0.0024, 46.67%)* |

Figure C.2.1.12.2-1: Case 1: InF-DH baseline scenario with clutter density of 40% and hc= 2m,  dc = 2m

Figure C.2.1.12.2-2: Case 2: InF-DH baseline scenario with clutter density of 40% and hc= 3m,  dc =5m

Figure C.2.1.12-4: Case 4: InF-SH optional scenario with clutter density of 20% and hc= 2m,  dc = 10m

Figure C.2.1.12.2-3: Case 3: InF-DH optional scenario with clutter density of 60% and hc= 6m,  dc = 2m

### C.2.2	Physical layer latency analysis for NR positioning enhancements

#### C.2.2.1	Results from source [4]

##### C.2.2.1.1	Description of evaluation scenarios

The physical layer latency for the following positioning methods are provided

- Enhanced UL E-CID

##### C.2.2.1.2	Latency analysis of NR positioning enhancements

Latency analysis for the Rel.16 NR positioning is provided ins Table C.2.2.1.2-1 and C.2.2.1.2-2.

Table C.2.2.1.2-1: NR positioning enhancements – latency analysis [4]

| Case L101, UL E-CID w/ measurements available Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A, Initial and Final RRC States CONNECTED Assuming UE Rx – Tx time difference is already available |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| gNB Rx – Tx time difference measurement and AoA measurements | 0-20ms | Assuming SRS periodicity is 20ms |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6-26ms |  |

Table C.2.2.1.2-2: NR positioning enhancements – latency analysis [4]

| Case L102, UL E-CID w/o measurements available Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A, Initial and Final RRC States CONNECTED Assuming UE Rx – Tx time difference is not available |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| gNB Tx higher layer processing | 3ms | Configuration of UE Rx – Tx time difference based on SRS and TRS/CSI-RS |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| UE Rx – Tx time difference measurement/gNB Rx – Tx time difference measurement and AoA measurements | 20ms | Assuming TRS/CSI-RS periodicity is 20ms Assuming SRS periodicity is 20ms |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for RRM measurement reporting, including UE Rx – Tx time difference measurement |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Processing UL-SCH containing the measurement and including gNB measurements |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 46-53.5ms |  |

#### C.2.2.2	Results from source [8]

##### C.2.2.2.1	Description of evaluation scenarios

For DL positioning enhancement, aperiodic PRS (A-PRS) could be introduced to reduce positioning latency. In this section, evaluation scenarios and parameters for positioning latency analysis of A-PRS based positioning solution are given as below:

- Case ID: 1.

- SCS: 15kHz.

- Frequency Band: FR1.

- Positioning Technique: DL-TDOA.

- Periodicity of PRS: Aperiodic PRS [R1-2007860]

- Duration time of PRS: 4ms

##### C.2.2.2.2	Latency analysis of NR positioning enhancements

For DL positioning enhancement, aperiodic PRS could be introduced. From our point of view, aperiodic measurement gap can be configured for aperiodic PRS [R1-2007860]. Therefore, whenever triggering, UE should measure those PRS resources. In addition, for measurement reporting, the SR request and UL grant procedures could be arranged before A-PRS triggering. Therefore, the reporting time could be reduced further. The enhanced procedure of DL-TDOA is shown in Figure C.2.2.2.2-1.

Figure C.2.2.2.2-1: Enhanced PHY procedure for DL-TDOA

The total delay could be calculated as

Ttotal=Ttrig+Tariv+Tprocess+Treport

where,

- Ttrig denotes the time to receive and decode the A-PRS triggering signaling.

- Tariv denotes the delay when DL PRS is received, which is related to the PRS offset value configured by the A-PRS triggering signaling

- Tprocess denotes the time from UE begins to measure PRS until the measurement result is ready, which is related to UE capability and the number of PRS resources needed to be measured.

- Treport denotes the time for measurement reporting, which includes SR request, PDCCH-based UL grant and PUSCH-based measurement result reporting..

Assume DCI signalling is used to trigger A-PRS, 1ms could be assumed for Ttrig. Current UE capability is defined as N ms of DL PRS symbols being processed in every T ms for a given maximum bandwidth (B) in MHz supported by UE. N = {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50}ms and T = {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}ms. Assume Y ms of PRS resources are configured, the processing time could be calculated as Tprocess=YN∙T.

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.2.2-1 with the assumption of SCS 15 kHz, and a moderate UE capability, i.e. N=4, T=8.

Table C.2.2.2.2-1: NR positioning enhancements – latency analysis [8]

| Case 1, 15kHz, FR1, DL-TDOA Source UE / Destination NW Positioning technique DL-TDOA, type DL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | 1 | receive and decode the A-PRS triggering signaling |
| PRS arrival delay | 4 | DL PRS arrived for measurement, which is related to the PRS offset value configured by the A-PRS triggering signaling |
| Process time | ≤X, X=YN∙T (e.g. X=8 for Y=4, N=4, T=8 ) | UE begins to measure PRS until the measurement result is ready. This value depends on UE capability the number of PRS resources to be measured. |
| End trigger | 0.5 | PUSCH-based measurement result reporting are included |
| Total values | 13.5 |  |

#### C.2.2.3	Results from source [5]

##### C.2.2.3.1	Description of evaluation scenarios

Brief description of evaluation scenarios and key evaluation parameters.

##### C.2.2.3.2	Latency analysis of NR positioning enhancements

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.3.2-1- Table C.2.2.3.2-6.

Table C.2.2.3.2-1: NR positioning enhancements – latency analysis [5]

| [Case 7], [IIoT/ Commercial], [Frequency Band], [Aperiodic PRS] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | 2.5-34.5 | Dynamic Scheduling latency for activating aperiodic PRS Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand PRS message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. Scheduling offset(TSlot offset) The slot offset can be 0-32 slots if the scheduling offset of SRS and PRS are consistent Process delay is assumed as 2ms |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 44.5~ | The minimum total value is 44.5 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-2: NR positioning enhancements – latency analysis [5]

| [Case 8], [IIoT/ Commercial], [Frequency Band], [on-demand/aperiodic MG] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG | 0.5 | Dynamic Scheduling latency for activating aperiodic/on-demand MG Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. |
| 3: DL measurement &process delay. | [6~10240] Or [130~32770] | DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity. TPRS, which value is {4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120, 10240}ms Process delay is assumed as 2ms |
| 4: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 27.5~ | The minimum total value is 27.5 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-3: NR positioning enhancements – latency analysis [5]

| [Case 9], [IIoT/ Commercial], [Frequency Band], [Positioning BWP] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PDCCH from the gNB carrying activate positioning BWP | 1.5 | Dynamic Scheduling latency for activating positioning BWP Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. BWP switching latency. The value of BWP switching time can refer to Table 8.6.2-1 in TS38.133. Here we take a typical value of 1ms as the BWP switching latency. |
| 3: DL measurement &process delay. | [6~10240] Or [130~32770] | DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity. TPRS, which value is {4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120, 10240}ms. Process delay is assumed as 2ms |
| 4: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 28.5~ | The minimum total value is 28.5 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-4: NR positioning enhancements – latency analysis [5]

| [Case 10], [IIoT/ Commercial], [Frequency Band], [Physical layer triggered] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA, AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range, ms | Description of Latency Component |
| Start trigger | 0.5 | Dynamic Scheduling latency for physical layer triggered positioning request Transmission of the PDCCH from the gNB carrying Positioning requesting message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. |
| 1: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 2: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 3: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 4: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 5: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| End trigger | 0.5 | PUCCH latency for physical layer triggered positioning reporting Transmission of the PUCCH from the UE carrying measurement/location message. Which value less than the DL/UL data transmission time, including Tprocess time_pucch, Talign time andTPUCCH and smaller than 0.5ms. |
| Total values | 44~ | The minimum total value is 44 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-5: NR positioning enhancements – latency analysis [5]

| [Case 11], [IIoT/ Commercial], [Frequency Band], [Combination of on-demand/aperiodic PRS, on-demand/aperiodic MG or Positioning BWP, and physical layer triggered request/report] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA, AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 0.5ms | Dynamic Scheduling latency for physical layer triggered positioning request Transmission of the PDCCH from the gNB carrying Positioning requesting message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms |
| 1: Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG/Positioning BWP switch message | 0.5 ms/1.5ms | Dynamic Scheduling latency for activating aperiodic/on-demand MG/Positioning BWP switch Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG/Positioning BWP switch message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. BWP switching latency: 1ms |
| 2.DL measurement &process delay. | 2.5-34.5 | Dynamic Scheduling latency for activating aperiodic/on-demand PRS Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand PRS message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. Scheduling offset(TSlot offset) The slot offset can be 0-32 slots if the scheduling offset of SRS and PRS are consistent Process delay is assumed as 2ms |
| End trigger | 0.5 ms | PUCCH latency for physical layer triggered positioning reporting Transmission of the PUCCH from the UE carrying measurement/location message. Which value less than the DL/UL data transmission time, including Tprocess time_pucch, Talign time andTPUCCH and smaller than 0.5ms. |
| Total values | 5 ms~ | The minimum total value is 5 ms if only consider the minimum value for every step. |

Table C.2.2.3.2-6: NR positioning enhancements – latency analysis [5]

| [Case 12], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE, Network]/Destination [UE,Network] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-A], Initial and Final RRC States [Idle/inactive] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| 1: Start trigger: DL measurement &process delay. | [6~10240] Or [130~32770] | DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity. TPRS, which value is {4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120, 10240}ms -Process delay is assumed as 2ms |
| 2. Positioning report by RACH procedures | [11.3~168.5] | Positioning report steps in idle/inactive state include below components: -Delay due to RACH scheduling period -Transmission of RACH Preamble -Preamble detection and processing in gNB -Transmission of RA response -UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Resume Request) -Transmission of RRC Resume Request Comparing to the steps of inacitve to connected state as in TR37910 table 5.7.2.1-1, steps 7~10 are ignored, which can save about 10ms. In addition, considering the delay due to RACH period, additional delay of 10~160ms should be added. Therefore, the latency of positioning report in idle/inactive state can be assumed as 11.3~168.5ms. (11.3~18.5ms-10ms+10~160ms) |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 27.3~ | The minimum total value is 27.3ms if only consider the minimum value for every step. |

#### C.2.2.4	Results from source [10]

##### C.2.2.4.1	Description of evaluation scenarios

In order to perform latency analysis for enhanced NR positioning solutions, we use the following assumptions:

- gNB location request: 2 symbols

  - Provides activation of DL PRS, UL SRS transmission, and scheduling information for UE measurement report (e.g. resources for UE measurement report transmission can be associated with resources of SRS for positioning or directly indicated)

  - DL PRS and UL SRS for positioning transmission are performed sequentially

- No MG configuration and BWP switching considerations

- DL PRS resource: 2 symbols, Comb-6, 4 symbols in total per DL PRS Resource Set

- SRS for positioning configuration – same as in reference system configuration

  - DL PRS processing delay – 2 ms (in scope of RAN WG4)

- UE measurement report

  - 7 symbols PUSCH duration (same as in reference system configuration)

Other assumptions are the same as in reference system configuration

##### C.2.2.4.2	Latency analysis of NR positioning enhancements

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.4.2-1.

Table C.2.2.4.2-1: NR positioning enhancements – latency analysis

| Case 4, InF, FR1, R17 DL-TDOA/DL-AoD Case 5, InF, FR1, R17 UL-TDOA/UL-AOA Case 6, InF, FR1, R17 Multi-RTT Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: R17 NR Positioning techniques, type DL+UL, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB measurement request message |
| Step 1. gNB location request | 24.25 symbols 0.8861 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [7] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L2 UE MAC CE processing time] | [~28-84 symbols 1-3 ms] | [In this analysis MAC CE based on PDCCH+PDSCH is assumed for gNB location request. Other possible option is to use PDCCH only that can further reduce latency TBD by RAN WG2] |
| Step 2: DL PRS transmission | 64 symbols 2.2858 ms | Value is provided for L1 components only |
| 2A. DL PRS alignment time | 4 symbols 0.1429 ms | Worst case: 4ms for selected reference system configuration to reflect slot boundary |
| 2B. DL PRS transmission time | 4 symbols 0.1429 ms |  |
| 2C. DL PRS processing and report delay | [~56 symbols 2 ms] | [ Not defined. It is assumed that new UE capabilities for low latency NR positioning are to be introduced. Definition is in scope of RAN WG4] |
| Step 3. SRS for positioning transmission | 29 symbols 1.0357ms | Value is provided for L1 components only |
| 3A: SRS for positioning preparation time | 0 ms | Not defined. 0ms is assumed for analysis (can be prepared in advance, e.g. during DL PRS processing or earlier) |
| 3B: SRS for positioning alignment time | 14 symbols 0.5ms | Depends on SRS resource allocation. Worst case: 14 symbols for selected reference configuration |
| 3C: SRS for positioning transmission time | 1 symbol 0.0357 | 1 symbol for selected reference configuration |
| 3D: SRS for positioning processing time | [14 symbols 0.5 ms] | [Not defined: 0.5 ms is assumed. Definition is in scope of RAN WG4] |
| Step 4. UE measurement report | 20.5symbols 0.7321ms | Value is provided for L1 components only |
| 4A: [L2 MAC CE preparation] | [~28 symbols 1 ms] | [In this analysis MAC CE based on PUSCH is assumed. TBD by RAN WG2] |
| 4B: PUSCH alignment time | 7 symbols 0.2500 ms |  |
| 4C: PUSCH transmission time | 7 symbols 0.2500 ms | 7 symbols for selected reference system configuration |
| 4D: PUSCH processing time | 6.5 symbols 0.2321 ms | Not specified. For analysis, the assumption from the [7] is used. gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| End trigger |  | Transmission by the gNB measurement response message |

Based on analysis in table above, we have following observations

- DL-TDOA latency of L1 components = Step 1 + Step 2 + Step 4 = 3.8839 ms

  - Estimated higher layer signaling time (L2/L3 components) is up to 4ms

  - Total time is about 8 ms

- UL-TDOA latency of L1 components = Step 1 + Step 3 = 1.9018 ms

  - Estimated higher layer signaling time (L2/L3 components) is up to 3ms

  - Total time is about 5 ms

- Multi-RTT latency of L1 components = Step 1 + Step 2 + Step 3 = 4.1875 ms

  - It is assumed that Step 4 is done in parallel with Step 3B,3C,3D

  - Estimated higher layer signaling time (L2/L3 components) is up to 4ms

  - Total time is about 8.2 ms

#### C.2.2.5 Results from source [16]

##### C.2.2.5.1 Description of evaluation scenarios

The latency analysis of enhancements for the UE-assisted DL methods are presented here. All analysis are based on 30kHz SCS. The minimum latency value is presented here. Specifically, we perform the latency analysis of three following enhancements:

- No measurement gap for DL PRS reception

- Measurement gap activation/deactivation

- On-demand PRS

##### C.2.2.5.2 Latency analysis of NR positioning enhancements

Table C.2.2.5.2-1: Latency analysis for no measurement gap DL methods

| Source [NW]/Destination [NW] Positioning technique: DL methods, mode ; UE-A Initial and Final RRC States [CONNECTED] Positioning technique: No measurement gap for PRS reception methods |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message and PRS configuration activation. | 1 |  |
| Step 2: The UE decodes the LPP Request location | 10 | NAS message |
| Step 3: The UE receives PRS without MG | 2 |  |
| Step 4: UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 |  |
| Total values | 16ms |  |

Table C.2.2.5.2-2: Latency analysis for measurement gap activation/deactivation DL methods

| Source [NW]/Destination [NW] Positioning technique: DL methods, mode ; UE-A Initial and Final RRC States [CONNECTED] Positioning technique: Measurement gap activation/deactivation DL methods |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message, MG and PRS configurations | 1 |  |
| Step 2: The UE decodes the LPP Request location | 10 | NAS message |
| Step 3: The UE transmits MG activation request in an MAC CE or UCI | 1 |  |
| Step 4: The gNB successfully decodes the MG activation request. | 3 | MAC CE or UCI |
| Step 5: The gNB activates the MG for the UE using MAC CE or UCI | 1 |  |
| Step 6: UE receive MG activation and apply the MG configuration | 3 | MAC CE or UCI |
| Step 7: UE receives PRS in the MG | 2 |  |
| Step 8: The UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 |  |
| Total values | 24ms |  |

Table C.2.2.5.2-3: Latency analysis for on-demand DL PRS methods

| Source [NW]/Destination [NW] Positioning technique: DL methods, mode ; UE-A Initial and Final RRC States [CONNECTED] Positioning technique: On-demand PRS |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: The UE transmits measurement gap (MG) request in an RRC message. | 1 |  |
| Step 2: The gNB successfully decode the MG request message | 5 | RRC message |
| Step 3: The gNB configures MG in an RRC message | 1 |  |
| Step 4: The UE receive the MG configuration message and apply the MG configuration. | 10 | RRC message |
| Step 5: The UE receives PRS in the MG | 2 |  |
| Step 6: The UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 |  |
| Total values | 22ms |  |

#### C.2.2.6	Results from [17]

##### C.2.2.6.1	Description of evaluation scenarios

Brief description of evaluation scenarios and key evaluation parameters.

##### C.2.2.6.2	Latency analysis of NR positioning enhancements

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.1.6-1.

Table C.2.2.1.6-1: NR positioning enhancements – latency analysis

| Proposal | L1 Latency | Explanation |
| --- | --- | --- |
| Support Low-layer (e.g., unicast/group-common DCI, MAC-CE) triggering of DL/UL PRS transmission/muting/Location-Request for DL-only and DL/UL methods. | < 1 slot (for DCI-based) 3 msec (for DL MAC-CE) | Instead of the UE receiving through a PDSCH carrying an RRC message the Location Information Request, a DCI-based triggering may be interpreted by the UE, depending on the scenario within the same slot that the PDCCH is received. A MAC-CE Request may be interested within 3 msec. |
| Support DCI/MAC-CE triggering of Measurement gaps (MG) for the purpose of positioning measurements | < 1 slot (for DCI-based) 3 msec (for DL MAC-CE) | Instead of the gNB transmitting an RRC configuration that includes the MG request, a DCI-based triggering may be interpreted by the UE, depending on the scenario within the same slot that the PDCCH is received. A MAC-CE Request may be interested within 3 msec. The smallest MG periodicity currently is 20 msec, so a UE may have to wait, after it has interpreted the RRC message, an additional 20 msec before receiving the MG in NR Rel-16. If the MG is triggered by DCI or MAC-CE, the UE would not have to wait for a long period of time for the first occasion of the MG. |
| Fast/real-time processing of short PRS instances: Support Enhanced PRS processing capabilities Support partially-staggered or no-staggered DL-PRS transmissions | 1-4 slots | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before it is ready to report. Furthermore, the maximum capability with respect to the number of PRS resources within a slot that is currently supported is 64 resources. If we want the UE to process, e.g. 18 TRPs, each one with at least 4 beams, with single-symbol comb-6 PRS, we would need UEs that can process 72 resources per slot. |
| Support Low-layer (e.g. UL MAC-CE or UCI) Measurement Reporting towards the serving gNB | 1 slot (for UCI) 3 msec (form UL MAC-CE) | Instead of the UE reporting the measurements as part of RRC in a NAS container (2-5 msec), a UCI-based would require < 1 slot, whereas a MAC-CE alternative would require 3 msec. |

#### C.2.2.7 Results from source [15]

##### C.2.2.7.1	Description of evaluation scenarios

To evaluate physical layer latency for the enhanced NR positioning solution(s), we use followings assumption as below:

- The gNB provides the UE with reporting resource of positioning measurements when the gNB configures MG.

  - Numerology: 15 kHz (for PDCCH, PDSCH, PUCCH, PUSCH, SRS)

- For PUSCH transmission:

  - Uplink switching gap is not configured.

  - No BWP switching

  - No overlapping symbols of the PUCCH and the scheduled PUSCH

  - The number of PUSCH symbols = from 4 to 14 for Type A

  - The number of PUSCH symbols = from 1 to 14 for Type B

  - According to the size of data (e.g. LPP message) to be transmitted, the number of symbols can be changed.

- For PDSCH transmission:

  - No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH

  - The number of PDSCH symbols = from 3 to 14 for Type A

  - The number of PDSCH symbols = from 2 to 14 for Type B

  - According to the size of data (e.g. LPP message) to be transmitted, the number of symbols can be changed.

- For SRS transmission:

  - One shot transmission (2 OS ~ 12 OS)

- According to resource type (periodic, Semi-persistent and aperiodic), the number of symbols can be changed.

#### C.2.2.7.2	Latency analysis of NR positioning enhancements

Summary of latency performance analysis for NR positioning enhancements is provided in Table C.2.2.7.2-1- Table C.2.2.7.2-5.

Table C.2.2.7.2-1: NR positioning enhancements – latency analysis [15]

| Source/Destination: NW/UE Positioning technique DL-TDOA and DL-AOD, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Request Location Information message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Request Location Information message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Constructing measurement gap request message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | [X1] | [X1] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| *Transportation of Measurement gap request message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5. The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| *Constructing Provide Location information message(RRC processing time, UE) | 10 ms | 38.331 section 12. |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | [X2] | [X2] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 73.12+[X1]+[X2]ms ~ 217.12+[X1]+ [X2]ms For UE capability-2: 71.68+[X1]+[X2]ms ~ 215.26+[X1]+ [X2]ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 23.12ms ~ 167.12ms For UE capability-2: 21.68ms ~ 165.26ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.2.2.7.2-2: NR positioning enhancements – latency analysis [15]

| Source/Destination: UE/NW Positioning technique DL-TDOA, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | [X1] | [X1] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| *Transportation of measurement gap request message (RRC processing time, gNB) | 10ms | 38.331 section 12. |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Measurement gap configuration message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5 The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| *Provide Location information message (UE) | 10 ms | 38.331 section 12. |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | [X2] | [X2] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 62.41+[X1]+[X2]ms ~ 205.34+[X1] +[X2]ms For UE capability-2: 61.33+[X1]+[X2]ms ~ 204.05+[X1] +[X2]ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 22.41ms ~ 165.34ms For UE capability-2: 22.33ms ~ 164.05ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.2.2.7.2-3: NR positioning enhancements – latency analysis [15]

| Source/Destination: NW/UE Positioning technique UL-TDOA and UL-AOA, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| SRS configuration (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS configuration (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS configuration message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of SRS activation message (RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Transmission of SRS activation (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS activation (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS activation message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| SRS transmission (UE) | 0.07ms ~ 0.86ms | 38.214 section 6.2.1. |
| *Reception of SRS and preparation time for gNB to make NRPPa message (gNB) | [X3] | [X3] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 31.49+[X3] ms ~ 34.42+[X3] ms For UE capability-2: 30.77+[X3] ms ~ 33.28+[X3] ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 1.49ms ~ 4.42ms For UE capability-2: 0.77ms ~ 3.28ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.2.2.7.2-4: NR positioning enhancements – latency analysis [15]

| Source/Destination: NW/UE Positioning technique: Multi-RTT, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| SRS configuration (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS configuration (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS configuration message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of SRS activation message (RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Transmission of SRS activation (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS activation (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS activation message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of provide assistance message (RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Transmission of LPP provide assistance data (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of LPP provide assistance data (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of provide assistance message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of request location information (RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Request Location Information message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Location Information message Reception (PHY processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of LPP message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Transportation of measurement gap request (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | [X1] | [X1] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| *Transportation of measurement gap request message (RRC processing time, gNB) | 10ms | 38.331 section 12. |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 10 ms | 38.331 section 12. |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Measurement gap configuration message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5 The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| SRS transmission (UE) | 0.07ms ~ 0.86ms | 38.214 section 6.2.1. |
| *Reception of SRS and preparation time for gNB to make NRPPa message (gNB) | [X3] | [X3] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| *Constructing Provide Location information message(RRC processing time, UE) | 10 ms | 38.331 section 12. |
| measurement report transmission(including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | [X2] | [X2] is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 145.34+[X1]+[X2]+[X3]ms ~ 293.32+[X1]+[X2]+[X3]ms For UE capability-2: 142.8+[X1]+[X2]+[X3]ms ~ 289.75+[X1]+[X2]+[X3] ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 25.34ms ~ 173.32ms For UE capability-2: 22.8ms ~169.75ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.2.2.7.2-5: NR positioning enhancements – latency analysis [15]

| Source/Destination: NW/UE Positioning technique: E-CID, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| LPP Request Location Information message transmission(gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of LPP Request Location Information message (RRC processing time, UE) | 10 ms | 38.331 section 12. |
| *Constructing Provide Location information message(RRC processing time, UE) | 10 ms | 38.331 section 12. |
| measurement report transmission(including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement report, SRS and preparation time for gNB to make NRPPa message (gNB) | [X2] | [X2]is up to gNB processing capability and it can vary depending on LS’s response from higher layer |
| Total values | For UE capability-1: 21.56+[X2]ms ~ 23.56+[X2] ms For UE capability-2: 20.84+[X2] ms~22.63+[X2] ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 1.56ms ~ 3.56ms For UE capability-2: 0.84ms~2.63ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

## C.3	Efficiency analysis for NR positioning enhancements

### C.3.1	Network efficiency analysis for NR positioning enhancements

#### C.3.1.1	Results from source [4]

##### C.3.1.1.1	Description of evaluation scenarios

1-symbol PRS with comb-4 and comb-12 is evaluated compared to 4-symbol and 12-symbol, respectively, assuming

- PRS transmission periodicity is 160ms

- PRS numerology is 30kHz

- 8 PRS resources with 12 symbols or 4 symbols per PRS resource

- The PRS bandwidth is 100MHz

In addition, E-CID is also compared against PRS-based methods.

##### C.3.1.1.2	Network efficiency analysis of NR positioning enhancements

The evaluation results are summarized in Table C.3.1.1.2-1.

Table C.3.1.1.2-1: NR positioning enhancements – network efficiency summary [4]

| Case | Positioning resource utilization |
| --- | --- |
| Case E1: PRS with 12 symbols | 2.14% |
| Case E2: PRS with 4 symbols | 0.714% |
| Case E3: PRS with 1 symbol | 0.179% |
| Case E4: E-CID enhancement | 0% |

By reducing the PRS symbols from 12 and 4 to 1 for comb-12 and comb-4, respectively, the overhead of PRS transmission is reduced by 11/12 and 3/4, respectively.

Moreover, by allowing 1-symbol PRS transmission, network may reuse CSI-RS for mobility for positioning, further reducing the PRS overhead.

Since enhanced E-CID may only use existing communication signals, it has not extra positioning overhead.

#### C.3.1.2	Results from source [8]

##### C.3.1.2.1	Description of evaluation scenarios

In this section, evaluation scenarios and parameters of network efficiency analysis for NR positioning enhancements are given in Table C.3.1.2.1-1.

Table C.3.1.2.1-1: NR positioning enhancements - evaluation scenarios and parameters [8]

| Parameter | [Case 1], [InF-HH-2D], [FR1], [DL-TDOA+ DL-CPP] | [Case 2], [InF-HH-2D], [FR1], [UL-TDOA+ UL-CPP] |
| --- | --- | --- |
| Channel model (baseline) | InF-HH-2D | InF-HH-2D |
| Carrier frequency | 3.5GHz | 3.5GHz |
| Subcarrier spacing | 30KHz | 30KHz |
| System Bandwidth | 100MHz | 100MHz |
| Reference Signal Transmission Bandwidth | 100MHz | 100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL PRS, Comb- 6 | UL SRS, Comb- 8 |
| Reference signal (type of sequence, number of ports, …) | Gold sequence, Port-1 | ZC sequence, Port-1 |
| Number of sites | 6 | 6 |
| Number of symbols used per occasion | 12 | 12 |
| number of occasions used per positioning estimate | 1 | 1 |
| Evaluated Enhancement for Rel.17 | DL-TDOA+ DL-CPP | UL-TDOA+ UL-CPP |
| Periodicity of reference signal | 20ms | 20ms |
| Number of PRS resource sets in one periodicity | 1 | 1 |
| Number of PRS Resources per PRS resource set | 1 | 1 |
| Number of users per site | 1,30 | 1,30 |

##### C.3.1.2.2	Network efficiency analysis of NR positioning enhancements

For the proposed NR positioning enhancements, we use DL-TDOA+DL-CPP or UL-TDOA+UL-CPP as the Rel.17 enhancement scheme. Carrier Phase Positioning can be supported by using Rel.16 DL-PRS or UL-SRS, i.e., the UE or gNB can get the carrier phase measurements together with other NR positioning measurements without additional RF resources and new reference signals. Therefore, the network efficiency of our proposed NR positioning enhancements is the same as that of Rel.16 positioning solutions.

The definition of DL-PRS/UL-SRS resource utilization and the calculation of DL-PRS/UL-SRS resource utilization according to the described definition are shown as in below.

Definition of DL-PRS resource utilization

Assume gNB x has M TRPs (the index of TRP is j, and j=1~M-1) which are configured to transmit DL-PRS, and each TRP is configured with N DL-PRS resource sets (the index of DL-PRS resource set is i, i=1~N-1). In order to avoid the interference among these DL-PRS resource sets, either the time-domain resources or the frequency-domain resources are orthogonal among these DL-PRS resource sets belongs to the same gNB.

We use DL_PRS_RU to denote DL-PRS resource utilization, then the DL-PRS resource utilization of gNB x (DL_PRS_RUgNB x) can be derived as follows,

DL_PRS_RUPRS resource set i = DL_PRS_RUPRS resource set i, time-domain×DL_PRS_RUPRS resource set i, frequency-domain

DL_PRS_RUPRS resource set i, time-domain= NumOfSymbolsPRS resourcePeriodicityPRS resource set i×NumOfSymbolsPer millisecond×NumofPRSResourcesPRS resource set i×RepetitionFactorPRS resource set i

DL_PRS_RUPRS resource set i, frequency-domain=BandWidthPRS resource set iBandWidthSystem×1CombSizeNPRS resource set i

DL_PRS_RUTRP j = i=1NDL_PRS_RUPRS resource set i

DL_PRS_RUgNB x = j=1MDL_PRS_RUTRP j

Calculation of DL-PRS resource utilization

The calculation of DL-PRS resource utilization according to the above described definition are shown as follows,

DL_PRS_RUPRS resource set i, time-domain=1220×28×1×1=2.1%

DL_PRS_RUPRS resource set i, frequency-domain=100MHz100MHz×16=16.7%

DLPRSRUPRS resource set i = DL_PRS_RUPRS resource set i, time-domain×DL_PRS_RUPRS resource set i, frequency-domain=2.1%×16.7%=0.36%

DL_PRS_RUTRP j = i=11DL_PRS_RUPRS resource set i=0.36%

DL_PRS_RUgNB x = j=16DL_PRS_RUTRP j=2.1%

Definition of SRS-Pos resource utilization

Assume gNB x serves M UEs (the index of UE is j, and j=1~M-1) which are configured to transmit SRS-Pos, and each UE is configured with N SRS-Pos resource sets (the index of SRS-Pos resource set is i, i=1~N-1). In order to avoid the interference among these SRS-Pos resource sets, either the time-domain resources or the frequency-domain resources are orthogonal among these SRS-Pos resource sets belongs to the same gNB.

We use SRS_Pos_RU to denote SRS-Pos resource utilization, then the SRS-Pos resource utilization of gNB x (SRS_Pos_RUgNB x) can be derived as follows,

SRS_Pos_RUSRS resource set i = SRS_Pos_RUSRS resource set i, time-domain×SRS_Pos_RUSRS resource set i, frequency-domain

SRS_Pos_RUSRS resource set i, time-domain= NumOfSymbolsSRS resourcePeriodicitySRS resource set i×NumOfSymbolsPer millisecond×NumofSRSResourcesSRS resource set i

SRS_Pos_RUSRS resource set i, frequency-domain=BandWidthSRS resource set iBandWidthSystem×1CombSizeNSRS resource set i

SRS_Pos_RUUE j = i=1NSRS_Pos_RUSRS resource set i

SRS_Pos_RUgNB x = j=1MSRS_Pos_RUUE j

Calculation of SRS-Pos resource utilization

The calculation of UL-SRS resource utilization according to the above described definition are shown as follows,

SRS_Pos_RUSRS resource set i, time-domain= 1220×28×1=2.1%

SRS_Pos_RUSRS resource set i, frequency-domain=100MHz100MHz×18=12.5%

SRS_Pos_RUSRS resource set i = SRS_Pos_RUSRS resource set i, time-domain×SRS_Pos_RUSRS resource set i, frequency-domain=4.3%×12.5%=0.5%

SRS_Pos_RUUE j = i=11SRS_Pos_RUSRS resource set i=0.27%

When there are 1 user per site,

SRS_Pos_RUgNB x = j=11×6SRS_Pos_RUUE j=1.6%

When there are 30 users per site,

SRS_Pos_RUgNB x = j=130×6SRS_Pos_RUUE j=48.2%

#### C.3.1.3	Results from source [5]

##### C.3.1.3.1	Description of evaluation scenarios

Brief description of evaluation scenarios and key parameters of evaluation. section

##### C.3.1.3.2	Network efficiency analysis of NR positioning enhancements

For the network efficiency based on PRS, at least the comb size, number of symbols, TRP number, PRS periodicity, resource repetition factor, muting pattern, numerology and beam sweeping assumptions of PRS should be included in resource utilization. It can be represented as equation (1)

PRS:Network efficiency = TRPNum×NumSymbolsCombSizeN×BeamNumPRS periodicity×symbolNumOfms×100%                                    (1)

For the network efficiency based on SRS, at least the comb size, number of symbols, UE number, SRS periodicity, numerology and beam sweeping assumptions of SRS should be included in resource utilization. It can be represented as below (2)

SRS:Network efficiency = UENum×NumSymbolsCombSizeN×BeamNumSRS periodicity×symbolNumOfms×100%                        (2)

The evaluation result of network and UE efficiency in terms of the percentage of PRS in the total transmission being presented as Table C.3.1.3.2-2 based on the below assumptions listed in Table C.3.1.3.2-1.

Table C.3.1.3.2-1: The parameter assumption for evaluation

| Parameter | Configuration |
| --- | --- |
| Reference Signal Transmission Bandwidth | 100 MHz for FR1 400 MHz for FR2 |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Rel-16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) |
| Number of sites | 18 |
| Number of beams | 1 for FR1 64 for FR2 |

Table C.3.1.3.2-2 The network and UE efficiency for different physical layer latency

| u | TPRS (ms) | Tmeas Gap ms | Physical layer Latency (ms) (case 3 in 3.1.3) | MGL ms For max{TPRS, Tmeasuement gap} | Network efficiency (percentage) | UE efficiency |
| --- | --- | --- | --- | --- | --- | --- |
| 30K (FR1) | 4 | 20 | 55.5 | 3ms | 16.07% | 15.00% |
|  | 5 | 20 | 55.5 | 3ms | 12.85% | 15.00% |
|  | 8 | 40 | 75.5 | 3ms | 8.03% | 7.50% |
|  | 10 | 20 | 55.5 | 3ms | 6.42% | 15.00% |
|  | 16 | 80 | 115.5 | 3ms | 4.01% | 3.75% |
|  | 20 | 20 | 55.5 | 3ms | 3.21% | 15.00% |
|  | 32 | 160 | 195.5 | 3ms | 2.0% | 1.88% |
|  | 40 | 40 | 75.5 | 3ms | 1.6% | 7.50% |
|  | 64 | 160 | 355.5 | 3ms | 1.0% | 1.88% |
|  | 80 | 80 | 115.5 | 3ms | 0.8% | 3.75% |
|  | 160 | 160 | 195.5 | 3ms | 0.4% | 1.88% |
|  | 320 | 160 | 355.5 | 3ms | 0.2% | 1.88% |
|  | 640 | 160 | 675.5 | 3ms | 0.1% | 1.88% |
|  | 1280 | 160 | 1315.5 | 3ms | 0.05% | 1.88% |
|  | 5120 | 160 | 5155.5 | 3ms | 0.01% | 1.88% |
|  | 10240 | 160 | 10275.5 | 3ms | 0.006% | 1.88% |
| 120K (FR2) | 4 | 20(160Note) | 55.5 | 20ms | 257.14% | 100.00% |
|  | 5 | 20(160) | 55.5 | 20ms | 205.71% | 100.00% |
|  | 8 | 40(160) | 75.5 | 20ms | 128.57% | 50.00% |
|  | 10 | 20(160) | 55.5 | 20ms | 102.85% | 50.00% |
|  | 16 | 80(160) | 115.5 | 20ms | 64.28% | 25% |
|  | 20 | 20(160) | 55.5 | 20ms | 51.42% | 100.00% |
|  | 32 | 160(160) | 195.5 | 20ms | 32.14% | 12.5% |
|  | 40 | 40(160) | 75.5 | 20ms | 25.71% | 50.00% |
|  | 64 | 160(160) | 355.5 | 20ms | 16.07% | 12.5% |
|  | 80 | 80(160) | 115.5 | 20ms | 12.85% | 12.50% |
|  | 160 | 160(160) | 195.5 | 20ms | 6.42% | 12.5% |
|  | 320 | 160(160) | 355.5 | 20ms | 3.21% | 12.5% |
|  | 640 | 160(160) | 675.5 | 20ms | 1.60% | 12.5% |
|  | 1280 | 160(160) | 1315.5 | 20ms | 0.80% | 12.5% |
|  | 5120 | 160(160) | 5155.5 | 20ms | 0.20% | 12.5% |
|  | 10240 | 160(160) | 10275.5 | 20ms | 0.10% | 12.5% |

It is observed that the network efficiency exceeds 100%, or the MGL/MGRP(UE efficiency) exceed 30% in some FR2 cases.

So, reducing the amount of PRS that sending per unit time can effectively reduce the Network efficiency. On-demand PRS can adjust the configuration of PRS based on the requirement to achieve low network efficiency. For example, on-demand PRS can determine which repetition and TRP transmitting if assumed 16 repetitions in 160ms for every TRP. The minimum Network and UE efficiency of on-demand PRS is 0% if no positioning requesting in 160ms or no positioning requesting for the UE. It is observed that the network and device efficiency will be reduced by on-demand PRS within the same level latency.

While, the network efficiency of aperiodic PRS are affected by the number of activations within unit time (such as 160ms). It can be observed the network efficiency of aperiodic PRS less than the Periodic PRS with 160ms periodicity if the number of activations less than 1 and less than the Periodic PRS with 10ms periodicity if the number of activations less than 10. Meanwhile, the latency can be reduced by nearly 20ms.

Table C.3.1.3.2-3: Network efficiency analysis for periodic PRS VS on demand PRS [5]

| [Case N1], [IIoT/ Commercial], [Frequency Band], [Periodic PRS VS On demand PRS] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Option | Periodic PRS |  | On-demand PRS |
| Configuration | Period 10ms | Period 160ms | Period 10ms, repetition :16 Muting repetition for on-demand service |
| PHY Latency (minimum) | Latencymin= 62ms Where DL measurement &process delay equals MGL(20ms). |  | Latencymin=46ms Where MGL and process time for minimum TRP (such as 4ms). |
| PHY Latency (maximum) | Latencymax=64mss | Latencymax=204ms | Latencymax=64ms |
| Network efficiency | TRPNum×NumSymbolsCombSizeN×beamNumPRS periodicity×symbolNumOfms =51.42% | 6.42% | i=015Boolean(Yi>0)* TRPNum* NumSymbols / CombSizeN160 =0~51.4% |

Table C.3.1.3.2-4: Network efficiency analysis for periodic PRS VS aperiodic PRS [5]

| [Case N2], [IIoT/ Commercial], [Frequency Band], [Periodic PRS VS Aperiodic -PRS] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Option | Periodic PRS |  | Aperiodic -PRS |
| Configuration | PRS Periodicity: 10ms MG periodicity: 20ms MGL:20ms | PRS Periodicity :160ms MG periodicity: 160ms MGL:20ms | SlotOffset INTEGER (1..32) |
| PHY Latency (minimum) | Latencymin=62ms |  | Latencymin=46ms Where MGL and process time for minimum TRP (such as 4ms). |
| PHY Latency (maximum) | Latencymax=64ms | Latencymax=204ms | Latencymax=48ms Where MGL and process time for minimum TRP equals 6ms if slot offset is 32 and scs is 120kHZ |
| Network efficiency | TRPNum×NumSymbolsCombSizeN×beamNumPRS periodicity×symbolNumOfms =51.42% | 6.42% | NumActivation×TRPNum×NumSymbolsCombSizeN×beamNum160ms×symbolNumOfms = NumActivation×6.42% |

#### C.3.1.4	Results from source [12]

##### C.3.1.4.1	Description of evaluation scenarios

We consider the measurement gap as the resource overhead for DL PRS reception. The UE can measure DL PRS resource outside the active BWP or with a numerology different from that of the active BWP. Furthermore, as specified in rel16 UE feature, measurement gap is necessary for a UE to receive and process DL PRS resource.  During the measurement gap, the UE cannot receive and transmit the normal NR traffic. Thus we can consider all the radio resource within each measurement gap are ‘occupied’ by the NR positioning. The resource utilization can be calculated as:

ρPRS=LMGTMG

Where TMG and LMG are the measurement gap repetition period and length of measurement gap respectively.

C.3.1.4.2	Network efficiency analysis of NR positioning enhancements

As specified in rel16, the length of measurement gap can be : 1.5/3/3/5/4/5.5/6 ms and the periodicity of measurement gap can be 20/40/80/160 ms. Given that, the resource overhead for DL PRS reception are as follows:

| measurement gap repetition period | Overhead of resource for DL PRS: 1.5/3/3.5/4/5.5/6 ms length |
| --- | --- |
| 20ms | 7.5% 15% 17.5% 20% 27.5% 30% |
| 40ms | 3.75% 7.5% 8.75% 10% 13.75% 15% |
| 80ms | 1.875% 3.75% 4.375% 5% 6.8755 7.5% |
| 160ms | 0.9375% 1.875% 2.1875% 2.5% 3.4375% 3.75% |

#### C.3.1.5	Results from source [21]

##### C.3.1.5.1	Description of evaluation scenarios

Brief description of evaluation scenarios and key parameters of evaluation. Section

Network efficiency was evaluated for 1-symbol PRS, using comb 2,4,6 and 12.

##### C.3.1.5.2	Network efficiency analysis of NR positioning enhancements

A comparative table of rel16 PRS with 1symbol PRS is shown for comb 2,4,6,12 in table C.3.1.5.2-1:

table C.3.1.5.2-1:1-symbol PRS resource utilization  summary

| Scenario | Configuration details | L1 RS occupancy, ms | Resource utilization vs Rel16 |
| --- | --- | --- | --- |
| Inf-SH | comb-2, 2 symbols, no interference | 18symbols / 0.64ms |  |
| Inf-SH | comb-2, 1 symbol, time reuse 9, interference | 9symbols /0.32ms | -50% |
| Inf-SH | comb-6, 6 symbols, no interference | 18 symbols /0.64ms |  |
| Inf-SH | comb-6, 1 symbol, time reuse 3, interference | 3symbols /0.11ms | -84% |
| Inf-SH | comb-12, 12 symbols, no interference | 24symbols /0.85ms |  |
| Inf-SH | comb-12, 1 symbol, time reuse 1, interference | 2symbols /0.07ms | -91% |
| IOO | comb-2, 1 symbol, time reuse 6, interference | 6symbols/0.22ms | -50% |
| IOO | comb-2, 2 symbols, interference | 12symbols/0.44ms |  |
| IOO | comb-4, 4 symbols, no interference | 12symbols/0.44ms |  |
| IOO | comb-4, 1 symbol, time reuse 3, interference | 3symbols /0.11ms | -75% |
| IOO | comb-12, 12 symbols, no interference | 12symbols/0.44ms |  |
| IOO | comb-12, 1 symbol, time reuse 1, interference | 1symbols/0.03ms | -91% |

#### C.3.1.6 Results from source [18]

##### C.3.1.6.1	Description of evaluation scenarios

Network efficiency is evaluated for UL SRS resource utilization for the following cases:

- UL SRS resource utilization w.r.t to the cyclic shift enhancements

- UL SRS resource utilization w.r.t to the number of OFDM symbols

- UL SRS resource utilization w.r.t to the number of antenna ports

##### C.3.1.6.2	Network efficiency analysis of NR positioning enhancements

UL SRS resource utilization w.r.t to the cyclic shift enhancements:

The number of simultaneous UEs of CS separated is chosen based on the evaluation so that the configured with different cyclic shifts on the same REs are detected without ambiguity.

A comparative table of Rel16 SRS with cyclic shift enhancements is provided in Table C.3.1.6.2-1:

Table C.3.1.6.2-1: Cyclic shift SRS enhancements

| Scenario | Rel-16/ Rel-17 enhancements | COMB | nbSym | Cyclic shift range | max number of CSs | Number of simultaneous UEs of CS separated | Improvement factor vs Rel16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Umi | CS enhancement | 4 | 4 | fftlength | 48 | 8 | 4 |
|  | Rel-16 | 4 | 4 | fftlength/KTC | 12 | 2 |  |
| Umi | CS enhancement | 8 | 8 | fftlength | 48 | 8 | 8 |
|  | Rel-16 | 8 | 8 | fftlength/KTC | 6 | 1 |  |
| InF | CS enhancement | 2 | 1 | fftlength/KTC | 8 | 8 | - |
|  | Rel-16 | 2 | 1 | fftlength/KTC | 8 | 8 |  |

The following analysis shows that UMi performance with resource sharing doesn’t degrade compared with the case when no resource sharing is configured:

Config 1011.2 (blue): resource sharing : Comb 4, 4 symbols, CS enhancement, phase correction in transmitter (Rel-17), 6 UEs use the same REs and are separated by different cyclic shift values

Config 1011.2 (red) , no resource sharing : Comb 4, 4 symbols, no phase correction, Coherent Destaggering with phase correction in receiver (Rel-16)

Figure C.3.1.6.2-1 2D-Error in UMi scenario SRS (Comb 4, 4 symbols) applying ATOA model. Blue: CS enhancement (Rel-17), 6 UEs use the same REs and are separated by different cyclic shift values. Red: no resource sharing.

UL SRS resource utilization w.r.t to the number of OFDM symbols:

For scenarios with low delay spread and low propagation delay difference (e.g. indoor with low ISD) COMB=4 may be sufficient. Rel-16 does not allow SRS for positioning with COMB=4, 1 Symbol. Alternatively a Rel-15 SRS may be supported for positioning purpose.

A comparative table of Rel16 SRS and Comb-4 one symbol is provided in Table C.3.1.6.2-2:

Table C.3.1.6.2-2: UL SRS resource utilization w.r.t to the number of OFDM symbols:

| Scenario | Rel-16/ Rel-17 enhancements | COMB | nbSym | max number of CSs | Number of simultaneous UEs of CS separated | REs per UE and per RB | Improvement factor vs Rel16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| InF | SRS with one symbol for COMB 4 | 4 | 1 | 12 | 6 | 3 | 2 |
|  | Rel-16 | 4 | 2 | 12 | 6 | 6 |  |

UL SRS resource utilization w.r.t to the number of antenna ports:

For UL SRS resource utilization we compare the improvement factor in terms of the Capacity per UE for Rel-16 and Rel-17 enhancements. We derive the improvement factor as Capacity per UERel-16 Capacity per UERel-17  ,  here  the Capacity per UE=OFDMsymbolsUE CombsizeN.

A comparative table of Rel16 SRS and multi-port SRS configurations is provided in Table C.3.1.6.2-3:

Table C.3.1.6.2-3 UL SRS resource utilization w.r.t to the number of antenna ports:

| Scenario | Rel-17 enhancements | # of Tx Beams | COMB | nbSym | OFDM symbols per UE | Capacity per UE | Improvement factor vs Rel16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1-Umi | 2-ports | 2 | 4 | 4 | 4 | 1 | 2 |
|  | Rel-16 | 2 | 4 | 4 | 8 | 2 |  |
| FR1-InF | 2-ports | 2 | 2 | 1 | 1 | 0,5 | 2 |
|  | Rel-16 | 2 | 2 | 1 | 2 | 1 |  |
| FR2-Umi | 4-ports | 4 | 8 | 8 | 8 | 2 | 4 |
|  | Rel-16 | 4 | 8 | 8 | 32 | 8 |  |
| FR2-InF (SCS=120kHz) | 4-ports | 4 | 4 | 4 | 4 | 1 | 4 |
|  | Rel-16 | 4 | 4 | 16 | 16 | 4 |  |

### C.3.2	UE efficiency analysis for NR positioning enhancements

#### C.3.2.1	Results from source [4]

##### C.3.2.1.1	Description of evaluation scenarios

The UE efficiency is evaluated based on UE power consumption. The following evaluation assumptions on UE power model besides what is already defined TR 38.840 are considered for PRS Rx and SRS Tx.

The both DL PRS Rx related and UL SRS Tx related power evaluation,

- The following C-DRX configuration are assumed

  - C-DRX cycle 160msec

  - 8 msec on-duration timer

- 100 msec inactivity timer

- The following I-DRX configuration are assumed

- I-DRX cycle 1.28msec

Table C.3.2.1.1-1 shows the UE power model for IDLE/INACTIVE state

Table C.3.2.1.1-1: UE power model for IDLE/INACTIVE state [4]

| Power State | Relative Power (FR1 reference from TR 38.840) | Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz) |
| --- | --- | --- |
| Deep Sleep (PDS) | 1 | 1 |
| Light Sleep (PLS) | 20 | 20 |
| Micro sleep (PMS) | 45 | 45 |
| PDCCH-only (PPDCCH) | 100 | 50Note |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 300 | 120 |
| PDSCH-only (PPDSCH) | 280 | 112 |
| SSB/CSI-RS proc. (PSSB) | 100 (synchronization or serving cell measurement) | 50 |
| Intra-frequency RRM measurement (Pintra) | · 150 (synchronous case, N=8, measurement only; Pintra, meas-only) · 200 (combined search and measurement; Pintra, search+meas) | · [60] (synchronous case, N=8, measurement only; Pintra, meas-only) · [80] (combined search and measurement; Pintra, search+meas) |
| Inter-frequency RRM measurement (Pinter) | · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer | · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer |
| Note: Power scaling to 20MHz reception bandwidth follows the rule in Section C.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}. |  |  |

For DL PRS Rx related power evaluation:

- Table C.3.2.1.1-2 shows the PRS Rx power model

- Figures C.3.2.1.1-1 and C.3.2.1.1-2 show the UE power state transit for IDLE/INACTIVE state and CONNECTED state, respectively

Table C.3.2.1.1-2: UE PRS Rx power model [4]

| N: Number of TRPs for intra-frequency measurement & search | Synchronous case |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| N=8 | 200 | 320 |

Figure C.3.2.1.1-1: UE power state transit for IDLE/INACTIVE state from [4]

Figure C.3.2.1.1-2: UE power state transit for CONNECTED state from [4]

For UL SRS Tx related power evaluation:

- Figure C.3.2.1.1-3 shows the slot-level UE behaviour for the purpose SRS Tx and Paging DRX

- Figures C.3.2.1.1-4 and C.3.2.1.1-5 show the UE power state transit for IDLE/INACTIVE state and CONNECTED state, respectively

Figure C.3.2.1.1-3: Slot-level behaviour for UE to transmit SRS in IDLE/INACTIVE state from [4]

Figure C.3.2.1.1-4: UE power state transit for IDLE/INACTIVE state from [4]

Figure C.3.2.1.1-5: UE power state transit for CONNECTED state from [4]

##### C.3.2.1.2	UE efficiency analysis of NR positioning enhancements

The UE power consumption for PRS Rx is shown in Table C.3.2.1.2-1.

Table C.3.2.1.2-1: UE PRS Rx power consumption [4]

| Case P1: IDLE/INACTIVE state in every 1.28s |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 50 | 2 | 100 | 0.0781% | 0.3789% |
| SSB RRM | 80 | 4 | 320 | 0.1563% | 1.2125% |
| Micro-sleep | 45 | 6 | 270 | 0.2344% | 1.0230% |
| PRS RRM | 200 | 96 | 19200 | 3.7500% | 72.7493% |
| Deep sleep | 1 | 2452 | 2452 | 95.7813% | 24.6363% |
| -- State transit | 450 | 9 | 4050 |  |  |
| Total |  | 2560 | 26392 | 100.0000% | 100.0000% |
| Case P2: CONNECTED state inside on-duration in every 1.28s |  |  |  |  |  |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 100 | 32 | 3200 | 1.2500% | 11.2549% |
| PRS RRM | 200 | 96 | 19200 | 3.7500% | 67.5295% |
| Deep sleep | 1 | 2432 | 2432 | 95.0000% | 21.2155% |
| -- State transit | 450 | 8 | 3600 |  |  |
| Total |  | 2560 | 28432 | 100.0000% | 100.0000% |
| Case P3: CONNECTED state outside on-duration in every 1.28s |  |  |  |  |  |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 100 | 128 | 12800 | 5.0000% | 33.7410% |
| PRS RRM | 200 | 96 | 19200 | 3.7500% | 50.6116% |
| Deep sleep | 1 | 2336 | 2336 | 91.2500% | 15.6474% |
| -- State transit | 450 | 8 | 3600 |  |  |
| Total |  | 2560 | 37936 | 100.0000% | 100.0000% |

The UE power consumption for SRS Tx is shown in Table C.3.2.1.2-2.

Table C.3.2.1.2-2: UE SRS Tx power consumption [4]

| Case P4: IDLE/INACTIVE state in every 1.28s |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 50 | 2 | 100 | 0.0781% | 0.8396% |
| SSB RRM | 80 | 32 | 2560 | 1.2500% | 21.4945% |
| Micro-sleep | 45 | 33 | 1485 | 1.2891% | 12.4685% |
| SRS Tx | 210 | 8 | 1680 | 0.3125% | 14.1058% |
| Deep sleep | 1 | 2485 | 2485 | 97.0703% | 51.0915% |
| -- State transit | 450 | 8 | 3600 |  |  |
| Total |  | 2560 | 11910 | 100.0000% | 100.0000% |
| Case P5: CONNECTED state outside on-duration in every 1.28s |  |  |  |  |  |
| Power state | Power unit | # slot | Power | Slot ratio | Power ratio |
| PDCCH-only | 100 | 120 | 12000 | 4.6875% | 60.8766% |
| SRS Tx | 210 | 8 | 1680 | 0.3125% | 8.5227% |
| Deep sleep | 1 | 2432 | 2432 | 95.0000% | 30.6006% |
| -- State transit | 450 | 8 | 3600 |  |  |
| Total |  | 2560 | 19712 | 100.0000% | 100.0000% |

#### C.3.2.2	Results from source [5]

##### C.3.2.2.1	Description of evaluation scenarios

Model and assumptions of UE efficiency for power consumption

General PRS measurements assumption

- Frequency range: FR1 has high priority

- Bandwidth: 100MHz/30kHz (connected state); 20MHz/30kHz (idle state)

- Periodicity: 160ms (connected state); 1280ms (idle state)

- RE mapping: Comb-6, 6 symbols

- 2 resources per TRP and 2 resources per slot

- Repetition factor=8, time gap=1

- 1 positioning frequency layer is baseline

- 8 TRPs per frequency layer

Slot-averaged  power  for single positioning frequency layer measurements

We list two tables for UE power consumption for measurement on a PRS slot in connected state and idle state, respectively.

Table C.3.2.2.1-1: UE power consumption for PRS measurements in connected state（100MHz）

| N: Number of TRPs for single frequency layer measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 610 | 690 |
| N=4 | 490 | 570 |

Table C.3.2.2.1-2: UE power consumption for PRS measurements in idle state（20MHz）

| N: Number of TRPs for single frequency layer measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 96 | 108 |
| N=4 | 76 | 88 |

In the above tables, all above values are slot-averaged power (PPRS ) in FR1. The synchronous case means actual PRS transmissions from TRPs are time-aligned, while the asynchronous case means actual PRS transmissions from TRPs are not time-aligned. The power values in the above tables are calculated and assumed based on the SSB power and scaling schemes in TR38.840.

Total power for single and multiple positioning frequency layers measurements

This model can refer to the model for inter-frequency RRM measurements in TR38.840 and make some minor changes.

A UE is expected to process one frequency layer at a time. Therefore, we propose that, for frequency layer i, the power of PRS measurement is represented as:

Ei=PPRS*NPRS+EMG_switch

where

-    Ei is total power over slots over which measurements are carried out in frequency layer i

-    PPRS is the slot average power for PRS measurements in frequency layer i

-    NPRS is the number of slot over which measurements are carried out

-    EMG_switch is the power for measurement gap switching, where EMG_switch= Pt * Tt

-	Pt is the switching power consumption

-	Assume micro sleep power for Pt which equals to 45 power unit

-	Tt is switching time (including switching on and off) for FR1 for measurement gap switching, which equals to 1ms (0.5ms*2)

-	If gap is not configured, gap switching power is 0

For Nf frequency layers, the total power is

ENf=i=0Nf-1Ei

It can be simplified to the following if Ei is the same across frequency layers (i.e. Ei = E for different frequency layers ).

ENf = E*Nf

Power components considered for PRS measurement

In idle state, at least the following power components are recommended to be considered for PRS measurement power saving evaluation.

- PRS occasion

- For simplicity, UE only performs intra-frequency PRS measurements every I-DRX cycle (1280ms)

- 4ms PRS length, 1PRS occasion

- 20MHz

- Relative power: 96

- Paging occasion

- 4ms length

- PDCCH only or PDCCH+PDSCH

- Relative power: 50 for PDCCH only; 120 for PDCCH+PDSCH

- Note1: for PDCCH only, no PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot;  for PDCCH+PDSCH, it is used when the UE decodes the PDCCH for paging and obtains the need to decode the corresponding PDSCH

- Note2: PDCCH only is assumed as baseline in idle state

- SSB burst for inter-frequency RRM measurement

- 5ms window after paging occasion, 1 SSB burst

- UE performs inter-frequency RRM every I-DRX cycle (1280ms)

- Relative power: 60

- SSB burst for serving/intra-frequency RRM measurement

- 2ms window before paging occasion, 1 SSB burst

- UE performs intra-frequency RRM every I-DRX cycle (1280ms)

- SSB burst period: 20ms

- Relative power: 60

- SSB burst for fine time-frequency sync. and RSRP measurement of serving/camping cell

- 2ms window before paging occasion

- 3 SSB bursts: 2 SSB burst is used standalone, 1 SSB burst reuse SSB burst for serving/intra-frequency RRM measurement

- Relative power: 50

- Sleep

- Sleep type is determined by duration between two power conponients above

- Deep sleep: duration>20ms

- Relative power: 1, transition energy: 450

- Light sleep: duration=6~20ms

- Relative power: 20, transition energy: 100

- Micro sleep: duration<6ms

- Relative power: 45, transition energy: 0

While in connected state, at least the following power components are recommended to be considered for PRS measurement power saving evaluation.

- PRS occasion

- For simplicity, UE only performs intra-frequency PRS measurements every C-DRX cycle (160ms)

- 4ms PRS length, 1PRS occasion

- 100MHz

- Relative power: 610

- Measurement gap for PRS measurement

- Length: 5ms

- Period: 160ms

- Total switching time: 1ms

- Relative power for switching: 45

- PDCCH only monitoring during active time

- For simplicity, only performs PDCCH decoding, traffic model is not introduced

- 4ms length

- Relative power: 100

- Note:  For PDCCH-only, no PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot.

- SSB burst for inter-frequency RRM measurement

- 5ms window after PDCCH monitoring, 1 SSB burst

- UE performs inter-frequency RRM every C-DRX cycle (160ms)

- Relative power: 150

- SSB burst for serving/intra-frequency RRM measurement

- 2ms window before PDCCH monitoring, 1 SSB burst

- UE performs intra-frequency RRM every C-DRX cycle (160ms)

- SSB burst period: 20ms

- Relative power: 150

- SSB burst for fine time-frequency sync. and RSRP measurement of serving/camping cell

- 2ms window before PDCCH monitoring

- 2 SSB bursts: 1 SSB burst is used standalone, 1 SSB burst reuse SSB burst for serving/intra-frequency RRM measurement

- Relative power: 100

- Sleep

##### C.3.2.2.2	UE efficiency analysis of NR positioning enhancements

UE efficiency analysis for power consumption

###### 1 - Baseline power consumption evaluation results

A - Power consumption for PRS measurement in idle state (baseline)

- A procedure of PRS measurement in idle state is shown in the figure below.

Figure C.3.2.2.2-1 Procedure of PRS measurements in idle state

Then we analyze the time of multiple power components and calculate the average energy consumption, the specific content can refer to the following Table.

Table C.3.2.2.2-1 Power components analysis for PRS measurement in idle state [5]

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Deep sleep | 1 | 1215 |
| Light sleep | 20 | 41.5 |
| Micro sleep | 45 | 4.5 |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 2+2 |
| Single positioning frequency layer measurement | 96 | 4 |
| Paging Occasion | 50 | 4 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 4 |
| Average power | 3.3605 |  |

B - Power consumption for PRS measurement in connected state (baseline)

- A procedure for PRS measurement in connected state is shown in Figure C.3.2.2.2-2 below. Based on the above assumptions and analysis, Table C.3.2.2.2-2 shows the power consumption for PRS measurement in connected state.

Figure C.3.2.2.2-2 Procedure of PRS measurements in connected state

Table C.3.2.2.2-2 Power components analysis for PRS measurement in connected state [5]

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Deep sleep | 1 | 115 |
| Light sleep | 20 | 23 |
| Micro sleep | 45 | 4 |
| SSB for Inter-frequency measurement | 150 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 |
| SSB Proc. | 100 | 2 |
| Single positioning frequency layer measurement | 610 | 4 |
| Gap switching | 45 | 1 |
| PDCCH-only monitoring | 100 | 4 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 3 |
| Average power | 35.2500 |  |

###### 2 - Power consumption evaluation for different PRS configurations

A - PRS measurement impacted by DRX

For PRS reception impacted by DRX configuration, we set the PRS period to be consistent with the DRX period which is equal to 160ms (the actual PRS period is 80ms and the DRX cycle is 160ms) as shown in Figure C.3.2.2.2-2(baseline), which assumes that the UE only measures PRS during DRX active time. While for PRS reception not impacted by DRX, we set the PRS period is 80ms and the DRX cycle is 160ms as shown in Figure C.3.2.2.2-3, which assumes that  the UE receives PRS during DRX active time and DRX non active time.

Figure C.3.2.2.2-3 Procedure of PRS measurements in DRX cycle (160ms) for PRS reception not impacted by DRX (1 DRX cycle with 2 PRS occasion to measure)

In addition, in Table C.3.2.2.2-3, we compared the UE power consumption of the two cases above.

Table C.3.2.2.2-3 Power components analysis for PRS measurement with/without DRX impact [5]

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | PRS reception impacted by DRX | PRS reception not impacted by DRX |
| Deep sleep | 1 | 115 | 110 |
| Light sleep | 20 | 23 | 23 |
| Micro sleep | 45 | 4 | 4 |
| SSB for Inter-frequency measurement | 150 | 5 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 | 2 |
| SSB Proc. | 100 | 2 | 2 |
| Single positioning frequency layer measurement | 610 | 4 | 8 |
| Gap switching | 45 | 1 | 2 |
| PDCCH-only monitoring | 100 | 4 | 4 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 1 | 2 |
| Light sleep transition | 100 | 3 | 3 |
| Calculation |  |  |  |
| DRX cycle | - | 160 | 160 |
| Average power | - | 35.2500 | 53.5625 |
| Power saving gain | - | 34.19% | 0 |

It can be observed that,

when PRS measurement is impacted by DRX (reception 1 PRS occasion every DRX cycle (160ms)), 34.19% power saving gain is shown, comparing with PRS measurement regardless of DRX(reception 2 PRS occasions every DRX cycle (160ms)) . If PRS reception is impacted by DRX, a great power saving gain will be obtained.

B -Extending PRS period

As we all know, increasing the period of PRS can reduce the power consumption of PRS measurement. Therefore, we extend PRS period 2 times (320ms) and 4 times (640ms). The procedure and power components analysis of PRS measurements are shown in Figure C.3.2.2.2-4 and Table C.3.2.2.2-4 respectively.

Figure C.3.2.2.2-4 Procedure of PRS measurements in DRX cycle with PRS and DRX cycle without PRS in connected state

Table C.3.2.2.2-4 Power components analysis for PRS measurement by extending PRS period in connected state [5]

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | PRS period(320ms) | PRS period(640ms) |
| Deep sleep | 1 | 230 | 460 |
| Light sleep | 20 | 55 | 119 |
| Micro sleep | 45 | 4 | 4 |
| SSB for Inter-frequency measurement | 150 | 10 | 20 |
| SSB for Intra-frequency measurement | 150 | 4 | 8 |
| SSB Proc. | 100 | 4 | 8 |
| Single positioning frequency layer measurement | 610 | 4 | 4 |
| Gap switching | 45 | 1 | 1 |
| PDCCH-only monitoring | 100 | 8 | 16 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 2 | 4 |
| Light sleep transition | 100 | 6 | 12 |
| Calculation |  |  |  |
| DRX cycle | - | 160 | 160 |
| Average power | - | 27.4844 | 23.6016 |
| Power saving gain | - | 22.03% | 33.05% |

It can be observed that,

By extending the PRS period to 2 times(160ms to 320ms), 22.03% power saving gain is shown ,comparing with the baseline assumption.

By extending the PRS period to 4 times(160ms to 640ms), 33.05 % power saving gain is shown ,comparing with the baseline assumption.

C - Concentrated PRS distribution

In this subsection, we compare two PRS distribution methods, namely concentrated PRS (baseline, as in Figure C.3.2.2.2-2) and distributed PRS (as in Figure C.3.2.2.2-5), which is based on our power consumption model, method and assumptions. The duration of concentrated PRS distribution is 5ms with 4ms PRS length and 1ms MG switching time. While for distributed PRS, we divide the concentrated PRS occasion of 4ms (baseline) into 4 PRS occasions with 1ms, and the adjacent PRS occasions are separated by 40ms. In this case, 4 measurement gap occasions with 2ms duration is assumed. Then we compare the power consumption performance of the 2 cases in one DRX cycle as shown in Table  C.3.2.2.2-5.

Figure C.3.2.2.2-5 Procedure of distributed PRS measurements in one DRX cycle

Table C.3.2.2.2-5 Power components analysis for concentrated and distributed PRS measurement [5]

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | Concentrated PRS measurement (baseline) | Distributed PRS measurement |
| Deep sleep | 1 | 115 | 103 |
| Light sleep | 20 | 23 | 36 |
| Micro sleep | 45 | 4 | 0 |
| SSB for Inter-frequency measurement | 150 | 5 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 | 2 |
| SSB Proc. | 100 | 2 | 2 |
| Single positioning frequency layer measurement | 610 | 4 | 4 |
| Gap switching | 45 | 1 | 4 |
| PDCCH-only monitoring | 100 | 4 | 4 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 1 | 3 |
| Light sleep transition | 100 | 3 | 5 |
| Calculation |  |  |  |
| DRX cycle | - | 160 | 160 |
| Average power | - | 35.2500 | 43.3937 |
| Power saving gain | - | 18.77% | 0 |

It can be seen that

when configuring concentrated PRS measurement (1 concentrated PRS occasion every 160ms), 18.77% power saving gain is shown, comparing with the distributed PRS measurement (4 distributed PRS occasion every 160ms).

Concentrated PRS measurement can mainly reduce the power consumption of deep sleep transition, which is the main part of power saving gain.

D - Adding PRS measurement window (PRS-MTC)

In Rel-15, SMTC has been defined. SSB-MTC is used to configure measurement timing configurations and includes duration which is the duration of the measurement window in which to receive SS/PBCH blocks. When SMTC is configured, the UE is not expected to measure SSB outside the SMTC window which reduces unnecessary power consumption. For PRS measurement, a similar window within a constrained duration (which can be called PRS-MTC) may be introduced in Rel-17 for UE power efficient positioning. PRS-MTC can limit the processing for measurement within a constrained time window as shown in Figure C.3.2.2.2-6. In this case, the UE is only required to measure PRS of 2ms length when configuring 2ms PRS MTC window.

Figure C.3.2.2.2-6  Procedure of PRS measurements with PRS MTC (2ms) in the DRX cycle

In this subsection, we compare power consumption between PRS measurement with 2ms PRS-MTC window and without PRS MTC window (baseline, as in Figure C.3.2.2.2-2). The corresponding power analysis is shown below in Table C.3.2.2.2-6.

Table C.3.2.2.2-6 Power components analysis for PRS measurement by adding PRS MTC window in connected state [5]

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | Without PRS-MTC(baseline) | PRS MTC(2ms) |
| Deep sleep 23 | 1 | 115 | 115 |
| Light sleep | 20 | 23 | 24 |
| Micro sleep | 45 | 4 | 5 |
| SSB for Inter-frequency measurement | 150 | 5 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 | 2 |
| SSB Proc. | 100 | 2 | 2 |
| Single positioning frequency layer measurement | 610 | 4 | 2 |
| Gap switching | 45 | 1 | 1 |
| PDCCH-only monitoring | 100 | 4 | 4 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 1 | 1 |
| Light sleep transition | 100 | 3 | 3 |
| Calculation |  |  |  |
| DRX cycle | - | 160 | 160 |
| Average power | - | 35.2500 | 28.0313 |
| Power saving gain | - | 0 | 20.48% |

It can be observed that,

by adding the PRS MTC window to limit PRS measurement in 2ms (from 4ms to 2ms), 20.48% power saving gain is shown, comparing with PRS measurement without PRS-MTC (the baseline assumption).

E - Reducing number of TRPs to be measured

When reducing the number of TRPs to be measured, the power component analysis is shown in Table C.3.2.2.2-7.

Table C.3.2.2.2-7 Power components analysis for PRS measurement for 4 TRPs in connected state [5]

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Deep sleep | 1 | 115 |
| Light sleep | 20 | 23 |
| Micro sleep | 45 | 4 |
| SSB for Inter-frequency measurement | 150 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 |
| SSB Proc. | 100 | 2 |
| Single positioning frequency layer measurement | 490 | 4 |
| Gap switching | 45 | 1 |
| PDCCH-only monitoring | 100 | 4 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 3 |
| Average power | 32.2500 |  |
| Power saving gain | 8.51% |  |

It can be observed that,

by reducing the number of TRPs for PRS measurement (from 8 TRPs to 4 TRPs),  8.51% power saving gain is shown ,comparing with the baseline assumption.

F - Reducing number of positioning frequency layers to be measured

To simplify the evaluation, we only list the power component analysis for a single frequency layer in the baseline. In this subsection, more analysis are shown for multiple positioning frequency layers as shown in Figure C.3.2.2.2-7, Figure C.3.2.2.2-8 and Table C.3.2.2.2-8.

Figure C.3.2.2.2-7 Procedure of PRS measurements of 2 frequency layers in the DRX cycle in connected state

Figure C.3.2.2.2-8 Procedure of PRS measurements of 4 frequency layers in the DRX cycle in connected state

Table C.3.2.2.2-8 Power components analysis for multiple frequency layers PRS measurement in connected state [5]

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | 2 frequency layers | 4 frequency layers |
| Deep sleep | 1 | 115 | 115 |
| Light sleep | 20 | 14 | 0 |
| Micro sleep | 45 | 8 | 12 |
| SSB for Inter-frequency measurement | 150 | 5 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 | 2 |
| SSB Proc. | 100 | 2 | 2 |
| Multiple positioning frequency layers measurement | 610 | 8 | 16 |
| Gap switching | 45 | 2 | 4 |
| PDCCH-only monitoring | 100 | 4 | 4 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 1 | 1 |
| Light sleep transition | 100 | 2 | 0 |
| Average power | - | 52.0313 | 82.4688 |
| Power consumption gain | - | 47.61% | 133.95% |

It can be observed that,

By increasing the number of frequency layers to 2 (from 1 to 2), 47.61% power consumption gain is shown, comparing with the baseline assumption.

By increasing the number of frequency layers to 4 (from 1 to 4), 133.95% power consumption gain is shown, comparing with the baseline assumption.

Correspondingly, if we take the case of 4 frequency layers as the baseline, then by reducing the number of frequency layer to 2 (from 4 to 2), 36.91% power saving gain is shown; by  reducing number of frequency layer to 1 (from 4 to 1), 57.26% power saving gain is shown.

###### 3 - Power consumption evaluation for idle states positioning

A - Idle state measurement and report

Figure C.3.2.2.2-9  Procedure of PRS measurements and report in idle state

In this subsection, we discuss power consumption for positioning measurement and report in idle states. For positioning report in idle state, let’s take the positioning information reported by EDT as an example. If the UE and NG-RAN node both support EDT, the UE sends an RRCEarlyDataRequest message to the NG-RAN node and includes a NAS control plane service request. The NAS control plane service request includes a positioning report message. From the perspective from physical layer, this message is carried in Msg3. Furthermore, the UE can receive RRCEarlyDataComplete containing positioning report completion information which is also included in Msg4.

The average power for the components introduced by positioning report in idle state is listed below in Table C.3.2.2.2-9.

Table C.3.2.2.2-9 Average power for the components introduced by positioning report in idle state [5]

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Coreset0+SIB1 | Equivalent to PDCCH + PDSCH. For idle state, scaling factor is assumed as 0.4 compared with connected states. | 120 |
| RAR | Equivalent to PDCCH + PDSCH. | 120 |
| Msg4 | Equivalent to PDCCH + PDSCH. Msg4 contains contention resolution information and RRCEarlyDataComplete information, doesn’t contain additional RRC information to establish RRC connection. | 120 |
| Msg3 | Equivalent to PUSCH. For idle state, scaling factor is assumed as 0.4. Considering positioning report carried in Msg3, the length of Msg3 is assumed as 4ms. | 280 |
| PRACH | Sequence length is 839. SCS is 1.25kHz. | 175 |
| Paging occasion | Equivalent to PDCCH + PDSCH. | 120 |

Therefore, the power components analysis for measurement and report in idle state is below in Table C.3.2.2.2-10. The power calculation cycle is 1280ms.

Table C.3.2.2.2-10 Power components analysis for positioning measurement and report in idle state [[5]]

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Deep sleep | 1 | 1177 |
| Light sleep | 20 | 57.5 |
| Micro sleep | 45 | 16.5 |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 6 |
| Single positioning frequency layer measurement | 96 | 4 |
| Paging Occasion | 120 | 4 |
| Coreset0+SIB1 | 120 | 1 |
| PRACH | 175 | 1 |
| RAR | 120 | 1 |
| Msg3(carrying positioning report) | 280 | 4 |
| Msg4 | 120 | 1 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 6 |
| Average power | 5.7488 |  |

B - Idle state measurement and connected state report

In this subsection, we discuss power consumption for positioning measurement in idle state and report in connected state. For simplicity, it is assumed that the only thing the UE does in the connected state is to report positioning information. The related procedure is shown in Figure C.3.2.2.2-10.

Figure C.3.2.2.2-10  Procedure of PRS measurements in idle state and report in connected state

The average power for the components introduced by positioning report in connected state and RRC state switching is listed below in Table C.3.2.2.2-11.

Table C.3.2.2.2-11 Average power for the components introduced by positioning report in connected state and RRC state switching [5]

| Power State |  | Characteristics | Relative Power |
| --- | --- | --- | --- |
| RRC state switching | Coreset0+SIB1 | Equivalent to PDCCH + PDSCH. For idle state, scaling factor is assumed as 0.4 compared with connected states. | 120 |
|  | RAR | Equivalent to PDCCH + PDSCH. | 120 |
|  | Msg4 | Equivalent to PDCCH + PDSCH. Msg4 contains contention resolution information and RRC setup information to establish RRC connection. | 120 |
|  | Msg3 | Equivalent to PUSCH. For idle state, scaling factor is assume as 0.4. | 280 |
|  | PRACH | Sequence length is 839. SCS is 1.25kHz. | 175 |
|  | Paging occasion | Equivalent to PDCCH + PDSCH. | 120 |
| Positioning report in connected state | SSB burst in connected state | SSB can be used for fine time-frequency sync. | 100 |
|  | Msg5 | Equivalent to PUSCH. For connected state, scaling factor is assume as 1. | 700 |
|  | Uplink grant | Equivalent to PDCCH only. | 100 |
|  | PUCCH SR | Equivalent to short PUCCH. Short PUCCH power = 0.3 x uplink power. | 210 |
|  | Pos report | Equivalent to PUSCH. | 700 |
|  | RRC release | Equivalent to PDSCH only. RRC connected state switches to idle state. | 280 |

Therefore, the power components analysis for measurement in idle state and report in connected states is below in Table C.3.2.2.2-12. The power calculation cycle is 1280ms.

Table C.3.2.2.2-12 Power components analysis for positioning measurement and report in idle state [5]

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1129 |
| Light sleep | 20 | 69.5 |
| Micro sleep | 45 | 35.5 |
| Idle states |  |  |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 8 |
| Single positioning frequency layer measurement | 96 | 4 |
| Paging Occasion | 120 | 4 |
| Coreset0+SIB1 | 120 | 1 |
| PRACH | 175 | 1 |
| RAR | 120 | 1 |
| Msg3 | 280 | 1 |
| Msg4 | 120 | 5 |
| Connected states |  |  |
| Uplink grant | 100 | 2 |
| SSB Proc. | 100 | 4 |
| Msg5 | 700 | 2 |
| PUCCH SR | 210 | 1 |
| Positioning report | 700 | 3 |
| RRC release | 280 | 2 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 8 |
| Average power | 10.3246 |  |

It can be observed that

compared to reporting in the idle state, the power consumption of reporting in the connected state increases 79.6%.

correspondingly, if we take the case of reporting in the connected state as the baseline, for reporting in the idle state , 44.32% power saving gain is shown.

C- Connected state measurement and report

In this subsection, we evaluate the case for connected states measurement and report. In order to be comparable with the above subsections, it is assumed that the UE still starts positioning from idle state. Furthermore, for simplicity, some additional assumptions are below.

- In connected state, LPP procedures including capability transfer, positioning assistant data transfer, some request signalings between the UE and the LMF are ignored.

- One shot positioning measurement and report related procedures are considered in connected state.

- Procedures related to measurement gap configuration are ignored.

- Once the positioning report is completed, the RRC connection is released.

- Measurement and report cycle is equal to idle state (1280ms).

- PRS bandwidth in connected state is equal to PRS bandwidth in idle state.

- Paging occasion power is equivalent to ‘PDCCH+PDSCH’, considering it may lead to RRC state transition.

Therefore, related procedure is shown in  Figure C.3.2.2.2-11.

Figure C.3.2.2.2-11  Procedure of PRS measurements and report in connected state

The average power for the components introduced by positioning measurement/report in connected state and RRC state switching is listed below in Table C.3.2.2.2-13.

Table C.3.2.2.2-13 Average power for the components introduced by positioning measurement/report in connected state and RRC state switching [5]

| Power State |  | Characteristics | Relative Power |
| --- | --- | --- | --- |
| RRC state switching | Coreset0+SIB1 | Equivalent to PDCCH + PDSCH. For idle state, scaling factor is assumed as 0.4 compared with connected states. | 120 |
|  | RAR | Equivalent to PDCCH + PDSCH. | 120 |
|  | Msg4 | Equivalent to PDCCH + PDSCH. Msg4 contains contention resolution information and RRC setup information to establish RRC connection. | 120 |
|  | Msg3 | Equivalent to PUSCH. For idle state, scaling factor is assume as 0.4. | 280 |
|  | PRACH | Sequence length is 839. SCS is 1.25kHz. | 175 |
|  | Paging occasion | Equivalent to PDCCH + PDSCH. | 120 |
| Positioning measurement and report in connected state | SSB burst in connected state | SSB can be used for fine time-frequency sync. | 100 |
|  | Msg5 | Equivalent to PUSCH. For connected state, scaling factor is assume as 1. | 700 |
|  | Uplink grant | Equivalent to PDCCH only. | 100 |
|  | PUCCH SR | Equivalent to short PUCCH. Short PUCCH power = 0.3 x uplink power. | 210 |
|  | Pos report | Equivalent to PUSCH. | 700 |
|  | RRC release | Equivalent to PDSCH only. RRC connected state switches to idle state. | 280 |
|  | PRS measurement | PRS bandwidth is equal to 20M. | 240 |

Therefore, the power components analysis for measurement and report in connected states is below in Table C.3.2.2.2-14. The power calculation cycle is 1280ms.

Table C.3.2.2.2-14 Power components analysis for positioning measurement and report in connected state [5]

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1115 |
| Light sleep | 20 | 84 |
| Micro sleep | 45 | 32 |
| Idle states |  |  |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 8 |
| Paging Occasion | 120 | 4 |
| Coreset0+SIB1 | 120 | 1 |
| PRACH | 175 | 1 |
| RAR | 120 | 1 |
| Msg3 | 280 | 1 |
| Msg4 | 120 | 5 |
| Connected states |  |  |
| Single positioning frequency layer measurement | 240 | 4 |
| Gap switching | 45 | 1 |
| Uplink grant | 100 | 2 |
| SSB Proc. | 100 | 6 |
| Msg5 | 700 | 2 |
| PUCCH SR | 210 | 1 |
| Positioning report | 700 | 3 |
| RRC release | 280 | 2 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 9 |
| Average power | 11.1367 |  |

It can be observed that

compared to reporting in the idle state, the power consumption of reporting in the connected state increases 93.72%.

correspondingly, if we take the case of reporting in the connected state as the baseline, for reporting in the idle state, 48.38% power saving gain is shown.

Furthermore, considering that we make some assumptions to simplify power consumption evaluation, such as ignoring complicated steps for LPP procedures, aligning the bandwidth and period with idle state measurement, it will consume more power when positioning in the connected state in general.

UE efficiency analysis for resource utilization

For Rel-16 UE efficiency, it is defined as the percentage of the MGL in the MG periodicity considering the UE is expected to process DL PRS within measurement gap, and the UE is not expected to conduct reception/transmission data from/to all carriers at least for measurement gap duration time. It can be represented as below equation (3)

UE efficiency=MGLMG periodicity×100%                                          (3)

Whether the MG is suitable or not for PRS measurement will directly affect to UE efficiency. e.g. 12ms length for PRS and 20ms length for MG, extra MG length (e.g. 8ms) will cause the wasting of UE efficiency. Meanwhile, the distribution of PRS also affects to UE efficiency. For example, 20ms MG is needed even the duration of PRS symbol only 2ms but the buffer window of PRS is 20ms because of the distributed PRS. So, on-demand MG with concentrated PRS is beneficial for reducing UE efficiency and the evaluation results are listed in case U1.

Furthermore, the UE efficiency only is affected by the PRS that UE needs to measure if positioning BWP is introduced and the UE efficiency will be improved. It can be represented as below equation (4) and the evaluation results are listed in case U2.

UE efficiency=the duration K ms of DL PRS symbolsP ms window of unit time×100%                                          (4)

Table C.3.2.2.2-15 UE efficiency analysis for periodic MG and PRS VS on demand MG with concentrated PRS [5]

| [Case U1], [IIoT/ Commercial], [FR2], [Periodic MG and PRS VS On demand MG with concentrated PRS] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Option | Periodic MG and PRS |  | On-demand MG with concentrated PRS |
| Configuration | PRS Periodicity: 10ms MG periodicity: 20ms MGL:20ms | PRS Periodicity :160ms MG periodicity: 160ms MGL:20ms | Period 10ms, repetition :16 MGL for minimum TRP: 4ms The num of MG is 0-16 |
| PHY Latency (minimum) | Latencymin=62ms |  | Latencymin=46ms Where MGL and process time for minimum TRP (such as 4ms). |
| PHY Latency (maximum) | Latencymax=64ms | Latencymax=204ms | Latencymax=64ms |
| UE efficiency | MGLMG periodicity =100% | 12.5% | i=015Boolean(Yi>0)×MGL160 =0~16×4/160 =0%~40% |

Table C.3.2.2.2-16 UE efficiency analysis for periodic MG and PRS VS positioning BWP [5]

| [Case U2], [IIoT/ Commercial], [FR2], [Periodic MG and PRS VS Positioning BWP] Source [UE, NW] / Destination [UE, NW] Positioning technique [DL-TDOA,AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Option | Periodic PRS |  | Positioning BWP Without MG |  |
| Configuration | PRS Periodicity: 10ms MG periodicity: 20ms MGL:20ms | PRS Periodicity:160ms MG periodicity: 160ms MGL:20ms | PRS Periodicity: 10ms The duration k ms of PRS for minimum TRP: 3ms | PRS Periodicity: 160ms The duration k ms of PRS for minimum TRP: 3ms |
| PHY Latency (minimum) | Latencymin=62ms |  | Latencymin=25.5ms Where DL measurement &process delay equals k ms (such as 3ms). |  |
| PHY Latency (maximum) | Latencymax=64ms | Latencymax=204ms | Latencymax=32.5ms Where DL measurement &process delay equals periodicity 10ms | Latencymax=182.5ms Where DL measurement &process delay equals periodicity 160ms |
| UE efficiency | MGLMG periodicity =100% | 12.5% | K ms of DL PRS symbolsP ms window of periodicity =3/10=30% | K ms of DL PRS symbolsP ms window of periodicity =3/160=1.88% |

Note, the total number of NLOS links in the baseline case for this scenario is only 4 and the number of LOS is 2592.