C.1.2	Physical layer latency analysis for Rel-16

C.1.2.1	Results from source [4]

C.1.2.1.1	Description of evaluation scenarios

The physical layer latency for the following positioning methods are provided

- UE-A DL-only positioning

- UL-only positioning

- DL E-CID

- UL E-CID

- UE-B DL-only positioning

C.1.2.1.2	Latency analysis of Rel.16 solutions

Latency analysis for the Rel.16 NR positioning is provided in Tables C.1.2.1.2-1 to C.1.2.1.2-6.

The latency for multi-RTT positioning should be the maximum between DL-TDOA/DL-AoD and UL-TDOA/UL-AoA, which is the same as DL-TDOA/DL-AoD considering UL delay is much smaller than DL.

Table C.1.2.1.2-1: Rel.16 NR positioning latency [4]

| Case L1, DL-TDOA/DL-AoD/Multi-RTT w/ Gap request and PRS periodicity 20ms Source NW/Destination NW Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-A Initial and Final RRC States CONNECTED Gap is requested, and PRS periodicity is 20ms This can also be used for Multi-RTT positioning. |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for gap request |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Decoding measurement gap request |
| gNB Tx higher layer processing | 3ms | Reconfiguration with a measurement gap of MGRP = 20ms, MGL = 3ms (MG pattern 10) |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| PRS measurement delay | 23 (1 samp.) 83ms (4 samp. CSSF = 1) 143 (4 samp. CSSF = 2) | According to RAN4, assuming PRS periodicity being 20ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 Ti = 20ms as the UE capability T Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5ms-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) |  |

Table C.1.2.1.2-2: Rel.16 NR positioning latency [4]

| Case L2, DL-TDOA/DL-AoD/Multi-RTT w/o Gap request and PRS periodicity 160ms Source NW/Destination NW Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-A Initial and Final RRC States CONNECTED Gap is not requested, and PRS periodicity is 160ms This can also be used for Multi-RTT positioning. |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| PRS measurement delay | 163ms (1 samp.) 643ms (4 samp. CSSF = 1) | According to RAN4, assuming PRS periodicity being 160ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 Ti = 20ms as the UE capability T Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) |  |

Table C.1.2.1.2-3: Rel.16 NR positioning latency [4]

| Case L3, UL-TDOA/UL-AoA Source NW/Destination NW Positioning technique UL-TDOA and/or UL-AoA, type UL Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| SRS measurement delay | 0.5-20ms (1 samp.) 60.5-80ms (4 samp.) | Assuming SRS periodicity is 20ms. Using 1 or 4 samples. |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6.5-26ms (1 samp.) 66.5-86.5ms (4 samp) |  |

Table C.1.2.1.2-4: Rel.16 NR positioning latency [4]

| Case L4, DL E-CID Source NW/Destination NW Positioning technique DL E-CID, type DL, mode UE-A Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| DL E-CID measurement | 0ms | Using only existing RRM measurement |
| UE Tx higher layer processing | 3ms | UL LPP message preparation |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 8.5-15ms |  |

Table C.1.2.1.2-5: Rel.16 NR positioning latency [4]

| Case L5, UL E-CID Source NW/Destination NW Positioning technique UL-E-CID, type DL+UL, mode UE-A Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| gNB Rx higher layer processing | 3ms | Processing NRPPa message from the NGAP message |
| AoA measurement delay | 0-20ms | Assuming SRS periodicity is 20ms. Either AoA is not measured or AoA is measured by 1 sample. |
| gNB Tx higher layer processing | 3ms | Processing NRPPa message into the NGAP message |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 6-26ms |  |

Table C.1.2.1.2-6: Rel.16 NR positioning latency [4]

| Case L6, UE-based DL-TDOA/DL-AoD w/ gap request t and PRS periodicity 20ms Source UE/Destination UE Positioning technique DL-TDOA and/or DL-AoD, type DL, mode UE-B Initial and Final RRC States CONNECTED MO-LR, i.e. UE as the location consumer |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data |
| UE Rx higher layer processing | 5ms | Handling of the DL-SCH payload to upper layers, including ASN.1 decoding of the LPP message |
| UE Tx higher layer processing | 3ms | UL RRC message preparation for gap request |
| PUSCH scheduling | 0.5-7.5ms | Considering TDD configuration DDDSU, using U to transmit PUSCH, the maximum delay could be SR BSR PUSCH using 3 cycles. The minimum delay could be a slot. |
| gNB Rx higher layer processing | 3ms | Decoding measurement gap request |
| gNB Tx higher layer processing | 3ms | Reconfiguration with a measurement gap of MGRP = 20ms, MGL = 3ms (MG pattern 10) |
| PDSCH scheduling | 0.5-1ms | Considering TDD configuration DDDSU, using D or S to transmit PDSCH. |
| UE Rx higher layer processing | 10ms | RRC reconfiguration delay in TS 38.331 |
| PRS measurement delay | 23 (1 samp.) | According to RAN4, assuming PRS periodicity being 20ms and single frequency layer processing TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast , Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i Assuming NRxBeam, i=1, i.e. the number of Rx beams is 1 at UE NPRS,islotN'LPRS,iN=1, i.e. UE can process all PRS in a slot and UE can buffer all PRS within a P-ms window Nsample = 1 or 4, i.e., the measurement average time is 1 or 4 Ti = 20ms as the UE capability T Tavailable_PRS, i=20ms, i.e., the LCM of PRS periodicity and MGRP is 20ms |
| Location calculation | 3ms | UE |
| End trigger |  | Calculation of Location Estimate at the UE |
| Total values | 51-58.5ms (1 samp.) |  |

C.1.2.2	Results from source [7]

C.1.2.2.1	Description of evaluation scenarios

In this contribution, the following positioning methods are included in physical layer latency analysis,

- DL-TDOA, UE-A

- DL-TDOA, UE-B

- DL-ECID

C.1.2.2.2	Latency analysis for Rel.16 solutions

Table C.1.2.2.2-1 physical layer latency analysis for UE-assisted positioning based on DL-TDOA method

| [Case PHY-L1] Source [UE]/Destination [NW] Positioning technique [DL-TDOA],type [DL], mode [UE-A] Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Components | Value Range (ms) | Description of Latency Component |
| Start trigger (step 1) | N/A | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE decodes and applies the Location Request (step 2) | 10 | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| Measurement gap request (step 3) | Corresponding values can be found in right column. | UL user plane latency defined in 37.910 clause 5.7.1.2.2 is applicable to measurement gap request, where the latency includes UE processing delay UL frame alignment (transmission alignment) TTI for UL data packet transmission BS processing delay The grant free based UL transmission is assumed in this contribution, some typical cases can be found in 37.910 clause 5.7.1.2.2. We choose following cases as examples, UL user plane latency for NR FDD with grant free transmission: UL user plane latency (Grant free)(ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 0.43; 0.30. UL user plane latency for NR TDD with grant free transmission: UL user plane latency (Grant free,DDDXU, with ‘X’=’U’ for UL traffic)(ms) – NR TDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 1.09; 0.64. |
| Serving gNB decodes and interprets the measurement gap request (step 4) | 3 | Regarding the RRC processing time at the gNB, 37.910 clause 5.7.1.1.1 defines processing delay in gNB for L2 and RRC. |
| Measurement gap configuration(step 5) | Corresponding values can be found in right column. | DL user plane latency defined in 37.910 clause 5.7.1.2.1 is applicable to measurement gap configuration, where the latency includes BS processing delay DL Frame alignment (transmission alignment) TTI for DL data packet transmission UE processing delay The latency may be different according to various configurations,some typical cases can be found in in 37.910 clause 5.7.1.2.2. We choose following cases as examples, DL user plane latency for NR FDD: DL user plane latency (ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=4 (7OS non-slot); p=0; 0.37; 0.27. DL user plane latency for NR TDD: DL user plane latency (ms – NR TDD (DDDXU,’X’=’D’ for DL traffic); UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=4 (7OS non-slot); p=0; 0.45; 0.32. |
| UE interprets and applies the measurement gap configuration(step 6) | 10 | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| UE positioning measurement(step 7) | 82 ( FR1) or 644 ( FR2) | Measurement period requirements defined in 38.133 clause 9.9.2.5 should be satisfied, more detailed configurations can refer to Table C.1.2.2.2-2. |
| UE positioning measurement transmission(step 8) | Corresponding values can be found in right column. | The same latency is assumed as “Measurement gap request”. |
| End trigger (step 9) | N/A | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | Corresponding values can be found in right column. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.23. FDD and 60KHz SCS; 667.87. TDD and 30KHz SCS; 107.63. TDD and 60KHz SCS; 668.60. |

Table C.1.2.2.2-2 Measurement period requirements for DL-TDOA method

| Parameters | FR1 | FR2 | Additional Explanations |
| --- | --- | --- | --- |
| MGRP ( Measurement Gap Repetition Period ) | 20 ms | 20 ms |  |
| MGL ( Measurement Gap Length ) | 4 ms | 4 ms | PRS is configured in a compact way, we assume the MGL can cover all PRS occasions from different TRPs. |
| TPRS ( DL PRS periodicity ) | 4 ms | 4 ms | All PRS resource sets have the same periodicity. |
| (N,T) ( Duration of DL PRS symbols N in units of ms a UE can process every T ms ) | (4, 20) ms | (4, 20) ms | The DL PRS buffering time fully aligns the MGL. |
| L ( Number of positioning frequency layer ) | 1 | 1 |  |
| Number of TRP | 4 | 4 |  |
| SCS ( Sub-carrier Spacing ) | 30 KHz | 60 KHz | MGL will span 8 slots for FR1, while the value is 16 slots for FR2. |
| LPRS ( the duration of DL PRS symbols within any a window ) | 2 ms | 4 ms | Type II DL PRS buffering capability is assumed. |
| ( carrier-specific scaling factor ) | 1 | 1 |  |
| ( UE Rx beam sweeping factor ) | 1 | 8 |  |
| ( Number of PRS RSTD samples ) | 4 | 4 |  |
| Total measurement period | 82 ms | 644 ms |  |
| Other assumptions | The number of PRS resources in a slot as configured by the assistance data is smaller the UE capability of max number of DL PRS resources that UE can process in a slot. UE is not required to measure PRS resources from multiple TRPs on the same OFDM symbol in FR2. |  |  |

Table C.1.2.2.2-3 physical layer latency for UE-based positioning based on DL-TDOA method

| [Case PHY-L2] Source [UE]/Destination [NW] Positioning technique [DL-TDOA],type [DL], mode [UE-B] Initial and Final RRC States [CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Start trigger | End trigger | Components that are different from UE-assisted positioning in Table C.1.2.2.2-1. | Total values (ms) |
| Transmission of the PDSCH from the gNB carrying the LPP Request Location Information | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message | Update the start and end triggers accordingly. Step 8 should be removed since UE is the location consumer, which is replaced by “ UE location calculation” ( assume the latency is [1 slot ]) All other steps are the same as UE-assisted positioning in Table C.1.2.2.2-1. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.73. FDD and 60KHz SCS; 668.02. TDD and 30KHz SCS; 108.13. TDD and 60KHz SCS; 668.85. |
| Transmission of the PDSCH from the gNB carrying the LPP Request Location Information | Calculation of Location Estimate at the UE | Update the start and end triggers accordingly. Step 8 should be removed since UE is the location consumer, which is replaced by “ UE location calculation” ( assume the latency is [1 slot ]) All other steps are the same as UE-assisted positioning in Table C.1.2.2.2-1. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.30. FDD and 60KHz SCS; 667.82. TDD and 30KHz SCS; 107.08. TDD and 60KHz SCS; 668.51. |
| Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message | Update the start and end triggers accordingly. Step 2 is replaced by “UE decodes and applies the LPP message containing the assistance data”. Another step for “ UE location calculation” ( assume the latency is [1 slot ] ) is added between step 7 and step 8. All other steps are the same as UE-assisted positioning in Table C.1.2.2.2-1. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.73. FDD and 60KHz SCS; 668.02. TDD and 30KHz SCS; 108.13. TDD and 60KHz SCS; 668.85. |
| Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data | Calculation of Location Estimate at the UE | Update the start and end triggers accordingly. Step 2 is replaced by “UE decodes and applies the LPP message containing the assistance data”. Step 8 should be removed since UE is the location consumer, which is replaced by “ UE location calculation” ( assume the latency is [1 slot ] ) All other steps are the same as UE-assisted positioning in Table C.1.2.2.2-1. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 106.30. FDD and 60KHz SCS; 667.82. TDD and 30KHz SCS; 107.08. TDD and 60KHz SCS; 668.51. |

Table C.1.2.2.2-3 physical layer latency for UE-based positioning based on DL-ECID method

| [Case PHY-L3] Source [UE]/Destination [NW] Positioning technique [DL-ECID],type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | N/A | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| UE interprets and applies the measurement configuration | 10 | RRC Processing time at the UE are captured in 38.331 Section 12, where defines the following RRC procedure delay, reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message |
| UE ECID measurement time | 0 | If RRM measurement is available (i.e. buffered) at UE side, UE can forward the ECID measurement directly without consuming additional time. |
| UE positioning measurement transmission | Corresponding values can be found in right column. | UL user plane latency defined in 37.910 clause 5.7.1.2.2 is applicable to measurement gap request, where the latency includes UE processing delay UL frame alignment (transmission alignment) TTI for UL data packet transmission BS processing delay The grant free based UL transmission is assumed in this contribution, some typical cases can be found in 37.910 clause 5.7.1.2.2. We choose following cases as examples, UL user plane latency for NR FDD with grant free transmission: UL user plane latency (Grant free)(ms) – NR FDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 0.43; 0.30. UL user plane latency for NR TDD with grant free transmission: UL user plane latency (Grant free,DDDXU, with ‘X’=’U’ for UL traffic)(ms) – NR TDD; UE capability 2. SCS. 30 KHz; 60 KHz. Resource mapping Type B; M=7 (7OS non-slot); p=0; 1.09; 0.64. |
| End trigger | N/A | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | Corresponding values can be found in right column. | FDD/TDD, SCS; Total values. FDD and 30KHz SCS; 10.43 ms. FDD and 60KHz SCS; 10.30 ms. TDD and 30KHz SCS; 11.09 ms. TDD and 60KHz SCS; 10.64 ms. |

C.1.2.3	Results from source [8]

C.1.2.3.1	Description of evaluation scenarios

C.1.2.3.2	Latency analysis for Rel.16 solutions

In this section, we analyse the physical layer latency for both DL-TDOA and UL-TDOA. For DL-TDOA, the physical layer procedure is shown in Figure C.1.2.3.2-1.

Figure C.1.2.3.2-1: PHY procedure for DL-TDOA

According to Figure C.1.2.3.2-1, the total PHY delay could be calculated as

Ttotal=Tgap,req+Tgap,cfg+Tariv+Tprocess+Treport

where,

- Tgap,req denotes the time to require measurement gap, which is 1ms according to the result of URLLC latency in TR 38.824.

- Tgap,cfg denotes the time for measurement gap configuration, which could be assumed as 10ms according to RRC reconfiguration procedure delay in TS 38.331.

- Tariv denotes the delay between the time when DL PRS is received and the time when measurement gap configuration is received, which is related to the maximum value of the configured periodicity of PRS and measurement gap.

- Tprocess denotes the time from UE begins to measure PRS until the measurement result is ready to report. Tprocessis related to UE capability and the number of PRS resources needed to be measured.

- Treport denotes the time for measurement reporting, which includes SR request, PDCCH-based UL grant and PUSCH-based measurement result reporting.

The value of Treport could be assumed as 0.5ms for URLLC according to section 5.2.1 of TR 36.881. The value of Tprocess may be inferred from the minimum periodicity of PRS and measurement gap, since the measurement result has to be reported before the next PRS occasion.

In Rel-16, the periodicity of PRS includes the following values: 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots. For the periodicity of measurement gap, the following values are supported: 20, 40, 80, 160, [320, 640]ms.

For example, assume SCS is 15kHz, the periodicity of PRS is 4 slots and the periodicity of measurement gap is 20ms, the PHY latency could be summarized in the following table.

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning (DL-TDOA) is provided in Table C.1.2.3.2-1.

Table C.1.2.3.2-1: Rel.16 NR positioning latency for DL-TDOA [8]

| Case 1, 15kHz, FR1, DL-TDOA Source UE/Destination NW Positioning technique DL-TDOA, type DL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger | 1 | Measurement gap request |
| Measurement gap configuration | 10 | Measurement gap configuration delay by RRC |
| PRS arrival delay | ≤X1, X1=max⁡{TcPRS,Tcmgap} (e.g. X1=4 when TcPRS=4 and Tcmgap=20) | DL PRS arrived for measurement, which is related to the maximum value of the configured periodicity of PRS TcPRS and the configured periodicity of measurement gap Tcmgap |
| Process time | ≤X2, X2=max⁡{min⁡(TPRS),min⁡(Tmgap)} (X2=20, as min⁡(TPRS)=4 and min⁡(Tmgap)=20) | UE begins to measure PRS until the measurement result is ready. This value depends on UE capability the number of PRS resources to be measured. The value is assumed to be determined by the supported minimum periodicity of PRS TPRS and measurement gap Tmgap. |
| End trigger | 0.5 | SR request, PDCCH UL grant and PUSCH-based measurement result reporting are included, wherein URLLC is assumed |
| Total values | ≤51.5 |  |

For UL-TDOA, the physical layer procedure is shown in Figure C.1.2.3.2-2.

Figure C.1.2.3.2-2: PHY procedure for UL-TDOA

The total delay could be calculated as

Ttotal=TSRS,act+Tprep+Tprocess

where,

- TSRS,act denotes the time to activate the SRS transmission.

- Tprep denotes the delay from effective time of SRS activation until UE begins to transmit SRS, which is related to the value of the configured periodicity of SRS.

- Tprocess denotes the time from gNB begins to measure SRS until the measurement result is ready, which is related to UE capability and the number of SRS resources needed to be measured.

If SRS is activated by MAC-CE, 3ms delay could be assumed. Similar to Case 1, the value of Tprocess may be inferred from the minimum periodicity of SRS. In Rel-16, the periodicity of SRS for 15 kHz SCS includes the following values:

- {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots. Assume the periodicity of SRS is 1 slot, the PHY latency could be get.

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning (UL-TDOA) is provided in Table C.1.2.3.2-2.

Table C.1.2.3.2-2: Rel.16 NR positioning latency for UL-TDOA [8]

| Case 2, 15kHz, FR1, UL-TDOA Source UE/Destination NW Positioning technique UL-TDOA, type UL, mode UE-assisted, Initial and Final RRC States CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger | 3 | UE SRS transmission is activated |
| UL SRS arrival delay | ≤X1, X1=TcSRS (e.g. X1=1 when TcSRS=1) | UL SRS arrived for measurement, which is related to the value of the configured periodicity of SRS TcSRS |
| End trigger | ≤X2, X2=min⁡(TSRS) (X2=1, as min⁡(TSRS)=1) | gNB begins to measure SRS until the measurement result is ready. This value depends on gNB capability, and the number of SRS resources to be measured. The value is assumed to be determined by the minimum periodicity of SRS, TSRS. |
| Total values | ≤5 |  |

C.1.2.4	Results from source [13]

C.1.2.4.1	Description of evaluation scenarios

We analyse the case of Network Initiated location request where the positioning method is DL-TDOA/DL-AoD in Case 1/2 and also for UL-TDOA/UL-AoA in Case 3. We only consider the RRC state as connected mode. Note that this analysis assumes that the higher layer components are taken into account in separate analysis and the definition of start and stop times for PHY layer latency are used. For Case 3 we assume that SRS-Pos already being configured (e.g., higher layer latency takes this into account, see RAN2 analysis).

C.1.2.4.2	Latency analysis for Rel.16 solutions

| Case 1 (FR1)/ Case 2 (FR2) Source: NW/Destination: NW Positioning technique: DL-TDOA, type: DL, mode: UE-A Initial and Final RRC States: Initial/Final - CONNECTED |  |  |  |
| --- | --- | --- | --- |
|  | Latency Component | Value Range | Description of Latency Component |
| Start trigger |  |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| Step 1: Processing of “LPP Request Location Information message” | 1a. Processing of PDSCH | FR1: [3-8 symbols] FR2: [20-24 symbols] | UE processing – based on N1 values from 38.214 |
|  | 1b. Processing of LPP message | [10 ms] | From 38.331 |
| Step 2 (optional): measurement gap request and configuration | 2a. SR (for MG request) preparation and transmission | FR1: [2 symbols – 80 slots] FR2: [2 symbols – 640 slots] | generate SR, as the message of measurement gap request may be new data coming Wait for SR transmission occasion and transmit SR to gNB |
|  | 2b. SR processing at gNB | [1 symbol] | SR decoding, should be much smaller than N2. |
|  | 2c. UL grant (for MG request) preparation and transmission | [3 symbols – 1 slot ] | UL grant generation and waiting for scheduling opportunity (based on CORESET configuration). Typical value. |
|  | 2d. UE processing of PDCCH and UL preparation | FR1: [5-10 symbols] FR2: 36 symbols | UL data includes requested measurement gap request. Based on N2 values from 38.214. |
|  | 2e. Scheduling delay - K2 | [0-32 slots] | Scheduling delay from UL grant to PUSCH |
|  | 2f. PUSCH reception and decoding | [1-2 symbols] | Including 1. PUSCH (containing MG request information) processing at gNB |
|  | 2g. RRC processing of MG request | [10 ms] | gNB RRC processing time |
| Step 3 (optional): measurement gap configuration | 3a. gNB prepares to transmits RRC (for MG configuraiton) | [3 symbols – 1 slots] | PDCCH and PDSCH preparation |
|  | 3b. UE PHY processing for RRC (for MG configuraiton) | FR1: [3-8 symbols] FR2: [20-24 symbols] | PDSCH decoding; not specified, but should be smaller than N1 |
|  | 3c. UE time to applie the MG configuration | [10 ms] | Based on 38.331 |
| Step 4: PRS processing | 4a. Transmission of DL PRS | [4-10240 slots] | Worst case time between periodic DL PRS based on values from 38.211 |
|  | 4b. Processing of DL PRS at UE | [8-1280 ms] | Assume average value of T=8 (lowest value) is reported by UE as capability and UE is able to process all DL PRS resources (i.e., N is sufficient). |
| Step 5: measurement report processing | 5a. Transmit SR | FR1: [2 symbols – 80 slots] FR2: [2 symbols – 640 slots] | Wait for SR transmission occasion and transmit SR to gNB |
|  | 5b. Processing of SR at gNB | [1 symbol] | SR decoding |
|  | 5c. UL grant preparation and transmission of UL grant by gNB | [3 symbols – 1 slot ] | UL grant generation and waiting for scheduling opportunity (based on CORESET configuration). Typical value. |
|  | 5d. UE processing of PDCCH and UL preparation | FR1: [5-10 symbols] FR2: 36 symbols | Based on N2 values from 38.214. |
|  | 5e. Scheduling delay - K2 | [0-32 slots] | Scheduling delay from UL grant to PUSCH |
|  | 5f. gNB processing of RSTD Report | [1-2 symbol] |  |
| End trigger |  |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total PHY latency |  | FR1: [44.35 – 10500] ms FR2: [35.08 – 2118.93 ms] | Assuming 15 kHz SCS for FR1 and 120 kHz SCS for FR2 |

| Case 3 Source: NW/Destination: NW Positioning technique: UL-TDOA, type: UL, mode: UE-A Initial and Final RRC States: Initial/Final - CONNECTED |  |  |  |
| --- | --- | --- | --- |
|  | Latency Component | Value Range | Description of Latency Component |
| Start trigger |  |  | Reception by the gNB of the NRPPa measurement request message (assumption is that UE has already been configured with periodic SRS-Pos) |
| Step 1: SRS-pos configuration | 1a. gNB prepares and transmits RRC (for SRS-pos configuraiton) | [3 symbols – 1 slot ] | PDCCH and PDSCH preparation |
|  | 1b. UE PHY processing for RRC | [3-24 symbols] | PDSCH decoding |
| Step 2: SRS-pos processing | 2a. Wait for transmission of SRS-Pos | [1-81920 slots] | Periodicity of SRS-Pos |
|  | 2b. Transmission of SRS-Pos | [2 symbols- 8 symbols] | Assuming effective comb-1. In FR2 case this will be much longer (e.g., multiple resources per set) |
|  | 2c. Processing of SRS-Pos at gNB/RP-only | [1-5 slots] |  |
| Step 3: positioning report | 3a. Preparation of UL measurement report | [3 symbols ] | Typical value |
| End trigger |  |  | The transmission by the gNB of the NRPPa measurement response message |
| Total PHY latency |  | [2.78 – 81928.5] ms | Assuming 15 kHz SCS |

C.1.2.5 Results from source [11]

C.1.2.5.1 Description of evaluation scenarios

The physical layer latency evaluations are captured for the following Rel-16 DL-based positioning scenarios:

- Case ID: 1- DL-TDOA / DL-AoD, UE-Assisted (UE-A) Positioning with MG configuration.

- Case ID: 2- DL-TDOA/ DL-AoD, UE-Assisted (UE-A) Positioning without MG configuration.

- Case ID: 3- DL-TDOA/ DL-AoD, UE-Based (UE-B) Positioning with MG configuration.

- Case ID: 4- DL-TDOA/ DL-AoD, UE-Based (UE-B) Positioning without MG configuration.

C.1.2.5.2 Latency analysis for Rel.16 solutions

Table C.1.2.5.2-1 presents the UE-Assisted DL-TDOA/DL-AoD physical layer latency evaluations by taking into account the minimum and cautious latency estimates (within the value range) of the various delay components (starting with the earliest component) with a Measurement Gap (MG) configuration, while Table C.1.2.5.2-2 presents the same scenario without a Measurement Gap (MG) configuration.

Table C.1.2.5.2-3 presents the UE-Based DL-TDOA/DL-AoD physical layer latency evaluations by taking into account the minimum and cautious latency estimates (within the value range) of the various delay components (starting with the earliest component) with a Measurement Gap (MG) configuration, while Table C.1.2.5.2-4 presents the same scenario without a Measurement Gap (MG) configuration.

Table C.1.2.5.2-1: Rel.16 NR positioning latency [11]

| Case ID: 1, Scenario: UE-Assisted Positioning with MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA or R.16 DL-AoD Source: NW, Destination: NW Positioning technique(s): DL-TDOA or DL-AoD, Type: DL, Mode: UE-Assisted (UE-A) Initial RRC and Final RRC States: CONNECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Latency Component | Value Range (ms) |  | Description of Latency Component |
|  |  | Min Value | Cautious Estimate7 |  |
| PHY Start |  |  |  |  |
| 1.1. | Start trigger (Air Interface1 Latency) | 1 – FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message (Air Interface Latency) Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 – FR2: 120 kHz SCS |  |
| 1.2. | UE Processing4 of LPP Request Location Information message | 3 | 10 | UE Processing of the PDSCH carrying the LPP Request Location Information message |
| PUSCH Preparation Time2 |  |  |  |  |
| 2. | UL Transmission of Measurement Gap Request (SR-based Scheduling) | 1 | 3.8 | Cautious Estimate = max (FDD, TDD) = max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| Measurement Gap Configuration |  |  |  |  |
| 3.1 | Measurement Gap Request (Air Interface Latency1) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the MG Request/Need message (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2:120 kHz SCS | 0.5 – FR2: 120 kHz SCS |  |
| 3.2 | gNB Processing3 of Measurement Gap Request | 3 | 10 | gNB Processing of the PUSCH carrying the MG Request message |
| 3.3 | Measurement Gap Configuration (Air Interface1 Latency) | 1 - FR1:30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the Measurement Gap Configuration (Air Interface Latency) |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 – FR2: 120 kHz SCS |  |
| 3.4 | UE Processing4 of Measurement Gap Configuration message | 3 | 10 | UE Processing of the PDSCH carrying the Measurement Gap Configuration message |
| DL-PRS Configuration within TMGRP |  |  |  |  |
| 4.1 | MG Gap Repetition Period (TMGRP), NOcc5= 1 | 1×20 | 1×160 | UE processing of PRS within a MG |
| 4.2 | UE Processing of DL-PRS Units (NProc,T) in addition to the TMGRP | (NProc,T) = (6,8) 0 | (NProc,T)6 = (20, 160) 20 | Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. UE Processing time =max (0, T-(TMGRP-N)) |
| PUSCH Preparation Time2 |  |  |  |  |
| 5.1 | UL Transmission of Measurement Report without UL grant | 1 | 3.8 | Cautious Estimate = max (FDD, TDD) = max (2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| PHY End |  |  |  |  |
| 6.1 | End Trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the LPP Provide Location Information (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 – FR2: 120 kHz SCS |  |
| 6.2 | End Trigger (Processing) | 3 | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message containing the measurement report. |
|  | Total Values | 38 - FR1: 30 kHz SCS | 235.6- FR1: 30 kHz SCS |  |
|  |  | 35 – FR2: 120 kHz SCS | 229.6 – FR2: 120 kHz SCS |  |
| Notes: 1: The Air Interface is a function dependent on the selected numerology, TDD/FDD configuration, and FR1/FR2 band. 2: The PUSCH processing time is based on the SR-based scheduling of PUSCH in [4]. 3: gNB Processing: The minimum processing estimates in [11, Ref 4] served as a guideline. The cautious estimates of the gNB processing times of the RRC messages are based on the delay requirements in [5]. The value is used for evaluation only; gNB processing delay may vary depending on implementation/deployment. 4: UE Processing: The minimum estimate in [11, Ref 4] served as a guideline. The cautious estimates of the UE processing times of the RRC messages are based on the delay requirements in [11, Ref 5]. 5: The number of DL-PRS occasions, NOcc = 1, although this may not be a typical value for obtaining accurate positioning measurements. The positioning measurements would need to be averaged over a higher number of DL-PRS occasions for accurate measurements. 6: The symbol processing time of N=20 ms was selected to align with the associated MGL=20, which has a corresponding TMGRP = 160 ms. 7: The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |  |  |  |

Table C.1.2.5.2-2: Rel.16 NR positioning latency [11]

| Case ID: 2, Scenario: UE-Assisted Positioning without MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA or R.16 DL-AoD Source: NW, Destination: NW Positioning technique(s): DL-TDOA, DL-AoD, Type: DL, Mode: UE-Assisted (UE-A) Initial RRC and Final RRC States: CONNECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Latency Component | Value Range (ms) |  | Description of Latency Component |
|  |  | Min Value | Cautious Estimate6 |  |
| PHY Start |  |  |  |  |
| 1.1. | Start trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message (Air Interface Latency) Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 - FR1: 120 kHz SCS |  |
| 1.2. | UE Processing4 of LPP Request Location Information message | 3 | 10 | UE Processing of the PDSCH carrying the LPP Request Location Information message |
| DL-PRS Configuration = TPRS × NOcc5 |  |  |  |  |
| 2.1 | DL-PRS Resources (DL-PRS Periodicity (TPRS) & (NOcc5= 1)) | 1×4 | 1×5120 | TPRS of 5120 selected as cautious estimate. It is a function of the PRS periodicity and number of occasions (NOcc5=1) over which the measurements will be averaged. |
| 2.2 | UE Processing4 of DL-PRS Units (NProc,T) | 4 | 0 | Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz. Minimum estimate is based on minimum T of 8 ms while cautious estimate is based on maximum T of 1280 ms. Processing time =max (0, T-TPRS ) |
| PUSCH Preparation Time2 |  |  |  |  |
| 3.1 | UL Transmission of Measurement Report without UL grant | 1 | 3.8 | Cautious Estimate = max (FDD, TDD)= max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| PHY End |  |  |  |  |
| 4.1 | End Trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the LPP Provide Location Information (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 4.2 | End Trigger (Processing) | 3 | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message containing the measurement report. |
|  | Total Values | 17 - FR1: 30 kHz SCS | 5147.8 - FR1: 30 kHz SCS |  |
|  |  | 15.5 - FR2: 120 kHz SCS | 5144.8 - FR2: 120 kHz SCS |  |
| Notes: 1: The Air Interface is a function dependent on the selected numerology, TDD/FDD configuration, and FR1/FR2 band. 2: The PUSCH processing time is based on SR-based scheduling of PUSCH in [11, Ref 4]. 3: gNB Processing: The minimum estimate in [11, Ref 4] served as a guideline. The value is used for evaluation only; gNB processing delay may vary depending on implementation/deployment. 4: UE Processing: The minimum estimate in [11, Ref 4] served as a guideline. The cautious estimates UE processing times of the RRC messages are based on the delay requirements in [11, Ref 5]. 5: The number of DL-PRS occasions, NOcc = 1, although this may not be a typical value for obtaining accurate positioning measurements. The positioning measurements would need to be averaged over a higher number of DL-PRS occasions for accurate measurements. 6: The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |  |  |  |

Table C.1.2.5.2-3: Rel.16 NR positioning latency [11]

| Case ID: 3, Scenario: UE-based Positioning with MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA or R.16 DL-AoD Source: UE, Destination: UE Positioning technique(s): DL-TDOA, DL-AoD Type: DL, Mode: UE-Based (UE-B) Initial RRC and Final RRC States: CONNECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Latency Component | Value Range (ms) |  | Description of Latency Component |
|  |  | Min Value | Cautious Estimate9 |  |
| PHY Start (If applicable)7 |  |  |  |  |
| 1.1. | Start trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message (Air Interface Latency) Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 – FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 1.2. | UE Processing4 of LPP Request Location Information message | 3 | 10 | UE Processing of the PDSCH carrying the LPP Request Location Information message |
| PHY Start (Alt. 1): PUSCH Preparation Time2 |  |  |  |  |
| 2.1 | UL Transmission Preparation of Measurement Gap Request (SR-based Scheduling) | 1 | 3.8 | Cautious Estimate = max (FDD, TDD)= max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| Measurement Gap Configuration |  |  |  |  |
| 3.1 | Measurement Gap Request (Air Interface Latency1) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the MG Request/Need message (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 3.2 | gNB Processing3 of Measurement Gap Request | 3 | 10 | gNB Processing of the PUSCH carrying the MG Request message |
| 3.3 | Measurement Gap Configuration (Air Interface Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the Measurement Gap Configuration (Air Interface Latency) |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 3.4 | UE Processing4 of Measurement Gap Configuration message | 3 | 10 | UE Processing of the PDSCH carrying the Measurement Gap Configuration message |
| With MG Gap of DL-PRS Configuration |  |  |  |  |
| 4.1 | MG Gap Repetition Period (TMGRP), NOcc5= 1 | 1×20 | 1×160 | UE processing of PRS within a MG |
| 4.2 | UE Processing of DL-PRS Units (NProc,T) in addition to the TMGRP | (NProc,T) = (6,8) 0 | (NProc,T)8 = (20, 160) 20 | Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. UE Processing time =max (0, T-(TMGRP-N)) |
| PUSCH Preparation Time (If applicable) 2 |  |  |  |  |
| 5.1 | UL Transmission of Measurement Report without UL grant | 1 | 3.8 | Cautious Estimate = max (FDD, TDD)= max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| PHY End (If applicable) 7 |  |  |  |  |
| 6.1 | End Trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the LPP Provide Location Information (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 6.2 | End Trigger (Processing) | 3 | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message containing the UE location estimate. |
|  | Total Values without Request and Provide Location Information Message Components = {2-5} | 29 - FR1: 30 kHz SCS | 207.8- FR1: 30 kHz SCS |  |
|  |  | 27.5 - FR2: 120 kHz SCS | 204.8 - FR2: 120 kHz SCS |  |
|  | With MG Gap and with Request and Provide Location Information message (If applicable) Components = {1-6} | 38 - FR1: 30 kHz SCS | 265.6- FR1: 30 kHz SCS |  |
|  |  | 35 - FR2: 120 kHz SCS | 259.6 - FR2: 120 kHz SCS |  |
| Notes: 1: The Air Interface is a function dependent on the selected numerology, TDD/FDD configuration, and FR1/FR2 band. 2: The PUSCH processing time is based on SR-based scheduling of PUSCH in [11, Ref 4]. 3: gNB Processing: The minimum estimate in [11, Ref 4] served as a guideline. The value is used for evaluation only; gNB processing delay may vary depending on implementation/deployment. 4: UE Processing: The minimum estimate in [11, Ref 4] served as a guideline. The cautious estimates UE processing times of the RRC messages are based on the delay requirements in [11, Ref 5]. 5: The number of DL-PRS occasions, NOcc = 1, although this may not be a typical value for obtaining accurate positioning measurements. The positioning measurements would need to be averaged over a higher number of DL-PRS occasions for accurate measurements. 6: For the evaluation scenario, the starting trigger for UE-based positioning considers the transmission of the PUSCH carrying the MG Request from the UE. 7: The LPP Request Location Information and Provide Location Information messages are considered in the evaluation as the UE may transmit the location estimate to the LMF, when applicable. 8: The symbol processing time of N=20 ms was selected to align with the associated MGL=20, which has a corresponding TMGRP = 160 ms. 9: The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |  |  |  |

Table C.1.2.5.2-4: Rel.16 NR positioning latency [11]

| Case ID: 4, Scenario: UE-based Positioning without MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA or R.16 DL-AoD Source: UE, Destination: UE Positioning technique(s): DL-TDOA, DL-AoD Type: DL, Mode: UE-Based (UE-B) Initial RRC and Final RRC States: CONNECTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Latency Component | Value Range (ms) |  | Description of Latency Component |
|  |  | Min Value | Cautious Estimate7 |  |
| PHY Start (If applicable)7 |  |  |  |  |
| 1.1. | Start trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message (Air Interface Latency) Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 1.2. | UE Processing4 of LPP Request Location Information message | 3 | 10 | UE Processing of the PDSCH carrying the LPP Request Location Information message |
| DL-PRS Configuration = TPRS × NOcc5 |  |  |  |  |
| 2.1 | DL-PRS Resources (DL-PRS Periodicity (TPRS) & (NOcc5= 1)) | 1×4 | 1×5120 | TPRS of 5120 selected as cautious estimate. It is a function of the PRS periodicity and number of occasions (NOcc5=1) over which the measurements will be averaged. |
| 2.2 | UE Processing4 of DL-PRS Units (N,T) | 4 | 0 | Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz. Minimum estimate is based on minimum T of 8 ms while cautious estimate is based on maximum T of 1280 ms. Processing time =max(0, T-TPRS ) |
| PUSCH Preparation Time (If applicable) 2 |  |  |  |  |
| 3.1 | UL Transmission of Measurement Report without UL grant | 1 | 3.8 | Cautious Estimate = max (FDD, TDD)= max(2.8, 3.8). Minimum value based on capability 2 (URLLC) device with readily available UL grant. |
| PHY End (If applicable) 6 |  |  |  |  |
| 4.1 | End Trigger (Air Interface1 Latency) | 1 - FR1: 30 kHz SCS | 2 - FR1: 30 kHz SCS | Transmission of the PUSCH from the UE carrying the LPP Provide Location Information (Air Interface Latency). Minimum value based on capability 2 (URLLC) device, while cautious estimate based on capability 1 (eMBB) device. |
|  |  | 0.25 - FR2: 120 kHz SCS | 0.5 - FR2: 120 kHz SCS |  |
| 4.2 | End Trigger (gNB Processing3) | 3 | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message containing the UE location estimate. |
|  | Total Values Components = {2} | 8 | 5120 |  |
|  | Without MG Gap and with Request and Provide Location Information message (If applicable) Components = {1-4} | 17 - FR1: 30 kHz SCS | 5147.8 - FR1: 30 kHz SCS |  |
|  |  | 15.5 - FR2: 120 kHz SCS | 5144.8 - FR2: 120 kHz SCS |  |
| Notes: 1: The Air Interface is dependent on the selected numerology, TDD/FDD configuration, and FR1/FR2 band. 2: The PUSCH processing time is based on SR-based scheduling of PUSCH in [11, Ref 4]. 3: gNB Processing: The minimum estimate in [11, Ref 4] served as a guideline. The value is used for evaluation only; gNB processing delay may vary depending on implementation/deployment. 4: UE Processing: The minimum estimate in [11, Ref 4] served as a guideline. The cautious estimates UE processing times of the RRC messages are based on the delay requirements in [11, Ref 5]. 5: The number of DL-PRS occasions, NOcc = 1, although this may not be a typical value for obtaining accurate positioning measurements. The positioning measurements would need to be averaged over a higher number of DL-PRS occasions for accurate measurements. . 6: The LPP Request Location Information and Provide Location Information messages are considered in the evaluation as the UE may transmit the location estimate to the LMF, when applicable. 7: The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |  |  |  |

C.1.2.6	Results from source [5]

C.1.2.6.1	Description of evaluation scenarios

C.1.2.6.2	Latency analysis for Rel.16 solutions

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest one. Latency analysis for the Rel.16 NR positioning is provided in Table C.1.2.6.2-1 to Table C.1.2.6.2-7.

Table C.1.2.6.2-1: Rel.16 NR positioning latency [5]

| [Case 1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE, Network]/Destination [UE,Network] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 64~ | The minimum total value is 64ms if only consider the minimum value for every step. |

Table C.1.2.6.2-2: Rel.16 NR positioning latency [5]

| [Case 1-1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE, Network]/Destination [UE,Network] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-A], Initial and Final RRC States [Idle/inactive] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Additional latency for idle/inactive state to connected state | Inactive: [21.3-178.5] Idle:[40-200] | The latency for inactive to connected is 11.3-18.5 based on Table 5.7.2.1-1 of 37.910 which don’t consider the periodicity of PRACH, So the total latency is 21.3-178.5 as the periodicity of PRACH is 10 ms -160 ms; In addition, compared to the latency for inactive to connected, the latency for idle to connected may have additional high layer steps, such as steps of gNB to AMF interactive, AMF processing etc. Therefore, we assume that additional 20ms for higher layer latency is needed at least, so the latency for idle to connected is assumed as 40~200ms. |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 85.3/104~ | The minimum total value is 85.3 for inactive and 104ms for idle state if only consider the minimum value for every step. |

Table C.1.2.6.2-3: Rel.16 NR positioning latency [5]

| [Case 2], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [Network]/Destination [Network] Positioning technique [DL-TDOA], type [DL], mode [UE-B], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | For Physical Layer Latency Start time of UE based method, Alt.2 is selected. Transmission of the PDSCH from the gNB carrying LPP message containing the assistance data. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Calculation of Location Estimate at the UE | 2 | Calculation of Location Estimate at the UE |
| 8: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 66~ | The minimum total value is 66ms if only consider the minimum value for every step. |

Table C.1.2.6.2-4: Rel.16 NR positioning latency [5]

| [Case 3], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE]/Destination [UE] Positioning technique [DL-TDOA], type [DL], mode [UE-B], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying LPP message containing the assistance data. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Calculation of Location Estimate at the UE | 2 | Calculation of Location Estimate at the UE |
| Total values | 55.5~ | The minimum total value is 55.5ms if only consider the minimum value for every step. |

Table C.1.2.6.2-5: Rel.16 NR positioning latency [5]

| [Case 4], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA] Source [UE,Network]/Destination [UE,Network] Positioning technique [UL-TDOA/UL-AoA], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 10 | Reception and Successful decode the NRPPa measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 1: UL measurement &process delay. | FR1: 0.5-2560 FR2: 0.125~640 | For case 1: one occasion and without considering beam sweeping UL measurement equals to the periodicity of SRS SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560}slots 15kHz 1ms-2560ms 30kHz 0.5ms-1280ms 60kHz 0.25ms-640ms 120kHz 0.125ms-320ms For case 2: Multiple positioning occasion and beam sweeping UL measurement equals to the periodicity of SRS×NRxBeam×NPosOccasion gNB processing delay is assumed as zero |
| End trigger | 0.5 | The transmission by the gNB of the NRPPa measurement response message. Which value less than the DL/UL data transmission time. It can be seen as the gNB processing time (Tprocess time_pdsch) and smaller than 0.5ms. |
| Total values | 30.5~ | The minimum total value is 30.5 ms if the periodicity of SRS is 20ms and the same as the DL minimum periodicity. |

Table C.1.2.6.2-6: Rel.16 NR positioning latency [5]

| [Case 5], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA] Source [UE,Network]/Destination [UE,Network] Positioning technique [UL-TDOA/UL-AoA], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 10 | Reception and Successful decode the NRPPa SRS activation and measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 1. Transmission of the PDCCH from the gNB carrying activate SRS message | 0.5 | Transmission of the PDCCH from the gNB carrying activate SRS message. Which value less than the DL/UL data transmission time. It needs to consider Tprocess time_pdcch, Talign time andTPDCCH as below and smaller than 0.5ms. - the gNB processing time (Tprocess time_pdcch )and the alignment delay(Talign time ) - The alignment delay includes the gap between the two consecutive PDCCH monitoring occasions for FDD, the UL/DL configuration for TDD, and the scheduling constraint due to the slot boundaries. - PDCCH duration (TPDCCH ) |
| 2: UL measurement &process delay. | 0-32 | The slot offset of aperiodic is 0-32 slots 15kHz 0ms-32ms 30kHz 0ms-16ms 60kHz 0ms-8ms 120kHz 0ms-4ms |
| End trigger | 0.5-1 | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 11~ | The minimum total value is 11ms if only considers the minimum value for every step. |

Table C.1.2.6.2-7: Rel.16 NR positioning latency [5]

| [Case 6], [IIoT/ Commercial], [Frequency Band], [Multi-RTT] Source [UE,Network]/Destination [UE,Network] Positioning technique [Multi-RTT], type [DL], mode [UE-A], Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 8. Successful decoding of the PUSCH carrying the LPP Provide Location Information message | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| 9. The alignment delay is the gap between End trigger of DL positioning and Start trigger of UL positioning | [TBD] | TAlign_DL_UL: The alignment delay is the gap between End trigger of DL positioning and Start trigger of UL positioning. If TAlign_DL_UL>0 , Start trigger timing of UL positioning is later than End trigger timing of DL positioning and the multi-RTT latency will bigger than the sum(TUE assisted DL only, TUE assisted UL only). If - TUE assisted DL only < TAlign_DL_UL<0, Start trigger timing of UL positioning is earlier than End trigger timing of DL positioning and the multi-RTT latency will less than the sum(TUE assisted DL only, TUE assisted UL only). |
| 10. Reception and Successful decode the NRPPa measurement request message from the LMF at gNB | 10 | Reception and Successful decode the NRPPa measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 11: UL measurement &process delay. | 0.125-2560 | For case 1: one occasion and without considering beam sweeping UL measurement equals to the periodicity of SRS SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560} slots 15kHz 1ms-2560ms 30kHz 0.5ms-1280ms 60kHz 0.25ms-640ms 120kHz 0.125ms-320ms For case 2: Multiple positioning occasion and beam sweeping UL measurement equals to the periodicity of SRS×NRxBeam×NPosOccasion gNB processing delay is assumed as zero |
| End trigger | 0.5 | The transmission by the gNB of the NRPPa measurement response message. Which value less than the DL/UL data transmission time. It can be seen as the gNB processing time (Tprocess time_pdsch) and smaller than 0.5ms. |
| Total values | 94.5+TAlign_DL_UL |  |

C.1.2.7	Results from source [12]

C.1.2.7.1	Description of evaluation scenarios

The following positioning method are included for physical layer latency analysis

- UE-assisted DL-based method: DL-TDOA

- UE-assisted UL-based method: UL-TDOA

- UE- based method: UE-based positioning calculation

C.1.2.7.2	Latency analysis for Rel.16 solutions

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning are provided in Tables C.1.2.7.2-1~3.

Table C.1.2.7.2-1: Rel.16 NR positioning latency DL-TDOA

| Case 1 and 4: DL-TDOA. 60KHz and 120 KHz Source UE/Destination NW Positioning technique:DL-TDOA, Initial and Final RRC States:CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | gNB sending PDSCH carrying LPP location request message |
| Process LPP msg | 10ms | UE process LPP location request information message |
| measurement gap configuration | 10ms+10ms: RRC reconfiguration delay | UE requests MG and gNB configures MG |
| DL PRS reception and prcoess | Minimum value is 20ms + 1ms DL PRS periodicity can be 4, 5, 8, 10, 16, 20, 32, 40, 64, 80,160, 320,640, 1280, 2560, 5120 or10240 ms MG periodicity can be 20, 40, 80 or 160ms Processing time depends on the UE capability: Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms | DL PRS reception and measurement |
| End trigger | 12.5ms for 15KHz 6.25ms for 30KHz 3.125ms for 60KHz 1.56 ms for 120KHz UE sends PUCCH SR to request UL grant UE receives DCI scheduling PUSCH UE sends PUSCH carrying measurement report PUSCH decodes PUSCH | UE sends PUSCH carrying location measurement report and gNB decode PUSCH |
| Total values | For FR1 (60kHz): 54.125ms For FR2 (120KHz): 52.56ms |  |

Table C.1.2.7.2-2: Rel.16 NR positioning latency of UL-TDOA

| Case 2 and 5: UL-TDOA. 60KHz and 120 KHz Source UE/Destination NW Positioning technique:DL-TDOA, Initial and Final RRC States:CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | gNB receives NPPa measurement request |
| gNB process NPPa msg | 10ms | gNB process the NPPa measurement request |
| SRS configuration | 10ms+10ms: RRC reconfiguration delay | gNB configures SRS |
| SRS transmission | Assume periodic SRS resource. The periodicity of SRS can be 1, 2, 4, 5, 8, 10, 16, 20 .. slots. So the minimum latency is: 1ms for 15KHz 0.25ms for 60KHz. 0.125ms for 120KHz | UE transmits SRS for positioning |
| End trigger | Assume that 3ms is needed by gNB to measure SRS and process the NPPPa measurement response | gNB measures SRS for positioning and reports the NPPPa measurement response |
| Total values | For FR1 (60kHz): 23.25ms For FR2 (120KHz): 23.125ms |  |

Table C.1.2.7.2-3: Rel.16 NR positioning latency of UE-based method

| Case 3 and 6 :UE-based method 60KHz and 120 KHz Source UE/Destination NW Positioning technique:DL-TDOA, Initial and Final RRC States:CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger | 10ms RRC processing delay | UE sends RRC Location measurement indication |
| measurement gap configuration | 10ms: RRC reconfiguration delay | gNB sends RRC measurement gap configuration |
| DL PRS reception and measurement | Minimum value is 20ms + 1ms DL PRS periodicity can be 4, 5, 8, 10, 16, 20, 32, 40, 64, 80,160, 320,640, 1280, 2560, 5120 or 10240ms. However, the measurement gap periodicity can be 20, 40, 80 or 160ms Processing time depends on the UE capability. Assume extra 1ms is needed for the UE to finish measurement | DL PRS reception and measurement |
| End trigger | 3 ms | UE completes location calculation |
| Total values | For FR1 (60kHz): 54.125ms For FR2 (120KHz): 52.56ms |  |

C.1.2.8	Results from source [10]

C.1.2.8.1	Description of evaluation scenarios

In order to provide NR positioning physical layer latency analysis, we use the following reference system configuration which is in favor of low latency positioning and were used in NR URLLC studies [3GPP 38.824, v16.0.0]

- Spectrum – FDD / FR1

- SCS – 30 kHz

- PDCCH,

  - Monitoring occasions – 7 per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0]

  - Duration – 1 symbol

- PDSCH

  - One symbol overlap with PDCCH

  - Duration – 2, 4, 7 symbols (Type B mapping)

  - Slot boundary is respected (i.e. transmission does not cross slot boundary)

- PUSCH

  - Any symbol, subject to slot boundary constraint (i.e. transmission does not cross slot boundary)

  - Duration – 2, 4, 7 symbols (Type B mapping w/ front loaded DMRS)

- PUCCH

  - 7 occasions per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0] for SR and HARQ feedback

  - Duration – 1 symbol

  - No HARQ – initial transmission is successful

- SRS for positioning

  - Single resource, 1 symbol duration

  - Periodicity – each slot

- DL PRS

  - 18 resources / 4 symbols per resource / 12 Comb-6 symbols per period

  - Periodicity – 20 ms

  - UE DL PRS processing capability – N = 0.5 ms (~12 symbols @30kHz), T = 8 ms

- UE capability for Tproc,1 Tproc,2

  - Capability # 2 (for low latency)

- Measurement gap

  - Type 1: BWP switching = 2 slots @ 30kHz

  - MGL = 5.5 ms, MGRP = 20ms

    - 2 BWP switching times per occasion (from and to active DL BWP) + DL PRS duration => 5 slots

  - Nsample = 4 (RAN4 core measurements requirements)

    - UE is expected to perform measurements on DL PRS resource 4 times (i.e. across 4 periods)

C.1.2.8.2	Latency analysis for Rel.16 solutions

Latency analysis for the Rel.16 NR positioning is provided in Table C.1.2.8.2-1, C.1.2.8.2-2 and C.1.2.8.2-3.

Table C.1.2.8.2-1: Rel.16 NR positioning latency for DL-TDOA/DL-AoD

| Case 1, InF, FR1, R.16 DL-TDOA/DL-AoD Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: DL-TDOA/DL-AoD, type DL only, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  |  |
| Step 1. Location Request | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [38.824] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.25 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.25 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L3 UE LPP processing] | [~280symbols ~10 ms] | Not specified. [The following number is assumed – 5ms DL RRC processing time and 5ms LPP processing time. TBD by RAN WG2] |
| Step 2. Measurement Gap Request | 39.75 symbols 1.4196 ms | Value is provided for L1 components only |
| 2A: [L3 UE RRC processing delay] | 56-140 symbols 2-5ms | [Not defined: 2-5ms UE RRC processing delay is assumed in this analysis TBD by RAN WG2] |
| 2B: SR preparation time | 0 symbols 0 ms | 0ms is assumed since SR can be prepared in advance based on UE implementation |
| 2C: SR alignment time | 2 symbols 0.0714 ms | Depends on configuration of SR monitoring occasions, TDD configuration Worst case: 2 symbols@30kHz for selected reference system configuration |
| 2D: SR transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2E: SR processing time | 4.5 symbols 0.1607 ms | Not specified. For analysis, the assumption from [38.824] is used. For SR-based PUSCH, gNB's processing time for SR is UE's N1. |
| 2F: PDCCH preparation time | 2.25 symbols 0.0804 ms | Not specified. For analysis, the assumptions from the [38.824] are used to derive PDCCH preparation time equal to N1/2: gNB's PUSCH-to-PDCCH processing time (PDCCH alignment has to be included separately) is UE's N1 + X; gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| 2G: PDCCH alignment time | 2 symbols 0.0714 ms | Depends on configuration of PDCCH monitoring occasions and TDD configuration |
| 2H: PDCCH transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2I: PUSCH preparation time | 6.5 symbols 0.2321 ms | Specified as UE PUSCH preparation time Tproc,2 = N2+d21+d2 (if no BWP switch) N2 = 5.5@30kHz, d21 = 1, d2 = 0. |
| 2J: PUSCH alignment time | 7 symbols 0.2500 ms | Worst case:7 symbols for given reference configuration |
| 2K: PUSCH transmission time | 7 symbols 0.2500 ms | 7 symbols for selected reference system configuration |
| 2L: PUSCH processing time | 6.5 symbols 0.2321 ms | Not specified. For analysis, the assumption from the [38.824] is used. gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |

| 2M: [L3 gNB RRC processing delay] | [~84 symbols ~3 ms] | [Not specified. 3ms are assumed in this analysis for gNB RRC processing delay TBD by RAN WG2] |
| --- | --- | --- |
| Step 3. Measurement Gap Configuration | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 3A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Same as for 1A |
| 3B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Same as for 1B |
| 3C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Same as for 1C |
| 3D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Same as for 1D |
| 3E: [L3 UE RRC processing delay] | [~280symbols 10 ms] | [Specified - 10ms TBD by RAN WG2] |
| Step 4. DL PRS Processing | 88,5 ms | Value is provided for L1 components only |
| 4A: DL PRS alignment time | 560symbols 20 ms | Depends on DL PRS periodicity settings Worst case: 20ms for selected reference system configuration |
| 4B: DL PRS processing and report delay | 1918 symbols 68.5 ms | Depends on DL PRS configuration, MG configuration (length and period) and UE DL PRS processing capabilities(N,T). MG should accommodate DL BWP switching. The report delay is defined by 38.133 in Section 9.9.2.5. For considered, reference system configuration it is equal to 3*20+8+0.5 = 68.5ms Note: may be revised by RAN WG4 |
| Step 5. DL PRS Measurement Report | 39.75 symbols 1.4196 ms | Value is provided for L1 components only |
| 5A: [L3: UE RRC + LPP processing delay] | [56-140 symbols 2-5ms] | [Not defined 2-5 ms UE RRC processing time and 5ms UE LPP processing time are assumed TBD by RAN WG2] |
| 5B: SR preparation time | 0 symbols 0 ms | Same as for 2B |
| 5C: SR alignment time | 2 symbols 0.0714 ms | Same as for 2C |
| 5D: SR transmission time | 1 symbol 0.0357 ms | Same as for 2D |
| 5E: SR processing time | 4.5 symbols 0.1607 ms | Same as for 2E |
| 5F: PDCCH preparation time | 2.25 symbols 0.0804 ms | Same as for 2F |
| 5G: PDCCH alignment time | 2 symbols 0.0714 ms | Same as for 2G |
| 5H: PDCCH transmission time | 1 symbol 0.0357 ms | Same as for 2H |
| 5I: PUSCH preparation time | 6.5 symbols 0.2321 ms | Same as for 2I |
| 5J: PUSCH alignment time | 7 symbol 0.2500 ms | Same as for 2J |
| 5K: PUSCH transmission time | 7 symbols 0.2500 ms | Same as for 2K |
| 5L: PUSCH processing time | 6.5 symbols 0.2321 ms | Same as for 2L |
| 5M: [L3 gNB LPP processing time] | [84 symbols 3 ms] | [Not defined. 3 ms are assumed for gNB LPP processing time TBD by RAN WG2] |

Table C.1.2.8.2-2: Rel.16 NR positioning latency for UL-TDOA/UL-AoA

| Case 2, InF, FR1, R.16 UL-TDOA/UL-AoA Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: UL-TDOA/UL-AoA, type UL only, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| Step 1. Configuration of SRS for positioning | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [7] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,g1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L3 UE RRC configuration delay] | [280 symbols 10ms] | [Specified - 10ms TBD by RAN WG2] |
| Step 2 (Alt.1): Activation of SRS for positioning (DCI) | 8.5 symbols 0.3036 ms | Value is provided for L1 components only |
| 2A: PDCCH preparation time | 2.25 symbols 0.0804 ms | Not specified. For analysis, the assumptions from the [7] are used to derive PDCCH preparation time equal to N1/2: gNB's PUSCH-to-PDCCH processing time (PDCCH alignment has to be included separately) is UE's N1 + X; gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| 2B: PDCCH alignment time | 2 symbols 0.0714 ms | Depends on configuration of PDCCH monitoring occasions and TDD configuration |
| 2C: PDCCH transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2D: PDCCH processing time | 3.25 symbols 0.1161 ms | Not specified: For analysis, it is assumed to be equal to ½ of UE PUSCH preparation time Tproc,2 = N2+d21+d2 (if no BWP switch) i.e. Tproc,2/2 N2 = 5.5@30kHz, d21 = 1, d2 = 0. |
| Step 2’(Alt.2): Activation of SRS for positioning (MAC CE) | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 2’A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Same as for 1A |
| 2’B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Same as for 1B |
| 2’C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Same as for 1C |
| 2’D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Same as for 1D |
| 2’E: [L2/L3 component: UE MAC processing delay] | [28-84 symbols 1-3ms] | [Not defined: 1-3ms are assumed TBD by RAN WG2] |
| Step 3. SRS for positioning Tx & Rx | 29 symbols 1.0357 ms | Value is provided for L1 components only |
| 3A: SRS for positioning preparation time | 0 ms | Not defined. 0ms is assumed for analysis |
| 3B: SRS for positioning alignment time | 14 symbols 0.5ms | Depends on SRS resource allocation. Worst case: 14 symbols for selected reference configuration |
| 3C: SRS for positioning transmission time | 1 symbol 0.0357 | 1 symbol for selected reference configuration |
| 3D: SRS for positioning processing time by gNB | [14 symbols 0.5 ms] | [Not defined: 0.5 ms is assumed. Definition is in scope of RAN WG4] |
| Step 4. gNB Measurement Report | 0 | Value is provided for L1 components only |
| 4A: [L3 gNB NRPPa preparation time] | [84symbols 3 ms] | [Not defined: 3 ms is assumed. TBD by RAN WG2] |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |

Table C.1.2.8.2-3: Rel.16 NR positioning latency for Multi-RTT

| Case 3, InF, FR1, R.16 Multi-RTT Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: Multi-RTT, type DL+UL, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| Step 1. Configuration of SRS for positioning | L1: 0.8661 [L3: 10.0] | L1: gNB signaling to UE to configure SRS for positioning – 0.5088 ms [L3: UE RRC configuration delay – 10ms - TBD by RAN WG2] |
| Step 2: Activation of SRS for positioning (MAC CE) | L1: 0.8661 [L2: 1.0] | L1: gNB signaling to UE to activate SRS for positioning transmission – 0.5088 ms [L2: UE MAC CE processing delay – 1 ms - TBD by RAN WG2] |
| Step 3. SRS for positioning TX & RX | L1: 1.0357 | L1: transmission of SRS for positioning by UE and its reception by gNB – 1.0357 ms |
| Step 4. gNB NRPPa preparation time for report to LMF | [L3: 3.0] | [L3: gNB NRPPa preparation time for report to LMF – 3 ms - TBD by RAN WG2] |
| Step 5. Location information request | L1: 0.8661 [L3: 10.0] | L1: gNB transmits to UE location request - 0.5088 ms [L3: 5 ms DL RRC processing time and 5ms LPP processing time - TBD by RAN WG2] |
| Step 6. MG Request | L1: 1.4196 [L3: 5-8] | L1: UE requests gNB to provide MG – 1.0266 ms [L3: UE RRC processing delay – 2-5 ms and gNB RRC processing delay – 3ms TBD by RAN WG2] |
| Step 7. MG Configuration | L1: 0.8661 [L3: 10.0] | L1: gNB provides MG configuration to UE RRC signaling – 0.5088 ms [L3: UE RRC configuration delay – 10 ms - TBD by RAN WG2] |
| Step 8. DL PRS Processing | L1: 88.5 | DL PRS alignment time and DL PRS processing and report delay |
| Step 9. DL PRS Measurement Report | L1: 1.4196 [L3: 3.0] | L1: UE reports measurement results to gNB [L3: gNB LPP processing time - 3ms; 2-5ms UL RRC processing time; 5ms UE LPP processing time are assumed. TBD by RAN WG2] |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |

C.1.2.9 Results from source [16]

C.1.2.9.1 Description of evaluation scenarios

The latency analysis of DL, UL, and DL+UL methods are presented here. In each method, we also perform the analysis of UE-Assisted and UE-Based (if applicable). All analysis are based on 30kHz SCS and minimum value for latency is presented.

C.1.2.9.2 Latency analysis for R.16 solution

Table C.1.2.9.2-1: Latency analysis for UE-assisted DL methods (Case 1) [16]

| Source [NW]/Destination [NW] Positioning technique: DL methods Initial and Final RRC States [CONNECTED], mode: UE-A |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | 1 | Estimate |
| Step 2: The UE decodes the LPP Request location | 10 | NAS message, based on RC message decoding time |
| Step 3: The UE transmits measurement gap (MG) request in an RRC message. | 1 | Estimate |
| Step 4: The gNB successfully decode the MG request message | 5 | RRC message |
| Step 5: The gNB configures MG in an RRC message | 1 | Estimate |
| Step 6: The UE receive the MG configuration message and apply the MG configuration. | 10 | RRC message decoding including MG configuration |
| Step 7: The UE receives PRS in the MG | 2 | According to RRC, the UE can be configured with MGL of 1.5ms. Thefore, the best scenario is that the UE can be configured MG of 1.5ms, performing PRS monitoring and finish the MG within 2ms. |
| Step 8: The UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 | The UE prepares PUSCH (1ms), uses type-1 configured grant to report the positioning measurement (1ms). Then gNB successfully decodes PUSCH (1ms). Note that the minimum periodicity of type-1 configured grant is 2 symbols for all SCSs. |
| Total values | 33ms |  |

Table C.1.2.9.2-2: Latency analysis for UE-based DL methods (Case 2) [16]

| Source [NW]/Destination [UE, NW] Positioning technique: DL methods Initial and Final RRC States [CONNECTED] ], mode: UE-B |  |  |  |
| --- | --- | --- | --- |
| Latency Component |  | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |  | 1 | Estimate |
| Step 2: The UE decodes the LPP Request location |  | 10 | NAS message |
| Step 3: The UE transmits MG request in an RRC message. |  | 1 | Estimate |
| Step 4: The gNB decodes the MG request message. |  | 5 | NAS message |
| Step 5: The gNB configures MG in an RRC message |  | 1 | RRC message |
| Step 6: The UE receive the MG configuration message and apply the MG configuration |  | 10 | RRC message decoding including MG configuration |
| Step 7: The UE receives PRS in the MG |  | 2 | Estimate |
| Step 8: UE calculates its location |  | 20-40 | Estimate |
| Step 9: UE reports its location |  | 2 | The UE prepares PUSCH (1ms), uses type-1 configured grant to report the positioning measurement (1ms). |
| Total values | From Step 1 to Step 8 | 50-70ms | Applicable for the scenario starting at the step “the UE receives PDSCH from the gNB carrying the LPP Request Location Information message” and also applicable for Alt 2 |
|  | From Step 1 to Step 9 | 52-72ms |  |
|  | From Step 3 to Step 8 | 39-59ms | Applicable for Alt 1 |
|  | From Step 3 to Step 9 | 41-61ms |  |
|  | From Step 7 to Step 8 | 22-42ms | Applicable for Alt 3. |
|  | From Step 7 to Step 9 | 24-44ms |  |

Table C.1.2.5.2-3: Latency analysis for UE-assisted UL methods (Case 3) [16]

| Source [NW]/Destination [NW] Positioning technique: UL methods Initial and Final RRC States [CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Reception by the gNB of the NRPPa measurement request message from LMF | NA | Delay at core. |
| Step 1: gNB send SRS configuration via RRC | 1 | Estimate |
| Step 2: UE decodes the RRC message including SRS configuration message | 10 | RRC message |
| Step 3: UE transmit SRS | 1 | Estimate |
| gNB perform measurement and transmit the NRPPa measurement response message | NA | Delay at core |
| Total values | 12ms | Note: In this analysis we only consider the periodic SRS configuration. For aperiodic and semi-persistent SRS configuration, the PHY latency will be increased. |

Table C.1.2.9.2-4: Latency analysis for UE-assisted DL+UL methods (Case 4) [16]

| Source [NW]/Destination [NW] ], mode: UE-A Positioning technique: DL+UL methods |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | 1 | Estimate |
| Step 2: The UE decodes the LPP Request location | 10 | NAS message |
| Step 3: The UE transmits measurement gap (MG) request in an RRC message. | 1 | Estimate |
| Step 4: The gNB successfully decode the MG request message | 5 | RRC message |
| Step 5: The gNB configures MG in an RRC message | 1 | Estimate |
| Step 6: The UE receive the MG configuration message and apply the MG configuration | 10 | RRC message |
| Step 7: The UE receives PRS in the MG | 2 |  |
| Reception by the gNB of the NRPPa measurement request message | NA | Core message. |
| Step 8: The gNB send SRS configuration via RRC | 1 | Estimate |
| Step 9: The UE decodes the RRC message including SRS configuration | 10 | RRC message |
| Step 10: The UE transmit SRS | 1 | Estimate |
| Step 11: The UE reports the positioning measurement and the gNB successful decoding of the PUSCH carrying the LPP Provide Location Information message | 3 |  |
| Total values | 45ms |  |

C.1.2.10 Results from source [17]

C.1.2.10.1	Description of evaluation scenarios

C.1.2.10.2	Latency analysis for Rel.16 solutions

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning is provided in Table C.1.2.1.20-1.

Table C.1.2.1.20-1: Rel.16 NR positioning latency

| UE-Assisted DL-only Positioning, RRC Connected State |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Step 1: Serving gNB sends PDSCH that contains the Location Information Request and is successfully decoded in physical layer by the UE | 1 - 2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 2: UE interprets and applies the LPP message (Location Request) | 10 | RRC Processing time are captured in 38.331 Section 12 |
| Step 3: Request of Location Information has be received at the UE. UE transmits in PUSCH an MG-request through an RRC message | 1 - 4 | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 4: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 5: Serving gNB transmits the RRC configuration of the MG request | 1 - 2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 6: UE receives the message, interprets and applies the MG configuration | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 7: UE receives at least one occasion of the PRS within the MG. | 20 - 640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 8: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 - 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 9: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | 2 - 5 | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total values | 57 - 823 |  |

| UE-based DL-only Positioning, RRC Inactive State, External Client |  |  |
| --- | --- | --- |
| Steps | “Best-case” Estimate of Duration (msec) in NR Rel-16 | “Conservative” Estimate of the Duration (msec) in NR Rel-16 |
| Step 1: Request of Location Information has been received at the UE. UE transmits in PUSCH an MG-request through an RRC message | 1-4 | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 2: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 3: Serving gNB transmits the RRC configuration of the MG request | 1-2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 4: UE receives the message, interprets and applies the MG configuration | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 5: UE receives at least one occasion of the PRS within the MG. | 20-640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 6: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2-140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 7: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | 2-5 | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total PHY-layer Latency (msec) | 46-811 |  |

| UE-based DL-only Positioning, RRC Connected State, UE Internal-client |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Step 1: UE receives at least one occasion of the PRS | 20 - 640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 2: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 - 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 3: UE transition to RRC Connected state | 11.3 - 18.5 | Transition from RRC Inactive to Connected state. 11.3 ms – 18.5 ms dependent on configuration/deployment (TR 37.910 [8]). Assume an "average" value. |
| Step 4: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | 2 - 5 | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Total values | 35.3 - 803.5 |  |

| UE-based DL-only Positioning, RRC Inactive State, UE internal-client |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Step 1: UE receives at least one occasion of the PRS | 6 - 640 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (N,8), with N<8. We assume that a reasonable PRS instance in NR Rel-16 FR1 is 6 msec. Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 2: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 - 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Total values | 8 - 780 |  |

| UE-Assisted MRTT Positioning, RRC Connected State |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Step 1: Serving gNB sends PDSCH that contains the Location Information Request and is successfully decoded in physical layer by the UE | 1 – 2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 2: UE interprets and applies the LPP message (Location Request) | 10 | RRC Processing time are captured in 38.331 Section 12 |
| Step 3: Request of Location Information has be received at the UE. UE transmits in PUSCH an MG-request through an RRC message | 1 - 4 | Transmission of a small package that includes an RRC message with an MG-request could be as low as 1 msec in FDD with SCS 30 KHz and Cap-2. However, for Cap-1 UEs, in TDD, where there can be fewer occasions for PUSCH transmission, it may be required longer period of time for the package to be transmitted through the air interface |
| Step 4: Serving gNB successfully decodes and interprets the PHY-layer message that contains the MG-request | 10 | There are no RRC processing times for gNBs in the specification. We assume the same processing time as for the UE which are captured in 38.331 Section 12. |
| Step 5: Serving gNB transmits the RRC configuration of the MG request | 1 – 2 | “Best-case” corresponds to the case of cap-2 UE with 30 KHz SCS and FDD. |
| Step 6: UE receives the message, interprets and applies the MG configuration | 10 | RRC Processing time are captured in 38.331 Section 12. |
| Step 7: UE receives at least one occasion of the PRS within the MG. | 20-640 | A UE may have applied the MG configuration at the end of a previous MG instance, so the UE would have to way for a whole PRS period (or MG period, which ever is larger). The minimum MGRP in NR rel-16 is 20 msec. So, even if the UE can produce a report using one occasion, the best-case latency would be 20 msec (assuming all required PRS resources are unmuted within the single PRS occasion). Typically more than one occasions are needed for accurate measurement (see e.g. RAN4 discussions in NR Rel-16). As an example, we assume that at least 4 occasions are needed and a typical PRS periodicity and MGRP would be of the order of 160 msec. |
| Step 8: UE finishes the processing T-N time after the end of the last PRS occasion where (N,T) is the reported capability of processing N msec of PRS every T | 2 - 140 | The highest UE capability in NR Rel-16 is to process 6 msec PRS every 8 msec, (N,T) = (6,8), so, after the end of a PRS occasion that is 6 msec long, a UE requires at least 2 msec of processing before a new PRS occasion is received. A more conservative estimate would be a UE reporting (N,T)=(20,160) msec, in which case a UE may require 140 msec to complete the processing (UE receives 20 msec worth of PRS, and then kick-offs the processing after the end of the PRS, before a new PRS occasion is received). |
| Step 9: UE reports the measurements and are decoded successfully by the gNB. NAS container is ready to be forwarded to the AMF | 2 - 5 | Large UL payload needed. Even with cap-2 UE in FDD, 1 msec would not be possible. Note: Does not involve any required time for the gNB processing to have the NAS container ready to be forwarded to the AMF. |
| Step 10: All gNBs processes the transmitted SRS in parallel. | 2 | Assumption: SRS configuration is received before Step 1, and SRS can be triggered with DCI. |
| Total values | 59 - 823 | Latency needed from the time applies the location request up to the decoding of the PUSCH carrying the Positionign report. |

C.1.2.11 Results from source [15]

C.1.2.11.1	Description of evaluation scenarios

We assumed the UEs in the RRC connected mode for the physical layer latency analysis, and we provide the latency analysis on the physical layer only as well as physical layer latency with the consideration of higher layer. The following cases are considered.

- Case 1: DL-TDOA/DL-AoD positioning method for the NW initiated

- Case 2: DL-TDOA/DL-AoD positioning method for the UE initiated

- Case 3: UL-TDOA/UL-AoA positioning method for the NW initiated

- Case 4: Multi-RTT positioning method for the NW initiated

- Case 5: E-CID positioning method for the NW initiated

To evaluate physical layer latency for the enhanced NR positioning solution(s), we use the following assumptions as below:

- Numerology: 15 kHz (for PDCCH, PDSCH, PUCCH, PUSCH, SRS)

- For PUSCH transmission:

- Uplink switching gap is not configured.

- No BWP switching

- No overlapping symbols of the PUCCH and the scheduled PUSCH

- The number of PUSCH symbols = from 4 to 14 for Type A

- The number of PUSCH symbols = from 1 to 14 for Type B

- According to the size of data (e.g. LPP message) to be transmitted, the number of symbols can be changed.

- For PDSCH transmission:

- No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH

- The number of PDSCH symbols = from 3 to 14 for Type A

- The number of PDSCH symbols = from 2 to 14 for Type B

- According to the size of data (e.g. LPP message) to be transmitted, the number of symbols can be changed.

- For SRS transmission:

- One shot transmission (2 OS ~ 12 OS)

- According to resource type (periodic, Semi-persistent and aperiodic), the number of symbols can be changed.

- For PUCCH transmission (Scheduling request):

- One shot transmission

- Periodicity and offset for PUCCH: 2 OS ~ 80 slots

- The length of symbols for PUCCH: 1 OS~ 14 OS (format 0 and 2)

- According to PUCCH configuration (e.g. periodicity and offset, symbol length of PUCCH), the latency can be changed.

- For PDCCH transmission (UL grant):

- Slot for PDCCH Monitoring configured as periodicity and offset is 1slot (format 0_0, 0_1) even though 2560 slots can be configured.

- The first symbol(s) for PDCCH monitoring in the slots is zero

- The length of symbols for CORESET: 1 OS ~3 OS

- According to PUCCH configuration (e.g. periodicity and offset, symbol length of PDCCH), the latency can be changed.

C.1.2.11.2	Latency analysis for Rel.16 solutions

Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest oneLatency analysis for the Rel.16 NR positioning is provided in Table C.1.2.11.2-1- Table C.1.2.11.2-5.

Table C.1.2.11.2-1: Rel.16 NR positioning latency for DL method (Case 1) [15]

| Source/Destination: NW/UE Positioning technique DL-TDOA and DL-AOD, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Request Location Information message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Request Location Information message (RRC processing time, UE) | 5 ms ~ 5.5 ms | 38.331 section 12. RRC DL information transfer at UE : 5 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| *Constructing measurement gap request message (RRC processing time, UE) | 2 ms ~ 5 ms | 38.331 section 12. RRC UL information transfer: 2 - 5ms |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request (RRC processing time, gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3ms |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 3 ms | 38.331 section 12. RRC Processing at gNB : 3ms |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of measurement gap configuration (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5. The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| *Constructing scheduling request message (RRC processing time, UE) | 2 ms ~ 5 ms | 38.331 section 12. RRC UL information transfer: 2 - 5ms |
| Transmission of scheduling request for measurement report (UEgNB) | 0.21ms~81ms | 38.213 section 9.2.4 38.331 section 6.3.2 |
| *Reception of scheduling request and constructing UL grant (gNB) | 6 ms ~ 6.5 ms | RRC Processing at gNB : 3*2 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| Transmission of UL grant(gNB UE ) | 0.07ms~0.21ms | 38.213 section 10.1 |
| UL grant reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of UL grant (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| *Constructing Provide Location information message(RRC processing time, UE) | 5 ms | 38.331 section 12. Processing delay for provide Location Information at UE : 5 ms |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3 ms |
| Total values | For UE capability-1: 62.97 ms ~ 297.11ms For UE capability-2: 61.17 ms ~ 293.68 ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 23.97ms ~ 249.11ms For UE capability-2: 22.17ms ~ 245.68ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.1.2.11.2-2: Rel.16 NR positioning latency for DL method (Case 2) [15]

| Source/Destination: UE/NW Positioning technique DL-TDOA, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request and preparation time for measurement gap(gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3ms |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 3 ms | 38.331 section 12. RRC Processing at gNB : 3ms |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of measurement gap configuration (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5 The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| *Constructing scheduling request message (RRC processing time, UE) | 2 ms ~ 5 ms | 38.331 section 12. RRC UL information transfer: 2 - 5ms |
| Transmission of scheduling request for measurement report (UEgNB) | 0.21ms~81ms | 38.213 section 9.2.4 38.331 section 6.3.2 |
| *Reception of scheduling request And preparation time for UL grant (gNB) | 6 ms ~ 6.5 ms | RRC Processing at gNB : 3*2 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| Transmission of UL grant(gNB UE ) | 0.07ms~0.21ms | 38.213 section 10.1 |
| UL grant reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of UL grant (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| *Constructing Provide Location information message(RRC processing time, UE) | 5 ms | 38.331 section 12. Processing delay for provide Location Information at UE : 5 ms |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3 ms |
| Total values | For UE capability-1: 55.26ms ~ 284.83ms For UE capability-2: 53.82ms ~ 282.97ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 23.26ms ~ 247.33ms For UE capability-2: 21.82ms ~ 245.47ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.1.2.11.2-3: Rel.16 NR positioning latency for UL method (Case 3) [15]

| Source/Destination: NW/UE Positioning technique UL-TDOA and UL-AOA, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| SRS configuration (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of SRS configuration (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of SRS configuration message (RRC processing time, UE) | 10 ms ~ 10.5 ms | 38.331 section 12. RRC Reconfiguration processing at UE : 10 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| *Reception of SRS activation message (RRC processing time, gNB UE) | 1 ms ~ 3.5 ms | 38.331 section 12. Processing delay of MAC-CE SRS Activation at UE: 1 ms ~ 3 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| SRS transmission (UE) | 0.07ms ~ 0.86ms | 38.214 section 6.2.1. |
| *Reception of SRS and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | NRPPa Processing delay at gNB Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| Total values | For UE capability-1: 14.78 ms ~ 20.14 ms For UE capability-2: 14.42 ms ~ 19.57 ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 0.78 ms ~ 2.64ms For UE capability-2: 0.42ms ~ 2.07ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.1.2.11.2-4: Rel.16 NR positioning latency for DL+UL method (Case 4) [15]

| Source/Destination: NW/UE Positioning technique: Multi-RTT, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| *Constructing Provide assistance message (RRC processing time, gNB) | 3 ms | 38.331 section 12. |
| Transmission of LPP provide assistance data (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Reception of LPP provide assistance data (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of LPP provide assistance data (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| *Constructing request location information (RRC processing time, gNB) | 3 ms | 38.331 section 12. RRC Processing at gNB : 3ms |
| Transmission of Request Location Information message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| * Transportation of Request Location Information message (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| *Constructing measurement gap request message (RRC processing time, UE) | 2 ms ~ 5 ms | 38.331 section 12. RRC UL information transfer: 2 - 5ms |
| measurement gap request (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement gap request (RRC processing time, gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3ms |
| *Constructing measurement gap configuration message(RRC processing time, gNB) | 3 ms | 38.331 section 12. RRC Processing at gNB : 3ms |
| Measurement gap configuration message (gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Measurement gap configuration message reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of measurement gap configuration (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| PRS measurement within measurement gap (UE) | 20ms ~ 160ms | 38.133 section 9.9.2.5 The requirement for measurement period is represented by LCM (PRS resource periodicity, repetition periodicity of the measurement gap) |
| SRS transmission (UE) | 0.07ms ~ 0.86ms | 38.214 section 6.2.1. |
| *Reception of SRS and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | NRPPa Processing delay at gNB Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| *Constructing scheduling request message (RRC processing time, UE) | 2 ms ~ 5 ms | 38.331 section 12. RRC UL information transfer: 2 ms - 5ms |
| Transmission of scheduling request for measurement report (UEgNB) | 0.21ms~81ms | 38.213 section 9.2.4 38.331 section 6.3.2 |
| *Reception of scheduling request and constructing UL grant (gNB) | 6 ms ~ 6.5 ms | RRC Processing at gNB : 3*2 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| Transmission of UL grant(gNB UE ) | 0.07ms~0.21ms | 38.213 section 10.1 |
| UL grant reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of UL grant (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| *Constructing Provide Location information message(RRC processing time, UE) | 5 ms | 38.331 section 12. Processing delay for provide Location Information at UE : 5 ms |
| measurement report transmission(including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3 ms |
| Total values | For UE capability-1: 77.75 ms ~314.75 ms For UE capability-2: 75.59 ms ~ 311.75 ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 24.75 ms ~ 251.75 ms For UE capability-2: 22.59 ms ~ 248.75 ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.

Table C.1.2.11.2-5: Rel.16 NR positioning latency for E-CID (Case 5) [15]

| Source/Destination: NW/UE Positioning technique: E-CID, mode: UE-A Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| LPP Request Location Information message transmission(gNB UE) | 0.14 ms ~ 1 ms | 38.214 section 5.1.2. |
| Request Location Information message Reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Transportation of Request Location Information message (RRC processing time, UE) | 5 ms ~ 5.5 ms | 38.331 section 12. RRC DL information transfer at UE : 5 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| *Constructing Provide Location information message(RRC processing time, UE) | 5 ms | 38.331 section 12. Processing delay for provide Location Information at UE : 5 ms |
| *Constructing scheduling request message (RRC processing time, UE) | 2 ms ~ 5 ms | 38.331 section 12. RRC UL information transfer: 2 - 5ms |
| Transmission of scheduling request for measurement report (UEgNB) | 0.21ms~81ms | 38.213 section 9.2.4 38.331 section 6.3.2 |
| *Reception of scheduling request And preparation time for UL grant (gNB) | 6 ms ~ 6.5 ms | RRC Processing at gNB : 3*2 ms Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms |
| Transmission of UL grant(gNB UE ) | 0.07ms~0.21ms | 38.213 section 10.1 |
| UL grant reception (PHY Processing time for PDSCH, UE) | For UE capability-1: 0.57 ms ~ 0.78 ms For UE capability-2: 0.21 ms | 38.214 section 5.3. |
| *Reception of UL grant (RRC processing time, UE) | 5 ms ~ 5.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC DL information transfer at UE : 5 ms |
| Transmission of measurement report (including PHY preparation time for PUSCH, UE) | For UE capability-1: 0.85 ms ~ 1.78 ms For UE capability-2: 0.49 ms ~ 1.42 ms | 38.214 section 6.4. 38.214 section 6.1.2. |
| *Reception of measurement report and preparation time for gNB to make NRPPa message (gNB) | 3 ms ~ 3.5 ms | Signalling Propagation Delays between Nodes and UE: 0 ms – 0.5 ms RRC Processing at gNB : 3 ms |
| Total values | For UE capability-1: 28.41ms ~ 116.55 ms For UE capability-2: 27.33 ms ~ 115.05 ms | When procedure marked with(*) is excluded, total values shall be amended as follows: For UE capability-1: 2.41 ms ~ 85.55 ms For UE capability-2: 1.33 ms ~ 84.05 ms |

Note 1: Some components denoted by (*) can be excluded since they are related with higher layer procedure.

Note 2: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed.