3GPP TR 38.857 V0.3.0 (2020-11)

Technical Report

3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on NR Positioning Enhancements; (Release 17)

| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |
| --- | --- |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	General description of NR positioning enhancements	8

5	Target requirements for NR positioning enhancements in Rel-17	9

5.1 	Target requirements	9

5.2 	Performance evaluation metrics	9

5.2.1	Horizontal accuracy	10

5.2.2	Vertical accuracy	10

5.2.3	 Other metrics	10

5.2.3.1	Latency	10

5.2.3.1.1	Physical layer Latency	10

5.2.3.1.2	Higher layer Latency	10

5.2.3.2	Network efficiency	10

5.2.3.3	Device efficiency	10

6 	Additional scenarios and channel models for NR positioning enhancements	11

6.1 	IIoT use cases	13

6.2 	General commercial use cases	16

7	Studied NR positioning enhancements	16

8	Performance evaluations for Rel-17 targets	17

8.1	Performance analysis of Rel-16 positioning solutions	17

8.1.1	Positioning accuracy analysis	17

8.1.1.1	Observations  from source [4]	17

8.1.1.2	Observations  from source [7]	20

8.1.1.3	Observations  from source [8]	20

8.1.1.4	Observations  from source [13]	23

8.1.1.5	Observations  from source [5]	24

8.1.1.6	Observations  from source [12]	31

8.1.1.7	Observations  from source [10]	32

8.1.1.8	Observations  from source [14]	33

8.1.1.9	Observations  from source [20]	33

8.1.1.10	Observations  from source [17]	34

8.1.1.11	Observations  from source [18]	40

8.1.2	Physical layer latency analysis for Rel-16	40

8.1.2.1	Observations  from source [4]	40

8.1.2.2	Observations  from source [7]	40

8.1.2.3	Observations  from source [8]	41

8.1.2.4	Observations  from source [13]	41

8.1.2.5	Observations  from source [11]	41

8.1.2.6	Observations  from source [5]	42

8.1.2.7	Observations  from source [12]	43

8.1.2.8	Observations  from source [10]	43

8.1.2.9	Observations  from source [16]	44

8.1.2.10	Observations  from source [17]	44

8.1.2.11	Observations  from source [15]	44

8.2	Performance analysis of studied NR positioning enhancements	45

8.2.1	Positioning accuracy analysis for NR positioning enhancements	45

8.2.1.1	Observations  from source [4]	45

8.2.1.2	Observations  from source [7]	48

8.2.1.3	Observations  from source [8]	48

8.2.1.4	Observations  from source [13]	48

8.2.1.5	Observations  from source [5]	48

8.2.1.6	Observations  from source [12]	62

8.2.1.7	Observations  from source [10]	63

8.2.1.8	Observations  from source [18]	65

8.2.1.9	Observations  from source [14]	67

8.2.1.10	Observations  from source [21]	69

8.2.1.11	Observations from source [17]	70

8.2.1.12	Observations from source [9]	74

8.2.2	Physical layer latency analysis for NR positioning enhancements	74

8.2.2.1	Observations  from source [4]	74

8.2.2.2	Observations  from source [8]	75

8.2.2.3	Observations  from source [5]	75

8.2.2.4	Observations  from source [10]	75

8.2.2.5	Observations  from source [16]	76

8.2.2.6	Observations from source [17]	76

8.2.2.7	Observations  from source [15]	76

8.3	Efficiency analysis for NR positioning enhancements	77

8.3.1	Network efficiency analysis for NR positioning enhancements	77

8.3.1.1	Observations  from source [4]	77

8.3.1.2	Observations  from source [8]	77

8.3.1.3	Observations  from source [5]	77

8.3.1.4	Observations  from source [12]	78

8.3.1.5	Observations  from source [21]	78

8.3.1.6	Observations  from source [18]	78

8.3.2	UE efficiency analysis for NR positioning enhancements	78

8.3.2.1	Observations  from source [4]	78

8.3.2.2	Observations  from source [5]	79

8.4	Summary of performance evaluations	82

9	Positioning integrity and reliability	88

10	Identified NR impacts in Rel-17	88

10.1 	NR positioning for UEs in RRC_INACTIVE state	88

10.2	On-demand transmission and reception of DL PRS	88

10.3	Aggregation of DL PRS resources	89

10.4	Aggregation of SRS for positioning resources	89

10.5	Enhancements for  UE Rx/Tx and gNB Rx/Tx timing delays	89

10.6	Enhancements for angle based methods	89

10.7	Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation	89

10.8	Enhancements of signaling & procedures for reducing NR positioning latency	89

10.9	DL positioning measurement in RRC_IDLE state	90

11	Conclusions	90

Annex A: Change history	94

Annex B: Appendix to summary of performance evaluations	95

B.1 Evaluation of  horizontal positioning accuracy with gNB/UE TX/RX timing error	95

B.2 Evaluation of  physical layer latency	100

Annex C:	Appendix to Performance evaluations for R17 performance targets	113

C.1	Performance analysis of Rel-16 positioning solutions	113

C.2	Performance of studied NR positioning enhancements	113

C.3	Efficiency analysis for NR positioning enhancements	113

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document captures the findings of the study item "Study on NR positioning enhancements" [2]. The purpose of this technical report is to document the requirements, additional scenarios, evaluations and technical proposals treated during the study and provide a way forward toward enhancements to NR positioning in TSG RAN WGs.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-193237: "new SID on NR Positioning Enhancements".

[3]	3GPP TR 38.855: "Study on NR Positioning (Release 16)".

[4]		R1-2009433	Evaluation results for Rel-16 positioning and Rel-17 enhancement	Huawei, HiSilicon

[5]	R1-2007665	Evaluation of NR positioning performance	vivo

[6]	R1-2007720	Evaluation of achievable positioning accuracy	BUPT

[7]	R1-2007754	Evaluation of achievable accuracy and latency	ZTE

[8]	R1-2007859	Discussion of evaluation of NR positioning performance	CATT

[9]	R1-2007908	NLOS Identification and Mitigation	FUTUREWEI

[10]	R1-2009390	Update of Evaluation Results for NR Positioning Performance in I-IoT Scenarios	Intel Corporation

[11]	R1-2007997	NR Positioning Latency Evaluations	Lenovo, Motorola Mobility

[12]	R1-2008225	Evaluation of NR positioning in IIOT scenario	OPPO

[13]	R1-2009555	Results on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

[14]	R1-2009502 Discussion on Performance evaluation of Rel-17 positioning	Sony

[15]	R1-2008416	Discussions on evaluation of achievable positioning accuracy and latency for NR positioning	LG Electronics

[16]	R1-2008489	Evaluation of achievable positioning latency	InterDigital, Inc.

[17]	R1-2009708	Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

[18]	R1-2009428	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

[19]	R1-2008720	Positioning evaluation results on potential enhancements for additional use cases	CeWiT

[20]	R1-2008764	Evaluation of achievable positioning accuracy and latency	Ericsson

[21]	R1-2008765	Potential positioning enhancements	Ericsson

[22]	R1-2007666	Discussion on potential positioning enhancements	 vivo

[23]	R1-2005380	Evaluation of achievable positioning accuracy and latency 	viv

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	General description of NR positioning enhancements

(General description of NR positioning up to release 16 & NR positioning enhancements in rel17)

3GPP NR radio-technology is uniquely positioned to provide added value in terms of enhanced location capabilities. The operation in low and high frequency bands (i.e. below and above 6GHz) and utilization of massive antenna arrays provides additional degrees of freedom to substantially improve the positioning accuracy. The possibility to use wide signal bandwidth in low and especially in high bands brings new performance bounds for user location for well-known positioning techniques, utilizing timing measurements to locate UE. The recent advances in massive antenna systems can provide additional degrees of freedom to enable more accurate user location by exploiting spatial and angular domains of propagation channel in combination with time measurements.

3GPP Rel-16 has specified various location technologies to support regulatory as well as commercial use cases. The target horizontal positioning requirements for commercial use cases studied in Rel-16 were <3 m (80%) for indoor scenarios and <10 m (80%) for outdoor scenarios (TR 38.855[3]). The 5G service requirements specified in TS 22.261 [24] include High Accuracy Positioning requirements, which are characterized by ambitious system requirements for positioning accuracy in many verticals. For example, on the factory floor, it is important to locate assets and moving objects such as forklifts, or parts to be assembled. Similar needs exist in transportation and logistics, for example.

To address the higher accuracy location requirements resulting from new applications and industry verticals for 5G, a Rel-17 Study Item of “Study on NR Positioning Enhancements” was approved by 3GPP TSG RAN [2][25]. The study item covers the enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically IIoT use cases).

This technical report documents the following accomplishments obtained during the study:

    - [the target performance requirements for RAT dependent solutions for Rel-17 for both general commercial use cases and IIoT use cases;]

    - the additional scenarios and channel models for evaluating NR positioning enhancements;

    - the NR positioning enhancements candidates for improving accuracy, reducing latency, and improving network and device efficiency for Rel-17;

    - evaluation of the achievable positioning performance, including the performance analysis of Rel-16 positioning solutions, the performance analysis, the efficiency analysis, and the observations obtained from the investigations for Rel-17 NR positioning enhancements;

    - the identified NR impacts for normative work for Rel-17.

5	Target requirements for NR positioning enhancements in Rel-17

5.1 	Target requirements

In Rel-17 target positioning requirements for commercial use cases are defined as follows:

    - Horizontal position accuracy (< 1 m) for 90% of UEs

    - Vertical position accuracy (< 3 m) for 90% of UEs

    - End-to-end latency for position estimation of UE (< 100 ms)

    - Physical layer latency for position estimation of UE (< 10 ms)

In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

    - Horizontal position accuracy (< 0.2 m) for 90% of UEs

    - Vertical position accuracy (< 1 m) for 90% of UEs

    - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

    - Physical layer latency for position estimation of UE (<10ms)

Note 1: Target positioning requirements may not necessarily be reached for all scenarios and deployments

Note 2: For some scenarios the requirement for Horizontal position accuracy can be relaxed to < 0.5 m in IIoT use cases.

Note 3: All positioning techniques may not achieve the target positioning requirements over all scenarios

5.2 	Performance evaluation metrics

(Includes horizontal accuracy vertical accuracy and other metrics)

For evaluating performance of NR positioning technologies, the following metrics apply. The following percentiles of positioning error are analyzed: 50%, 67%, 80%, 90%.

5.2.1	Horizontal accuracy

Horizontal accuracy is the difference between the calculated horizontal position and the actual horizontal position of a UE.

5.2.2	Vertical accuracy

Vertical accuracy is the difference between the calculated vertical position and the actual vertical position of a UE.

5.2.3		Other metrics

5.2.3.1	Latency

5.2.3.1.1	Physical layer Latency

Latency includes higher layer and physical layer latency. Physical layer latency for DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted approaches are separately studied

The physical layer latency start- and end-time are defined for each positioning method in table 5.2.3.1-1

Table 5.2.3.1-1: Definition of physical layer latency start- and end-time

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| UL-only method & UL ECID & Multi-RTT | Reception by the gNB of the NRPPa measurement request message | The transmission by the gNB of the NRPPa measurement response message |
| UE-based | - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE |

5.2.3.1.2	Higher layer Latency

5.2.3.2	Network efficiency

PRS/SRS resource utilization is the metric used to evaluate network efficiency.

5.2.3.3	Device efficiency

The UE power consumption models developed in TR38.840 can be considered as the starting point for defining the UE power consumption model for the evaluation for NR positioning. For evaluations, it is up to each company to detail their methodology (including the power model) for evaluation.

6 	Additional scenarios and channel models for NR positioning enhancements

From justification, for the evaluation of solutions, the Rel-16 scenarios and channel models in TR 38.855 are reused where applicable, and additional scenarios for IIoT use cases should be defined.

from objective 1a. Includes definition of  additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases e.g. (I)IoT)

The scenario parameters common to all the scenarios in the study are detailed in table 6-1. Additionally, blockage model is not considered. For evaluations including UE mobility, the spatial consistency procedure defined in TR 38.901 is taken into consideration.

The evaluation methodology does not define any baseline reference signals. Configurations of DL PRS and SRS supported by Rel-16 specifications are used for evaluation of the achievable performance based on Rel-16 positioning technologies.

Table 6-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: 4-panels UE: - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: [X] ns for gNB and [Y] ns for UE - X and Y are up to companies - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

6.1 	IIoT use cases

For evaluating baseline performance, the following scenarios (with various options/configurations) are defined for RAT-dependent positioning techniques for the NR positioning enhancements study

-	Scenario 1. InF-SH for FR1 and FR2

-	Scenario 2. InF-DH for FR1 and FR2

In the evaluation of all scenarios, the absolute-time-of arrival model defined in TR 38.901 is considered, without modification. Parameters specific to scenario 1and 2 are detailed in table 6.1-1

Table 6.1-1: Parameters common to InF scenarios

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be - (baseline) at least the convex hull of the horizontal BS deployment. - (optional) It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(Inf-SH) and X2= for scenario 2 (InF-DH) |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: - Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height - (Optional): {40%, 3m, 5m} - (Optional): {60%, 6m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

6.2 	General commercial use cases

For general commercial use cases, Rel-16 scenarios and channel models in TR 38.855 are reused. For the absolute time of arrival modelling in IOO, UMa, Umi, companies may provide the details of their model, if any.

7	Studied NR positioning enhancements

(from objective 1c. Includes positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency for both RAN1 and RAN2.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case.  )

The following enhancements have been considered during this study:

- Partial staggering and non-staggering RE mapping of SRS for positioning with different combinations of comb-factors and symbol lengths, including the methods/signalling for addressing potential time-domain aliasing due to the partial/non-staggering RE mapping.

- Semi-persistent and a-periodic transmission and reception of DL PRS

  - Semi-persistent means MAC-CE triggered

  - Aperiodic would correspond to DCI-triggered

- On-demand transmission and reception of DL PRS

  - On-demand corresponds to the UE-initiated or network-initiated request of PRS and/or SRS, i.e. UE or LMF request/suggesting/recommending specific PRS pattern, ON/OFF, periodicity, BW, etc.

- Multipath mitigation techniques including but not limited to the following:

  - The applicable scenarios and performance benefits of multipath mitigation techniques

  - The methods/measurement/signaling for the LOS/NLOS detection and identification

  - The measurements for supporting the multipath mitigation/utilization

  - The procedure and signaling for supporting the multipath mitigation/utilization

  - Implementation-based solutions (e.g., outlier rejection) without the need of any additional specified method/measurements/procedures/signaling.

  - Note: The above study applies to DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted positioning.

- NR positioning for UEs in RRC_IDLE state and UEs in RRC_INACTIVE state, including the benefits on latency, network/UE efficiency and UE power consumption

- For reducing NR positioning latency, more efficient signaling & procedures enabling a device to request and report positioning information, which may include, but not limited to, the following aspects:

  - DL PRS/SRS configuration, activation or triggering.

  - The request for positioning information (the assistance data, etc.).

  - The report of positioning information (the measurement report, etc.).

  - Note: It is not within RAN1 scope to analyze positioning architecture enhancements to enable such more efficient signaling & procedures.

  - Note: RAN1 does not make any assumptions on whether the LCS architecture specified in TS 23.273 is enhanced or not.

- Simultaneous transmission by the UE and reception by the gNB of the SRS for positioning across multiple CCs and multiple slots, including

  - The scenarios and performance benefits of the enhancement

  - The impact of channel spacing, TA and timing offset, phase offset, frequency error, and power imbalance across slots or CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

- Scenario, benefits, and methods for improving the accuracy of the UL AoA and DL-AoD methods for both UE-based and network-based (including UE-assisted) positioning

- Scenario, benefits, methods and signaling for improving positioning accuracy in the presence of the UE Rx/Tx transmission delays, and/or and gNB Rx/Tx transmission delays for UE-based and network-based (including UE-assisted) positioning.

- Aggregating multiple DL positioning frequency layers of the same or different bands for improving positioning performance for both intra-band and inter-band scenarios

- The scenarios and performance benefits of aggregating multiple DL positioning frequency layers

- The impact of channel spacing, timing offset, phase offset, frequency error, and power imbalance among CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

- UE complexity considerations

8	Performance evaluations for Rel-17 targets

8.1	Performance analysis of Rel-16 positioning solutions

Including accuracy and latency (objective 1b) performance, compared to rel17 performance targets

8.1.1	Positioning accuracy analysis

8.1.1.1	Observations from source [4]

Table 8.1.1.1-1 captures observations based on NR positioning evaluations results for horizontal location error for baseline scenarios.

Table 8.1.1.1-2 captures observations based on NR positioning evaluations results for horizontal location error for modified DH and 3D positioning.

Table 8.1.1.1-3 captures observations based on NR positioning evaluations results for horizontal location error for UE/gNB calibration error.

Table 8.1.1.1-4 captures observations based on NR positioning evaluations results for vertical location error for modified DH and 3D positioning.

Table 8.1.1.1-1: Rel.16 NR positioning (baseline) – horizontal accuracy performance summary [4]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| 1, InF-SH, FR1, DL-TDOA | 1.964 | 0.964 | 1.764 | 1.464 |
| 2, InF-SH, FR1, UL-TDOA | 1.0277 | 0.0277 | 0.8277 | 0.5277 |
| 3, InF-SH, FR1, UL-TDOA/AoA | 0.2682 | Yes | 0.0682 | Yes |
| 4, InF-SH, FR1, Multi-RTT | 1.6992 | 0.6992 | 1.4992 | 1.1992 |
| 5, InF-DH422, FR1, DL-TDOA | 15.635 | 14.635 | 15.435 | 15.135 |
| 6, InF- DH422, FR1, UL-TDOA | 9.6631 | 9.163 | 9.963 | 9.663 |
| 7, InF-DH422, FR1, UL-TDOA/AoA | 0.8016 | Yes | 0.6016 | 0.3016 |
| 8, InF- DH422, FR1, Multi-RTT | 7.311 | 6.311 | 7.111 | 6.811 |
| 9, InF-SH, FR2, DL-TDOA | 0.9633 | Yes | 0.7633 | 0.4633 |
| 10, InF-SH, FR2, DL-TDOA/AoD | 0.0654 | Yes | Yes | Yes |
| 11, InF-SH, FR2, UL-TDOA/AoA | 0.0694 | Yes | Yes | Yes |
| 12, InF-SH, FR2, Multi-RTT | 0.4496 | Yes | 0.2496 | Yes |
| 13, InF-DH422, FR2, DL-TDOA | 9.6798 | 8.6798 | 9.4798 | 9.1798 |
| 14, InF- DH422, FR2, DL-TDOA/AoD | 0.7197 | Yes | 0.5197 | 0.2197 |
| 15, InF-DH422, FR2, UL-TDOA/AoA | 0.7086 | Yes | 0.5086 | 0.2086 |
| 16, InF- DH422, FR2, Multi-RTT | 4.2895 | 3.2895 | 4.0895 | 3.7895 |

Table 8.1.1.1-2: Rel.16 NR positioning (modified DH and 3D positioning) – horizontal accuracy performance summary [4]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| 101, InF-DH435, FR1, UL-TDOA/AoA | 1.3012 | 0.3012 | 1.1012 | 0.8012 |
| 102, InF-DH435, FR1, Multi-RTT | 9.8411 | 8.8411 | 9.6411 | 9.3411 |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | 4.3405(H) | 3.3405 | 4.1405 | 3.8405 |
| 104, InF-DH435-3D, FR1, Multi-RTT | 16.0515(H) | 15.0515 | 15.8515 | 15.5515 |
| 105, InF-DH435, FR2, UL-TDOA/AoA | 1.1486 | 0.1486 | 0.9486 | 0.6486 |
| 106, InF-DH435, FR2, Multi-RTT | 5.46 | 4.46 | 5.26 | 4.96 |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | 2.4365(H) | 1.4365 | 2.2365 | 1.9365 |
| 108, InF-DH435-3D, FR2, Multi-RTT | 15.5828(H) | 14.5828 | 15.3828 | 15.0828 |

Table 8.1.1.1-3: Rel.16 NR positioning (UE/gNB calibration error) – horizontal accuracy performance summary [4]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| 201, InF-SH, FR1, DL-TDOA, Group Delay Error | 1.458 | 0.458 | 1.258 | 0.958 |
| 202, InF-SH, FR1, UL-TDOA, Group delay error | 1.2343 | 0.2343 | 1.0343 | 0.7343 |
| 203, InF-SH, FR1, UL-TDOA/AoA, Group delay error | 0.3251 | Yes | 0.1251 | Yes |
| 204, InF-SH, FR1, Multi-RTT, Group delay error | 4.2662 | 3.2662 | 4.0662 | 3.7662 |
| 205, InF-DH422, FR1, DL-TDOA, Group delay error | 15.039 | 14.039 | 14.839 | 14.539 |
| 206, InF-DH422, FR1, UL-TDOA, Group delay error | 9.4102 | 8.4102 | 9.2102 | 8.9102 |
| 207, InF-DH422, FR1, UL-TDOA/AoA, Group delay error | 0.8662 | Yes | 0.6662 | 0.3662 |
| 208, InF-DH422, FR1, Multi-RTT, Group delay error | 9.5701 | 8.5701 | 9.3701 | 9.0701 |
| 209, InF-SH, FR1, UL-AoA | 0.1119 | Yes | Yes | Yes |
| 210, InF-SH, FR1, UL-AoA, Angle error 1 degree | 1.1676 | 0.1676 | 0.9676 | 0.6676 |
| 211, InF-SH, FR1, UL-AoA, Angle error 2 degrees | 2.1732 | 1.1732 | 1.9732 | 1.6732 |
| 212, InF-SH, FR1, UL-AoA, Angle error 5 degrees | 5.3982 | 4.3982 | 5.1982 | 4.8982 |

Table 8.1.1.1-4: Rel.16 NR positioning (modified DH and 3D positioning) – vertical accuracy performance summary [4]

| Simulation case (Vertical Error) | Accuracy achieved @[90]% | Commercial vertical accuracy requirements [3]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| 103, InF-DH435-3D, FR1, UL-TDOA/AoA | 1.1585(V) | Yes | 0.9585 | 0.1585 |
| 104, InF-DH435-3D, FR1, Multi-RTT | 1.6675(V) | Yes | 1.4675 | 0.6675 |
| 107, InF-DH435-3D, FR2, UL-TDOA/AoA | 0.4593(V) | Yes | 0.2593 | Yes |
| 108, InF-DH435-3D, FR2, Multi-RTT | 1.8800(V) | Yes | 1.68 | 0.88 |

8.1.1.2	Observations from source [7]

Table 8.1.1.2-1: Rel.16 NR positioning – horizontal accuracy performance summary[7]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1 | 0.603 |
| Case 2 | 0.568 |
| Case 3 | 0.704 |
| Case 4 | 0.943 |
| Case 5 | 1.479 |
| Case 6 | 0.092 |
| Case 7 | 0.090 |
| Case 8 | 0.300 |
| Case 9 | 0.615 |
| Case 10 | 1.224 |
| Case 11 | 12.433 |
| Case 12 | 12.345 |
| Case13 | 12.386 |
| Case 14 | 12.368 |
| Case 15 | 12.458 |
| Case 16 | 14.759 |
| Case 17 | 12.174 |
| Case 18 | 10.815 |
| Case 19 | 12.285 |
| Case 20 | 14.845 |

Table 8.1.1.2-2: Rel.16 NR positioning – vertical accuracy performance summary [7]

| Simulation case (Vertical Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 21 | 0.979 |
| Case 22 | 0.459 |
| Case 23 | 1.419 |
| Case 24 | 1.271 |

8.1.1.3	Observations from source [8]

Table 8.1.1.3-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.3-1: Rel.16 NR positioning – horizontal accuracy performance summary [8]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If No, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If No, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| [Case 1], [InF-SH-2D], [FR1], [DL-TDOA] | 0.1650 | Yes | Yes |
| [Case 2], [InF-SH-2D], [FR1], [UL-TDOA] | 0.1551 | Yes | Yes |
| [Case 3], [InF-SH-2D], [FR1], [Multi-RTT] | 0.1650 | Yes | Yes |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | 0.2045 | No （0.0045） | Yes |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | 0.2574 | No （0.0574） | Yes |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | 0.2677 | No （0.0677） | Yes |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | 0.2540 | No (0.054) | Yes |
| [Case 8], [InF-DH-2D], [FR1], [DL-TDOA] | 0.1693 | Yes | Yes |
| [Case 9], [InF-DH-2D], [FR1], [UL-TDOA] | 0.1184 | Yes | Yes |
| [Case 10], [InF-DH-2D], [FR1], [Multi-RTT] | 0.1237 | Yes | Yes |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | 0.7089 | No (0.5089) | No (0.2089) |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | 0.6937 | No （0.4937） | No (0.1937) |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | 0.151 | Yes | Yes |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | 0.692 | No (0.362) | No (0.062) |
| [Case 15], [IOO], [FR1], [DL-TDOA] | 0.2288 | —— | —— |
| [Case 16], [IOO], [FR1], [UL-TDOA] | 0.1836 | —— | —— |
| [Case 17], [IOO], [FR1], [UL-TDOA+UL-AOA] | 0.1219 | —— | —— |
| [Case 18], [IOO], [FR1], [Multi-RTT] | 0.283 | —— | —— |
| [Case 19], [IOO], [FR1], [DL-TDOA] | 32.4509 | —— | —— |
| [Case 20], [IOO], [FR1], [UL-TDOA] | 32.0927 | —— | —— |
| [Case 21], [IOO], [FR1], [UL-TDOA+UL-AOA] | 9.2356 | —— | —— |
| [Case 22], [IOO], [FR1], [Multi-RTT] | 1.3668 | —— | —— |
| [Case 23], [InF-SH-2D], [FR2], [DL-TDOA] | 0.0372 | Yes | Yes |
| [Case 24], [InF-SH-2D], [FR2], [UL-TDOA] | 0.0538 | Yes | Yes |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | 0.0789 | Yes | Yes |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | 0.0817 | Yes | Yes |
| [Case 27], [InF-DH-2D], [FR2], [DL-TDOA] | 0.0388 | Yes | Yes |
| [Case 28], [InF-DH-2D], [FR2], [UL-TDOA] | 0.0553 | Yes | Yes |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | 0.7033 | No （0.5033） | No （0.2033） |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | 0.6848 | No （0.4848） | No （0.1848） |
| [Case 31], [IOO], [FR2], [DL-TDOA] | 0.0406 | —— | —— |
| [Case 32], [IOO], [FR2], [UL-TDOA] | 0.0397 | —— | —— |
| [Case 33], [IOO], [FR2], [Multi-RTT] | 0.0402 | —— | —— |
| [Case 34], [IOO], [FR2], [DL-TDOA] | 32.3809 | —— | —— |
| [Case 35], [IOO], [FR2], [UL-TDOA] | 32.0887 | —— | —— |
| [Case 36], [IOO], [FR2], [Multi-RTT] | 1.2681 | —— | —— |

Table 8.1.1.3-2 captures observations based on NR positioning evaluations results for vertical location error.

Table 8.1.1.3-2: Rel.16 NR positioning – vertical accuracy performance summary [8]

| Simulation case (Vertical Error) | Accuracy achieved @[90]% | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If No, provide performance gaps@[90]% | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If No, provide performance gaps@[90]% |
| --- | --- | --- | --- |
| [Case 4], [InF-SH-3D], [FR1], [DL-TDOA] | 1.8954 | No （1.6954） | No （0.8954） |
| [Case 5], [InF-SH-3D], [FR1], [UL-TDOA] | 2.0599 | No （1.6599） | No （1.0599） |
| [Case 6], [InF-SH-3D], [FR1], [UL-TDOA+UL-AOA] | 2.256 | No (2.056) | No (1.256) |
| [Case 7], [InF-SH-3D], [FR1], [Multi-RTT] | 2.18 | No (1.98) | No (1.18) |
| [Case 11], [InF-DH-3D], [FR1], [DL-TDOA] | 2.9917 | No （2.7917） | No （1.9917） |
| [Case 12], [InF-DH-3D], [FR1], [UL-TDOA] | 2.049 | No （1.849） | No （1.049） |
| [Case 13], [InF-DH-3D], [FR1], [UL-TDOA+UL-AOA] | 0.633 | No (0.433) | Yes |
| [Case 14], [InF-DH-3D], [FR1], [Multi-RTT] | 2.82 | No (2.62) | No (1.82) |
| [Case 25], [InF-SH-3D], [FR2], [DL-TDOA] | 0.6283 | No （0.4283） | Yes |
| [Case 26], [InF-SH-3D], [FR2], [UL-TDOA] | 0.8304 | No （0.6304） | Yes |
| [Case 29], [InF-DH-3D], [FR2], [DL-TDOA] | 3.0578 | No （2.8578） | No （2.0578） |
| [Case 30], [InF-DH-3D], [FR2], [UL-TDOA] | 3.1267 | No （2.9267） | No （2.1267） |

8.1.1.4	Observations from source [13]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1, [InF-SH, DL-TDOA, FR1] | 4.35 |
| Case 2, [InF-DH, DL-TDOA, FR1] | 7.16 |
| Case 3, [IOO, DL-TDOA, FR1] | 4.31 |
| Case 4, [IOO, DL-TDOA, FR1] | 6.50 |
| Case 5, [UMi, DL-TDOA, FR1] | 23.81 |
| Case 6, [InF-SH, DL-TDOA, FR1] | 1.65 |
| Case 7, [InF-DH, DL-TDOA, FR1] | 4.99 |

8.1.1.5	Observations from source [5]

Table 8.1.1.5-1.1 to Table 8.1.1.5-1.3 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.5-1.1: Rel.16 NR positioning – horizontal accuracy performance summary for baseline with perfect synchronization [5]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| [Case 1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 4.15 | 3.95 |
| [Case 3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 2.97 | 2.77 |
| [Case 5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 5.92 | 5.72 |
| [Case 7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 5.77 | 5.57 |
| [Case 11], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | 0.094 | Yes |
| [Case 13], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | 0.031 | Yes |
| [Case 15], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | 0.60 | 0.4 |
| [Case 17], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | 0.049 | Yes |
| [Case 19], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | 4.22 | 4.02 |
| [Case 21], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | 4.07 | 3.87 |
| [Case 23], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | 5.85 | 5.65 |
| [Case 25], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | 5.76 | 5.56 |
| [Case 27], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | 0.087 | Yes |
| [Case 29], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | 0.032 | Yes |
| [Case 31], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | 0.60 | 0.40 |
| [Case 33], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | 0.051 | Yes |
| [Case 35], [SH, perfect sync], [FR1], [UL-AOA] | 5.93 | 5.73 |
| [Case 37], [DH, perfect sync], [FR1], [UL-AOA] | 5.48 | 5.28 |
| [Case 39], [SH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | 0.41 | 0.21 |
| [Case 41], [DH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | 0.68 | 0.48 |
| [Case 43], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | 4.25 | 4.05 |
| [Case 45], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | 3.96 | 3.76 |
| [Case 47], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | 5.88 | 5.68 |
| [Case 49], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | 5.74 | 5.54 |
| [Case 51], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | 0.10 | Yes |
| [Case 53], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | 0.031 | Yes |
| [Case 55], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | 0.60 | 0.40 |
| [Case 57], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | 0.051 | Yes |

Table 8.1.1.5-1.2: Rel.16 NR positioning – horizontal accuracy performance summary for baseline with 50ns synchronization error [5]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| [Case 2], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 24.06 | 23.86 |
| [Case 4], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 23.21 | 23.01 |
| [Case 6], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 23.79 | 23.59 |
| [Case 8], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 22.90 | 22.70 |
| [Case 12], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | 26.09 | 25.89 |
| [Case 14], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | 25.67 | 25.47 |
| [Case 16], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | 20.30 | 20.10 |
| [Case 18], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | 20.16 | 19.96 |
| [Case 20], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | 24.51 | 24.31 |
| [Case 22], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | 23.21 | 23.01 |
| [Case 24], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | 22.90 | 22.70 |
| [Case 26], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | 18.92 | 18.72 |
| [Case 28], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | 27.70 | 27.50 |
| [Case 30], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | 25.67 | 25.47 |
| [Case 32], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | 22.01 | 21.81 |
| [Case 34], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | 19.74 | 19.54 |
| [Case 36], [SH, sync error 50ns], [FR1], [UL-AOA] | 6.20 | 6.00 |
| [Case 38], [DH, sync error 50ns], [FR1], [UL-AOA] | 5.76 | 5.56 |
| [Case 40], [SH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | 0.43 | 0.23 |
| [Case 42], [DH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | 0.77 | 0.57 |
| [Case 44], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | 4.71 | 4.51 |
| [Case 46], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | 4.13 | 3.93 |
| [Case 48], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | 6.20 | 6.00 |
| [Case 50], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | 6.23 | 6.03 |
| [Case 52], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | 0.10 | Yes |
| [Case 54], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | 0.030 | Yes |
| [Case 56], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | 0.78 | 0.58 |
| [Case 58], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | 0.055 | Yes |

Table 8.1.1.5-1.3: Rel.16 NR positioning – horizontal accuracy performance summary for DH {60%,6,2} with perfect synchronization [5]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| [Case 9], [DH {0.6,6,2}, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | 18.71 | 18.51 |
| [Case 10], [DH {0.6,6,2}, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | 15.09 | 14.89 |

Table 8.1.1.5-2 captures observations based on NR positioning evaluations results for vertical location error.

Table 8.1.1.5-2: Rel.16 NR positioning – vertical accuracy performance summary [23]

| Simulation case (Vertical Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [1]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| [Case V1], [SH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 0.66 | Yes |
| [Case V2], [DH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 1.12 | 0.12 |
| [Case V3], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 0.82 | Yes |
| [Case V4], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 1.39 | 0.39 |
| [Case V5], [SH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 1.05 | 0.05 |
| [Case V6], [DH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 5.46 | 4.46 |
| [Case V7], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 1.21 | 0.21 |
| [Case V8], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [UL-AOA+ZOA, MUSIC, select based on first/median peak] | 9.06 | 8.06 |

8.1.1.6	Observations from source [12]

Table 8.1.1.6-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.6-1: Rel.16 NR positioning – horizontal accuracy performance summary [12]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| Case1, (InF-SH, FR1) | 8.5 | No, 8.3 | No, 8.00 |
| Case2, (InF-DH, FR1) | 14.95 | No, 14.75 | No, 14.55 |

8.1.1.7	Observations from source [10]

Table 8.1.1.7-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.7-1: Rel.16 NR positioning – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1, InF-SH, FR1, DL-TDOA | 0.85 |
| Case 2, InF-DH, FR1, DL-TDOA | 6.2 |
| Case 3, InF-SH, FR2, DL-TDOA | 0.65 |
| Case 4, InF-DH, FR2, DL-TDOA | 17.3 |
| Case 5, InF-SH, FR1, UL-TDOA | 0.77 |
| Case 6, InF-DH, FR1, UL-TDOA | 6.13 |
| Case 7, InF-SH, FR2, UL-TDOA | 0.9 |
| Case 8, InF-DH, FR2, UL-TDOA | 16.9 |
| Case 9, InF-SH, FR1, Multi-RTT | 0.25 |
| Case 10, InF-DH, FR1, Multi-RTT | 16.3 |
| Case 11, InF-SH, FR2, Multi-RTT | 0.9 |
| Case 12, InF-DH, FR2, Multi-RTT | 7.72 |

Table 8.1.1.7-2 captures observations based on NR positioning evaluations results for vertical location error.

Table 8.1.1.7-2: Rel.16 NR positioning – vertical accuracy performance summary

| Simulation case (Vertical Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1, InF-SH, FR1, DL-TDOA | 8.5 |
| Case 2, InF-DH, FR1, DL-TDOA | 12.6 |
| Case 3, InF-SH, FR2, DL-TDOA | 12.88 |
| Case 4, InF-DH, FR2, DL-TDOA | 63.4 |
| Case 5, InF-SH, FR1, UL-TDOA | 12.9 |
| Case 6, InF-DH, FR1, UL-TDOA | 12.9 |
| Case 7, InF-SH, FR2, UL-TDOA | 13 |
| Case 8, InF-DH, FR2, UL-TDOA | 62.78 |
| Case 9, InF-SH, FR1, Multi-RTT | 13.1 |
| Case 10, InF-DH, FR1, Multi-RTT | 66 |
| Case 11, InF-SH, FR2, Multi-RTT | 6.45 |
| Case 12, InF-DH, FR2, Multi-RTT | 7.07 |

8.1.1.8	Observations from source [14]

Table 8.1.1.8-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.8-1: Rel.16 NR positioning – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| [Case 1], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.24 |
| [Case 2], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 0.70 |
| [Case 3], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 2.80 |
| [Case 4], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 3.87 |
| [Case 5], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.35 |
| [Case 6], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | 1.00 |

8.1.1.9	Observations from source [20]

Table 8.1.1.9-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.9-1: Rel.16 NR positioning – horizontal accuracy performance summary [20]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| [20], source1, UMa, FR1, DL-TDOA (50% UEs indoor) | 11.97m (1 occasion) 7.04m (9 occasions) |
| [20], source1, UMa, FR1, DL-TDOA (100% UEs outdoor) | 4.5m (1 occasion) 2.5m (9 occasions) |
| [20], source2, UMi, FR1, DL-TDOA | 1.7m (1 occasion) 1.06m (9 occasions) |
| [20], source3, UMi, FR1, UL-TDOA | 3.10m (1 occasion) 2.43m (9 occasions) |
| [20], source4, UMi, FR2, DL-TDOA | 0.13m (1 occasion) 0.06m (9 occasions) |
| [20], source5, IOO, FR1, DL-TDOA | 1.51m (1 occasion) 0.93m (9 occasions) |
| [20], source6, IOO, FR1, UL-TDOA | 1.52m (1 occasion) 0.98m (9 occasions) |
| [20], source7, IOO, FR2, DL-TDOA | 0.18m (1 occasion) 0.07m (9 occasions) |
| [20], source8, InF-SH, FR1, DL-TDOA | 0.19m (convex hull UEs) 0.36m (all UEs) |
| [20], source9, InF-SH , FR1, UL-TDOA | 0.18m (convex hull) 0.35m (all UEs) |
| [20], source10, InF-DH, FR1, DL-TDOA | 7m (convex hull UEs) 14.3m (all UEs) |
| [20], source11, InF-DH , FR1, UL-TDOA | 7.5m (convex hull UEs) 14.55m (all UEs) |
| [20], source12, InF-SH, FR2, DL-TDOA | 0.0172m(no RX/Tx error-convex hull UEs) 3.34m (8ns Rx/Tx error- convex hull UEs) 0.0349m (no RX/Tx error-all UEs) 3.68m (8ns Rx/Tx error-all UEs) |
| [20], source13, InF-SH , FR2, UL-TDOA | 0.0163m (no RX/Tx error-convex hull UEs) 3.36m (8ns Rx/Tx error-convex hull UEs) 0.0313m (no RX/Tx error-all UEs) 3.85m (8ns Rx/Tx error-all UEs) |
| [20], source14, InF-DH, FR2, DL-TDOA | 7.06m (convex hull UEs) 14.95m (all UEs) |
| Ericsson15, InF-DH , FR2, UL-TDOA | 6.98m (convex hull UEs) 13.48m (all UEs) |

8.1.1.10	Observations from source [17]

Table 8.1.1.10-1 captures observations based on NR positioning evaluations results for horizontal location error.

Table 8.1.1.10-1: Rel.16 NR positioning – horizontal accuracy performance summary

|  |  | 90% |
| --- | --- | --- |
| Case 1, InF FR1 DH ISD20, 100MHz, Link Quality, DL TDOA | Convex UEs | 46.647m |
|  | (Optional) All UEs | 46.649m |
| Case 1, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.044m |
|  | (Optional) All UEs | 0.045m |
| Case 2, InF FR1 SH ISD50, 100MHz, Link Quality, DL TDOA | Convex UEs | 16.556m |
|  | (Optional) All UEs | 14.647m |
| Case 2, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA | Convex UEs | 0.038m |
|  | (Optional) All UEs | 0.034m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 12.66m |
|  | (Optional) All UEs | 30.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 13.1m |
|  | (Optional) All UEs | 17.62m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Variable UE heights | Convex UEs | 0.22m |
|  | (Optional) All UEs | 0.47m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.12m |
|  | (Optional) All UEs | 0.36m |

| Horizontal Positioning Error (all UEs) | Beam Pair | 90% |
| --- | --- | --- |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.019 |
|  | Strongest | 0.027 |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.015 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.025 |
|  | Strongest | 9.40 |
| Case 8 InF-DH FR2 3d mRTT | Earliest | 0.020 |
| Case 5 InF-SH FR2 DL-TDOA | Earliest | 0.0087 |
|  | Strongest | 0.0170 |
| Case 7 InF-DH FR2 DL-TDOA | Earliest | 0.0096 |
|  | Strongest | 1.6767 |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.040 |
|  | Strongest | 6.32 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.024 |
|  | Strongest | 0.053 |
| Case 12 UMi FR2 DL-TDOA | Earliest | 0.0403 |
|  | Strongest | 6.3233 |
| Case 13 InH FR2 DL-TDOA | Earliest | 0.0113 |
|  | Strongest | 0.0279 |

| Horizontal Positioning Error |  | 90% |
| --- | --- | --- |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.6 |
| Case 9, UMI, FR1, DL-TDOA, Without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 3.2 |
| Case 9, UMI, FR1, RTT, Without Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 0.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 8 |
| Case 10, UMI, FR1,RTT, with Δτ, Perfect Sync, No Timing Errors, Likelihood Fusion Algorithm | (Optional) All UEs | 9.4 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | (Optional) All UEs | 17.3 |
| Case 10, UMI, FR1,DL-TDOA, with Δτ, Perfect Sync, No Timing Errors, LQ Algorithm | (Optional) All UEs | 20.1 |
| Case 11, UMA, FR1, DL-TDOA, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Outdoor UEs | 1.5 |
| Case 11, UMA, FR1, RTT, without Δτ, Perfect Sync, No Timing Errors, RANSAC Algorithm | Indoor UEs | 96 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 1 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.3 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.4 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.6 |
| Case 14, UMI, FR1, RTT, With Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 7.9 |

| Horizontal Positioning Error |  | 90% |
| --- | --- | --- |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δ𝜏, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 0 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1,DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 10, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 20, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 9.3 |
| Case 15, UMI, FR1, DL-TDOA, With Δτ, Sync Error = 50, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs | 12.5 |

| Horizontal Positioning Error |  |  | 90% |
| --- | --- | --- | --- |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 0.87 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 1.37 |
| Case 16, UMI, FR1, RTT, Without Δ𝜏, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 2.22 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 2 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 0.87 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 5 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 1.37 |
| Case 16, UMI, FR1,RTT, Without Δτ, Perfect Sync, Timing Error = 10 ns, Likelihood Fusion Algorithm | (Optional) All UEs |  | 2.22 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 2.3 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 0 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 0.61 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 2 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 1 |
| Case 17, UMI, FR1, DL-TDOA, Without Δτ, Sync Error = 0, Timing Error = 10 ns, Likelihood Fusion Algorithm |  | (Optional) All UEs | 2.3 |

| Case ID | Beam Pair | Tx T1 | 90% |
| --- | --- | --- | --- |
| Case 18 InF-SH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.01 |
|  |  | 0.1ns | 0.06 |
|  |  | 0.2ns | 0.13 |
|  |  | 0.5ns | 0.40 |
|  |  | 1.0ns | 0.96 |
|  |  | 2.0ns | 1.59 |
|  | Strongest beam pair | 0.0ns | 0.02 |
|  |  | 0.1ns | 0.11 |
|  |  | 0.2ns | 0.25 |
|  |  | 0.5ns | 0.94 |
|  |  | 1.0ns | 2.30 |
|  |  | 2.0ns | 5.19 |
| Case 19 InF-DH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.02 |
|  |  | 0.1ns | 0.10 |
|  |  | 0.2ns | 0.24 |
|  |  | 0.5ns | 0.76 |
|  |  | 1.0ns | 2.10 |
|  |  | 2.0ns | 6.31 |
|  | Strongest beam pair | 0.0ns | 43.94 |
|  |  | 0.1ns | 36.56 |
|  |  | 0.2ns | 35.28 |
|  |  | 0.5ns | 44.16 |
|  |  | 1.0ns | 40.33 |
|  |  | 2.0ns | 46.31 |
| Case 20 InH, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.03 |
|  |  | 0.1ns | 0.31 |
|  |  | 0.2ns | 0.47 |
|  |  | 0.5ns | 1.16 |
|  |  | 1.0ns | 2.04 |
|  |  | 2.0ns | 4.11 |
|  | Strongest beam pair | 0.0ns | 0.21 |
|  |  | 0.1ns | 7.44 |
|  |  | 0.2ns | 8.22 |
|  |  | 0.5ns | 11.37 |
|  |  | 1.0ns | 14.94 |
|  |  | 2.0ns | 18.87 |
| Case 21 UMi, FR2, DL-TDOA | Earliest beam pair | 0.0ns | 0.03 |
|  |  | 0.1ns | 0.13 |
|  |  | 0.2ns | 0.20 |
|  |  | 0.5ns | 0.38 |
|  |  | 1.0ns | 0.73 |
|  |  | 2.0ns | 1.38 |
|  | Strongest beam pair | 0.0ns | 16.21 |
|  |  | 0.1ns | 23.54 |
|  |  | 0.2ns | 16.59 |
|  |  | 0.5ns | 19.30 |
|  |  | 1.0ns | 14.27 |
|  |  | 2.0ns | 21.65 |

Table 8.1.1.10-2 captures observations based on NR positioning evaluations results for vertical location error.

Table 8.1.1.10-2: Rel.16 NR positioning – vertical accuracy performance summary

| Vertical Positioning error |  | 90% |
| --- | --- | --- |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Variable UE heights | Convex UEs | 20.6m |
|  | (Optional) All UEs | 38.9m |
| Case 3, InF FR1 DH ISD20, 100MHz, RANSAC, DL TDOA, Fixed UE heights | Convex UEs | 18.44m |
|  | (Optional) All UEs | 22.98m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, DL TDOA, , Unequal gNBs heights, Variable UE heights | Convex UEs | 1.89m |
|  | (Optional) All UEs | 2.63m |
| Case 4, InF FR1 SH ISD50, 100MHz, RANSAC, Unequal gNBs heights, DL TDOA, Unequal gNBs heights, Fixed UE heights | Convex UEs | 0.9m |
|  | (Optional) All UEs | 1.34m |

| Vertical (Across All UEs) | Beam Pair | 90% |
| --- | --- | --- |
| Case 6 InF-SH FR2 3d mRTT | Earliest | 0.084 |
| Case 8 InF-SH FR2 3d mRTT | Earliest | 0.041 |

8.1.1.11	Observations from source [18]

Table 8.1.1.11-1: Rel.16 NR positioning – horizontal accuracy performance summary from [18]

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| Case 1- config ID 445 (FR1-InF DH) | ≥10 |
| Case 3- config ID 1112 (FR1-UMi) | 3.24 |
| Case 2- config ID 1011 (FR1-UMi with ATOA) | ≥10 |

8.1.2	Physical layer latency analysis for Rel-16

8.1.2.1	Observations from source [4]

Summary of latency performance analysis is provided in Table 8.1.2.1-1.

Table 8.1.2.1-1: NR Rel.16 positioning – latency performance summary [4]

| Description Evaluation Case | L1 Latency, ms | Commercial requirements [100]ms are met -Yes/No - If no, provide performance gaps | IIoT requirements of [10ms] are met - Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case L1, DL-TDOA/DL-AoD/Multi-RTT w/ Gap request and PRS periodicity 20ms | 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) | Yes (1 samp.) >=11.5ms (4 samp. CSSF = 1) >=71.5ms (4 samp. CSSF = 2) | >=41.5ms (1 samp.) >=101.5ms (4 samp. CSSF = 1) >=161.5ms (4 samp. CSSF = 2) | Yes (1 samp.) >=11.5ms (4 samp. CSSF = 1) >=71.5ms (4 samp. CSSF = 2) |
| Case L2, DL-TDOA/DL-AoD/Multi-RTT w/o Gap request and PRS periodicity 160ms | 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) | No (1 samp.) No (4 samp. CSSF = 1) | No (1 samp.) No (4 samp. CSSF = 1) | No (1 samp.) No (4 samp. CSSF = 1) |
| Case L3, UL-TDOA/UL-AoA | 6.5-26ms (1 samp.) 66.5-86.5ms (4 samp) | Yes (1 samp.) Yes (4 samp.) | Yes/No (1 samp.) >=56.5ms (4 samp.) | Yes (1 samp.) Yes (4 samp.) |
| Case L4, DL E-CID | 8.5-15ms | Yes | Yes/No | Yes |
| Case L5, UL E-CID | 6-26ms | Yes | Yes/No | Yes |
| Case L6, UE-based DL-TDOA/DL-AoD w/ gap request t and PRS periodicity 20ms | 51-58.5ms (1 samp.) | Yes | >=41ms | Yes |

8.1.2.2	Observations from source [7]

Summary of latency performance analysis is provided in Table 8.1.2.2-1.

Table 8.1.2.2-1: NR positioning enhancements – latency performance summary

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Case PHY-L1, UE-A, DL-TDOA, FR1, FDD | 106.23 |
| Case PHY-L1, UE-A, DL-TDOA, FR2, FDD | 667.87 |
| Case PHY-L2, UE-B, DL-TDOA, FR1, FDD | 106.30 |
| Case PHY-L2, UE-B, DL-TDOA, FR2, FDD | 667.82 |
| Case PHY-L3, UE-A, DL-ECID, FR1,FDD | 10.43 |
| Case PHY-L3, UE-A, DL-ECID, FR2, FDD | 10.64 |

8.1.2.3	Observations from source [8]

Summary of latency performance analysis is provided in Table 8.1.2.3-1.

Table 8.1.2.3-1: NR Rel.16 positioning - latency performance summary [8]

| Description Evaluation Case | L1 Latency, ms | Commercial requirements [100]ms are met -Yes/No - If No, provide performance gaps | IIoT requirements of [10ms] are met - Yes/No. If No, provide performance gaps | IIoT requirements of [100]ms are met - Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case 1, 15kHz, FR1, DL-TDOA | 51.5 | Yes | No (41.5ms gaps) | Yes |
| Case 2, 15kHz, FR1, UL-TDOA | 5 | Yes | Yes | Yes |

8.1.2.4	Observations from source [13]

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Case 1, DL-TDOA/DL-AoD, FR1 | [44.35 – 10500] |
| Case 2, DL-TDOA/DL-AoD, FR2 | [35.08 – 2118.93] |
| Case 3, UL-TDOA/UL-AoA, FR1 | [2.78 – 81928.5] |

8.1.2.5	Observations from source [11]

A summary of the physical layer latency performance analysis for the DL-based positioning methods is provided in  Table 8.1.2.5-1.

Table 8.1.2.5-1: NR positioning enhancements – latency performance summary [11]

| Description Evaluation Case | L1 Latency1, ms |
| --- | --- |
| Case ID: 1, Scenario: UE-Assisted Positioning with MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA/R.16 DL-AoD | [38 - 235.6]: 30 kHz SCS |
|  | [35 - 229.6]: 120 kHz SCS |
| Case ID: 2, Scenario: UE-Assisted Positioning without MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDO/ R.16 DL-AoD | [17 - 5147.8]: 30 kHz SCS |
|  | [15.5 - 5144.8]: 120 kHz SCS |
| Case ID: 3, Scenario: UE-based Positioning with MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA/R.16 DL-AoD | [29 - 207.8]: 30 kHz SCS |
|  | [27.5 - 204.8]: 120 kHz SCS |
| Case ID: 4, Scenario: UE-based Positioning without MG configuration, Frequency Band: FR1/FR2, Technique: R.16 DL-TDOA/R.16 DL-AoD | [8 – 5120]: 120 kHz SCS |
| Notes: 1: The presented L1 latency value ranges correspond to the minimum and cautious estimates. Due to the assumptions of a single DL-PRS occasion, this may not correspond to an accurate positioning measurement and serves a guideline for the achievable physical layer latency. The cautious estimate is not intended to indicate the physical layer latency upper bound. |  |

8.1.2.6	Observations from source [5]

Summary of latency performance analysis is provided in Table 8.1.2.6-1.

Table 8.1.2.6-1: NR positioning enhancements – latency performance summary [5]

| Description Evaluation Case | L1 Latency ms | Commercial requirements [100]ms are met -Yes/No - If no, provide performance gaps | IIoT requirements of [10ms] are met - Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- |
| [Case 1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-A] | 64ms~ |  | 54ms~ |  |
| [Case 1-1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-A], [idle,inactive] | 85.3ms~ or 104ms~ |  | 75.3ms~ or 94ms~ |  |
| [Case 2], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-B] Source [Network]/Destination [Network] | 66 ms ~ |  | 56ms~ |  |
| [Case 3], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-B] Source [UE]/Destination [UE] | 55.5ms~ |  | 45.5ms~ |  |
| [Case 4], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA], [periodic SRS] | 30.5ms~ |  | 20.5ms~ |  |
| [Case 5], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA], [A- SRS] | 11ms~ |  | 1ms~ |  |
| [Case 6], [IIoT/ Commercial], [Frequency Band], [Multi-RTT] | 94.5+ TAlign_DL_UL ~ |  | 84.5+TAlign_DL_UL~ |  |

8.1.2.7	Observations from source [12]

Summary of latency performance analysis is provided in Table 8.1.2.7-1.

Table 8.1.2.7-1: NR positioning enhancements – latency performance summary [12]

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Case1, DL-TDOA,FR1 | 54.125 |
| Case2, UL-TDOA,FR1 | 23.25 |
| Case3, UE- based method,FR1 | 54.125 |
| Case4, DL-TDOA,FR2 | 52.56 |
| Case5, UL-TDOA,FR2 | 23.125 |
| Case6, UE- based method,FR2 | 52.56 |

8.1.2.8	Observations from source [10]

Summary of latency performance analysis is provided in Table 8.1.2.8-1.

Table 8.1.2.8-1: NR positioning enhancements – latency performance summary

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Case 1, InF, FR1, R.16 DL-TDOA/DL-AoD | 4.5714 (L1 components) + [36] (L2/L3 components) + 88.5 (DL PRS processing) = 129.07 ms (total) |
| Case 2, InF, FR1, R.16 UL-TDOA/UL-AoA | 2.7678 (L1 components) + [16] (L2/L3 components) = 18.7678 (total) |
| Case 3, InF, FR1, R.16 Multi-RTT | 7.3393 (L1 components) + [45] (L2/L3 components) + 88.5 (DL PRS processing) = 140.8393 (total) |

8.1.2.9	Observations from source [16]

The latency analysis for each case is summarized in the following table:

| Description Evaluation Case | L1 Latency, ms |
| --- | --- |
| Latency analysis for UE-assisted DL methods (Case 1) | 33 |
| UE-based DL methods (Case 2) | 22-72 |
| UE-assisted UL methods (Case 3) | 12 |
| UE-assisted DL+UL methods (Case 4) | 45 |

8.1.2.10	Observations from source [17]

Summary of latency performance analysis is provided in Table 8.1.2.10-1.

Table 8.1.2.10-1: NR positioning enhancements – latency performance summary

| NR Rel-16 Scenario | L1-layer Latency Expected Range (ms) |
| --- | --- |
| UE-Assisted DL-only Positioning, RRC Connected State | 57 - 823 |
| UE-based DL-only Positioning, RRC Inactive State, External Client | 35.3 - 803.5 |
| UE-based DL-only Positioning, RRC Connected State, UE Internal-client | 46 - 811 |
| UE-based DL-only Positioning, RRC Inactive State, UE internal-client | 8 - 780 |
| UE-Assisted MRTT Positioning, RRC Connected State | 59 - 823 |

8.1.2.11	Observations from source [15]

Summary of latency performance analysis is provided in Table 8.1.2.11-1.

Table 8.1.2.11-1: NR positioning – latency analysis [15]

| Description Evaluation Case | Assumptions | L1 Latency, ms (including preparation/processing time at higher layer) | L1 Latency, ms (excluding preparation/processing time at higher layer) |
| --- | --- | --- | --- |
| Case 1, DL-TDOA, DL-AOD [NW initiated, UE-A] | FR1, 15kHz # of symbols for PUSCH: 1~14 OS # of symbols for PDSCH: 2~14 OS # of symbols for SRS: 2~12 OS Periodicity and offset for PUCCH: 2 OS ~ 80 slot The length of symbols for PUCCH: 1 OS~ 14 OS Slot for PDCCH Monitoring configured as periodicity and offset is 1slot. The first symbol(s) for PDCCH monitoring in the slots is zero # of symbols for CORESET: 1 OS ~3 OS -Uplink switching gap is not configured. -No BWP switching -No overlapping symbols of the PUCCH and the scheduled PUSCH -No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH *Note: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed. *Note: According to scheduling request configuration and UL grant configuration, the total values may change. For example, larger periodicity for SR and/or PDCCH monitoring periodicity are set. | For UE capability-1: 62.97 ms ~ 297.11ms For UE capability-2: 61.17 ms ~ 293.68 ms | For UE capability-1: 23.97ms ~ 249.11ms For UE capability-2: 22.17ms ~ 245.68ms |
| Case 2, DL-TDOA, DL-AOD [UE initiated, UE-A] |  | For UE capability-1: 55.26ms ~ 284.83ms For UE capability-2: 53.82ms ~ 282.97ms | For UE capability-1: 23.26ms ~ 247.33ms For UE capability-2: 21.82ms ~ 245.47ms |
| Case 3, UL-TDOA, UL-AOA [NW initiated, UE-A] |  | For UE capability-1: 14.78 ms ~ 20.14 ms For UE capability-2: 14.42 ms ~ 19.57 ms | For UE capability-1: 0.78 ms ~ 2.64ms For UE capability-2: 0.42ms ~ 2.07ms |
| Case 4, Multi-RTT [NW initiated, UE-A] |  | For UE capability-1: 77.75 ms ~314.75 ms For UE capability-2: 75.59 ms ~ 311.75 ms | For UE capability-1: 24.75 ms ~ 251.75 ms For UE capability-2: 22.59 ms ~ 248.75 ms |
| Case 5, E-CID [NW initiated, UE-A] |  | For UE capability-1: 28.41ms ~ 116.55 ms For UE capability-2: 27.33 ms ~ 115.05 ms | For UE capability-1: 2.41 ms ~ 85.55 ms For UE capability-2: 1.33 ms ~ 84.05 ms |

8.2	Performance analysis of studied NR positioning enhancements

Including performance of positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency  ((objective 1c).

8.2.1	Positioning accuracy analysis for NR positioning enhancements

8.2.1.1	Observations from source [4]

Table 8.2.1.1-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.1-1: NR positioning enhancements – horizontal accuracy performance summary [4]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| 311, InF-DH422, FR1, UL-TDOA | Rel-16 baseline | 9.6631 | No | No | No |
| 312, InF-DH422, FR1, UL-TDOA w/ RAIM | 9.0062 | 0.6569 | Yes | 0.4569 | 0.1569 |
| 313, InF-DH422, FR1, UL-TDOA w/ LOS/NLOS identification | 9.452 | 0.2111 | Yes | 0.0111 | Yes |
| 321, InF-DH422, FR1, UL-TDOA, 100M contiguous | Rel-16 baseline | 0.2022 | Yes | No | Yes |
| 322, InF-DH422, FR1, UL-TDOA, 200M contiguous | 0.1638 | 0.0384 | Yes | Yes | Yes |
| 323, InF-DH422, FR1, UL-TDOA, 50MHz+100MHz (Gap)+50MHz | 0.111 | 0.0912 | Yes | Yes | Yes |
| 331, InF-SH, FR1, DL-TDOA, Comb-4 and 4-symbol | Rel-16 baseline | 0.0939 | Yes | Yes | Yes |
| 332, InF-SH, FR1, DL-TDOA, Comb-4 and 1-symbol | -0.0184 | 0.1123 | Yes | Yes | Yes |
| 333, InF-SH, FR1, DL-TDOA, Comb-12 and 12-symbol | Rel-16 baseline | 0.1091 | Yes | Yes | Yes |
| 334, InF-SH, FR1, DL-TDOA, Comb-12 and 1-symbol | -0.0108 | 0.1199 | Yes | Yes | Yes |
| 341, InF-SH, FR1, DL-TDOA | Rel-16 baseline | 0.0939 | Yes | Yes | Yes |
| 342, InF-SH, FR1, DL-TDOA, 20-RB of PRS punctured by SSB | -0.0151 | 0.109 | Yes | Yes | Yes |
| 362, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ legacy AoA) | Rel-16 baseline | 4.8161 | No | No | No |
| 363, InF-DH422, FR1, UL-TDOA/AoA, ULA 4x1 w/ modified AoA | 4.6467 | 0.1694 | Yes | Yes | Yes |
| 371, InF-DH422, FR1, Multi-RTT | Rel-16 baseline | 0.1694 | Yes | Yes | Yes |
| 372, InF-DH422, FR1, E-CID w/ single cell RTT/AoA | -0.0701 | 0.2395 | Yes | No | Yes |
| 381, InF-SH, FR1, UL-TDOA, 100M，0ns gNB Sync error | Rel-16 baseline | 0.1136 | Yes | Yes | Yes |
| 382, InF-SH, FR1, UL-TDOA, 100M，0.2ns gNB Sync error | -0.0316 | 0.1452 | Yes | Yes | Yes |
| 383, InF-SH, FR1, UL-TDOA, 100M，0.5ns gNB Sync error | -0.1692 | 0.2828 | Yes | No | Yes |
| 384, InF-SH, FR1, UL-TDOA, 100M，1ns gNB Sync error | -0.2544 | 0.5372 | Yes | No | Barely |
| 385, InF-SH, FR1, UL-TDOA, 200M, 0ns gNB Sync error | FR1 CA baseline | 0.025 | Yes | Yes | Yes |
| 386, InF-SH, FR1, UL-TDOA, 200M, 0.2ns gNB Sync error | -0.0852 | 0.1102 | Yes | Yes | Yes |
| 387 InF-SH, FR1, UL-TDOA, 200M, 0.5ns gNB Sync error | -0.2409 | 0.2659 | Yes | No | Yes |
| 388 InF-SH, FR1, UL-TDOA, 200M, 1ns gNB Sync error | -0.509 | 0.534 | Yes | No | Barely |

8.2.1.2	Observations from source [7]

Table 8.2.1.2-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.2-1: NR positioning enhancements – horizontal accuracy performance summary [7]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% |
| --- | --- | --- |
| Case 26 | 5.140 | 7.205 |
| Case 27 | 10.342 | 2.003 |
| Case 29 | 2.926 | 9.248 |
| Case 30 | 11.946 | 0.228 |

8.2.1.3	Observations from source [8]

Table 8.2.1.3-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.3-1: NR positioning enhancements – horizontal accuracy performance summary [8]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If No, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- |
| [Case 1], [InF-HH-2D], [FR1], [DL- TDOA+ DL-CPP] | 0.114m Vs CASE 1 in section 8.1.1.3 | 0.051 | YES | YES |
| [Case 2], [InF-HH-2D], [FR1], [UL- TDOA+ UL-CPP] | 0.1061m Vs CASE 2 in section 8.1.1.3 | 0.049 | YES | YES |

8.2.1.4	Observations from source [13]

8.2.1.5	Observations from source [5]

Table 8.2.1.5-1.1 to Table 8.2.1.5-1.9 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.5-1.1: NR positioning enhancements – horizontal accuracy performance summary for baseline with RAIM [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | 3.95 | 0.099 | Yes |
| [Case E3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | 2.95 | 0.024 | Yes |
| [Case E5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | 1.49 | 4.43 | 4.23 |
| [Case E7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | 1.42 | 4.35 | 4.15 |
| [Case E9], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0 | 0.094 | Yes |
| [Case E11], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.007 | 0.024 | Yes |
| [Case E13], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.43 | 0.17 | Yes |
| [Case E15], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.015 | 0.034 | Yes |
| [Case E17], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | 4.12 | 0.10 | Yes |
| [Case E19], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | 4.04 | 0.034 | Yes |
| [Case E21], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | 0.37 | 5.48 | 5.28 |
| [Case E23], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | 1.21 | 4.55 | 4.35 |
| [Case E25], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.004 | 0.083 | Yes |
| [Case E27], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0 | 0.032 | Yes |
| [Case E29], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.41 | 0.19 | Yes |
| [Case E31], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.008 | 0.043 | Yes |
| [Case E33], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | 4.14 | 0.11 | Yes |
| [Case E35], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | 3.91 | 0.049 | Yes |
| [Case E37], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | 0.99 | 4.89 | 4.69 |
| [Case E39], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | 1.62 | 4.12 | 3.92 |
| [Case E41], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.008 | 0.092 | Yes |
| [Case E43], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.008 | 0.030 | Yes |
| [Case E45], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.41 | 0.19 | Yes |
| [Case E47], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.003 | 0.048 | Yes |

Table 8.2.1.5-1.2: NR positioning enhancements – horizontal accuracy performance summary with RAIM and LOS detection [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E49], [SH, perfect sync], [FR1], [DL-TDOA, RAIM] |  | 0.094 | Yes |
| [Case E50], [SH, perfect sync], [FR1], [100% LOS detection probability without RAIM] |  | 0.096 | Yes |
| [Case E51], [SH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] |  | 0.083 | Yes |
| [Case E52], [SH, perfect sync], [FR1], [DL-TDOA, 95% LOS detection probability without RAIM] |  | 2.86 | 2.66 |
| [Case E53], [SH, perfect sync], [FR1], [DL-TDOA, 90% LOS detection probability without RAIM] |  | 4.54 | 4.34 |
| [Case E54], [SH, perfect sync], [FR1], [DL-TDOA, baseline no LOS detection without RAIM] |  | 4.62 | 4.42 |
| [Case E55], [DH, perfect sync], [FR1], [DL-TDOA, RAIM] |  | 0.17 | Yes |
| [Case E56], [DH, perfect sync], [FR1], [DL-TDOA, 100% LOS detection probability without RAIM] |  | 0.33 | 0.13 |
| [Case E57], [DH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] |  | 0.17 | Yes |
| [Case E58], [DH, perfect sync], [FR1], [DL-TDOA, 95% LOS detection probability without RAIM] |  | 3.40 | 3.20 |
| [Case E59], [DH, perfect sync], [FR1], [DL-TDOA, 90% LOS detection probability without RAIM] |  | 3.43 | 3.23 |
| [Case E60], [DH, perfect sync], [FR1], [DL-TDOA, baseline no LOS detection without RAIM] |  | 8.64 | 8.44 |

Table 8.2.1.5-1.3: NR positioning enhancements – horizontal accuracy performance summary with different timing measurement reporting granularity [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E61], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] |  | 0.16 | Yes |
| [Case E62], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] |  | 0.21 | 0.1 |
| [Case E63], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] |  | 0.47 | 0.27 |
| [Case E64], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] |  | 0.17 | Yes |
| [Case E65], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] |  | 0.35 | 0.15 |
| [Case E66], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] |  | 0.59 | 0.39 |

Table 8.2.1.5-1.4: NR positioning enhancements – horizontal accuracy performance summary with different Rx/Tx timing error [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E67], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] |  | 0.30 | 0.10 |
| [Case E68], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] |  | 0.34 | 0.14 |
| [Case E69], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] |  | 0.36 | 0.16 |
| [Case E70], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] |  | 0.35 | 0.15 |
| [Case E71], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] |  | 0.37 | 0.17 |
| [Case E72], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] |  | 0.42 | 0.22 |
| [Case E73], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] |  | 0.83 | 0.63 |
| [[Case E74], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] |  | 1.07 | 0.87 |
| [Case E75], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] |  | 1.87 | 1.67 |
| [Case E76], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] |  | 0.31 | 0.11 |
| [Case E77], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] |  | 0.32 | 0.12 |
| [Case E78], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] |  | 0.32 | 0.12 |
| [Case E79], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] |  | 0.28 | 0.08 |
| [Case E80], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] |  | 0.31 | 0.11 |
| [Case E81], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] |  | 0.40 | 0.20 |
| [Case E82 [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] |  | 0.76 | 0.56 |
| [[Case E83], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] |  | 0.88 | 0.68 |
| [Case E84], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] |  | 1.94 | 1.74 |
| [Case E85], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] |  | 0.24 | 0.04 |
| [Case E86], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] |  | 0.23 | 0.03 |
| [Case E87], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] |  | 0.27 | 0.07 |
| [Case E88], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] |  | 0.33 | 0.13 |
| [Case E89], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] |  | 0.44 | 0.24 |
| [Case E90], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] |  | 0.28 | 0.08 |
| [Case E91], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] |  | 0.34 | 0.14 |
| [[Case E92], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] |  | 0.76 | 0.56 |
| [Case E93], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] |  | 1.26 | 1.06 |
| [Case E94], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] |  | 0.24 | 0.04 |
| [Case E95], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] |  | 0.24 | 0.04 |
| [Case E96], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] |  | 0.28 | 0.08 |
| [Case E97], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] |  | 0.36 | 0.16 |
| [Case E98], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] |  | 0.46 | 0.26 |
| [Case E99], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] |  | 0.34 | 0.14 |
| [Case E100], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] |  | 0.48 | 0.28 |
| [[Case E101], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] |  | 0.87 | 0.67 |
| [Case E102], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] |  | 1.28 | 1.08 |

Table 8.2.1.5-1.5: NR positioning enhancements – horizontal accuracy performance summary with aggregation of DL positioning frequency layers [5]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E103], [SH, perfect sync], [FR1], [50M] |  | 0.31 | 0.11 |
| [Case E104], [SH, perfect sync], [FR1], [100M] |  | 0.094 | Yes |
| [Case E105], [SH, perfect sync], [FR1], [50M+50M] |  | 0.21 | 0.01 |
| [Case E106], [DH, perfect sync], [FR1], [50M] |  | 0.44 | 0.24 |
| [Case E107], [DH, perfect sync], [FR1], [100M] |  | 0.17 | Yes |
| Case E108], [DH, perfect sync], [FR1], [50M+50M] |  | 0.23 | 0.03 |

Table 8.2.1.5-1.6: NR positioning enhancements – horizontal accuracy performance summary with aggregation of DL positioning frequency layers with timing offset [22]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E121], [SH, perfect sync], [FR1], [50M+50M] [timing offset 1ns] |  | 0.46 | 0.26 |
| [Case E122], [SH, perfect sync], [FR1], [50M+50M] [timing offset 5ns] |  | 2.03 | 1.83 |
| [Case E123], [SH, perfect sync], [FR1], [50M+50M] [timing offset 10ns] |  | 5.46 | 5.26 |
| [Case E124], [SH, perfect sync], [FR1], [50M+50M] [timing offset 20ns] |  | 10.05 | 9.85 |
| [Case E125], [DH, perfect sync], [FR1], [50M+50M] [timing offset 1ns] |  | 0.96 | 0.76 |
| [Case E126], [DH, perfect sync], [FR1], [50M+50M] [timing offset 5ns] |  | 3.90 | 3.70 |
| [Case E127], [DH, perfect sync], [FR1], [50M+50M] [timing offset 10ns] |  | 8.34 | 8.14 |
| [Case E128], [DH, perfect sync], [FR1], [50M+50M] [timing offset 20ns] |  | 10.69 | 10.49 |

Table 8.2.1.5-1.7: NR positioning enhancements – horizontal accuracy performance with reduced Rx/Tx timing error and synchronization error [22]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E115], [SH, sync error 50ns], [FR1], [DL-TDOA] [sync error reduced by differential positioning] |  | 0.11 | Yes |
| [Case E116], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] [Rx/Tx timing error reduced by differential positioning] |  | 0.13 | Yes |
| [Case E117], [SH, sync error 50ns], [FR1], [UL-TDOA] [sync error reduced by UL-TDOA+AOA] |  | 3.16 | 2.96 |
| [Case E118], [SH, perfect sync], [FR1], [UL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] [Rx/Tx timing error reduced by UL-TDOA+AOA] |  | 1.50 | 1.30 |
| [Case E119], [DH, (60%,6,2), perfect sync], [FR1], [machine learning] |  | 4.60 | 4.40 |
| [Case E120], [DH, (60%,6,2), sync error 50ns], [FR1], [machine learning] |  | 5.12 | 4.92 |

Table 8.2.1.5-1.8: NR positioning enhancements – horizontal accuracy performance summary for IOO scenario without absolute time of arrival modelling [23]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E109], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] (Case 63 in [5]) |  | 0.80 | Yes |
| [Case E110], [IOO scenario withlout absolute time of arrival modelling, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] (Case 64 in [5]) |  | 0.54 | Yes |
| [Case E111], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] (Case 65 in [5]) |  | 0.84 | Yes |
| [Case E112], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] (Case 66 in [5]) |  | 0.56 | Yes |
| [Case E113], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] (Case 67 in [5]) |  | 0.68 | Yes |
| [Case E114], [IOO scenario without absolute time of arrival modelling, perfect sync], [FR2], [Multi-RTT MUSIC, select based on first/median peak] (Case 68 in [5]) |  | 0.50 | Yes |

Table 8.2.1.5-1.9: NR positioning enhancements – horizontal accuracy performance summary for IOO scenario with absolute time of arrival modelling [23]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case E129], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] |  | 0.82 | Yes |
| [Case E130], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] |  | 0.56 | Yes |
| [Case E131], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] |  | 0.86 | Yes |
| [Case E132], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] |  | 0.62 | Yes |
| [Case E133], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] |  | 0.68 | Yes |
| [Case E134], [IOO scenario with absolute time of arrival modelling, perfect sync], [FR2], [Multi-RTT MUSIC, select based on first/median peak] |  | 0.54 | Yes |

Table 8.2.1.5-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

Table 8.2.1.5-2: NR positioning enhancements – vertical accuracy performance summary [23]

| Simulation case (Vertical Error) | Gain vs Rel16 solution @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [1]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| [Case E-V1], [SH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 0.58 | Yes |
| [Case E-V2], [DH, perfect sync], [FR1], [ BS height = 8m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 0.64 | Yes |
| [Case E-V3], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 1.25 | 0.25 |
| [Case E-V4], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =1.5m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 4.62 | 3.62 |
| [Case E-V5], [SH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 0.66 | Yes |
| [Case E-V6], [DH, perfect sync], [FR1], [ BS height = 8m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 3.16 | 2.16 |
| [Case E-V7], [SH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 1.27 | 0.27 |
| [Case E-V8], [DH, perfect sync], [FR1], [ BS height = {4,8}m UE height =[0.5,2]m] [DL-TDOA, MUSIC, select based on first/median peak] |  | 4.93 | 3.93 |

8.2.1.6	Observations from source [12]

Table 8.2.1.6-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.6-1: NR positioning enhancements – horizontal accuracy performance summary [12]

| Simulation cases (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If No, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case1, (InF-SH, NLOS classification FR1) | 7.7 | 0.87 | No, 0.67 | No, 0.3 |
| Case2, (InF-SH, NLOS mitigation FR1) | 5.46 | 3.12 | No, 2.9 | No, 2.62 |
| Case3, (InF-SH, NLOS classification + NLOS mitigation FR1) | 8.27 | 0.31 | No, 0.11 | Yes |
| Case4, (InF-DH, NLOS classification FR1) | 2.4 | 12.5 | No, 12.3 | No, 12 |
| Case5, (InF-DH, NLOS mitigation FR1) | 9.4 | 5.49 | No, 5.29 | No, 4.66 |
| Case6, (InF-DH, NLOS classification FR1) | 12.43 | 2.47 | No, 2.27 | No, 1.97 |
| Case7, (InF-SH, Ideal NLOS classification FR1) | 8.27 | 0.23 | No, 0.03 | Yes, |
| Case8, (InF-DH, Ideal NLOS classification FR1) | 14.59 | 0.31 | No, 0.11 | Yes |

8.2.1.7	Observations from source [10]

Table 8.2.1.7-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.7-1: NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% |
| --- | --- | --- |
| Case 13, InF-SH, FR1, DL-TDOA | 0.48 | 0.37 |
| Case 14, InF-DH, FR1, DL-TDOA | 4.26 | 1.94 |
| Case 15, InF-SH, FR2, DL-TDOA | 0.44 | 0.21 |
| Case 16, InF-DH, FR2, DL-TDOA | 16.41 | 0.89 |
| Case 17, InF-SH, FR1, UL-TDOA | 0.37 | 0.4 |
| Case 18, InF-DH, FR1, UL-TDOA | 1.73 | 4.4 |
| Case 19, InF-SH, FR2, UL-TDOA | 0.66 | 0.24 |
| Case 20, InF-DH, FR2, UL-TDOA | 16.22 | 0.68 |
| Case 21, InF-SH, FR1, Multi-RTT | 0.08 | 0.17 |
| Case 22, InF-DH, FR1, Multi-RTT | 16.00 | 0.3 |
| Case 23, InF-SH, FR2, Multi-RTT | 0.80 | 0.1 |
| Case 24, InF-DH, FR2, Multi-RTT | 7.55 | 0.17 |
| Case 25, InF-SH, FR1, Multi-RTT + vertical AoA | 0.15 | 0.1 |
| Case 26, InF-DH, FR1, Multi-RTT + vertical AoA | 16.11 | 0.19 |
| Case 27, InF-SH, FR2, Multi-RTT + vertical AoA | 0.82 | 0.08 |
| Case 28, InF-DH, FR2, Multi-RTT + vertical AoA | 7.52 | 0.2 |
| Case 33, InF-SH, FR1, Multi-RTT | 0.19 | 0.06 |
| Case 34, InF-DH, FR1, Multi-RTT | 16.19 | 0.11 |
| Case 35, InF-SH, FR2, Multi-RTT | 0.85 | 0.05 |
| Case 36, InF-DH, FR2, Multi-RTT | 7.65 | 0.07 |

Table 8.2.1.7-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

Table 8.2.1.7-2: NR positioning enhancements – vertical accuracy performance summary

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% |
| --- | --- | --- |
| Case 13, InF-SH, FR1, DL-TDOA | 1.60 | 6.9 |
| Case 14, InF-DH, FR1, DL-TDOA | 0.2 | 12.4 |
| Case 15, InF-SH, FR2, DL-TDOA | 6.28 | 6.6 |
| Case 16, InF-DH, FR2, DL-TDOA | 57.60 | 5.8 |
| Case 17, InF-SH, FR1, UL-TDOA | 0.1 | 12.8 |
| Case 18, InF-DH, FR1, UL-TDOA | 0. | 12.9 |
| Case 19, InF-SH, FR2, UL-TDOA | 1.70 | 11.3 |
| Case 20, InF-DH, FR2, UL-TDOA | 62.14 | 0.64 |
| Case 21, InF-SH, FR1, Multi-RTT | 12.21 | 0.89 |
| Case 22, InF-DH, FR1, Multi-RTT | 65.51 | 0.49 |
| Case 23, InF-SH, FR2, Multi-RTT | 5.45 | 1 |
| Case 24, InF-DH, FR2, Multi-RTT | 6.62 | 0.45 |
| Case 25, InF-SH, FR1, Multi-RTT + vertical AoA | 12.83 | 0.27 |
| Case 26, InF-DH, FR1, Multi-RTT + vertical AoA | 65.80 | 0.2 |
| Case 27, InF-SH, FR2, Multi-RTT + vertical AoA | 6.37 | 0.08 |
| Case 28, InF-DH, FR2, Multi-RTT + vertical AoA | 7.01 | 0.06 |
| Case 33, InF-SH, FR1, Multi-RTT | 12.70 | 0.4 |
| Case 34, InF-DH, FR1, Multi-RTT | 65.81 | 0.19 |
| Case 35, InF-SH, FR2, Multi-RTT | 6.06 | 0.39 |
| Case 36, InF-DH, FR2, Multi-RTT | 6.82 | 0.25 |

8.2.1.8	Observations from source [18]

Table 8.2.1.8-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.8-1: NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- |
| Case 4- config 220 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.70) | No (0.70) |
| Case 5- config 320 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.45) | Yes (0.45) |
| Case 6- config 420 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.43) | Yes |
| Case 7- config 421 (InF-DH, FR1,UL-TDOA) | N.A. | No(>3.00) | No(>3) | No(>3) |
| Case 8- config 422 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(0.44) | Yes |
| Case 9- config 423 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.59) | No (0.59) |
| Case 10- config 443 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.50) | Yes |
| Case 11- config 444 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.49) | Yes |
| Case 12- config 447 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(0.95) | Yes |
| Case 13- config 552 (InF-DH, FR1,UL-TDOA) | N.A. | No(1.70) | No(1.70) | Yes |
| Case 14- config 554 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | Yes | Yes |

Table 8.2.1.8-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

Table 8.2.1.8-2: NR positioning enhancements – vertical accuracy performance summary

| Simulation case (Vertical Error) | Gain vs Rel16 solution @[90]%, [m] | Commercial vertical accuracy requirements [3]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- | --- |
| Case 4- config 220 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.84) | No(1.84) |
| Case 5- config 320 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.19) | No(1.19) |
| Case 6- config 420 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.12) | No(1.12) |
| Case 7- config 421 (InF-DH, FR1,UL-TDOA) | N.A. | No(>3) | No(>3) | No(>3) |
| Case 8- config 422 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.15) | No(1.15) |
| Case 9- config 423 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.56) | No(1.56) |
| Case 10- config 443 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.12) | No(1.12) |
| Case 11- config 444 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(1.43) | No(1.43) |
| Case 12- config 447 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No(2.23) | No(2.23) |
| Case 13- config 552 (InF-DH, FR1,UL-TDOA) | N.A. | No(>3) | No(>3) | No(>3) |
| Case 14- config 554 (InF-DH, FR1,UL-TDOA) | N.A. | Yes | No (0.24) | Yes |

8.2.1.9	Observations from source [14]

Table 8.2.1.9-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.9-1: NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Accuracy achieved @[90]% |
| --- | --- |
| [Case 7], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.074 |
| [Case 8], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 0.38 |
| [Case 9], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 1.96 |
| [Case 10], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | 2.13 |
| [Case 11], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | 0.31 |
| [Case 12], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | 0.69 |
| [Case 21], [InF-SH], [FR2], [400 MHz], [Comb-4], [6dB PB] | 0.086 |
| [Case 22], [InF-SH], [FR2], [100 MHz], [Comb-4], [6dB PB] | 0.36 |
| [Case 23], [InF-SH], [FR2], [50 MHz], [Comb-4], [6dB PB] | 0.77 |
| [Case 24], [InF-SH], [FR2], [20 MHz], [Comb-4], [6dB PB] | 1.45 |

Table 8.2.1.9-2 captures observations based on evaluations results of NR positioning enhancements for vertical location error.

Table 8.2.1.9-2: NR positioning enhancements – vertical accuracy performance summary

| Simulation case (Vertical Error) | Accuracy achieved @[90]% |
| --- | --- |
| [Case 13], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB], [ UEH ∈ [0.5, 2] m] | 0.77 |
| [Case 14], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | 1.51 |
| [Case 15], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | 1.34 |
| [Case 16], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0.5, 2] m] | 1.47 |
| [Case 17], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | 1.34 |
| [Case 18], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | 1.73 |
| [Case 19], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | 2.11 |
| [Case 20], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB], [UEH ∈ [0, 8] m] | 2.57 |

8.2.1.10	Observations from source [21]

Table 8.2.1.10-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.10-1: NR positioning enhancements – horizontal accuracy performance summary [21]

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90]%, [m] | Accuracy achieved @[90]% |
| --- | --- | --- |
| [20] E2, Inf DH, FR1, DL TDOA LOS detection | 9.6m | 8.4m |
| [20] E20, InF SH FR2, UL TDOA multipanel | -0.002 | 0.034039 |
| [21] E21 InF SH FR2, UL TDOA multipanel | 3.81821 | 0.02989 |
| [21] E22 InF SH FR2, DL TDOA multipanel | -0.003 | 0.037037 |
| [21] E23 InF SH FR2, DL TDOA multipanel | 3.7881 | 0.03662 |
| [21] E30, Inf SH, FR1, DL TDOA 1-symbol PRS | -0.06 | 0.54665 |
| [21] E33, IOO, FR1, DL TDOA 1-symbol PRS | 0.02 | 1.404678 |
| [21] E36, Inf SH, FR1, DL TDOA 1-symbol PRS | 0.09 | 0.719702 |
| [21] E39, IOO, FR1, DL TDOA 1-symbol PRS | 0.08 | 1.512598 |
| [21] E42, Inf SH, FR1, DL TDOA 1-symbol PRS | 0.3 | 0.912933 |
| [21] E45, IOO, FR1, DL TDOA 1-symbol PRS | 0.31 | 1.732023 |
| [21] E48, Inf DH, FR1, DL TDOA thresholding | -0.05 | 15.788375 |
| [21] E49, Inf DH, FR1, DL TDOA thresholding | 0,29 | 16.022874 |
| [21] E50, Inf DH, FR1, DL TDOA thresholding | -1.25 | 16.984019 |
| [21] E51, Inf DH, FR1, DL TDOA thresholding | -1.68 | 17.412632 |
| [21] E52, Inf DH, FR1, DL TDOA thresholding | -4.05 | 19.787138 |
| [21] E53, Inf DH, FR1, DL TDOA thresholding | -6.88 | 22.610367 |
| [21] E54, Inf SH, FR1, DL TDOA SRS CS unfolding (PAPR preservation) | 13.4 | 2.52 |
| [21] E55, IOO, FR1, DL TDOA SRS CS unfolding (corr optimization) | 12.98 | 3.04 |
| [21] E58, Inf DH, FR1, DL TDOA PRS aggregation | 1.09 | 13.91966 |
| [21] E59, Inf DH, FR1, DL TDOA PRS aggregation | -0.005 | 15.01508 |
| [21] E61, Inf SH, FR1, DL TDOA PRS aggregation | 0.1798 | 0.079559 |
| [21] E62, Inf SH, FR1, DL TDOA PRS aggregation | -0.016 | 0.275798 |
| [21] E63, Inf SH, FR1, DL TDOA PRS aggregation | -1.18 | 1.447441 |

8.2.1.11	Observations from source [17]

Table 8.2.1.11-1 captures observations based on evaluations results of NR positioning enhancements for horizontal location error.

Table 8.2.1.11-1: NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- |
| Case 22, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | 0.5 |
| Case 22, InH, FR1, Phase offset = 2 degrees, Frequency-domain Stitching | 0.5 |
| Case 22, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | 0.5 |
| Case 22, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | 1.5 |
| Case 22, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | 1.5 |
| Case 23, InH, FR1, Phase offset = 0 degrees, Frequency-domain Stitching | Yes |
| Case 23, InH, FR1, Phase offset = 4 degrees, Frequency-domain Stitching | 1 |
| Case 23, InH, FR1, Phase offset = 8 degrees, Frequency-domain Stitching | 1.4 |
| Case 23, InH, FR1, Phase offset = 10 degrees, Frequency-domain Stitching | 1.5 |
| Case 24, InH, FR1, Channel Spacing = 0 MHz, Frequency-domain Stitching | 2 |
| Case 24, InH, FR1, Channel Spacing = 7.2 MHz, Frequency-domain Stitching | 3 |
| Case 24, InH, FR1, Channel Spacing = 14.4 MHz, Frequency-domain Stitching | 4 |
| Case 24, InH, FR1, Channel Spacing = 36 MHz, Frequency-domain Stitching | 4.5 |
| Case 25, UMI, FR1,Phase offset = 0 degrees, Frequency-domain Stitching | Yes |
| Case 25, UMI, FR1,Phase offset = 4 degrees, Frequency-domain Stitching | 0.2 |
| Case 25, UMI, FR1,Phase offset = 8 degrees, Frequency-domain Stitching | 0.5 |
| Case 25, UMI, FR1,Phase offset = 12 degrees, Frequency-domain Stitching | 1.2 |
| Case 25, UMI, FR1,Phase offset = 18 degrees, Frequency-domain Stitching | 3 |
| Case 26, UMI, FR1,Time offset = 0 ns, Frequency-domain Stitching | Yes |
| Case 26, UMI, FR1,Time offset = 0.5 ns, Frequency-domain Stitching | Yes |
| Case 26, UMI, FR1,Time offset = 1 ns, Frequency-domain Stitching | Yes |
| Case 26, UMI, FR1,Time offset = 2 ns, Frequency-domain Stitching | 0.5 |
| Case 26, UMI, FR1,Time offset = 5 ns, Frequency-domain Stitching | 0.5 |

| Simulation case (Horizontal Error) | Phase Shift between the two aggregated component carriers | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps [m] | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps [m] |
| --- | --- | --- | --- | --- |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | 0 degrees | 23.4 | 24.2 | 23.9 |
|  | 45 degrees | 24.6 | 25.4 | 25.1 |
|  | 90 degrees | 24.5 | 25.3 | 25 |
| Case 27, InF FR1 DH ISD20, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap | 0 degrees | 24.7 | 25.5 | 25.2 |
|  | 45 degrees | 24.4 | 25.2 | 24.9 |
|  | 90 degrees | 27.9 | 28.7 | 28.4 |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 1.8MHz Gap | 0 degrees | 2.4 | 3.2 | 2.9 |
|  | 45 degrees | 1.9 | 2.7 | 2.4 |
|  | 90 degrees | 2.4 | 3.2 | 2.9 |
| Case 27, InF FR1 SH ISD50, Link Quality, Freq. Agg. Of two 100MHz CCs with 100MHz Gap | 0 degrees | 3.25 | 4.05 | 3.75 |
|  | 45 degrees | 3.26 | 4.06 | 3.76 |
|  | 90 degrees | 3.43 | 4.23 | 3.93 |

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| 28 2*200MHz Perfect phase | Yes. | Yes. | Yes. |
| 28 200MHZ(baseline) | Yes. | Yes. | Yes. |
| 29 2*200MHz Perfect phase | 63% | 62% | 62% |
| 29 200MHZ(baseline) | 62% | 61% | 61% |
| 30 2*200MHz Perfect phase | Yes. | 78% | 83% |
| 30 200MHZ(baseline) | Yes. | 66% | 77% |
| 31 2*200MHz Perfect phase | Yes. | Yes. | Yes. |
| 31 200MHZ(baseline) | Yes. | Yes. | Yes. |
| 32 2*200MHz Perfect time | Yes. | 78% | 83% |
| 32 200MHZ(baseline) | Yes. | 66% | 77% |
| 33 2*200MHz Perfect time | Yes. | Yes. | Yes. |
| 33 200MHZ(baseline) | Yes. | Yes. | Yes. |

| Case ID | Kinematic constraint condition | 90% |
| --- | --- | --- |
| Case 34 InF-SH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.049 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.024 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.024 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.031 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.036 |
| Case 35 InF-DH, FR2, DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.058 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.029 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.034 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.038 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.044 |
| Case 36 InH, FR2 DL-TDOA, RANSAC | XY is unknown in the estimation. XY is unknown in the RMS calculation | 0.071 |
|  | X is unknown in the estimation. X is unknown in the RMS calculation | 0.026 |
|  | XY is unknown in the estimation. X is unknown in the RMS calculation | 0.028 |
|  | Y is unknown in the estimation. Y is unknown in the RMS calculation | 0.044 |
|  | XY is unknown in the estimation. Y is unknown in the RMS calculation | 0.053 |

| Horizontal Positioning Error | Gain vs Rel.16 solution, @[90]%, [m] | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- |
| Case 37, UMI, FR1,RTT, With Delta Tau, Perfect Sync, No Timing Errors, AoA & UL PDP Enhancement | 3.4 | 2.9 |
| Case 38, UMI, FR1,DL-TDOA, With Δτ, Perfect Sync, No Timing Errors, UL PDP Enhancement | 1 | 7 |
| Case 39, UMI, FR1, RTT, With Delta Tau, Perfect Sync, No Timing Errors, UL PDP Enhancement | 4.3 | 3.7 |

| Horizontal Positioning Error | Gain vs Rel.16 solution, @[90]%, [m] | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps @[90]% | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps @[90]% |
| --- | --- | --- | --- |
| Case 40, InF FR1 DH ISD20, 100MHz, LOS Genie + Link Quality | -0.009 | Yes | Yes |

8.2.1.12	Observations from source [9]

No observation found

8.2.2	Physical layer latency analysis for NR positioning enhancements

8.2.2.1	Observations from source [4]

Observations on NR positioning latency enhancements are provided in Table 8.2.2.1-1.

Table 8.2.2.1-1: NR positioning enhancements - physical layer latency performance summary [4]

| Description Evaluation Case | L1 Latency,ms | Gain over R16, ms | Commercial requirements [100]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [10]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| Case L101, UL E-CID w/ measurements available | 6-26ms | 0 | Yes | Yes/No | Yes |
| Case L102, UL E-CID w/o measurements available | 46-53.5ms | Negative | Yes | No | Yes |

8.2.2.2	Observations from source [8]

Table 8.2.2.2-1: NR positioning enhancements - physical layer latency performance summary [8]

| Description Evaluation Case | L1 Latency, ms | Gain over R16, ms | Commercial requirements [100]ms are met Yes/No. If No, provide performance gaps | IIoT requirements of [10]ms are met Yes/No. If No, provide performance gaps | IIoT requirements of [100]ms are met Yes/No. If No, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| Case 1, 15kHz, FR1, DL-TDOA | 13.5 | 38 | Yes | No (3.5ms gap) | Yes |

8.2.2.3	Observations from source [5]

Observations on NR positioning latency enhancements are provided in Table 8.2.2.3-1.

Table 8.2.2.3-1: NR positioning enhancements - physical layer latency performance summary [5]

| Description Evaluation Case | L1 Latency,ms | Gain over R16, ms | Commercial requirements [100]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [10]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| [Case 7], [on-demand/aperiodic PRS] | 44.5ms~ | 19.5ms~ |  | 34.5ms~ |  |
| [Case 8], [on-demand/aperiodic MG] | 27.5ms~ | 36.5ms~ |  | 17.5ms~ |  |
| [Case 9], [Positioning BWP] | 28.5ms~ | 35.5ms~ |  | 18.5ms~ |  |
| [Case 10], [physical layer triggered] | 44ms~ | 20ms~ |  | 32ms~ |  |
| [Case 11], [combination scheme] | 5ms~ | 59ms~ |  | Yes |  |
| [Case 12] [idle/inactive] | 27.3ms~ | 58ms or 76.7ms |  | 17.3ms |  |

8.2.2.4	Observations from source [10]

Observations on NR positioning latency enhancements are provided in Table 8.2.2.4-1.

Table 8.2.2.4-1: NR positioning enhancements - physical layer latency performance summary

| Description Evaluation Case | L1 Latency, ms | Gain over R16, ms |
| --- | --- | --- |
| Case 4, InF, FR1, R17, DL-TDOA/DL-AoD | 3.8839 (L1 components) + 4 (L2/L3 components) = 8 (total) | 0.6875 (L1 components) [32] (L2/L3 components) 121.07 (total) |
| Case 5, InF, FR1, R17, UL-TDOA/UL-AoA | 1.9018 (L1 components) + 3 (L2/L3 components) = 5 (total) | 0.8660 (L1 components) [13] (L2/L3 components) 13.7678 (total) |
| Case 6, InF, FR1, R17, Multi-RTT | 4.1875 (L1 components) + 4 (L2/L3 components) = 8.2 (total) | 3.1518 (L1 components) [41] (L2/L3 components) 132.6393 (total) |

8.2.2.5	Observations from source [16]

The latency analysis for each enhancement scenario is summarized in the following table:

| Description Evaluation Case | L1 Latency, ms | Gain over R16 (UE-assisted DL methods (Case 1)), ms |
| --- | --- | --- |
| No measurement gap DL methods | 16 | 17 |
| Measurement gap activation/deactivation DL methods | 24 | 09 |
| On-demand DL PRS methods | 22 | 11 |

8.2.2.6	Observations from source [17]

Observations on NR positioning latency enhancements are provided in Table 8.2.2.1.6-1.

Table 8.2.2.6-1: NR positioning enhancements - physical layer latency performance summary

| Description Evaluation Case | Gain over R16, ms |
| --- | --- |
| Support Low-layer (e.g., unicast/group-common DCI, MAC-CE) triggering of DL/UL PRS transmission/muting/Location-Request for DL-only and DL/UL methods. | 10 msec |
| Support DCI/MAC-CE triggering of Measurement gaps (MG) for the purpose of positioning measurements | >30 msec |
| Fast/real-time processing of short PRS instances: Support Enhanced PRS processing capabilities Support partially-staggered or no-staggered DL-PRS transmissions | >1.5 msec |
| Support Low-layer (e.g. UL MAC-CE or UCI) Measurement Reporting towards the serving gNB | 2 msec |

8.2.2.7	Observations from source [15]

Summary of latency performance analysis is provided in Table 8.2.2.7-1.

Table 8.2.2.7-1: NR positioning enhancements – latency analysis [15]

| Description Evaluation Case | Assumptions | L1 Latency, ms (including preparation/processing time at higher layer) | L1 Latency, ms (excluding preparation/processing time at higher layer) |
| --- | --- | --- | --- |
| Case 1, DL-TDOA, DL-AOD [NW initiated, UE-A] | FR1, 15kHz # of symbols for PUSCH: 1~14 OS # of symbols for PDSCH: 2~14 OS # of symbols for SRS: 2~12 OS -Uplink switching gap is not configured. -No BWP switching -No overlapping symbols of the PUCCH and the scheduled PUSCH -No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH *Note: The maximum latency for PDSCH/PUSCH transmission is assumed as one slot excluding preparation time. Total values may change when the information size related with LPP message is changed. | For UE capability-1: 49.12 ms ~ 198.12 ms For UE capability-2: 47.68 ms ~ 196.26 ms | For UE capability-1: 23.12ms ~ 167.12ms For UE capability-2: 21.68ms ~ 165.26ms |
| Case 2, DL-TDOA, DL-AOD [UE initiated, UE-A] |  | For UE capability-1: 41.41ms ~ 185.84ms For UE capability-2: 40.33ms ~ 184.55ms | For UE capability-1: 22.41ms ~ 165.34ms For UE capability-2: 21.33ms ~ 164.05ms |
| Case 3, UL-TDOA, UL-AOA [NW initiated, UE-A] |  | For UE capability-1: 14.78 ms ~ 20.14 ms For UE capability-2: 14.42 ms ~ 19.57 ms | For UE capability-1: 0.78 ms ~ 2.64 ms For UE capability-2: 0.42 ms ~ 2.07 ms |
| Case 4, Multi-RTT [NW initiated, UE-A] |  | For UE capability-1: 63.9 ms ~ 215.76 ms For UE capability-2: 62.1 ms ~ 213. 33 ms | For UE capability-1: 23.9 ms ~ 169.76 ms For UE capability-2: 22.1 ms ~167.33ms |
| Case 5, E-CID [NW initiated, UE-A] |  | For UE capability-1: 14.56 ms ~ 17.56 ms For UE capability-2: 13.84 ms16.63 ms | For UE capability-1: 1.56ms ~ 3.56ms For UE capability-2: 0.84ms~2.63ms |

8.3	Efficiency analysis for NR positioning enhancements

In this report, Network efficiency and UE efficiency is evaluated either via analytically or via simulations.

8.3.1	Network efficiency analysis for NR positioning enhancements

8.3.1.1	Observations from source [4]

8.3.1.2	Observations from source [8]

8.3.1.3	Observations from source [5]

PRS resource utilization with different periodicities were evaluated

- In FR1, for 20 ms DL PRS periodicity and MG periodicity, 3ms  MGL, 30 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 1 beams per site, PRS resource utilization is 3.21% while the MGL/MGRP (UE overhead)  is 15%.

- In FR2, for 20 ms DL PRS periodicity, 20ms for MGL and MGRP,120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, PRS resource utilization is 51.42% while the MGL/MGRP (UE overhead)  is 100%

The network and device efficiency of aperiodic PRS is multiple of the number of activations.

8.3.1.4	Observations from source [12]

Observation: measurement gap is the major contribution for PRS resource utilization.

8.3.1.5	Observations from source [21]

No observation found

8.3.1.6	Observations from source [18]

Summary of cyclic shift enhancements SRS is provided in Table 8.3.1.6.-1.

Table 8.3.1.6-1: NR positioning enhancements - Cyclic shift SRS enhancements

| Scenario | Rel-16/ Rel-17 enhancements | COMB | nbSym | Cyclic shift range | max number of CSs | Number of simultaneous UEs of CS separated | Improvement factor vs Rel16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Umi | CS enhancement | 4 | 4 | fftlength | 48 | 8 | 4 |
|  | Rel-16 | 4 | 4 | fftlength/KTC | 12 | 2 |  |
| Umi | CS enhancement | 8 | 8 | fftlength | 48 | 8 | 8 |
|  | Rel-16 | 8 | 8 | fftlength/KTC | 6 | 1 |  |
| InF | CS enhancement | 2 | 1 | fftlength/KTC | 8 | 8 | - |
|  | Rel-16 | 2 | 1 | fftlength/KTC | 8 | 8 |  |

Summary of multi-port SRS evaluation is provided in Table 8.3.1.6.-2.

Table 8.3.1.6-2: NR positioning enhancements - SRS resource utilization w.r.t to the number of antenna ports:

| Scenario | Rel-17 enhancements | # of Tx Beams | COMB | nbSym | OFDM symbols per UE | Capacity per UE | Improvement factor vs Rel16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1-Umi | 2-ports | 2 | 4 | 4 | 4 | 1 | 2 |
|  | Rel-16 | 2 | 4 | 4 | 8 | 2 |  |
| FR1-InF | 2-ports | 2 | 2 | 1 | 1 | 0,5 | 2 |
|  | Rel-16 | 2 | 2 | 1 | 2 | 1 |  |
| FR2-Umi | 4-ports | 4 | 8 | 8 | 8 | 2 | 4 |
|  | Rel-16 | 4 | 8 | 8 | 32 | 8 |  |
| FR2-InF (SCS=120kHz) | 4-ports | 4 | 4 | 4 | 4 | 1 | 4 |
|  | Rel-16 | 4 | 4 | 16 | 16 | 4 |  |

8.3.2	UE efficiency analysis for NR positioning enhancements

8.3.2.1	Observations from source [4]

Observations on NR positioning UE efficiency enhancements are provided in Table 8.3.2.1-1.

Table 8.3.2.1-1: NR positioning enhancements – UE efficiency summary [4]

| Description Evaluation Case | Power consumption | Power saved |
| --- | --- | --- |
| Case P1: IDLE/INACTIVE state in every 1.28s | 26392 | 7.2% to Case P2 30.4% to Case P3 |
| Case P2: CONNECTED state inside on-duration in every 1.28s | 28432 | - |
| Case P3: CONNECTED state outside on-duration in every 1.28s | 37936 | - |
| Case P4: IDLE/INACTIVE state in every 1.28s | 11910 | 39.6% to Case P5 |
| Case P5: CONNECTED state outside on-duration in every 1.28s | 19712 | - |

8.3.2.2	Observations from source [5]

Observations of UE efficiency for power consumption

Table 8.3.2.2-1: NR positioning enhancements – power consumption performance in connected state [5]

| Power saving scheme description | Average power consumption (power unit) | Power reduction compared to baseline | Note |
| --- | --- | --- | --- |
| PRS measurement impacted by DRX | 53.5625(baseline) 2 PRS occasions every DRX cycle(160ms) | - | In this case, PRS period=80ms DRX cycle=160ms If PRS measurement is impacted by DRX, UE is only expected to measure PRS in DRX active time. |
|  | 35.2500 1 PRS occasion every DRX cycle (160ms) | 34.19% |  |
| Extending PRS period | 35.2500(baseline) PRS period=160ms | - | - |
|  | 27.4844 PRS period=320ms | 22.03% |  |
|  | 23.6016 PRS period=640ms | 33.05% |  |
| Concentrated PRS distribution | 43.3937(baseline) 4 distributed PRS occasion every 160ms | - | In this case, the duration of concentrated PRS distribution is 5ms with 4ms PRS length and 1ms MG switching time. While for distributed PRS, we divide the concentrated PRS occasion of 4ms (baseline) into 4 PRS occasions with 1ms, and the adjacent PRS occasions are separated by 40ms. |
|  | 35.2500 1 concentrated PRS occasion every 160ms | 18.77% |  |
| Adding PRS-MTC window | 35.2500(baseline) without PRS-MTC PRS occasion duration=4ms | - | - |
|  | 28.0313 PRS-MTC to limit PRS measurement PRS occasion duration=2ms | 20.48% | - |
| Reducing number of TRPs to be measured | 35.2500(baseline) Number of TRPs=8 | - | - |
|  | 32.2500 Number of TRPs=4 | 8.51% |  |
| Reducing number of positioning frequency layers to be measured | 82.4688 (baseline) Number of FLs=4 | - | - |
|  | 52.0313 Number of FLs=2 | 36.91% |  |
|  | 35.2500 Number of FLs=1 | 57.26% |  |

Table 8.3.2.2-2: NR positioning enhancements – power consumption comparison in idle state and connected state [5]

| Power saving scheme description | Average power consumption (power unit) | Power reduction compared to baseline | Additional assumptions | Note |
| --- | --- | --- | --- | --- |
| 1. Connected state measurement and report | 11.1367(baseline) | - | 1. UE starts positioning from idle state 2.LPP/RRC procedures for positioning are ignored. 3. Only one shot positioning measurement and report considered. 4.Once the positioning report is completed, the RRC connection is released 5. Measurement/report cycle is equal to idle state (1280ms). 6. The power unit for PRS measurement in connected state is equal to PRS bandwidth in idle state 7. Paging occasion power is equivalent to ‘PDCCH+PDSCH’, considering it may lead to RRC state transition | Considering that some assumptions are made to simplify power consumption evaluation, such as ignoring complicated steps for LPP procedures, aligning the bandwidth and period with idle state measurement, it will consume more power when positioning in the connected state in general. |
| 2. Idle state measurement and connected state report | 10.3246 | 7.29% | UE switches to connected mode to report. Once the positioning report is completed, the RRC connection is released |  |
| 3. Idle state measurement and idle state report | 5.7488 | 48.38% | - |  |

The UE efficiency of periodic PRS and MG and on-demand MG with concentrated PRS in FR2 were evaluated:

    - For 20 ms DL PRS periodicity and MGRP, 20ms for MGL, 120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, the UE efficiency(  MGL/MGRP) is 100%

    - For 160 ms DL PRS periodicity and MGRP, 20ms for MGL, 120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, the UE efficiency(  MGL/MGRP) is 12.5%

    - For on-demand MG with concentrated PRS, the range of UE efficiency is 0%-40% depends on the configuration of on-demand MG and PRS

The UE efficiency of periodic PRS and MG and positioning BWP without MG in FR2 were evaluated:

    - For 20 ms DL PRS periodicity and MG, 20ms for MGL, 120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, the UE efficiency(  MGL/MGRP) is 100%

    - For 160 ms DL PRS periodicity and MGRP, 20ms for MGL, 120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, the UE efficiency(  MGL/MGRP) is 12.5%

    - For 20 ms DL PRS periodicity and without MG, the UE efficiency is 30% only depends on DL PRS symbols

    - For 160 ms DL PRS periodicity and without MG, the UE efficiency is 1.88% only depends on DL PRS symbols

8.4	Summary of performance evaluations

Performance analysis of baseline I-IoT InF scenarios shows that InF-SH scenario (Scenario 1) is characterized by high probability of LOS links. In InF-DH (Scenario 2) the probability of LOS links is reduced substantially while probability of NLOS links is increased accordingly.

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the InF-SH scenario (Scenario 1).

    - Based on the results provided by a majority of sources, sub-meter level @ 90% of horizontal positioning accuracy is achieved by Rel.16 solutions.

    - For horizontal accuracy, results were provided by 13 sources([4], [6], [7], [8], [9], [12], [13], [14], [19], [20], [17], [5], [10]) out of 17 sources for FR1 and by 9 sources ([4], [6], [7], [8], [14], [20], [17], [5], [10]) out of 17 for FR2

- For NR positioning evaluations in FR1 band, the following is observed with respect to horizontal positioning accuracy:

  - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from 4 sources ([8, [20], [5],[17]) and is not achieved in contributions from 9 sources ([4], [6], [7], [9], [12], [13], [14], [19], [10])

  - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from 6 sources ([4], [8], [20], [17], [5], [10])and is not achieved in contributions from 7 sources ([6], [7], [9], [12], [13], [14], [19])

- For NR positioning evaluations in FR2 band, the following is observed with respect to horizontal positioning accuracy:

  - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [7] ([4],[7], [8], [20], [17], [5], [10])sources and is not achieved in contributions from 2 sources ([6], [14])

  - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from 9 sources ([4], [6], [7], [8], [14], [20], [17], [5] ,[10]) and is not achieved in contributions from 0 sources

    - For vertical accuracy, results were provided by 4 sources ([7], [8], [5], [10]) out of 17 for FR1 and by 4 sources ([7], [8], [17], [10]) out of 17 for FR2 band

- For NR positioning evaluations in FR1 band, the following is observed with respect to vertical positioning accuracy:

  - Accuracy of ≤ 1m @ 90% is achieved in contribution from 2 sources ([7], [5]) and is not achieved from 2 sources ([8], [10])

- For NR positioning evaluations in FR2 band, the following is observed with respect to vertical positioning accuracy:

  - Accuracy of ≤ 1m @ 90% is achieved in contribution from 4 sources ([7], [8], [17], [10]) [and is not achieved by 0 sources]

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the baseline InF-DH scenario (Scenario 2), including evaluations with variable gNB/UE heights for vertical accuracy

- Based on the results provided by a majority of sources, sub-meter level @ 90% of horizontal positioning accuracy is not achieved by Rel.16 based solutions.

- For horizontal accuracy, results were provided by 14 sources ([4], [6], [7], [8], [9], [12], [13], [14], [19], [20], [17], [5], [10], [18]) out of 17 for FR1 and by 9 sources ([4], [6], [7], [8], [14], [20], [17], [5], [10]) out of 17 for FR2

- For NR positioning evaluations in FR1 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 0.2m @ 90% is achieved in contribution from [3] sources ([8],[17],[5]) and is not achieved in contributions from 11 sources ([4], [6], [7], [9], [12], [13], [14], [19], [20], [10], [18])

- Accuracy of ≤ 0.5m @ 90% is achieved in contributions from 4 sources ([6], [8], [17], [5]) and is not achieved in contributions from 10 sources ([4], [7], [9], [12], [13], [14], [19], [20], [10], [18])

- For NR positioning evaluations in FR2 band, the following is observed with respect to horizontal positioning accuracy:

  - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from 4 sources ([6], [17], [5], [8]) and is not achieved in contributions from 6 sources ([4], [7],, [14], [20], [10])

  - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from 4 sources ([6], [17], [5], [8]) and is not achieved in contributions from 6 sources ([4], [7], [14], [20], [10])

    - For vertical accuracy, results were provided by 6 sources ([7], [8], [5], [10], [4], [18]) out of 17 for FR1 and by 4 sources ([7], [8], [10], [4]) out of 17 for FR2 band

- For NR positioning evaluations in FR1 band, the following is observed with respect to vertical positioning accuracy:

  - Accuracy of ≤ 1m @ 90% is achieved in contribution from 2 sources ([8], [5]) and is not achieved from 4 sources ([7], [10], [4], [18])

- For NR positioning evaluations in FR2 band, the following is observed with respect to vertical positioning accuracy:

  - Accuracy of ≤ 1m @ 90% is achieved in contribution from 1 source ([4]) and is not achieved from [3] sources ([7], [8], [10])

For the issues related to mitigating effects of multipath/NLOS for positioning

- Evaluation results for LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms in indoor factory scenarios were provided by 12 sources ([12], [9], [5], [10], [17], [7], [4], [19], [13], [14], [18], [20]) out of 17 sources

- NR positioning utilizing LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms improve performance of positioning accuracy with respect to solutions that do not apply these techniques

- From the evaluations,

  - 9 sources ([9], [10], [7], [4], [19], [13], [14], [18], [20]) evaluated LOS/NLOS identification with additional specification changes relative to Rel.16 solutions

  - 2 sources ([5], [17]) evaluated outlier rejection algorithm (implementation-based algorithm that can be applied for Rel.16 solutions without specification changes)

  - 1 source ([12]) evaluated NLOS mitigation using triangle-based inequality algorithm (implementation-based algorithm that can be applied for Rel.16 solutions without specification changes)

- Comparative analysis of LOS/NLOS identification with specification changes vs implementation based methods (outlier rejection algorithms) was done by 6 sources ([10], [4], [5], [17], [7], [12])

  - Three sources ([10], [4], [7]) observe that NR positioning based on LOS/NLOS identification outperforms NR positioning utilizing outlier rejection

  - Three sources ([5], [17], [12]) observe that NR positioning utilizing outlier rejection outperforms NR positioning utilizing LOS/NLOS identification

For issues related to gNB/UE TX/RX timing errors

- Evaluation results of gNB/UE TX/RX timing errors (as per the optional model) are provided by 7 sources ([4], [7], [17], [10], [8], [20], [5]) out of 17 sources)

- Summary of results is provided in tables B.1-1 to B.1-4

For the issues related to aggregation of DL positioning frequency layers:

- Evaluation results for aggregation of DL positioning frequency layers were provided by 5 sources ([10], [17], [4], [22], [20]) out of 17.

- Aggregation of NR positioning frequency layers improves positioning accuracy under certain scenarios, configurations, and assumptions on modelled impairments such as: bandwidth and spacing of aggregated layers, timing offset and frequency offset over frequency layers, phase discontinuity and possible amplitude imbalance.

  - One source ([4]) observes that aggregation with phase continuity can help to improve the positioning accuracy, and discontinuous aggregation can approach the performance of contiguous aggregation with the same frequency span

  - One source ([10]) has shown that aggregation of frequency layers (without modeling impairments) improves the positioning accuracy for intra-band contiguous configuration and that further study is needed for other cases including impairments

  - One source ([20]) has observed that PRS aggregation shows potential gains without modeling phase error, but these gains are lost when the phase error between CCs becomes too large

  - One source ([17]) has analyzed aggregation of 2 and 4 frequency layers for different channel spacings, time and phase offset across frequency layers

  - One source ([22] has analyzed aggregation of 2 frequency layers for different time offset values and observed that:

- For the case without impairments modeling, aggregation of multiple DL positioning frequency layers 50MHz+50MHz, performance target [0.2m @ 90%] cannot be achieved in both InF-SH and InF-DH.

- For the case without impairments modeling, aggregation of multiple DL positioning frequency layers 50MHz+50MHz, the performance is worse than 100MHz but better than 50MHz.

- The performance of aggregation of frequency layers degrades if timing offset is increased

For issues related to physical layer latency for Rel.16 DL-TDOA/DL-AOD

- Summary of results is provided in table B.2-1

- Summary of physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning in FR1 was provided by 11 sources

- Summary of physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning in FR2 was provided by 5 sources

- For evaluation in FR1,

  - results from 11 sources out of 11 sources ([17], [4], [7], [5], [11], [15], [8], [13], [12], [16], [10]) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 10ms

  - results from [2] ([7], [10]) sources out of 11 sources ([17], [4], [7], [5], [11], [15], [8], [13], [12], [16], [10]) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 100ms

- For evaluation in FR2,

  - results from 5 sources out of 5 sources ([7], [5], [11], [12],[13]) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 10ms

  - results from 2 ([7], [5]) sources out of 4 sources ([7], [5], [11], [12]) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 100ms

- The following list provides the major physical layer latency components for Rel.16 DL TDOA/DL-AOD UE-assisted NR Positioning

  - DL PRS alignment, transmission, measurement (including processing time) and report delay

  - Measurement gap request, configuration and alignment time

  - UE/gNB higher layer (LPP/RRC) processing times

For issues related to physical layer latency for Rel.16 UL-TDOA/UL-AOA

- Summary of results is provided in table B.2-2

- Summary of physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning in FR1 was provided by 8 sources ([4], [5], [15], [8], [13], [12], [16], [10])

- Summary of physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning in FR2 was provided by 2 sources ([5], [12])

- For evaluation in FR1,

  - results from [3] sources ([4], [8], [13]) out of 8 sources ([4], [5], [15], [8], [13], [12], [16], [10]) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 10ms

  - results from 8 sources out of 8 sources ([4], [5], [15], [8], [13], [12], [16], [10]) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 100ms

- For evaluation in FR2,

  - results from 2 sources out of 2 sources ([5], [12]) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning exceeds 10ms

  - results from [1] ([12]) sources out of 2 sources ([5], [12]) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 UL-TDOA/UL-AOA NR Positioning

  - SRS for positioning processing time

  - SRS for positioning alignment time (depends on periodic or aperiodic SRS for positioning)

  - gNB higher layer processing delays (RRC/ NRPPa processing times)

For issues related to physical layer latency for Rel.16 Multi-RTT

- Summary of results is provided in table B.2-3

- Summary of physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning in FR1 was provided by 6 sources ([17], [4], [5], [15], [16], [10])

- Summary of physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning in FR2 was provided by 0 sources

- For evaluation in FR1,

  - results from 6 sources ([17], [4], [5], [15], [16], [10]) out of 6 sources ([17], [4], [5], [15], [16], [10]) show that minimum estimated physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning exceeds 10ms

  - results from 4 sources ([17], [4], [5], [16]) out of 6 sources ([17], [4], [5], [15], [16], [10]) show that minimum estimated physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 Multi-RTT UE-assisted NR positioning

  - DL PRS alignment, transmission, measurement time and report delay

  - Measurement gap request, configuration, alignment time

  - SRS for positioning processing time

  - SRS for positioning alignment time (depends on periodic or aperiodic SRS for positioning)

  - UE/gNB higher layer (LPP/RRC/NRPPa) processing times

For issues related to physical layer latency for Rel.16 E-CID NR positioning

- Summary of results is provided in table B.2-4

- Summary of physical layer latency for Rel.16 E-CID NR positioning in FR1 was provided by [3] sources ([4], [7], [15])

- Summary of physical layer latency for Rel.16 E-CID NR positioning in FR2 was provided by 0 sources

- For evaluation in FR1,

  - results from 2 sources ([7], [15]) out of 3 sources ([4], [7], [15]) show that minimum estimated physical layer latency for Rel.16 E-CID NR positioning exceeds 10ms

  - results from [3] sources ([4], [7], [15]) out of 3 sources ([4], [7], [15]) show that minimum estimated physical layer latency for Rel.16 E-CID NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 E-CID NR positioning

  - Higher layer signaling processing

For issues related to physical layer latency for Rel.16 DL-only UE-based NR positioning

- Summary of results is provided in table B.2-5

- Summary of physical layer latency for Rel.16 DL-only UE-based NR positioning in FR1 was provided by 6 sources ([17], [4], [5], [11], [12], [16])

- Summary of physical layer latency for Rel.16 DL-only UE-based NR positioning in FR2 was provided by 2 sources ([5], [11])

- For evaluation in FR1,

  - results from 4 sources ([4], [5], [12], [16]) out of 6 sources ([17], [4], [5], [11], [12], [16]) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 10ms

  - results from 6 sources out of 6 sources ([17], [4], [5], [11], [12], [16]) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning does not exceed 100ms

- For evaluation in FR2,

  - results from 2 sources out of 2 sources ([5], [11]) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 10ms

  - results from [1] ([5]) sources out of 2 sources ([5], [11]) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 100ms

- The following list provides the major physical layer latency components for Rel.16 DL-only UE-based NR positioning

  - DL PRS alignment, transmission, measurement time and, if requested, report delay

  - Measurement gap request, configuration, alignment time

  - Higher layer (LPP/RRC) processing times

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMa scenario

- Based on the results provided, 10 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMa scenario

- Results were provided by 2 sources ([20], [17]) out of 17 for FR1 band

- For NR positioning evaluations for UMa scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 80% is achieved for the outdoor UEs in contributions from 1 source ([17]) out of 2 sources ([20], [17]) in the scenario without absolute time of arrival modelling. Zero sources met an accuracy of ≤ 1m @ 90%.

- Accuracy of ≤ 10m @ 90% is achieved for the outdoor UEs in contributions from 2 sources ([20], [17]) out of 2 sources in the scenario without absolute time of arrival modelling

- Accuracy of ≤ 10m @ 90% is achieved for the indoor UEs in contributions from 1 source ([20]) out of 2 sources in the scenario without absolute time of arrival modelling

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMi scenario

- Results were provided by 4 sources ([13], [20], [17], [18]) out of 17 for FR1 band

- For NR positioning evaluations for UMi scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 90% is achieved in contributions from 2 sources ([20], [17]) and is not achieved from 2 sources ([13], [18]) in the scenario without absolute time of arrival modelling

Accuracy of ≤ 1m @ 90% is not achieved from 2 sources ([17], [18]) in a scenario with absolute time of arrival modellin

- For NR positioning evaluations for UMi scenario in FR2 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 90% is achieved in contributions from 1 source ([17]]) in the scenario without absolute time of arrival modelling.

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the IOO scenario

- Based on the results provided by a majority of the sources, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in IOO scenario

- Results were provided by 5 sources ([8], [13], [14], [20], [23]) out of 17 for FR1 and 5 sources ([8], [14], [20], [17], [23]) out of 17 for FR2 band

- For NR positioning evaluations for IOO scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 90% is achieved in contributions from 4 sources ([8], [14], [20], [23]) and is not achieved from 1 source ([13]) in the scenario without absolute time of arrival modelling

- Accuracy of ≤ 1m @ 90% is achieved from 1 source ([23]) in a scenario with absolute time of arrival modelling

- For NR positioning evaluations for IOO scenario in FR2 band, the following is observed with respect to horizontal positioning accuracy:

- Accuracy of ≤ 1m @ 90% is achieved in contributions from 5 sources ([8], [14], [20], [17], [23]) in the scenario without absolute time of arrival modelling

- Accuracy of ≤ 1m @ 90% is achieved from 1 source ([23]) in a scenario with absolute time of arrival modelling

The results for the UE efficiency (power saving) in the RRC_IDLE/RRC_INACTIVE states were analyzed by 2 sources ([4], [5]) out of 17 sources (assumptions may be different between the different sources)

- In one source ([4]), the following observations were made:

  - RRC_IDLE/RRC_INACTIVE state positioning can save about 7%-40% power consumption compared to C-DRX configuration

- In one source ([5]), the following observations were made:

  - Positioning report in the RRC_IDLE state can provide 44.32 % of power saving gain compared to the report in the RRC_CONNECTED state

  - Positioning measurement and report in the RRC_IDLE state can provide at least 48.38 % of power saving gain compared to the measurement and report in the RRC_CONNECTED state

The results for the PRS resource utilization were analyzed by 3 sources ([4], [5], [8]) out of 17 sources

- In one source ([4]), the PRS resource utilization was evaluated for the case of 160 ms DL PRS periodicity, 30 kHz subcarrier spacing, and 12, 4, and 1 symbol per PRS resource:

  - PRS with 12, 4, and 1 symbol has positioning resource utilization of 2.14 %, 0.714 %, and 0.179 %, respectively

- In one source ([5]), the PRS resource utilization was evaluated:

  - In FR1, for 20 ms DL PRS periodicity and MG periodicity, 3ms MGL, 30 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 1 beams per site, PRS resource utilization is 3.21% while the MGL/MGRP (UE overhead) is 15%.

  - In FR2, for 20 ms DL PRS periodicity, 20ms for MGL and MGRP, 120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, PRS resource utilization is 51.42% while the MGL/MGRP (UE overhead) is 100%

  - It was observed by the source that the network and device efficiency can be improved by on-demand PRS (assuming the same latency) compared to periodic PRS

- In one source ([8]), the PRS resource utilization was evaluated for the case of 20 ms DL PRS periodicity, 30 kHz subcarrier spacing, and 12 symbols per PRS resource:

  - PRS with 12 symbols has positioning resource utilization of 2.1 %.

9	Positioning integrity and reliability

From objective 2: Includes solutions necessary to support integrity and reliability of assistance data and position information:

10	Identified NR impacts in Rel-17

10.1	NR positioning for UEs in RRC_INACTIVE state

NR positioning for UEs in RRC_INACTIVE state is recommended for normative work, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

The details of how to enable the UE positioning in RRC_ INACTIVE state can be further discussed during normative work. These details may include, but are not limited to the following aspects:

  - UL reference signals (e.g., SRS for positioning, PRACH preambles) for UL measurements

  - Signalling and procedures for support the assistance data delivery, DL-PRS configuration, UL reference signals for positioning resource configuration, measurement reporting), which may be developed based on the enhancements of existing signalling and procedures (e.g., existing 2-step and/or 4-step PRACH procedures, paging procedure, small data transmission).

10.2	On-demand transmission and reception of DL PRS

From a physical layer perspective, on-demand transmission and reception of DL PRS, which includes at least the following is recommended

- UE-initiated request of on-demand DL PRS transmission

- LMF (network)-initiated request of on-demand DL PRS transmission

- Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

10.3	Aggregation of DL PRS resources

Simultaneous transmission by the gNB and reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs can be studied further and if needed, specified during normative work

- From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, bands and RF architectures, can be further studied

10.4	Aggregation of SRS for positioning resources

Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers can be studied further and if needed, specified during normative work.

- From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, particular bands and RF architectures, can be further studied.

10.5	Enhancements for UE Rx/Tx and gNB Rx/Tx timing delays

The methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or and gNB Rx/Tx timing delays are recommended for normative work, including

- DL, UL and DL+UL positioning methods

- UE-based and UE-assisted positioning solutions

- Note: The details of the solutions are left for further discussion in normative work.

10.6	Enhancements for angle based methods

The enhancements of the procedure, measurements, reporting, and signalling for improving the accuracy of

- UL AoA is recommended for normative work for network-based positioning solutions.

- DL-AoD is recommended for normative work for UE-based and network-based (including UE-assisted) positioning solutions.

10.7	Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation

Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation can be studied further, and if needed, specified during normative work for improving positioning accuracy.

- Note: The details of the enhancements of reporting are left for further discussion in normative work, which may include, but are not limited to the following information associated with multi-path, e.g., LOS/NLOS identification, time of arrival of the multi-path components, signal power and/or relative power, power delay profile, angle, and/or polarization information, coherence bandwidth, etc.

10.8	Enhancements of signaling & procedures for reducing NR positioning latency

- Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work.

- Note: Aperiodic reception would correspond to DCI-triggered reception

- Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work.

- Note: Semi-persistent reception in the above corresponds to MAC-CE activated reception

The following enhancements of signaling & procedures for reducing NR positioning latency are recommended for normative work, including DL and DL+UL positioning methods

  - The details of the solutions are left for further discussion in normative work, which may include the following aspects:

    - Latency reduction related to the measurement gap

    - Latency reduction related to the reporting and request (e.g., via RRC signaling, MAC-CE and/or physical layer procedure, and/or priority rules)

    - Latency reduction related to measurements

The following enhancements of signaling & procedures for reducing NR positioning latency can be studied and specified, if needed

  - Latency reduction related to the request and response of positioning assistance data (e.g., via RRC signaling, MAC-CE and/or physical layer procedure)

  - Latency reduction related to the reception of DL PRS (e.g., priority rules for the reception of DL PRS)

No assumptions are made on whether the LCS architecture specified in TS 23.273 is enhanced or not.

10.9	DL positioning measurement in RRC_IDLE state

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state.

  - Note: This does not imply that measurements have to be reported in RRC_IDLE state.

11	Conclusions

This study focused on the analysis of potential enhancements and solutions necessary to support the high accuracy, low latency, high network efficiency and device efficiency to NR positioning targeting both general commercial and IIOT use cases.

In the study item, Rel-17 target positioning requirements for RAT dependent solutions were discussed and defined for general commercial use cases and IIoT use cases, including horizontal and vertical positioning accuracy, and physical layer and end-to-end positioning latency (see Section 5). Additional scenarios and channel models for evaluating Rel-17 NR positioning enhancements were developed for the evaluation of the achievable positioning performance of the enhancements (see Section 6).

The potential positioning enhancements for improving positioning accuracy, reducing latency, and improving network and device efficiency of NR positioning were studied. The potential positioning enhancements, which were investigated rigorously in this study, are outlined in Section 7.

NR positioning accuracy with Rel.16 positioning solutions were evaluated under the condition that gNB time synchronization error and gNB/UE TX/RX timing errors are not modelled for InF-SH scenario and InF-DH scenario for both FR1 and FR2 bands. The evaluation results show:

- For horizontal positioning accuracy,

    - in the InF-SH scenario, based on the results provided [by a majority of sources], sub-meter level @ 90% is achieved in both FR1 and FR2 bands.

    - in the InF-DH scenario, based on the results provided [by a majority of sources], sub-meter level @ 90% is not achieved in both FR1 and FR2 bands.

- For vertical positioning accuracy

    - in the InF-SH scenario,

    - sub-meter level @ 90% is achieved by some sources but not achieved by some other sources in FR1 band

    - sub-meter level @ 90% is achieved by all sources in FR2 band;

    - in the InF-DH scenario,

    - sub-meter level @ 90% is achieved by some sources and is not achieved by some other sources in both FR1 and FR2 bands

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the IOO scenario

- Based on the results provided by a majority of the sources, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in IOO scenario

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMa scenario

- Based on the results provided, 10 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMa scenario

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMi scenario

- Based on the results provided by some of the companies, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMi scenario

The impact of NLOS/multipath on NR positioning accuracy and the resolutions for NLOS/multipath mitigation were investigated. NR positioning utilizing LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms improve performance of positioning accuracy with respect to solutions that do not apply these techniques.

The impact of gNB/UE TX/RX timing errors on NR positioning accuracy were investigated. Evaluation results show the gNB/UE TX/RX timing errors have significant impact on positioning accuracy.

Aggregation of NR positioning frequency layers for improving positioning accuracy were investigated. Evaluation results show that aggregation of NR positioning frequency layers improves positioning accuracy under certain scenarios, configurations, and assumptions on modelled impairments as outlined in Section [8.4].

Physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-Assisted, UL-TDOA/UL-AOA, Multi-RTT, E-CID and DL-only UE-based NR positioning were investigated, and the major physical layer latency components for these NR positioning techniques were also identified as shown in Section [8.4].

The UE efficiency (power saving) in the RRC_IDLE/RRC_INACTIVE states were also analysed, and power saving gains are observed with detailed observations related to power savings are outlined in Section [8.4].

The network efficiency in terms of resource utilization was analyzed and benefits of potential positioning enhancements observed are outlined in Section [8.4].

The potential positioning enhancements for improving positioning accuracy, reducing latency, and improving network and device efficiency of NR positioning were studied.

The following enhancements have been recommended for normative work

- NR positioning for UEs in RRC_INACTIVE state, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

- On-demand transmission and reception of DL PRS, which includes at least

  - UE-initiated request of on-demand DL PRS transmission

  - LMF (network)-initiated request of on-demand DL PRS transmission

  - Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

- The methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or and gNB Rx/Tx timing delays, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- The enhancements of the procedure, measurements, reporting, and signalling for improving the accuracy of

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions

- The enhancements of signaling & procedures for reducing NR positioning latency related to DL and DL+UL positioning methods

  - the measurement gap

  - the measurement request and reporting (e.g., via RRC signaling, MAC-CE and/or physical layer procedure, and/or priority rules)

  - the measurement time

The following enhancements are considered beneficial for the purpose of improving positioning accuracy, reducing latency, improving network and/or device efficiency and are being recommended to be studied further and if needed, specified during normative work

- Simultaneous transmission by the gNB and aggregated reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs

- Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers

- Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation

- Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs

- Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs

- Enhancements of signaling & procedures for reducing NR positioning latency related to

  - the request and response of positioning assistance data (e.g., via RRC signaling, MAC-CE and/or physical layer procedure)

  - the reception of DL PRS (e.g., priority rules for the reception of DL PRS

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. This does not imply that measurements have to be reported in RRC_IDLE state.

It is recommended to proceed with a normative work to support NR positioning enhancements.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-05 | RAN1#101-e | R1-2004948 |  |  |  | Baseline TR skeleton. | 0.0.1 |
| 2020-10 | RAN1#103-e | R1-2009430 |  |  |  | Update of TR based on RAN1#101-e and RAN1#102-e agreements. | 0.1.0 |
| 2020-10 | RAN1#103-e | R1- 2009544 |  |  |  | Update of TR based on RAN1#103-e agreements. | 0.1.1 |
| 2020-11 | RAN1#103-e | R1- 2009670 |  |  |  | Update of TR based on RAN1#103-e agreements. | 0.2.0 |
| 2020-11 | RAN1#103-e | R1- 2009745 |  |  |  | Update of TR based on RAN1#103-e agreements. | 0.3.0 |

Annex B:
Appendix to summary of performance evaluations

B.1 Evaluation of horizontal positioning accuracy with gNB/UE TX/RX timing error

Table B.1-1: Summary of evaluated gNB/UE TX/RX timing error parameters and achieved horizontal positioning accuracy in InF-SH baseline scenario for Rel.16 positioning method.

| Company name (Positioning method) | FR1 / FR2 | gNB/UE TX/RX timing error mitigation is on/off | Evaluated UE TX/RX timing error values (Y value) | Evaluated gNB TX/RX timing error values (X value) | Is horizontal positioning accuracy 0.2m @ 90% met? | Is horizontal positioning accuracy 0.5m @ 90% met? |
| --- | --- | --- | --- | --- | --- | --- |
| [10] (Multi-RTT) | FR1 | Off at gNB Off at UE | 10 ns | 5 ns | NO | NO |
|  | FR1 | Ideal at gNB On at UE | 0 ns | 5 ns | NO | YES |
| [7] (DL-TDOA) | FR1 | Off at gNB | 0 ns | 0.5 ns | NO | NO |
|  | FR2 | Off at gNB | 0 ns | 0.5 ns | NO | YES |
| [4] (DL/UL-TDOA) | FR1 | Off at gNB | N/A | 1.4ns (2ns inter-gNB difference) | NO | NO |
| [4] (UL-TDOA/AoA) | FR1 | Off at gNB | N/A | 1.4ns (2ns inter-gNB difference) | NO | YES |
| [4] (Multi-RTT) | FR1 | Off at gNB Off at UE | 5.6ns (8ns intra-UE Rx - Tx difference) | 1.4ns (2ns intra-gNB Rx – Tx difference) | NO | NO |
| [4] (UL-TDOA) | FR1 | On at gNB | N/A | 0ns inter-gNB difference | YES | YES |
|  |  |  |  | 0.2ns inter-gNB difference | YES | YES |
|  |  |  |  | 0.5ns inter-gNB difference | NO | YES |
|  |  |  |  | 1ns inter-gNB difference | NO | NO |
| [5] (DL-TDOA) | FR1 | Off at gNB Off at UE | 0 ns | 0 ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1ns | 0.5ns | NO | YES |
|  |  |  | 2ns | 0.5ns | NO | YES |
|  |  |  | 3ns | 0.5ns | NO | YES |
|  |  |  | 5ns | 0.5ns | NO | YES |
|  |  |  | 0.5ns | 1ns | NO | YES |
|  |  |  | 0.5ns | 2ns | NO | NO |
|  |  |  | 0.5ns | 3ns | NO | NO |
|  |  |  | 0.5ns | 5ns | NO | NO |
| [5] (Multi-RTT) | FR1 | Off at gNB Off at UE | 0 ns | 0 ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1ns | 0.5ns | NO | YES |
|  |  |  | 2ns | 0.5ns | NO | YES |
|  |  |  | 3ns | 0.5ns | NO | YES |
|  |  |  | 5ns | 0.5ns | NO | YES |
|  |  |  | 0.5ns | 1ns | NO | YES |
|  |  |  | 0.5ns | 2ns | NO | YES |
|  |  |  | 0.5ns | 3ns | NO | NO |
|  |  |  | 0.5ns | 5ns | NO | NO |
| [17] (DL-TDOA) | FR2 | Off at gNB Off at UE | 0.0ns | 0.0ns | YES | YES |
|  |  |  | 0.1ns | 0.1ns | YES | YES |
|  |  |  | 0.2ns | 0.2ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1.0ns | 1.0ns | NO | NO |
|  |  |  | 2.0ns | 2.0ns | NO | NO |
| [20] (DL-TDOA) | FR2 | Off at gNB Off at UE | N/A | 0ns | YES | YES |
|  |  | Off at gNB Off at UE | N/A | 1ns | NO | YES |
|  |  | Off at gNB Off at UE | N/A | 2ns | NO | NO |
|  |  | Off at gNB Off at UE | N/A | 4ns | NO | NO |
|  |  | Off at gNB Off at UE | N/A | 8ns | NO | NO |
|  |  | On at gNB | N/A | 0ns | YES | YES |
|  |  | On at gNB | N/A | 8ns | YES | YES |

Table B.1-2: Summary of evaluated gNB/UE TX/RX timing error parameters and achieved horizontal accuracy in InF-DH baseline scenario for Rel.16 positioning methods.

| Company name (Positioning method) | FR1 / FR2 | gNB/UE TX/RX timing error mitigation is on/off | Evaluated UE TX/RX timing error values (Y value) | Evaluated gNB TX/RX timing error values (X value) | Is positioning accuracy 0.2m @ 90% met? | Is positioning accuracy 0.5m @ 90% met? |
| --- | --- | --- | --- | --- | --- | --- |
| [10] (Multi-RTT) | FR1 | Off at gNB Off at UE | 10 ns | 5 ns | NO | NO |
|  | FR1 | Ideal at gNB On at UE | 0 ns | 5 ns | NO | NO |
| [7] (DL-TDOA) | FR1 | Off at gNB | 0 ns | 0.5 ns | NO | NO |
|  | FR2 | Off at gNB | 0 ns | 0.5 ns | NO | NO |
| [4] (DL/UL-TDOA) | FR1 | Off at gNB | N/A | 1.4ns (2ns inter-gNB difference) | NO | NO |
| [4] (UL-TDOA/AoA) | FR1 | Off at gNB | N/A | 1.4ns (2ns inter-gNB difference) | NO | NO |
| [4] (Multi-RTT) | FR1 | Off at gNB Off at UE | 5.6ns (8ns intra-UE Rx - Tx difference) | 1.4ns (2ns intra-gNB Rx – Tx difference) | NO | NO |
| [5] (DL-TDOA) | FR1 | Off at gNB Off at UE | 0 ns | 0ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1ns | 0.5ns | NO | YES |
|  |  |  | 2ns | 0.5ns | NO | YES |
|  |  |  | 3ns | 0.5ns | NO | YES |
|  |  |  | 5ns | 0.5ns | NO | YES |
|  |  |  | 0.5ns | 1ns | NO | YES |
|  |  |  | 0.5ns | 2ns | NO | NO |
|  |  |  | 0.5ns | 3ns | NO | NO |
|  |  |  | 0.5ns | 5ns | NO | NO |
| [5] (Multi-RTT) | FR1 | Off at gNB Off at UE | 0 ns | 0ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | NO | YES |
|  |  |  | 1ns | 0.5ns | NO | YES |
|  |  |  | 2ns | 0.5ns | NO | YES |
|  |  |  | 3ns | 0.5ns | NO | YES |
|  |  |  | 5ns | 0.5ns | NO | YES |
|  |  |  | 0.5ns | 1ns | NO | YES |
|  |  |  | 0.5ns | 2ns | NO | YES |
|  |  |  | 0.5ns | 3ns | NO | NO |
| [17] (DL-TDOA) | FR2 | Off at gNB Off at UE | 0.0ns | 0.0ns | YES | YES |
|  |  |  | 0.1ns | 0.1ns | YES | YES |
|  |  |  | 0.2ns | 0.2ns | YES | YES |
|  |  |  | 0.5ns | 0.5ns | No | No |
|  |  |  | 1.0ns | 1.0ns | No | No |
|  |  |  | 2.0ns | 2.0ns | No | No |

Table B.1-3: Summary of evaluated gNB/UE TX/RX timing error parameters and achieved horizontal positioning accuracy in IOO scenario for Rel.16 positioning method.

| Company name (Positioning method) | FR1 / FR2 | gNB/UE TX/RX timing error mitigation is on/off | Evaluated UE TX/RX timing error values (Y value) | Evaluated gNB TX/RX timing error values (X value) | Is horizontal positioning accuracy 0.2m @ 90% met? | Is horizontal positioning accuracy 0.5m @ 90% met? |
| --- | --- | --- | --- | --- | --- | --- |
| [8] (DL-TDOA) | FR1 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (UL-TDOA) | FR1 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (Multi-RTT) | FR1 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (DL-TDOA) | FR2 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (UL-TDOA) | FR2 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |
| [8] (Multi-RTT) | FR2 | Off at gNB Off at UE | 1.5 ns | 0.5 ns | NO | NO |

Table B.1-4: Summary of evaluated gNB/UE TX/RX timing error parameters and achieved horizontal positioning accuracy in UMi scenario for Rel.16 positioning method.

| Company name (Positioning method) | FR1 / FR2 | gNB/UE TX/RX timing error mitigation is on/off | Evaluated UE TX/RX timing error values (Y value) | Evaluated gNB TX/RX timing error values (X value) | Is horizontal positioning accuracy 1m @ 90% met? |
| --- | --- | --- | --- | --- | --- |
| [17], UMI with Δτ (RTT) | FR1 | Off at gNB Off at UE | 1 ns | 1 ns | No |
| [17], UMI with Δτ (RTT) | FR1 | Off at gNB Off at UE | 2 ns | 2 ns | No |
| [17], UMI with Δτ (RTT) | FR1 | Off at gNB Off at UE | 5 ns | 5 ns | No |
| [17], UMI with Δτ (RTT) | FR1 | Off at gNB Off at UE | 10 ns | 10 ns | No |
| [17], UMI without Δτ (RTT) | FR1 | Off at gNB Off at UE | 2 ns | 2 ns | Yes |
| [17], UMI without Δτ (RTT) | FR1 | Off at gNB Off at UE | 5 ns | 5 ns | No |
| [17], UMI without Δτ (RTT) | FR1 | Off at gNB Off at UE | 10 ns | 10 ns | No |

B.2 Evaluation of physical layer latency

Table B.2-1: physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-Assisted NR positioning

| Source Reference to Tdoc # | Physical layer latency for DL-TDOA/DL-AOD, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| [17] | [57-823] | Major Assumption: Connected state, FR1, (N,T) = (6,8) PRS capability Major components: Location Request reception, MG request & configuration, PRS/MG Alignment, PRS processing capabilities |
| [4] | FR1: 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) | Major assumptions: PRS periodicity is 20ms MG is requested Major components PRS measurement |
| [4] | FR1: 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) | Major assumptions: PRS periodicity is 160ms MG is not requested (sharing with existing RRM gap 6ms/40ms) Major components PRS measurement |
| [7] | FR1:106.23 ms FR2: 667.87 ms | Major assumptions: RRC Connected;4 samples;CSSF=1;Measurement Gap Repetition Period is 20ms. Major components: Measurement gap request procedures UE positioning measurement |
| [5] | FR1: [64-11556] FR2： [728-328996] | Major assumptions and components: For FR1: DL measurement &process delay=LCMTPRS, T measGap∪ TProcess time, PRS and MG is periodicity The minimum value is 22ms for LCMTPRS, T measGap=20ms，(N,T) = (6,8) The maximum value is 11514 ms for LCMTPRS, T measGap=10240ms，(N,T) = (6,1280) For FR2: DL measurement &process delay=LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time , NRxBeam=8 , NPosOccasion=4 The minimum value is 20*4*8+2ms =642ms The maximum value is (10240×8×4+1280-6)=328954ms MG request and configuration Location Request and report |
| [11] | FR1: [38-235.6] FR2: [35-229.6] | Major Assumptions: Start and End States: RRC_CONNECTED, MG configuration enabled, MGRP = 20ms-160ms, 1 DL PRS occasion, T=8-160 ms DL PRS processing time. Major Components: Request Location reception and processing, MG request & configuration, DL PRS Measurement and Processing, Provide Location transmission and processing. |
| [11] | FR1: [17-5147.8] FR2: [15.5-5144.8] | Major Assumptions: Start and End States: RRC_CONNECTED, Without MG configuration, DL PRS periodicity =4-5120ms, 1 DL PRS occasion, T=8ms DL PRS processing time. Major Components: Request Location reception and processing, DL PRS Measurement and Processing, Provide Location transmission and processing. |
| [15] | For UE capability-1: 62.97 ms ~ 297.11ms For UE capability-2: 61.17 ms ~ 293.68 ms | Major assumptions: -For PUSCH transmission: - Uplink switching gap is not configured. - No BWP switching - No overlapping symbols of the PUCCH and the scheduled PUSCH - # of PUSCH symbols = from 4 to 14 for Type A - # of PUSCH symbols = from 1 to 14 for Type B -For PDSCH transmission: - No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH - # of PDSCH symbols = from 3 to 14 for Type A - # of PDSCH symbols = from 2 to 14 for Type B Major components -RRC processing time for LPP message at both gNB and UE (LPP request location information message, measurement gap request message, LPP provide location information message) -PRS measurement (LCM of PRS resource periodicity and repetition periodicity of the measurement gap) -If the latency components related with higher layer are excluded, the physical layer latency is described as follows: - For UE capability-1: 23.97ms ~ 249.11ms - For UE capability-2: 22.17ms ~ 245.68ms |
| [8] | FR1: 51.5ms | Major Assumptions: Case 1, 15kHz, FR1, DL-TDOA Source UE/Destination NW Positioning technique DL-TDOA, type DL, mode UE-assisted, Initial and Final RRC States CONNECTED. Major Components: require measurement gap, measurement gap configuration, the delay between the time when DL PRS is received and the time when measurement gap configuration is received, the time from UE begins to measure PRS until the measurement result is ready to report, measurement reporting. |
| [13] | FR1: [44.35 – 10500] ms FR2: [35.08 – 2118.93 ms] | Major Assumptions: 15 kHz SCS for FR1 120 kHz SCS for FR2 Source NW/ Destination NW. UE-assisted. Including MG configuration. Major components: DL PRS periodicity DL PRS processing time SR related steps |
| [12] | FR1: 54.125ms for 60KHz FR2: 52.56ms for 120KHz | Major Assumptions: 60KHz for FR1 and 120KHz for FR2 Major components: Process Location Request reception, MG request & configuration, PRS measurement and processing PUSCH carrying measurement report |
| [16] | FR1: 33ms | Major assumptions: 30kHz SCS Initial and final state: RRC_CONNECTED. The UE is configured with MG of 1.5ms, receives the PRS within the MG to conduct positioning measurement. The UE uses a configured grant having periodicity of 1ms to report the measurement. Best case scenario Major components: Decoding the LPP request location by the UE Decoding the MG request by the gNB Receiving the MG configuration and apply the configuration. |
| [10] | FR1: 129.07 ms | Major assumptions: 30kHz SCS / FDD Initial and final state: RRC_CONNECTED. DL PRS: 18 resources / 4 symbols per resource / 12 Comb-6 symbols per period. Periodicity – 20 ms. UE DL PRS processing capability – N = 0.5 ms (~12 symbols @30kHz), T = 8 ms Dynamic DL/UL scheduling based on SR – based on URLLC assumptions [3GPP 38.824, v16.0.0] Measurement gap: MGL = 5.5 ms, MGRP = 20ms DL PRS processing Nsample = 4 (RAN4 core measurements requirements) UE is expected to perform measurements on DL PRS resource 4 times (i.e. across 4 periods) Higher layer latency components (RRC/LPP processing) are included into the physical layer analysis Major components: MG configuration and alignment time DL PRS processing time and report delay Multiple DL/UL transactions for location request, assistance information, measurement gap request and configuration and associated UE/gNB higher layer processing delays (RRC/LPP) Summary: 4.5714 (L1 components) + 36 (L2/L3 components) + 88.5 (DL PRS processing) = 129.07 ms (total) |

Table B.2-2: physical layer latency for Rel.16 UL-TDOA/DL-AOA UE-Assisted NR positioning

| Source Reference to Tdoc # | Physical layer latency for UL-TDOA/UL-AOA, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| [4] | FR1: 6.5-26ms (1 samp.) 66.5-86.5ms (4 samp) | Major assumptions: SRS periodicity is 20ms Major components SRS measurement |
| [5] 1 | FR1: 30.5-2570.5 FR2: 650.5-10250.5 | Major assumptions: FR1:SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560}slots - 15kHz 1ms-2560ms - 30kHz 0.5ms-1280ms - 60kHz 0.25ms-640ms - 120kHz 0.125ms-320ms FR2: Multiple positioning occasion (4) and beam sweeping (8) - UL measurement equals to the periodicity of SRS×NRxBeam×NPosOccasion gNB processing delay is assumed as zero； The minimum periodicity of SRS is 20ms and the same as the DL minimum periodicity. Major components: SRS measurement; NRPPa process time |
| [5] 2 | FR1: 11-43 | Major assumptions: SRS is aperiodic and the slot offset of aperiodic is 0-32 slots - 15kHz 0ms-32ms - 30kHz 0ms-16ms - 60kHz 0ms-8ms 120kHz 0ms-4ms Major components: SRS measurement; NRPPa process time; Activation; |
| [15] | For UE capability-1: 14.78ms ~ 20.14ms For UE capability-2: 14.42ms ~ 282.97ms | Major assumptions: -For SRS transmission: One shot transmission (2 OS ~ 12 OS) -For PDSCH transmission: - No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH - # of PDSCH symbols = from 3 to 14 for Type A - # of PDSCH symbols = from 2 to 14 for Type B Major components -RRC processing time for LPP message at both gNB and UE (SRS configuration, SRS activation message) -When the latency related with higher layer is excluded, physical layer latency is described as follows: - For UE capability-1: 0.78ms ~ 2.64ms - For UE capability-2: 0.42ms ~ 2.07ms |
| [8] | FR1: 5ms | Major Assumptions: Case 2, 15kHz, FR1, UL-TDOA Source UE/Destination NW Positioning technique UL-TDOA, type UL, mode UE-assisted, Initial and Final RRC States CONNECTED Major Components: the time to activate the SRS transmission, the delay from effective time of SRS activation until UE begins to transmit SRS, the time from gNB begins to measure SRS until the measurement result is ready. |
| [13] | FR1: [2.78 – 81928.5] ms | Major Assumptions: 15 kHz SCS Source NW/ Destination NW. Excluding SRS-Pos RRC configuration Major Components: - SRS-Pos periodicity - Processing of SRS-Pos at gNB/RP-only |
| [13] | FR1: [2.78 – 81928.5] ms | Major Assumptions: 15 kHz SCS Source NW/ Destination NW. Excluding SRS-Pos RRC configuration Major Components: SRS-Pos periodicity Processing of SRS-Pos at gNB/RP-only |
| [13] | FR1: [2.35 – 81925] ms | Major Assumptions: 15 kHz SCS Source NW/ Destination NW. Excluding SRS-Pos RRC configuration Major Components: - SRS-Pos periodicity - Processing of SRS-Pos at gNB/RP-only |
| [12] | FR1: 23.25 ms for 60kHz FR2: 23.125ms for 120kHz | Major Assumptions: 60KHz for FR1 and 120KHz for FR2. Major Components: - gNB process NPPa measurement request - Configure SRS - SRS-Pos periodicity - gNB processing SRS |
| [16] | FR1: 12ms | Major assumptions: - Initial and final state: RRC_CONNECTED. - SRS transmission resources occur immediately after decoding the SRS configuration. - 30kHz SCS - Best case scenario Major components: - Decoding the SRS configuration message. |
| [10] | FR1: 18.77 ms | Major assumptions: - 30kHz SCS / FDD - Initial and final state: RRC_CONNECTED. - Dynamic DL/UL scheduling based on SR – see URLLC assumptions [3GPP 38.824, v16.0.0] - PUSCH: Any symbol, subject to slot boundary constraint (i.e. transmission does not cross slot boundary); Duration – 2, 4, 7 symbols (Type B mapping w/ front loaded DMRS) - PUCCH: 7 occasions per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0] for SR and HARQ feedback, Duration – 1 symbol. - No HARQ – initial transmission is successful - SRS for positioning: Single resource, 1 symbol duration, Periodicity – each slot - Higher layer latency components (RRC/LPP processing) are included into the physical layer latency analysis Major components: - SRS for positioning configuration - UE/gNB higher layer processing delays (RRC/LPP processing) Summary = 2.7678 (L1 components) + 16 (L2/L3 components) = 18.7678 ms (total) |

Table B.2-3: physical layer latency for Rel.16 UE-Assisted Multi-RTT Positioning

| Source Reference to Tdoc # | Physical layer latency for Multi-RTT, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| [17] | [59-823] | Major assumptions: Connected state, FR1, (N,T) = (6,8) PRS capability Major components: Location Request Reception, MG Request & Configuration, PRS/MG Alignment, PRS processing capabilities |
| [4] | FR1: 51.5-66ms (1 samp.) 111.5-126.5ms (4 samp. CSSF = 1) 171.5-186ms (4 samp. CSSF = 2) | Major assumptions: PRS periodicity is 20ms MG is requested Major components PRS measurement |
| [4] | FR1: 171.5-178.5ms (1 samp.) 651.5-658.5ms (4 samp. CSSF = 1) | Major assumptions: PRS periodicity is 160ms MG is not requested (sharing with existing RRM gap 6ms/40ms) Major components PRS measurement |
| [5] | FR1: [94.5-14126.5] +TAlign_DL_UL | Major assumptions and components: For FR1: DL measurement &process delay =LCMTPRS, T measGap∪ TProcess time, PRS and MG is periodicity the minimum value is 22ms for LCMTPRS, T measGap=20ms，(N,T) = (6,8) the maximum value is 11514 ms for LCMTPRS, T measGap=10240ms，(N,T) = (6,1280) FR1:SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560}slots 15kHz 1ms-2560ms 30kHz 0.5ms-1280ms 60kHz 0.25ms-640ms 120kHz 0.125ms-320ms TAlign_DL_UL: The alignment delay is the gap between End trigger of DL positioning and Start trigger of UL positioning. MG request and configuration Location Request and report |
| [15] | For UE capability-1: 77.75 ms ~ 314.75 ms For UE capability-2: 75.59 ms ~ 311.75 ms | Major assumptions: -For PUSCH transmission: - Uplink switching gap is not configured. - No BWP switching - No overlapping symbols of the PUCCH and the scheduled PUSCH - # of PUSCH symbols = from 4 to 14 for Type A - # of PUSCH symbols = from 1 to 14 for Type B -For PDSCH transmission: - No overlapping symbols of the scheduling PDCCH and the scheduled PDSCH - # of PDSCH symbols = from 3 to 14 for Type A - # of PDSCH symbols = from 2 to 14 for Type B -For SRS transmission:One shot transmission (2 OS ~ 12 OS) Major components -RRC processing time for LPP message at both gNB and UE (SRS configuration, SRS activation message, LPP request location information message, measurement gap request message, LPP provide location information message) -PRS measurement (LCM of PRS resource periodicity and repetition periodicity of the measurement gap) -When the latency related with higher layer is excluded, physical layer latency is described as follows: - For UE capability-1: 24.75 ms ~ 251.75ms - For UE capability-2: 22.59ms ~ 248.75ms |
| [16] | FR1: 45ms | Major assumptions: Initial and final state: RRC_CONNECTED. The UE is configured with MG of 1.5ms, receives the PRS within the MG to conduct positioning measurement. The UE uses a configured grant having periodicity of 1ms to report the measurement. SRS transmission resources occur immediately after decoding the SRS configuration. 30kHz SCS Best case scenario Major components: Decoding the LPP request location by the UE Decoding the MG request by the gNB Receiving the MG configuration and apply the configuration. Receiving PRS in the MG Decoding the SRS configuration message. |
| [10] | 140.84 ms | Major assumptions: 30kHz SCS / FDD Initial and final state: RRC_CONNECTED. DL PRS: 18 resources / 4 symbols per resource / 12 Comb-6 symbols per period. Periodicity – 20 ms. UE DL PRS processing capability – N = 0.5 ms (~12 symbols @30kHz), T = 8 ms Dynamic DL/UL scheduling based on SR – based on URLLC assumptions [3GPP 38.824, v16.0.0] Measurement gap: MGL = 5.5 ms, MGRP = 20ms DL PRS processing Nsample = 4 (RAN4 core measurements requirements) UE is expected to perform measurements on DL PRS resource 4 times (i.e. across 4 periods) PUSCH: Any symbol, subject to slot boundary constraint (i.e. transmission does not cross slot boundary); Duration – 2, 4, 7 symbols (Type B mapping w/ front loaded DMRS) PUCCH: 7 occasions per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0] for SR and HARQ feedback, Duration – 1 symbol. No HARQ – initial transmission is successful SRS for positioning: Single resource, 1 symbol duration, Periodicity – each slot Higher layer latency components (RRC/LPP processing) are included into the physical layer latency analysis Major components: MG configuration and alignment time DL PRS processing time and report delay Multiple DL/UL transactions and associated UE/gNB RRC/LPP processing delays Summary: 7.3393 (L1 components) + 45 (L2/L3 components) + 88.5 (DL PRS processing) = 140.8393 (total) |

Table B.2-4: physical layer latency for Rel.16 UE-Assisted E-CID Positioning

| Source Reference to Tdoc # | Physical layer latency for ECID, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| [4] | FR1 8.5-15ms | Major assumptions: DL E-CID RRM measurement available Major components Higher layer signaling processing |
| [4] | FR1 6-26ms | Major assumptions: UL E-CID RRM measurement available Major components Higher layer signaling processing, or Additional AoA measurement at gNB |
| [7] | FR1 10.30 ms | Major assumptions: DL E-CID RRM measurement is available at UE side. Major components: UE interprets and applies the measurement configuration |
| [15] | For UE capability-1: 28.41 ms ~116.55 ms For UE capability-2: 27.33 ms ~ 115.05 ms | Major assumptions: -For PUSCH transmission: - Uplink switching gap is not configured. - No BWP switching - No overlapping symbols of the PUCCH and the scheduled PUSCH - # of PUSCH symbols = from 4 to 14 for Type A - # of PUSCH symbols = from 1 to 14 for Type B Major components - RRC processing time for LPP message at both gNB and UE (LPP provide location information message) -When the latency related with higher layer is excluded, physical layer latency is described as follows: - For UE capability-1: 2.41ms ~ 85..55ms (FR1) - For UE capability-2: 1.33ms ~ 84.05ms (FR1) |

Table B.2-5: physical layer latency for Rel.16 UE-Based DL Only Positioning

| Source Reference to Tdoc # | Physical layer latency for UE-based DL only positioning, ms | Comments on major assumptions and physical layer latency components |
| --- | --- | --- |
| Source #1: | FR1: FR2: | Major assumptions: Major components: |
| [17]1 | [46-811] | Major assumptions: Start from RRC Connected, FR1, (N,T)=(6,8), External client Major components: Location Request reception, MG request & configuration, MG/PRS alignment, PRS processing capabilities |
| [17]2 | [8-780] | Major assumptions: Start from RRC Inactive, FR1, (N,T)=(6,8) , Internal client Major components: PRS alignment time, PRS processing capabilities |
| [4] | FR1 51-58.5ms (1 samp.) | Major assumptions: PRS periodicity is 20ms MG is requested MO-LR Major components PRS measurement |
| [5] 1 | FR1: [66-11558] FR2： [730-328998] | Major assumptions and components: For FR1: DL measurement &process delay=LCMTPRS, T measGap∪ TProcess time, PRS and MG is periodicity MG request and configuration Calculation of Location Estimate at the UE Location Request and report MT-LR |
| [5] 2 | FR1: [55.5-11547.5] FR2： [719.5-328987.5] | Major assumptions and components: For FR1: DL measurement &process delay=LCMTPRS, T measGap∪ TProcess time, PRS and MG is periodicity MG request and configuration Location Request Calculation of Location Estimate at the UE MO-LR |
| [11] | FR1: [29-207.8] FR2: [27.5 -204.8] | Major Assumptions: Start and End States: RRC_CONNECTED, MGRP = 20ms-160ms, 1 DL PRS occasion, T=8ms-160ms PRS processing time, Request and provide location information messages omitted. Major Components: MG request & configuration, DL PRS Measurement and Processing. |
| [11] | [8-5120] | Major Assumptions: Start and End States: RRC_CONNECTED, Without MG configuration, DL PRS periodicity=4-5120ms, 1 DL PRS occasion, T=8ms DL PRS processing time, Request and provide location information messages omitted. Major Components: DL PRS Measurement and Processing. |
| [12] | 44ms | Major Assumption: - Start time: UE sends MG request - End time: UE finish location calculation Major component: - MG request and configuration - Measurement gap periodicity - UE calculating location |
| [16] | FR1 : [39-61] ms for Alt. 1, [50-72] ms for Alt. 2, [22-44] ms for Alt. 3, where different alternatives correspond to different starting points for latency evaluation of UE-B positioning | Major assumptions: - 30kHz SCS - Initial and final state: RRC_CONNECTED. - The UE is configured with MG of 1.5ms, receives the PRS within the MG to conduct positioning measurement. (for Alt 1 & 2) - The UE uses a configured grant having periodicity of 1ms to report the measurement. (for Alt 1 & 2 & 3) - Best case scenario Major components: - Decoding the LPP request location by the UE (for Alt 2) - Decoding the MG request by the gNB (for Alt 1 & 2) - Receiving the MG configuration and apply the configuration. (for Alt 1 & 2) - UE calculating location (for Alt 1 & 2 & 3) |

Annex C:	Appendix to Performance evaluations for R17 performance targets

C.1	Performance analysis of Rel-16 positioning solutions

See separate word file.

C.2	Performance of studied NR positioning enhancements

See separate word file.

C.3	Efficiency analysis for NR positioning enhancements

See separate word file.