3GPP TSG RAN WG1 #104b-e		      R1-2103996

e-Meeting, April 12th – 20th, 2021

Source:	Moderator (CATT)

Title:	Moderator summary #3 on M-TRP simultaneous transmission with multiple Rx panels

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

Background

This is FL summary #3 of Rel.17 MIMO AI 8.1.2.3.

Company inputs in the previous two rounds of discussion are documented in [1] and [2].

# Discussion

Beam measurement/reporting

Issue 1: CMR configuration

Proposal: On CMR resource configuration for beam reporting option 2 , decide in RAN1#105b-e whether to adopt “set” or “subset”

- NOTE: the following has been agreed

  - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

    - FFS: extension to aperiodic CMR resource setting

  - Each reported beam pair in a single CSI-report consists of M = 2 SSBRI/CRI values, where each SSBRI/CRI points to a CMR resource in a different CMR resource set or subset.

- FFS: bitwidth of each CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

| Company | Views |
| --- | --- |
| Moderator Summary | Observation from last round: - Support “set” (9): Qualcomm, ZTE, Apple, Lenovo/MoM, CMCC, Huawei, HiSilicon, Futurewei, - Support “subset” (8): vivo, DOCOMO, LGE, CATT, TCL, Nokia/NSB, Ericsson |
| ZTE | It seems that we need to make decision this meeting,regarding ‘set’ and ‘subset’. So, we suggest to be decided in the next GTW based on current situation. At least, the last bullet ’ FFS: bitwidth of each CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets’ should be added in Chariman notes. |
| Nokia/NSB | We think more discussion is required to make conclusion. We prefer to postpone the decision by RAN1 #105-e. |
| Mod | Tend to agree with Nokia’s view. Proposed to postpone decision to RAN1#105-e. |
| Mod | Updated company list |

Issue 2: UE panel/antenna related feedback

Proposal: For potential UE panel related information feedback for beam reporting option 2, further study the following alternatives:

- Alt-1: UE reports panel ID / antenna-group ID or the reporting setting is associated with panel ID/antenna-group ID

  - the reporting setting is associated with panel ID/ antenna-group ID

- Alt-2: UE indicates if reported beams are associated to different RX spatial filters, or maximum number of supported layers corresponding to DL RS in a group, or whether two beams in a beam pair can be used for spatial multiplexing or diversity

- Other alternatives are not precluded

- NOTE: study of these alternatives with beam reporting option 1/3 is not precluded

| Company | Technical views |
| --- | --- |
| Moderator Summary | Observation from last round. There is no consensus. Alt-1: - Support (8): vivo, Lenovo/MotM, AT&T, Huawei, HiSilicon, Futurewei, LGE - Concern (7): OPPO, DOCOMO, MediaTek, Nokia/NSB, Qualcomm, Ericsson Alt-2: - Support (17): CMCC, vivo, ZTE, MediaTek, Lenovo/MotM, Xiaomi, Apple; Samsung, AT&T, Huawei, HiSilicon, Futurewei, Qualcomm, DOCOMO, LGE, Ericsson - Concern (3): OPPO, Nokia/NSB |
| Xiaomi | Remove our name from conern of Alt-1. |
| ZTE | Suggest to agree above FFS and make decision next meeting. |
| Nokia/NSB | The gain of each alternative for option 2 should be clarified. We think Alt-1 and Alt-2 are more related to option 1 or option 3 which are not agreed yet. For Option 2, there is no clear gain from the proposed operations. |
| Mod | Added a note by Nokia/NSB’s suggestion. |
| Mod | Updated company list |

Issue 3: L1-SINR and interference measurement

Proposal 2.4: For beam reporting option 2, evaluate the performance, specification, and implementation aspects of L1-SINR based beam measurement/feedback, including at least the following aspects

- Physical resource for interference measurement, e.g.

  - CMR of the other reported beam within the beam pair,  and/or

  - CMR of the reported beam, and/or

  - Dedicated IMR resource

- UE behavior of interference measurement

- Note: L1-RSRP report has been agreed for option 2

| Company | Technical views |
| --- | --- |
| Moderator | Observation from last round: There is no consensus. - Support (13): DOCOMO, ZTE, Lenovo&MotM, LGE, CMCC, ETRI, AT&T, Huawei, HiSilicon, Futurewei, Qualcomm, Ericsson - Concern (5): vivo, Apple, OPPO, Nokia/NSB (except dedicated IMR) |

Issue 4: Value of N

The following has been agreed and a down-selection is due in RAN1#105-e.

Agreement: For beam reporting option 2

- On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e:

  - Alt1: Support maximum value N = {1, 2}

  - Alt2: Support maximum value N = {1, 2, 3, 4}

- FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17

- On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e

  - Alt1: The value of N is fixed by RRC configuration

  - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE

| Company | Technical views |
| --- | --- |
| Moderator Summary | Observation from last round: There is no consensus at the moment. Moderator recommends to follow the RAN1#104b-e agreement and down select in RAN1#105-e. Q1: - Alt-1: OPPO, Xiaomi, Apple, OPPO, - Alt-2: DOCOMO, CATT, ZTE, MediaTek, CMCC, TCL, Nokia/NSB, Samsung, Huawei, HiSilicon, Futurewei, Qualcomm, Ericsson, Lenovo/MotM Q2: - Alt-1: CATT, DOCOMO, OPPO, MediaTek, Xiaomi, Apple (with a researched SSBRI/CRI value to indicate no beam identified), LGE, CMCC, Huawei, HiSilicon, Futurewei, Qualcomm, Lenovo/MotM - Alt-2: CATT, ZTE, MediaTek, OPPO, TCL, Nokia/NSB, Samsung, Ericsson |
| Lenovo&MotM | Q1: support Alt 2. Q2 : support Alt 1. |

M-TRP Beam failure recovery

Issue 0: Simultaneous configuration of cell-specific and TRP-specifc BFR on the same cell

Proposal:

- FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

| Company | Technical views |
| --- | --- |
| Moderator summary | Observation from last round: - There is no consensus. - Some companies (e.g. Xiaomi, Lenovo/MotM, NEC, LGE, APT/FGI, Apple, Samsung, AT&T, Nokia/NSB, MediaTek, Ericsson, Xiaomi) think this is possible, while some (OPPO, Convida, MediaTek, Huawei, HiSilicon, Futurewei, Qualcomm, DOCOMO, ZTE) disagree. - Some companies suggested that SpCell and SCell may be discussed separately. |
| Qualcomm | We support not to simultaneously configure both on same CC to simplify the design/complexity. Otherwise, UE may have to monitor BFD RS for both, and new rules are needed for the interaction between the two, e.g. if both per-TRP and cell level BFR are triggred, do we need to report both or report only one? We are also open to discuss the design for simultaneous configuration of both before certain deadline. If no consensus, then we should agree no simultaneous configuration. |
| Xiaomi | We think if Multi-TRP PDCCH repetition is supported, both TPRs fail doesn’t mean the cell fails. In this case, it is reasonable to configure both cell-specific and TRP-specific BFR in the same CC. |
| Sony | We think either cell-specific BFR or TRP-specific BFR mechanism is workable. At current stage, we see no strong reason to simultaneously configure both on the same CC, no matter the CC belongs to SpCell or SCell. On the contrary, if both BFR mechanisms supported, we have to handle the interaction or possibly collision. But we are fine to have a FFS on this issue. |
| DOCOMO | I think our comment was that, sometimes RACH (cell-specific BFR) can be performed as fallback scheme for per-TRP BFR on SpCell, but it does not mean we support ‘configured in the same CC’. We do not think cell-specific BFD-RS and per-TRP BFR-RS sets need to be configured in the same CC. Only one type of BFD-RS can be configured. Hence, we do not support this proposal. |
| ZTE | FFS seems to be needed, and let’s make further discussion next meeting. |
| LGE | Support simultaneous configuration for SpCell. |
| Lenovo&MotM | Support simultaneous configuration for SpCell. When both TRPs are failed, it can fallback to RACH based BFRQ. But for SCell, since the PUCCH-SR is configured in SpCell or PUCCH-SCell, it’s no need to support simultaneous configuration. Therefore, we support to discuss this issue for SpCell and SCell separately. |
| Nokia/NSB | Support simultaneous configuration for SpCell, which is up to NW. |
| MediaTek | Support simultaneous configuration ONLY for SpCell |

Issue 1: S-DCI vs. M-DCI

Proposal:

- Support S-DCI and M-DCI in TRP-specific BFR in Rel.17

| Company | Technical views |
| --- | --- |
| Moderator summary | Observation: There is no consensus. Individual company view appears stable. - Support (23): Huawei, HiSilicon, CATT, vivo, CMCC, Intel, Samsung, Nokia/NSB, AT&T, Ericsson, Spreadtrum, Qualcomm, Futurewei, APT/FGI, Convida, Xiaomi, NEC, Sony, DOCOMO, TCL, ETRI - Concern on S-DCI (3): OPPO, MediaTek, Apple - Prioritize S-DCI, postpone M-DCI: ZTE, Levono/MotM, Qualcomm |
| Qualcomm | We support both sDCI and mDCI. They are identical in terms of use case. Yes, we agree a TRP ID may be needed for sDCI to work. Perhaps we can prioritize mDCI first, then sDCI if time allows? |
| Xiaomi | We support both S-DCI and M-DCI. Since even in S-DCI, it is possible that each TRP is configured with CORESET. Any TRP configured with CORESET is necessary to be configured with TRP specific BFR. |
| Sony | Support both S-DCI and M-DCI as a unified solution. We sympathize what FL mentioned during online session that even for S-DCI, NW could dynamically choose which TRP to send the DCI scheduling DL transmission from two TRPs. From this sense, the DL control channel from each TRP should be monitored by UE. |
| Apple | We would like to clarify whether sDCI/mDCI covers PDCCH repetitions or SFN approach that is under discussion in 8.1.2.1. In addition, it should be noted that BFR for sTRP should not be considered in this AI. However, currently sTRP and sDCI is dynamically switched by MAC CE. Does it mean the whole BFR procedure for sDCI should be dynamically turned on/off? We also want to understand the benefit for TRP-specific BFR for sDCI. We noticed some commentes are like both sDCI and mDCI are equaly important. But we also noticed some feature, e.g. joint/separate HARQ report is only supported for mDCI. |
| ZTE | We share the same views with Qualcomm that a TRP ID is needed for sDCI to work. Based on current situation, we suggest to treat M-DCI firstly, and postpone S-DCI until M-DCI solution is stable. |
| LGE | No strong preference on this proposal, but we are curious about the point that S-DCI is related to from spec perspective. To our understaning, S-DCI MTRP is only related to PDSCH MTRP transmission but hypothetical BLER is about PDCCH beam quality. For explicit BFD, no difference between S-DCI and M-DCI from spec perspective. For implicit BFD, we could introduce two CORESET groups when single CORESET pool is configured in the BWP/CC as FL initially proposed for issue 4, but these CORESET groups is configured by gNB and there is no relevance to S-DCI MTRP to our understanding. So, it would be better to focus on whether we support M-TRP BFR framework to be applicable for single CORESET pool or not, rather than S-DCI vs M-DCI. So, we’d like to suggest to modify the proposal as below, which is clearer to us. Proposal: Support TRP-specific BFR in the case of single CORESET pool as well as two CORESET pools in a BWP/CC. |
| Ericsson | Support the proposal |
| Lenovo&MotM | We share the same view of ZTE which is to treat M-DCI firstly and postpone S-DCI. |

Issue 3: 1-to-1 association between BFD-RS and NBI-RS set

Proposal:

- On the 1-to-1 association between BFD-RS sets and NBI-RS sets when two NBI-RS sets are configured, down-selecte from the following two alternatives in RAN1#105-e

  - Alt-1: First BFD-RS set associated with first NBI-RS set, and second to the second(NOTE: how to capture this can be up to RAN2)

  - Alt-2: RRC configurable association between BFD-RS se k (k=0, 1) and NBI-RS set j (j=0, 1). (NOTE: how to capture this can be up to RAN2)

Revised Proposal:

- On the 1-to-1 association between BFD-RS sets and NBI-RS sets, support the following association

  - Alt-1: First BFD-RS set associated with first NBI-RS set, and second to the second (NOTE: how to capture this can be up to RAN2)

| Company | Technical views |
| --- | --- |
| Moderator summary | Observation from last round: - Alt1 (16): CATT, DOCOMO, Convida, MediaTek, Lenovo&MotM, Xiaomi, Apple, NEC, APT/FGI, TCL, Ericsson, Futurewei, Huawei, HiSilicon (both acceptable) - Alt2 (3): ZTE, Huawei, HiSilicon (both acceptable) - Can be left to RAN2 (4): OPPO, Nokia/NSB, AT&T, Qualcomm - A revised proposal is provided based on majority view. |
| Qualcomm | Suggest to add the following text to clarify how those sets are finally linked to CORESETPoolIndex for both sides to be in sync on the failed TRP. Otherwise, we are also fine to leave the signaling to RAN2 - On the 1-to-1 association between BFD-RS sets and NBI-RS sets, support the following association - Alt-1: First BFD-RS set associated with first NBI-RS set, and second to the second (NOTE: how to capture this can be up to RAN2) - For mDCI, both first sets map to CORESETPoolIndex 0, and both second sets map to CORESETPoolIndex 1 - FFS: sDCI |
| Xiaomi | Support the revised proposal |
| Sony | Support FL’s proposal. In our view, Alt.-2 RRC configurable association can be somehow implemented in Alt-1 by setting which BFD-RS is the 1st set and correspondingly which NBI-RS set is the 1st set via RRC signaling. |
| vivo | We still would like to add the condition that this is only applicable for the case when two NBI-RS sets are configured. - On the 1-to-1 association between BFD-RS setsand NBI-RS sets when two NBI-RS sets are configured, support the following association - Alt-1: First BFD-RS set associated with first NBI-RS set, and second to the second (NOTE: how to capture this can be up to RAN2) |
| DOCOMO | Support the revised proposal |
| ZTE | This is not our preference, but we can live it for progress. |
| Ericsson | Support revised proposal. |
| Lenovo&MotM | Support. |
| Nokia/NSB | We don’t see any RAN1 aspect on this design. It is up to RAN2. |
| MediaTek | Suppoty the revised proposal |
| Mod | @Nokia/NSB: An association needs to be known so that UE knows which NBI-RS set to search when a BFD-RS set fail. If a fixed association is sufficient, RAN1 can make a conclusion and RAN2 can finalize the signaling structure. |

Issue 4: Explicit/implicit BFD-RS set

Version 1

Proposal 3.3.c: BFD-RS set configuration

- Support both explicit and implicit BFD-RS set configuration in Rel.17 with 1 activated TCI state for each CORESET

- For implicit BFD-RS configuration, down select from the following two alternatives

  - Alt-1: support implicit BFD-RS configuration for M-DCI, where BFD-RS set k is derived from TCI states of CORESETs with the same CORESETPoolIndex

  - Alt-2:  Support implicit BFD-RS configuration, where BFD-RS set k is derived from TCI states of CORESET with the same CORESETGroupIndex

Revised version 2: BFD-RS set configuration

- Support both explicit and implicit BFD-RS set configuration in Rel.17 with 1 activated TCI state for each CORESET.

- For implicit BFD-RS configuration, down select from the following two alternatives

  - Alt-1: support implicit BFD-RS configuration for M-DCI, where BFD-RS set k is derived from TCI states of CORESETs with the same CORESETPoolIndex

  - Alt-2:  Support implicit BFD-RS configuration, where BFD-RS set k is derived from TCI states of CORESET with the same CORESETPoolIndex. Extend configuration of CORESETPoolIndex to S-DCI in Rel.17 when TRP-specific BFR is configured.

Revised version 3: BFD-RS set configuration

- Support both explicit and implicit BFD-RS set configuration in Rel.17 with 1 activated TCI state for each CORESET.

Revised version 4: BFD-RS set configuration

- Support both explicit and implicit BFD-RS set configuration in Rel.17 at least in case of 1 activated TCI state for each CORESET for both M-DCI and S-DCI based M-TRP.

- For implicit BFD-RS configuration for M-DCI and S-DCI M-TRP

  - For M-DCI, the first BFD-RS set is derived from TCI states of CORESETs with the same CORESETPoolIndex  = 0, and the second BFD-RS set is derived from TCI states of CORESETs with CORESETPoolIndex = 1

  - FFS: details for S-DCI

| Company | Technical views |
| --- | --- |
| Moderator summary | Summary from last round: - 2 companies support implicit only (Apple, OPPO). - 30 companies support both implicit and explicit. Some companies think explicit is not strictly necessary but are are OK to accept both (e.g. Intel, CATT). - Some companies support implicit for M-DCI only. A revised proposal (version 4) is provided based on Nokia/NSB’s suggestion. |
| Qualcomm | Suggest the following wording change, since 2-TCI CORESET is still pending Revised version 4: BFD-RS set configuration - Support both explicit and implicit BFD-RS set configuration in Rel.17 with at least in case of 1 activated TCI state for each CORESET. - For implicit BFD-RS configuration for M-DCI and S-DCI M-TRP - For M-DCI, BFD-RS set k is derived from TCI states of CORESETs with the same CORESETPoolIndex - FFS: details for S-DCI |
| Convida Wireless | Support the proposal, but a suggested rewording for clarity, similar to Qualcomms suggestion above: - For M-DCI, the first BFD-RS set is derived from TCI states of CORESETs with CORESETPoolIndex = 0 and the second BFD-RS set is derived from TCI states of CORESETs with CORESETPoolIndex = 1. |
| Moderator | Updated based on Qualcomm and Convida |
| Xiaomi | Support the revised version 4 |
| Sony | Support FL’s updated proposal to handle these two cases (1 active TCI state for CORESET and more than 1 TCI states for CORESET) separately. |
| vivo | Support. |
| DOCOMO | Support |
| Apple | We have to wait for decision for issue 0. |
| ZTE | Support the revised version 4 for M-DCI only. |
| LGE | OK with the recent revision, but we think both Alt 1 and Alt 2 can be supported regardless of whether S-DCI is supported for Rel-17 BFR or not. |
| Ericsson | We do not support S-DCI for implicit configuration. Suggest the following revision: Revised version 4: BFD-RS set configuration - Support both explicit and implicit BFD-RS set configuration in Rel.17 at least in case of 1 activated TCI state for each CORESET for both M-DCi and S-DCI M-TRP. - For implicit BFD-RS configuration for M-DCI M-TRP - For M-DCI, the first BFD-RS set is derived from TCI states of CORESETs with the same CORESETPoolIndex = 0, and the second BFD-RS set is derived from TCI states of CORESETs with CORESETPoolIndex = 1 - FFS: whether implicit BFD-RS configuration is supported for S-DCI MTRP and corresponding details |
| Lenovo&MotM | Our preference is implicitly only, however, we can accept the revised proposal. |
| Nokia/NSB | Support the revised version 4 |
| MediaTek | Support the Revised version 4 |
| Mod | Updated company list. |

Issue 5: Implicit configuration for BFD-RS set with more than 1 activated TCI

Proposal:

- For a CORESET associated with more than 1 activated TCI states

  - For implicit BFD-RS set determination, BFD-RS set associated with this CORESET is based on QCL-typeD source RS of all activated TCI states

  - FFS: BLER determination based on two TCI states, e.g. whether separate BLER are independently derived from each TCI state, or a common BLER is derived from all TCI states, or leave to RAN4

| Company | Technical views |
| --- | --- |
| Moderator summary | Observation from last round: - There is no consensus. - Some companies think the case with two activated TCI states relates to AI 8.1.2.1/8.1.2.4 and should be postponed until the other two AIs are sufficiently clear. |
| Qualcomm | We are fine for the Proposal. |
| Xiaomi | We want to clarify that the BFR-RS set is for cell specific BFR or TRP specific BFR. If both cases are not precluded, it is better to update it as follows. Proposal: - For a CORESET associated with more than 1 activated TCI states - BFD-RS set(s) associated with this CORESET are based on QCL-typeD source RS of all activated TCI states - FFS: BLER determination based on two TCI states, e.g. whether separate BLER are independently derived from each TCI state, or a common BLER is derived from all TCI states, or leave to RAN4 [mod]: My personal understanding of the intention is that a CORESET with 2 activated TCI is still associated with one BFD-RS set, which comprises two QCL-type D source RS. If there is one BFD-RS set it is cell-specific BFR, otherwise if there are more than 1 BFD-RS sets, it’s TRP-specific BFR. |
| Sony | Support in principle. For BFD-RS set determination, we think it’s for implicit BFD-RS determination. If explicit BFD RS set is applied, it’s up to NW to configure. So we would suggestion following change as Proposal: - For a CORESET associated with more than 1 activated TCI states - For implicit BFD-RS set determination, BFD-RS set associated with this CORESET is based on QCL-typeD source RS of all activated TCI states - FFS: BLER determination based on two TCI states, e.g. whether separate BLER are independently derived from each TCI state, or a common BLER is derived from all TCI states, or leave to RAN4 As for BLER determination, we believe how to calculate it depends on the exact transmission scheme of PDCCH. For instance, common BLER or let’s say joint BLER of SFN PDCCH might be more proper to evaluate the PDCCH reliability. |
| Mod | Updated with Sony’s input. |
| LGE | Fine but I think similar discussion is ongoing in AI 8.1.2.4. |
| Ericsson | Support the proposal |
| Nokia/NSB | We can discuss later. |

Issue 7: Whether two NBI-RS sets can overlap

Proposal:

- When two NBI-RS sets are configured , set k and j are disjoint (k, j = 0, 1)

  - This applies to at least SCell. FFS for SpCell (e.g. whether NBI-RS set associated with TRP associated with CORESET #0 may include NBI-RS associated with the other TRP)

| Company | Technical views |
| --- | --- |
| Moderator summary | Observation from last round: - There is no consensus. |
| Qualcomm | We are fine for the proposal |
| Xiaomi | Support the proposal |
| LGE | We think the proposal is not needed. |
| Ericsson | Support the proposal |
| Lenovo&MotM | Not support |
| Nokia/NSB | This is up to NW confiruation. No further discussion is required. |
| MediaTek | Up to NW implementation |

Issue 8: RACH-based fallback

Proposal: CBRA-based transmission can be triggered on a SpCell for per-TRP BFR as least in the following scenarios

- Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell

- FFS: other scenarios

  - Scenario 2: at least one TRP fails on SpCell

  - Scenario 3: at least one pre-defined TRP fails on SpCell

  - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available

  - Scenario 5: If MAC-CE based reporting does not work (details FFS)

  - Scenario 6: When no PUCCH-SR is configured

- NOTE: It is RAN1’s understanding that RAN1 decision does not preclude RAN2 from studying other scenarios.

| Company | Technical views |
| --- | --- |
| Moderator summary | Observation from last round: - Support proposal and scenario 1: DOCOMO, Convida, Lenovo, ZTE, Xiaomi, NEC, LGE, APT/FGI, TCL, ETRI, Samsung, Huawei/HiSilicon, Futurewei, Ericsson. Qualcomm, CATT - Focus on single-TRP failure first: CMCC, vivo - When PUCCH-SR is not configured (added Scenario 6): Apple - No restriction (by UE implementation): OPPO, Nokia / NSB Moderator feels this is not the most urgent issue, so discussion may be postponed. |
| Qualcomm | Our preference is Scenario 1 |
| Xiaomi | Support the proposal |
| DOCOMO | Our preference is Scenario 1 |
| Lenovo&MotM | Support the proposal and our preference is Scenario 1. |

Issue 9: PUCCH-SR

Proposal:

- A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group

  - NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group

- FFS: if a PUCCH-SR resource can be associated with 2 UL spatial filters, and if so, transmission schemes (e.g. multi-TRP PUCCH scheme agreed in AI 8.1.2.1, or selection of UL spatial filter)

- For a UE configured with two PUCCH-SR resources in a cell group

  - When beam failure is detected in one BFD-RS set in a CC, one PUCCH-SR resource is selected for failure event indication. Down select from the following PUCCH-SR resource selection rule in RAN1#104b-e:

    - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS.

    - Alt-2: PUCCH-SR resource associated with failed  BFD-RS set, association details FFS.

    - Alt-3: Leave to UE implementation

  - FFS: When beam failure is detected in two BFD-RS sets in a CC

Revised proposal (version 2):

For a UE configured with two PUCCH -SR resources in a cell group when beam failure is detected in a single/multiple CCs in one or more of BFD-RS sets configured in one or more of CCs ,

- Down select one of the following PUCCH-SR resource selection rules (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e

  - Alt-1: PUCCH -SR resource associated with other/non-failed BFD-RS set, association details FFS .

  - Alt-2: PUCCH -SR resource associated with failed BFD-RS set, association details FFS .

  - Alt-3: Leave it up to UE implementation

Additional proposal 9.b

When two PUCCH-SR resources are configured in a cell-group, discuss on the following two alternatives

- Alt-1:  the two dedicated PUCCH-SR resources are configured in a SR configuration for BFR.

- Alt-2: the two dedicated PUCCH-SR resources correspond to two SR configurations for BFR, and each PUCCH-SR resource is configured in a SR configuration for BFR.

| Company | Technical views |
| --- | --- |
| Moderator summary | Observation from last round: There is an ongoing email discussion. Company positions (for single-TRP failure with 2 configured PUCCH-SR resources) are summarized below. - Alt-1: Huawei, HiSilicon, Qualcomm, Futurewei, ZTE, NEC, Sony, ETRI, DOCOMO - Alt-2: CMCC, CATT, Spreadtrum, Qualcomm, IDCC, vivo, Xiaomi, Fujitsu, Nokia/NSB, - Alt-3: Intel, CATT, Apple, Convida, LGE, Spreadtrum, MediaTek, APT/FGI, TCL Prioriizing single-CC: DOCOMO, vivo, Not priorizing single-CC: LGE, Convida A revised proposal (version 2) is provided for further discussion. |
| Qualcomm | We are fine for the revised proposal. For the conclusion, what is the use case for 1 BFD RS set for per-TRP BFR? Good to clarify |
| Moderator | One possible use case is that TRP2 and TRP1 do not always operate in the same frequency, e.g. TRP2 operates in CC2 only. |
| Xiaomi | We are fine for the revised proposal. For the clarification of conclusion, if TRP 2 operates in CC2 only, it means that there is only TRP1 in CC1. Thus is it necessary to configure TRP specific BFR on CC1? |
| Sony | We are fine with Revised proposal (version 2). As for proposed conclusion, given the example from FL to respond to QC’s question, can we understand the case one BFD-RS set in one CC as cell-specific (single-TRP operation) BFD RS? If so, it seems not so necessary to have a conclusion, and perhaps it’s fine that RAN1 are in the same page on BFD RS set configuration within a cell group. |
| vivo | From the email discussion, we have some comments as following: - Further add a condition that “when the SR is triggered” and we are open to discuss it. - The case of single CC should be prioritized. - We don’t see any difference between the proposed conclusion and the agreement that cell-specific BFR and TRP-specific BFR can be configured in different CCs, so that we think it can be removed. Consider all the above, we revise the proposal as follows: Revised proposal (version 3): For a UE configured with “n” two PUCCH -SR resources in a cell group, (n=1,2), For when beam failure is detected in a single/multiple CCs in one or more of BFD-RS sets configured in one or more of CCs, - Study the “n” number of PUCCH -SR resources (“n”) selected for beam failure event indication, and selection rule (if needed), (n=1,2) - Down select one of the following PUCCH -SR resource selection rules (and their combinations) for the study when the SR is triggered, without precluding other alternatives, in RAN1#105-e - Alt-1: PUCCH -SR resource associated with other/non-failed BFD-RS set, association details FFS. - Alt-2: PUCCH -SR resource associated with failed BFD-RS set, association details FFS. - Alt-3: Leave it up to UE implementation - The above study focuses on the case of single CC first. Proposed conclusion - For TRP-specific BFR, UE can be configured with one BFD-RS set in one CC and two BFD-RS sets in another CC in the same cell group. |
| DOCOMO | We support vivo’s revision. And our preference is Alt.-1, not Alt-2 (we revised Moderator summary). |
| Moderator | The proposed conclusion is based on request from one company. If everyon’s understanding is the same, I think there is no harm clarifying this common understanding. However I am fine to remove it as long as we are on the same page. On prioritizing “single-CC” case, we have heard different views on the email reflector already. Companies are free to raise it on the GTW session. |
| Moderator | Added “additional proposal 9.b” by DOCOMO. |
| LGE | Fine with FL’s revised proposal. For additional proposal, is it RAN1 issue? |
| Lenovo&MotM | Support the FL’s revised proposal. |
| Nokia/NSB | Fine with FL’s revised version 2. Also, al1 and alt2 are equivalent, either way looks fine. For additional proposal, whether to support alt 1 can be discussed, but we don’t support. Alt 2 is aligned with the existing design. |

Issue 10: BFRA MAC-CE content

Proposal:

- A single MAC-CE is used for BFRQ report for all TRPs in all CCs in a cell group

- The MAC-CE carries information of failed TRP identifiers, e.g.

  - Alt-1: indices of BFD-RS set where beam failure is detected,

  - Alt-2: implicitly through new beam index, if found, else explicitly through BFD-RS set index

  - other alternatives are not precluded

- For each failed TRP for a CC, BFRQ carries information whether a new candidate beam is found, and new beam index (if found).

| Company | Technical views |
| --- | --- |
| Moderator | Observation from last round: Majority companies are OK with the proposal, except the first bullet on 1. vs 2 MAC-CE. Majority companies prefer a single MAC-CE. - 1 MAC-CE (22): Apple, vivo, Spreadtrum, MediaTek, Huawei, HiSilicon, Qualcomm, Futurewei, APT/FGI, Convida, Xiaomi, Sony, DOCOMO, Fujitsu, LGE, TCL, ETRI, CATT, Nokia/NSB (details up to RAN2) - 2 MAC-CE (2): ZTE, NEC - Delete the 1st bullet: OPPO |
| Qualcomm | We prefer 1 MAC-CE. The MAC-CE ID is also limited resource |
| Convida Wireless | Support the proposal, but suggested editorial update: - The MAC-CE carries information of failed TRP identitiesfiers, e.g. In the 3rd bullet, the terminology used is “new beam index”. I suggest to use the same term in Alt-2, i.e.: Alt-2: implicitly through resource indicator that corresponds to the identified new beam index |
| Moderator | The latest update was based on inputs from Huawei/HiSilicon. They look quite similar to me. Aligning them is fine. @CW/HW: would you have a strong opinion on any of them? |
| Xiaomi | Support the proposal |
| vivo | For the first bullet, we don’t think Alt-2 is a good solution to indicate the failed TRP, especially when no new beam satisfying the requirement has been found and reported. Therefore, we prefer to revise Alt-2 as follows: - Alt-2: implicitly through resource indicator that corresponds to the identified new beam index if found when two NBI-RS sets are configured. For the third bullet, we would like to add the condition that this is only applicable for the case when the NBI-RS set corresponding to the failed TRP is configured. - For each failed TRP for a CC, BFRQ carries information whether a new candidate beam is found when two NBI-RS sets are configured, and new beam index (if found when two NBI-RS sets are configured). [mod]: This request has been made numerous times, and I have referenced the past agreement that there is a one-to-one association between BFD-RS set and NBI-RS set, so there must be a NBI-RS set configured corresponding to each BFD-RS set. Also in Rel.16 there was an agreement that NBI-RS set must be configured. Can you please explain in what situation the NBI-RS is not configured? |
| DOCOMO | Do not understand how Alt-2 works if there is no new beam found? |
| Convida Wireless2 | @DOCOMO: In Alt-2, the failed TRP identity would be included if no new beam is found. If no new beam is found, there are plenty of reserved bits in the corresponding octet in the MAC CE. @FL: To clarify Alt-2, could we please revise as follows, first to align with the 3rd bullet wording (new beam index if found) and second to clarify based on DOCOMO’s question)? - Alt-2: implicitly through new beam index, if found, else explicitly through BFD-RS set index |
| Apple | The implicit rule in alt2 cannot work since we failed to each any conseunsus for issue 7. If CBD-RS set index is reported, there is no difference to report it compared to report BFD-RS set index. |
| ZTE | We share the same concerns with DOCOMO about Alt-2, besides that we can not support 1 MAC-CE. For m-DCI, we think that straightforwardly, the TRP related CORESETPoolID can be reported directly. |
| Moderator | Revised alt-2 based on Convida’s view. @ZTE: On 1 vs 2 MAC-CE, back in Rel.16 SCell BFR, a single MAC-CE was agreed for multiple CC. When it comes to TRP level, can you elaborate why 2 MAC-CEs must be used? |
| Lenovo&MotM | Support Alt 1. |
| Nokia/NSB | Support the proposal. We support alt 1 between the alternatives. |
| MediaTek | Regarding Alt2, we don't see the benefit from the revised Alt-2 proposed by CW. It seems some bits in MACCE are still resevered for reporting of BFD-RS set index if no beam is found. Furthermore, as mentioned by Apple, RAN1 doesn't agree that two NBI-RS sets are always disjoint. |

Issue 11: UE assumption after BFR response

Proposal 3.8 : After receiving BFR response

- For each failed TRP, the DL QCL-typeD assumption of all CORESETs associated with that TRP with 1 activated TCI state is updated by the RS associated with the latest reported new candidate beam (if found when NBI-RS set is configured).

  - FFS: How to associate CORESET(s) with each TRP

  - FFS: timeline for the new beam updte after receiving BFR response

- FFS: Update of QCL-type D  assumption UL spatial filter/power control assumption for PUCCH, and other channels/RSs.

- The above applies at least to SCell; FFS SpCell

| Company | Technical views |
| --- | --- |
| Moderator | The proposal is updated based on inputs from MediaTek, vivo, Huawei, HiSilicon. |
| Qualcomm | Do we need the last bullet, given the 1st bullet? |
| Moderator | @Qualcomm: deleted the last (duplicated) bullet |
| Xiaomi | Support the proposal |
| DOCOMO | Support |
| Apple | We feel this is connected with issue 1. We suggest to postpone the decision. |
| ZTE | In the main bullets, we need to consider whether ‘28 symbols’ after receiving BFR procedure is needed. If needed, we need to FFS the SCS determination for the 28 symbols. [mod]: I think these details can be discussed later. Would this be OK? |
| LGE | OK |
| Lenovo&MotM | Support. |
| MediaTek | Okay to this proposal. Re ZTE’s comment, we suggest to we can add one FFS: FFS: Timeline for the new beam update after receiving BFR response |
| Mod | Added the 2nd note under the 1st bullet. |

Previous agreements

RAN1#102-e

Agreement

For L1-RSRP, consider measurement / reporting enhancement to facilitate inter-TRP beam pairing

- Option-1: Group-based reporting,

  - e.g., beam restriction to facilitate inter-TRP pairing.

- Option-2: Non-group-based reporting

Agreement

Evaluate and study at least but not limited to the following issues for multi-beam enhancement

- Issue 1: Consideration of inter-beam interference

- Issue 2: For group-based reporting, increased number of groups and/or beams per group

- Issue 3: UE Rx panel related beam measurement/report

  - NOTE: “UE panel” is used for discussion purpose only

Agreement

- Evaluate enhancement to enable per-TRP based beam failure recovery starting with Rel-15/16 BFR as the baseline.

- Consider following potential enhancement aspects to enable per-TRP based beam failure recovery

  - Issue 1: TRP-specific BFD

  - Issue 2: TRP-specific new candidate beam identification

  - Issue 3: TRP-specific BFRQ

  - Issue 4: gNB response enhancement

  - Issue 5: UE behavior on QCL/spatial relation assumption/UL power control for DL and UL channels/RSs after receiving gNB response

Agreement

Study Rel.17 enhancements on beam management for multi-TRPs with following priority

- High priority:

  - Beam measurement/reporting enhancement

  - Beam failure recovery for multi-TRP

- Low priority

  - Simultaneous reception of same type of channel/RS with different QCL-TypeD

  - Simultaneous reception of different type of channel/RS with different QCL-TypeD

RAN1#103-e

Agreement

Down-select at least one of the following options for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing in RAN1 #104-e

- Option 1: In a CSI-report, UE can report N>1 pair/groups and M>=1 beams per pair/group

  - Different beams in different pairs/groups can be received simultaneously

  - FFS: whether M is equal or can be different across different pair/group

- Option 2: In a CSI-report, UE can report N(N>=1) pairs/groups and M (M>1) beams per pair/group

  - Different beams within a pair/group can be received simultaneously

- Option 3: UE report M(M>=1) beams in N (N>1) CSI-reports corresponding to N report setting

  - Different beams in different CSI-reports can be received simultaneously

  - FFS: whether/how to introduce an association between different CSI-reports

  - FFS: whether/how to differentiate reported measurements for beams that are received simultaneously vs. beams that are not received simultaneously

    - whether/how to introduce an indication along with the CSI-reports to indicate whether the beams in different CSI-reports can be received simultaneously

- FFS: value of N and M in each option

- FFS: Association between different beams in above options and different TRP/UE panels

- FFS: Identify new use cases per option compared with R16 (including backhaul)

- FFS: whether different beams in different pairs/groups/reports can be received by same spatial filter per option

Agreement

- For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set.

  - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP

  - Support at least one of explicit and implicit BFD-RS configuration

    - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured

      - FFS: Further study QCL relationship between BFD-RS and CORESET

    - FFS: How to determine implicit BFD-RS configuration, if supported

- For M-TRP new beam identification

  - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured

    - FFS: detail on association of BFD-RS and NBI-RS

    - Support the same new beam identification and configuration criteria as Rel.16, including  L1-RSRP, threshold

Agreement

- Support TRP-specific BFD counter and timer in the MAC procedure

  - The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS

Agreement

- Support a BFRQ framework based on Rel.16 SCell BFR BFRQ

  - In RAN1#104-e, select one from the following options

    - Option 1: Up to one dedicated PUCCH-SR resource in a cell group

      - A cell group refers to either MCG, SCG, or PUCCH cell group

      - FFS: number of spatial filters associated with the PUCCH-SR resources

      - FFS: How the SR configuration is done

    - Option 2:  Up to two (or more) dedicated PUCCH-SR resources in a cell group

      - A cell group refers to either MCG, SCG, or PUCCH cell group

      - FFS: whether each PUCCH-SR resource is restricted to be associated to one spatial filter

      - FFS: How the SR configuration is done

  - FFS: Whether no dedicated PUCCH-SR resource can be supported in addition to Option 1 or Option 2

- Study whether and how to provide the following information in BFRQ MAC-CE

  - Index information of failed TRP(s)

  - CC index (if applicable)

  - New candidate beam index (if found)

  - Indication whether new beam(s) is found

  - FFS: whether/how to incorporate multi-TRP failure

RAN1#104-e

Agreement

For beam measurement in support of M-TRP simultaneous transmission

- Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously

  - Support M = 2

  - Support extending the maximum value of N > 1, exact value FFS

  - N=1 and N=2

    - FFS: Other values larger than 2

    - FFS: Whether the UE could report beams are received with different RX beams

- Further study the support of option 1 and option 3

- The above applies at least for L1-RSRP

  - FFS: L1-SINR

Agreement

- For M-TRP BFR Support 1-to-1 association between each BFD-RS set and an NBI-RS set

  - FFS: Association details

Agreement

For M-TRP BFR

- Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set

  - FFS: value of N (e.g. fixed in specification, or UE capability)

- FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability)

Agreement

For BFRQ of M-TRP BFR

- Option 3: Up to two dedicated PUCCH-SR resources in a cell group

- FFS: Whether PUCCH-SR for SCell can be reused for M-TRP

- Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found

  - Support at least indication of a single TRP failure

    - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE

    - FFS: whether/how to support  indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell)

- FFS: UE behavior when TRP failure status is different across cells

- FFS: Whether PUCCH SR resource can be configured with 2 spatial relations

RAN1#104b-e

Agreement: For beam reporting option 2

- On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e:

  - Alt1: Support maximum value N = {1, 2}

  - Alt2: Support maximum value N = {1, 2, 3, 4}

- FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17

- On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e

  - Alt1: The value of N is fixed by RRC configuration

  - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE

Agreement:

On CMR resource configuration for beam reporting option 2, adopt the following alternative:

- Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

  - FFS: extension to aperiodic CMR resource setting

- Each reported beam pair in a single CSI-report consists of M = 2 SSBRI/CRI values, where each SSBRI/CRI points to a CMR resource in a different CMR resource set or subset.

- Decide in RAN1#104b-e whether to adopt “set” or “subset” in the above.

Agreement

- Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs.

- FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

Agreement

On BFD-RS of TRP-specific BFR

- BFD-RS resource number:

  - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability

  - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e

    - Alt1: max value is 2

    - Alt2: max value is a UE capability, including possible candidate value of 1

Agreement

Adopt the following beam failure detection criteria for each BFD-RS set

- The physical layer in the UE assesses the radio link quality per BFD-RS set and indicates the BFD-RS set index to higher layers every X ms, if the hypothetical PDCCH BLER of all BFD-RS in the corresponding set of BFD-RS is higher than a threshold

  - X is max{minimal periodicity of BFD RS in the set, 2ms}

Agreement

- A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group

  - NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group

Reference

R1-2103858, “Moderator summary #1 on beam management enhancement for M-TRP with multiple Rx panels”, Moderator (CATT)

R1-2103906, “Moderator summary #2 on beam management enhancement for M-TRP with multiple Rx panels”, Moderator (CATT)