3GPP TSG RAN WG1#104b-e	 	 	R1-2103582

e-Meeting, April 12th – 20th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on DL-AoD positioning enhancements

Agenda Item:	8.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#91-e meeting, updated WID on NR positioning was agreed [1]. The work item includes RAN1-led objectives as follows:

The objective of this work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. The objective of this work item also includes the support of GNSS enhancements.

The specific objectives of this work are:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

Notes:

Solutions for RAT-dependent positioning enhancements are designed to operate in both frequency ranges (i.e. FR1 & FR2)

RAN working groups will take into account the work from other working group SA/CT which involves same objectives.

- The WID is subject to further update in RAN #91 for RAN1/2/3/4 scoping.

In this contribution, we present our view on DL-AoD enhancements for Rel-17 NR positioning.

- Potential enhancements for improving accuracy

  - Reporting and measurement of first arrival path

At the last meeting, potential enhancements regarding reporting and measurement of first arrival path were discussed and the following agreement was made [2].

Agreement:

- For both UE-based and UE-assisted DL-AOD study the following enhancements that enable the UE to measure and report (for UE-assisted) information related to the first arriving path

  - Option 1: Information corresponds to PRS-RSRP of the first arriving path

  - Option 2: Information corresponds to the angle of departure of the first arriving path

  - Option 3: Information corresponds to the arrival time of the first path

  - Option 4: Information corresponds to phase of the CIR corresponding to the first arriving path

  - Option 5: Information corresponds to received signal value (amplitude and phase of the channel estimated from the first path which can be achieved as a combination of option 1 and option 4) of the first arriving path

- FFS: Reporting of additional path to the first arriving path.

- FFS: Measurement definition details

- FFS: additional assistance data to support these enhancements

- FFS: how the “first path” is selected among PRS resources in a PRS resource set

- Note 1: Supporting multiple options as well as none of the options above is not precluded.

In the current NR, the angle based positioning methods are supported such as DL-AoD and UL-AoA. In the DL-AoD positioning, a UE measures RSRP of DL-PRS as positioning metric. In other words, a UE/NW (i.e. UE-based positioning/UE-assisted positioning) can calculate the UE’s location from the location information of TRP, the direction information of TRP Tx beams and the RSRP. When we consider the case that the UE is located to NLOS environment from a TRP, UE-assisted DL-AoD positioning based on RSRP reporting may be insufficient. For example, when a UE is located to NLOS or multi-path effective environment, the UE may calculate positioning metric by using an inappropriate direction of arrival beam and it may impact on positioning accuracy since RSRP information may be insufficient to identify whether the beam is NLOS or not. In order to solve above issue in the UE-assisted DL-AoD positioning, one potential scheme is to use more information than RSRP. At the last meeting, the candidate options are proposed. Option 1, Option 2 and Option 3 may need to support reporting on additional path in addition to that on the first arrival path in order to identify whether the UE is in LOS or NLOS environment. On the other hand, Option 4 may not need to support such reporting on additional path since to the phase information of the first arrival path would be sufficient to identify whether the UE is in LOS or NLOS. Based on the above discussion, we made the following observation.

Observation 1:

- Option 1, Option 2 and Option 3 may need to support the report of information on additional path in addition to that on the first arrival path to identify whether the UE is in LOS or NLOS environment

- Option 4 may need to support the report of information on the first arrival path only to identify whether the UE is in LOS or NLOS environment

  - Adjacent beam reporting

Regarding enhancements related to adjacent beam reporting, the following agreement was made at the last meeting [2].

Agreement:

For UE-assisted DL-AOD positioning method, study the following options to enable the UE to measure/report a PRS resource with an additional, adjacent PRS resources measurement/report:

- Option 1: UE can be requested to measure and report on specific PRS resources

- Option 2: Enhancing the assistance data to identify adjacent beams

- Option 3: Enhancing the reporting to include the measurements of adjacent beams

- FFS: Detailed signaling and procedure

- FFS: How to define adjacent beams

- Note: Depending on the discussion results, none/one/multiple of above options may be adopted in Rel-17

Rel-16 NR supports multiple RSRP measurements (up to 8 measurements) within a DL-AoD reporting. However, the UE may not report adjacent beams of best beam in a measurement reporting since the UE does not understand correspondence between DL-PRS resources and Tx beams. If the UE can measure and report these adjacent beams in addition to the best beam, it may be beneficial to improve DL-AoD positioning accuracy. Since NW understand corresponding DL-PRS resources and Tx beams, signaling specific PRS resources from NW side (i.e. Option 1) is simpler and may be enough to achieve the objective.

Observation 2:

- Rel-17 can consider the following option for further study

  - Option 1: UE can be requested to measure and report on specific PRS resources

  - RSRP measurement reporting

Regarding enhancements related to RSRP measurement reporting, the following agreement was made at the last meeting [2].

Agreement:

For UE-assisted DL AOD, select one of the following options for reporting of RSRP measurements per TRP

- Option 1: Up to 8 measurements in a measurement report (as in release 16)

- Option 2: Up to 8 measurements in a measurement report, for the same Rx beam index

- Option 3: Up to N>=8 measurements

  - Note: Multiple measurements corresponding to different Rx Beam index may be  reported for a given PRS resource.

  - FFS: value for N.

As we mentioned in the previous section, Rel-16 NR supports multiple RSRP measurements within a DL-AoD reporting. However, whether the multiple RSRP measurements correspond to the same Rx beam or not is up to implementation. If the multiple RSRP measurements correspond to the same Rx beam index, we think it will be helpful to improve positioning accuracy. Therefore, we prefer Option 2 compared with Option 1.

If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is enough to achieve sufficient positioning accuracy, Option 2 would be enough. On the other hand, if additional reporting based on different Rx beams is beneficial to improve the positioning accuracy, Option 3 can be considered.

Observation 3:

- Rel-17 can consider the following two alternatives for further study

  - Alt 1: If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is enough to achieve sufficient positioning accuracy, Option 2 is adopted

  - Alt 2: If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is not enough to achieve sufficient positioning accuracy, Option 3 is adopted and it includes Option 2 like measurement results (i.e., based on same Rx beam) and additional measurement result(s) (i.e., based on different Rx beam(s))

- Conclusion

In this contribution, we discussed on potential enhancements for Rel-17 DL-AoD positioning. Based on the discussion, we made following observations.

Observation 1:

- Option 1, Option 2 and Option 3 may need to support the report of information on additional path in addition to that on the first arrival path to identify whether the UE is in LOS or NLOS environment

- Option 4 may need to support the report of information on the first arrival path only to identify whether the UE is in LOS or NLOS environment

Observation 2:

- Rel-17 can consider the following option for further study

  - Option 1: UE can be requested to measure and report on specific PRS resources

Observation 3:

- Rel-17 can consider the following two alternatives for further study

  - Alt 1: If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is enough to achieve sufficient positioning accuracy, Option 2 is adopted

  - Alt 2: If a DL-AoD reporting where all RSRP measurements correspond to the same Rx beam index is not enough to achieve sufficient positioning accuracy, Option 3 is adopted and it includes Option 2 like measurement results (i.e., based on same Rx beam) and additional measurement result(s) (i.e., based on different Rx beam(s))

References

- RP-210903, 3GPP RAN#91-e, “Revised WID on NR Positioning Enhancements”, Electronic Meeting, Mar. 2021.

- 3GPP RAN1#104-e, Chairman’s Notes, Feb. 2021.