3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103748

E-meeting, April 12-20, 2021

Agenda Item:	7.2.7

Source:	Huawei, HiSilicon

Title:	Remaining issues for Rel-16 UE power saving

Document for:	Discussion and Decision

# Introduction

In RAN1#102-e, most of the critical issues were resolved [1], but there are still some issues to be corrected. This contribution discusses the remaining clarifications.

# Discussion

## Alignment of dormancy adaptation description with RAN2 specification

Regarding the description of dormancy adaptation, the parameters of dormancyGroupOutsideActiveTime and dormancyGroupWithinActiveTime are generally used to describe how UE interprets the bitmap used for dormancy indication.

10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= - a bitmap, when the UE is provided a number of groups of configured SCells by dormancyGroupOutsideActiveTime, where - the bitmap location is immediately after the Wake-up indication bit location - the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0 - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP

Table 1 DormancyGroupWithinActiveTime and dormancyGroupOutsideActiveTime defined in TS38.331

WithinActiveTimeConfig-r16 ::= SEQUENCE { firstWithinActiveTimeBWP-Id-r16 BWP-Id OPTIONAL, -- Need M dormancyGroupWithinActiveTime-r16 DormancyGroupID-r16 OPTIONAL -- Need R } OutsideActiveTimeConfig-r16 ::= SEQUENCE { firstOutsideActiveTimeBWP-Id-r16 BWP-Id OPTIONAL, -- Need M dormancyGroupOutsideActiveTime-r16 DormancyGroupID-r16 OPTIONAL -- Need R } ……skipped part…… DormantBWP-Config field descriptions. dormantBWP-Id This field contains the ID of the downlink bandwidth part to be used as dormant BWP. If this field is configured, its value is different from defaultDownlinkBWP-Id, and at least one of the withinActiveTimeConfig and outsideActiveTimeConfig should be configured..

Accroding to RAN1 specification, it seems a number of groups of configured SCells are provided directly by the parameter “dormancyGroupWithinActiveTime” or “dormancyGroupOutsideActiveTime”. However, according to TS 38.331 v16.3.1, RAN2 just defines the parameter dormancyGroupWithinActiveTime as a parameter configured for each SCell. And one dormancyGroupWithinActiveTime just indicates one SCell group ID for the configured SCell. Therefore, a number of SCell groups are actually figured out according to a number of parameters "dormancyGroupWithinActiveTime" configured for a number of configurd SCells. Therefore, it is suggested to reflect this in TS 38.213 by using “according to” to replace “provided by”.

RAN1 should also use the plural form of the parameters “dormancyGroupWithinActiveTime” and “dormancyGroupOutsideActiveTime” to align with RRC parameter definition of dormancyGroupWithinActiveTime and dormancyGroupOutsideActiveTime”.

- Adopt the following change for dormancy indication in TS 38.213.

10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= - a bitmap, when the UE is provided a number of groups of configured SCells by higher layer parameter(s) dormancyGroupOutsideActiveTime, where - the bitmap location is immediately after the Wake-up indication bit location - the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by according to higher layer parameter(s) dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0 - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP

# Conclusions

According to the previous discussion, the following updates are needed:

- Adopt the following change for dormancy indication in TS 38.213.

10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ======skipped part======= - a bitmap, when the UE is provided a number of groups of configured SCells by high layer parameter(s) dormancyGroupOutsideActiveTime, where - the bitmap location is immediately after the Wake-up indication bit location - the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ======skipped part======= If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by according to high layer parameter(s) dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0 - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP

# References

- RAN1#103-e Chairman’s Notes.

- RP-190727, "New WID: UE Power Saving in NR", CATT, CAICT.