3GPP TSG RAN WG1 Meeting #106-e 	R1- 2106462

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	RAN1 aspects of RedCap UE type and identification

Document for:	Discussion and Decision

# Introduction

In RAN#92-e meeting, the new WID on support of the reduced capability NR devices was approved [1],

- Specify definition of one RedCap UE type including capabilities for RedCap UE identification and for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. [RAN2, RAN1]

  - The existing UE capability framework is used; changes to capability signalling are specified only if necessary.

- Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and MsgA if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1]

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

- Specify necessary updates of UE capabilities (38.306) and RRC parameters (38.331). [RAN2]

and the following objectives in the WID were included:

In this contribution, three issues are discussed. The first issue is that how to define UE type for RedCap, the second issue is how to identify RedCap UEs, and the third issue is the capability indication and access restriction.

# Discussion

## 2.1   Device type and definition

Agreements: - For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. - Detailed signalling is up to RAN2 Working assumption: - RedCap UE type is defined based on one of the following options - Option 2: Only include the reduced capabilities that the network needs to know during initial access, if any. - Option 4: The corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support - FFS: details of the set of reduced capabilities Working assumption: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled - FFS How to support enable/disable the early indication - FFS details e.g.: - separate initial UL BWP - separate PRACH resource - PRACH preamble partitioning - FFS the possibility of supporting Msg3 for the early indication Agreements: (if the above working assumption is confirmed) - Early indication of RedCap UEs in Msg1 can be enabled/disabled via SIB

In our view, the UE type for RedCap is defined by a minimum (mandatory) capability set, on top of which additional optional UE features can be reported with existing capability framework, where details can be discussed in RAN2. Only one RedCap UE type is specified as mandated by the WID [1], and early identification of RedCap UE type is used to differentiate RedCap UEs from non-RedCap UEs. So only initial access related capabilities need to be included in the minimum capability set for RedCap UEs. As agreed in RAN1#105-e, there is no need for the number of Rx branches to be reported during initial access. Moreover, if the existing switching times for UE not capable of full duplex are reused for HD-FDD RedCap UEs, in general there is no specification impact during RedCap UE’s initial access and random access procedure, so the half-duplex capability can be considered as an optional capability and reported to the network after random access. Only the maximum UE channel bandwidth supported by RedCap UE can affect the initial access, in consideration of the initial UL BWP configuration vs. RedCap UE maximum bandwidth, and thus early identification is justified and necessary.

Although the current agreements have provided two options, progress for different cost/complexity techniques is different. It would be preferred to specially determine what needs to be included to the UE type definition by directly digging into the details of the reduced capabilities, as the remaining FFS, instead of using a general statement. Therefore,

Proposal 1: The RedCap UE type is defined by the maximum UE channel bandwidth only.

## 2.2   Device identification

The feasibility, necessity, pros and cons from RAN1 perspective for RedCap UE identification during Msg1, Msg3 or post Msg4 acknowledgement are captured in the TR [3]. Following analysis are listed:

### Necessity of early identification

According to the objectives of the revised WID, the functionality that enables RedCap UEs to be explicitly identifiable through an early indication need to be specified. The bandwidth and peak data rate of NR RedCap UE will be less than that of non-RedCap UEs. Early identification can avoid unnecessary limitations on non-RedCap UE’s scheduling and DL/UL data transmission during initial access procedure, when considering the coexistence of NR RedCap UEs with NR non-RedCap UEs.

If the RedCap UEs can be identified during initial access, the earlier the network can identify the RedCap UEs, the better for the gNB to separately schedule the data transmission and make network control on RedCap UEs.

### Identification via Msg1

The main necessity for identifying RedCap UEs during Msg1 is to enable frequency hopping of PUSCH for Msg3 or PUCCH for Msg4 HARQ feedback within the maximum channel bandwidth for RedCap UEs. In RAN1 #105-e meeting, the identified methods include separate initial UL BWP between RedCap UEs and non-RedCap UEs, separate PRACH resource between RedCap UEs and non-RedCap UEs, or PRACH preamble partitioning. We have the following analysis:

Separate initial UL BWP between RedCap UEs and non-RedCap UEs

In RAN1#105-e, the following working assumption have been made:

- For enabling/supporting that the RACH occasion (RO) associated with the best SSB falls within the RedCap UE bandwidth, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth), and this separate initial UL BWP for RedCap includes ROs for RedCap UEs.

  - Note: these ROs can be dedicated for RedCap UEs or shared with non-RedCap UEs.

According to the working assumption, RedCap UEs and non-RedCap UEs can be configured with separate initial UL BWP respectively, but share the same PRACH configuration (PRACH resource and preambles range). In that case, the RedCap UEs and the non-RedCap UEs still cannot be identified. So separate PRACH resource or PRACH preamble partitioning shall be the identifying methods for RedCap UEs and non-RedCap UEs.

Separate PRACH occasions between RedCap UEs and non-RedCap UEs

For TDM method, e.g. separate PRACH occasions, the gNB still has to reserve separate occasion(s) for the PRACH transmission, which may lead to inefficient resource utilization. Moreover, some PRACH resource configuration restriction (especially for TDD) to non-RedCap UEs may be introduced and the latency of random access for non-RedCap UEs may be prolonged considering the PRACH opportunities of non-RedCap UEs are probably scattered. One alternative of the standardization work to support TDM is to define dedicate prach-ConfigurationIndex to configure PRACH resource sets separately for the RedCap UEs on the time dimension, and new table to designate available radio frames and slots corresponding to the values of prach-ConfigurationIndex for RedCap UEs would be needed.

For FDM manner, parameter msg1-FDM is used in NR to configure the number of PRACH transmission occasions multiplexing in one time instance. Msg1-FrequencyStart indicates offset of lowest PRACH transmission occasion in frequency domain with respective to PRB#0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. In RedCap, a frequency offset between RedCap UEs and non-RedCap UEs, or a separate frequency offset for RedCap UEs can be used.

Separate preambles between RedCap UEs and non-RedCap UEs

If PRACH occasions are shared, preambles can be used to distinguish RedCap UEs and non-RedCap UEs. The maximum number of available preambles (for both contention-based random access and non-contention-based random access) in a cell is 64 in the current specification. If the number of available preambles in a cell is increased to consider the CDM of UEs, it will cause standardization work on how to determine the root sequence for the increased preambles, and it may also impact the correlation detection of non-RedCap UE’s preamble reception. Therefore, under the condition that high connection density is not considered in WID, and considering the limited time, the maximum number of available preambles in a cell should be kept to 64 in R17.

The current parameter totalNumberOfRA-Preambles configures the number of preambles for random access for non-RedCap UEs. Preamble partitioning between RedCap UEs and non-RedCap UEs can be used, for example, a separate pool of preambles can be configured as shown in figure 1.

Figure 1. Example for separate preambles between RedCap UEs and non-RedCap UEs

Either separate PRACH occasions or separate preambles are feasible but may have benefits only in applicable scenarios, considering tradeoff between access latency, user capacity, system overhead, and resource utilization in uplink and downlink. For some bands, the maximum channel bandwidth is not larger than 20MHz, then there is no need to identify RedCap UEs during Msg1 transmission for FR1. Thus early identification is also configurable by SIB1, explicitly or implicitly. The use of the two approaches is also left to gNB configuration while, from RedCap UE point of view, a separate IE would be cleaner if early identification by separate RACH resources (PRACH occasions or preambles) is enabled.

### Identification via Msg3

Identify RedCap UEs during Msg3 transmission can be beneficial for PUCCH for Msg4 HARQ feedback. But according to the agreement in RAN1 #105e meeting, a separate initial UL BWP for RedCap UEs is configured if the initial UL BWP for legacy UEs is larger than maximum channel bandwidth for RedCap UEs, which needs identifying RedCap UEs during Msg1Therefore, it is preferred to identify RedCap UEs during Msg1 transmission in this case. While if the initial UL BWP for legacy UEs is no larger than maximum channel bandwidth for RedCap UEs and the RedCap UEs share the same BWP with legacy UEs, then identify RedCap UEs during Msg3 transmission may be preferred for avoiding  PRACH resource  partitioning.

### Identification via MsgA

The RedCap UEs can be also identified by MsgA transmission if 2-step RACH is supported by these RedCap UEs. Following resources in MsgA can be used for identification, e.g. preamble, RO, PRU. The details of identification via MsgA can be further discussed after the identification via Msg1 or Msg3 is confirmed, since the identification method in 4-step RACH can be the reference of 2-step RACH.

### Configurable by network

Above options can be configured by the network with considering different situation in real deployment. Since identifying RedCap UEs via Msg1 could lead to potential reduction in PRACH user capacity and potential increase on gNB processing resources for preamble, identifying RedCap UEs via Msg3 can be configured by network when it is not necessary to identify RedCap UEs via Msg1. The network can configure early identification through broadcast signaling. The network can also configure separate PRACH resource for RedCap UE if early identification via Msg1 is configured. If 2-step RACH is supported by the network and the RedCap UEs, the network can also configure separate preamble, RO or PRU resources for RedCap UE in SIB1.

Proposal 2: A RedCap UE can be early identified via Msg1 or Msg3 for 4-step RACH, or MsgA for 2-step RACH if 2-step RACH is supported, which is configurable via SIB1.

- For Msg1 based early identification, separate PRACH occasions or separate preambles on shared PRACH occasion can be used between RedCap UEs and non-RedCap UEs. For MsgA based early identification, details can be discussed after the discussion of Msg1/Msg3 based early identification.

## 2.3   Capability indication and access restriction

As specified in the updated WID, system information can indicate whether a RedCap UE can camp on the cell/frequency, and the indication can be specific to 1Rx or 2Rx.

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

In our understanding, there are two potential motivations for restricting the access of RedCap UEs to avoid unnecessary network access attempt of the RedCap UEs.

- Motivation 1: Network does not support RedCap UEs.

- Motivation 2: Network supports RedCap UEs, but restricts the access of all or part of RedCap UEs for load balancing and system performance.

For the first motivation, the network should indicate to the RedCap UEs whether the network supports RedCap UEs accessing or not. For the commercialization of networks, support of NR RedCap UEs could be deployed gradually. In the actual network environment, especially for the initial deployment phase, it is possible that some cells do not support RedCap UEs in the early stage. If the network indicates no support of NR RedCap UEs, the UEs will not attempt to access the network again or not attempt to access for a much longer time to avoid unnecessary power consumption. In this case, the UE may continue to scan SSB raster to find another suitable cell to camp on. Therefore, the earlier the network indicates its capability, the better the RedCap UEs can save power consumption.

Proposal 3: It is necessary for network to indicate whether it supports RedCap UEs accessing or not.

For the second motivation as stated in WID, if the network supports RedCap UEs, the network can indicate whether it allows the RedCap UEs or part of RedCap UEs (e.g. RedCap UEs with 1Rx) access or not. For example, the network wants to load balance or protect the access of non-RedCap UEs.

Based on the above discussion, it should be allowed that network can restrict the access of all RedCap UEs or part of RedCap UEs (e.g. RedCap UEs with 1Rx) according to the network decision.

According to the current NR specification, there are some mechanisms for the network to indicate whether the network allows the UE’s access or not, for example, via the indications in MIB or SIB1 to inform the UE to exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. As RedCap UEs are not typically latency sensitive during initial access, if the restriction on the time for cell selection/reselection could be adjusted by the gNB according to the traffic load of the network and the RX branches number of the UEs, it will be beneficial for power saving of the UEs and control flexibility of the gNB. Use of 2 more spare bits in PBCH payload in FR1 can also be considered.

Currently, there are 15 spare bits in DCI associated with SIB1, which can be used to make access restriction of RedCap UEs. Compared with access restriction via SIB1, if restricting the access of RedCap UEs via DCI associated with SIB1, the RedCap UEs would stop the system information acquisition procedure once the RedCap UE successfully decodes the DCI with the access restricting information. Since unnecessary SIB1 decoding can be avoided for the RedCap UEs, it is beneficial for power saving. It shall be possible to enable access control separately for 1Rx/2Rx RedCap UEs, for example, two bits in DCI associated with SIB1 specific for 1Rx and 2Rx RedCap UEs are utilized. For different number of Rx branches, network may choose to use a different decision, e.g. time for re-access to indicate how it may expect various RedCap UEs.

Observation 1: Compared with access restriction via SIB1, access restriction via DCI associated with SIB1 is beneficial for RedCap UE’s power saving and has minor specification impact.

Proposal 4: Consider to restrict the access of RedCap UEs via SIB1

- Access control specific to RedCap UEs with 1Rx or 2Rx via DCI associated with SIB1

- Different cell selection/reselection time for 1Rx or 2Rx can be configured by gNB.

# Conclusions

In this contribution, preliminary considerations are provided on defining and constraining reduced capabilities, as well as the identification and access restriction of reduced capabilities devices. Moreover, the following observations and proposals are given:

Observation 1: Compared with access restriction via SIB1, access restriction via DCI associated with SIB1 is beneficial for RedCap UE’s power saving and has minor specification impact.

Proposal 1: The RedCap UE type is defined by the maximum UE channel bandwidth only.

Proposal 2: A RedCap UE can be early identified via Msg1 or Msg3 for 4-step RACH, or MsgA for 2-step RACH if 2-step RACH is supported, which is configurable via SIB1.

- For Msg1 based early identification, separate PRACH occasions or separate preambles on shared PRACH occasion can be used between RedCap UEs and non-RedCap UEs. For MsgA based early identification, details can be discussed after the discussion of Msg1/Msg3 based early identification.

Proposal 3: It is necessary for network to indicate whether it supports RedCap UEs accessing or not.

Proposal 4: Consider to restrict the access of RedCap UEs via SIB1

- Access control specific to RedCap UEs with 1Rx or 2Rx via DCI associated with SIB1

- Different cell selection/reselection time for 1Rx or 2Rx can be configured by gNB.

# References

- RP-211574, “Revised WID on support of reduced capability NR devices”, RAN#92e.

- “Chairman’s Notes RAN1 #105-e final”, E-meeting, May, 2021.

- TR 38.875, “Study on support of reduced capability NR devices”.