3GPP TSG RAN WG1 Meeting #106-e	                                            R1-2106466

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.2.3

Source:	Huawei, HiSilicon

Title:	Enhancements on beam management for multi-TRP in Rel-17

Document for:	Discussion and Decision

# Introduction

In RAN1 #105-e meeting [1], the following agreements were reached for M-TRP BM and M-TRP BFR. In this paper, we provide our views on the remaining issues.

Agreements on M-TRP BM Agreement For CMR configuration for option 2, adopt - Alt-1: “set” Agreement The bitwidth of each SSBRI/CRI is determined based on the number of SSB/CSI-RS resources in the associated CMR resource set - FFS: specify the association between SSBRIs/CRIs in a reported group and CMR resource sets Agreement - For beam measurement/reporting option 2, the maximum number of beam groups in a single CSI-report is a UE capability and may take value from Nmax = {1,2,3,4} in Rel.17. - FFS: If UCI payload reduction for Nmax>=2 is needed and if so, how - The number of beam groups (N) reported in a single CSI-report - Alt1: The value of N is configured by RRC signalling Agreements on M-TRP BFR Agreement Select one of the following alternatives with possible modification in RAN1#106-e - Alt 2.5.2 A: - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 B: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 C: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 D: - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

# M-TRP BM

The reporting format of Rel-17 group based reporting are undefined yet. With Rel-15 and Rel-16 beam reporting, differential reporting of L1-RSRP/L1-SINR was supported to reduce reporting overhead. With a maximum of 4 beam groups in Rel-17 group based reporting, the reporting overhead can be even larger. Hence, it is imperative to support differential reporting for Rel-17 group based reporting.

Figure 1. Three differential reporting mechanisms

Assuming N beam groups (i.e., 2*N CRI/SSIRI) and their RSRP values (i.e., 2*N RSRP values) are going to be reported. For each beam group, 2 CRI/SSBRI and their RSRP values are to be reported. For each resource set, N CRI/SSBRI and their RSRP values are to be reported. As illustrated in Figure 1, three differential reporting mechanisms can be considered as candidates. Among the three options, the first one has the lowest reporting overhead.

- Global differential reporting: For all the 2*N RSRP values, one of them (e.g., the largest one) is reported with 7-bit quantization and each of the remaining values is reported with 4-bit differential quantization.

- Differential reporting within each resource set: For N RSRP values corresponding to each resource set, one of them is reported with 7-bit quantization, and each of the remaining values is reported with 4-bit differential quantization.

- Differential reporting within each beam group: For 2 RSRP values corresponding to each beam group, one of them is reported with 7-bit quantization, and the other is reported with 4-bit differential quantization.

Proposal 1: For group-based beam reporting in Rel-17, support differential reporting for multiple beam groups, where the largest RSRP value across all beam groups is reported with 7-bit quantization and the other RSRP values are reported with 4-bit differential quantization.

# M-TRP BFR

In the last meeting, it was discussed whether implicit BFD-RS derivation can be supported for S-DCI case. In our view, this depends on whether two CORESET pools can be configured in S-DCI case. In Rel-16, two CORESET pools are mainly intended for M-DCI based M-TRP transmission. However, it does not mean that it is infeasible to configure two CORESET pools for S-DCI case. One benefit of allowing for two CORESET pools for S-DCI case is that implicit BFD-RS derivation can be supported based on configured CORESET pools, which enables faster BFD-RS update comparing to explicit RRC reconfiguration and also reduces signaling overhead.

Proposal 2: Support implicit BFD-RS derivation for S-DCI case when two CORESET pools are configured.

In the previous meetings, it was discussed how to report the index of the failed TRP. Basically, there were two options mentioned.

- Alt-1: Index of the BFD-RS set with beam failure is reported explicitly in BFR MAC CE

- Alt-2: If a resource index representing the identified candidate beam exists in BFR MAC CE, the index of the BFD-RS set with beam failure is implicitly indicated via this resource index; otherwise, the index of the BFD-RS set with beam failure is reported explicitly in the MAC CE.

In our view, Alt-2 looks overly complicated, such as indexing of NBI-RS across two NBI-RS sets, but without a clear benefit. So, we prefer Alt-1 at this point.

Proposal 3: Support reporting index of BFD-RS set with beam failure explicitly in BFR MAC CE.

In R17, RAN1 should focus or prioritize the use case where a cell (SpCell or SCell) is configured with two BFD-RS sets (i.e., M-TRP BFR). Possible BFR strategy for these two types of cells is summarized in Table 1.

Table 1 BFR strategy for different cases

| Cell type | Condition | UE behavior |
| --- | --- | --- |
| SpCell with two BFD-RS sets | Beam failure detected in one BFD-RS set | Report a new beam or indication of no new beam for the failed BFD-RS set via MAC CE |
|  | Beam failure detected in both BFD-RS sets, and one new beam is found | Fallback to RACH based BFR |
| SCell with two BFD-RS sets | Beam failure detected in one BFD-RS set | Report a new beam or indication of no new beam for the failed BFD-RS set via MAC CE |
|  | Beam failure detected in both BFD-RS set | Report a new beam or indication of no new beam for each failed BFD-RS set via MAC CE |

For SpCell configured with two BFD-RS sets, if beam failure is detected in one BFD-RS set, the UE should identify a new beam from the NBI-RS set associated with this BFD-RS set and report it to the gNB via MAC CE (possibly over the link towards the other non-failed TRP). If there is no new beam identified, indication of no new beam identified can be reported via MAC CE. While, if beam failure is detected in both BFD-RS sets (the links to two TRPs have both failed), the UE should identify a new beam from the two NBI-RS sets and report it to the gNB via RACH-based BFR procedure, if configured. If there is no new beam identified, the UE does nothing same as R15 BFR procedure.

Proposal 4: For SpCell configured with two BFD-RS sets,

- if beam failure is detected in only one BFD-RS set, the UE should report one new beam or indication of no new beam identified for that BFD-RS set via MAC CE;

- if beam failure is detected in both BFD-RS sets and a new beam is identified from any of configured NBI-RS sets, the UE should fall back to RACH based BFR procedure if configured.

For SCell configured with two BFD-RS sets, if beam failure is detected in one BFD-RS set, the UE shall identify a new beam in the NBI-RS set associated with the BFD-RS set and report it to the gNB via MAC CE (possibly over the link towards the other non-failed TRP). If there is no new beam identified, indication of no new beam identified is reported via MAC CE (different from the handling of SpCell). While, if beam failure is detected in both BFD-RS sets, the UE shall try to identify a new beam from the NBI-RS set associated with each BFD-RS set. In each NBI-RS set, the UE may or may not found new beam available. The UE report such information to the gNB via MAC CE. Namely, for each BFD-RS set, the UE reports one new beam or indication of no new beam identified via MAC CE (possibly over SpCell).

Proposal 5: For SCell configured with two BFD-RS sets,

- if beam failure is detected in one BFD-RS set, the UE should report one new beam or indication of no new beam identified for that BFD-RS set via MAC CE;

- if beam failure is detected in both BFD-RS sets, the UE should report one new beam or indication of no new beam identified for each BFD-RS set via MAC CE.

# Conclusion

This paper discusses issues on BM and BFR for multi-TRP transmission. In summary, the following proposals are made:

Proposal 1: For group-based beam reporting in Rel-17, support differential reporting for multiple beam groups, where the largest RSRP value across all beam groups is reported with 7-bit quantization and the other RSRP values are reported with 4-bit differential quantization.

Proposal 2: Support implicit BFD-RS derivation for S-DCI case when two CORESET pools are configured.

Proposal 3: Support reporting index of BFD-RS set with beam failure explicitly in BFR MAC CE.

Proposal 4: For SpCell configured with two BFD-RS sets,

- if beam failure is detected in only one BFD-RS set, the UE should report one new beam or indication of no new beam identified for that BFD-RS set via MAC CE;

- if beam failure is detected in both BFD-RS sets and a new beam is identified from any of configured NBI-RS sets, the UE should fall back to RACH based BFR procedure if configured.

Proposal 5: For SCell configured with two BFD-RS sets,

- if beam failure is detected in one BFD-RS set, the UE should report one new beam or indication of no new beam identified for that BFD-RS set via MAC;

- if beam failure is detected in both BFD-RS sets, the UE should report one new beam or indication of no new beam identified for each BFD-RS set via MAC CE.

# References

“3GPP RAN1#105-e Chair’s Notes”, May 10th – 27th, 2021.