3GPP TSG RAN WG1 Meeting #106-e	R1-2106480

E-meeting, August 16th-27th, 2021

Agenda Item:	8.7.1.2

Source:	Huawei, HiSilicon

Title:	Assistance RS occasions for IDLE/inactive mode

Document for:	Discussion and Decision

# Introduction

In this contribution, the remaining issues of assistance TRS for idle/inactive UE power saving enhancements are discussed.

# Availability indication for assistance TRS

## L1 based availability indication

In RAN1#105-e meeting, regarding the L1 based availability indication of assistance TRS the following agreement and working assumption were made [1]:

Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signaling Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB

It was agreed that L1 based availability indication of TRS/CSI-RS for IDLE/INACTIVE UEs is supported. When PEI is enabled, the IDLE/INACTIVE UEs can get the power saving gain by avoiding receiving paging PDCCH/PDSCH. In this case, to maintain the power saving gain, PEI should also indicate the availability. However, some companies showed concern on the information capacity of sequence based. It is the common understanding that PDCCH-based PEI can provide larger information capacity. Therefore, in the working assumption, it is proposed the availability is indicated at least for PDCCH-based PEI.

- PDCCH-based PEI can carry more information bits than SSS-based PEI and TRS based PEI.

Based on the above analysis, there is no gap to confirm the working assumption, therefore we have the following proposal.

- Confirm the working assumption, i.e.

- Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

  - FFS how to enable/disable L1 based availability indication configurable by SIB

## Indication content

In RAN1#105-e meeting, regarding the indication content of availability indication the following agreement was made [1]:

Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - FFS maximum number of configured RS resources per physical layer availability indication to support. - FFS whether availability/unavailability information is for all or some of configured RS resources

It is possible for gNB to configure multiple assistance RS resources. When the availability is indicated to the UE, a simple way is to indicate all the resources together, where for example only 1-bit may be needed. It is possible to indicate the availability of different RS resources separately, and it is more flexible to indicate the availability in a finer granularity, such as to use a bitmap where each bit indicate a separate RS resource or a group of resources.  On the contrary, it is questionable whether the codepoint-based indication can provide additional benefit than bitmap-based manner.

- Bitmap is the baseline for availability indication, where each bit indicates a RS or a group of RS.

It is also possible to indicate availability in beam selective manner. The availability of assistance TRS in a beam direction may change over time. Note that the assistance TRS can be the TRS configured for CONNECTED UEs. At a time, not each beam direction has CONNECTED UE. Besides, the beam directions that have CONNECTED UEs may vary over time. For example, a UE in a beam direction may enter connected-mode when data arrives and leave connected-mode after the data exchange is completed. In addition, a CONNECTED UE may move in or out of a beam direction due to its mobility. Therefore, in order to save resource overhead or gNB energy, the gNB may only transmit TRS in some of the beam directions, and for a specific beam, the gNB may not always keep the TRS alive.

As a result, to efficiently reflect the availability change of the assistance TRS in beam specific manner, the gNB may also need to indicate the availability indication in beam specific manner. For example, a bit in a bitmap corresponds to an SSB index and this bit can indicate the availability of the assistance TRS(s) that are QCLed with the corresponding SSB index, considering SSB can serve as the QCL resource of the assistance TRS.

Hence, we have the following proposal.

- Support to indicate the availability of assistance TRS occasion(s) per beam direction by a bitmap, where each bit corresponds to the assistance TRS(s) that are QCLed with the same associated SSB index.

## Validity time

During RAN1 #105-e, the validity time was discussed. And there are several options according to the FL summary [2].

Moderator proposal #5 L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration that can be determined based on one or more alternatives from the following: - Alt 1: configured by higher layer - Alt 2: a window before a PO. - E.g. Time duration between the end of L1 signal/channel carries the availability indication and the start of an associated PO. - Alt3: Included in the availability indication.

First of all, Alt 2 should be supported, i.e., to define the validity time as a window before a PO. The assistance TRS occasions is used for time/frequency tracking for IDLE mode UE. Only the RS transmitted inside a time duration which is before the PO makes sense for the UE as the assistance TRS for power saving. There is no need to require the gNB to transmit assistance TRS which is far away from a PO in time domain, considering it will not improve UE power consumption. Therefore, regarding the indicated range of assistance TRS in time domain, as an example shown in Figure 1, only indicating the availability of assistance RS in a specific time window associated with the PO would make the availability indication more precise and give gNB more flexibility. It is understood that PO1 and PO2 in  are for different UEs.

Furthermore, when multiple RS are configured, it is possible that the indication overhead could be further reduced if the availability of concerned RS in the specified window needs to be informed. As an example in Figure 1, for PO1 only the availability of RS2 needs to be indicated, while for PO2 only the availability of RS3 needs to be indicated. Then the signaling overhead for availability indication can be reduced to 1-bit from 3-bit (if bitmap is used).

It should be noted that defining the validity time as a window before a PO can provide the above two benefits (i.e. more flexibility and reduced overhead), no matter whether availability is indicated in paging DCI or in PEI. The details of the time window, e.g. the gap between the end of the window and the start of PO, could be further discussed.

Figure 1 Using paging DCI or PEI to indicate the availability of assistance RS

- Support to indicate the availability of assistance TRS in a window before the PO for both paging DCI based availability indication and PEI based availability indication.

Regarding Alt 1, we support to configure indication period, which is also related with the validity time.

For paging DCI based availability indication, when the availability change is infrequent, from gNB’s point of view, it would be a concern for frequently transmitted paging DCI for availability indication for every paging cycle. To transmit paging DCI less frequently, an indication period of multiple DRX cycle length, can be introduced during which the availability status of assistance TRS is assumed the same. As an example, gNB could only indicate the availability by the paging DCI transmitted at the beginning of the indication period, as shown in Figure 2

, where the indication period equals to 4 paging cycles. As another option, it would be also possible not to restrict that availability is only transmitted in the specified duration in indication period, UE can continue receive POs and can utilize available TRS until it is detected in a paging DCI in one PO . This can avoid additional paging DCI transmission due to the indication of availability.

It should be noted that in our view, the indication period is a different concept with validity time. Specifically, the validity time is a time duration, during which the availability of assistance TRS is indicated by paging PDCCH and/or PEI. But the indication period is a periodic time duration, during each one the availability of assistance TRS is assumed to be the same. So the indication period can work together with Alt2 (i.e. a window before PO). When the availability is indicated for an indication period, the signaling can be at the beginning of the indication period, or can be at any paging cycle of the indication period.

When availability is indicated in PEI, the indication period can be fixed to 1 paging cycle. Or, the indication period can also be configured for PEI when Behv-A is used..

Figure 2 Illustration of indication period

- At least for paging DCI based availability indication, an indication period is introduced during which the availability of assistance TRS is assumed to be the same.

In current specification, two paging cycle length can be configured, i.e. the default paging cycle configured in SIB and the UE specific paging cycle configured in NAS signaling. UE specific paging cycle is supported to deal with different delay requirement among a lot of UEs. The applied paging cycle length of a UE is the shorter one of the configured UE specific paging cycle and the default paging cycle. From UE perspective, different UEs may have different paging cycles. From the network perspective, same PO can be shared among UEs with different paging cycles to save network resource overhead. As shown in Figure 3, four UEs with different paging cycles can be paged on the same PO, i.e. the first PO in Figure 3. If there is availability indication in the paging DCI, different UE may have different understanding due to different paging cycles. As default paging cycle is common to all UEs, so the straight forward and simple way is to use default paging cycle as the granularity of indication period length.

Figure 3 Illustration of paging cycle

- Indication period is several default paging cycle length, which is common to all UEs and can avoid different understanding among UEs paged on the same PO.

The indication period can be fixed to one default paging cycle to save signaling overhead, or configured as N default paging cycles to provide some flexibility to gNB. For L1-based indication, only availability indicated is needed. In our view, selecting the value of N is a tradeoff between the signaling overhead and the capability for gNB to predict the status of TRS. To give gNB flexibility as much as possible, the value range of N can be e.g. {2, 4, 8, 16, 32}, where large value (like 32) can be used for short paging cycle (like 320ms).

- The length of the indication period can be configured to one default paging cycle or configured as N default paging cycles.

It should be noted that, the motivation of configuring validity time by higher layer is to reduce the signaling overhead of L1-based indication. The validity time in this case is just derived naturally and implicitly, or the validity time is configured by higher layer signaling for more precise control. Regarding the Alt3, i.e. including the validity time in the availability indication, we fail to see obvious benefit to dynamically indicate the validity time. Alt3 will increase also the signaling overhead. So we don’t support Alt3.

- Including the validity time in the availability indication provides no obvious benefit and increases the signaling overhead.

## SIB based indication

The following agreements were made in RAN1 #105-e [1].

Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB Agreement: Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured. - FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

In the RAN1 #105-e, it is proposed by some companies to enable/disable L1 based availability indication in SIB. Specifically, according to the discussion during RAN1 #105-e, our understanding of the solution is that if a TRS resource is configured but the L1 based indication of the TRS is disabled then the TRS resource will be viewed as always available. In other words, gNB ensures that the TRS is always transmitted. However, in the WID [3], there is a NOTE: “always-on TRS/CSI-RS transmission by gNodeB is not required”. Therefore, in our view the solution for enabling/disabling L1 based availability indication is not aligned with the WID. On the other hand, additional control on enabling/disabling L1 based indication seems not to be beneficial since it may be hard for gNB to predict which TRS resource will be transmitted or not for a very long time period. So dynamic control by L1 based indication for every configured TRS resource is more preferred. The L1 based indication can be enabled/disabled implicitly by the configuration of the TRS. In other words, if a TRS resource is configured in SIB, it is enabled to be indicated by L1 based signaling implicitly.

- The L1 based availability indication for a TRS resource is enabled/disabled implicitly by the presence/absence of the configuration of the TRS resource in SIB.

With the above understanding, the L1 based availability indication is always enabled once the TRS resource is configured in SIB. So there is no need to introduce ‘SIB based availability indication’. Besides, in our view, it is natural and a basic 3GPP procedure that if a TRS resource configuration is not present, then the TRS resource is not known (and thus not available) for IDLE/INACTIVE UEs. We don’t need agreement for the normal procedure. At last, according to the agreement, it is RAN1 understanding that existing SI update procedure is used for SIB based signaling. Therefore, any SIB based indication provide no power saving gain compared with acquiring the presence/absence of the TRS resources in SIB_X.

- No SIB based availability indication is supported for availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

# Configuration of assistance TRS

## QCL

In RAN1 #105-e, the following agreement was made.

Agreement: The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. - FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration - FFS: QCL type, which is predetermined

For the first FFS, we prefer to configure QCL per RS resource set. As it is analyzed in Section 3.2, configuring the QCL per RS resource set instead of per resource can reduce configuration signaling overhead. On the other hand, as it is analyzed in Section 2.2, the granularity of L1-based indication can be per resource set to reduce signaling overhead. If the configuration is also per resource set, it is easier to indicate the availability per RS resource set (i.e. per QCL)

- The QCL information is configured per resource set.

For the second FFS, in FR1 for IDLE/INACTIVE UEs, the DMRS of paging PDCCH and corresponding paging PDSCH is QCLed with an SSB with respect to ‘typeA’ implicitly. If the assistance TRS is QCLed with DMRS of paging PDCCH and PDSCH with respect to ‘typeC’, the assistance TRS can only provide coarse T/F tracking for IDLE/INACTIVE UEs and the power saving gain will be limited.

On the contrary, if the QCL type of assistance TRS is ‘typeA’, the TRS can provide delay spread and Doppler spread estimation, which is beneficial for finer T/F tracking. Only with the finer T/F tracking functionality, can the UE use the assistance TRS to replace SSB, and thus gets more power saving gain. Therefore, in our view the assistance TRS should be allowed to be QCLed with SSB with respect to ‘typeA’. To give gNB more flexibility, we suggest that QCL type for assistance TRS is configurable. An example is illustrated in Figure 4, where the QCL type of TRS1 is ‘typeA’ while the QCL type of TRS2 is ‘typeC’.

In FR2, to get the beam alignment, QCL-D should be additionally ensured.

Figure 4 Configurable QCL type to provide more power saving gain for UE to utilize the assistance TRS for time-frequency tracking

- The QCL type for TRS in IDLE/INACTIVE mode is configurable

  - In FR1, a TRS is QCLed  with an SSB with respect to either ‘typeA’ or ‘typeC’;

  - In FR2, a TRS is QCLed with an SSB with respect to either ‘typeA+D’ or ‘typeC+D’.

## Configuration index

In RAN1 #105-e, the following agreement was made.

Agreement: Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset {10, 20, 40, 80} ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - FFS Configuration index - details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source

For the FFS on configuration index we don’t see a need to introduce it. According to TS38.331 [4], when multiple resources (or resource sets) are configured, there is a ‘list’ to configure them. The order within the list is taken as the index of the corresponding resource (or resource set). In our view, such a design principle can be used for assistance TRS for IDLE/INACTIVE UEs. So neither explicit configuration index nor implicit index based on QCL source is needed.

- The configuration of assistance TRS for IDLE/INACTIVE UEs does not include configuration index.

## The consideration on reducing configuration overhead

When assistance TRS is configured in SIB it is important to reduce the signalling overhead. Otherwise, there may be not enough space in a SIB message to contain the configuration of multiple RS resources. In RAN1 #105-e, several methods to reduce configuration overhead were proposed and summarized in the moderator’s summary [2]. In the following, we will provide analyses for each method.

Alt1: The common configuration parameter per RS resource set, or group of sets

In our view, Alt1 can help to ‘compress’ the TRS configuration when multiple TRS resources are in the same resource set. In real deployment, the values for the parameters need to be selected carefully. It always happens that different RSs are with the same periodicity and the same transmitting power. So it brings the possibility that some parameters with the same value can be configured only once instead of multiple times.

It can be further discussed which parameter can be configured as common part.

Alt2: gNB provides a ‘reference configuration’, and each configured resource can have a ‘delta-configuration’ compared with the reference one

Alt2 can provide similar benefit as Alt1. For example, TRS0 is the reference configuration. If all the parameters of TRS1 are the same as those of TRS0 except the QCL and startingRB, gNB can only configure QCL and startingRB for TRS1. When UE receives the configuration, the UE can determine the values for parameters other than QCL and startingRB for TRS1 based on the reference configuration (i.e. TRS0).

Alt3: Predefine or fix a part of TRS parameters in specification

According to the agreement, there are some parameters that can be determined by some implicit way instead of explicitly configured just like in CONNECTED mode, i.e. the trs-info, and the time domain position of the second symbol. For other parameters, e.g. scramblingID, firstOFDMSymbolInTimeDomain, startingRB, and nrofRBs, they should be configurable to avoid restriction on gNB side.

Alt4: Number of RS resources/configurations be minimized

For FR2, there can be as many as 64 SSBs in a SSB burst set, which implies that there can be at most 64 beam directions. It is open to further discuss whether the number of configured RS resource needs to be limited.

Alt5: Configure TRS/CSI-RS SMTC, e.g. similar as SMTC for mobility

The main motivation to configure TRS resources in a SMTC-like manner is to ensure that the TRS(s) appear within a time period which is near to the PO(s). However, as we discussed, if the validity time is defined as a window before PO, the benefit can be already obtained. There is no need to increase the complexity on configurations of assistance TRS. Furthermore, it is not clear for us how the configuration signalling overhead could be reduced by this way. Some further clarification may be needed.

Alt6: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s)

It is already agreed that the time domain location of TRS occasion is configured by ‘periodicityAndOffset’. So Alt6 was already excluded.

Based on the above analysis, we support Alt1 and Alt2

- The following ways can be used to reduce signaling overhead for the TRS resource configuration

  - Alt1: The common configuration parameter per RS resource set, or group of sets

  - Alt2: gNB provides a ‘reference configuration’, and each configured resource can have a ‘delta-configuration’ compared with the reference one

# Conclusions

According to the previous discussion, we have the following observations and proposals:

- PDCCH-based PEI can carry more information bits than SSS-based PEI and TRS based PEI.

- Including the validity time in the availability indication provides no obvious benefit and increases the signaling overhead.

- Confirm the working assumption, i.e.

- Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

  - FFS how to enable/disable L1 based availability indication configurable by SIB

- Bitmap is the baseline for availability indication, where each bit indicates a RS or a group of RS.

- Support to indicate the availability of assistance TRS occasion(s) per beam direction by a bitmap, where each bit corresponds to the assistance TRS(s) that are QCLed with the same associated SSB index.

- Support to indicate the availability of assistance TRS in a window before the PO for both paging DCI based availability indication and PEI based availability indication.

- At least for paging DCI based availability indication, an indication period is introduced during which the availability of assistance TRS is assumed to be the same.

- Indication period is several default paging cycle length, which is common to all UEs and can avoid different understanding among UEs paged on the same PO.

- The length of the indication period can be configured to one default paging cycle or configured as N default paging cycles.

- The L1 based availability indication for a TRS resource is enabled/disabled implicitly by the presence/absence of the configuration of the TRS resource in SIB.

- No SIB based availability indication is supported for availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- The QCL information is configured per resource set.

- The QCL type for TRS in IDLE/INACTIVE mode is configurable

  - In FR1, a TRS is QCLed  with an SSB with respect to either ‘typeA’ or ‘typeC’;

  - In FR2, a TRS is QCLed with an SSB with respect to either ‘typeA+D’ or ‘typeC+D’.

- The configuration of assistance TRS for IDLE/INACTIVE UEs does not include configuration index.

- The following ways can be used to reduce signaling overhead for the TRS resource configuration

  - Alt1: The common configuration parameter per RS resource set, or group of sets

  - Alt2: gNB provides a ‘reference configuration’, and each configured resource can have a ‘delta-configuration’ compared with the reference one

# References

- Chairman's Notes RAN1#105-e.

- R1-2106117, “Summary for TRS/CSI-RS occasion(s) for idle/inactive UEs”, Samsung.

- RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek.

- TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”.

- Chairman's Notes RAN1#104-e.

# Appendix A. Assistance TRS agreements in RAN1#104-e

Agreements: Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least: - powerControlOffsetSS, - scramblingID - firstOFDMSymbolInTimeDomain, - startingRB. - nrofRBs, - FFS other parameters - FFS applicable values Agreements: The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#104b-e: - Alt1: same as initial BWP - Alt2: configurable parameter Agreements: Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details (including whether or not to restrict the RS to be TRS only) Decision: As per email posted on Jan 31st, Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable). Conclusion From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs. Agreements: The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e: - Alt-1: within initial DL BWP - Alt-2: is not restricted by initial BWP - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. Decision: As per email posted on Feb 4th, Agreements: To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives: - Alt-1: From higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS - Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO - FFS details - Other alternatives are not precluded Conclusion: Decide at RAN1#104b-e, whether or not to support periodic CSI-RS in addition to periodic TRS for TRS/CSI-RS occasion(s) for idle/inactive UEs.

# Appendix B. Assistance TRS agreements in RAN1#104bis-e

Agreement: SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0. Agreement: Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details of the QCL information, e.g. associated SSB index Agreement: IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. - Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. Working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signaling To further check on 4/19 Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported: - Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources - FFS whether and how to indicate the ‘availability’ in beam selective manner. - Other alternatives are not precluded

# Appendix C. Assistance TRS agreements in RAN1#105-e

Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - FFS maximum number of configured RS resources per physical layer availability indication to support. - FFS whether availability/unavailability information is for all or some of configured RS resources Agreement: Support applicable values for the following configuration parameters as below. - powerControlOffsetSS: {-3, 0, 3, 6}dB - scramblingID: 0 to 1023 - firstOFDMSymbolInTimeDomain: 0 to 9 - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4 - startingRB: 0 to 274 - nrofRBs: 24 to 276 Agreement: The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. - FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration - FFS: QCL type, which is predetermined Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB Agreement: Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset {10, 20, 40, 80} ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - FFS Configuration index - details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source Agreement: Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured. - FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously