3GPP TSG RAN WG1 #106-e			      R1-2106644

e-Meeting, August 16th – 27th, 2021

Agenda Item:		8.1.2.4

Source:				InterDigital, Inc.

Title:						M-TRP Operation for HST-SFN Deployment

Document for:		Discussion and Decision

- Introduction

In WG1 Meetings #105e, RAN1 continued the discussion on HST-SFN scenario. In the meeting, the discussion was mainly focused on aspects related to the TCI states in a M-TRP HST-SFN framework. Based on the discussion, the following agreements were reached [1],

- Confirm the following working assumption from RAN1#104b-e: All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1. - UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with scheme 1 PDSCH by RRC , but not capable to support dynamic switching between scheme 1 and single-TRP by TCI state field in DCI Format 1_1/1_2 - For specification based TRP-based frequency offset pre-compensation scheme - Support dynamic (DCI -based) switching with single-TRP scheme by TCI state field in DCI format 1_1/1_2 - This feature is UE optional - UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with TRP-based frequency PDSCH by RRC , but not capable to support dynamic switching between TRP-based frequency and single-TRP by TCI state field in DCI Format 1_1/1_2 - Support semi-static (RRC based) switching with Rel-16 schemes 1a, 2a, 2b, 3, 4 - Support semi-static (RRC based) switching with Rel-17 scheme 1 (PDSCH) - Enhanced MAC CE signaling is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP. - For TRP-based pre-compensation QCL assumptions is provided to the UE by using the existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type - FFS rule or signalling to determine which TCI state with dropped QCL parameters - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH. - FFS whether this restriction is per UE or per CC - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for different CORESETs. - FFS whether this restriction is per UE or per CC - Enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is identified by the number of TCI states activated per CORESET and RRC parameter - FFS: Configuration detail of RRC parameter - Including whether the same RRC parameter is used for PDCCH and PDSCH - If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP -based pre-compensation) is configured and a CORESET is activated with two TCI states and UE is configured with enableTwoDefaultTCI-States and time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, down-select rule to determine default beam(s) for Rel-17 SFN PDSCH reception in RAN1#106-e: - Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a - Alt 2: Introduce new rules to determine TCI states based on two TCI state(s) of the CORESET - If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, support the following configuration of RS for BFD - Down-select one alternative for implicit configuration - Alt 1-2: RS of CORESETs with both single and two TCI states are used - Alt 1-3: RS of CORESETs with only two TCI states are used - Down-select one alternative for explicit configuration - Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS - FFS other details - Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration - Note: down-selection can be done separately for Rel-15/16 cell specific BFR and Rel-17 TRP-specific BFR, Rel-17 TRP-specific BFR to be discussed under AI 8.1.2.3 Working Assumption - For TRP-based pre-compensation, Variant A (based on RAN1#103-e meeting agreement) are supported as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states. - FFS: Additional support of Variant B

In this contribution, following the directions laid out in agreements, we further discuss the enhancements for M-TRP to support HST-SFN deployment scenario and present our views.

# Background

HST is one of the important 5G NR deployment scenarios as captured in TR 38.913 [2]. HST deployment scenario focuses on providing a consistent user experience for the passengers and high-reliability low-latency communications for the high-speed trains. High-speed trains are expected to travel at a velocity of up to 500 km/h. Providing connectivity to passengers at such a high velocity would pose numerous challenges and may demand several enhancements to the existing technologies.

Figure 1 shows an HST-SFN scenario where multiple TRPs are spread out along the track path. These TRPs are connected to one BBU and share the same cell ID to reduce the number of handovers and increase the robustness of the air-interface. BBUs are usually connected via optical fiber aiming to provide enough backhaul capacity with a manageable level of backhaul delay.

Figure 1 - Use of a M-TRP configuration for HST-SFN

# Enhancements for HST-SFN

## TCI/QCL Indication

A TCI state may comprise of at least one combination of serving cell, bandwidth part identity, and at least one reference signal such as a CSI-RS or SSB. The UE may assume QCL relationship between the ports of such reference signals and the DM-RS ports to assist in the reception and demodulation of PDCCH or PDSCH. It would be inefficient to employ the existing mechanism of TCI indication for an HST-SFN scenario. Thus, modifying TCI/QCL framework targeting the unique requirements of HST-SFN deployment scenario is essential.

Figure 2: HST-SFN transmission with SFNed DM-RS and PDCCH/PDSCH transmission.

In the last meeting it was agreed to introduce an enhanced MAC CE signaling for PDCCH to activate two TCI states for SFN-based PDCCH transmission where the corresponding MAC CE indicates at least serving cell ID, CORESET ID and the two TCI state IDs [1].

Group-based Beam Indication

Given the fact that in an HST scenario, many UEs will require simultaneous update on their TCI states, from signalling overhead perspective it does not make much sense if the TCI update is done for each UE in an individual manner.  Therefore, it would be relevant if a group-based beam indication is used for beam indication of a CORESET.

For example, a CORESET can be configured for a group of UEs which its configuration can be provided via a broadcasting signal (e.g., SIB). Then, a group-based beam indication can determine a beam for a CORESET based on an explicit indication using a DCI or a broadcasting signal, wherein the DCI can be a group-common DCI monitored by a group of UEs. The DCI may be monitored in a common search space and its associated RNTI may be a group RNTI.

Observation 1: From signalling overhead perspective it does not make much sense if the TCI update is done for each UE in an individual manner.

Proposal 1: Support DCI-based group indication to indicate a beam for a CORESET.

Zone-based Beam Indication

Also, we can consider implicit determination based on a geographical location information of the UE (e.g., Zone-id) or time window information (e.g., a set of slots, subframe, radio frame, etc.). In a multi-TRP HST deployment, as UEs move from TRP to TRP, TCI info needs to be updated every few seconds for all the UEs in the train, making it very inefficient in terms of resource usage and its related signalling to perform TCI indication per individual UE basis. Therefore, as shown in Figure 3, a zone-based identification could be helpful to mitigate the signalling overhead.

A zone can be configured or determined based on the geographical coordinates of a UE, and it may be associated with a zone identity (e.g., zone-id). For example, information on TCI states could be shared by UEs within the same zone, therefore a gNB does not have to update TCI information of every UE individually every few seconds.

Observation 2: In a multi-TRP HST deployment, TCI/QCL info of all UEs need to be updated every few seconds.

Proposal 2: Study zone-based configuration for TCI/QCL information to mitigate potential high signalling overhead.

Figure 3- Zone-based TCI/QCL indication in HST-SFN Transmission

## PDCCH in SFN Manner

If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured, then two TCI states can be activated for at least one CORESET. In such scenario, we could have either an implicit or explicit configurations of RSs that are intended for BFD.

Implicit configuration of RSs

In the last meeting, two alternatives for implicit configuration of BFD RSs were identified for further discussion and down-selection [1],

- Alt 1-2: RS of CORESETs with both single and two TCI states are used

- Alt 1-3: RS of CORESETs with only two TCI states are used

In Alt 1-2, RS of CORESETs with both single and two TCI states are used. Therefore, from our perspective Alt 1-2, in certain cases, may have a better understanding of channel condition. For example, in case RSs of two TCI state could not be combined properly for measurement and cause an unnecessary BFD event, use of RS of CORESETs with single TCI state may be advantageous. However, use of Alt 1-2 would obviously require more processing and measurement effort than Alt 1-3 from UE side that may not be preferred. However, since the main purpose of SFN PDCCH is enhancing the reliability and performance, it may be worthwhile to use Alt 1-2 in certain deployment scenarios.

Observation 3: Alt 1-2 may provide a more reliable BFD process at the expense of more UE processing and complexity.

Proposal 3: For implicit BFD RS configuration, support RS of CORESETs with only two TCI states, and consider use of RS of CORSETs with single TCI state as an optional UE feature.

Explicit configuration of RSs

In the last meeting, two alternatives for explicit configuration of BFD RSs were identified for further analysis and down-selection [1],

- Down-select one alternative for explicit configuration

  - Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS

    - FFS other details

  - Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration

Given configuration of two TCI states per CORESET for SFN PDDCH, we are not sure how the existing process of Rel-15/Rel-16 that is based on single TCI per CORESET could provide satisfactory performance. On the other hand, Alt 2-2 requires additional specification work to support definition and configuration of CSI-RS resource or SSB pairs as BFD RS. However, despite extra specification work required by Alt 2-1, from performance perspective, Alt 2-1 may be a better solution than Alt 2-2.

Observation 4: Since in SFN PDCCH, two TCI states are configured per CORESET, it is not clear whether the existing process of Rel-15/Rel-16 could provide satisfactory performance.

Proposal 4: For explicit BFD RS configuration, support defining CSI-RS resource or SSB pairs as BFD RS.

SFN PDCCH for Scheme 2

A successful reception of PDSCH depends on the successful reception of the related PDCCH. Since two TRPs are geographically separated, the radio channels could experience completely different propagation conditions. Therefore, harvesting spatial diversity could increase the reliability of the communication. As in HST-SFN deployments, UEs move extremely fast, deep fading conditions that last only for a very short time period can occur very often. Since PDCCH is a maximum of up to 3 OFDM symbols long, it could be very susceptible to this deep fading compared to PDSCH.

Based on recent agreements, it is only Scheme 1 that supports SFN transmission of PDCCH. However, as argued earlier, it is important to support a similar level of reliability for PDCCH when using Scheme 2. One way to enhance the performance of PDCCH would be to rely on non-SFN PDCCH transmission. However, such an approach would not be efficient. First, it requires the use of additional resources, which impacts the spectrum efficiency of the system. And secondly, it increases the required number of blind decoding attempts by a UE, leading to an increase in power consumption. Therefore, non-SFN PDCCH transmission is not preferred.

To enable PDCCH transmission in an SFN manner, some considerations may be needed for PDCCH DM-RS transmission. In Scheme 2, PDSCH DM-RS are non-SFN, however such an approach cannot be considered for PDCCH DM-RS, as it is based on a single port transmission. Therefore, to support SFN transmission of PDCCH, one of the followings can be considered,

- PDCCH DM-RS can be sent in an SFN manner. Therefore, PDSCH DM-RS would be TRP-specific, but PDCCH DM-RS will remain in an SFN manner.

- PDCCH DM-RS can be enhanced to support non-SFN DM-RS transmission.  As such, UE will be able to perform separate channel estimation for processing of the SFNed PDCCH transmitted from multiple TRPs.

Observation 5: Transmitting PDCCH SFN manner could help to increase the reliability of the HST-SFN transmission.

Proposal 5: Support PDCCH SFN transmission for Scheme 2.

## Scheme 1 - Scheme 1a Switching

In the last meeting, DCI-based switching between Scheme 1 (PDSCH) and single-TRP transmission scheme by utilizing TCI state field in DCI format 1_1/1_2 was agreed. Also, due to potential implementation complexity issues, it was decided that this feature to be considered as a UE optional feature [1].

Furthermore, as for switching between HST-SFN Scheme 1 and Rel-16 scheme 1a, it was agreed to support semi-static (RRC-based) switching between Scheme 1 (PDSCH) and Rel-16 scheme 1a. As for support of dynamic switching, there does not seem to be any compelling reason for such requirement. HST-SFN Scheme 1 and Rel-16 Scheme 1a are developed for very different objectives, where the former is intended for achieving higher reliability and robustness and the later is mainly for enhancing throughput. Also, the impact of implementation aspects is much more severe than the case of switching between Scheme 1 and single-TRP. To enable such switch between the two modes, various receiver processing blocks need to be reconfigured and re-initialized.

Observation 6: For support of dynamic switching, there does not seem to be any viable reason for such requirement.

Proposal 6: RAN1 does not support dynamic switching between Scheme 1 (PDSCH) and Rel-16 scheme 1a.

## Doppler Pre-compensation Schemes

A network-based frequency offset pre-compensation scheme for HST-SFN scenario can be implemented base on the following steps,

- 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation

- 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step

- 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step

Based on the agreed three step high-level procedure, different compensation schemes with different implications can be implemented. For example, Figure 4 shows two potential schemes that can be implemented based on the agreement.

In the last meeting, it was agreed to support specification-based TRP Doppler pre-compensation scheme for FR1 [1]. It was also stated that at least one of UL RS based Doppler estimation by gNB and/or DL RS based Doppler feedback by UE should be supported. Since the performance of pre-compensation schemes depends heavily on the accuracy of Doppler estimation, some enhancements for UL/DL-RS-based Doppler estimation may be needed. Herein, we discuss two main issues that need to be studied for either approach,

Issue 1: Accuracy

In either approach of Doppler estimation, there are some factors that could impact the accuracy of the Doppler estimation.

In the case of Doppler estimation using SRS, it is not clear how robust SRS signals would behave in a high Doppler environment, and whether they can maintain their properties to be useful as a source of Doppler shift estimation. Also, it should be reminded that frequent transmission of SRS by all UEs increase collision likelihood and the resulting interference could further reduce accuracy of the Doppler estimation.

In the case of Doppler estimation using TRS, it is not clear how accurate such estimation would be as Doppler in an HST deployment is a time-varying phenomenon where fixed-rate TRS transmission and the delay involved in estimation and reporting may create a situation where the applied pre-compensation would always lag the actual value of Doppler.

Observation 7: In either approach of Doppler estimation, there are some aspects that could impact the accuracy of the Doppler estimation.

Proposal 7: Study accuracy of Doppler measurement, delay cycle between Doppler measurement to the instant of pre-compensation, and enhance RS design if needed.

Issue 2: Overhead

In either approach of Doppler estimation, overhead is an important issue that could severely impact the spectral efficiency of the system.

In the case of Doppler estimation using SRS, on one hand we need to have frequent SRS transmission by UEs to make sure that the estimated Doppler by gNB is not outdated, however on the other hand we need to minimize uplink resource usage to reduce interference and UE power consumption.

In the case of Doppler estimation by TRS, the problem is two-fold. Firstly, TRS transmissions need to be frequent enough to allow an accurate Doppler estimation by UE. Secondly, the updated Doppler estimate need to be continuously reported to gNB to allow an accurate tracking of Doppler value for pre-compensation.

Observation 8: In either approach of Doppler estimation, there are some aspects that could impact the uplink signalling overhead.

Proposal 8: Study overhead associated to transmission of SRS and CSI-report for Doppler measurement and report, respectively, and enhance related procedures if needed.

(a)

(b)

Figure 4 – Frequency pre-compensation schemes

## TRS Enhancements

Figure 5 – Experienced Doppler shift by a UE

One of the challenging issues in an HST deployment scenario is to have an accurate estimation of Doppler shift. In an HST deployment, the value of the Doppler shift is changing continuously, and its rate of change increases significantly as an HST travels in the vicinity of a TRP. This high rate of change in Doppler shift is due to the fact that a train has a much higher displacement with respect to a TRP in its vicinity than when it is far off a TRP. Figure 5 shows the perceived Doppler shift of different TRSs transmitted by different TRPs. Based on Figure 5, we can make a couple of observations.

- When an HST is far from both TRPs, the measured Doppler shifts on TRSs transmitted by the TRP pair is almost constant, equal and opposite in sign.

- When an HST is in the vicinity of a TRP, the measured Doppler shifts on TRSs transmitted by the TRP pair change at a very high rate.

- When an HST passes by a TRP, the sign of the Doppler shift changes abruptly.

Due to high mobility and high Doppler shift, the HST scenario's performance could be susceptible to Doppler measurement errors. To have an accurate estimate of the Doppler shift, we need to have a frequent transmission of TRSs. In Rel-16, TRS can be configured with 10 ms, 20 ms, 40 ms, or 80 ms periodicity. While a lower rate of TRS may be sufficient for areas where HST is relatively far from TRPs, they may not be frequent enough for areas in the vicinity of TRPs. Thus, TRSs are required to be transmitted frequently. However, based on the above observations, a high rate transmission of TRS is only needed when UE is in the TRP vicinity. Other than that, a UE can operate with a lower rate of TRS transmission.

.

Observation 9: To have an accurate estimation of Doppler shift, a high rate transmission of TRS is only needed when a UE is in the vicinity of a TRP, other than that a UE can operate with a lower rate of TRS transmission.

Proposal 9: Support variable-rate TRS transmission for HST deployment scenario.

# Performance Evaluation

# Performance Comparison of Schemes 1 and 2

In this section, we share our throughput evaluation results of both Scheme 1 and Scheme 2 considering CDL-D channels for an FDD system. UE is traveling at 350 km/h. Two modulation and coding schemes, MCS 13 and MCS 17, and antenna configurations, 2Tx2Rx and 8Tx2Rx, are considered. The network setup considered in the simulation is shown in Figure 6, and the complete set of simulation parameters are summarized in Table 1.

Figure 6: HST-SFN network setup for link-level simulation

Table 1 –Parameters for link-level simulation

| Parameter | Value |
| --- | --- |
| Duplexing | FDD (FR1) |
| TRP layout (Ds, Dmin, etc) | Ds=700m, Dmin=150m CDL based model – TRP height: 35m, UE height: 1.5m |
| TRP antenna configuration including number of antennas, pattern, ports, orientation, etc | CDL based extension: 2 ports: [Mg, Ng, M, N, P]=[1, 1, 1, 1, 2], 2 ports: [Mg, Ng, M, N, P]=[1, 1, 2, 2, 2], 2 ports: [Mg, Ng, M, N, P]=[1, 1, 8, 2, 2], Directional antenna: 2D-Gaussian |
| UE antenna configuration including number of antennas, pattern, ports, orientation, etc | 2 ports: [Mg, Ng, M, N, P]=[ 1, 1, 1, 1, 2] , Omni-directional antenna, fixed UE orientation angle |
| DMRS | Single port DMRS (port-0) for Scheme-1, two port DMRS (Ports 0&2) for Scheme-2. DM-RS type-1, # of DMRS 1 + 1 +1, |
| MCS | 13 and 17 |
| System BW | 10MHz |
| SCS | 30kHz |
| Allocated no. of PRBs | 10 PRBs |
| Carrier frequency and max doppler shift | 3.5GHz, 350kmph |
| Propagation condition | CDL extension (CDL-D = 100ns) |
| Precoder | None |
| UE Velocity | 350km/h |
| Doppler estimation | Ideal |
| Antenna downtilt and azimuth | elementMaxGainAz for RRH1=-23.2 degrees elementMaxGainAz for RRH2=-156.8 degrees Downtilt angle = 5.02 rad |

| Figure 7-Throughput of Scheme 1 and 2 in CDL-D channels with MCS 13 and 2Tx2Rx | Figure 8-Throughput of Scheme 1 and 2 in CDL-D channels with MCS 17 and 2Tx2Rx |
| --- | --- |
| Figure 9-Throughput of Scheme 1 and 2 in CDL-D channels with MCS 13 and 8Tx2Rx | Figure 10-Throughput of Scheme 1 and 2 in CDL-D channels with MCS 17 and 8Tx2Rx |

Figures 7 – 10, show throughput vs SNR of both HST-SFN Scheme 1 and 2 at different UE locations. Further, Figures 11-18 give variation of the throughput with UE’s location at four different SNR values, -5 dB, 5 dB, 10 dB, and 20 dB. In both sets of figures, MCS values 13 and 17 with 2 and 8 transmit antennas at the TRPs are considered. As demonstrated in the presented results, a UE experiences better throughput as it moves towards the midpoint (i.e., D1 = Ds/2= 350 m) of the two TRPs, especially at the low SNR region. The reason can be due to the orientation of the antenna beams, i.e., towards the railway track’s centre location between two TRPs. Further, figures indicate that at low to medium SNR region, UEs located towards the centre of the two TRPs achieve better throughput with Scheme 2 than Scheme 1. A similar observation could be made for D1 = 175 m, and 525 m cases with 8Tx2Rx antenna configuration. This indicates that having two orthogonal DM-RS resources helps achieve better channel estimates in low to medium SNR values. And having better channel estimates could help improve the performance.  However, in all the cases at high SNR values, Scheme 1 achieves better throughput than Scheme 2. This is mainly due to Scheme 1’s lower DM-RS overhead, which enables having more time-frequency resources for the PDSCH transmission in Scheme 1. In conclusion, in the considered simulation setup, the better choice of HST-SFN transmission scheme highly depends on the operating SNR region, location of the UE (including transmit beam orientation), and the number of transmit antennas at TRPs.

Observation 10: The better choice of HST-SFN transmission scheme highly depends on the operating SNR region, location of the UE (including transmit beam orientation), and the number of transmit antennas at TRPs.

Proposal 10: Support both Schemes 1 and 2 for HST-SFN transmission.

| Figure 11-Throughput vs D1 of Scheme 1 in CDL-D channels with MCS13 and 2Tx2Rx | Figure 12-Throughput vs D1 of Scheme 1 in CDL-D channels with MCS17 and 2Tx2Rx |
| --- | --- |
| Figure 13-Throughput vs D1 of Scheme 1 in CDL-D channels with MCS13 and 8Tx2Rx | Figure 14-Throughput vs D1 of Scheme 1 in CDL-D channels with MCS17 and 8Tx2Rx |

| Figure 15-Throughput vs D1 of Scheme 2 in CDL-D channels with MCS13 and 2Tx2Rx | Figure 16-Throughput vs D1 of Scheme 2 in CDL-D channels with MCS17 and 2Tx2Rx |
| --- | --- |
| Figure 17-Throughput vs D1 of Scheme 2 in CDL-D channels with MCS13 and 8Tx2Rx | Figure 18-Throughput vs D1 of Scheme 2 in CDL-D channels with MCS17 and 8Tx2Rx |

# Performance Evaluation of a 4-Tap Channel

This section shares our throughput evaluation results for an HeUE case considering a 4-Tap channel for an FDD system. UE is traveling at 500 km/h. Three modulation and coding schemes, MCS 4, MCS 13 and MCS 17; and antenna configuration, 2Tx2Rx, is considered. The complete set of simulation parameters are summarized in Table 2.

Table 2 –Parameters for link-level simulation

| Parameter | Value |
| --- | --- |
| Propagation condition | 4-tap channel model |
| TRP layout | Ds=700m, Dmin=150m |
| Carrier frequency | 3.5 GHz |
| Speed | 500 km/h |
| BW | 10 MHz |
| SCS | 30 kHz |
| Number of RBs for PDSCH | 50 PRBs |
| Duplexing | FDD |
| gNB antenna configuration | 2Tx |
| UE antenna configuration | 2Tx |
| DMRS configuration | DMRS type 1, 1+1+1 |
| MCS | MCS 4, 13, 17 based on 64QAM table |
| Rank | Rank 1 |
| Channel Estimation | Practical |

| Figure 19- Throughput performance comparison among UE receiver enhancement, the legacy UE with traditional channel estimation and the ideal channel estimation for MCS 4 | Figure 20- Throughput performance comparison among UE receiver enhancement, the legacy UE with traditional channel estimation and the ideal channel estimation for MCS 13 |  |
| --- | --- | --- |
| Figure 21- Throughput performance comparison among UE receiver enhancement, the legacy UE with traditional channel estimation and the ideal channel estimation for MCS 17 |  |

Figure 19-21 illustrates the throughput performance comparison of the legacy UE, HeUE, and the ideal channel estimation for MCS 4, 13, and 17. Here, HeUE corresponds to UE estimating the doppler shifts from each TRP separately and compensating the impact of multiple Doppler shifts by their weighted linear combination. These linear weights can be determined with respect to the UEs distance or received power relative to each TRP.

It can be observed from the throughput results in Figure 19 (i.e., MCS 4 case) and Figure 20 (i.e., MCS 13 case) that HeUE outperforms legacy UE in low SNR region and closely matches throughput of ideal channel estimation case in high SNR region. Moreover, the throughput results in Figure 21 (i.e., MCS 17 case) show that the HeUE outperforms legacy UE in low to high SNR region. In conclusion, in the considered simulation setup, the linear weighted Doppler shift compensation by HeUE performs significantly better than the legacy UE and can achieve notable throughput gains throughout the SNR region.

Figure 22- Throughput performance comparison, for MCS-17, among UE based doppler compensation, NW based pre-compensation with ideal doppler reporting and NW based pre-compensation with residual error in doppler reporting

The performance of the network based pre-compensation depends much on the accuracy of the indicated Doppler frequency. Figure 22 shows TP performance under different values of frequency residual. From the presented results in Figure 22, it can be observed that the performance difference between UE based compensation and NW based pre-compensation is almost nonexistent as long as the residual frequency error is low. In this exemplary configuration, it can be seen that as long as the accuracy of Doppler indication is within +/- 200Hz, the performance of NW based compensation does not incur any degradation.

# Conclusions

This contribution discussed potential enhancement for HST-SFN. Based on the presented discussion, we make the following observations and proposals,

Observation 1: From signalling overhead perspective it does not make much sense if the TCI update is done for each UE in an individual manner.

Observation 2: In a multi-TRP HST deployment, TCI/QCL info of all UEs need to be updated every few seconds

Observation 3: Alt 1-2 may provide a more reliable BFD process at the expense of more UE processing and complexity.

Observation 4: Since in SFN PDCCH, two TCI states are configured per CORESET, it is not clear whether the existing process of Rel-15/Rel-16 could provide satisfactory performance.

Observation 5: Transmitting PDCCH SFN manner could help to increase the reliability of the HST-SFN transmission.

Observation 6: For support of dynamic switching, there does not seem to be any viable reason for such requirement.

Observation 7: In either approach of Doppler estimation, there are some aspects that could impact the accuracy of the Doppler estimation.

Observation 8: In either approach of Doppler estimation, there are some aspects that could impact the uplink signalling overhead.

Observation 9: To have an accurate estimation of Doppler shift, a high rate transmission of TRS is only needed when a UE is in the vicinity of a TRP, other than that a UE can operate with a lower rate of TRS transmission.

Observation 10: The better choice of HST-SFN transmission scheme highly depends on the operating SNR region, location of the UE (including transmit beam orientation), and the number of transmit antennas at TRPs.

Proposal 1: Support DCI-based group indication to indicate a beam for a CORESET.

Proposal 2: Study zone-based configuration for TCI/QCL information to mitigate potential high signalling overhead.

Proposal 3: For implicit BFD RS configuration, support RS of CORESETs with only two TCI states, and consider use of RS of CORSETs with single TCI state as an optional UE feature.

Proposal 4: For explicit BFD RS configuration, support defining CSI-RS resource or SSB pairs as BFD RS.

Proposal 5: Support PDCCH SFN transmission for Scheme 2.

Proposal 6: RAN1 does not support dynamic switching between Scheme 1 (PDSCH) and Rel-16 scheme 1a.

Proposal 7: Study accuracy of Doppler measurement, delay cycle between Doppler measurement to the instant of pre-compensation, and enhance RS design if needed.

Proposal 8: Study overhead associated to transmission of SRS and CSI-report for Doppler measurement and report, respectively, and enhance related procedures if needed

Proposal 9: Support variable-rate TRS transmission for HST deployment scenario.

Proposal 10: Support both Schemes 1 and 2 for HST-SFN transmission.

# References

[1]    Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104b-e, April 12th – 20th, 2021

[2]	   3GPP, TR 38.913 Rel-15 ‘Study on Scenarios and Requirements for Next Generation Access Technologies’.