3GPP TSG RAN WG1 #106-e                                                                            R1-2106733

e-Meeting, August 16th – 27th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on cross-carrier scheduling and cross-carrier CSI-RS triggering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-08-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Change#1: Based on the previous RAN1 agreements shown below, in the following two cases, UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell. Case1: when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL Case2: when the DL DCI does not have the TCI field present Each case (Case1 and Case2) above is described via “when …” in the current spec. However, in CR R1-2102218 from RAN1#104-e, another “when …” is added in this paragraph. Three “when ….” are followed by “for both the cases”, it is not clear how to interpret the two cases. For example, it is not clear whether the first two “when …” is mapped to the first case or whether the last two “when …” is mapped to the second case. Agreements: (RAN1#97 meeting) - When PDSCH and its scheduling PDCCH are in the different CCs, if the PDCCH-to-PDSCH delay < Threshold-Sched-Offset timeDurationForQCL or if the TCI information is absent from the DCI, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell Change#2: In current specification, if the sub-carrier spacings of scheduling PDCCH and scheduled PDSCH are different, the UE gets timeDurationForQCL based on the sub-carrier spacing of the scheduled PDSCH. The UE gets default beam of scheduled PDSCH according to the determined timeDurationForQCL. But for AP-CSI-RS, there is no clarification about which sub-carrier spacing is used to get the beamSwitchTiming in case that the sub-carrier spacings of scheduling PDCCH and scheduled AP-CSI-RS are different. It will lead to some ambiguities to get the default beam of AP-CSI-RS. In addition, there is some ambiguity on whether ‘beamSwitchTiming + d ∙2μCSIRS/2μPDCCH’ or beamSwitchTiming is threshold reported by the UE. So we propose to add clarification about the sub-carrier spacing for getting the beamSwitchTiming, following the same approach as for timeDurationForQCL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change#1: Clarify the two cases when UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell. Change#2: Clarify about the sub-carrier spacing for getting the beamSwitchTiming, following the same approach as for timeDurationForQCL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Change#1: It is not clear what the two intended cases are when UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell. Change#2: It will lead to some ambiguities to get the default beam of AP-CSI-RS. In addition, there is some ambiguities on whether ‘beamSwitchTiming + d ∙2μCSIRS/2μPDCCH’ or beamSwitchTiming is threshold reported by the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5, 5.2.1.5.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The two changes are based on RAN1’s previous agreements and RAN1’s common understanding, which is expected to be no isolated impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

5.1.5	Antenna ports quasi co-location

<----------------------Unchanged parts are omitted--------------->

If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier:

-	The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero;

-	When the UE is configured with enableDefaultBeamForCCS, for both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell.

<----------------------Unchanged parts are omitted--------------->

5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

Beam switch timing:

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value beamSwitchTiming associated with the subcarrier spacing of the scheduled aperiodic CSI-RS is one of the values of {14, 28, 48} and enableBeamSwitchTiming is not provided, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameter repetition, or is smaller than beamSwitchTiming-r16 + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, wherein the reported value beamSwitchTiming-r16 is based on the subcarrier spacing of the scheduled aperiodic CSI-RS, when enableBeamSwitchTiming is provided and the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter repetition set to 'on', where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero

<----------------------Unchanged parts are omitted--------------->