3GPP TSG RAN WG1#106e       	        	                                   R1-2106789    E-meeting August.16th-27th, 2021

Agenda Item:					    8.1.1

Source:	Sony

Title:	Further enhancement on multi-beam operation

Document for:	Discussion

# Introduction

In RANP#86 meeting, the WID on multi-beam enhancement of Rel.17 was established as below

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

In this contribution, we present our considerations and thoughts on how to step forward.

# Unified TCI state framework

In Rel.15/16, the DL beam indication depends on the signaling of TCI states. It is configured in PDSCH-Config per BWP and can be viewed as a pool of DL Tx beams. The essential function of TCI state is to conduct QCL relation between two DL RSs, e.g. QCL-TypeA and/or QCL-TypeD between TRS and PDSCH DMRS.

As for UL, the beam indication is built on spatial relations which are separately configured and/or activated for UL channels or signals in a per PUCCH/SRS resource level. The spatial relation may contain an SRS resource or a DL RS as spatial source for determining UL Tx spatial filter, whereas a TCI state could only contain a DL RS (either SSB or CSI-RS) as a QCL source in Rel.15/16.

Next, we begin our discussion with source RS for conveying QCL-TypeD in the framework of unified TCI state.

## Source RS for unified TCI state

DL QCL

In RAN1#104e, the following agreement was achieved which implies that the basic QCL rule in Rel.16 can be reused for unified TCI state.

Agreement On Rel.17 unified TCI framework, the supported source/target QCL relations in the current TS38.214 V16.4.0 is supported for QCL Type D. - Note: This implies that the following source RS types for DL QCL (Type D, for DL RX spatial filter reference) information for DL UE-dedicated reception on PDSCH and all/subset of CORESETs are supported: - CSI-RS for beam management - CSI-RS for tracking - FFS (to be decided by RAN1#104bis-e): If SSB, CSI-RS for CSI, and/or SRS for BM are also supported as source RS types

However, in Rel.15/16, only DL RS can be referred for DL reception. With indicated DL RS, UE determines the corresponding DL Rx beam. In other words, UE has flexibility in implementing its Rx beam. But if SRS for BM is indicated as a source RS for determining its Rx beam, the UE should revert the Tx beam of the indicated SRS for BM as Rx beam. Therefore, the flexibility in implementing UE’s Rx beam would be impaired. Not to mention that once the UE experiences unpredicted rotation, mobility change, etc, the spatial Tx filter of indicated SRS for BM may not be suitable for DL reception anymore. When applying DL RS as source RS, this can be less than an issue, since UE could track the DL RS and refine its Rx beam accordingly.

- : For determining DL Rx beam, reverting the Tx beam of indicated SRS for BM as the DL Rx beam seems restrictive for UE implementation and may cause DL beam pair misalignment when UE rotates or moves.

In RAN1#105e, two additional constraints on using SRS for BM as DL QCL-TypeD source RS were brought up, i.e. a) TRS as QCL-TypeA as a companion RS and b) DL RS providing spatial relation for SRS for BM. For constraint a), it seems natural to indicate UE both QCL-TypeA RS (TRS) and QCL-TypeD (SRS, if supported) RS. However, for constraint b), we think if DL RS is configured as spatial relation for SRS (source RS in unified TCI state), then why not use the DL RS directly as DL QCL source? In addition, spatial relation is not applicable in FR1 in which UE is not configured with any spatial relation from UE capability reporting perspective. With above being said, we have

- : Following the Rel.15/16 QCL rules, RAN1 do not support SRS for BM as DL QCL-TypeD source RS.

In Rel.15/16, when UE enters the state of RRC connected, CSI-RS for CSI can serve as source RS for QCL-TypeA and TypeD for either PDCCH DMRS or PDSCH DMRS. In RAN1#104, RAN1 achieved consensus on reusing the existing QCL rule in Rel.16 as in above agreement. But it seems that the interpretation on Rel.16 QCL rule is incomplete, by missing CSI-RS for CSI, which was left to FFS.

In the framework of unified TCI state in Rel.17, we believe that CSI-RS for CSI can serve as QCL source RS without any issue and it’s time for RAN1 to fix it, at least for above agreement.

- : For backward compatibility with Rel.15/16 beam management, support CSI-RS for CSI as QCL source RS in unified TCI state.

## Target RS for unified TCI state

In RAN1#105e, RAN1 agreed that any DL RS that is valid target DL RS of Rel.15/16 TCI state can be configured as a target DL RS of Rel.17 DL TCI state. But whether CSI-RS and non-UE-dedicated CORESETs and PDSCH can share the same TCI state(s) with UE-dedicated CORESET(s) and PDSCH is still open as in following agreements.

Agreement On Rel.17 unified TCI framework, discuss and decide by RAN1#106-e (August 2021) - Whether each of the following DL RSs can share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC - CSI-RS resources for CSI - Some CSI-RS resources for BM, if so, which ones (e.g. aperiodic, repetition ‘ON’) - CSI-RS for tracking - DMRS(s) associated with non-UE-dedicated reception on PDSCH and all/subset of CORESETs - Whether some SRS resources or resource sets for BM can share the same indicated Rel-17 TCI state as dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC Agreement On Rel.17 unified TCI framework, for any DL RS that does not share the same indicated Rel-17 TCI state(s) as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, but can be configured as a target DL RS of a Rel-17 DL TCI (hence the Rel-17 DL TCI state pool), discuss and down-select by RAN1#106-e (August 2021) between the following two alternatives: - Alt1. Rel-15/16 TCI state update signaling/configuration mechanism(s) are reused to update/configure the Rel-17 TCI state - Alt2. Rel-17 TCI state update signaling/configuration mechanism(s) are used, e.g. with Rel-17 MAC-CE/DCI-based beam indication for Rel-17 joint/separate TCI Note: The DL RS includes CSI-RS and DMRS for PDSCH or PDCCH Note: For some channels/signals, only one of the above two alternatives may apply (to be discussed).

CSI-RS

Considering UE capability reporting, if UE supports tci-StatePDSCH, then DL control and data channel can share the same configured and activated TCI state(s). All these TCI states per BWP are configured in PDSCH-Config. Though the interpretation of tci-StatePDSCH doesn’t mention anything about CSI-RS, we find that the TCI state ID configured for periodic and aperiodic CSI-RS or activated for semi-persistent CSI-RS is also from the same pool of TCI states per DL BWP. No matter what purpose of the CSI-RS is. Hence from configuration aspects, we think both CSI-RS can also share the same configured unified TCI states as those configured for UE-dedicated PDCCH/PDSCH.

- : From unified TCI state configuration aspects, CSI-RS and UE-dedicated PDCCH and PDSCH can share the same pool of unified TCI states.

Given the same TCI state pool configured for both CSI-RS and UE-dedicated PDCCH/PDSCH, let’s next move to dynamic signaling, e.g. DCI-based beam indication. For CSI-RS for BM with repetition OFF, multiple DL Tx beams are to be measured by UE to select best K (<=4) beams. For this use case, it seems the indication of M (only M = 1 supported) beams is not efficient. But for other use cases, e.g. time/frequency tracking, CSI acquisition and UE Rx beam sweeping (CSI-RS for BM with repetition ON), assigning control and data channel beam to RS seems to inverse the whole beam management procedure. But that’s not correct interpretation in our view. For instance, if a UE identifies a good DL Tx beam and reports to NW, NW may chance the beams of TRS, CSI-RS for CSI or even CSI-RS for BM with repetition ON, and CORESET(s) and PDSCH via single DCI. That saves DL overhead. Whether to do it can be up to NW side. Then from spec perspective, we should allow NW to do so. Hence, we have

- : From dynamic signaling perspective, support CSI-RS to share the same indicated TCI state(s) of Rel.17 as UE-dedicated PDCCH and PDSCH.

SRS for BM

In RAN1#105e, the following agreement was made with an FFS whether SRS for BM can be a target RS of Rel.17 UL TCI state.

Agreement On Rel.17 unified TCI framework, - Any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) - Note: This does not imply that all such DL RSs necessarily share a same TCI state - The DL RS includes CSI-RS and DMRS for PDSCH or PDCCH - FFS: Whether some SRS resources or resource sets for BM can be configured as a target signal/channel of a Rel-17 UL TCI (hence the Rel-17 UL TCI state pool) - Note: This does not imply that DL and UL TCI state pools are separate or shared for separate DL/UL TCI (this issue is still TBD)

In Rel.15/16, SRS resources for UL Tx beam sweeping are typically not configured with any spatial relation. The reason lies in the fact that UE has to refer to the spatial relation for SRS transmission. Assuming the same SSBs configured for all SRS resources within a resource set, then UE cannot carry out UL beam sweeping, but only with single Tx beam referred from the configured SSB. But situation may change when comes to UL Rx beam sweeping (NW Rx beam refinement). To this end, NW configures a DL RS as spatial relation for SRS resources. Then UE transmits SRS resources with one single Tx beam to facilitate NW’s Rx beam refinement. From this sense, we believe

- : SRS for UL Rx beam sweeping can be a target RS of Rel.17 unified TCI state.

Similar reason as CSI-RS for BM, if some SRS resource, e.g. UL Rx beam sweeping can be supported as target RS of Rel.17 unified TCI state, we would like to allow it be indicated with the same TCI state as DG/CG PUSCH and dedicated PUCCH.

- : If SRS for BM is supported as target RS of Rel.17 unified TCI state, then some of it should be allowed to share the same dynamically indicated TCI state as DG/CG PUSCH and dedicated PUCCH.

## TCI state pooling

With the unified TCI states, what remains questionable is whether DL and UL share a common TCI state pool, e.g. tci-StatesToAddModList configured in PDSCH-Config, or whether NW configures a similar unified TCI state pool in PUSCH-Config for UL separately. In most recent meetings (from RAN1#102e to RAN1#104e), agreements were made as below

b) In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives for accommodating the case of separate beam indication for UL and DL - Alt1. Utilize the joint TCI to include references for both DL and UL beams - Alt2. Utilize two separate TCI states, one for DL and one for UL. The TCI state for the DL is the same as agreed in 1a. The TCI state for the UL can be newly introduced. - Alt 2-1: The UL TCI state is taken from the same pool of TCI states as the DL TCI state - Alt 2-2: The UL TCI state is taken from another pool of TCI states than the DL TCI state - Note: The resulting beam indication directly refers to the associated source RS(s) - FFS (RAN1#103-e): Details on extension to intra- and inter-band CA - Note: This may be related to issue 5 as well as other reasons for different TCIs such as network flexibility/scheduling Agreement On Rel-17 unified TCI framework: - A pool of joint DL/UL TCI state is used for joint DL/UL TCI state update (beam indication). - FFS: The pool for separate DL and UL TCI state update (beam indication) - Note: Here, TCI state pool refers to a pool configured via higher-layer (RRC) signaling FFS: Whether joint TCI may include UL specific parameter(s) such as UL PC/timing parameters, PL RS, panel-related indication, etc. and if it is included, it is used only for UL transmission of the DL and UL transmissions to which the joint TCI is applied

Our thought is to maintain flexibility of DL and UL beam indication as much as possible. We illustrate the methodology of TCI state pooling in Figure 1. Two separate TCI state pools can be configured via RRC signaling, meaning one for DL and the other one for UL. Each configured TCI state pool can be activated/deactivated by MAC CE and then indicated by DCI. In addition, we noticed that in separate DL/UL beam indication, only DL RS can be applied for DL beam indication, meaning SRS for UE to determine DL Rx beam is not supported yet. But both DL RS (SSB and CSI-RS) and UL RS (SRS) can be configured into UL TCI state. From this sense, it is reasonable to separately configured TCI state pools for DL and UL.

- : Unified TCI state framework for both DL and UL

- : To achieve the flexibility of DL and UL beam updating/indication, support separately configured unified TCI state pools per BWP/CC.

## Common beam operation for CA

Intra-band CA

In RAN1#105e, the common beam operation for intra-band CA was progressed with following working assumption.

Working Assumption For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP /CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC

For TCI states pooling for CA, there are two options which can be simply denoted as 1) single shared TCI state pool among CCs; 2) per individual CC TCI state pooling. Though option 1) can reduce RRC signaling and UE storage on each configured TCI state pool, legacy TCI state configuration method, i.e. per BWP/CC in Rel.15/16 was acceptable as well. In our understanding, both RRC configuration approaches are listed as working assumption.

Regarding QCL-TypeA/TypeD source RS, we observe that one common TCI state ID points to 1 or 2 QCL-Info(s) which contains cell-specific 1 or 2 DL RS(s). For conveying QCL-TypeD, when BWP/CC ID is absent, UE refers to one DL RS in target CC with the same resource ID as the RS in reference BWP/CC. And of course, the BWP/CC ID can be present in QCL-Info, then UE can refer the indicated DL RS from the reference BWP/CC. We believe that’s the design facilitate most use cases one may have in mind, so we would like to

- : Confirm the working assumption on RRC-configured TCI state pool(s) and methods on QCL-TypeA/D source RS indication when BWP/CC ID is present or absent.

Inter-band CA

For the case of multiple CCs in different bands, i.e., inter-band CCs, there are, as well, opportunities to reduce the burden of signaling overhead. With no optimizations, multiple beam management procedures may be required in the inter-band case to cater for multiple CCs, BWPs and UE panels, as illustrated for the intra-band case. This has several adverse effects:

- Time-frequency resources need to be reserved for initial beam establishment and beam refinement on multiple CCs.

- The burden of signaling overhead, e.g., to configure suitable TCI states, increases with the number of CCs and UE panels.

- The UE burden and energy consumption associated with additional beam management procedures scales up with the number of CCs, UE panels and BWPs.

It is possible, however, to reuse beam management procedures across bands. For instance, when TRPs of different bands (e.g., 28 GHz and 39 GHz bands) are co-located, signals between NW and UE tend to propagate through the same physical directions. In this case, a beam sweep procedure common to multiple CCs may be feasible, including the reuse of TCI states. To harbor opportunities for optimizations and also considering above agreement, we make the following proposal

- : For inter-band CAs, support common beam operation for data/control, DL/UL, multiple BWPs, multiple TRPs and multiple UE panels.

The feasibility of the above proposal depends, of course, on the UE ability to simultaneously produce beams in different bands which roughly point in the same directions. It is apparent that the ability to do so will depend on the UE implementation. For example, some UEs might not be able to simultaneously control directions of beams in different bands, or they might be able to do so for certain band combinations, but not for others. Therefore, the achievable savings depend on certain UE capabilities. RAN4 has been discussing a potential new UE capability on independent beam management (IBM) and/or common beam management (CBM) for inter band CA operation. Note that if such capability would be eventually agreed, Proposal 8 supporting a common beam management procedure states across bands would then become necessary.

- : The UE capability related to simultaneously steering beams in the same direction belonging to CCs in different bands should be reported.

Furthermore, RAN4 is discussing possible test setups for inter-band CCs wherein the beams of different CCs may be configured to use different polarization directions. In addition, some UEs may only be able to have independent beam control on different CCs with different polarizations due to limitations from the RF implementation. Therefore, when optimizing beam indication across bands, polarization needs to be taken into account. In particular, the polarization property of beams shall not impair optimizations of cross CC beam indication while, at the same time, it should be possible to select orthogonal polarizations for beams in different bands pointing in the same/different directions.

- : For the optimization of beam indication across bands, the polarization property of beams should be considered.

# L1/L2 inter-cell mobility

In Rel.15/16 NR, the intra-cell beam level mobility was designed and specified. In the WID of Rel.17, RAN plans to support the inter-cell mobility with as less latency as possible, i.e. by using the L1/L2 signaling. The motivation for introducing the L1/L2-centric inter-cell mobility is to address mobility issue associated with moderate- to high-speed UE. When a UE moves across the border of its SpCell and enters the coverage of a non-serving cell (NSC), the benefit comes from early beam-level activation/indication from NSC before any RRC signaling involved (if needed).

## DL measurement for NSC RS

In RAN1#105e, the following agreement on DL measurement was achieved

Agreement On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, decide by RAN1#106-e whether to support the following RS types as measurement RS or not: - CSI-RS for mobility/RRM associated with a non-serving cell - CSI-RS for BM associated with a non-serving cell - CSI-RS for tracking associated with a non-serving cell Note: If another beam metric other than L1-RSRP is supported (e.g. L3-RSRP is still FFS), the above also applies Note: An RS is associated with a non-serving cell means that it is either configured for a non-serving cell or configured for a serving cell but is QCLed with a non-serving cell SSB

In addition to SSB from NSC, a UE normally would be configured with CSI-RS for mobility for RRM purpose. In comparison with SSB, CSI-RS for mobility can be allowed with larger bandwidth in frequency domain and more density in time domain for more accurate measurement results. In addition, to get time-domain filtered results in L3, UE first needs to produce measurement results of mobility RS in L1 which can be reused for L1/L2 inter-cell mobility. Furthermore, CSI-RS for mobility can be configured with an SSB from the same NSC as its QCL-TypeD source RS. In such case, UE can track back the QCL chain of this non-serving cell and retune its proper DL Rx beam accordingly. Therefore, we failed to see a reason to refuse CSI-RS for mobility as measurement RS from NSC

- : Support CSI-RS for mobility as measurement RS for L1/L2 inter-cell mobility.

## Beam reporting for NSC RS

In RAN1#105e, beam reporting in one reporting instance has been discussed and finally agreed in following agreement.

Agreement On Rel.17 L1-RSRP multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - Support at least K=4, where K is defined as the number of beams associated at least with non-serving cell(s) reported in a single CSI reporting instance - The maximum value of supported K is a UE capability - K is configured by NW based on the UE capability - FFS: The support of K=8 and 16 - For K>4, the maximum number of beams associated with one cell is 4 - FFS: Support L1-based event-driven reporting based on Rel-16 SCell BFR framework or analogous to L3-based event-driven reporting, including the definition of L1-based event, if needed Note: If another beam metric other than L1-RSRP is supported (e.g. L3-RSRP is still FFS), the above also applies

In Rel.15/16, higher layer events are defined in TS 38.331 for L3 inter-cell mobility. When the triggering condition of those L3 events satisfied, UE reports to NW and then possibly triggers L3 handover procedure. Note that from time-domain perspective, such reporting is not periodic/semi-persistent/aperiodic, but UE triggered. From latency perspective, L3 filtered RSRP reporting normally takes more time than the L1-RSRP reporting due to the nature of time domain filtering. On the other hand, the stability of L3-RSRP outperforms that of L1-RSRP in making handover decision. But since RAN1 manages to set up L1/L2 inter-cell mobility, we believe L1 metric would be more suitable.

For L1/L2 inter-cell mobility, it seems straight-forward to define L1 events by using L1 metric, e.g. L1-RSRP or L1-SINR. For example, one L1 event could be defined as when the quality of NSC is better than current NS with an offset. Once such newly defined L1 event happens, UE triggers corresponding L1 reporting which contains RS from NSC and corresponding L1 metric.

- : Define L1-metric based events for L1/L2 inter-cell mobility and support UE triggered beam reporting based on L1 events.

## Beam activation/indication

For L1/L2 inter-cell mobility, when it’s necessary, NW sends beam-related signaling to UE. The signaling of beam activation in L2 MAC CE and/or beam indication in L1 DCI conveys unified TCI state(s) which implies serving cell change to UE. In RAN1#105e, the following working assumption was achieved with a few of FFS points.

Working Assumption On Rel.17 beam indication enhancements for L1/L2-centric inter-cell mobility, support the following: - Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) - FFS (to be decided in RAN1#106-e): Whether this also applies to PDSCH/PUSCH associated with UE-dedicated CORESETs only or additional target channels (e.g. UE-dedicated PDCCH/PUCCH) - FFS: Whether the above is supported only for joint TCI, or both joint TCI and separate DL/UL TCI (including that, if separate DL/UL TCI is supported, the DL TCI and UL TCI associated with a same cell) - FFS: Whether to support activation of TCI states for more than one cells simultaneously - FFS: Whether down-selection between MAC-CE only based and MAC-CE+DCI-based beam indication scheme is necessary - The DL QCL and UL spatial relation rules already agreed for intra-cell scenario - FFS: The use of SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference for UE-dedicated PDSCH - FFS (to be decided in RAN1#106-e): Whether this also applies to UE-dedicated PDCCH - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel - FFS (to be decided in RAN1#106-e): Whether SSB associated with a physical cell ID different from that of the serving cell can also be used as a direct QCL reference (source RS) for UE-dedicated PDCCH/PDSCH

From signaling perspective, the beam indication mechanism can follow the same rule of Rel.15/16, i.e. MAC CE + DCI. Particularly, serving cell sends Rel.17 MAC CE to activate 8 or more unified TCI states which contains RS only from non-serving cell. Here we would like to put a constraint that all the RSs in unified TCI states come from the same NSC. That implicitly tells UE to be prepared for serving cell change and UE doesn’t need to maintain its reception from current serving cell. Somehow this operation could reduce UE complexity in DL reception. After ACK feedback of the MAC CE, NW may send DL DCI without assignment to dynamically indicate DL Tx beam(s) from NSC.

- : Support Rel.17 MAC CE-based and DCI-based beam indication mechanism, in which MAC CE activates unified TCI states containing RS only from the same non-serving cell.

For SSB from NSC, it is used as DL measurement and reporting RS. But due to its nature (narrow channel bandwidth, 20 PRBs), we think it would be safe to apply such SSB as indirect QCL source.

- : Support SSB (from NSC) only as indirect QCL source for PDSCH DMRS and PDCCH DMRS.

In section 3.2, we discuss the possibility to include CSI-RS for mobility as measurement RS. If newly defined L1 event occurs, CSI-RS for mobility could be reported by UE in existing L1 reporting. Considering the benefits, we think it would be reasonable to support such feature in UE’s measurement and reporting. If so, it also seems nature to indicate CSI-RS for mobility as source RS in TCI state along with other CSI-RS, e.g. CSI-RS for BM or for tracking.

- : Support CSI-RS for mobility as source RS for conducting QCL info from non-serving cell.

# Dynamic TCI state update/indication

In RAN1#102e, Item a) of Issue 3 was identified as following

[Issue 3] For Rel.17 NR FeMIMO, on dynamic TCI state update signaling medium: a) In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives: - Alt1. DCI - Alt2. MAC CE - Note: Combination between DCI and MAC CE for, e.g. different use cases or control information partitioning can also be considered - Note: The study should consider factors such as feasibility for pertinent use cases, performance (based on at least the agreed EVM), overhead (including PDCCH capacity), latency, flexibility, reliability including the support of retransmission - Note: This may be related to outcome of issue 1a), 1b), and 6a)

During RAN1#103e, the DCI based TCI state updating was discussed when considering the impacts on reliability, overhead, and latency, and finally supported. In this section, we would like to discuss pending issues on DCI based approach.

## DCI formats for signaling TCI state(s)

In RAN1#103e, RAN1 agreed to reuse DL DCI format 1_1 and 1_2 with DL assignment for beam indication and leave other possibility open for further discussion. In RAN1#104e, this DCI format down selection was formulated with more concrete details on its HARQ-ACK, beam application timing, etc.

In RAN1#104bis-e, DCI format 1_1 and 1_2 without DL assignment (Alt1) was supported and all related design details, including HARQ mechanism, RNTI, DCI field combinations, were almost done. However, there were a few points FFS, for example how to handle the case when only UL data is available.

Agreement On the Rel.17 DCI-based beam indication, in RAN1#104bis-e, down-select at least one of the following alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e): - Alt0: No additional DCI format is supported - Alt1: DCI formats 1_1 and 1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy - FFS : How to identify DCI formats 1_1/1_2 used for beam indication only (not for scheduling a PDSCH reception, not indicating a SPS PDSCH release, or not indicating SCell dormancy), considering impacts on PDCCH coverage and scheduling mechanism - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt2: Dedicated DCI format other than 1_1/1_2 without DL assignment, applicable for joint TCI as well as separate DL/UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS, based on DCI indicating SCell dormancy - FFS : If the format is based on an existing DCI format, how to identify the DCI format used for beam indication only - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt3: UL-related DCI formats 0_1/0_2 with UL grant, applicable only for UL-only TCI of separate DL /UL TCI

DCI 0_1/0_2 with UL grant

As for UL scheduling, the function of UL beam indication was designed to be dependent on SRS resource in Rel.15/16. Specifically, the SRI in DCI 0_1/0_2 indicates SRS resource(s) which was/were configured in SRS resource set with usage set as either ‘codebook’ or ‘non-codebook’. Besides UL Tx beam(s), SRI also couples with other UL aspects, e.g. UL power control, precoder and rank selection, which cannot be provided by TCI state in DL DCI. In addition, when UL scheduling comes, the other fields (rather than TCI state) in DL DCI are not identical with that of UL scheduling DCI. Hence, it seems not feasible to only apply DL DCI to convey TCI state(s) when separate UL beam indication is needed. Therefore, we have

- : The multiple roles of SRI in UL DCI are very important to UL scheduling, which could not be fully replaced by the TCI state.

If DL DCI is the only signaling medium to indicate UL Tx beam, then the UL transmission depends on two DCIs, one for UL beam indication and the other one for UL grant. In other words, at most two DCIs should be signaled to UE for one PUSCH transmission. Unless the DCI for UL beam indication lasts for a few of slots, it would be not efficient when compared with UL scheduling DCI 0_1/0_2, hence we still prefer

- : Support (Alt.3) UL DCI format 0_1/0_2 with UL grant to indicate UL Tx beam by joint DL/UL TCI state or UL-only TCI state.

GC-DCI

In RAN1#103e, the possibility of using group-common DCI to carry TCI state(s) for a group of UE has been discussed and it is now for further study. From signaling perspective, assuming K users in a group and instead of sending K UE-dedicated DCI to each of K UEs, NW only need to send one GC-DCI in the best case. Hence, a certain amount of DL overhead (i.e. (K-1)/K) can be saved, therefore alleviating congestion for DL control channel.

In our view, for some specific deployment scenarios, e.g. high-speed train or vehicles in highway in which a group of UEs are located in close proximity. Though due to different UE rotation or UE position the DL Rx beams can be different from UE to UE, the DL Tx beam(s) from NW seems common for the group of UEs. Therefore, as inspired by GC-PDCCH designed in Rel.15/16, it seems reasonable to apply GC-DCI to carry DL TCI state to the group of UEs.

Agreement On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework: - Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states - The existing DCI formats 1_1 and 1_2 are reused for beam indication - Support a mechanism for UE to acknowledge successful decoding of beam indication - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI - FFS: Whether any additional specification support is needed - Support activation of one or more TCI states via MAC CE analogous to Rel.15/16: - At least for the single activated TCI state, the activated TCI state is applied - The content for the MAC CE is determined based on the outcome of issue 1 - FFS: If supported, default TCI state when more than one TCI states are activated by MAC CE - Note: There is no implications on the support of single TRP or multi-TRP - FFS: Additional enhancement such as L1-based beam indication with group-common DCI - FFS: Whether the Rel.17 beam indication can also apply to beam indication for single channel (e.g. PDSCH only, single CORESET) or a subset of channels - FFS: Additional details on extending the support of L1-based beam indication when separate UL (from DL) common beam indication is configured

Moreover, this GC-DCI conveying DL TCI states should be identifiable by UE. Different from repurposing DL DCI with assignment, there are not many different fields in GC-DCI, therefore special combinations of some fields would not work. But one way to do so is to scramble the GC-PDCCH with a new RNTI.

- : Study standard impacts of introducing group common DCI to indicate/update DL TCI state(s) for a group of UEs.

## HARQ mechanism for DCI

It does matter that both NW and UE synchronously maintain the same knowledge of Tx-Rx beam pair. As for DCI based beam indication/updating, NW should know whether UE decodes the DCI carrying TCI state(s) successfully or not. Consequently, the HARQ mechanism for this type of DCI should be specified.

DCI 0_1/0_2 with UL grant

For DCI format 0_1/0_2 with UL grant, there is no explicit HARQ procedure specified, since NW by nature is aware of whether PUSCH has been decoded successfully or not as recipient. NW only sends UL grant again with same HARQ process ID and un-toggled NDI to UE for retransmitting PUSCH, if NW failed in decoding previously transmitted PUSCH or UE failed in decoding UL grant DCI. It seems straight forward to reuse this UL implicit HARQ mechanism for DCI 0_1/0_2 with UL grant.

Let’s below take the examples in Figure 2. At the beginning, NW sends UL grant DCI to schedule PUSCH transmission. If UL DCI has been successfully decoded by a UE, the UE would transmit PUSCH accordingly. No matter PUSCH is decoded successfully or not at NW corresponding to case a) and b), NW can assume that the UE has decoded UL grant DCI. If no PUSCH transmitted from UE corresponding to case c), NW infers that UE did not decode the UL grant DCI successfully. Then it would be up to NW to re-schedule the UE with UL grant DCI carrying unified TCI state(s). In summary, even for UL grant DCI (no explicit HARQ feedback), the conventional implicit HARQ mechanism can be reused or slightly enhanced to support UL grant DCI carrying TCI state(s).

- : HARQ procedure for dynamic granted PUSCH

- : For UL grant DCI carrying TCI state(s), if supported, then reuse the implicit HARQ mechanism of PUSCH for UL grant DCI.

GC-DCI

For GC-DCI carrying TCI state(s) for a group of UEs, different from other GC-DCI, e.g. containing SFI or TPC, HARQ mechanism is needed anyway to maintain the alignment of beam pair between NW and UE.

- : For GC-DCI carrying unified TCI state(s) for a group of UEs, if supported, study its HARQ mechanism for each UE within the indicated group.

# Uplink multi-panel fast operation

Regarding the UL panel-specific beam selection, certain progress was made until RAN1#98 in Rel.16 and then one conclusion came that this feature cannot be supported in Rel.16 due to no consensus in RAN1. In Rel.17, we would like to continue to discuss, design and specify (if necessary) the feature of fast UL panel selection. In what follows, we would like to touch a fundamental issue for UL fast panel selection based on unified TCI state, i.e. association between panel and TCI state, then discuss how to update such association in a dynamic manner.

## Association with UE panels

Intuitively for designing a panel-specific UL transmission, one may create a panel ID for each panel entity. Afterwards UE reports its panel-related information to NW associated with such panel ID. Note that such ID may not directly reflect UE’s physical antenna panel implementation, it could map to proper terminology in specification. One good example in our view would be SRS resource set with usage as ‘Beam Management’. From Rel.15, UE report its capability on how many SRS resource set(s) it supports for simultaneous UL beam sweeping. Such capability can be viewed by NW as how many UL antenna panels deployed at this UE.

Hence, we would like to keep our proposal to get align with the highlighted part in following agreement made in RAN1#104e.

Agreement On Rel.17 enhancement for facilitating fast uplink panel selection, - Rel.17 TCI state update (based on MAC CE + DCI along with the necessary TCI state activation, or MAC CE only) can be used for UE UL panel selection: - FFS : Whether specification support for this feature is necessary and if so the details of such spec support, e.g. - Additional spec support in TCI state definition to accommodate UL panel - UE reporting to facilitate UL panel selection - UE reporting, e.g. panel-specific report, including UE -panel state, e.g. inactive, active for DL /UL measurement, active for DL reception only, active for UL transmission, or other combination(s) of UE -panel states - Support for linking or association of UE panels with CSI-RS/SSB resources or resource sets, SRS resource sets, and/or PUCCH resource groups, etc.

- : Support to link or associate UL panels with SRS resource sets for UL fast panel selection.

However, as some UE vendors pointed out in a few of previous meetings, the specification should not be designed in a way that forces UE to expose its antenna implementation. It implies that such explicit signaling on panel ID would be not be acceptable. In addition, when NR system operates based on DL beam sweeping and beam correspondence, there could be no SRS for BM configured for UE. With this compromised understanding, RAN1 achieved following agreements on implicit method for panel-specific selection.

Agreement On Rel.17 enhancement for facilitating fast uplink panel selection, for discussion purpose, a panel entity corresponds to one or more RS resources: - For CSI/beam reporting, the RS resource is an RS associated with measurement and/or reporting - For beam indication, the RS resource is a source RS for UL TX spatial filter information - Note: For one RS resource, the corresponding panel entity may vary and is controlled by the UE, and whether/how to maintain a common understanding between gNB and UE can be further discussed/decided - Note: The above does not preclude possibility that an RS resource can be mapped to multiple panels - Note: The one or more RS resources may correspond to one or more RS resource set(s) depending on further discussion/decision - Note: Specification should not be designed in such a way that the UE is required to disclose its antenna implementation

The implicit logic chain of associating UE panel(s) can be depicted as follows. In beam measurement/reporting procedure, a UE measures DL RS resources with its antenna panel(s) and reports specific DL RS resource(s) which can be deemed as associated with specific UE panel(s). We have to note that those selected DL RS could be already configured in joint TCI states or UL-only TCI states. Then in beam indication, UE can be indicated with such TCI state(s) to prepare UL transmission with specific UL panel. In summary, the implicit approach of conducting panel-specific selection is to link or associate UE panel(s) with joint or UL-only TCI state via DL RS which is measured by specific UE panel(s) and then reported.

To this end, RAN1 may need to check current beam reporting format on whether panel status and/or panel ID (or let’s say a Tag) should be reported too.

- : Support to implicitly link or associate UE panel(s) with joint or UL-only TCI state(s) via beam reporting.

Furthermore, in RAN1#104bis-e, 3 options were listed to associate a panel entity with potential terminology in spec, such as reported CSI-RS and/or SSB resource, or a new panel ID within CSI/beam reports. From the discussion above, both Opt1-1 and Opt1-2 can provide such linking function.

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, for CSI/beam measurement/ reporting, down select and/or modify from the following candidates: - Opt1-1: A panel entity corresponds to a reported CSI-RS and/or SSB resource index in a beam reporting instance - The correspondence between a panel entity and a reported CSI-RS and/or SSB resource index is informed to NW - FFS: How to inform through CSI/beam reporting framework - FFS: Detailed design of the correspondence including the conveyed information - Note: the correspondence between a CSI-RS and/or SSB resource index and a panel entity is determined by the UE (analogous to Rel-15/16) - Opt1-2: A panel entity is referring to a new panel ID within CSI/beam reports - FFS: Detailed design of the new panel ID including the information conveyed by the new panel ID - Note: The association between the new panel ID and the panel entity is determined by the UE - Opt1-3: No additional specification support - The duration in which the above panel entity reference is valid and the respective setting are FFS Note: “panel entity” is only used for discussion purpose

- : Support to link a panel entity with a new panel ID (Opt1-2, 1st priority) or with CSI-RS and/or SSB resource in beam reporting (Opt1-1, 2nd priority).

## UE panel status update

In RAN1#103e, the UE-initiated UL panel control was agreed as

Agreement In Rel.17 enhancement for facilitating fast uplink panel selection, UE-initiated UL panel selection/activation are supported: - FFS: Whether NW-initiated panel selection/activation is also supported - FFS: Whether specification support for this feature is necessary and if so the details of such spec support.

Considering power saving purpose or MPE issue, a UE may turn off or let’s say deactivate some of its antenna panels. If NW schedules UL transmission involving such inactive antenna panels at UE, the UE may consume more time to turn on the inactive antenna panel(s) than the scheduled PDCCH to PUSCH time gap or for aperiodically triggered SRS. Consequently, the scheduled UL transmission cannot be fulfilled in time by UE which is surely undesirable either from NW side or UE side. In other words, both NW and UE should be on the same page on the ‘ON/OFF’ status of UE panels. Specifically, when a UE would turn off/on some UL panel(s), it should let NW know in a synchronous manner.

- : For scheduled UL transmission, it matters that both NW and UE should be mutually aware of status of UE panel(s) in a synchronous manner.

- : UE panel status updating due to rotation

In Figure 3, one may observe that the UE equipped with multiple panels rotates by certain angle. From L1 metric wise, such rotation may result in association change from CSI-RS resource #3 linking “green” panel to CSI-RS resource #4 linking “red” panel. When detected necessary association modification, it would be better for UE to report it to NW either via UCI (similar as beam reporting) or MAC CE.

- : Support UE-initiated UL panel(s) linking/association status update via UCI or MAC CE.

A UE has been granted to charge its antenna panel(s) for UL transmission. Not only because UE antenna panels are UE implementation, but the reason also lies in the fact that only UE can observe its battery lifetime and then determine to turn off some panel(s) to increase standby time, or detect potential panel-specific extensive RF emission to human tissue and turn off specific panel(s) to avoid MPE. This kind of local information at UE cannot be easily known by NW in a timely manner.

- : Do not support NW-initiated panel selection/activation, unless the benefits for supporting this feature can be fully justified.

# MPE mitigation

In Rel.16, RAN1 received an LS from RAN4 to handle the MPE issue to comply with regulatory RF exposure limits. In RAN4, two approaches to handle the MPE issue have been agreed. One approach is to directly back off UE Tx power from the configured maximum transmit power by a quantity P-MPR. The other one is to control the ratio of UL transmission based on UE capability, i.e. maxUplinkDutyCycle.

Though aforementioned RAN4 solutions in Rel.16 have a touch on the MPE issue, they came with a price of UL coverage deterioration, especially at FR2. For instance, when power back-off applied by a UE, the UE may need to lower its maximum power by, i.e.10dB to satisfy RF exposure limits. This can possibly result in dissatisfactory measurements of an UL Tx beam at NW side, though the corresponding DL beam selected still brings high L1-RSRP at UE.

- : Due to UL Tx power back-off, the Rel.15/16 beam management established on beam correspondence may result in suboptimal UL Tx beam selection.

As a further observation, we notice that the MPE issue only affects UL. Therefore, it is possible to only switch the UL beam or panel to overcome the UL coverage limitation caused by the MPE restriction. Depending on whether beam correspondence holds or not at the UE side, the UE may be scheduled with an UL Tx beam which is not the corresponding DL Rx beam. From Rel.15/16 spec,  different UL Tx beam and DL Rx beam, as mentioned above, is surely not restricted, thanks to two parallel beam management mechanisms, i.e. TCI states for DL and SpatialRelationInfo for UL in Rel.15/16 and separate UL/DL TCI states in Rel.17. So we have following observation

- : Depending on whether beam correspondence holds or not at UE side, the UE can be scheduled with an UL Tx beam which is not the corresponding DL Rx beam due to UL MPE issue.

As early as in Rel.16 (RAN1#97), the MPE issue was brought up to RAN1. But due to lack of consensus in RAN1#98, the discussion on MPE was concluded unfortunately. In Rel.17, the WID formally includes the MPE issue and Issue 5 was identified to handle it in the way below

- [Issue 5] For Rel.17 NR FeMIMO, on MPE mitigation (that is, minimizing the UL coverage loss due to the UE having to meet the MPE regulation), in RAN1#103-e: - If needed, identify candidate solutions to be down-selected in future meeting(s). The following sub-categories can be used: - CAT0. The need for specification support for MPE event detection and, if needed, candidate solutions - CAT1. The need for UE reporting associated with an MPE and/or a potential/anticipated MPE event if the UE selects a certain UL spatial resource, e.g., corresponding to DL or UL RS - CAT2. The need for NW signaling in response to the reported MPE event (taking into account issue 1) and UE behavior after receiving the NW signalling - Note: RAN4 has agreed to specify P-MPR reporting (cf. CRs for TS 38.101/102/133) which can be used as a baseline scheme for further enhancement - Note: This may be related to outcome of issue 4b) - Companies are encouraged to submit evaluation results based on the agreed EVM to justify the benefits of the candidate solutions

## MPE-aware beam reporting

In RAN1#104bis-e, to facilitate MPE mitigation, all options were thoroughly discussed and listed to be down selected in RAN1#105e.

Agreement On Rel.17 enhancements to facilitate MPE mitigation, in RAN1#105-e, further discuss to down-select at least one or combine from the following options: - Opt 1A. {Rel.16 P-MPR based (beam/panel-level)} + Virtual PHR or a modified version - The modified version may be associated with each activated UL TCI or, if applicable, joint TCI, or associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) from candidate pool, if reported. - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - FFS: how to determine the virtual PHR or the modified version. - Opt 1D. {Rel.16 P-MPR based (beam/panel-level)} - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - Opt 2A. {SSBRI(s)/CRI(s) and/or panel indication} + L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR] - FFS: Whether/how to enhance existing beam reporting format to support Option 2A - FFS: When multiple SSBRIs/CRIs and their corresponding metrics are reported in the same reporting instance, whether to allow mixture between the SSBRI(s)/CRI(s)) intended for MPE mitigation and for DL beam reporting - FFS: Whether the reporting is UE-initiated (event-driven) and/or NW-initiated - FFS: If Opt2A is selected and there is no consensus on a modified L1-RSRP definition, at least the Rel-15 L1-RSRP definition is reused and virtual PHR may be added FFS: If gNB acknowledges MPE report from UE for UE-initiated (event-driven) reporting FFS: If differential report is supported when multiple UL beams are reported in the same report

Regarding the progress for Issue 4, UE panel entity can be either associated with DL RS resource in CSI/beam reporting. Hence, we think the same rule could also apply to Issue 5. There is no need for UE to report panel, if the report contains SSBRI(s)/CRI(s) which are associated with UE panel already.

- : To facilitate MPE mitigation, support UE reports (a subset of Opt 2A) SSBRI(s)/CRI(s) + modified L1-RSRP that accounts for MPE effect.

For some other NW, SRS based UL beam sweeping is necessary tool at NW to determine UL Tx beam. When a UE carries out this UL beam sweeping procedure, the UE should keep the MPE issue in mind and therefore apply power back-off on a per Tx beam basis. More specifically, different SRS resources within the SRS resource set with usage set to ‘beam management’ may have different P-MPR values. SRS resources with their Tx beams experiencing the MPE issue should apply a power back-off value, e.g. 10dB, when compared to other SRS resources not experiencing MPE. Therefore, considering the MPE impact, NW may select the ‘best’ SRS resource for UL transmission.

- : For UL beam sweeping, when taking MPE into account, UE should transmit SRS resource(s) by certain power back-off, if needed.

## Beam/panel-specific P-MPR reporting

The UL transmit power is strictly limited by regulators’ limitations on MPE. Therefore, a power back off (P-MPR) can be used by UEs to meet the MPE limitation defined in the Rel-15 RAN4 specification. However, since the P-MPR value can be significant and unpredictable in real life, it will severely reduce the UL coverage and potentially result in radio link failures (RLFs). In Rel-16, enhancements of the MPE solution target avoiding RLFs and connection releases caused by FR2 UE RF exposure compliance. In RAN4#93, it was agreed that the P-MPR shall be indicated to the network through MAC-CE [2].

For Rel-17 multiple-beam operation with MPE impact, it is reasonable for RAN1 to consider supporting the existing RAN4 solutions. As we mentioned earlier, RAN4 has agreed in RAN4#93 to indicate the P-MPR value to the network, so that the network can estimate a reasonable UL duty cycle for the UE to avoid potential radio link failure and UL coverage loss. From RAN1 aspect, it can further associate the P-MPR value with spatial relation, e.g., reporting P-MPR of multiple candidate UL beams, so that the gNB can select the optimal beam pair for the uplink transmission.

For multi-panel operation, a straightforward way is to let the UE report a P-MPR value associated to a UE panel. Please note here that the MPE level is related to the physical antenna panels. Therefore, a panel ID likely needs to be related to a single physical antenna panel. If the P-MPR of each UE-panel is indicated to the network, the gNB can specify which panel to use and further optimize of the UL scheduling, resource allocation, etc.

- : Support UE reports P-MPR value(s) associated with panel(s) (Opt 1D) to mitigate UL coverage loss due to MPE.

## UL beam/panel switching to avoid MPE

Enhancements from RAN4 are limited to transmitted power control and uplink scheduling. However, even for the same antenna panel, the exposure level can be different between different beams, and even larger differences can be observed between different panels. An example from [3] is shown below in Figure 4. The free space power density of three different array designs averaged over 4 cm2 is plotted: For the same array, the power density level varies for each spatial filter (beam). For different array designs, the difference is even more pronounced. As a result, one can expect the actual max UL duty cycle, and P-MPR to be different for each beam in a real implementation. Therefore, we believe that beam/panel switching can be an effective solution and enhancement to mitigating the UL coverage loss due to MPE.

(a)

(b)

- : The view of simulation models of 28 GHz UE mock-ups of three different 4×1 arrays, and phase excitation scheme with a progressive phase shift βi (b) maximum 4 cm 2-averaged incident power density when d (distance to the antenna array)  and βi vary for 28 GHz 4×1 array.

- : Beam/panel switching can be an effective solution to mitigate the UL coverage loss due to MPE.

In addition to the efforts on enhancing MPE related beam reporting and P-MPR reporting, another way to avoid MPE is to switch from a beam which exceeds the maximum exposure limit. For instance, when a UE is served by two TRPs, if one UL Tx beam with one TRP shall back-off Tx power due to MPE, this UE could choose the other TRP for UL transmission. In such case it would be beneficial to study the UE triggered UL Tx beam change. Hence, we have

- : To avoid UL MPE issue, it would be beneficial for RAN1 to study and, if necessary, specify the mechanism of UE triggered UL beam/panel switch.

# Definition on UE antennas

## Antenna panel definition

During RAN1#104b_e meeting it was agreed to further study possible need for gNB awareness of UE panel configuration.

Because different UE vendors may implement UE antenna panels in different ways, the conventional panel definition, a physical antenna array with either single or dual polarization, doesn’t cover all aspects for panel-specific beam selection.

Agreement On Rel.17 enhancements for MPUE, investigate and, if needed, specify the following: - UE reporting of panel-specific information as a UE capability, for example: - Information related to the total number of DL/UL panel entities - Information related to the number of (max) antenna ports/layers per panel entity - Information related to the maximum number of resources per panel entity for SRS BM - Information related to panel selection delay - Information related to panel activation delay - UE reporting information related to minimal activation/selection delay for a panel based on L1 or L2 signaling - UE reporting of panel activation/selection status of a panel entity, e.g. active state for both DL and UL, or active state for DL only - FFS: details of this information (e.g. minimal activation/selection delay for a panel) and signaling (e.g. L1 or L2 signaling) - UE-reported information in MPE report (if supported) is used to indicate the minimal activation/selection delay and panel activation/selection status Note: above ‘panel entity’ is a logical entity and how to map physical panels to the logical entities is up to UE implementation. Note: This will depend on the final outcome of whether specification support for UE-initiated panel activation/selection is agreed.

The “if needed” reflect that there is no consensus among companies regarding the need for gNB awareness of UE panel configuration. Some companies expressed that it was not clear how a gNB could use this information. It is our understanding that current networks are fully operational, while in practice, being far from optimal from a UE beam management (BM) perspective. With progress in technology (e.g. number of layers supported, beam refinement steps, 1D arrays vs. 2D arrays, etc.) and increase in the UE variety (e.g. introduction of red-cap UEs), UEs with different capabilities will be more common. Awareness of the UE-panel properties at gNB side is a first step toward enhanced BM algorithms.

- : Awareness of the UE-panel properties is a first step toward enhanced BM algorithms.

- : We see a clear benefit of specifying UE reporting of panel-specific information.

## Analog beam definition

Besides UE antenna panel definition, we also observe that there is no explicit definition of what a beam is. Though analog beamforming is highly up to UE implementation, but in our understanding, a beam can be defined as

- : A beam can be defined as a spatial filtering associated with one or two antenna ports carrying one or two layers separated in the polarization domain.

With the beam definition in hand, next one may consider whether to signal the UE capability on whether single or dual layer(s) are supported by a panel. In RAN4#90bis main session’s chairman notes, it was agreed to use 8 beams for an UL beam sweep in beam-correspondence test. In addition it was also agreed to further study whether there is an advantage to specify the polarization aspect in the UL beam sweep. This may have system benefits both from a MU perspective as well as from a link level capacity perspective. Hence we have following

- : RAN1 needs to study and specify (if necessary) whether additional signaling is necessary when a beam can support up to two independent layers separated by polarization.

# Conclusions

Finally, allow us to repeat our proposals to draw attention.

- : Following the Rel.15/16 QCL rules, RAN1 do not support SRS for BM as DL QCL-TypeD source RS.

- : For backward compatibility with Rel.15/16 beam management, support CSI-RS for CSI as QCL source RS in unified TCI state.

- : From dynamic signaling perspective, support CSI-RS to share the same indicated TCI state(s) of Rel.17 as UE-dedicated PDCCH and PDSCH.

- : SRS for UL Rx beam sweeping can be a target RS of Rel.17 unified TCI state.

- : If SRS for BM is supported as target RS of Rel.17 unified TCI state, then some of it should be allowed to share the same dynamically indicated TCI state as DG/CG PUSCH and dedicated PUCCH.

- : To achieve the flexibility of DL and UL beam updating/indication, support separately configured unified TCI state pools per BWP/CC.

- : Confirm the working assumption on RRC-configured TCI state pool(s) and methods on QCL-TypeA/D source RS indication when BWP/CC ID is present or absent.

- : For inter-band CAs, support common beam operation for data/control, DL/UL, multiple BWPs, multiple TRPs and multiple UE panels.

- : The UE capability related to simultaneously steering beams in the same direction belonging to CCs in different bands should be reported.

- : For the optimization of beam indication across bands, the polarization property of beams should be considered.

- : Support CSI-RS for mobility as measurement RS for L1/L2 inter-cell mobility.

- : Define L1-metric based events for L1/L2 inter-cell mobility and support UE triggered beam reporting based on L1 events.

- : Support Rel.17 MAC CE-based and DCI-based beam indication mechanism, in which MAC CE activates unified TCI states containing RS only from the same non-serving cell.

- : Support SSB (from NSC) only as indirect QCL source for PDSCH DMRS and PDCCH DMRS.

- : Support CSI-RS for mobility as source RS for conducting QCL info from non-serving cell.

- : Support (Alt.3) UL DCI format 0_1/0_2 with UL grant to indicate UL Tx beam by joint DL/UL TCI state or UL-only TCI state.

- : Study standard impacts of introducing group common DCI to indicate/update DL TCI state(s) for a group of UEs.

- : For UL grant DCI carrying TCI state(s), if supported, then reuse the implicit HARQ mechanism of PUSCH for UL grant DCI.

- : For GC-DCI carrying unified TCI state(s) for a group of UEs, if supported, study its HARQ mechanism for each UE within the indicated group.

- : Support to link or associate UL panels with SRS resource sets for UL fast panel selection.

- : Support to implicitly link or associate UE panel(s) with joint or UL-only TCI state(s) via beam reporting.

- : Support to link a panel entity with a new panel ID (Opt1-2, 1st priority) or with CSI-RS and/or SSB resource in beam reporting (Opt1-1, 2nd priority).

- : Support UE-initiated UL panel(s) linking/association status update via UCI or MAC CE.

- : Do not support NW-initiated panel selection/activation, unless the benefits for supporting this feature can be fully justified.

- : To facilitate MPE mitigation, support UE reports (a subset of Opt 2A) SSBRI(s)/CRI(s) + modified L1-RSRP that accounts for MPE effect.

- : For UL beam sweeping, when taking MPE into account, UE should transmit SRS resource(s) by certain power back-off, if needed.

- : Support UE reports P-MPR value(s) associated with panel(s) (Opt 1D) to mitigate UL coverage loss due to MPE.

- : To avoid UL MPE issue, it would be beneficial for RAN1 to study and, if necessary, specify the mechanism of UE triggered UL beam/panel switch.

- : We see a clear benefit of specifying UE reporting of panel-specific information.

- : A beam can be defined as a spatial filtering associated with one or two antenna ports carrying one or two layers separated in the polarization domain.

- : RAN1 needs to study and specify (if necessary) whether additional signaling is necessary when a beam can support up to two independent layers separated by polarization.

# Reference

- RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung

- 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”

- B. Xu, K. Zhao, Z. Ying, D. Sjöberg, W. He and S. He, "Analysis of Impacts of Expected RF EMF Exposure Restrictions on Peak EIRP of 5G User Equipment at 28 GHz and 39 GHz Bands," in IEEE Access, vol. 7, pp. 20996-21005, 2019.