3GPP TSG RAN WG1 #106-e	R1- 2106921

# e-Meeting, August 16th – 27th, 2021

Agenda Item:		8.15.2

Source:				Samsung

Title:					Timing relationship enhancements

Document for:	Discussion

# Introduction

A WID on NB-IoT/eMTC support for NTN was approved in RAN#92-e [1]. One objective is related to timing relationships enhancements.

Specify the following IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763: - Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763 - UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD - Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

This contribution discusses aspects of timing relationship enhancements.

# Timing relationships

The propagation delays in NTN are much longer than the propagation delays in terrestrial mobile systems which are usually less than 1ms, ranging from several milliseconds to hundreds of milliseconds depending on the altitudes of the airborne platforms. The cell size in NTN is much larger and UEs in different parts of the cell experience different delays. Thus, the long propagation delays and large cell size require modifications of the timing aspects defined in terrestrial networks so that DL and UL frame timing is aligned at the eNB side.

During the SI phase, the following timing relationships were identified for enhancements in NTN IoT [2].

For NB-IoT:

- 	NPDCCH to NPUSCH format 1

-	RAR grant to NPUSCH format 1

-	NPDSCH to HARQ-ACK on NPUSCH format 2

-	Timing advance command activation

-	FFS: NPDCCH order to NPRACH

For eMTC:

- 	MPDCCH to PUSCH

-	RAR grant to PUSCH

-	MPDCCH to scheduled uplink SPS

-	PDSCH to HARQ-ACK on PUCCH

-	CSI reference resource timing

-	MPDCCH to aperiodic SRS

-	Timing advance command activation

-	FFS: MPDCCH order to PRACH

NPDCCH to NPUSCH format 1

NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0. As specified in [3] for terrestrial NB-IoT, a UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-	n+k0 	DL subframe for FDD,

-	k0 		NB-IoT UL subframes following the end of n+8 subframe for TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1.

For NTN, an additional delay k_offset needs to be added.

-	n+k0 +k_offset 	DL subframe for FDD,

-	k0 +k_offset 		NB-IoT UL subframes following the end of n+8 subframe for TDD,

Proposal 1: For NB-IoT in NTN, introduce an additional delay of k_offset subframes before transmission of NPUSCH format 1 scheduled by a NPDCCH with DCI format N0.

RAR grant to NPUSCH format 1

The higher layers indicate the Narrowband Random Access Response Grant to the physical layer, which includes 2 bits for the scheduling delay field () with k0 = 12 for IDelay = 0 , where NB-IoT DL subframe n is the last subframe in which the NPDSCH associated with the Narrowband Random Access Response Grant is transmitted. An additional delay offset k_offset should be added and the relationships described above for NPUSCH format 1 transmission scheduled by a NPDCCH with DCI format N0 are also valid for the scheduling by a RAR grant.

Proposal 2: For NB-IoT in NTN, introduce an additional delay of k_offset subframes before transmission of NPUSCH format 1 scheduled by a RAR grant.

NPDSCH to HARQ-ACK on NPUSCH format 2

The same logic for an additional delay offset k_offset used in the above timing relationships applies also to the transmission of NPUSCH format 2. Thus, the relationships for NTN NB-IoT are as follows.

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	  + k_offset		DL subframes for FDD,

-	   + k_offset			NB-IoT UL subframes following the end of n+12 subframe for TDD,

transmission of the NPUSCH format 2 in N consecutive NB-IoT UL slots.

Proposal 3: For NB-IoT in NTN, introduce an additional delay of k_offset subframes before transmission of NPUSCH format 2 scheduled by a RAR grant.

NPDCCH order to NPRACH

Whether the timing relationship of a preamble transmission initiated by a PDCCH order channel needs to be enhanced for NTN IoT was identified as FFS during the SI phase. For terrestrial NB-IoT, in case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe , , where a NPRACH resource is available. Similar to the timing relationships of other channels agreed to be enhanced for essential minimum functionality in NTN, an additional delay k_offset needs to be added to k2 to take into account the large round trip delay. The gNB would then have knowledge of when to expect a NPRACH transmission.

Proposal 4: For NB-IoT in NTN, introduce an additional delay of k_offset subframes before transmission of a random access preamble when the random access procedure is initiated by a PDCCH order.

NPRACH preamble retransmission

In terrestrial NB-IoT [3],

- If a random access response is received and the corresponding DL-SCH transport block ending in subframe n does not contain a response to the transmitted preamble sequence, or

- If no NPDCCH scheduling random access response is received in subframe n, where subframe n is the last subframe of the random access response window, or

- If an NPDCCH scheduling random access response with associated RA-RNTI is detected and the corresponding DL-SCH transport block reception ending in subframe n cannot be successfully decoded,

the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n (subframe n is a DL frame).

This timing relationship for PRACH retransmission should also apply for NTN IoT.

Proposal 5: The timing relationship for PRACH retransmission in NB-IoT is reused for NTN NB-IoT.

Timing Advance

In Ran1#105-e it was debated whether in the specification of terrestrial NB-IoT and eMTC it is clear that the value ns of the subframe that is used in the initialization of the scrambling sequence generator is the subframe number after applying the TA. For NTN, the TA value can be several seconds (multiple subframes), and whether the TA is applied or not to determine the subframe value ns used in the initialization is expected to affect the scrambling sequence generation. From TS36.211 Clause 5.3.1 ns is the starting frame of the PUSCH transmission, hence the TA has already been taken into account in determining ns.  It seems there is no need for a clarification in the specifications.

Observation 1: From TS36.211 Clause 5.3.1 ns is the starting frame of the PUSCH transmission, hence the TA has already been taken into account in determining ns.

# Cell/UE-specific time offset

Cell-specific timing offset is required at least for the transmission timing of RAR grant scheduled PUSCH. A cell-specific timing offset is based on a maximum time advance (TA) within one cell which can be large due to the large round-trip delay in NTN, and can be overestimated for some UEs. A UE-specific timing offset can reduce the transmission delay for UEs that need a smaller timing offset, at the cost of additional signaling overhead and power consumption since an estimated TA should be reported to the eNB.

Configurability of cell-specific or UE-specific timing offset can be useful depending on scenarios.  For small NTN cells, the variation of the TA values among the users of a cell is relatively small and a cell-specific Koffset is expected to be accurate. For large NTN cells, the variation of the TA values among the users of a cell can be large, in the order of several subframes [3], especially for GEO scenarios, and UE-specific timing offsets are needed.

Thus, a cell-specific timing offset can be used for initial access at least for adjusting the transmission timing of the RAR grant scheduled PUSCH. A UE can be reconfigured with a UE-specific timing offset for connected mode.

Proposal 6: Support cell-specific timing offset during initial access. The cell-specific timing offset is transmitted in SIB.

Proposal 7: Support cell-specific and UE-specific timing offset after initial access. Timing offset is configurable.

# Conclusion

This contribution discusses remaining aspects related to timing relationship enhancements. Proposals made in this contribution are summarized below.

Proposal 1: For NB-IoT in NTN, introduce an additional delay of k_offset subframes before transmission of NPUSCH format 1 scheduled by a NPDCCH with DCI format N0.

Proposal 2: For NB-IoT in NTN, introduce an additional delay of k_offset subframes before transmission of NPUSCH format 1 scheduled by a RAR grant.

Proposal 3: For NB-IoT in NTN, introduce an additional delay of k_offset subframes before transmission of NPUSCH format 2 scheduled by a RAR grant.

Proposal 4: For NB-IoT in NTN, introduce an additional delay of k_offset subframes before transmission of a random access preamble when the random access procedure is initiated by a PDCCH order.

Proposal 5: The timing relationship for PRACH retransmission in NB-IoT is reused for NTN NB-IoT.

Observation 1: From TS36.211 Clause 5.3.1 ns is the starting frame of the PUSCH transmission, hence the TA has already been taken into account in determining ns.

Proposal 6: Support cell-specific timing offset during initial access. The cell-specific timing offset is transmitted in SIB.

Proposal 7: Support cell-specific and UE-specific timing offset after initial access. Timing offset is configurable.

# References

- RP-211601, WID on NB-IoT/eMTC support for NTN, RAN#92-e, June 2021.

- RP-211456, 3GPP TR 36.763 V1.0.0 (2021-05).

- 3GPP TS 36.213.