3GPP TSG RAN WG1 #106-e		                                                                 R1-2106995

e-Meeting, August 16th – 27th , 2021

Title:	Draft Reply LS on resource reselection trigger sl-reselectAfter

Response to:	R1-2106406/ R2-2106625

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	CATT, GOHIGH

To:	RAN2

Cc:

Attachment:	-

Contact Person:

Name:	Rui Zhao

E-mail Address:	zhaorui@gohigh.com.cn

Overall description

RAN1 thank RAN2 for the LS on resource reselection trigger sl-reselectAfter, and would provide our response to the following questions from RAN2.

Question 1: In NR V2X, whether “unused retransmission opportunities in case of HARQ feedback is enabled” shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

Response to question 1:

From RAN1’s understanding, if a MAC PDU has been positively acknowledged, the remaining retransmission of the MAC PDU should not be counted towards “consecutive unused transmission opportunities” on resources indicated in the selected SL grant.

And the current RAN’s spec has a clear idea on this issue. In clause 5.22.1.1 of 38.321, if a MAC PDU has been positively acknowledged, the remaining retransmission(s) of the MAC PDU has been cleared from selected SL grant. And the “consecutive unused transmission opportunities” is only for the resource(s) of the SL grant. Therefore, the remaining retransmission of the MAC PDU with positively acknowledged is not counted for the resource reselection trigger.

1> if a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledged as specified in clause 5.22.1.3.3: 2> clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

There is another issue on the range of sl-reselectAfter values, since there is only up to one retransmission for LTE-V2X, then the corresponding sl-reselectAfter for LTE-V2X is [1, … ,9]. However, NR-V2X allows much larger retransmissions which could be up to 31.  If NR-V2X reuse the same sl-reselectAfter values as that in LTE-V2X, it may lead to release the sidelink grant even when only one TB is skipped. Therefore, it would be better to enlarge the range of sl-reselectAfter values.

Question 2: In NR V2X, according to RAN1 agreement, is it the correct understanding that only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

Response to question 2:

No, from RAN1’s understanding, the sidelink grant is maintained by MAC layer regardless of SCI indicated or not.

Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take the above responses into account in the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#106b-e			11-19 October 2021   			E-Meeting

RAN1#107-e			11-19 November 2021			E-Meeting