3GPP TSG RAN WG1 #106-e		R1-2107616

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.14.1

Source: 	Intel Corporation

Title:	Traffic model for XR

Document for:        Discussion/Decision

- Introduction

Agreement

PDB value of the stream in UL AR aggregating streams of scene, video, data, and audio, i.e., Option 2, Stream 2 in Option 1, and Stream 2 in Option 3.

- 30ms (baseline), 10/15/60ms (optional)

Agreement

For DL video stream, separate packet arrivals in time for dual-eye buffer can be optionally evaluated, based on the single stream model by doubling the packet arrival rate and halving the packet size compared to the single stream, while all other parameters (e.g., jitter, PDB) are the same as for single stream.

- For companies who are evaluating separate packet arrivals in time for dual-eye buffer in addition to single stream (baseline), it is recommended to evaluate at least the following scenarios in the table.  It is encouraged to evaluate additional baseline/optional scenarios/configurations.

| Application | AR/VR 30Mbps |  |  |
| --- | --- | --- | --- |
| Traffic model | Single stream for dual-eye buffer | Separate packet arrival for dual-eye buffer |  |
| Data rate (Mbps) | 30 | 30 |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |
| Mean packet size (Bytes) | 62500 | 31250 | Data rate / FPS / 8 [bytes] |
| STD of packet size (Bytes) | 6563 | 3281 | 10.5% x mean packet size |
| Max packet size (Bytes) | 93750 | 46875 | 150% x mean packet size |
| Min packet size (Bytes) | 31250 | 15625 | 50% x mean packet size |
| Packet arrival interval (ms) | 1000/60 | 1000/120 |  |
| PDB (ms) | 10 |  |  |

Agreement

When companies are submitting evaluation results to RAN1, it is recommended to submit results at least the following parameters in the below table.

- Note 1: This is only intended to have more results from more companies at least for the corresponding configuration. RAN1 agreements regarding baseline vs. optional for simulation scenarios, configurations, parameters, remain the same.

- Note 2: Companies are encouraged to submit results for other baseline/optional configurations as much as they can.

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

In RAN1#105-e, the following agreements were made on the traffic model for XR evaluations [1]

Agreement

For the optional evaluation scenario, two streams of I-frame and P-frame for DL video stream (option 1), the traffic models described in the below table are assumed.

- FFS: Parameter values of α, A, B, C, D, E, F, G, H

  - Including the possibility of using multiple set of parameter values

- For companies who are evaluating this option, it is recommended to evaluate at least the following scenario: AR/VR, 30Mbps, Dense Urban for FR1 and InH for FR2.  It is encouraged to evaluate additional baseline/optional scenarios/configurations.

| Two data streams, i.e. M1 = 2 | Option 1A: slice-based |  |  |  | Option 1B: GOP-based |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I-stream |  | P-stream |  | I-stream |  | P-stream |
| Packet modelling | Slice-level |  |  |  | Frame-level |  |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  |  |  | Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream. |  |  |
| Number of packets per stream at a time | 1 |  | N-1 |  | I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet |  |  |
|  | N = 8: the number of slices per frame. |  |  |  |  |  |  |
| Average data rate per stream |  |  |  |  | RI=R*αK-1+α |  | RP=R*K-1K-1+α |
|  | - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice, e.g. α = 1.5, 2, 3 |  |  |  |  |  |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
|  | Mean = RIFPS | Mean = RPFPS*(N-1) |  |  | Mean = RI*KFPS | Mean = RP*KFPS*(K-1) |  |
|  | - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video |  |  |  |  |  |  |
| PER, PDB | [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms] |  |  | [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms] |  |  |  |

In this paper, we provide SA4 trace analysis and discuss the remaining issues.

- SA4 trace analysis (focusing on VR2)

Overview:

In [2] SA4 provided an overview of software tools, trace formats and configurations for VR2, CG and AR2 traffic. The document also provides initial recommended configurations for each of the above scenarios and provides relevant P-Traces. An overview of the different software modules and APIs that are provided for the purposes of RAN1 simulation is shown in the following

Figure 1: Overview of RAN evaluation framework

Figure 1: Overview of RAN evaluation framework

The following describes the individual functional blocks shown above and specifically the details for VR2 use-case is indicated below:

Content Model

This block provides typical characteristics for the XR content model for video, audio and potentially other data. This content is rendered for XR/CG consumption. For VR2

  - Game output with 2 eye buffers at 2Kx2K at 60 fps, 8bit.

  - Content and Trace Preview is here: http://dash.akamaized.net/WAVE/3GPP/XRTraffic/Traces/Qualcomm-VR2

  - No audio

Encoding Model

This functional block provides the details for the content encoding in order to meet certain objectives. This includes the generation of sequences of application data units (slices, video frames, audio frames, etc.) and the incurred timestamp of each of the units is available.

- HEVC model as defined in S4aV200575

- target bitrate 30 Mbit/s, equally split across eye buffers, independently encoded.

- Pre-encoding delay: Encoder pre-delay is varying between 10 to 20ms

Encoding delay is modelled to vary with truncated Gaussian

Content Delivery Model:

This functional block provides details on content delivery, for example packetization, delay jitter, but possibly also more sophisticated models such as retransmission, TCP operations and so on. This also includes emulation of 5G Core Network. It produces packet traces (timestamp and size of each packet) based on traces of application data units. For VR2:

- Edge/Cloud to gNB bitrate is 1.5 media bitrate

The following configurations are recommended with priority according to order in [2].

| Configuration | Basic Content Parameters |
| --- | --- |
| VR2-1 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-2 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |
| VR2-3 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, 1500 byte max packet size packets |
| VR2-4 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, unlimited packet size |
| VR2-5 | 1 slice per eye buffer, every 8th frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-6 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffers sent interleaved, 1500 byte max packet size |
| VR2-7 | 8 slices per eye buffer, 1 slice per frame is intra coded, 45Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size packets |
| VR2-81 | 8 slices per eye buffer, 1 slice per frame is intra coded, 45Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |

## P-Trace analysis for VR2

A summary of the P-Trace analysis for VR2 use-case based on the configurations VR2-1 to VR2-8 is shown below.

|  | VR2-1 | VR2-3 | VR2-5 | VR2-7 | VR2-2 | VR2-4 | VR2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| #slice per eye buffer | 8 | 8 | 1 | 8 | 8 | 8 | 8 |
|  | 1 slice per frame is intra coded | 1 slice per frame is intra coded | every 8th frame is intra coded | 1 slice per frame is intra coded | 1 slice per frame is intra coded | 1 slice per frame is intra coded | 1 slice per frame is intra coded |
| Target bitrate | 30Mbit/s capped VBR with window 200ms | 30Mbit/s CBR with window 1 frame | 30Mbit/s capped VBR with window 200ms | 45Mbit/s capped VBR with window 200ms | 30Mbit/s capped VBR with window 200ms | 30Mbit/s CBR with window 1 frame | 45Mbit/s capped VBR with window 200ms |
| Max packet size (byte) | 1500 byte max packet size | 1500 byte max packet size packets | 1500 byte max packet size | 1500 byte max packet size | unlimited packet size | unlimited packet size | unlimited packet size |
| Bitrate (Mbps) | 28.81 | 29.83 | 26.15 | 41.01 | 28.15 | 29.15 | 40.08 |
| mean. frame-size (kbytes) | 59.02 | 61.12 | 53.58 | 84.02 | 57.68 | 59.73 | 82.11 |
| max/mean. frame-size | 1.18 | 1.06 | 1.94 | 1.24 | 1.23 | 1.06 | 1.23 |
| min/mean frame-size | 0.48 | 0.93 | 0.24 | 0.34 | 0.48 | 0.94 | 0.34 |
| std/mean frame-size | 0.07 | 0.02 | 0.12 | 0.14 | 0.07 | 0.02 | 0.14 |

Observations-1:

- for CBR configurations the frame-size variations are quite small

  - the max/mean frame-size ratio is ~ 1.06

  - the min/mean frame-size ratio is ~0.93

  - the std/mean frame-size ratio is ~0.02

- for VBR configurations the frame-size variations are larger

  - the max/mean frame-size ratio is ~ 1.18 – 1.94 with smaller ratios corresponding to 8 slice/eye buffer case while large ratio corresponding to 1 slice case

  - the min/mean frame-size ratio is ~ 0.24 – 0.48 with larger ratios corresponding to 8 slice/eye buffer case while small ratio corresponding to 1 slice case

  - the std/mean frame-size ratio is ~ 0.07 – 0.14

The detailed analysis for the above configurations are shown below:

## 1500 byte max packet size cases (VR2-1, VR2-3, VR2-5, VR2-7)

| Figure 2: Bitrate variation around target bitrate | Figure 3: packet-size CDFs (max 1.5 kbyte) |
| --- | --- |

| Figure 4: frame-size probability (calculated from P-trace) | Figure 5: frame-size CDFs (calculated from P-trace) |
| --- | --- |

| Figure 6: # of packets vs frame-size scatter plot | Figure 7: burst-length (ms) vs frame-size scatter plot |
| --- | --- |

## Unlimited max packet size configurations (VR2-2, VR2-4, VR2-8)

| Figure 8: Bitrate variation around target bitrate | Figure 9: packet-size CDFs (max unlimited) |
| --- | --- |

| Figure 10: frame-size probability (calculated from P-trace) | Figure 11: frame-size CDFs (calculated from P-trace) |
| --- | --- |

| Figure 12:# of packets vs frame-size scatter plot | Figure 13: burst-length (ms) vs frame-size scatter plot |
| --- | --- |

Observations-2:

We observe that

- there is significant variation in traffic observed at L2 (P-trace) for a given content model (V-trace) and use-case example: VR2 30 Mbps 2 eye buffers at 2Kx2K at 60 fps, 8bit

- this variation is due to encoding model (slices, encoding delay, etc.) and content delivery model (packetization etc.)

- distribution of frame-size based on P-trace is asymmetric (heavy tail below mean, light tail above mean)

# 3. Two stream model

## 3.1  PER, PDB requirement

In RAN1#105-e, it was agreed that two streams of I-frame and P-frame for DL video stream (option 1) can be optionally evaluated. The parameter values of α, A, B, C, D, E, F, G, H are to be further discussed and potentially confirmed in RAN1#106-e.

For GOP-based model, the underlying assumption is temporal prediction for P frames and purely intra-coded for I-frames, where the exact nature of temporal dependency of P-frames will be up to encoder configuration. Therefore, it is desirable to set the PER for I frames (PER_I) smaller than the PER for P frames (PER_P) considering the effect of error propagation. If arrival time of a I frame and a P frame is similar at the gNB (based on encoding delay etc. for DL for example) we expect that the PDB_I can be similar to PDB_P.

For slice-based model, smaller PER for I slice is desirable due to the same reason as above. If arrival time of the I slice and P slice is similar at the gNB (based on encoding delay etc. for DL for example) we expect that the PDB_I can be similar to PDB_P.

## 3.2 Average size ratio between I-slice and P-slice

The statistics of average size ratio between I-slice and P-slice obtained from P-trace [3] is shown in the following table, where α is defined as the ratio of the average I-slice data size to the average P-slice data size.

|  | VR2-1 | VR2-3 | VR2-5 | VR2-7 | VR2-2 | VR2-4 | VR2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| α | 1.85 | 1.90 | 1.07 | 1.81 | 1.85 | 1.89 | 1.80 |

It should be noted that 1 slice per frame is intra coded for configuration VR2-1, VR2-3, VR2-7, VR2-2, VR2-4 and VR2-4 with 8 slices per frame, whereas every 8th frame is intra coded with 1 slice per frame for configuration VR2-5. Therefore, for VR2-5, α is equivalent to the size ratio between I-frame and P-frame.

Observations-3:

We observe that

- The average size ratio between one I-frame/slice and one P-frame/slice observed from SA4 traces is ~ 1.07– 1.90.

# 4. Conclusions

Observations-1 (from SA4 trace):

- for CBR configurations the frame-size variations are quite small

  - the max/mean frame-size ratio is ~ 1.06

  - the min/mean frame-size ratio is ~0.93

  - the std/mean frame-size ratio is ~0.02

- for VBR configurations the frame-size variations are larger

  - the max/mean frame-size ratio is ~ 1.18 – 1.94 with smaller ratios corresponding to 8 slice/eye buffer case while large ratio corresponding to 1 slice case

  - the min/mean frame-size ratio is ~ 0.24 – 0.48 with larger ratios corresponding to 8 slice/eye buffer case while small ratio corresponding to 1 slice case

  - the std/mean frame-size ratio is ~ 0.07 – 0.14

Observations-2 (from SA4 trace):

We observe that

- there is significant variation in traffic observed at L2 (P-trace) for a given content model (V-trace) and use-case example: VR2 30 Mbps 2 eye buffers at 2Kx2K at 60 fps, 8bit

- this variation is due to encoding model (slices, encoding delay, etc.) and content delivery model (packetization etc.)

- distribution of frame-size based on P-trace is asymmetric (heavy tail below mean, light tail above mean)

Observations-3 (from SA4 trace):

We observe that

- The average size ratio between one I-frame/slice and one P-frame/slice observed from SA4 traces is ~ 1.07– 1.90.

# References

| [1] | "Draft Report of 3GPP TSG RAN WG1 #105-e v0.2.0". |
| --- | --- |
| [2] | "R1-2101765, LS to on XR-Traffic Models, SA4". |
| [3] | "http://dash.akamaized.net/WAVE/3GPP/XRTraffic/Traces/Qualcomm-VR2". |

# Appendix

The trace format for P-trace is shown below:

| Name | Type | Semantics |
| --- | --- | --- |
| number | BIGINT | Unique packet number in the delivery |
| time_stamp_in_micro_s | BIGINT | Availability time of packet for next processing step relative to start time 0 in microseconds (0 means lost). |
| size | BIGINT | packet size in bytes. |
| user_id | BIGINT | assigns an id to the user in order to differentiate |
| buffer | BIGINT | The associated eye buffer 1=left 2=right In general, differentiates application traffic for different buffers, for example audio, video, left eye, right eye. For example mapped to port or track. |
| delay | BIGINT | Delay observed of the packet in the last processing step (-1 means lost) |
| render_timing | BIGINT | the rendering generation timing associated to the media included in the packet. |
| number_in_unit | BIGINT | The number of the packet within the unit (slice), start at 1 |
| last_in_unit | BIGINT | Indicates if this is the last packet in the slice/unit 0=no, 1=yes |
| type | BIGINT | The data type of the unit 0 unknown For video 1=intra 2=inter |
| importance | BIGINT | assigned relative importance information (higher number means higher importance) |
| index | BIGINT | Unique index increased by 1 and indexing this row in the S-Trace file. |
| s_trace | STRING | Reference to s_trace file containing information for each slice |

This configuration is not mentioned in [5]