3GPP TSG RAN WG1 #106-e	R1- 2107756

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.8.2

Source:	Apple Inc.

Title:	PUCCH coverage enhancements

Document for:	Discussion, Decision

# Introduction

In 3GPP RAN#90-e, a work item for Rel-17 NR coverage enhancements was approved. The objectives of Rel-17 coverage enhancement WI, is to specify solutions to enhance coverage for different uplink channels mainly PUSCH, PUCCH and Msg3 PUSCH, as it can be found below [1]:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

In this contribution, we present our views on potential techniques and required signalling to enhance NR coverage for PUCCH channel.

# Coverage Enhancements for NR PUCCH

## DMRS bundling across PUCCH repetitions

Joint channel processing across different PUCCH repetition occasions requires several conditions to be met, one of them is the phase continuity across different repetitions. The following agreement was made in RAN1# 104e [2]:

Agreements: Subject to the prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration.

- FFS: the configuration is per UE or per PUCCH resource.

- FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling

- FFS: necessity of additional signaling/configuration of DMRS bundling duration/window and associated size

Further in RAN1# 105e it was agreed that:

Agreement: For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements.

- Strive for common design of the time domain window for PUSCH/PUCCH with DMRS bundling as much as possible.

In our view, the RRC indication to enable DMRS bundling, subject to UE capability, should be per PUCCH resource. Once enabled, it is not expected that UE keep phase out of a configured window for DMRS bundling or over consecutive opportunities for PUCCH repetitions per hop, whichever is smaller. In this way, the existing mechanism in DCI for dynamic indication of PUCCH resource via PRI is sufficient to give flexibility to scheduler to schedule a PUCCH with repetitions with (or without) DMRS bundling. Based on what we discussed, we have the following proposals:

Proposal 1: DMRS bundling for PUCCH repetitions, subject to UE capability, is enabled via RRC configuration per PUCCH resource.

Proposal 2: Subject to UE capability, if conditions of DMRS bundling for PUCCH repetitions is met, UE is not expected to keep phase out of consecutive opportunities for PUCCH repetitions per hop.

In current NR, once inter-slot PUCCH frequency hopping is configured, the hopping procedure is per slot. In other words, if there are more than 2 repetitions, every other repetition will be on the same frequency hop. It was agreed in RAN1 #104e that [2]:

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling.

- FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16.

- Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible

In our view, given that the achievable gain due to DMRS bundling for a limited bundle-size is marginal, additional signaling for inter-slot frequency hopping with DMRS bundling is not well justified. For a UE indicated with inter-slot frequency hopping and also DMRS bundling, in order to benefit from both of DMRS bundling and frequency hopping, the first half of the repetitions should happen in the first hop and the second half of the repetitions on the second hop.

Proposal 3: Specify the inter-slot frequency hopping pattern to enable the conjunction operation of repetition, frequency hopping and joint channel estimation.

In current NR specification, once PUCCH with repetition overlaps with another PUCCH, we have (38.213, 9.2.6):

- If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI:

  - if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority

We should note than in current NR, all PUCCHs with repetitions are RRC indicated and there is no indication of DMRS bundling. If we just follow the current specification, we may end up two problems:

- a PUCCH repetition occasion, dynamically indicated with repetition factor, within a bundle of repetitions may be dropped, if another overlapping PUCCH without dynamic indication of repetition number has a UCI type with a higher priority, as mentioned above.

- a PUCCH repetition occasion within a bundle of repetitions with DMRS bundling may be dropped, e.g. if another overlapping PUCCH has a UCI type with a higher priority, as mentioned above. Subsequently, the phase continuity will be lost for the first PUCCH with DMRS bundling.

Proposal 4: Specify conditions under which a PUCCH with dynamic indication of repetition number may overlap with another PUCCH repetitions without dynamic indication of repetitions.

Proposal 5: If DMRS bundling is supported, specify conditions under which phase continuity is kept for a PUCCH with DMRS bundling overlapping in one (or more) occasions with a second PUCCH without DMRS bundling.

In current NR, TPC command is applied per transmission occasion. If UE receives other DCIs, UE-specific or GC-DCI 2_2, in between different PUSCH/PUCCH repetitions, then different repetitions may have different transmit power. This leads to a phase dis-continuity. For PUCCH/PUSCH repetitions with DMRS bundling, one simple solution is to prevent any DCI indication with a new TPC within the bundle of repetitions. This approach is a bit restrictive, from both UE and gNB perspective.

Proposal 6: For a PUCCH (or PUSCH) repetition with DMRS bundling, only TPC indicated by a unicast DCI is applied, i.e. TPC on GC-DCI 2-2 is ignored.

Proposal 7: Unicast DCI with a TPC command implicitly indicates that DMRS bundling is off, from the occasion that new TPC is applied.

## Dynamic indication of PUCCH repetition number

Dynamic indication of repetition factor for PUCCH is discussed and a working assumption was agreed in RAN1 #105e as follows [2]:

Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication.

- Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor.

  - FFS: RRC signaling enhancement details

Given that configuration of each PUCCH resource of format 1/3/4 can be associated with a RRC repetition factor, potentially there will be more PUCCH resources per resource set, e.g. some with repetition factor 2, 4, etc. In current specification, there are two mechanisms to indicate a PUCCH resource within a set with more than 8 resources. The first mechanism is specified in 38.213 Sec. 9.2.1 for cell-specific PUCCH indication (fixed to 16 resources), and the second mechanism is specified in 38.213 Sec. 9.2.3, which allows resource indication for dedicated PUCCH config when the set size is up to 32. The latter mechanism is only applicable to the first PUCCH resource set (set0) which may have more than 8 resources (up to 32 resources). In our view, we can combine the existing mechanisms to map 3bits in PRI to a PUCCH resource in a resource set (not limited to set0) with more than 8 resources (and up to 64 resources), as follows:

- Step1: if the PUCCH resource set size is more than 32, use the mechanism in 38.213 Sec. 9.2.1 to split the resources within the set into two sub-sets, where the first subset has 32 resources and NCCE and nCCE,0 indicate which sub-set is selected.

- Step2: reuse the procedure in 38.213 Sec. 9.2.3 to determine the final PUCCH resource within the sub-set selected by Step1.

Proposal 8: Support the working assumption as an agreement, and combine existing mechanisms based on PRI, NCCE and nCCE,0 to indicate the PUCCH resource with repetition factor within a PUCCH resource set up to 64 resources

- FFS: indication of repetition factor for a FM1 PUCCH provided by pucch-ResourceCommon

# Conclusions

In this contribution, we provided our thoughts on techniques NR coverage enhancement. Based on views shared in this paper, we make the following proposals:

Proposal 1: DMRS bundling for PUCCH repetitions, subject to UE capability, is enabled via RRC configuration per PUCCH resource.

Proposal 2: Subject to UE capability, if conditions of DMRS bundling for PUCCH repetitions is met, UE is not expected to keep phase out of consecutive opportunities for PUCCH repetitions per hop.

Proposal 3: Specify the inter-slot frequency hopping pattern to enable the conjunction operation of repetition, frequency hopping and joint channel estimation.

Proposal 4: Specify conditions under which a PUCCH with dynamic indication of repetition number may overlap with another PUCCH repetitions without dynamic indication of repetitions.

Proposal 5: If DMRS bundling is supported, specify conditions under which phase continuity is kept for a PUCCH with DMRS bundling overlapping in one (or more) occasions with a second PUCCH without DMRS bundling.

Proposal 6: For a PUCCH (or PUSCH) repetition with DMRS bundling, only TPC indicated by a unicast DCI is applied, i.e. TPC on GC-DCI 2-2 is ignored.

Proposal 7: Unicast DCI with a TPC command implicitly indicates that DMRS bundling is off, from the occasion that new TPC is applied.

Proposal 8: Support the working assumption as an agreement, and combine existing mechanisms based on PRI, NCCE and nCCE,0 to indicate the PUCCH resource with repetition factor within a PUCCH resource set up to 64 resources

- FFS: indication of repetition factor for a FM1 PUCCH provided by pucch-ResourceCommon

# References

- RP-202928, “New WID on NR Coverage Enhancements”, December 2020

- Chairman’s Notes RAN1#105-e final