3GPP TSG RAN WG1 #106-e			R1-2107878

e-Meeting, August 16th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Other enhancements for simultaneous operation of IAB-node’s child and parent links

Agenda Item:	8.10.2

Document for: 	Discussion and Decision

- Introduction

At RAN#86 meeting, a new WID [1] on “Enhancements to Integrated Access and Backhaul” was approved. The detailed objectives for RAN1 are as follows.

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In this contribution, we discuss enhancements on IAB-node timing mode(s), extensions for DL/UL power control, and interference management to support simultaneous operation.

- Discussion on enhancements on case #6 and #7 timing modes

  - Timing mode switching

Agreement

An IAB-node is indicated when Case 6 timing is performed at the IAB-node.

- FFS details of the indication (e.g. semi-static and/or dynamic, implicit and/or explicit, linkage to multiplexing capability, etc.).

FFS whether an IAB-node is also indicated when Case 7 timing is performed at the IAB-node.

Agreement

An IAB-node is indicated when Case 6 timing is performed at the IAB-node.

- FFS details of the indication (e.g. semi-static and/or dynamic, implicit and/or explicit, linkage to multiplexing capability, etc.).

FFS whether an IAB-node is also indicated when Case 7 timing is performed at the IAB-node.

At the RAN1#105-e meeting, IAB-node timing mode switching was discussed and following agreement was made [2].

Example of timing modes setting for IAB-node is shown in Fig.1. As shown in Fig.1, IAB-node may start with Case #1 timing mode for MT Tx timing as a UE, since during initial access IAB-DU is not activated so that Case #6 timing mode is not necessary for IAB-node. And a parent node doesn’t know that the connecting node is a normal UE or IAB-node, so that Case #7 timing mode is not indicated during initial access. Therefore, IAB-node certainly has TA for Case #1 timing mode which is indicated by RAR. In addition, once IAB-node establishes a connection with a parent node, IAB-node may have T_delta, or it has own synchronization source, such as GNSS, for DU Tx timing with Case #1 timing mode.

When IAB-node supports simultaneous IAB-DU and IAB-MT operation, IAB-node may also have additional configurations, e.g. TDD pattern for IAB-MT and information of Case #6 and #7 timing modes. In that sense, IAB-node has the information of Case #1 and Case #6/#7 timing modes, and the timing mode may be changed dynamically or semi-statically since Case #6 timing mode may be required by IAB-node and Case #7 timing mode may be required by a parent node based on their resource configuration, Tx/Rx direction, and conditions of IAB-DU or the parent node MT. Thus, we conclude following proposal.

Proposal 1: Mechanism of dynamic/semi-static switching among different timing modes needs to be considered, so that IAB-node should be indicated both Case #6 and #7 timing indications

Figure 1 : Example of timing modes setting for IAB-node

At the RAN1#105-e meeting, an indication for simultaneous MT/DU operation was discussed and following agreement was made in 8.10.1 “Enhancements to resource multiplexing between child and parent links of an IAB node” [2].

Agreement

In case of simultaneous MT/DU operation,

- the parent node can dynamically indicate to the child node at least a set of restricted beams at the IAB-DU of the child node

- the child node can dynamically report to the parent node a set of recommended beams, not preferred beams, or both recommended and not preferred beams of the IAB-MT of the child node

  - FFS: Whether the specification supports all reporting combinations.

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Additional semi-static signaling

- FFS: Per-panel granularity in addition to per-beam granularity

- FFS: Relationship between child IAB-MT beam indication and parent IAB-DU beam indication

- Note: This does not preclude any enhancements for either DU or MT-based CLI measurement and reports

Agreement

In case of simultaneous MT/DU operation,

- the parent node can dynamically indicate to the child node at least a set of restricted beams at the IAB-DU of the child node

- the child node can dynamically report to the parent node a set of recommended beams, not preferred beams, or both recommended and not preferred beams of the IAB-MT of the child node

  - FFS: Whether the specification supports all reporting combinations.

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Additional semi-static signaling

- FFS: Per-panel granularity in addition to per-beam granularity

- FFS: Relationship between child IAB-MT beam indication and parent IAB-DU beam indication

- Note: This does not preclude any enhancements for either DU or MT-based CLI measurement and reports

For the simultaneous MT/DU operation, specific indications can be considered as well as the indication of beam information, and the timing mode can be also indicated together with other related indications for simultaneous MT/DU operation.

Proposal 2: Timing mode can be indicated together with other related indications for simultaneous MT/DU operation (e.g. restricted beams at IAB-DU)

  - Derivation of Tx timing of Case #6 and #7 timing mode

At the RAN1#105-e meeting, derivations of Tx timing of case #6 and #7 were discussed and following agreement was made [2].

Agreement

RAN1 to downselect how the IAB-MT Tx timing is set for Case 6 timing at a given IAB-node:

- Alt1: the IAB-MT Tx timing is obtained by the node via the legacy TA loop plus an offset from the parent node.

  - FFS details of the required offset.

- Alt2: the IAB-MT Tx timing is set by the node to the timing obtained for the node’s DL Tx.

- Alt3: the IAB-MT Tx timing is obtained by the node jointly with the IAB-DU Tx timing via a common offset from the parent node.

Downselection to consider at least the following aspects:

- Dependency of DL synchronization schemes at the IAB-DU

- Potential additional signaling overhead.

- Achievable DU Tx / MT Tx alignment error tolerance.

- Suitability for switching between timing modes.

Agreement

RAN1 to downselect how the IAB-MT Tx timing is set at an IAB-node for Case 7 timing at the parent node:

- Alt1: the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

  - FFS details of the required offset

- Alt2: the IAB-MT Tx timing of the node is obtained via the legacy TA loop from the parent node.

- Alt3: the IAB-MT Tx timing of the node is obtained via a Case 7 specific TA loop from the parent node.

Downselection to consider at least the following aspects:

- Potential impact to OTA synchronization availability for DU Tx at the IAB-node.

- Potential additional signaling overhead.

- Suitability for switching between timing modes.

Agreement

RAN1 to downselect how the IAB-MT Tx timing is set for Case 6 timing at a given IAB-node:

- Alt1: the IAB-MT Tx timing is obtained by the node via the legacy TA loop plus an offset from the parent node.

  - FFS details of the required offset.

- Alt2: the IAB-MT Tx timing is set by the node to the timing obtained for the node’s DL Tx.

- Alt3: the IAB-MT Tx timing is obtained by the node jointly with the IAB-DU Tx timing via a common offset from the parent node.

Downselection to consider at least the following aspects:

- Dependency of DL synchronization schemes at the IAB-DU

- Potential additional signaling overhead.

- Achievable DU Tx / MT Tx alignment error tolerance.

- Suitability for switching between timing modes.

Agreement

RAN1 to downselect how the IAB-MT Tx timing is set at an IAB-node for Case 7 timing at the parent node:

- Alt1: the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

  - FFS details of the required offset

- Alt2: the IAB-MT Tx timing of the node is obtained via the legacy TA loop from the parent node.

- Alt3: the IAB-MT Tx timing of the node is obtained via a Case 7 specific TA loop from the parent node.

Downselection to consider at least the following aspects:

- Potential impact to OTA synchronization availability for DU Tx at the IAB-node.

- Potential additional signaling overhead.

- Suitability for switching between timing modes.

As discussed in section 2.1, IAB-node has information of Case #1 timing mode for non-simultaneous operation, so that the legacy TA loop is necessary for Case #1 timing mode. Therefore the legacy TA loop can be a baseline for the derivation of MT Tx timing for Case #6 and #7. In addition, it’s better to define the same approach (alternative) for Case #6 and #7 Tx timing derivation to achieve a unified mechanism. Therefore, the legacy TA loop and an offset value can be a candidate solution for MT timing derivation for both Case #6 and #7 timing mode. The offset value for Case #6 timing mode can be T_delta which IAB-node may have for IAB-DU timing derivation.

Proposal 3: Legacy TA loop and offset value should be used for MT Tx timing derivation for both Case #6 and #7 timing modes (Alt1).

Case #6 and #7 timing mode can be applied for Rel-17 IAB-nodes, and UEs and Rel-16 IAB-nodes support Case #1 timing mode. Therefore reception timing at parent nodes for configured resources, e.g. CG-PUSCH, PUCCH for SR, need to be aligned. One of the simple rule is that Case #1 timing mode needs to be applied for the transmission of CG-PUSCH, PUCCH (e.g. for SR), SRS to align the reception timing of signals transmitted by Rel-16 IAB-node, UEs and Rel-17 IAB-node at parent nodes.

Observation 1: Case #1 timing mode should be applied for transmission of CG-PUSCH, PUCCH (e.g. for SR), SRS to align the reception timing of signals transmitted by Rel-16 IAB-node, UEs and Rel-17 IAB-node.

- Discussion on UL power adjustment between DU and MT

Agreement

Decide in RAN1#106-e whether to support an IAB-node indicating assistance information to help with its MT’s UL TX power control. The assistance information can be:

- FFS: Desired TX power

- FFS: Offset to a baseline PHR

- FFS: Desired dynamic range

FFS: whether this information is provided to the parent-node, the CU, or both.

FFS: whether the MT’s UL TX power control formula needs to be changed

Agreement

Decide in RAN1#106-e whether to support an IAB-node indicating assistance information to help with its MT’s UL TX power control. The assistance information can be:

- FFS: Desired TX power

- FFS: Offset to a baseline PHR

- FFS: Desired dynamic range

FFS: whether this information is provided to the parent-node, the CU, or both.

FFS: whether the MT’s UL TX power control formula needs to be changed

At the RAN1#105-e meeting, IAB-node UL power adjustment was discussed and following agreements were made [2].

For the simultaneous operation of MT and DU transmission by IAB-node’s child and parent links using a single antenna panel or transceiver with FDM, the Tx power difference between MT and DU needs to be smaller, since out of band emission of the stronger signal may degrade the performance of the weaker signal. In order to configure smaller Tx power difference between MT and DU, IAB-node can send an information of desired IAB-MT Tx power, which may be higher than the Tx power controlled by parent node.

Proposal 4: IAB-node reports desired TX power for IAB-MT as an assistance information.

When IAB-node reports the desired IAB-MT Tx power, IAB-node may have two different IAB-MT Tx power, e.g. the desired Tx power for simultaneous transmission and the Tx power controlled by a parent node for non-simultaneous transmission. Therefore, IAB-node needs the indication of which Tx power can be applied for the simultaneous transmission from a parent node.

Proposal 5: Applicability of the desired Tx power for IAB-MT transmission needs to be indicated by parent node.

- Discussion on interference management for IAB

Agreement

RAN1 to select among the following options to support DU-to-DU measurement and report.

- For DU-to-DU CLI measurement:

  - Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 1.2. enhanced legacy DU-based measurement procedures (e.g., enhanced Rel-16 RIM)

  - Option 1.3. enhanced MT-based measurements (e.g., MT-based CLI, MT RRM measurements)

- For DU-to-DU CLI report:

  - Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 2.2. enhanced legacy DU-based report (e.g., enhanced Rel-16 RIM)

  - Option 2.3. enhanced MT-based report (e.g., MT-based CLI, MT RRM measurements)

Agreement

RAN1 to select among the following options to support DU-to-DU measurement and report.

- For DU-to-DU CLI measurement:

  - Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 1.2. enhanced legacy DU-based measurement procedures (e.g., enhanced Rel-16 RIM)

  - Option 1.3. enhanced MT-based measurements (e.g., MT-based CLI, MT RRM measurements)

- For DU-to-DU CLI report:

  - Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 2.2. enhanced legacy DU-based report (e.g., enhanced Rel-16 RIM)

  - Option 2.3. enhanced MT-based report (e.g., MT-based CLI, MT RRM measurements)

At the RAN1#104-e meeting, interference management for IAB-node was discussed and following agreement was made [3].

Cross link interference (CLI) for IAB is one of the discussion points in Rel-17 IAB discussion, and interference between IAB-nodes is shown in Fig.3. Example of DU to DU interference is shown in Fig.3 (a). Interference due to DL signals transmitted by IAB-DU may be handled by the deployment of IAB nodes. Example of MT to MT interference is shown in Fig.3 (b). Interference due to UL signals transmitted by IAB-MT may be handled by using mechanism of CLI as in Rel-16, e.g. IAB node #2 may measure SRS transmitted by IAB node #1 MT and report SRS RSRP to provide the information whether the IAB node #1 MT Tx power is sufficient for IAB node #2 or not. Therefore, no additional mechanism is necessary for the IAB interference management.

Proposal 6: No additional mechanism is necessary for IAB interference management.

- DU to DU interference				 (b) MT to MT interference

Figure 2: Interference between IAB nodes

- Conclusion

In this contribution, we discussed enhancements on IAB-node timing mode(s), extensions for DUL power control, and interference management to support simultaneous operation. Based on the discussion, we made following proposals.

Proposal 1: Mechanism of dynamic/semi-static switching among different timing modes needs to be considered, so that IAB-node should be indicated both Case #6 and #7 timing indications

Proposal 2: Timing mode can be indicated together with other related indications for simultaneous MT/DU operation (e.g. restricted beams at IAB-DU)

Proposal 3: Legacy TA loop and offset value should be used for MT Tx timing derivation for both Case #6 and #7 timing modes (Alt1).

Observation 1: Case #1 timing mode should be applied for transmission of CG-PUSCH, PUCCH (e.g. for SR), SRS to align the reception timing of signals transmitted by Rel-16 IAB-node, UEs and Rel-17 IAB-node.

Proposal 4: IAB-node reports desired TX power for IAB-MT as an assistance information.

Proposal 5: Applicability of the desired Tx power for IAB-MT transmission need to be indicated by parent node.

Proposal 6: No additional mechanism is necessary for IAB interference management.

References

- RP-193251, New WID on Enhancements to Integrated Access and Backhaul, Dec. 2019

- 3GPP, RAN1#105e, RAN1 Chairman’s Notes, May 2021.

- 3GPP, RAN1#104e, RAN1 Chairman’s Notes, Feb. 2021.