3GPP TSG RAN WG1 #106-e                                                                                            R1-2107972

e-Meeting, August 16th – 27th, 2021

Title:	Draft reply LS on physical layer aspects of small data transmission

Response to:	R2-2106561

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	RAN1

To:	RAN2

Cc:

Attachments:

# Contact Person:

# Name:

E-mail Address:

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS on physical layer aspects of small data transmission in R2-2106561.

RAN1 has discussed the following question regarding the CG-SDT and would like to provide response as follows.

- Working assumption: UE-specific search space is configured for UEs performing CG-SDT. RAN2 asks RAN1 whether this working assumption can be confirmed

Answer: RAN1 confirms this working assumption on UE-specific search space configured for UEs performing CG-SDT.

- CG-SDT resource can be configured on either initial BWP or separate SDT BWP. Ask RAN1 to confirm

Answer: RAN1 concluded that either initial BWP or separate SDT BWP can be configured for CG-SDT.

- RAN2 thinks that some feedback may be beneficial in case CG is used for subsequent transmission. RAN2 assumes that existing mechanism can be used.

Answer: RAN1 discussed the potential feedback for CG-SDT. RAN1 thinks that the existing mechanism e.g. DL/UL grant scheduling subsequent transmission can be used for the feedback response to CG-SDT transmission.

Regarding RA-SDT, RAN1 discussed the configuration parameters for the PRACH resource configuration when PRACH occasions are shared between SDT and non-SDT and when PRACH occasions are separately configured for SDT and non-SDT. Following configurations for RA-SDT are provided from RAN1.

For RA-SDT, when PRACH occasions are separate between SDT and non-SDT, the PRACH resource configurations are separately configured for both 4-step RACH and 2-step RACH, including time resources and frequency resources of PRACH resources, preamble formats, root sequence and total number of preambles for SDT. For RA-SDT, when PRACH occasions are shared between SDT and non-SDT, the preambles for SDT and non-SDT in the shared ROs are separately configured. - the preambles for SDT in the shared RO are determined from the non-CBRA preambles for non-SDT RA type. For RA-SDT, when PRACH occasions are shared or separate between SDT and non-SDT, - the existing mapping rule of SSB-to-preamble mapping for non-SDT can be reused - the SCS of preamble for SDT can be reused from that for non-SDT, i.e. configured by msg1-subcarrierSpacing. For power control for PRACH of RA-SDT, - the preambleReceivedTargetPower for non-SDT RA is reused. - The power ramping counter is common for both non-SDT and SDT RA. - For SDT RACH with shared ROs, powerRampingStep for SDT RACH are indicated by that for the non-SDT RACH. For SDT RACH with separated ROs, powerRampingStep for SDT RACH can be separately configured.

# 2. Actions:

RAN1 respectfully asks RAN2 to take into account the above information.

# 3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting #106b-e				11th Oct – 19th Oct 2021		                              		e-meeting

TSG-RAN WG1 Meeting #107-e			11th Nov – 19th Nov 2021 	    		     			e-meeting