3GPP TSG RAN WG1#106-e		R1-2107976

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussion on frequency hopping for multi-PUSCH scheduling

Agenda Item:	7.2.2

Document for:	Discussion and Decision

- Introduction & Background

In RAN#82 meeting, the WID of the NR access to the unlicensed spectrum has been approved [1]. The following physical layer procedures have been identified as the objectives of the work item.

-	Physical layer procedure(s) including [RAN1, RAN2]:

- 	HARQ operation: NR HARQ feedback mechanisms are the baseline for NR-U operation with extensions in line with agreements during the study phase (NR-U TR section 7.2.1.3.3), including immediate transmission of HARQ A/N for the corresponding data in the same shared COT as well as transmission of HARQ A/N in a subsequent COT. Potentially support mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities. (RAN1)

-	Scheduling multiple TTIs for PUSCH in-line with agreements from the study phase (TR 38.889, Section 7.2.1.3.3). (RAN1)

In RAN#86 meeting, the work item has been completed with some remaining issues. In this contribution, we discuss frequency hopping for multi-PUSCH scheduling.

- Discussion

In TS38.214 v16.6.0 [2], there are some descriptions as follows.

6.1.2.1	Resource allocation in time domain

……

-	for PUSCH scheduled by DCI format 0_1, if pusch-RepTypeIndicatorDCI-0-1  is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. For PUSCH scheduled by DCI format 0_2, if pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. Otherwise, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH.

……

Since multi-PUSCH scheduling in conjunction with PUSCH repetition Type B has not been discussed so far, it is not expected to set pusch-RepTypeIndicatorDCI-0-1 to 'pusch-RepTypeB', so multi-PUSCH scheduling by DCI format 0_1 should follow PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH.

Observation 1: Multi-PUSCH scheduling by DCI format 0_1 follows PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH.

In TS38.214 v16.6.0 [2], the following description was agreed during RAN1#104-e meeting.

6.1.2.1	Resource allocation in time domain

……

If a UE is configured with higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, the UE does not expect to be configured with pusch-AggregationFactor.

……

Based on the above description, when pusch-TimeDomainAllocationListForMultiPUSCH is configured, no actual PUSCH repetition with Type A will be expected, even only a single PUSCH is scheduled in actual based on one row from pusch-TimeDomainAllocationListForMultiPUSCH. In other words, no multi-slot PUSCH transmission will be expected if multi-PUSCH scheduling is configured or enabled.

Observation 2: No multi-slot PUSCH transmission will be expected if multi-PUSCH scheduling is configured or enabled.

In TS38.214 v16.6.0 [2], frequency hopping for PUSCH repetition Type A is specified as follows.

6.3.1	Frequency hopping for PUSCH repetition Type A

For PUSCH repetition Type A (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. One of two frequency hopping modes can be configured:

-	Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission.

-	Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

……

Based on the above description, since inter-slot frequency hopping is only applicable to multi-slot PUSCH transmission, only intra-slot frequency hopping can be considered for multi-PUSCH scheduling.

In actual, multiple PUSCHs scheduled by a single DCI without repetition are totally the same as that scheduled by respective DCI where each DCI schedules only a single PUSCH without repetition, in terms of transmission properties. In other words, multi-PUSCH scheduling can be regarded as single slot transmission. Therefore, intra-slot frequency hopping is applicable to multi-PUSCH scheduling naturally.

Proposal 1: Clarify that intra-slot frequency hopping is applicable to multi-PUSCH scheduling, while inter-slot frequency hopping is not applicable to multi-PUSCH scheduling.

- Conclusion

In this contribution, frequency hopping for multi-PUSCH scheduling has been discussed, and the proposals and observations made are summarized below:

Observation 1: Multi-PUSCH scheduling by DCI format 0_1 follows PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH.

Observation 2: No multi-slot PUSCH transmission will be expected if multi-PUSCH scheduling is configured or enabled.

Proposal 1: Clarify that intra-slot frequency hopping is applicable to multi-PUSCH scheduling, while inter-slot frequency hopping is not applicable to multi-PUSCH scheduling.

- References

- Chairman’s Notes, RAN#82, Sorrento, Italy, Dec. 10-13, 2018.

- TS 38.214, Physical layer procedures for data, v16.6.0, Jun. 2021.