3GPP TSG RAN WG1 #106-e	R1-2106413

e-Meeting, August 16th – 27th, 2021

3GPP TSG-RAN WG2 Meeting #114-e						R2-2106623

E-Meeting, May 2020

Title:	LS on time gap information in SCI

Response to:

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

#### Name:	Qianxi Lu

E-mail Address:	<qianxi.lu@oppo.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

In RAN2#113bis, RAN2 made the following working assumption

19:	Working assumption: SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource. FFS whether explicitly configured SL HARQ RTT timer may be still required. If big problem is identified next meeting, we can revisit it.

This working assumption was made based on the assumption that the RX UE can determine the time location of the next retransmission resource(s) of the TX UE (assuming that resource reserved by SCI is not reselected by the TX UE due to e.g. pre-emption/UL-SL prioritization) based on the “time resource assignment” field in SCI. In RAN2#114, some companies believed this is not feasible, while others believed that the network always guarantees that this is feasible. RAN2 would therefore like to ask RAN1.

Q1: For R17 SL DRX design, from RAN1 perspective, whether it is feasible for the Rx-UE to determine the time location of the next retransmission resource(s) of the TX UE (assuming that resource is not reselected by the TX UE) based on the “Time resource assignment” field in SCI?

2. Actions:

To RAN WG1: RAN2 respectfully requests RAN1 to feedback on Q1 above.

3. Dates of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #115-e	16 August – 27 August 2021	eMeeting

TSG RAN WG2 Meeting #116-e	1 November– 12 November 2021	eMeeting