3GPP TSG RAN WG1 #106bis-e			R1-2109907

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.2.4

Source:	Moderator (InterDigital, Inc.)

Title:	Discussion Summary #5 for Beam Management for new SCSs

Document for:	Discussion and Decision

Introduction

In this contribution, we summarize all issues discussed on beam management and timings associated with beam-based operation for new SCSs to support NR from 52.6 GHz to 71 GHz in RAN#106bis-e.

Timings Associated with Beam-based Operation

Support of additional values of beamSwitchTiming, beamReportTiming and timeDurationForQCL

Observations and Proposals from Contributions

| - Company | - Observations and Proposals from Contributions |
| --- | --- |
| - [Huawei/HiSi, 1] | Proposal 1: There is no need to introduce any additional candidate value of “timeDurationForQCL”, “beamSwitchTiming” and “beamReportTiming” for 120kHz, 480kHz and 960kHz SCS. (Alt-1) |
| - [FUTUREWEI, 2] | Proposal 5: Support Alt-1: For candidate values of timeDurationForQCL, beamSwitchTiming and beamReportTiming no additional candidate values are supported for 120 kHz, 480 kHz and 960 kHz |
| - [ZTE/Sanechips, 4] | Proposal 2: For candidate values of timeDurationForQCL, beamSwitchTiming and beamReportTiming, Alt-1 is preferred. - Alt-1: No additional candidate values are supported for 120 kHz, 480 kHz and 960 kHz |
| - [vivo, 5] | Proposal 1: For timeDurationForQCL, no additional candidate value is needed for 480 and 960 kHz. |
| - [CATT, 8] | Proposal 4: 28 and 56 symbols are supported as additional candidate values for 480 kHz and 960 kHz for the timeDurationForQCL, beamSwitchTiming and beamReportTiming. |
| - [xiaomi, 9] | Proposal 1: For candidate values of timeDurationForQCL, beamSwitchTiming and beamReportTiming, Support Alt-1. |
| - [Ericsson, 10] | Proposal 5 For candidate values of timeDurationForQCL, beamSwitchTiming and beamReportTiming, support Alt-2 from the RAN1#106-e agreement, i.e., 28 and 56 symbols are supported as additional candidate values for 480 kHz and 960 kHz, respectively |
| - [Samsung, 12] | Proposal 1: Support Alt-1 (no additional candidate values) for timeDurationForQCL, beamSwitchTiming and beamReportTiming for new SCSs |
| - [MediaTek, 13] | Proposal 1: Defer the discussion of additional values of timeDurationForQCL, beamSwitchTiming and beamReportTiming for 480 kHz and 960kHz till the BD/CCE limits are specified for 480kHz and 960kHz. |
| - [Intel, 14] | Proposal 1: Support additional candidate values of 28 OFDM symbols and 56 OFDM symbols for SCS 480 kHz and SCS 960 kHz, respectively, for each of the parameters: timeDurationQCL, beamReportTiming and beamSwitchTiming. |
| - [InterDigital, 18] | Observation 1: The current minimum candidate values, 56 and 12 symbols, take only 125 us, which is short enough for efficient operation. Observation 2: If 28 and 56 symbols are supported for 480 kHz and 960 kHz, UE needs to receive and decode PDCCH, change UE reception beam for PDSCH and generate beam report within 62.5 us, which can lead processing burden for the UE implementation. Proposal 1: No additional candidate values are supported for 480 kHz and 960 kHz (Alt-1). |
| - [LGE, 19] | Proposal #1: For candidate values of timeDurationForQCL, beamSwitchTiming and beamReportTiming, 28 and 56 symbols are supported as additional candidate values for 480 kHz and 960 kHz, respectively (i.e., Alt-2 in the agreement made in RAN1#106-e). |
| - [Qualcomm, 22] | Proposal 4: For timeDurationForQCL, beamSwitchTiming and beamReportTiming, no additional candidate values are supported for 120 kHz, 480 kHz and 960 kHz. |

Summary of views

In RAN1#106-e, the following agreement on timeDurationForQCL. beamSwitchTiming and beamReportTiming is agreed.

Agreement: For candidate values of timeDurationForQCL, beamSwitchTiming and beamReportTiming, - Support one of the following alternatives - Alt-1: No additional candidate values are supported for 120 kHz, 480 kHz and 960 kHz - Alt-2: 28 and 56 symbols are supported as additional candidate values for 480 kHz and 960 kHz, respectively - For UE capability signaling, UE reports one value of the candidate values in OFDM symbols per each SCS

Based on the above agreement, the following companies’ views are observed.

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1.1 | Support additional values of beamSwitchTiming, beamReportTiming and timeDurationForQCL | No additional candidate values (Alt-1) - Huawei/HiSi, FUTUREWEI, ZTE/Sanechips, vivo, xiaomi, Samsung, IDCC, Qualcomm - [Huawei/HiSi]: The motivation to introduce additional candidate values are not clear. Due to limited time in this agenda item, we do not see a need to introduce any additional candidate values for these parameters. - [Qualcomm]: For the candidate values of timeDurationForQCL, beamSwitchTiming, and beamReportTiming, we prefer not to support 28 and 56 symbols as additional candidate values for 480 kHz and 960 kHz, respectively. Those values are too short compared with scaled values from 120 kHz. Support 28 and 56 symbols for 480 kHz and 960 kHz, respectively (Alt-2) - CATT, Ericsson, Intel, LGE - [Ericsson]: Since these are only candidate values, this does not mandate the UE to support them. However, if the UE is capable of supporting such values it makes for more responsive (faster) beam management, and we think this is good for the technology. - [Intel]: However, the simple scaling means no speed up in beam management operations for the new NR UE devices, which are intended to work on these frequencies, comparing with current UE devices operating in NR FR2. Therefore, it seems necessary to provide optional UE capabilities for faster beam switching and beam reporting assuming some reasonable progress in signal processing of UE devices for NR extension from 52.6 GHz up to 71 GHz. Hold the discussion - MediaTek - [MediaTek]: Defer the discussion of additional values of timeDurationForQCL, beamSwitchTiming and beamReportTiming for 480 kHz and 960kHz till the BD/CCE limits are specified for 480kHz and 960kHz. |

1st round discussion

Observation 1

For timeDurationForQCL, beamSwitchTiming and beamReportTiming, majority of companies indicated no additional candidate values (Alt-1). The moderator requests additional inputs of the companies which did not express their positions. In addition, MediaTek proposed to defer the discussion until BD/CCE limits are specified for 480kHz and 960kHz. The moderator asks MediaTek’s detailed technical background to defer the discussion.

Proposal 1

For candidate values of timeDurationForQCL, beamSwitchTiming and beamReportTiming,

- No additional candidate values are supported for 120 kHz, 480 kHz and 960 kHz

- Note: this is Alt-1 from the RAN1#106 agreement.

| Company | Input |
| --- | --- |
| LG Electronics | We support Alt-2, i.e., 28 and 56 symbols for 480 kHz and 960 kHz, respectively. We couldn’t understand why we cannot support additional values as UE capability. |
| OPPO | support |
| Ericsson | Agree with LGE. These are candidate values, hence it seems reasonable not to support them for UEs that are capable of faster beam management. |
| Lenovo, Motorola Mobility | We support the Proposal 1 |
| vivo | Support Proposal 1. |
| Futurewei | Support proposal-1. Questions on Feasibility of new values have to be settled first. |
| Nokia/NSB | Do not support. Share view with LGE and Ericsson. |
| MediaTek | Thanks to Moderator for asking the reasoning of our proposal. The existing candidate values in Rel-15/16 timeDurationForQCL, beamSwitchTiming and beamReportTiming have been verified to achieve the balance of implementation and system efficiency, which is the reason we support reusing the same associated absolute time duration of 120kHz for the candidate values of 480kHz and 960kHz in previous agreement. For the new candidate values, we are open to discuss but it is important to us the evaluation of new values. To achieve this, we at least need the BD/CCE limit of PDCCH monitoring for 480kHz and 960kHz since timeDurationForQCL, beamSwitchTiming and beamReportTiming are related to how fast UE can obtain the control information for the upcoming scheduling. The agreement for the basic PDCCH monitoring capability for 480kHz and 960kHz is still missing and we simply can’t justify the smaller values from the implementation point of view. However, we are also aware that there are only two meetings left and some conclusion on the new values should be made sooner than later. Therefore, we would like to hear the technical background on 28symbols for 480kHz and 56 symbols for 960kHz to facilitate the discussion. We don’t against the new values but we just prefer to be more careful on determining the new candidate values to ensure those values are meaningful. I hope this clarify our intention. |
| Qualcomm | Support Proposal 1 |
| Apple | Support Proposal 1. Our view is that adopting smaller value should be justified by a possibility to implement it in a meaningful future by some chipset vendors. The approach of introducing smaller values with ‘optional’ UE capability is not enough; otherwise, we can even introduce a smaller values than those proposed in Alt-2. |
| Samsung | Support Proposal 1 |
| Huawei, HiSilicon | Support Proposal 1 |

1st round discussion summary

TBU

maxNumberRxTxBeamSwitchDL

Observations and Proposals from Contributions

| - Company | - Observations and Proposals from Contributions |
| --- | --- |
| - [Huawei/HiSi, 1] | Proposal 2: For maxNumberRxTxBeamSwitchDL, support 4 as a candidate value for 960kHz SCS in addition to the agreed candidate value 2. (Alt-2) Proposal 4: UL-DL or DL-UL w/o spatial domain filter change should be counted as a beam switch in maxNumberRxTxBeamSwitchDL. |
| - [FUTUREWEI, 2] | Proposal 3: Support Alt-1: For maxNumberRxTxBeamSwitchDL support {1, 4 and [7]} as candidate values for 960 kHz in addition to the agreed candidate values 2. |
| - [Spreadtrum, 3] | Proposal 1: For maxNumberRxTxBeamSwitchDL, support 1 and 4 for 960 kHz. |
| - [ZTE/Sanechips, 4] | Proposal 1: For FR2-2, the values of maxNumberRxTxBeamSwitchDL in Alt-1 are preferred for 960 kHz SCS. - Alt-1: Support 1, 4 and 7 as candidate values for 960 kHz in addition to the agreed candidate value 2 |
| - [vivo, 5] | Proposal 2: the definition of maxNumberRxTxBeamSwitchDL should be discussed in case of different numerology configuration for DL and UL. |
| - [OPPO, 6] | Proposal 1: Support 1 and 4 as candidate values for 960 kHz in addition to the agreed candidate values 2. |
| - [CATT, 8] | Observation 2: When SCS is 480 kHz or 960 kHz, the duration of each OFDM symbol would be shorter. UE may not support performing beam switching as much as 14 times within a slot. Proposal 3: For SCS 480/960 kHz, the candidate value of maxNumberRxTxBeamSwitchDL can be {2, 4, 7} and {1, 2, 4} respectively. |
| - [Ericsson, 10] | Proposal 7 For maxNumberRxTxBeamSwitchDL for 960 kHz, support Alt-2 from the RAN1#106-e agreement, i.e., support candidate values 2 and 4 for 960 kHz SCS. |
| - [Nokia/NSB, 11] | Proposal 1: Support 4 as a candidate value for 960 kHz in addition to the agreed candidate values 2. |
| - [Samsung, 12] | Proposal 2: Support Alt-2 (Support 4 as a candidate value for 960 kHz) for maxnumberRxTxBeamSwitchDL |
| - [MediaTek, 13] | Proposal 2: For maxNumberRxTxBeamSwitchDL for 960kHz, support {1,4,[<=7]} in the condition that [<=7] should be discussed after the RAN4 decision on the beam switching time for 960kHz. |
| - [Intel, 14] | Proposal 2: For maxNumberRxTxBeamSwitchDL in case of SCS 960 kHz: Candidate value set is {1, 2, 4, 7} switches. |
| - [InterDigital, 18] | Observation 4: Switching Rx/Tx beam 7 times within 15.625 us can be serious burden for UE implementation. Proposal 3: Support 4 as a candidate value for 960 kHz (Alt-2). |
| - [LGE, 19] | Proposal #3: For maxNumberRxTxBeamSwitchDL of 960 kHz, support 4 as a candidate value in addition to the agreed candidate values 2 (i.e., Alt-2 in the agreement made in RAN1#106-e). |
| - [Qualcomm, 22] | Proposal 3: For maxNumberRxTxBeamSwitchDL, support at least 1 and 4 as candidate values for 960 kHz in addition to the agreed candidate value 2. |

Summary of views

In RAN1#106-e, the following agreements on maxNumberRxTxBeamSwitchDL are agreed.

Agreement: For maxNumberRxTxBeamSwitchDL, - Support at least 2 and 4 as candidate values for 480 kHz - FFS: 7 - Support at least 2 as a candidate value for 960 kHz - FFS: Support for additional candidate value(s) including 4 Agreement: For maxNumberRxTxBeamSwitchDL, - For 480 kHz, support 7 as a candidate value for 480 kHz in addition to the agreed candidate values 2 and 4 - For 960 kHz, support one of the following alternatives - Alt-1: Support 1, 4 and [7] as candidate values for 960 kHz in addition to the agreed candidate values 2 - Alt-2: Support 4 as a candidate value for 960 kHz in addition to the agreed candidate values 2 - No additional candidate values are supported

Based on the above agreement, the following companies’ views are observed.

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1 | maxNumberRxTxBeamSwitchDL | Support 1, 4 and [7] as additional candidate values for 960 kHz (Alt-1) - FUTUREWEI, Spreadtrum (without 7), ZTE/Sanechips, Oppo (without 7), CATT (without 7), MediaTek, Intel, Qualcomm (without 7) - [CATT]: When SCS is 480 kHz or 960 kHz, the duration of each OFDM symbol would be shorter. UE may not support performing beam switching as much as 14 times within a slot. - [MediaTek]: For maxNumberRxTxBeamSwitchDL for 960kHz, support {1,4,[<=7]} in the condition that [<=7] should be discussed after the RAN4 decision on the beam switching time for 960kHz. Support 4 as an additional candidate value for 960 kHz (Alt-2) - Huawei/HiSi, Ericsson, Nokia/NSB, Samsung, IDCC, LGE - [Samsung]: Since the candidate value {7} for 480 kHz has been agreed, we think ‘4’ for 960 kHz which has similar capability can be supported. However, in our view, the value ‘1’ is too restrictive for the UE which supports multi-panel operation. Consequently, we support Alt-2 for maxNumberRxTxBeamSwitchDL - [IDCC]: Switching Rx/Tx beam 7 times within 15.625 us can be serious burden for UE implementation. |

1st round discussion

Observation 2

For maxNumberRxTxBeamSwitchDL, it is observed that majority of companies indicated their support on 1 and 4 as additional candidate values for 960 kHz (Alt-1 without [7]). Based on the majority view, the moderator provides proposal 2. Please continue discussion based on proposal 2.

Proposal 2

For maxNumberRxTxBeamSwitchDL, support 1 and 4 as candidate values for 960 kHz in addition to the agreed candidate value 2.

- Note: this is Alt-1 without [7] from the RAN1#106 agreement.

| Company | Input |
| --- | --- |
| LG Electronics | Support Proposal 2. |
| OPPO | support |
| Ericsson | We are fine to remove [7]. We support candidate value 4. However, we think that 1 beam switch per slot is too low a number. Consider a CSI-RS resource set configured with repetition = ‘on’ (i.e., the so-called P3 procedure for UE Rx beam refinement). Given that CSI-RS resource sets are restricted to 1 slot, this means that a maximum of 2 CSI-RS resources could be configured for L1-RSRP measurements. This is very restrictive, e.g., for multi-panel UEs as pointed out by Samsung. |
| Lenovo, Motorola Mobility | We support proposal 2 |
| vivo | Support proposal 2 |
| Futurewei | Support proposal 2 |
| Nokia/NSB | Do not support. We have the same view as Ericsson. We are fine to remove 7 but also want to remove 1 which is too restrictive for multi-beam management not supporting measuring of multiple SSB or CSI-RS resources in a slot. |
| MediaTek | Support the proposal. |
| Qualcomm | Support Proposal 2 |
| Apple | We are fine with Proposal 2. It should be noted that value of ‘1’ is the lowest value and advanced UE can always indicate the larger value, e.g., 4 for 960kHz SCS. If we compare BM efficiency with this value of 960kHz SCS with 120kHz SCS case, value of ‘1’ with 960kHz SCS equivalently means 8*2=16 times of beam switching within a 120kH slot duration, which is quite faster compared to the values defined for 120kHz SCS. In other words, even this value already makes it challenging to reuse the existing hardware of 120kHz SCS for higher SCS. |
| Samsung | Do not support. As we mentioned in our tdoc, the value 1 seems too restrictive. |
| Huawei, HiSilicon | We prefer to support {2,4} only. |

1st round discussion summary

TBU

Additional beam switching time delay d

Observations and Proposals from Contributions

| - Company | - Observations and Proposals from Contributions |
| --- | --- |
| - [Huawei/HiSi, 1] | Proposal 3: For the additional beam switching time delay d, when triggering PDCCH with 120kHz (480kHz) has a smaller subcarrier spacing than AP-CSI-RS or PDSCH, the supported value is obtained by multiplying a factor of two (eight) to the corresponding value for 60 kHz SCS. |
| - [ZTE/Sanechips, 4] | Proposal 3: The following values can be considered for additional beam switching time delay d for triggering AP-CSI-RS when triggering PDCCH with 120/480kHz has a smaller SCS than AP-CSI-RS. µPDCCH; d [PDCCH symbols]. 3; 28. 5; 56. |
| - [CATT, 8] | Proposal 5: 28 symbols are supported as additional candidate values for 120 kHz for the additional beam switching time delay d. |
| - [Ericsson, 10] | Proposal 6 For additional beam switch delay d, support Alt-1 from the RAN1#106-e agreement, i.e., d = 14 symbols for 120 kHz. For 480 kHz support the value for 120 kHz scaled by 4x, i.e., d = 56 symbols for 480 kHz. |
| - [Nokia/NSB, 11] | Proposal 2: Additional beam switching time delay d of 120 kHz is 14 symbols. Proposal 3: Additional beam switching time delay d of 480 kHz is 28 symbols. |
| - [Samsung, 12] | Proposal 3: Support Alt-2 (28 symbol) for 120 kHz and 56 symbol for 480 kHz for additional beam switching time delay d |
| - [MediaTek, 13] | Proposal 3: For the additional beam switching time delay d of 120 kHz and 480kHz when the scheduling DCI SCS 15×2μPDCCH is smaller than the triggered aperiodic CSI-RS SCS 15×2μCSIRS, support d=28 symbols for DCI SCS=120kHz and d=112 symbols for DCI SCS=480kHz. |
| - [Intel, 14] | Proposal 3: For additional beam switching delay d, support [14] PDCCH symbols when μPDCCH=3 (SCS 120 kHz), support [56] PDCCH symbols when μPDCCH=5 (SCS 480 kHz). |
| - [InterDigital, 18] | Observation 3: As well as timeDurationForQCL, beamSwitchTiming and beamReportTiming, further processing time reduction is difficult considering the similar DCI payload size and the more complex UE implementation. Proposal 2: It is preferred to support values 28 symbols for 120 kHz (Alt-2) and 112 symbols for 480 kHz, respectively. |
| - [LGE, 19] | Proposal #2: For beam switching time delay d of 120 kHz, support 28 symbols (i.e., Alt-2 in the agreement made in RAN1#106-e). In addition, for beam switching time delay d of 480 kHz, support 112 symbols. |
| - [Apple, 20] | Proposal 1: For additional beam switching time delay ‘d’, reuse the absolute time defined for 60kHz i.e., 28 symbols for 120kHz and 112 symbols for 480kHZ. |
| - [Qualcomm, 22] | Proposal 5: For additional beam switching time delay d for PDCCH SCS = 120 kHz, support 28 symbols. |

Summary of views

In RAN1#106-e, the following agreements on additional beam switching time delay d are agreed.

Agreement: For the threshold values 48 or 48+ d∙2μCSIRS/2μPDCCH mentioned in Clauses 5.2.1.5.1 and 5.2.1.5.1a of 38.214, scale 48 to 4*48 for 480 kHz and 8*48 for 960 kHz. Agreement: - For additional beam switching time delay d of 120 kHz, support one of the following alternatives - Alt-1: 14 symbols - Alt-2: 28 symbols - FFS: value for 480 kHz

Based on the above agreement, the following companies’ views are observed.

| # | Issue | Companies’ views |
| --- | --- | --- |
| 3.1 | Value of additional beam switching time delay d for 120 kHz | 14 symbols (Alt-1) - Ericsson, Nokia/NSB, Intel - [Ericsson]: An upper bound on the value of the additional beam switching delay for cross-carrier triggering of aperiodic CSI-RS on carriers with different numerologies is d = 14 and 56 for µPDCCH = 3 and 5, respectively. Further discuss if these values can be tightened, e.g., by a factor of 2. 28 symbols (Alt-2) - Huawei/HiSi, ZTE/Sanechips, CATT, Samsung, MediaTek, IDCC, Apple, Qualcomm, LGE - [Apple]: Given the fact that the higher frequency band is sort of FR2-1 extension to beyond 52.6GHz, processing time for new SCSs is preferrable to allow reusing the existing FR2-1 hardware regardless of SCS. It is the reason that RAN1 agreed the following in the RAN1 #104 e-meeting as a general guideline for timeline related discussions [1] |
| 3.2 | Value of additional beam switching time delay d for 480 kHz | 28 symbols - Nokia/NSB, Intel - [Nokia/NSB]: As already the main delay component is scaled with 4 or 8 for 480 and 960 kHz, respectively, compared to 120 kHz, the additional beam switching timing delay d does not need to scale similarly. 56 symbols - ZTE/Sanechips, Ericsson - [Ericsson]: For additional beam switch delay d, support Alt-1 from the RAN1#106-e agreement, i.e., d = 14 symbols for 120 kHz. For 480 kHz support the value for 120 kHz scaled by 4x, i.e., d = 56 symbols for 480 kHz.112 symbols (Value of 60 kHz SCS scaled by 8x) 112 symbols - Huawei/HiSi, MediaTek, IDCC, Apple, LGE - [LGE]: For beam switching time delay d of 120 kHz, support 28 symbols (i.e., Alt-2 in the agreement made in RAN1#106-e). In addition, for beam switching time delay d of 480 kHz, support 112 symbols. |

1st round discussion

Observation 3

For 120kHz, majority of companies indicated their support on 28 symbols while 4 other companies support 14 symbols. Based on the majority view, the moderator provides For 480 kHz, no proposal supported by majority companies is observed. Please continue discussion.

Please indicate your preference on beam switching time delay d for 480 kHz.

| Company | Input |
| --- | --- |
| Ericsson | As we discuss below, we think that for 120 kHz d should be 14 symbols. For 480 kHz we agree with Nokia’s view that “the main delay component is scaled with 4 or 8 for 480 and 960 kHz, respectively, compared to 120 kHz, the additional beam switching timing delay d does not need to scale similarly.” Hence, we can be open to 2 x 14 = 28 symbols in addition to our original proposal of 4 x 14 = 56 symbols. |
| Nokia/NSB | As we already indicated, no need for further scaled up of “d”. We have spent already 4 slots of 120kHz SCS. Further addition of 2 slots or 8 slots strongly restrict the scheduling of AP-CSI RS with given TDD frame structure. Support 14 symbols for 120kHz, and 28 symbols for 480kHz. |
| MediaTek | Unless there will be capability signaling for this parameter, we prefer to use the same scaling principle for 120kHz and 480kHz to cover the required processing time in all the scenarios. |
| Qualcomm | We prefer 112 symbols for 480 kHz |
| Samsung | In order to keep similar capability as 120 kHz d, we prefer 112 symbols for 480 kHz. |

Proposal 3

- For additional beam switching time delay d of 120 kHz, support 28 symbols.

  - Note: this is Alt-2 from the RAN1#106 agreement.

| Company | Input |
| --- | --- |
| LG Electronics | We support Proposal 3 and our preference on Alt-2 is added in the above summary. |
| OPPO | Support |
| Ericsson | Our preference is still Alt-1. We don’t see why for 120 kHz that d needs to be larger than 1-slot. Our understanding is that this allows time for PDCCH decoding. On a 120 kHz carrier, is this expected to be longer than 1 slot? Furthermore, the We note that in FR1, for PDCCH on either a 15 or 30 kHz carrier that schedules a carrier with larger SCS, d is roughly ½ slot (8 symbols) in both cases. For FR2, following the same scaling principle, d can be the same value for either a 60 or 120 kHz carrier scheduling a carrier with larger SCS. For the case of 60 kHz, d = 1 slot (14 symbols is specified). Hence, we think for 120 kHz d should also be 14 symbols, i.e., Alt-1. |
| Lenovo, Motorola Mobility | We support proposal 3 |
| vivo | Support proposal 3 |
| Futurewei | Support proposal 3 |
| Nokia/NSB | Do not support. We prefer alt-1. The same view as Ericsson. |
| Qualcomm | Support Proposal 3 |
| Apple | Support proposal 3. |
| Samsung | Support Proposal 3 |
| Huawei, HiSilicon | We Support Proposal 3. We prefer to be consistent and support the absolute time domain values with respect to the values in FR2-1. Therefore, we support 28 symbols for 120 kHz and 112 symbols for 480 kHz. It is noteworthy that d∙2μCSIRS2μPDCCHis added to timeDurationForQCL and beamSwitchTiming which are obtained for 480 and 960 kHz following the same principle (using the same absolute time values for 120 kHz in FR2-1). |

1st round discussion summary

TBU

Beam switching gap and scheduling restrictions for higher SCSs

Observations and Proposals from Contributions

| - Company | - Observations and Proposals from Contributions |
| --- | --- |
| - [Huawei/HiSi, 1] | Observation 1: For 960 kHz and 480 kHz SCS, current scheduling restrictions cannot protect the reception or transmission of a signal with a higher priority when an adjacent symbol carries a signal with a lower priority and using a different beam. Proposal 5: For 960 kHz and 480 kHz SCS and within one slot, UE is not expected to be scheduled/configured with a signal/channel with a lower priority on one symbol before to and one symbol after of a signal/channel with a higher priority and a different QCL-D assumption. - Use the following table to prioritize DL signal/channels where a smaller priority index indicates a higher priority Priority index; Signal(s). 0; SSB, RLM-RS. 1; CORESET 0. 2; CORESET N, N=1, 2, …. 3; CSI-RS for tracking/CQI. 4; PDSCH DMRS. 5; PDSCH data symbol. Proposal 6: A UE can drop the low priority signal(s)/channel(s) in a slot when additional beam switch is required and the number of beam switches has already reached maxNumberRxTxBeamSwitchDL. |
| - [FUTUREWEI, 2] | Proposal 1: For both 480 kHz and 960 kHz SCS, UE is not expected to be able receive downlink data or control channel or reference signals with different QCL-TypeD properties on adjacent symbols within a slot if that exceeds its signaled beam switch capability. Proposal 2: Analogous to the overlapping PDCCH/PDSCH and PDCCH/PDCCH with different QCL-TypeD situation in FR2-1, in FR2-2 a precedence relation is necessary for UEs incapable of adjacent symbol reception with beam switching. Proposal 4: UE is expected to prioritize reception based on a priority ranking and is expected to receive at-least those symbols in a slot (using their corresponding beams) that are associated with signals (or channels) whose priorities are in the top maxNumberRxTxBeamSwitchDL highest priorities among signals of all symbols in that slot. FFS: Details of priority ranking |
| - [ZTE/Sanechips, 4] | Observation 1: Rel-15/16 NR specifications have enough flexibility to support beam switching for non-SSB channels/signals with new SCSs 480 kHz and 960 kHz, even if the lengths of CP are not enough for beam switching. |
| - [NEC, 7] | Proposal 2: A gap for beam switching or directional LBT should be introduced for multiple QCL assumption in multiple-PDSCH scheduling. |
| - [CATT, 8] | Observation 4: In order to guarantee the reception performance of PDSCH, the additional beam switching gap need to be reserved before the PDSCH. Proposal 7: When the additional beam switching gap is introduced, QCL assumption needs to be investigated. |
| - [Ericsson, 10] | Proposal 8 To allow efficient configuration of reference signal resource sets for beam management for 480/960 kHz SCS, RAN1 should further discuss the introduction of some form of UE capability signalling that can provide the network with knowledge related to the UE beam switch time (on the order of 10s of ns, rather than 10s or 100s of OFDM symbols). |
| - [Samsung, 12] | Proposal 4: Reserve one symbol for beam switching gap when using 480 kHz and 960 kHz SCSs. Proposal 6: If beam switching gap is required, use indicated QCL assumption when an enough gap for beam switching is provided, otherwise keep default QCL assumption. |
| - [NTT DOCOMO, 15] | Proposal 1: For timing parameters associated with beam based operation, - For beam switching between SRS/PUCCH/PUSCH, whether/how to define the beam switching gap depends on RAN4 conclusion on “transient period” and “UE beam switching time (beam direction switch only)”. - Value(s) for the SRS antenna switching gap should be defined for 480 and 960kHz SCS. |
| - [Lenovo/MotM, 17] | Observation 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, with higher subcarrier spacings (numerologies) such as 960kHz for SSB, beam switching issue would appear between the contiguous SSB beams since the CP length would not be enough for beam switching, and an extra gap might be needed to prevent performance degradation Observation 2: For NR operation between 52.6 GHz and 71 GHz, the new indicated common TCI state may not be applicable for the scheduled PDSCHs even the PDSCHs are received after the application time when the UE cannot switch it RX beams to the new indicated common TCI state between two continuous PDSCH transmissions Proposal 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, for higher subcarrier spacings (numerologies) such as 960kHz for SSB, to allow the beam switching between contiguous SSBs and between SSB and CORESET#0, a gap (for example a symbol gap or post-fix) should be supported for beam switching at least for 960kHz |
| - [InterDigital, 18] | Observation 6: When multiple PDSCHs are scheduled by a single DCI, suitable slot level or symbol level gaps between the scheduled PDSCHs could be used to switch the QCL assumption in between reception of the first and the last scheduled PDSCHs. |
| - [Qualcomm, 22] | Proposal 1: Introduce a minimum interval between start of two consecutive beam switches. - The value can be X symbols per SCS and can be UE capability. Proposal 2: Introduce explicit beam switch gaps at least in the following scenarios for 480 and 960 KHz SCSs. - Between different SSBs. - Between CSI-RS resources in a resource set with higher layer parameter Repetition configured as ON. |

Summary of views

| # | Issue | Companies’ views |
| --- | --- | --- |
| 4.1 | Introduction of beam switching gap or scheduling restriction | Beam switching gap: NEC, CATT, Samsung, IDCC, Qualcomm - [NEC]: A gap for beam switching or directional LBT should be introduced for multiple QCL assumption in multiple-PDSCH scheduling. - [Lenovo/MotM]: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, for higher subcarrier spacings (numerologies) such as 960kHz for SSB, to allow the beam switching between contiguous SSBs and between SSB and CORESET#0, a gap (for example a symbol gap or post-fix) should be supported for beam switching at least for 960kHz. Scheduling restriction: Huawei/HiSi, FUTUREWEI - [Huawei/HiSi]: For 960 kHz and 480 kHz SCS, current scheduling restrictions cannot protect the reception or transmission of a signal with a higher priority when an adjacent symbol carries a signal with a lower priority and using a different beam. - [FUTUREWEI]: For both 480 kHz and 960 kHz SCS, UE is not expected to be able receive downlink data or control channel or reference signals with different QCL-TypeD properties on adjacent symbols within a slot if that exceeds its signaled beam switch capability. UE capability: Ericsson - [Ericsson]: To allow efficient configuration of reference signal resource sets for beam management for 480/960 kHz SCS, RAN1 should further discuss the introduction of some form of UE capability signalling that can provide the network with knowledge related to the UE beam switch time (on the order of 10s of ns, rather than 10s or 100s of OFDM symbols). Hold the discussion: Docomo - [Docomo]: For timing parameters associated with beam based operation, - For beam switching between SRS/PUCCH/PUSCH, whether/how to define the beam switching gap depends on RAN4 conclusion on “transient period” and “UE beam switching time (beam direction switch only)”. - Value(s) for the SRS antenna switching gap should be defined for 480 and 960kHz SCS. No: ZTE/Sanechips - [ZTE/Sanechips]: Rel-15/16 NR specifications have enough flexibility to support beam switching for non-SSB channels/signals even if the lengths of CP are not enough for beam switching |
| 4.2 | Introduction of SRS antenna switching gap | Value(s) for the SRS antenna switching gap should be defined for 480 and 960 kHz SCS: Docomo Table: The minimum guard period between two SRS resources of an SRS resource set for antenna switching Y [symbol]. 0; 15; 1. 1; 30; 1. 2; 60; 1. 3; 120; 2. |

1st round discussion

Observation 4

No proposal supported by majority companies is observed by the moderator. Please answer following questions.

Q1. Do you think RAN1 needs to hold the discussion for beam switching gap/scheduling restrictions until RAN4 conclusion on transient period?

Q2. If not, please share your views on whether/how to support beam switching gaps/scheduling restrictions.

Q3. Please share your views on whether to support SRS antenna switching gap for new SCSs.

| Company | Input |
| --- | --- |
| Ericsson | Q1: It is not clear to us what is the intended scope of the discussion on beam switching gap/scheduling restrictions. It seems that different companies have different ideas in mind. Could the moderator try to focus the discussion to a particular case or set of cases? Until there is an issue that has been clearly identified, then we suggest to hold discussion. Q3: Regarding the minimum guard time between SRS resources for antenna switching, the values of Y are based on RAN4 requirements. Hence, RAN1 should wait until RAN4 discusses this issue. |
| Lenovo, Motorola Mobility | Q1: We agree to postpone the discussion until a feedback from RAN4 is received |
| Nokia/NSB | Q1: similar view as Ericsson Q3: this should be based on RAN4 decision. |
| Qualcomm | Q1: Prefer not to wait given the limited remaining time. Q2: Below are our preference, which can be UE capability - 1 symbol gap for both 480 and 960 kHz at least - Between adjacent SSBs - Between adjacent CSI-RS resources in a resource set with higher layer parameter Repetition as ON (for Rx beam sweep) - Between adjacent DL symbols with different QCL-TypeD sources - Between adjacent UL symbols with different spatial relation RSs - 2, 4, 8 symbol gap for 240, 480, and 960 kHz between DL and UL switching Q3: Support, the gap should also be scaled based on SCS |
| Samsung | Q1: We prefer to wait until a feedback from Ran4 is received. |
| Huawei, HiSilicon | Q1: We don’t think RAN1 should hold the discussion for beam switching gap/scheduling restrictions until RAN4 reach a conclusion on transient period. Up to 200ns beam switch time at the UE side is suggested in ongoing discussions in RAN4. Even if the 100ns beam switching gap is agreed, this cannot be absorbed the CP length of 73ns in 960kHz. Also, considering other impairments such as effects of channel dispersion, synchronization errors, and gNB MIMO TAE, 146ns CP in 480 kHz does not seem to be enough. We think that it is quite clear already that some scheduling restrictions are required at least for 960 kHz. If we wait for RAN4, we essentially have to take on this important issue entirely in the maintenance phase. We don’t think this would be such a good idea. Q2: Scheduling restrictions should be supported for 960 kHz and 480 kHz. In our view, for 960 kHz and 480 kHz, UE should not be scheduled/configured with a signal/channel with a lower priority on one symbol before to and one symbol after of a signal/channel with a higher priority and a different QCL-D assumption. Q3: This can be discussed as a part of Q2. |
| LG Electronics | Q1: As we introduced beam switching gap (at least 1 symbol) for adjacent SSBs in the last meeting, we can consider to support symbol-level beam switching gap between DL signals/channel or between UL signals/channels, before RAN4 reply to RAN1’s LS. Q3: As Ericsson pointed out, we also think gap between SRSs need to be investigated in RAN4. On the other hand, we can consider at least 1 symbol gap for 960 kHz for the examples from Qualcomm. |

Proposal 4

TBU

1st round discussion summary

TBU

Multiple QCL Assumptions for Multiple PDSCHs/PUSCHs

Multiple QCL assumptions based on timeDurationForQCL for single TRP and multi-TRP

Observations and Proposals from Contributions

| - Company | - Observations and Proposals from Contributions |
| --- | --- |
| - [Huawei/HiSi, 1] | Observation 2: In the slots with offset smaller than timeDurationForQCL, UE may receive and buffer signals in each slot using a different beam associated with the lowest CORESET ID of the latest monitored slot. Proposal 7: For multi-PDSCHs scheduled by a single DCI, when a PDSCH scheduling offset is less than timeDurationForQCL, UE receives the PDSCH with the same QCL assumption used to monitor the CORESET with the lowest controlResourceSetId in the latest slot that precedes the PDSCH and in which one or more CORESETs are monitored by the UE (Case 2 Alt 2 in RAN1 106-e Agreement). Proposal 8: For multi-PDSCHs scheduled by a single DCI, when some of PDSCHs collide with semi-static UL symbols and skipped, no additional QCL assumption specification is required for the remaining non-colliding PDSCHs. |
| - [FUTUREWEI, 2] | Proposal 6: For Case-2, default QCL assumption is derived using a configured value of the first scheduled PDSCH slot offset, i.e., configured choice of the slot containing PDSCH with the smallest scheduling offset. The default QCL assumption derived for this PDSCH slot, is the same as that specified in Rel. 16 for single-PDSCH scheduling when the scheduling offset is less than timeDurationForQCL. FFS: Details of configured value. Proposal 7: Any PDSCH whose one or more symbols collide with semi-statically configured UL symbols is ignored for the purpose of QCL determination. The remaining symbols of that PDSCH can be regarded as gap symbols. |
| - [Spreadtrum, 3] | Proposal 4: For PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL, support Alt 1 where Single QCL assumption is applied for all scheduled PDSCHs. |
| - [ZTE/Sanechips, 4] | Proposal 4: If all PDSCHs scheduled by a single DCI with scheduling offsets less than the threshold timeDurationForQCL, same default QCL assumption(s) can be adopted. - All scheduled PDSCHs follows a same default QCL assumption as the first PDSCH Proposal 5: If some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL - All PDSCH(s) that has scheduling offset less than timeDurationForQCL follows a same default QCL assumption, as given in Proposal 4 - All PDSCH(s) that has scheduling offset equal to or greater than timeDurationForQCL follows a same QCL assumption Proposal 6: When some of PDSCHs are collided with semi-static UL symbols, the following can be considered: - These PDSCHs can be skipped when determining the scheduling offset from the scheduled PDSCH to the scheduling PDCCH The first PDSCH for deriving default QCL assumption should be a first non-collided PDSCH Proposal 7: For multi-PDSCH scheduling with M-DCI based mechanism, each TRP in multi-TRP can adopt the same method of QCL acquisition as for single-TRP. Proposal 8: For multi-PDSCH scheduling with S-DCI based mechanism, the same method of QCL acquisition as for single-TRP can be applied, including: - All PDSCH(s) that has scheduling offset less than timeDurationForQCL follows a same default QCL assumption - All PDSCH(s) that has scheduling offset equal to or greater than timeDurationForQCL follows a same QCL assumption |
| - [vivo, 5] | Proposal 3: Support Alt1 (single QCL assumption is applied for all scheduled PDSCHs) for default beam determination of Multi-PDSCHs when PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL. Proposal 4: PDSCH skipping does not affect the original default beam rule. Proposal 5: Legacy default beam behavior in Rel16 can be reused for Multi-PDSCHs scheduling in the scenario of M-TRP. |
| - [OPPO, 6] | Proposal 2: For multiple PDSCHs/PUSCHs scheduled by a single DCI for single TRP, if the offset between the reception of the DL DCI and any scheduled PDSCH is less than the threshold timeDurationForQCL, the UE shall follow the QCL assumption of the CORESET on the latest slot to determine the QCL assumption of each PDSCH. |
| - [NEC, 7] | Proposal 1: For case 2 PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL, the current Rel-16 behavior can be directly extended to multiple-PDSCH scheduling, and Alt 2 multiple QCL assumption can be supported for multiple-PDSCH transmission. |
| - [CATT, 8] | Observation 3: For the case 2, two options could be selected for multi-PDSCH scheduled by single DCI, and they could achieve reception gain in different scenarios. The scenarios benefit to Alt1 could be easily satisfied by gNB scheduling. And considering UE complexity, Alt1 is preferred than Alt2. Proposal 6: When some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL and some have scheduling offset equal to or greater than timeDurationForQCL, Alt-1 below should be supported. - Alt 1: Single QCL assumption is applied for all scheduled PDSCHs. The scheduled PDSCHs quasi co-located with the RS(s) based on the activated TCI states in the first slot with the scheduled PDSCH. - Alt 2: Multiple QCL assumptions are applied. The scheduled PDSCHs quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state in DCI. |
| - [Ericsson, 10] | Proposal 1 QCL Indication for single-TRP operation - In the RAN1#106-e agreement for single-TRP operation, Alt-1 is supported for Case 2, i.e., single QCL assumption is applied for all scheduled PDSCHs - The single QCL assumption is given by the existing Rel-16 QCL assumption for single-PDSCH scheduling - Note: The existing Rel-16 QCL assumption corresponds to the CORESET with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE - For multi-PDSCH scheduling, the “latest slot” monitored by the UE for PDCCH is determined with respect to the earliest scheduled PDSCH that does not overlap any OFDM symbol classified as ‘U’ in the semi-static TDD DL/UL pattern |
| - [Nokia/NSB, 11] | Proposal 4: Support single QCL assumption for the multi-PDSCH transmission in case of some of the PDSCHs are having lower scheduling offset than timeDurationForQCL. Proposal 5: QCL assumption for the all the PDSCHs is the same as the one used for the first PDSCH in case of some of the PDSCHs are having lower scheduling offset than timeDurationForQCL. I.e. the same default QCL assumption for all scheduled PDSCHs as for the for the first PDSCH. Proposal 6: In multi-TRP case, single QCL assumption is applied for the multiple scheduled PDSCHs per TRP. |
| - [Samsung, 12] | Proposal 5: Support Alt-2 of Case 2 (multi QCL assumptions). Use indicated QCL assumption if scheduling offset is equal or greater than timeDurationForQCL. |
| - [MediaTek, 13] | Proposal 4: For the reception of multi-PDSCHs scheduled by a single DCI within the duration specified by timeDurationForQCL, current Rel-15/16 default beam assumption should be applied. |
| - [Intel, 14] | Proposal 5: For Case 2 when PDSCH scheduling offset is less than timeDuraionForQCL, the UE should apply the default QCL assumption(s) which corresponds to one of the semi-statically configured PDSCH TCI states for the UE. - FFS: Which TCI state from the dedicated UE configuration is the default. |
| - [NTT DOCOMO, 15] | Proposal 2: For multi-PDSCH/PUSCH scheduled by single DCI for single TRP case, - Regardless of the offset between scheduling DCI and PDSCHs relative to timeDurationForQCL, TCI states for PDSCHs scheduled by a single DCI follows the TCI state applied for the first PDSCH. - If the offset between the first PDSCH and PDCCH is less than timeDurationForQCL, default QCL assumption of the first PDSCH is same as CORESET with the lowest CORESETID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. |
| - [Sony, 16] | Proposal 3 : For the case of single DCI scheduled multiple PDSCH of single-TRP, when any of the scheduled PDSCHs < timeDurationForQCL, UE applies the same default Rx beam from the 1st PDSCH to the last PDSCH. |
| - [Lenovo/MotM, 17] | Proposal 3: For NR operation between 52.6 GHz and 71 GHz when multiple PDSCHs are scheduled via single DCI to be received from two TRPs (not all PDSCHs received from one TRP), then the duration for which each TCI state is valid should also be indicated Proposal 4: For NR operation between 52.6 GHz and 71 GHz with high subcarrier spacing values such as 480kHz and 960kHz, specify enhancements to support multiple default beams association for multiple PDSCHs scheduled by single DCI: - PDCCH CORESET can be associated with multiple QCL assumptions (beams) that can be used to determine multiple default beams based on lowest CORESET ID Duration/applicability for each of the default beams can also be associated to allow UE to determine when to switch from one default beam to another during the duration of multiple PDSCH transmission Proposal 5: For NR operation between 52.6 GHz and 71 GHz with high subcarrier spacing values such as 480kHz and 960kHz, if a UE is going to transmit a set of consecutive PUSCH transmissions including both dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions, the UE can select the latest indicated UL Tx beam to transmit the consecutive UL CG and DG transmissions |
| - [InterDigital, 18] | Observation 5: More performance gain can be achieved by using an optimized beam indicated by a TCI state field in DCI rather than using a default beam. Proposal 4: Support multiple QCL assumptions (Alt 2) when PDSCH scheduling offset for any PDSCH < timeDurationForQCL (Case 2). Observation 8: While multi-TRP scenarios are one of the important deployment scenarios, given the remaining time for Rel-17, it is preferred to minimize the required specification efforts as much as possible. Proposal 6: To minimize the specification efforts, it is preferred to apply the identical TCI state determination mechanism as single TRP. - When PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL (Case 1), support single QCL assumption (i.e., one TCI state for all PDSCHs from 1st TRP and another one TCI state for all PDSCHs from 2nd TRP). When PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL (Case 2), RAN1 jointly decides whether to apply single QCL assumption (Alt 1) or multiple QCL assumption (Alt 2) for both single TRP and multi-TRP scenarios. |
| - [LGE, 19] | Proposal #4: If PDSCH scheduling offset for any of PDSCHs scheduled by a single DCI is less than timeDurationForQCL (i.e., Case 2), the single QCL assumption is applied for all scheduled PDSCHs and is determined based on the lowest index CORESET in the latest slot from the first valid PDSCH (which is not collided with semi-static UL symbols). Proposal #5: In order to determine Case 1 (i.e., PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL) or Case 2 (PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL), only valid PDSCHs are taken into account and PDSCHs skipped due to collision with semi-static UL symbols are excluded. |
| - [Apple, 20] | Proposal 2: Support a mechanism to allow a single QCL assumption at least for multi-PDSCH scheduled by a single DCI that have scheduling offset less than timeDurationForQCL. |
| - [Convida, 21] | Proposal 2: Alt-2 (i.e. multiple QCL assumptions are applied) is preferred when the scheduling offset shorter than timeDurationForQCL for single DCI scheduling multi-PDSCH with single TRP. |
| - [Qualcomm, 22] | Proposal 8: In case of PDSCH scheduling offset for any scheduled PDSCH with offset < timeDurationForQCL, UE applies the single default PDSCH beam to remaining scheduled PDSCH with offset > timeDurationForQCL. - This is regardless of whether the indicated TCI state is same as the single default PDSCH beam or not. Proposal 9: In case of mDCI mTRP for any scheduled PDSCH with offset < timeDurationForQCL, support a single default PDSCH beam applied to all PDSCHs scheduled by a DCI associated with a CORESET for a given CORESETPoolIndex. - gNB can dynamically update the single default PDSCH beam per CORESETPoolIndex via MAC-CE. Proposal 10: In case of sDCI mTRP for any scheduled PDSCH with offset < timeDurationForQCL, support the two default PDSCH beams defined in R16 to be applied to all PDSCHs scheduled by a DCI. Proposal 11: Support new RRC parameter to indicate UE to follow either the R17 enhanced PDSCH default beam & QCL rule for multi-PDSCH scheduling or the corresponding R15/16 rule. - The RRC parameter can be introduced separately for sTRP, sDCI mTRP, and mDCI mTRP. |

Summary of views

In RAN1#106-e, the following agreements on QCL assumptions of PDSCH for single TRP are agreed.

Agreement: For the single TRP case, for multi-PDSCHs scheduled by a single DCI with a single DCI field ‘Transmission Configuration Indication’ that indicates a single TCI state (if the DCI field is present), - Case 1: PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL - Case 1-1: tci-PresentInDCI enabled - Single QCL assumption based on the indicated codepoint of the single DCI field ‘Transmission Configuration Indication’ is applied for all scheduled PDSCHs - Case 1-2: tci-PresentInDCI not present - Single QCL assumption of the single scheduling DCI scheduled multi-PDSCHs is applied for all scheduled PDSCHs - Case 2: PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL - Down select one of the following alternatives - Alt 1: Single QCL assumption is applied for all scheduled PDSCHs - FFS: Details of single QCL assumption - Alt 2: multiple QCL assumptions are applied - FFS: Details of multiple QCL assumptions - FFS: When some of PDSCHs are collided with semi-static UL symbols and then skipped - FFS: The multi-TRP case

Based on the above agreement, the following companies’ views are observed.

| # | Issue | Companies’ views |
| --- | --- | --- |
| 5.1 | Whether to support single QCL assumption or multiple QCL assumptions for Case 2 | Single QCL assumption (Alt 1): FUTUREWEI, Spreadtrum, ZTE/Sanechips (including multi-TRP), vivo, CATT, Ericsson, Nokia/NSB, Intel, NTT Docomo, Sony, LGE, Apple, Qualcomm - [Spreadtrum]: For Case 2, we don’t see any need to introduce multiple QCL assumptions. Single QCL assumption is enough and is the simplest solution. Specifically, the QCL corresponding lowest CORESET-id can be chosen as the default QCL assumption. - [ZTE/Sanechips]: If some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL - All PDSCH(s) that has scheduling offset less than timeDurationForQCL follows a same default QCL assumption, as given in Proposal 4 - All PDSCH(s) that has scheduling offset equal to or greater than timeDurationForQCL follows a same QCL assumption - [Sony]: For the case of single DCI scheduled multiple PDSCH of single-TRP, when any of the scheduled PDSCHs < timeDurationForQCL, UE applies the same default Rx beam from the 1st PDSCH to the last PDSCH. Multiple QCL assumption (Alt 2): Huawei/HiSi, OPPO, NEC, Samsung, MediaTek, Lenovo/MotM, IDCC, Convida - [Oppo]: For multiple PDSCHs/PUSCHs scheduled by a single DCI for single TRP, if the offset between the reception of the DL DCI and any scheduled PDSCH is less than the threshold timeDurationForQCL, the UE shall follow the QCL assumption of the CORESET on the latest slot to determine the QCL assumption of each PDSCH. - [NEC]: For case 2 PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL, the current Rel-16 behavior can be directly extended to multiple-PDSCH scheduling, and Alt 2 multiple QCL assumption can be supported for multiple-PDSCH transmission. - [IDCC]: More performance gain can be achieved by using an optimized beam indicated by a TCI state field in DCI rather than using a default beam. |
| 5.2 | Handling of skipped PDSCHs due to collision with semi-static UL symbols | No additional specification support: Huawei/HiSi, vivo, NTT Docomo - [vivo]: PDSCH skipping does not affect the original default beam rule. Ignore skipped PDSCH symbols: FUTUREWEI, ZTE/Sanechips, - [Futurewei]: Any PDSCH whose one or more symbols collide with semi-statically configured UL symbols is ignored for the purpose of QCL determination. The remaining symbols of that PDSCH can be regarded as gap symbols. |

1st round discussion

Observation 5

No proposal supported by majority companies is observed for QCL assumptions based on timeDurationForQCL and handling of skipped PDSCHs due to collision with semi-static UL symbols. Please share your views on the following questions.

Q1. Whether to support single QCL assumption or multiple QCL assumptions for case 2 for single TRP?

Q2. Whether to introduce new specification impacts to handle skipped PDSCHs due to collision with semi-static UL symbols. If needed, what would be the new specification impacts?

| Company | Input |
| --- | --- |
| OPPO | We are fine to support single QCL assumption for case 2 for single TRP if majority wants to go this direction. Our original concern is that if there is a CORESET/SS set configured within the set of multi-PDSCH scheduled, single QCL assumption may result in a PDSCH reception cancellation if the CORESET TCI state is different from the single QCL assumption. Thus, enhancement could be done to improve this aspect. But single QCL assumption is also acceptable to us. |
| Ericsson | Q1: We support single QCL assumption for simplicity. Furthermore, the QCL assumption can be based on a straightforward extension of the Rel-16 as follows: - The single QCL assumption is given by the existing Rel-16 QCL assumption for single-PDSCH scheduling - Note: The existing Rel-16 QCL assumption corresponds to the CORESET with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE Regarding how to interpret the “latest slot” in the case of multi-PDSCH scheduling, a straightforward approach is as follows (let’s call this Alt-1): - Alt-1: For multi-PDSCH scheduling, the “latest slot” monitored by the UE for PDCCH is determined with respect to the earliest scheduled PDSCH that does not overlap any OFDM symbol classified as ‘U’ in the semi-static TDD DL/UL pattern The following could work too: - Alt-2: For multi-PDSCH scheduling, the “latest slot” monitored by the UE for PDCCH is determined with respect to the earliest scheduled PDSCH regardless of potential overlap with any OFDM symbol classified as ‘U’ in the semi-static TDD DL/UL pattern Q2: Please see the above two alternatives (Alt-1 and Alt-2) for how to handle this. |
| Lenovo, Motorola Mobility | Considering the limited remaining time, we are OK to consider a single QCL assumption. |
| Vivo | Q1: we support Single QCL assumption (Alt 1). Q2: Two cases are defined in previous agreement. Case 1: PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL and Case 2: PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL. For a single DCI scheduled multi-PDSCH, it’s either case 1 or case 2. However, we don’t see any specification impact needed to handle “when some of PDSCHs are collided with semi-static UL symbols and then skipped”. |
| Futurewei | Q1: We appreciate the simplicity of single QCL operation and prefer Alt-1 using an approach that determines QCL-D based on first scheduled PDSCH. Our concern is on UE determination of its first scheduled PDSCH in terms of how soon can it decode its DCI and retrieve the indicated row of the TDRA table. In case this operation itself imposes a delay of several slots, some simple specification in terms of UE assumption of its first scheduled PDSCH slot is also needed. |
| Nokia/NSB | Q1: support single QCL assumption. We don’t see any usecase having multiple QCL assumption other than repetition scheme. Q3: we can consider possible options similar as Ericsson proposals later. |
| MediaTek | Q1: We prefer to adopt the same default beam assumption used in Rel-15/16. Otherwise, if a new behavior is adopted, how can UE know which default beam assumption need to apply before DCI decoding? Q2: if Rel-15/16 default beam assumption can be adopted, we don’t see the need for further spec update. |
| Qualcomm | Q1: Support single QCL assumption for selecting best beam to maximize SNR. We are also fine to have multiple QCL assumption, e.g. R15 default PDSCH beam rule, as baseline, i.e. gNB can configure UE which mode to use Q2: We don’t see the need for new spec change |
| Apple | Q1: Support single QCL due to quite short slot duration with larger SCS. Q2: We do not see the additional specification impact for ‘PDSCH skipping’ due to collision with UL symbol indicated by semi-static as the following rule defined in TS 38.213 seems sufficient to handle this case: For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL- ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot.. |
| Convida Wireless | Rel-15/16 default beam can be considered for the baseline which the UE can determine the latest monitored slot and use the QCL assumption associated with the CORESET with lowest ID in that slot for the PDSCH reception QCL assumption. |
| Samsung | Q1: Support multi QCL assumptions for Case2 for Single TRP to achieve better SNR. Q2: We don’t see the need for new spec change |
| Huawei, HiSilicon | Q1: We support Alt 2 of Case 2 (multiple QCL assumptions are applied in the case of PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL) due to the following reasons: - In the case that some PDSCHs are before timeDurationForQCL and some after, we think using a single QCL assumption for all PDSCHs is quite sub-optimal. UE has to use default QCL assumptions(s) for the PDSCHs before timeDurationForQCL which are not necessarily associated with optimal beams for the corresponding PDSCHs. However, we don’t think it is well-justified to deprive UE from using the optimal QCL assumption (indicated by the TCI field in the scheduling DCI to receive PDSCHs after timeDurationForQCL) just because UE has used other default beam(s) for the PDSCHs before timeDurationForQCL. - We don’t think using a single default QCL assumption reduces UE processing complexity for the case that some scheduled PDSCH are before timeDurationForQCL. Let’s assume that DCI schedules 8 PDSCHs in 8 slots (which may even be non-consecutive slots and, hence, span a much longer set of slots) were one, some, or all of the scheduled PDSCHs are before timeDurationForQCL. It is quite likely that UE has to monitor CSS in some of the scheduled PDSCHs slots (or some slots within the span of the scheduled PDSCH slots) for, eg, CORESET#0. If we are tempted to use only a single default QCL assumption for all the scheduled slots under the pretext of simplifying UE operation, then we may need to also note that UE anyways has to change its receive beam to monitor CORESET#0 unless it always use the QCL assumption used for monitoring CORESET#0 as the default QCL assumption for all PDSCHs. Such an assumption would certainly be suboptimal. - The definition of “timeDurationForQCL” is brought below: timeDurationForQCL Defines minimum number of OFDM symbols required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [12] clause 5.1.5. UE shall indicate one value of the minimum number of OFDM symbols per each subcarrier spacing of 60kHz and 120kHz.. Based on our understanding of the above definition, for up to timeDurationForQCL symbols after the last symbol of DCI arrives at UE buffer, it is undetermined whether or not UE has completed PDCCH reception which requires channel estimation for PDCCH, demodulation, decoding, and DCI content parse. In other words, for up to timeDurationForQCL symbols after the last symbol of DCI arrives at UE buffer, it is undetermined whether or not UE knows a DCI whose CRC scrambled by its C-RNTI has already arrived in its buffer. Of course, one may argue that it takes less than timeDurationForQCL symbols (let’s say X symbols) to decode the DCI as timeDurationForQCL symbols also include the time for DCI content parsing and applying spatial QCL information received in DCI for PDSCH processing. However, to our understanding, the value of X symbols is unspecified. The only thing that is guaranteed about its value is that X<= timeDurationForQCL. Therefore, any reasonable design should assume that X= timeDurationForQCL. Given above, note that timeDurationForQCL for 480 and 960 kHz can be quite large in terms of number of slots. We already agreed to 28 symbols * 4 = 8 slots for 480 kHz and 28 symbols * 8 = 16 slots for 960 kHz. Now, let’s for the sake of argument, assume that UE’s timeDurationForQCL is equal to 16 slots in 960 kHz. Let’s say a DCI whose CRC scrambled by UE’s RNTI arrives to UE’s buffer in some slot. This DCI schedules 8 PDSCHs in the 3rd slot, 4th slot, 5th slot,…, 10th slot after. However, considering that UE timeDurationForQCL is 16 slots, UE may know that that there is a DCI for it only at 16th slot after the DCI arrives at its buffer! So, for such a scenario, how is it possible to mandate the UE to use a “single QCL assumption” to receive PDSCHs that arrive in its buffer 3rd slot, 4th slot, 5th slot,…, 10th slot after the DCI arrived in its buffer when UE would know that there is a DCI in its buffer only after 16 slots? By the time that UE realizes that there is a DCI for it, all PDSCHs have already been buffered! For such scenario, during the time interval that UE is still unaware of the presence of a DCI in its buffer, the only choice for UE is to keep buffering the received signals using its default behavior: Each slot, including PDSCH slots, is buffered based on the QCL assumption of the lowest CORESET ID in the latest monitored slot. Q2: In our view, no additional QCL assumption specification is required for the remaining non-colliding PDSCHs. Among all PDSCHs that have not collided with semi-static UL symbols, QCL assumption for the PDSCHs whose offsets are larger than timeDurationForQCL is derived from Case 1 in the Agreement in RAN1 106-e and the QCL assumption for PDSCHs whose offsets are smaller than timeDurationForQCL is the same as the QCL assumption of the lowest CORESET ID in the latest monitored slot. |
| LG Electronics | Q1: Support single QCL assumption to have a common behavior with Case 1. Q2: The specification impact to handle invalid PDSCH may not exist as Apple pointed out. However, the main point of the previous FFS point regarding invalid PDSCH is, how to determine Case 1 or 2 if some PDSCHs are skipped, e.g., due to collision with semi-static UL symbols. Let’s take an example, as shown above. 4 PDSCHs are scheduled by a multi-PDSCH scheduling DCI but the first two PDSCHs are cancelled owing to the conflict with semi-static UL symbols. Then, if the scheduling offset from the DCI to the first two (but cancelled) PDSCHs are less than timeDurationForQCL but the scheduling offset from the DCI to the last two PDSCHs are larger than timeDurationForQCL, which between Case 1 and Case 2 will be applied? Our preference for that example is that UE applies Case 1. |

Proposal 5

TBU

1st round discussion summary

TBU

Multiple TCI states/SRIs for multiple PDSCHs/PUSCHs with multi-TRPs

Observations and Proposals from Contributions

| - Company | - Observations and Proposals from Contributions |
| --- | --- |
| - [Huawei/HiSi, 1] | Proposal 9: The design of multi-TRP multi-PDSCH scheduling should be considered as a lower priority discussion in this WI. |
| - [FUTUREWEI, 2] | Proposal 8: Multi-TRP scenario should be considered once all remaining details in the single-TRP case have been addressed. |
| - [Spreadtrum, 3] | Proposal 2: Confirm the following working assumption: Working assumption: For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs - The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism - The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism - Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states FFS: Details of multiple TCI state association with multiple PDSCHs Proposal 3: Cyclic mapping/Sequential mapping/half-half mapping can be considered for association among multiple TCI state and multiple PDSCHs. |
| - [vivo, 5] | Proposal 6: Confirm the work assumption without FFS. |
| - [OPPO, 6] | Proposal 3: Confirm the working assumption for multi-PDSCH scheduling for the multi-TRP case. |
| - [CATT, 8] | Proposal 8: The multi-PUSCH/PDSCH scheduling by the same DCI should be grouped when the number of scheduling PUSCH/PDSCH is more than two. |
| - [xiaomi, 9] | Proposal 2: In case of multi-DCI based multi-TRP, there is no need to discuss the beam indication for the multiple PDSCHs/PUSCHs, because each TRP has its own DCI which is similar with single TRP case. Proposal 3: In the case of single DCI based mTRP, there are two alternatives for the association between the multiple PDSCHs/PUSCHs and the indicated TCI state(s): - Alt.1: Some of these PDSCHs/PUSCHs are from TRP 1 and others are from TRP2 - Alt.2: There is a so-called “multiple PDSCHs/PUSCHs” for each TRP |
| - [Ericsson, 10] | Proposal 3 Confirm the working assumption on multi-PDSCH for multi-TRP from RAN1#106-e Proposal 4 QCL Indication for multi-TRP operation - For multi-PDSCH scheduling for the multi-TRP case, support the following for Case 1-1, Case 1-2, and Case 2 where the definition of the 3 cases is the same as in the RAN1#106-e agreement for single-TRP: - For single-DCI based multi-TRP mechanism - The existing Rel-16 QCL assumption per TRP for single-PDSCH scheduling is applied for all scheduled PDSCHs - For multi-DCI based multi-TRP mechanism - The existing Rel-16 QCL assumption for single-PDSCH scheduling is applied to all scheduled PDSCHs per TRP - For multi-PDSCH scheduling, the "latest slot" monitored by the UE for PDCCH in the Rel-16 QCL assumption (if applicable) is interpreted with respect to the earliest scheduled PDSCH that does not overlap any OFDM symbol classified as 'U' in the semi-static TDD DL/UL pattern - Note: The existing Rel-16 QCL assumption for the 3 cases is given as follows - Case 1: PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL - Case 1-1: tci-PresentInDCI enabled - The existing Rel-16 QCL assumption corresponds to the one or two TCI states associated with a codepoint of the detected DCI - Case 1-2: tci-PresentInDCI not present - The existing Rel-16 QCL assumption corresponds to the CORESET used for the PDCCH transmission within the active BWP of the serving cell - Case 2: PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL - If neither enableTwoDefaultTCI-States nor enableDefaultTCI-StatePerCoresetPoolIndex is configured - The existing Rel-16 QCL assumption corresponds to the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE - For single-DCI based multi-TRP if enableTwoDefaultTCI-States is configured - The existing Rel-16 QCL assumption corresponds to the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states - For multi-DCI based multi-TRP mechanism if enableDefaultTCI-StatePerCoresetPoolIndex is configured - The existing Rel-16 QCL assumption corresponds to CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of coresetPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE |
| - [Nokia/NSB, 11] | Proposal 7: Confirm working assumption: Working assumption: For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs - The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism - The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism - Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states - FFS: Details of multiple TCI state association with multiple PDSCHs |
| - [Samsung, 12] | Proposal 7: Reuse Rel-16 TCI indication mechanism of single DCI based multi-TRP configured by repetitonNumber for that of multi-PDSCH scheduled by single DCI for single DCI based multi-TRP Proposal 8: Reuse TCI indication mechanism of multi-PDSCH scheduled by single DCI for single-TRP for that of multi-PDSCH scheduled by single DCI for multi DCI based multi-TRP |
| - [MediaTek, 13] | Proposal 5: Deprioritize the discussion of multi-PDSCH scheduling enhancements for multi-TRP application. |
| - [Intel, 14] | Proposal 4: Confirm the working assumption on multi-PDSCH scheduling for multi-TRP. The indicated TCI states should correspond to all scheduled PDSCHs. |
| - [NTT DOCOMO, 15] | Proposal 3: For multi-PDSCH/PUSCH scheduled by single DCI for multi-TRP case, - Multi-PDSCH scheduling by single DCI is supported only for multi-DCI based multi-TRP case in Rel-17. - Regardless of the offset between scheduling DCI and PDSCHs relative to timeDurationForQCL, TCI states for PDSCHs scheduled by a single DCI follows the TCI state applied for the first PDSCH. If the offset between the first PDSCH and PDCCH is less than timeDurationForQCL, default QCL assumption of the first PDSCH is same as CORESET with lowest CORESETID among the CORESETs with same value of CORESETPoolIndex as the PDCCH scheduling the PDSCH in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE. |
| - [Sony, 16] | Proposal 2 : For single-DCI scheduled multi-PUSCH for multi-TRP, support up to 2 SRIs and each SRI is pointed to each TRP. |
| - [Lenovo/MotM, 17] | Proposal 2: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, confirm the following working assumption: Working assumption: For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs - The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism - The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism - Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states FFS: Details of multiple TCI state association with multiple PDSCHs |
| - [InterDigital, 18] | Observation 7: Multi-TRP deployment is one of the important deployment scenarios for NR in higher frequencies. Proposal 5: Confirm the working assumption on TCI state indication mechanism for multi-TRPs. |
| - [LGE, 19] | Proposal #6: For multi-PDSCH scheduling for multi-TRPs, the followings can be considered for each multiplexing scheme that was introduced for Rel-16 single DCI based multi-TRP operation. - SDM: Similar to Rel-16, when DCI indicates two TCI states and two CDM groups, UE can apply SDM scheme to each of scheduled PDSCHs. - FDM scheme A/B: Similar to Rel-16, when DCI indicates two TCI states and one CDM group, UE can apply FDM scheme A or B to each of scheduled PDSCHs (or only to the first PDSCH) - TDM scheme A: When DCI indicates two TCI states and one CDM group, UE can apply TDM scheme A only to the first PDSCH, or apply only the first TCI state to all of PDSCHs and ignore the second TCI state. - TDM scheme B: TDRA row index can be used to differentiate TDM scheme B (configured with repetitionNumber) with multi-PDSCH scheduling (configured with multiple SLIV values). |
| - [Qualcomm, 22] | Proposal 6: For multi-PDSCH scheduling for multi-TRPs, confirm the WA to support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for both sDCI and mDCI based multi-TRPs. - The indicated one or two TCI states are applied to each scheduled PDSCH. |

Summary of views

In RAN1#106-e, the following assumption on the support of multiple TCI states for multi-TRP are agreed.

Working assumption: For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs - The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism - The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism - Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states - FFS: Details of multiple TCI state association with multiple PDSCHs

Based on the above agreement, the following companies’ views are observed.

| # | Issue | Companies’ views |
| --- | --- | --- |
| 6.1 | Whether to support multi-PDSCH scheduling by a single DCI for single-DCI based multi-TRP transmission | Confirm working assumption: Spreadtrum, vivo (without FFS), OPPO, CATT, xiaomi, Ericsson, Nokia/NSB, Samsung, Intel, Lenovo/MotM, IDCC, LGE, Qualcomm Deprioritize multi-TRP scheduled by a single DCI: Huawei/HiSi (low priority), MediaTek, NTT Docomo - [Huawei/Hisi]: The design of multi-TRP multi-PDSCH scheduling should be considered as a lower priority discussion in this WI. |

1st round discussion

Observation 6

The moderator observed that majority of companies indicated their support on confirmation of the working assumption. Based on the observation, the moderator provides proposal 6.

Proposal 6

The following working assumption is confirmed:

For multi-PDSCH scheduling for multi-TRPs, support a single DCI field ‘Transmission Configuration Indication’ as in Rel-16 TCI state indication mechanism for multi-TRPs

- The single DCI field ‘Transmission Configuration Indication’ indicates one or two TCI states associated with a code point for single DCI based multi-TRP mechanism

- The single DCI field ‘Transmission Configuration Indication’ indicates only one TCI state associated with a code point for multi-DCI based multi-TRP mechanism

- Reuse Rel-16 RRC configuration and MAC CE activation/deactivation methods for the one or two TCI states

FFS: Details of multiple TCI state association with multiple PDSCHs

| Company | Input |
| --- | --- |
| OPPO | Support |
| Ericsson | We support confirming the WA. Question to moderator: In addition to the above, it is also necessary to discuss how to extend the default QCL assumptions for multi-TRP to the case of multi-PDSCH scheduling, i.e., for the case when any of the scheduling offsets < timeDurationFor QCL. We had a long discussion in the last meeting on Case 1-1,1-2, and Case 2 for single TRP. We think the same framework applies to multi-TRP, and that for each case a straightforward extension of the Rel-16 rules will work for each of these cases. However, we still need to have that discussion. Does the moderator plan to discuss it this meeting? Also, please see our comment with respect to Proposal 8 below. There is a need to also discuss the default QCL assumptions for cross-carrier scheduling + multi-PDSCH scheduling depending on the configuration of the RRC parameter enableDefaultBeamForCCS. |
| Lenovo, Motorola Mobility | We support confirming the working assumption. Regarding the FFS, we need to further discuss the details on how the two TCI states are applied to multiple PDSCHs |
| vivo | We’d like to get a clarification on what exactly is the intention of FFS. 1. Does the FFS only apply to the first bullet when “the single DCI field ‘Transmission Configuration Indication’ indicates two TCI states associated with a code point for single DCI based multi-TRP mechanism”? 2. The main sentence of the WA says “as in Rel-16 TCI state indication mechanism for multi-TRPs”. It is our understanding that we will follow that. However, FFS seems can be interpreted differently. |
| Nokia/NSB | Support. |
| Qualcomm | Support |
| Apple | We are fine to confirm the working assumption. It is also unclear for us regarding the FFS. TCI state association for PDSCH repetition has been agreed in MIMO session. The difference between Multi-PDSCH and PDSCH repetition is the TBs transmitted over multiple SLIVs. The TCI association rule defined in MIMO session can be simply reused here for multi-PDSCH scheduling. |
| Convida Wireless | We are ok with the moderator’s proposals. |
| Samsung | Support Proposal 6 |
| Huawei, HiSilicon | We still don’t think WA should be confirmed. We do not see the WA confirmation as a progress or move in a right direction considering the very limited remaining time in Rel-17. We have the following two major reasons: - Extensions of supported Rel-16 m-TRP schemes to Rel-17 m-TRP multi-PDSCH are not clearly specified in Rel-17 yet. Many relevant details for such extensions are not even discussed. We are not sure that is well-justified to make an agreement on the QCL assumption for m-TRP multi-PDSCH when more fundamental aspects of m-TRP multi-PDSCH are not even discussed in Rel-17. For instance, note that in Rel-16 single-DCI m-TRP, the following four schemes are supported (RP-202803): - 'FDMSchemeA': When two TCI states, i.e. two TRPs, are indicated in a DCI and the UE is set to 'FDMSchemeA', the UE shall receive single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation in a manner of comb-like PRGs allocation (or half/half for wideband). - 'FDMSchemeB': When two TCI states, i.e. two TRPs, are indicated in a DCI and the UE is set to 'FDMSchemeB', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to non-overlapping frequency domain resource allocation in a manner of comb-like PRGs allocation (or half/half for wideband). - 'TDMSchemeA' (Intra-slot): When two TCI states are indicated in a DCI and the UE is set to 'TDMSchemeA', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion. Both PDSCH transmission (corresponding to two TRPs respectively) occasions with mapping Type B only shall be received within a given slot with a symbol-level gap configured by StartingSymbolOffsetK. - “repetitionNumber-r16” (Inter-slot): When a UE is configured by the higher layer parameter repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, the UE may expect to be indicated with one or two TCI states in a codepoint of the TCI field. When two TCI states are indicated in a DCI, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with two TCI states associated to repetitionNumber-r16 consecutive slots (up to 16). Each PDSCH transmission occasion is expected to have the same SLIV. The UE may be configured with either cyclicMapping or sequentialMapping for given TCI state mapping pattern so that the first TCI state is applied to the first PDSCH transmission occasion and the second TCI state is applied to the second PDSCH transmission occasion.. In our view, before discussing the QCL assumption for m-PDSCH m-TRP, we first need to discuss whether and how to extend above four cases to m-PDSCH m-TRP in Rel-17. This is far from being a trivial problem. For instance, we need to answer the following questions: - Whether and how to extend the TDMASchemeA (intra-slot) or repetitionNumber-r16 (Inter-slot) scheme to multi-PDSCH scenario? How would be the resource allocation of N PDSCHs of TRP1 relative to N PDSCHs of TRP2 (Interlaced in time domain? no special order? or all the PDSCHs of the second TRP are scheduled after those of the first TRP? can some PDSCHs of the two TRPs overlap?....) - Whether and how to extend FDMSchemeA and FDMSchemeB to m-PDSCH case? Are 2N PDSCHs allocated to 2 Frequency beans each with N shared time slots where Frequency bean 1 carries N PDSCHs of TRP 1 and Frequency bean 2 carries N PDSCHs of TRP 2 or interlacing of N PDSCHs of each TRP in Frequency domain is allowed? Similar problems should also be resolved in the case of multi-DCI m-PDSCH m-TRP. For instance, can some or all of the N PDSCHs of TRP1 overlap with some or all N PDSCHs of TRP2? For instance, do we support the time domain location of N PDSCHs of TRP1 to be M slot shifted version of the time domain location of N PDSCHs of TRP2 so that N-M PDSCHs of TRP1 are fully overlapped with N-M PDSCHs of TRP2 while the rest of M slots are non-overlapping? In the case that only non-overlapped PDSCHs are supported and the PDSCHs span at least 2N slots what are the possible implications on, for instance, the UE buffer? - Even if the WA is confirmed, the first immediate problem that needs to be answered is to specify QCL assumption for the case that some or all PDSCHs are before timeDurationForQCL. However, in our view, it is clear that this problem first needs to be resolved for the single TRP case (in Section 3.1). This means that we have to first choose between Case 2 Alt 1 and Case 2 Alt 2 for single TRP and, in the second step, clearly specify the default CQL assumption (e.g., QCL assumption corresponding to the lowest CORESETId in the latest monitoring slot or the QCL assumption corresponding to the first scheduled PDSCH or other alternatives). |
| LG Electronics | We do not object to confirm the previous working assumption. But we prefer to clarify the FFS part. If we simply apply the rule defined in MIMO session for multi-PDSCH scheduling case (as Apple stated), we wonder if simple extension is possible especially for TDM schemes. |

1st round discussion summary

TBU

Beam Management for Shared Spectrum Operation

Observations and Proposals from Contributions

| - Company | - Observations and Proposals from Contributions |
| --- | --- |
| - [Huawei/HiSi, 1] | Proposal 10: In order to mitigate the impact of LBT failure in BFD procedure, support transmitting complementary aperiodic CSI-RS when LBT failure occurs on periodic BFD-RS. |
| - [FUTUREWEI, 2] | Proposal 9: Utilize aperiodic CSI-RS transmission to address impact of LBT failure on periodic RS transmissions intended to support beam failure recovery. Proposal 10: Consider support for low latency beam (QCL-TypeD) switch of periodic RS transmissions after persistent or sustained LBT failure. |
| - [Spreadtrum, 3] | Proposal 5: Aperiodic CSI-RS should be a complement for BFD-RS. |
| - [ZTE/Sanechips, 4] | Proposal 9: Study and evaluate the impact of LBT and the limitation of COT length on the procedure of beam failure detection. |
| - [OPPO, 6] | Proposal 4: Holding the discussion on AP-CSI-RS for BFR/BFD until the LBT procedure has been made clear in agenda item 8.2.6. |
| - [NEC, 7] | Proposal 3: UE should apply the QCL assumption(s) of the smallest CORESET ID on which directinal LBT succeed in the latest slot for each PDSCH when some or all of the scheduled PDSCHs of the multiple PDSCH have scheduling offset less than timeDurationForQCL for shared spectrum. Proposal 4: If the indicated beam in the DCI scheduling the PDSCH is QCLed with the directional LBT beam for the DCI, then no additional LBT is needed for the PDSCHs have scheduling offset equal to or greater than timeDurationForQCL in shared spectrum. |
| - [CATT, 8] | Observation 1: When UE cannot measure the periodic CSI-RS at the scheduled transmission instance for beam management due to LBT failure, gNB could transmit aperiodic CSI-RS and indicate to the UE as the alternative measurement. Proposal 1: Aperiodic CSI-RS could be used as the alternative solution of missed L1 RSRP measurement of periodic CSI-RS due to LBT failure with minimized specification impact. |
| - [xiaomi, 9] | Observation 1: There may be performance loss caused by non-transmitted periodic CSI-RS for beam measurement because gNB cannot get the latest beam measurement results especially for high speed UE. Proposal 4: Aperiodic RS transmission can be triggered to patch a non-transmitted periodic CSI-RS. Observation 2: The existing BFD mechanism may not work well if the periodic CSI-RS for BFD cannot transmitted because of LBT failure. Proposal 6: The beam failure detection procedure should be enhanced if triggering aperiodic CSI-RS to complement the non-transmitted BFD-RS is supported. Observation 3: The beam switching in the same COT will be influenced by the LBT mechanism. |
| - [Ericsson, 10] | Proposal 9 Enhancement of existing BFD procedures by introduction of ap-CSI-RS is not needed for operation in shared spectrum. The existing BFI counter and timer can be adjusted to compensate for occasional LBT failure causing a missing instance (period) of a periodic BFD RS (SS/PBCH block and/or p-CSI-RS). |
| - [Nokia/NSB, 11] | Observation 1: For P-TRS transmissions in the cell, it would be beneficial to have a mechanism to be able to transmit P-TRSs dropped due to LBT failure. Proposal 8: Consider solutions to provide robustness for TRS transmission due to LBT failures, for instance: - A beam specific (SSB specific) aperiodic TRS transmission that could be triggered for one or multiple UEs at a time to “patch” non-transmitted P-TRS using certain beam (certain SSB as QCL-TypeD source) - Multiple transmission opportunities for the P-TRS within a time period Observation 2: More transmission opportunities for the BFD-RS against LBT failures can be supported by the same mechanism used for peridic CSI-RS such as TRS. Proposal 9: In case of directional LBT (if applied), consider impacts on beam management in the COT, e.g. - impact on validity of the configured DL RSs for L1-RSRP measurement and reporting and - impact on beam switching application time within the COT (e.g. the case when the new beam is or is not QCLed with the LBT beam of the COT). Proposal 10: If multi-slot CSI-RS is supported, use slot offset (by reusing the parameter CSI-ResourcePeriodicityAndOffset currently applicable only for periodic and semi-persistent resource) parameter for the aperiodic CSI-RS resource where the offset would be calculated from the slot where the first CSI-RS resource of the same set is allocated. |
| - [Samsung, 12] | Proposal 9: Support multi-slot aperiodic CSI-RS/SRS scheduled by a single DCI for beam management in 60 GHz unlicensed band. Proposal 10: Further investigate the issue on the uncertainty of RS transmission due to LBT for 60 GHz unlicensed band. |
| - [MediaTek, 13] | Proposal 6: Deprioritize the discussion of beam management enhancements for LBT failure handling. |
| - [Intel, 14] | Proposal 6: No special handling of periodic RS transmissions is needed to address interruptions due to LBT failure as well as no special means are needed to distinguish between LBT failures and beam failures. |
| - [NTT DOCOMO, 15] | Proposal 4: For beam management in 52.6-71GHz, discuss the following: - whether to support reporting more than 4 beams for beam reporting in one report instance, if the number of configured CSI-RS resources in a resource set for beam management is increased. - whether to increase the number of configured CSI-RS resources for beam management. |
| - [Sony, 16] | Proposal 1: Support aperiodic CSI-RS for beam failure detection (BFD) and candidate beam determination (CBD) at least for unlicensed band operation. Proposal 4: Study and specify if needed single DCI scheduled multiple aperiodic CSI-RS and/or aperiodic SRS across multiple slots. |
| - [Lenovo/MotM, 17] | Proposal 6: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, the following potential enhancements related to periodic transmissions of RS such as P-TRS should be specified to deal with LBT failure: - Termination of periodic RS transmission on beams where consecutive LBT failures are encountered - Dynamic switching of the QCL assumption (beams) for periodic RS transmission where consecutive LBT failures are encountered, where: - Multiple QCL assumptions (multiple beams) can be configured to the RS resource and beam switch can be triggered once the continuous number of LBT failures reach a certain threshold value |
| - [InterDigital, 18] | Observation 9: Absence of periodic/semi-persistent RSs may impact on performance of fine time/frequency tracking, beam failure recovery and beam/CSI reporting. Proposal 7: Introduce an enhanced mechanism to patch non-transmitted periodic/semi-persistent RSs due to LBT failures. Proposal 8: Support RS transmission based on candidate RSs when LBT fails for periodic/semi-persistent RSs. Proposal 9: Support RS pre-emption based on gNB indication to achieve accurate fine time/frequency tracking, beam failure recovery and beam/CSI. |
| - [LGE, 19] | Proposal #7: The following aspects can be considered to enhance beam management operation when channel access scheme is used for unlicensed spectrum. - How to provide more opportunities of CSI-RS or SRS transmission considering LBT failure - How to enhance beam failure procedure considering not transmitted BFD-RS due to LBT failure |
| - [Convida, 21] | Proposal 3: Enhancement of beam operation for unlicensed bands should be investigated to mitigate interference and optimize system performance due to hidden node for NR from 52.6 GHz to 71 GHz. |

Summary of views

| # | Issue | Companies’ views |
| --- | --- | --- |
| 7.1 | Whether to enhance RS transmissions to deal with LBT failure | Yes: Huawei/HiSi, FUTUREWEI, Spreadtrum, CATT, Xiaomi, Nokia/NSB, Sony, IDCC, LGE, Convida - [Huawei/HiSi]: In order to mitigate the impact of LBT failure in BFD procedure, support transmitting complementary aperiodic CSI-RS when LBT failure occurs on periodic BFD-RS. - [Lenovo/MotM]: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, the following potential enhancements related to periodic transmissions of RS such as P-TRS should be specified to deal with LBT failure: - Termination of periodic RS transmission on beams where consecutive LBT failures are encountered - Dynamic switching of the QCL assumption (beams) for periodic RS transmission where consecutive LBT failures are encountered, where: - Multiple QCL assumptions (multiple beams) can be configured to the RS resource and beam switch can be triggered once the continuous number of LBT failures reach a certain threshold value Need further study/hold the discussion/No support: OPPO, CATT, Ericsson, Intel, NTT Docomo - [CATT]: Aperiodic CSI-RS could be used as the alternative solution of missed L1 RSRP measurement of periodic CSI-RS due to LBT failure with little specification change. - [Ericsson]: Enhancement of existing BFD procedures by introduction of ap-CSI-RS is not needed for operation in shared spectrum. The existing BFI counter and timer can be adjusted to compensate for occasional LBT failure causing a missing instance (period) of a periodic BFD RS (SS/PBCH block and/or p-CSI-RS). - [Intel]: No special handling of periodic RS transmissions is needed to address interruptions due to LBT failure as well as no special means are needed to distinguish between LBT failures and beam failures |
| 7.2 | Multi-slot aperiodic RS | Yes: Nokioa/NSB, Samsung, LGE - [Samsung]: Support multi-slot aperiodic CSI-RS/SRS scheduled by a single DCI for beam management in 60 GHz unlicensed band. - [LGE]: How to provide more opportunities of CSI-RS or SRS transmission considering LBT failure |

1st round discussion

Observation 7

The moderator observed no proposal supported by majority companies in this topic. In addition, 12 companies supporting periodic RS enhancement indicated different preferences on detailed enhancements (e.g., use of aperiodic RS for BFR, periodic TRS enhancement and etc.). Given the remaining time for Rel-17, the moderator believes that there’s not enough time to finalize the required specification enhancement for periodic RS enhancement.

Proposal 7

Conclusion

- No periodic RS enhancement is supported for Rel-17 NR 52-71.

| Company | Input |
| --- | --- |
| Ericsson | Agree with the conclusion proposed by the moderator. As we analyze in our contribution, we don't think enhancement is needed for BFR. Furthermore, it is too late in the WI to be discussing such enhancements. |
| vivo | We support Proposal 7 to conclude this topics. |
| Nokia/NSB | Several companies are still supporting this enhancement, we prefer not to make conclusion in this meeting. |
| MediaTek | We support the conclusion. |
| Qualcomm | Support |
| Samsung | Support Proposal 7 |
| Huawei, HiSilicon | We agree with Nokia. We don’t think this conclusion is necessary. |
| LG Electronics | Support |

1st round discussion summary

TBU

Others

Observations and Proposals from Contributions

| - Company | - Observations and Proposals from Contributions |
| --- | --- |
| - [CATT, 8] | Proposal 2: The beam management frame work should be reused for NR operation in 52.6-71 GHz. Proposal 9: In initial access, the beam adaptation for Msg3 and Msg4 transmission can be adapted based on the beam measurement report from UE. |
| - [xiaomi, 9] | Proposal 5: To support more beams, the maximal number of reference singles in one CSI-RS resource set should be increased. Or, multiple aperiodic CSI-RS resource sets associated with one aperiodic trigger state should be allowed to be used for beam measurement. |
| - [Ericsson, 10] | Proposal 2 QCL indication for cross-carrier scheduling - Support cross-carrier scheduling for multiple PDSCHs scheduled by a single DCI - For single-TRP operation, the following is supported in order to enable multi-PDSCH scheduling: - If the UE is not configured with enableDefaultBeamForCCS - The UE expects that the PDSCH scheduling offset for all scheduled PDSCHs ≥ timeDurationForQCL analogous to Rel-16 - If the UE is configured with enableDefaultBeamForCCS - The existing Rel-16 default QCL assumption for single-PDSCH scheduling is applied for all scheduled PDSCHs when either or both of the following apply: - The PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL - The TCI field is absent from the scheduling DCI - Note: the existing Rel-16 default QCL assumption corresponds to the activated TCI state with the lowest ID applicable to the PDSCH in the active BWP of the scheduled cell - FFS: Details for multi-TRP operation Proposal 10 Enhancement of the number of explicitly configured RSs for BFD (SS/PBCH blocks and/or p-CSI-RS) is not needed. Proposal 11 For the new beam identification (NBI) procedure, the 28 symbol window for decoding PDCCH in recoverySearchSpaceId may need to be revisited for the case that a serving cell is configured with 480 or 960 kHz SCS. |
| - [NTT DOCOMO, 15] | Proposal 5: Beam failure detection/recovery procedure in NR 52.6-71GHz can consider following potential enhancements, - whether to introduce a new time gap to apply new beam configuration after receiving BFR response from gNB - whether to increase the number of candidate beams included in set - if the discussion on the number of CSI-RS resources for beam management is deprioritized in Rel-17, it can also be low priority. |
| - [Sony, 16] | Proposal 5 : Beam alignment during initial access procedure should be considered for NR above 52.6 GHz. |
| - [Lenovo/MotM, 17] | Proposal 7: For NR operation between 52.6 GHz and 71 GHz, Rel-17 common TCI state indication should be supported for multi-PDSCH scheduling Proposal 8: For NR operation between 52.6 GHz and 71 GHz, how to determine the applied TCI state for the multiple PDSCH in continuous slots when the indicated common TCI state is changed but the UE cannot switch it RX beam within the CP should be further discussed Proposal 9: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs with different TBs are scheduled by the DCI indicating a common TCI state, the ACK/NACK of any one scheduled PDSCH can be used as the ACK for the DCI |
| - [Apple, 20] | Proposal 3: Consider a mechanism to enable/disable the PDCCH monitoring within the multi-PDSCH duration to improve resource efficiency. |
| - [Convida, 21] | Proposal 1: Rel-17 FeMIMO unified TCI framework like TCI state indication for PDCCH can be considered for NR from 52.6 GHz to 71 GHz. |
| - [Qualcomm, 22] | Proposal 7: Support dedicated configuration of default PDSCH beam for better optimization flexibility. - gNB can dynamically update the default PDSCH beam via MAC-CE. Proposal 12: Support partial BFR for single TRP. Proposal 13: Support UE report of recommended SSB in Msg3/A in initial access. Proposal 14: Support dynamic beam update of periodic channel/RS. Proposal 15: Investigate sub-band based beam report. Proposal 16: The contents of configured TCI states can be dynamically updated. - The contents may include any QCL source RS ID, e.g. both TypeA/D RS IDs, and corresponding BWP/CC ID. |

1st round discussion

Summary of views

| # | Issue | Companies’ views |
| --- | --- | --- |
| 8.1 | Beam reporting/alignment during initial access procedure | Yes: CATT, Sony, Qualcomm No: |
| 8.2 | Dynamic beam update | Yes: Lenovo/MotM, Qualcomm, NTT Docomo No: |
| 8.3 | Increase the number of configured CSI-RS resources | Yes: Xiaomi, NTT Docomo No: Ericsson |
| 8.4 | Considering Rel-17 feMIMO unified TCI framework for multi-PDSCH/PUSCH scheduling | Yes: Lenovo/MotM, Convida No: |
| 8.5 | Enhancement on 28 symbol window for decoding PDCCH for the NBI procedure of BFR | Yes: Ericsson, NTT Docomo No: |
| 8.6 | PDCCH monitoring activation/deactivation within multi-PDSCH | Yes: Apple No: |
| 8.7 | Partial BFR | Yes: Qualcomm No: |
| 8.8 | QCL indication for cross carrier scheduling | Yes: Ericsson No: |
| 8.9 | Default PDSCH beam indication via MAC CE | Yes: Qualcomm No: |

Observation 8

No proposal supported by majority companies is observed by the moderator. Please provide views on the above proposals in the summary.

| Company | Input |
| --- | --- |
| Ericsson | We think a small spec clarification is needed for the case of cross-carrier scheduling when multi-PDSCHs are scheduled with single DCI. The existing spec text in 38.214 Section 5.1.5 has a default QCL assumption rule depending on whether or not the RRC parameter enableDefaultBeamForCCS is configured. Currently, the rule is written in terms of single-PDSCH scheduling. The update that is needed for multi-PDSCH scheduling is to clarify that when the RRC parameter is configured, that the default QCL assumption is applied for all scheduled PDSCHs when any scheduled PDSCH has scheduling offset < timeDurationForQCL. Hence, we think this issue should be discussed in parallel with the other QCL indication issues in Section 3 of this FL summary. |

    - Proposal 8

TBU

1st round discussion summary

TBU

References

- R1-2108770	Discussion on the beam management procedures for 52-71GHz spectrum	Huawei, HiSilicon,

- R1-2108785	Views on Beam management for Beyond 52.6 GHz		FUTUREWEI,

- R1-2108903	Discussion on beam management for above 52.6GHz	Spreadtrum Communications,

- R1-2108937	Discussion on the beam management for 52.6 to 71GHz	ZTE, Sanechips,

- R1-2108962	Discussions on beam management for new SCSs for NR operation from 52.6GHz to 71GHz	vivo,

- R1-2109073	Discussion on beam management for new SCSs	OPPO,

- R1-2109119	Beam management enhancement for NR from 52.6GHz to 71GHz	NEC,

- R1-2109211	Beam management for new SCSs for up to 71GHz operation	CATT,

- R1-2109403	Discussion on beam management for new SCSs	xiaomi,

- R1-2109437	Beam Management for New SCSs	Ericsson,

- R1-2109445	Beam Management Aspects	Nokia, Nokia Shanghai Bell,

- R1-2109479	Beam management for new SCSs for NR from 52.6 GHz to 71 GHz	Samsung,

- R1-2109561	Beam management discussion for 52.6-71 GHz NR operation	MediaTek Inc.,

- R1-2109601	Discussion on Beam management aspects for extending NR up to 71 GHz	Intel Corporation,

- R1-2109668	Beam based operation for new SCSs for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.,

- R1-2109780	Beam management enhancement for NR from 52.6GHz to 71GHz	Sony,

- R1-2109899	Beam-management enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility,

- R1-2109906	Discussions on beam management for new SCSs	InterDigital, Inc.,

- R1-2109964	Enhancements for beam management to support NR above 52.6 GHz	LG Electronics,

- R1-2110024	Beam Management for New SCSs	Apple,

- R1-2110112	Beam management for NR from 52.6 GHz to 71 GHz	Convida Wireless,

- R1-2110175	Beam managment for new SCS	Qualcomm Incorporated.