3GPP TSG RAN WG1 #106bis-e			 R1-2110608

e-Meeting, October 11th – 19th, 2021

Title:	Reply LS on UE Power Saving

Response to:	LS on UE power saving from RAN2 (R2-2108917)

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN1, MediaTek

To:	RAN2

Cc:

Contact person:	Weide Wu

Weide.wu@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

First RAN1 would like to thanks RAN2 for sharing RAN2 agreements on Rel-17 UE power saving enhancements.

For RAN2 request on TRS/CSI-RS for RRC Idle and Inactive, RAN1 would like to provide the following information.

- For the information on TRS/CSI-RS availability, RAN1 has discussed availability indication and achieved the following agreements:

From RAN1#104-e: Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

From RAN1#104-bis-e: Working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signaling

From RAN1#105-e: Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - FFS maximum number of configured RS resources per physical layer availability indication to support. - FFS whether availability/unavailability information is for all or some of configured RS resources Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB Agreement: Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured. - FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

From RAN1#106-e: Agreement Support at least one of the following alternatives - Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion. - Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion Note: The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication. Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only. Agreement L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where - the time duration can be determined based on at least one from the following (to be down-selected): - Alt-1: configured by higher layer - Alt-2: a predefined/configured window - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s) - Alt-4: until when the UE receives another availability indication - A combination of alternatives or other alternatives is not precluded. - the reference point can be determined as at least one from the following (to be down-selected): - Alt-1: start of next PO or DRX cycle - Alt-2: time location where UE receives the indication - Note: the time location is subject to application delay if agreed - Alt-3: start of current PO or DRX cycle where UE receive the indication - Alt-4: a time location which is configured by higher layer - A combination of alternatives or other alternatives is not precluded. Agreement For a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block

From RAN1#106-bis-e: Conclusion No consensus to support SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration.

- For the information on list of the parameters and a potential structure for TRS/CSI-RS information, RAN1 has discussed the configurations and achieved the related agreements as summarized in the Appendix. RAN1 will summarize all the list of potential RRC parameters and structures for RAN1-related Rel-17 features in a common LS on RRC parameters to RAN2 #116-e meeting.

In addition to TRS/CSI-RS for RRC Idle and Inactive, RAN1 also achieved the following agreements related to UE paging subgrouping and would also like to share the information for the purpose of cross-WG alignment:

From RAN1 #105-e: Agreement: For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported.

From RAN1 #106-bis-e: Agreement For NR Rel-17, paging indications to UE subgroups are carried only in PEI. Agreement For PEI, a new DCI format is supported to include at least paging indications to UE group(s)/subgroups of the associated PO(s) - One bit in the DCI payload indicating one UE subgroup of a PO or one UE group/PO - The maximum number of total bits for paging indication field in PEI DCI format is x - One PEI can be configured to indicate up to 4 PO(s) in a PF - FFS whether to supporting map PEI to 3 POs in a PF - FFS: 1 PEI for POs across multiple PFs - FFS: value of x

# 2	Actions

To RAN2:

RAN1 respectfully asks RAN2 to take the above information into account for their future work.

# 3	Dates of next RAN1 meetings

TSG RAN WG1 Meeting #107-e		11 November – 19 November, 2021	e-Meeting

TSG RAN WG1 Meeting #107-bis-e	17 January – 25 January, 2022 		e-Meeting

TSG RAN WG1 Meeting #108-e		21 February – 03 March, 2022 		e-Meeting

Appendix: Collection of Agreements Related to TRS/CSI-RS for RRC Idle and Inactive

The following table summarizes all related RAN1 agreements to list of the parameters and a potential structure for TRS/CSI-RS information:

From RAN1#104-e: Agreements: Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least: - powerControlOffsetSS, - scramblingID - firstOFDMSymbolInTimeDomain, - startingRB. - nrofRBs, - FFS other parameters - FFS applicable values Agreements: Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details (including whether or not to restrict the RS to be TRS only) Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

From RAN1#104-bis-e: Agreement: SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0. Agreement: Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details of the QCL information, e.g. associated SSB index Agreement: IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. - Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration

From RAN1#105-e: Agreement: Support applicable values for the following configuration parameters as below. - powerControlOffsetSS: {-3, 0, 3, 6}dB - scramblingID: 0 to 1023 - firstOFDMSymbolInTimeDomain: 0 to 9 - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4 - startingRB: 0 to 274 - nrofRBs: 24 to 276 Agreement: The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. - FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration - FFS: QCL type, which is predetermined Agreement: Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset {10, 20, 40, 80} ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - FFS Configuration index - details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source

From RAN1#106-e: Agreement For a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block