3GPP TSG RAN WG1 #106bis-e    			       	 R1-2110091

e-Meeting, Oct 11th – Oct 19th, 2021

Agenda Item:	8.5.4

Source: 	LG Electronics

Title: 	Discussion on latency improvements for NR positioning

Document for:   Discussion and decision

Introduction

This contribution discusses Rel-17 NR positioning enhancements specifically in physical layer latency reduction for NR positioning.

Physical layer latency reduction

Reduced the number of samples for positioning measurement

For latency reduction, reduced number of samples was discussed and following agreement was made in the previous meeting [1].

Agreement: Subject to UE capability, support LMF to explicitly request UE to report the measurement with either M-sample or 4-sample, if RAN4 has supported M-sample measurement. - FFS signalling details.

When a reduced samples are used for measurement for purpose of latency reduction, the measurement accuracy would be degraded. So, when LMF requests positioning measurement through NRPP message, it needs for LMF to provide a parameter for distinguishing between a specific case (e.g. 1<=M<4 sample(s)) and the normal case (e.g. 4 samples) depending on use cases and UE capability. Currently, there are only two types of measurement request information elements such as common IEs and positioning method-specific IEs. The information for distinguishing use case could be included at least in one of them. An example is shown as below:

Example 1) In positioning method-specific IEs:

-- ASN1START NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE { nr-DL-PRS-RstdMeasurementInfoRequest-r16 ENUMERATED { true } OPTIONAL,-- Need ON nr-RequestedMeasurements-r16 BIT STRING { prsrsrpReq (0) } (SIZE(1..8)), nr-AssistanceAvailability-r16 BOOLEAN, nr-DL-TDOA-ReportConfig-r16 NR-DL-TDOA-ReportConfig-r16 OPTIONAL, -- Need ON additionalPaths-r16 ENUMERATED { requested } OPTIONAL, -- Need ON reducedsample-r17 ENUMERATED { requested } or {SIZE(1..4)} OPTIONAL, -- Need ON ... }

Example 2) In common IEs:

CommonIEsRequestLocationInformation ::= SEQUENCE { locationInformationType LocationInformationType, triggeredReporting TriggeredReportingCriteria OPTIONAL, -- Cond ECID periodicalReporting PeriodicalReportingCriteria OPTIONAL, -- Need ON additionalInformation AdditionalInformation OPTIONAL, -- Need ON qos QoS OPTIONAL, -- Need ON environment Environment OPTIONAL, -- Need ON locationCoordinateTypes LocationCoordinateTypes OPTIONAL, -- Need ON velocityTypes VelocityTypes OPTIONAL, -- Need ON reducedsample-r17 ENUMERATED { requested } or {SIZE(1..4)} OPTIONAL, -- Need ON ..., [[ messageSizeLimitNB-r14 MessageSizeLimitNB-r14 OPTIONAL -- Need ON ]], [[ segmentationInfo-r14 SegmentationInfo-r14 OPTIONAL -- Need ON ]] }

Proposal 1:

- For request location information, introduce a parameter for distinguishing between a specific case (e.g. 1<=M<4 sample(s)) and the normal case (e.g. 4 samples) which is accompanied in request location information. The parameter can be included in the following IEs:

  - Common IEs for request location information (e.g. CommonIEsRequestLocationInformation)

  - Positioning method specific IEs (e.g. NR-DL-TDOA-ProvideLocationInformation, NR-DL-AoD-ProvideLocationInformation, NR-Multi-RTT-ProvideLocationInformation, etc.)

Latency improvements with respect to measurement gap configuration

Regarding latency reduction for measurement gap (MG), followings were agreed in the previous meeting [1].

Agreement: For the purpose of positioning latency reduction, with potential support of a new mechanism of MG request, consider the following options with a decision to be made in RAN1#106b. - Option. 1: by LMF (via a NRPPa message) - Option. 2: by UE (via UCI or UL MAC CE) Agreement: For the purpose of positioning latency reduction, with potential support a new MG activation and deactivation procedure, consider the following options with a decision to be made in RAN1#106b (and RAN4 to be informed about any decision made) - Option. 1: DCI - Option. 2: DL MAC CE - Option. 3: UE autonomously applies the MG FFS whether deactivation can be implicit via configurable number of the MG occasions.

Regarding MG request, LMF generally requests that UE operates positioning measurement through NRPP message, and then UE would check the suitability of pre-configured MG and may request MG configuration depending on the result of the suitability check. In this respect, if information related with MG request is delivered by LMF via a NRPPa message in parallel (option#1, by LMF), it can provide a benefit of latency reduction. In case of option #2(by UE), it also seems that delivering message through low layer signaling (UCI or MAC CE) can provide a benefit of  latency reduction rather than current RRC signaling. In this respect, we are supportive of all options.

Proposal 2:

- For the purpose of positioning latency reduction, RAN1 can support all options (initiated by LMF (option #1) and by UE (option #2)) for MG request.

For MG activation/deactivation, both option #1 (DCI) and option #2 (MAC-CE) are supportive because e of same reason that low layer singling has benefit of latency rather than RRC signaling. In general, obtaining information through DCI is faster than MAC-CE, but available resources like a bits are limited. Considering it, it seems that supporting both options (DCI and MAC-CE) is appropriate and it can be used for supporting various use case. Even though the decision on details is up to higher layer, some examples such as that major components of information are transmitted through DCI and the detail (or minor) components are transmitted through MAC-CE respectively.

Proposal 3:

- For the purpose of positioning latency reduction, RAN1 can support both DCI and MAC-CE for MG activation/deactivation. Also, following can be applied.

  - Major information through DCI, detail (or minor) information through DCI.

Latency improvements with respect to PRS measurement reporting

In the previous RAN2 #115-e meeting, some discussions regarding low-layer triggering and CG-based measurement reporting has been discussed as shown below:

Proposal 9: RAN2 is proposed to down-prioritize discussion on enhancements related to lower-layer triggering of measurement request/reporting for latency reduction for this release. Proposal 10 (Low priority): With regard to configured UL grant for location reporting, RAN2 can discuss the following aspects for CG-based solution in RRC_CONNECTED mode:  How the CG parameters are configured: o Based on the PRS measurement period and starting position in time of the other TRPs o Definition of additional finer time granularities for both reportingAmount and reportingInterval IEs within the periodicalReporting configuration in LPP message  How the CG information is indicated to the gNB: o CG configuration information via LMF o CG configuration information via UE

We think that RAN1 needs to discuss about it more closely and providing result of our discussion would be obviously helpful for RAN2 to discuss about it. If CG-based PUSCH is applied for measurement report, it can be expected to reduce latency reduction by omitting some delays related to SR and UL grant.

Proposal 4:

- For latency reduction of positioning measurement reporting, preconfigured resource based measurement reporting (e.g., CG-based PUSCH) should be introduced.

Since gNB cannot see the LPP message, it is hard to know the UE’s exact measurement time. For this reason, the signaling of activation is necessary even though a CG-based PUSCH is provided. To resolve the ambiguity, ‘the lower layer signaling for triggering/activation of measurement gap(s) (MG(s)) (which is discussed as a method for MG enhancement in the previous meeting [2]) can be reused for activation of CG-based PUSCH resources for positioning measurement reporting as shown Figure 2.

Figure 2: The example of CG-based positioning measurement report (with MG)

Proposal 5:

- If CG-based PUSCH is applied for positioning measurement report, ‘the lower layer signaling for triggering/activation of measurement gap(s) (MG(s)) (which is discussed as a method for MG enhancement in the previous meeting [2]) can be reused for activation of CG-based PUSCH resources for positioning measurement reporting.

Similarly, the case where UE measures PRS without MG can be considered. If lower layer signaling for triggering/activation of MG(s) indicates which CG-based PUSCH is used for measurement report as shown in Figure 3, gNB can decode the measurement report properly. So, the information on indicating which CG-based PUSCH is used is necessary to be included in lower layer signaling when CG-based PUSCH is supported for the without MG case.

Figure 3: The example of CG-based positioning measurement report (without MG)

Proposal 6:

- The information for indicating which CG-based PUSCH is used for is necessary to be included in lower layer signaling for triggering/activation of MG(s) when CG-based PUSCH is supported for the MG without case.

Conclusion

In this contribution, we have discussed physical layer latency reduction for DL-positioning in Rel-17, and our proposals are summarized below.

Reduced the number of samples for positioning measurement

Proposal 1:

- For request location information, introduce a parameter for distinguishing between a specific case (e.g. 1<=M<4 sample(s)) and the normal case (e.g. 4 samples) which is accompanied in request location information. The parameter can be included in the following IEs:

  - Common IEs for request location information (e.g. CommonIEsRequestLocationInformation)

  - Positioning method specific IEs (e.g. NR-DL-TDOA-ProvideLocationInformation, NR-DL-AoD-ProvideLocationInformation, NR-Multi-RTT-ProvideLocationInformation, etc.)

Latency improvements with respect to measurement gap configuration

Proposal 2:

- For the purpose of positioning latency reduction, RAN1 can support all options (initiated by LMF (option #1) and by UE (option #2)) for MG request.

Proposal 3:

- For the purpose of positioning latency reduction, RAN1 can support both DCI and MAC-CE for MG activation/deactivation. Also, following can be applied.

  - Major information through DCI, detail (or minor) information through DCI.

Latency improvements with respect to PRS measurement reporting

Proposal 4:

- For latency reduction of positioning measurement reporting, preconfigured resource based measurement reporting (e.g., CG-based PUSCH) should be introduced.

Proposal 5:

- If CG-based PUSCH is applied for positioning measurement report, ‘the lower layer signaling for triggering/activation of measurement gap(s) (MG(s)) (which is discussed as a method for MG enhancement in the previous meeting [2]) can be reused for activation of CG-based PUSCH resources for positioning measurement reporting.

Proposal 6:

- The information for indicating which CG-based PUSCH is used for is necessary to be included in lower layer signaling for triggering/activation of MG(s) when CG-based PUSCH is supported for the MG without case.

Reference

- 3GPP RAN1, Chairman’s Notes RAN1#106-e meeting.

- 3GPP RAN1, Chairman’s Notes RAN1#105-e meeting.