3GPP TSG RAN WG1 #106bis-e		R1- 2110255

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.12.1

Source:	ASUSTeK

Title:	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs

Document for:		Discussion and Decision

- Introduction

The support of NR Multicast and Broadcast Services (MBS) is introduced in Rel-17 as a work item [1] to achieve group-scheduling. The objectives of NR MBS are specified in [1] and some agreements on the mechanisms to support group scheduling for RRC_CONNECTED UEs were reached at the RAN1#102-e through RAN1#106-e meeting [2]~[7]. In this contribution, we discuss the issues about BWP switching handling, SPS multicast handling when switching BWPs, and possible solutions for supporting group-scheduling for the MBS WI and some proposals are given.

- Discussion

2-1. BWP switch handling for multicast

It was agreed that for multicast of RRC-CONNECTED UEs, a common frequency resource (CFR) for group-common PDCCHs and PDSCHs is confined within the frequency resource of a dedicated unicast BWP configured for a UE to support simultaneous receptions of unicast and multicast PDCCHs/PDSCHs in the same slot. Also, it was agreed that the common frequency resource for multicast of RRC_CONNECTED UEs includes one PDCCH-config and one PDSCH-config separate from the PDCCH-config and PDSCH-config of the dedicated unicast BWP.

For simplicity, here we denote a dedicated unicast BWP in which a common frequency resource is configured/confined and in which a UE is capable of receiving multicast PDCCHs/PDSCHs as an “MBS-capable BWP”. On the other hand, we denote a dedicated unicast BWP which does not contain a common frequency resource or in which a UE is incapable of receiving multicast PDCCHs/PDSCHs as an “MBS-incapable BWP”.

A UE may be configured with an MBS-capable BWP and an MBS-incapable BWP simultaneously. If active BWP is the MBS-capable BWP, the UE is able to receive multicast PDCCHs/PDSCHs. However, if at a moment the active BWP is switched to the MBS-incapable BWP, the UE is not able to receive the multicast PDCCHs/PDSCHs. In this case, it needs some studies on how to resume multicast PDCCH/PDSCH receptions, e.g. the may automatically switches back to the MBS-capable BWP after a certain time duration for multicast PDCCH/PDSCH receptions.

Observation 1: A UE is not able to receive multicast PDCCHs/PDSCHs if the UE’s active BWP is switched to an MBS-incapable BWP.

Proposal 1: If a UE’s active BWP is switched from an MBS-capable BWP to an MBS-incapable BWP, it needs some studies for the UE to resume multicast PDCCH/PDSCH receptions, e.g. the UE automatically switches back to the MBS-capable BWP after a certain time duration.

2-2. CFR sharing between BWPs

Based on the current agreements, the CFR is configured within a UE’s dedicated BWP for multicast receptions. Consider the scenario that a UE has two dedicated BWPs (e.g. BWP 1 and BWP 2) where one of the two BWPs (e.g. BWP 1) is configured with a CFR. If the UE’s active BWP is BWP 1, the UE is able to receive multicast PDCCH/PDSCH in the CFR in BWP 1. However, if the frequency resource of BWP 2 also fully covers the CFR, the UE is unable to receive multicast PDCCH/PDSCH in the CFR in BWP 2, which limits the multicast capabilities/throughputs for the UE. The design of the configuration of a CFR can be further studied or some mechanisms can be specified to better improve the multicast scheduling capability.

Proposal 2: CFR sharing mechanisms should be further studied to improve the multicast scheduling capability.

2-3. Semi-persistent scheduling (SPS) for multicast

2-3-1. SPS handling when switching BWPs

In NR Rel-15/16, an SPS configuration is configured per BWP (as in BWP-DownlinkDedicated RRC IE for a BWP). When a UE’s active BWP is switched from BWP 1 to BWP 2, any activated SPS configuration on BWP 1 would be terminated (no matter whether BWP 2 comprises frequency resources of the activated SPS or not), which is specified in 38.321.

This situation can be improved for the multicast reception scenario. When BWP 1 and BWP 2 both fully covers a CFR and if the CFR can be shared between the two BWPs (based on some mechanisms), the SPS multicast PDSCH receptions which are activated on BWP 1 may not be interrupted if the UE’s active BWP is switched to BWP 2. In this case, no (re-)activation PDCCH is needed on BWP 2 to resume the SPS multicast PDSCH receptions.

Proposal 3: SPS multicast PDSCH receptions are not interrupted in a CFR when switching between two BWPs if the CFR can be shared between the two BWPs.

2-3-2. SPS multicast PDSCH activation via UE-specific PDCCH

It was discussed in the previous meetings whether to support using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration. Using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration may be beneficial. For example, the transmission reliability of a UE-specific PDCCH is typically higher than a multicast PDCCH due to possibly better UE-specific beam selection. Also, for a UE that newly joins the group, using a UE-specific PDCCH, instead of a multicast PDCCH, for activating an SPS multicast PDSCH configuration is more reasonable because the SPS multicast PDSCH configuration may be already activated for some UEs and it is unnecessary for them to receive the activation PDCCH of the same SPS multicast PDSCH configuration again.

However, if using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration is supported, how to indicate the time-domain resources, e.g. the slot number and the symbols in the slot, of the next SPS multicast PDSCH occasion corresponding to the SPS multicast PDSCH configuration still need some discussion.

Proposal 4: NR multicast supports using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration.

2-4. Multicast receptions for carrier aggregation (CA)

Based on an agreement (quoted below) made on the RAN1#105-e meeting [6] and the RAN1#106-e meeting [7], the search space set for monitoring multicast PDCCHs of PTM scheme 1 would be a kind of common search space (CSS) set.

[6] Agreement: For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, Alt 2 is supported: - Alt 2: support a Type-x CSS - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS. - FFS: Whether the Type-x CSS is a Type-3 CSS [7] Conclusion: The specification impact of having a new Type-x CSS for GC-PDCCH in RRC_CONNECTED state can be studied and discussed further.

As a baseline, a multicast PDCCH will reuse existing fields in DCI format 1_0 for the first DCI format and will reuse existing fields in DCI format 1_1 for the second DCI format. Therefore, the CSS set for monitoring multicast PDCCHs may be similar to a CSS set in which a UE monitors DCI formats with CRC-scrambled by a C-RNTI, a MCS-C-RNTI, or a CS-RNTI, i.e. a Type-3 CSS with DCI formats with CRC-scrambled by a C-RNTI, a MCS-C-RNTI, or a CS-RNTI. Currently in the NR Rel-15/16 standard, such a CSS can only be configured on a PCell, which means that a UE may only be configured to monitor multicast PDCCHs of PTM scheme 1 on a PCell. In this situation, when the UE requires more and more MBS/multicast services, the traffic, including control channels and data channels, on the PCell may become congested. To overcome this issue, MBS service may need to utilize carrier aggregation to get higher multicast throughputs.

Observation 2: A UE may only be configured to monitor multicast PDCCHs of PTM scheme 1 on a PCell.

Observation 3: When a UE requires more and more MBS/multicast services, the traffic on the PCell may become congested.

In order not to increase the UE monitoring capability requirement too much, cross-carrier scheduling (CCS) is supported for CA scenarios. Cross-carrier scheduling may be supported for the multicast scheduling feature as well. Here we provide some possible solutions for multicast PDCCH monitoring and multicast PDSCH receptions for CA and CCS.

Concept 1: The Rel-15/16 CSS configuration restriction is retained, where a CSS can only be configured on a PCell. The UE can be configured with one or more G-RNTI associated with one or more serving cell, including the PCell and SCell(s) in a cell group, where each G-RNTI is associated with only one cell. The UE monitors the one or more G-RNTI on the multicast CSS only on the PCell. The data channel resources on the SCell(s) may be utilized for multicast PDSCH receptions. The control resources on the PCell may still have congestion issues.

Concept 2: The Rel-15/16 CSS configuration restriction is slightly modified. In this case, the multicast CSS can be considered as a special USS, which can be monitored on an SCell using one or more G-RNTI. The control channel resources on the SCell(s) can be further utilized.

It is important that RAN1 further studies the possibilities of supporting carrier aggregation and cross-carrier scheduling for multicast.

Proposal 5: RAN1 further studies the possibilities of supporting carrier aggregation and cross-carrier scheduling for multicast.

2-5. The support of PTM transmission scheme 2

PTM transmission scheme 2 uses a UE-specific PDCCH to schedule a group-common PDSCH. PTM scheme 2 may bring respective benefits to control channel and data channel like better PDCCH reliability due to UE-specific beam selection and reducing data channel congestion by using a group-common PDSCH, instead of many UE-specific PDSCHs, to a group of UE. Therefore, we think introducing PTM scheme 2 for initial transmissions and retransmissions for multicast may be beneficial. Also, the design or fields of a PTM scheme 2 UE-specific PDCCH may be similar to the existing DCI format 1_0, 1_1 with CRC-scrambled by a C-RNTI, which may not take as much effort as the PTM scheme 1 PDCCH design does.

Also, we found that using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration is very similar to PTM transmission scheme 2, where both of them use a UE-specific PDCCH for scheduling group-common PDSCHs. Therefore, if using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration is supported, PTM transmission scheme 2 should be supported as well.

Proposal 6: PTM transmission scheme 2 for initial transmissions and retransmissions is supported for multicast.

- Conclusion

According to the discussion above, some observations and proposals are given.

Observation 1: A UE is not able to receive multicast PDCCHs/PDSCHs if the UE’s active BWP is switched to an MBS-incapable BWP.

Observation 2: A UE may only be configured to monitor multicast PDCCHs of PTM scheme 1 on a PCell.

Observation 3: When a UE requires more and more MBS/multicast services, the traffic on the PCell may become congested.

Proposal 1: If a UE’s active BWP is switched from an MBS-capable BWP to an MBS-incapable BWP, it needs some studies for the UE to resume multicast PDCCH/PDSCH receptions, e.g. the UE automatically switches back to the MBS-capable BWP after a certain time duration.

Proposal 2: CFR sharing mechanisms should be further studied to improve the multicast scheduling capability.

Proposal 3: SPS multicast PDSCH receptions are not interrupted in a CFR when switching between two BWPs if the CFR can be shared between the two BWPs.

Proposal 4: NR multicast supports using a UE-specific PDCCH to activate an SPS multicast PDSCH configuration.

Proposal 5: RAN1 further studies the possibilities of supporting carrier aggregation and cross-carrier scheduling for multicast.

Proposal 6: PTM transmission scheme 2 for initial transmissions and retransmissions is supported for multicast.

Reference

- RP-201038, ”WID revision: NR Multicast and Broadcast Services”, Huawei, HiSilicon.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #102-e, e-Meeting, August 17th - 28th, 2020.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #103-e, e-Meeting, October 26th - November 13th, 2020.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #104-e, e-Meeting, January 25th - February 5th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #104b-e, e-Meeting, April 12th – 20th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #105-e, e-Meeting, May 10th – 27th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #106-e, e-Meeting, August 16th – 27th, 2021.