3GPP TSG-RAN WG1 Meeting #106-bis-e	R1-2110348

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.12

Source:	Ericsson

Title:	Views on UE features for NR MBS

Document for:	Discussion, Decision

- Introduction

This contribution provides our view on the list of UE features for NR MBS in release17, based on the initial email discussion outcome in R1-2108679.

- UE Features for NR MBS

## (33-1)  Broadcast feature

In the broadcast features, DCI format should be in bracket to signify that a new DCI format value should be devised as “Fallback MBS DCI”.  Moreover, the feature 33-1/1 mention MCCH-RNTI, which has not been agreed, and should be removed for now. Finally, the support of interslot TDM between unicast and broadcast is limited to RRC CONNECTED state.

The following shows the revise feature 33-1 (unchanged columns removed)

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format [1_0] with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots in RRC_CONNECTED. - Support MCCH change notification indication via DCI. |
| --- | --- | --- | --- |

## (33-2)  Dynamic Scheduling for multicast

Feature 33-2/5 is based on the following agreement:

Agreement: For Rel-17 MBS UE, the UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).

Therefore, feature 5 in 33-2 should be per CC, not per UE.  Additionally, the new fallback/non fallback DCIs for MBS are not yet agreed but will be different from formats 1_0 or 1_1. We propose to put the format in brackets for now. Finally, the support of CFR configuration feature can be very large, and it is not entirely clear yet whether CFR needs to be configured at all for some cases (for example, when MBS bandwidth is the same as the active BWP.  Therefore we propose to put it in bracket for now and make progress on what is included under the support of CFR.

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - [Support of CFR configuration for multicast.] - Support of CORESET and common search space configuration for multicast. - Support of DCI format [1_0 / 1_1] with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. |  | Yes |  |  | Per UE Feature 32-2-5 per CC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## (33-3-1, 33-3-2)  FDM and TDM of unicast and multicast

For both of these features, HARQ codebook support also should include unicast and multicast, as it is possible to create a common codebook. The following change is proposed:

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast and unicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast and unicast. |
| --- | --- | --- | --- |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM-ed Type-1 HARQ-ACK codebook for multicast and unicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast and unicast. |

## (33-5-1) SPS for multicast

For SPS it is up to the gNB to configure an SPS configuration for multicast or unicast, and there can be more than one SPS configuration. Moreover, how retransmission is supported for SPS should also be a separate feature.  Finally we propose that feature 4 is clarified to be relevant for SPS activation and release. The following change is proposed:

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common activation with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH - Support of PTM retransmission for SPS multicast - Support of PTP retransmission for SPS multicast |
| --- | --- | --- | --- |

# Conclusion

We have proposed to modify/append 4 UE features for NR MBS. A full UE feature list in tracked changes is provided in section 4.

- References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#106e

- UE Feature List for NR_MBS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - . - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format ¦1_0] with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots in RRC_CONNECTED. - Support MCCH change notification indication via DCI. |  | Yes |  |  | Per UE | No | No |  |  | Optional without capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - [Support of CFR configuration for multicast.] - Support of CORESET and common search space configuration for multicast. - Support of [DCI format 1_0 / 1_1] with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. |  | Yes |  |  | Per UE Feature 32-2-5 per CC | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast and unicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast and unicast. | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM-ed Type-1 HARQ-ACK codebook for multicast and unicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast and unicast. | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Intra-slot TDM-ed multiple unicast PDSCH and/or multiple group-common PDSCH | - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common activation with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH - Support of PTM retransmission for SPS multicast - Support of PTP retransmission for SPS multicast | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |