3GPP TSG RAN WG1 Meeting #106bis-e 	R1-2110421

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon (Moderator)

Title:	Summary of CSI enhancements for MTRP and FDD (Round 1)

Document for:	Discussion and Decision

# Introduction

# Summary of CSI enhancement for FDD

Proposal 1: For Rel-17 PS codebook, down-select one from the following Alts for rank 3 and 4:

- Alt 1: Support layer-common port selection

- Alt 2: Support layer-common port-selection among layers 3 and 4. Moreover, port-selection among layers 1~2 and layers 3~4 can be different.

Alt 1: Qualcomm, Lenovo/Mot, ZTE, vivo, Ericsson, Nokia/NSB, DOCOMO, OPPO, Spreadtrum, CATT, LGE, Fraunhofer IIS/Fraunhofer HHI, MTK

Alt 2: Samsung, Huawei/Hisi

Proposal 2: For Rel-17 PS codebook, down-select one from the following Alts for rank 3 and 4:

- Alt 1: Support layer-common Wf , when N>Mv and Mv=2

- Alt 2: Support layer-common Wf among layers 3 and 4 layer-common when N>Mv and Mv=2. Moreover, reported Wf among layers 1~2 and layers 3~4 can be different.

- Alt 3: Support Mv=1 for rank 3 and 4

Alt 1: Qualcomm, Lenovo/Mot, ZTE, vivo, Nokia/NSB, DoCoMo, Oppo, Spreadtrum, CATT, LGE, Fraunhofer IIS/Fraunhofer HHI, MTK(2nd)

Alt 2: Huawei/Hisi

Alt 3: Samsung, MTK(1st)

Proposal 3: For Rel-17 PS codebook Rank 3-4, support layer-specific non-zero coefficient selection of W2.

Proposal 4: To mitigate CSI overhead of Rel-17 PS codebook rank 3~4, down-select one from the following Alts:

- Alt 1: the value of beta34 for rank 3 and 4 is smaller than that for rank 1 and 2, i.e. beta34=beta12/2

  - Limit the maximum number of non-zero coefficients across all layers to 2K1*Mv*beta12 and per layer to K1*Mv*beta34

- Alt 2: the value of beta34 for rank 3 and 4 is smaller than that for rank 1 and 2

  - Limit the maximum number of non-zero coefficients across all layers to 2K1*Mv*beta34 and per layer to K1*Mv*beta34

- Alt 3: the value of beta for rank 3 and 4 is the same with that for rank 1 and 2

  - Limit the maximum number of non-zero coefficients across all layers to 2K1*Mv*beta and per layer to K1*Mv*beta

Alt 1: ZTE

Alt 2: Lenovo/Mot, Huawei/Hisi

Alt 3: Qualcomm, ZTE, Vivo, Ericsson, Samsung, Nokia/NSB, DoCoMo, Oppo, Spreadtrum, CATT, LGE, Fraunhofer IIS/Fraunhofer HHI, MTK

Proposal 6: For Rel-17 PS codebook, down-select one from following alternatives:

- Alt 1: Support RI restriction which is the same with Rel-16 PS codebook, i.e., 4 bits are used to indicate the number of ranks respectively.

- Alt 2: RI restriction is not needed for Rel-17 PS codebook

Proposal 7: For Rel-17 PS codebook, down-select one from following alternatives:

- Alt 1: Support CBSR to restrict port and corresponding amplitude

- Alt 2: CBSR to restrict port and corresponding amplitude is not needed

Proposal 9-1: For Rel-17 PS codebook, whether the bitmap for indicating non-zero coefficients can be absent

- Alt 1: the bitmap for indicating non-zero coefficient can be absent

- Alt 2: the bitmap for indicating non-zero coefficient is always present

Alt 1: Lenovo/Mot, Samsung, DOCOMO, Spreadtrum, CATT,

Alt 2: Qualcomm, ZTE, vivo, Ericsson, Nokia/NSB, OPPO, LGE, Fraunhofer IIS/Fraunhofer HHI, MTK

Proposal 13 (updated): If M=2 and N>M, the lower FD index of Wf is always assumed to be 0 and not reported. The higher FD index of Wf is reported by using ceiling (log2(N-1)) bits

- Note: The phase shift/remapping of FD basis is up to UE implementation

Yes: Qualcomm, Lenovo/Mot, ZTE (with note), Ericsson, Samsung, DoCoMo, OPPO, Spreadtrum, CATT (with note), LGE (with note), Fraunhofer IIS/Fraunhofer HHI, MediaTek

No: Nokia (Not ok with Note)

# Summary of CSI enhancement for MTRP

Proposal 22-1: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support configuring RI restriction for Multi-TRP measurement hypothesis.

- FFS detailed solution in RAN1 106bis.

Yes: Lenovo/Mot, vivo, QC, Nokia/NSB, Futurewei, InterDigital, NEC, DOCOMO, OPPO, Spreadtrum, LGE (Alt 1 secondly), CATT, CMCC

No: ZTE, Ericsson

Proposal 23-1: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting,

- Alt 1: CBSR is supported and can be applied for both single-TRP and Multi-TRP measurement hypotheses.

  - FFS how CBSR is configured as legacy or with dedicated RRC signing for Multi-TRP

- Alt 2: CBSR is only applied for single-TRP measurement hypotheses and CBSR for Multi-TRP measurement hypotheses is not needed/applied.

Alt 1:  Lenovo/Mot, vivo, QC, InterDigital, NEC, DOCOMO, OPPO, Spreadtrum, Fraunhofer IIS/

Fraunhofer HHI, CATT, CMCC

Alt 2: Nokia/NSB, ZTE

Proposal 25: Parent IE of RRC parameters configuring “N CMR pairs”, “Two CMR groups” and RRC parameter “sharedCMR” is

- Alt 1: CSI-ReportConfig

- Alt 2: NZP-CSI-RS-ResourceSet

Alt 1: Lenovo/Mot, Samsung, QC, Nokia/NSB, Futurewei, InterDigital, OPPO, Spreadtrum, LGE, CATT, Ericsson

Alt 2: vivo, ZTE, DOCOMO, CMCC