3GPP TSG RAN WG1 #108-e			R1-2200894

e-Meeting, February 21st – March 3rd, 2022

3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200081

Electronic Meeting, January 17-25, 2022

Title	LS for the channelization for up to 71 GHz

Release:	Release 17

Work Item:	NR_ext_to_71GHz-Core

Source:	RAN4

To:	RAN1

Cc:	-

Contact person:

- Name:		Huiping Shan

- E-mail Address:	shanhuiping [at] catt [dot] cn

1. Overall description

RAN4 has discussed the channelization and sync raster design for several meetings and reached the following agreements in RAN4#101-bis-e meeting.

Agreement: Consider the different channelization for licensed band(s) and unlicensed band(s)

- Fixed sync raster for unlicensed bands

  - Fixed scheme should not be constrained by IEEE channel raster

  - Send LS to RAN1 to make sure that RAN1 accommodates the solution already now for both fixed and floating sync raster.

  - For the contiguous carrier aggregation, the channel spacing of adjacent channels should be multiple of the larger SCS, i.e., 960KHz, used by two channels/CCs

- Floating sync raster for licensed bands

- Stick to the agreement last meeting for 3 x 17.28MHz as the minimum granularity.

  - Refer to gap between adjacent GSCN values is not smaller than 3 x 17.28MHz

- FFS: Unlicensed bands tries to use the sub-set of sync raster for licensed bands

In order to facilitate discussions in RAN1, RAN4 would like to also note for licensed operations the minimum granularity, RAN4 is considering 3 x 17.28 MHz for 120 kHz, 12 x 17.28 MHz for 480 kHz and 6 x 17.28 MHz for 960 kHz. RAN1 may use this information for further discussions.

2. Actions

To RAN1 group:

ACTION: 	RAN4 respectfully asks RAN1 to take the above information into account in the conclusions related to both fixed and floating channelization.

3. Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #102-e	21th February – 3rd March 2022 	Online, E-meeting

TSG RAN WG4 Meeting #103-e	16th May – 27th May 2022 	Online, E-meeting