3GPP TSG RAN WG1 #108-e			R1-2200895

e-Meeting, February 21st – March 3rd, 2022

3GPP TSG RAN WG4 Meeting #101bis-e 		R4-2202413

Electronic Meeting, Jan. 17 – 25, 2022

Title:	LS on Rel-17 FeMIMO SRS related impact

Response to:

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	RAN4

To:	RAN1

Cc:	-

Contact person:

#### Name:	Xiang Gao

#### E-mail Address:	gaoxiang74@huawei.com

Attachments: None

# 1	Overall description

In RAN4#101bis-e meeting, SRS related issues were raised in [1]. To be specific, in Rel-17 UE is allowed to indicate its capability on SRS transmission starting on any OFDM symbol within the slot. Then several OFDM symbols gap can exist between two SRS resource sets. Consequently, it is required to clarify that whether UE is allowed to transmit other UL signals during the gap. A figure is provided as below to better understand the scenario:

Furthermore, if it is allowed to transmit other UL signals during the gap between two SRS resource sets, when the Tx antenna for SRS and PUSCH/PUCCH is different, whether the time masks defined in TS 38.101-1 clause 6.3.3.7 is sufficient needs to be clarified. RAN4 has discussed on the following questions considering the aforementioned two clarification points:

Question 1: Is UE expected to transmit other signals between SRSs belonging to different set where usage is set to antenna switching?

Question 2: Should there be a guard period defined in RAN1 specs or a switching time defined in ran4 specs in between SRSs and other signals?

It is noticed that during RAN1#107-e meeting, similar issues have been discussed as summarized in [2]. Thus RAN4 would like to inform RAN1 the following initial discussion outcomes:

- RAN4 thinks not sending PUSCH and PUCCH between SRS resources belonging to different sets where usage is set to antenna switching is inefficient.

- RAN4 clarifies that the transient period between SRS resources is 15us.

Further, RAN4 will continue working on these issues.

# 2	Actions

To RAN1

ACTION: 	RAN4 respectfully asks RAN1 to consider the above information during their discussion.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #102-e		Feb. 21 – Mar. 3, 2022		Electronic Meeting

TSG RAN WG4 Meeting #103-e		May 16 – May 27, 2022		Electronic Meeting

# 4 References

- R4-2200342, Two sets of SRS sets and GP needed in between, Qualcomm, RAN4#101bis-e

- R1-2112589, FL summary #2 on SRS enhancements, Moderator (ZTE), RAN1#107e