3GPP TSG-RAN WG1 Meeting #108-e 	R1-2200921

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.5.2

Source:	Huawei, HiSilicon

Title:	Correction to SRS-RSRP and SRS-RSRPP

Document for:	Discussion and decision

Introduction

- In this paper, we discuss the remaining issues of the definition of SRS-RSRP and SRS-SRSPP.

Discussion

Based on the LS from RAN4 [1], we think that the reference point of SRS-RSRP and SRS-RSRPP needs to be revised according to different gNB types.

Working assumption: The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [1]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [1]: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104 [1]: the Rx Transceiver Array Boundary connector.

We think the previous working assumption should updated as below.

- Proposal 1: Revert the working assumption and update the reference as below.

Agreement Apply the following changes to the definition for UL SRS-RSRPP in the previous agreement: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. - The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104: the Rx Transceiver Array Boundary connector. - FFS: For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRPP value shall not be lower than the corresponding UL SRS-RSRPP of any of the individual receiver branches - FFS: Note: First and additional paths RSRP, when provided in the same report, use the same RX branch(es) selected for the first arrival path and for the UL SRS-RSRP if the UL SRS-RSRP is reported - FFS: whether/how to capture the note in the specifications Note: The following two options are supported by gNB to LMF: - Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for UL SRS-RSRP) - The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

- Proposal 2: Agree to the following TP to section 5.2.5 and 5.2.6 of TS 38.215.

### 5.2.5 UL SRS reference signal received power (UL SRS-RSRP) Definition; UL SRS reference signal received power (UL SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). UL SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. The reference point for UL SRS-RSRP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS-RSRP of any of the individual receiver branches.. ### 5.2.6 UL SRS reference signal received path power (UL SRS-RSRPP) Definition; UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path..

In addition, according to the LS from RAN4 on DL-PRS RSRPP, we think the definition should also revised to reflect how the per-RE power is measured.

For detailed discussion, please refer to our companion contribution [3] [4].

- Proposal 3: Agree to the following TP to section 5.2.6 of TS 38.215.

### 5.2.6 UL SRS reference signal received path power (UL SRS-RSRPP) Definition; UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power (in [W]) of the linear average of the channel response compensated by the i-th path delay on the resource elements that carry the UL SRS signals, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path..

Conclusion

In this contribution, we have the following proposals with regards to the UL RSRP and UL RSRPP definition.

- Proposal 1: Revert the working assumption and update the reference as below.

Agreement Apply the following changes to the definition for UL SRS-RSRPP in the previous agreement: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. - The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104: based on the combined signal from antenna elements corresponding to a given receiver branch, - for type 1-H base station TS 38.104: the Rx Transceiver Array Boundary connector. - FFS: For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRPP value shall not be lower than the corresponding UL SRS-RSRPP of any of the individual receiver branches - FFS: Note: First and additional paths RSRP, when provided in the same report, use the same RX branch(es) selected for the first arrival path and for the UL SRS-RSRP if the UL SRS-RSRP is reported - FFS: whether/how to capture the note in the specifications Note: The following two options are supported by gNB to LMF: - Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for UL SRS-RSRP) - The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

- Proposal 2: Agree to the following TP to section 5.2.5 and 5.2.6 of TS 38.215.

### 5.2.5 UL SRS reference signal received power (UL SRS-RSRP) Definition; UL SRS reference signal received power (UL SRS-RSRP) is defined as linear average of the power contributions (in [W]) of the resource elements carrying sounding reference signals (SRS). UL SRS-RSRP shall be measured over the configured resource elements within the considered measurement frequency bandwidth in the configured measurement time occasions. The reference point for UL SRS-RSRP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS-RSRP of any of the individual receiver branches.. ### 5.2.6 UL SRS reference signal received path power (UL SRS-RSRPP) Definition; UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path..

- Proposal 3: Agree to the following TP to section 5.2.6 of TS 38.215.

### 5.2.6 UL SRS reference signal received path power (UL SRS-RSRPP) Definition; UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power (in [W]) of the linear average of the channel response compensated by the i-th path delay on the resource elements that carry the UL SRS signals, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path..

- Reference

- R1-2200901, LS reply on UL SRS-RSRPP definition, RAN4 (Ericsson) , RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar., 2022.

- R1-2200905, Reply LS on reporting and definition of DL PRS path RSRP, RAN4 (Nokia), RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar., 2022.

- R1-2200922, Maintenance of DL-AoD enhancements, Huawei, HiSilicon, RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar. 2022.

- R1-2202461, Discussion on DL PRS-RSRPP definition, Huawei, HiSilicon, RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar. 2022.