3GPP TSG RAN WG1 Meeting #108-e	R1-2200930

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.1.2.1

Source:	Huawei, HiSilicon

Title:	Remaining issues on multi-TRP for reliability and robustness in Rel-17

Document for:	Discussion and Decision

Introduction

In this contribution, remaining issues of supporting multi-TRP for reliability and robustness in Rel-17 are summarized with corresponding text proposals.

Discussion

Remaining issue for span-based PDCCH repetition

For Rel-16 URLLC, the span-based PDCCH monitoring is introduced. In order to reduce the implementation complexity, the following conclusion was achieved in RAN1#103-e meeting [1] that the UE is not required to do the CCE/BD counting in spans except the first one within a slot, including the determination of whether overlapped candidates are counted for monitoring.

Conclusion: - UE may not do the CCE/BD counting for the purpose of dropping in spans except the first one within a slot for Rel-16 PDCCH monitoring capability on the primary cell, as defined by the following two paragraphs in TS 38.213. A PDCCH candidate with index msj,nCI for a search space set Sj using a set of CCEs in a CORESETp on the active DL BWP for serving cell nCI is not counted for monitoring if there is a PDCCH candidate with index msi,nCI for a search space set Si<Sj, or if there is a PDCCH candidate with index nsj,nCI and nsj,nCI<msi,nCI, in the CORESET p on the active DL BWP for serving cell nCI using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index msj,nCI is counted for monitoring. … CCEs for PDCCH candidates are non-overlapped if they correspond to - different CORESET indexes, or - different first symbols for the reception of the respective PDCCH candidates..

In Rel-17, the CCE/BD counting has been enhanced with the following agreement, to handle the case of overlapping between individual candidate and linked candidates. The overlapping individual candidate is not counted and whether it’s monitored depends on UE capability. To avoid any ambiguity, the interpretation of overlapping candidates is based on Rel-17 PDCCH rule, no matter it’s transmitted in the individual candidate or linked candidate.

Agreement When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET - Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate). - Whether the individual candidate is monitored or not is determined by a UE capability - FFS (In UE feature session): The details including reusing the reported number of BDs for this purpose, or relation to reported number of BDs - In both cases, the individual candidate is not counted toward the BD limit. - UE capability for max number of such overlaps is introduced - FFS: Value of 0 is included as a candidate value for the UE capability - The details to be discussed as part of UE capability discussions - FFS: When the individual candidate is monitored, the scenario where the other linked candidate is also “overlapping” (same CORESET, DCI size, CCEs, scrambling) with a second individual candidate

However, with the above conclusion of Rel-16 span based PDCCH monitoring, the behaviours for the first span and the remaining spans are different:

  - In the first span, the UE is able to determine whether the overlapped individual candidate is counted, and whether is monitored or not if the UE supports;

  - In the remaining spans, the UE does not do the CCE/BD counting. Therefore, the UE just monitors all the configured candidates and does not determine the overlapping of the candidates.

As a result, in the spans except the first one, if one DCI is transmitted on a linked candidate overlapped with an individual candidate, which are of the same CCE sets, the same DCI size and the same scrambling, the UE may perform decoding twice and detects two DCIs in the PDCCH candidates from the same DCI, with one considered as individual candidate, and the other considered as linked candidate. Then there will be ambiguity on timing reference for the two DCIs, as gNB assumes they should follow the Rel-17 PDCCH repetition rule, while the UE assumes one as Rel-17 PDCCH repetition and an additional one as legacy Rel-15/16 PDCCH transmission.

In order to address the ambiguity issue while not increasing UE complexity, we have the following proposal.

Proposal 1: UE does not expect one of the linked PDCCH candidates overlapping with an individual PDCCH candidate in spans except for the first span in a slot, when the candidates use the same set of CCEs in the same CORESET and have the same scrambling and DCI size.

SRI based power control mechanism for PUSCH repetition with mTRP

In R17, dynamic switching between sTRP and mTRP based reception is supported. In current spec, when two SRI fields and related two SRS resource sets are configured, separate power control parameters would be used according to the related two SRIs, if mTRP based PUSCH transmission is used (SRS resource set indicator set to “10” or “11”). While when single TRP based PUSCH is used (SRS resource set indicator set to “00” or “01”), only one set of power control parameters is used. So current spec of section 7.1.1 in TS 38.213 should be updated:

Section 7.1.1 of TS 38.213 - if the DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the two SRI fields and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines first and second values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values that are mapped to the values of the first and second SRI fields, respectively. - if the DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between < Unchanged parts are omitted > - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' - if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the two SRI values corresponding to each SRS resource set with usage set to ‘codebook’. - if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the two SRI values corresponding to each SRS resource set with usage set to ‘codebook’, respectively. - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', - if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI field value corresponding to the first SRS resource set with usage set to ‘nonCodebook’ and to a second value, that is associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to ‘nonCodebook’. < Unchanged parts are omitted > - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the two SRI fields and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first and second values of αb,f,cj from the P0-PUSCH-AlphaSetID values that are mapped to the values of the first and second SRI field values, respectively. - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first value of αb,f,cj from the P0-PUSCH-AlphaSetID value that is mapped to the first SRI field value, and - the second value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value

Proposal 2: Endorse the above text proposal.

Conclusion

This contribution has provided the discussion of remaining issues on reliability/robustness enhancements using multi-TRP in Rel-17, and made following proposals:

Proposal 1: UE does not expect one of the linked PDCCH candidates overlapping with an individual PDCCH candidate in spans except for the first span in a slot, when the candidates use the same set of CCEs in the same CORESET and have the same scrambling and DCI size.

Proposal 2: Endorse the text proposal in section 2.2.

- References

- Chairman's Notes RAN1#103-e final.