3GPP TSG-RAN WG1 Meeting #108-e                                          R1-2200932

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.1.2.3

Source:	Huawei, HiSilicon

Title:	Remaining issues on beam management for multi-TRP in Rel-17

Document for:	Discussion and Decision

- Introduction

In previous meetings, group-based beam reporting for M-TRP was supported. In this paper, we further provide our views on some critical issues.

- Technique proposals

In group-based beam reporting for M-TRP, UE can report up to 4 beam groups to the gNB. The beams in a beam group can be received simultaneously by the UE via the same or different Rx beams. It is not knowable for the gNB whether the beams in a beam group are received with the same or different Rx beams. Without such information, the flexibility of implementation of gNB will be restricted. For example, gNB does not know whether it is proper to conduct multiplexing or diversity-based transmission with a beam group as it does not know how many Rx beams are used at the UE side.

Proposal 1: Support UE reporting whether the same or different Rx beams are used for the two beams in each reported beam group.

- Text proposals

  - Triggering of RACH based BFR

Reason for changes

It was agreed [1] that when beam failure is detected on both BFD-RS sets of SpCell, RACH based BFR procedure can be triggered. However, this is not captured in current spec.

Agreement

RACH-based transmission can be triggered on a SpCell at least in the following scenarios

- Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell

Summary of changes

Capture the missed agreement.

Consequences if not approved

The functionality is incomplete. The UE does not know what to do when both BFD-RS sets of SpCell are detected with beam failure.

To summarize, we have the following proposal.

Proposal 2: Capture the missed agreement on RACH based BFR as shown in TP #1.

TP #1

< Start of text proposal 38.213 v17.0.0 Section 6> < Unchanged parts are omitted > For serving cells associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0,0 and/or q0,1 having radio link quality worse than Qout,LR, the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from corresponding sets q1,0 and/or q1,1 for the serving cells. For SpCell associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, when both q0,0 and q0,1 have radio link quality worse than Qout,LR, the UE can trigger RACH procedure for beam failure recovery. < Unchanged parts are omitted > < End of text proposal 38.213 v17.0.0 Section 6>

- Conclusion

The proposals of this paper are summarized as follows.

Proposal 1: Support UE reporting whether the same or different Rx beams are used for the two beams in each reported beam group.

Proposal 2: Capture the missed agreement on RACH based BFR as shown in TP #1.

- References

“3GPP RAN1#106bis-e Chair’s Notes”, December 11th – December 19th, 2021.