3GPP TSG RAN WG1 Meeting #108-e	    R1-2200947

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.16.7

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for UE power saving enhancements

Document for:	Discussion and Decision

- Introduction

This contribution includes our views on UE features for Rel-17 UE power saving enhancement based on RAN1 agreements/conclusions to address remaining issues for UE power saving features since RAN1#107-bis meeting.

- Discussion

- UE feature for paging enhancement (UE feature 29-1)

In RAN1#107bis, the following UE feature 29-1 was agreed to be introduced. Regarding the details highlighted by yellow, it is still FFS.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |

Regarding the type of the UE feature, “per band” type is preferred considering it could reduce the IODT work to deploy the feature. Even if the UE type is defined as “per UE”, it should be at least “per UE” with FR1/FR2 differentiation.

Regarding the capability of monitoring PDCCH for DCI format 2_7, the common understanding is based on the legacy mandatory UE capability, i.e. FG 3-1 in 38.822.

The related description in FG 3-1 is as following:

“- For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot”

There is no description about the new agreed Type2A CSS for PEI PDCCH in the FG3-1. To keep no touch on FG 3-1, which is stable from Rel-15, it is proposed to add a similar sentence to FG 29-1 to capture the above UE mandatory capability to support 29-1:

“- For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot”

Proposal 1: Make the following update for UE feature 29-1:

- Update the UE feature 29-1 as ‘per band’ or “per UE” with FR1/FR2 differentiation.

- Add a component 3 to capture the PDCCH monitoring of type2A-CSS based on legacy UE capability FG3-1: “For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot”.

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication 3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot |  |  |  | UE does not support paging enhancement | Per UE Per band | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- TRS resources for idle/inactive UEs (UE feature 29-2)

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

It was agreed in RAN1#107-e that the UE feature 29-2 is introduced with some FFS details of highlight, as shown above. We have the following proposals for the FFS part of the UE feature:

- A controversial discussion is whether to take FG 29-1 as the pre-requisite feature. To move forward as a compromise, it is proposed to update the component 2 as “Support receiving L1 indication for TRS availability via DCI format 2_7 and DCI format 1_0” if we could not achieve consensus on the pre-requisite feature.

- We are fine to update the feature group as “optional without capability signaling”.

- The UE feature 29-2 should be ‘per band’, considering it could accelerate the deployment of the feature. Even if the UE feature is defined as “per UE” type, it should be at least with FR1/FR2 differentiation. However, if we could agree the FG is “Optional without capability signaling”, it seems no need to discuss the UE type and we could simply leave the column blank.

- ‘Consequence if the feature is not supported by the UE’ can be updated to “UE does not support TRS resources for idle/inactive UEs”.

Proposal 2: Make the following update on the FFS part of UE feature 29-2:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability via DCI format 2_7 and DCI format 1_0 |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE does not support TRS resources for idle/inactive UEs | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- PDCCH monitoring adaptation within an active BWP (29-3)

The current UE features for PDCCH monitoring adaptation are shown below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

The following updates are suggested:

- Remove the highlight of component of 29-3a, which seems not controversial.

- “per band” features are preferred, considering it could accelaerate deployment of the feature on some bands. Even if the UE feature is defined as “per UE” type, it should be at least with FR1/FR2 differentiation.

Proposal 3: Make the following update on the FFS part of UE feature 29-3:

- Remove the highlight of component of 29-3a to endorse the component description.

- The UE features of 29-3a, 29-3b, 29-3c and 29-3d are ‘per band’ UE feature or “per UE” with FR1/FR2 differentiation.

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE Per band | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE Per band | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching | 29-3b | Y |  |  | Per UE Per band | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-3b | Y |  |  | Per UE Per band | N | N | N |  | Optional with capability signaling |

- Conclusion

In this contribution, we have following proposals on UE features for Rel-17 UE power saving enhancement:

Proposal 1: Make the following update for UE feature 29-1:

- Update the UE feature 29-1 as ‘per band’ or “per UE” with FR1/FR2 differentiation.

- Add a component 3 to capture the PDCCH monitoring of type2A-CSS based on legacy UE capability FG3-1: “For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot”.

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication 3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot |  |  |  | UE does not support paging enhancement | Per UE Per band | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Make the following update on the FFS part of UE feature 29-2:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability via DCI format 2_7 and DCI format 1_0 |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE does not support TRS resources for idle/inactive UEs | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Make the following update on the FFS part of UE feature 29-3:

- Remove the highlight of component of 29-3a to endorse the component description.

- The UE features of 29-3a, 29-3b, 29-3c and 29-3d are ‘per band’ UE feature or “per UE” with FR1/FR2 differentiation.

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE Per band | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE Per band | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching | 29-3b | Y |  |  | Per UE Per band | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-3b | Y |  |  | Per UE Per band | N | N | N |  | Optional with capability signaling |

- References

- R1-2200780	Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e , RAN1#107-bis

- R1-2200678, Summary on UE features for UE power saving enhancements, Moderator (NTT DOCOMO, INC.)

- Chair’s note of RAN1#107b-e.